3GPP TSG-RAN WG4 meeting #80	R4-165645

Göteborg, Sweden, 22 – 26 August, 2016

Source:	Ericsson

Title:	TP to TR 36.714-03-01: UE requirements for CA_2A-7A-66A

Agenda item:	8.3.2

Document for:	Approval

# 1. Introduction

This is a text proposal for 36.714-03-01 for introduction of 3DL/1UL CA combination CA_2A-7A-66A.

# 2. Text Proposal for TR 36.714-03-01

##### ---Start of changes---

### 6.50.4	∆TIB and ∆RIB values

For deriving ΔTIB,c and ΔRIB,c values for Band (2 + 7 + 66) we use filter data based on a hexplexer implemented in FBAR technology, but also make a comparison to the 2DL band combinations Band (2 + 7), Band (2 + 66) and Band (4 + 7) already specified in view of other implementations. The first two of these 2DL combinations are also fallback modes of Band (2 + 7 + 66). The requirements for the 2DL fallback modes should not be more stringent than the corresponding 2DL combinations for UE supporting the 3DL combination Band (2 + 7 + 66), even though the filter technologies improve. Furthermore, most UE support a multiple of combinations so filter replication is a challenge.

The hexplexer considered also supports operation in the G-block above Band 2 and thus operation in Band 25, which is used in North America and parts of Latin America. Hence the requirements for Band (2 + 7 + 66) thus derived would also be relevant for Band 25 and Band (7 + 25 + 66) operation and a larger fleet of UEs, which also facilitates component reuse. In fact, nowadays many duplexers for Band 2 are also designed to support Band 25.

Figure 6.50.4-1 displays the broadband response of a hexplexer supporting both Band (2 + 7 + 66) and Band (7 + 25 + 66). The plots below represent the performance of typical filters at room temperature (25C). However, reading the plots 5 MHz outside the nominal frequency band corresponds fairly well to extended temperature conditions (ETC) for -20 to +85 C. The TX and RX passband responses for B2 (B25), B7 and B66 are shown in detail for frequencies up to 5 MHz outside the nominal frequency range in Figure 6.50.4-2, Figure 6.50.4-3 and Figure 6.50.4-4, respectively.

Figure 6.50.4-1: broadband response for a Band (7 + 25 + 66) FBAR hexplexer with B25TX (pink), B25RX (light blue), B7TX (brown), B7RX (navy) and B66TX (red) and B66RX (blue).

Figure 6.50.4-2: traces for B25TX/B2TX (left) and B25RX/B2RX (right) at room temperature.

Figure 6.50.4-3: traces for B7TX (left) and B7RX (right) at room temperature.

Figure 6.50.4-4: traces for B66TX (left) and B66RX (right) at room temperature.

In order to estimate the ΔTIB,c and ΔRIB,c we compare the maximum IL 5 MHz outside the passband for each band displayed in the figures above to the specified IL for commercial B2 and B4 and B7 duplexers also implemented in FBAR. The results are shown in Table 6.50.4-1 and Table 6.50.4-2 with the “IL quadplexer” obtained by reading 5 MHz outside the nominal frequency range in the figures above and the “IL duplexer” the quoted minimum performance at ETC for the corresponding duplexer (in production). The duplexer data for Band 66 represent estimates for a pre-commercial product. The estimated ΔTIB,c and ΔRIB,c are obtained by using the shared-pain rule (50% of the IL), the values in brackets are the maximum values for the existing 2DL combinations Band (2 + 7) and Band (2 + 66) shown in Table 6.50.4-3.

Table 6.50.4-1: TX insertion loss across temperature

| Inter-band CA Configuration | E-UTRA Band | IL quadplexer [dB] | IL duplexer [dB] | Estimated ΔTIB,c [dB] |
| --- | --- | --- | --- | --- |
| CA_2A-7A-66A | 2 (25) | 3.4 | 3.0 | 0.2 (0.5) |
|  | 7 | 3.1 | 3.0 | 0.1 (0.5) |
|  | 66 | 3.4 | 2.2 | 0.6 (0.5) |

Table 6.50.4-2: RX insertion loss across temperature

| Inter-band CA Configuration | E-UTRA Band | IL quadplexer [dB] | IL duplexer [dB] | Estimated ΔRIB,c [dB] |
| --- | --- | --- | --- | --- |
| CA_2A-7A-66A | 2 (25) | 3.8 | 3.8 | 0 (0.3) |
|  | 7 | 2.6 | 3.0 | 0 (0) |
|  | 66 | 2.8 | 2.6 | 0.1 (0.3) |

Table 6.50.4-3: Performance for other 2DL and 3DL combiantions used for deriving ΔTIB,c and ΔRIB,c values

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] | ΔRIB,c [dB] |
| --- | --- | --- | --- |
| CA_2A-7A | 2 | 0.5 | 0 |
|  | 7 | 0.5 | 0 |
| CA_4A-7C | 4 | 0.5 | 0.5 |
|  | 7 | 0.5 | 0.5 |
| CA_2A-66A | 2 | 0.5 | 0.3 |
|  | 66 | 0.5 | 0.3 |
| CA_2A-4A-7A | 2 | 0.5 | 0.3 |
|  | 4 | 0.5 | 0.5 |
|  | 7 | 0.5 | 0.5 |

The requirement for a constituent band of a 2DL mode of the 3DL combination should not be more stringent than the requirement for the corresponding “standalone” requirements for a UE not supporting the 3DL combination.

The requirements for Band (2 + 7 + 66) above would also be relevant for Band (7 + 25 + 66) that includes the G-block.

Other technologies than FBAR need to be considered to avoid dependency a specific filter technology. Therefore, it is proposed to re-use the requirements from existing combination 4A-7A, see requirements in Table 6.50.4-4 and Table 6.50.4-5.

Table 6.50.4-4: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| Ca_2A-7A-66A | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |

Table 6.50.4-5: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| Ca_2A-7A-66A | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |

##### ---End of changes---