3GPP TSG-RAN WG4 Meeting #80	                                   R4-165754

# Gothenburg, Sweden, 22 - 26 Aug 2016

Source: 	CATT

Title: 	Further consideration on 5G NR co-existence study

Agenda Item:	10.3.1

Document for:	Discussion

Introduction

- In last RAN4 meeting, some simulation assumptions for co-existence study for WP 5D were discussed and summarised in the WF [1]. After the meeting, there were some e-mail discussions among interested companies in order to calibrate the simulation results.

In this contribution, we provide further considerations on the simulation assumptions based on the agreed WF [1] and RAN1 discussion.

Simulation assumptions

General assumptions

- The simulation assumptions used in our simulation are illustrated in Table 2.1-1. Considering the current stage of the discussion in RAN1 and RAN4 email reflector, only parameters for 30GHz is proposed in this paper .

Table 2.1-1 general assumptions

| Carrier frequency | 30GHz |  |  |
| --- | --- | --- | --- |
| Victim system carrier frequency | 30.2GHz |  |  |
| Channel Bandwidth | 200MHz |  |  |
| ALCR/ACS model | The same as in TR 36.942 |  |  |
| Evaluation metrics | Throughput degradation compared to single operator case, i.e. no ACI. Both mean and 5%-tile are considered. |  |  |
| Link-level performance model | Scaled Shannon’s formula in TR 36.942 before RAN1’s result |  |  |
| Deployment scenario | Urban Macro | Dense Urban | Indoor Hotspot |
| Network layout | Hex. Grid with 19 sites, tri-sector (wrap-around) | Micro layer: Manhattan grid pattern | Indoor floor: 12BSs per 120m x 50m |
| ISD | 500m |  | 20m |
| System loading | Full buffer | Full buffer | Full buffer |
| DL power control | No | No | No |
| UL power control | Using TR 36.942 [2] as starting point, and further evaluate whether the parameter sets are applicable for >6GHz | Using TR 36.942 [2] as starting point, and further evaluate whether the parameter sets are applicable for >6GHz | Using TR 36.942 [2] as starting point, and further evaluate whether the parameter sets are applicable for >6GHz |
| Victim system location | Uncoordinated | Uncoordinated | Uncoordinated |
| No. of UEs per cell (DL) | 10 | 10 | 10 |
| No. of UEs per cell (UL) | 10 | 10 | 10 |
| Penetration loss for O-2-I UE | 50% low loss, 50% high loss (80% indoor UE) | 50% low loss, 50% high loss (80% indoor UE) | 0 |
| Shadowing fading standard deviation | The same as TR 38.900 [4], described in section 2.2 | The same as TR 38.900 [4], described in section 2.2 | The same as TR 38.900 [4], described in section 2.2 |
| Shadowing correlation | Between cells: 0.5; Between sites:1.0 | FFS | No |
| Path-loss model | The same as TR 38.900 [4], described in section 2.2 | The same as TR 38.900 [4], described in section 2.2 | The same as TR 38.900 [4], described in section 2.2 |
| Minimum coupling loss | 70 dB as a starting point | 53Db as a starting point | 45dB as a starting point |
| Handover margin | 0 dB for calibration | 0 dB for calibration | 0 dB for calibration |
| Maximum Tx power | BS: 46dBm UE: 23dBm | BS: 33dBm UE: 23dBm | BS: 23dBm UE: 23dBm |
| Antenna height | BS: 25m UE: 1.5m | BS: 10m UE: 1.5m | BS: 3m UE:1.5m |
| Beamforming pattern | BS: The same as TR 37.842 [3], described in section 2.4 UE: FFS | BS: The same as TR 37.842 [3], described in section 2.4 UE: FFS | BS: The same as TR 37.842 [3], described in section 2.4 UE: FFS |
| Number of Beamforming pattern per cell | BS: 10 UE: 1 | BS: 10 UE: 1 | BS: 10 UE: 1 |
| Maximum antenna gain | BS: 8dBi UE: 0dBi | BS: 6dBi UE: 0dBi | BS: 0dBi UE: 0dBi |
| Noise figure | BS: 8dBm (Take 8dBm as the starting point) UE: 13dBm | BS: 8dBm (Take 8dBm as the starting point) UE: 13dBm | BS: 8dBm (Take 8dBm as the starting point) UE: 13dBm |

Path-loss model

The path-loss models and LOS probability are the same as TR 38.900 [4] and provided in Table 2.2-1 and Table 2.2-2, respectively.

Table 2.2-1: Path loss models [4]

| Scenario | Path loss [dB], fc is in GHz and d is in meters (6) |  | Shadow fading std [dB] | Applicability range, antenna height default values |
| --- | --- | --- | --- | --- |
| RMa LOS |  |  |  | 10 m < d2D < dBP (5) dBP < d2D < 10 000 m, hBS = 35 m, hUT = 1.5 m, W = 20 m, h = 5 m h = avg. building height W = avg. street width The applicability ranges: 5 m < h < 50 m 5 m < W < 50 m 10 m < hBS < 150 m 1 m < hUT < 10 m |
| RMa NLOS |  |  |  | 10 m < d2D < 5 000 m, hBS = 35 m, hUT = 1.5 m, W = 20 m, h = 5 m h = avg. building height W = avg. street width The applicability ranges: 5 m < h < 50 m 5 m < W < 50 m 10 m < hBS < 150 m 1 m < hUT < 10 m |
| UMa LOS |  |  | σSF=4.0 σSF=4.0 | 10m < d2D < d'BP 1) d'BP < d2D <5000m 1.5m ≦ hUT≦ 22.5m hBS = 25 m |
| UMa NLOS |  |  | σSF =6 | 10 m < d2D < 5 000 m 1.5 m ≦ hUT ≦ 22.5 m hBS = 25 m Explanations: see note 3 |
|  | Optional |  | σSF =7.8 |  |
| UMi - Street Canyon LOS |  |  | σSF=4.0 σSF=4.0 | 10m < d2D < d'BP 1) d'BP < d2D <5000m 1.5m ≦ hUT≦ 22.5m hBS = 10 m |
| UMi – Street Canyon NLOS |  |  | σSF=7.82 | 10 m < d2D < 5000m 1.5m ≦ hUT≦ 22.5m hBS = 10 m Explanations: see note 4 |
|  | Optional |  | σSF=8.2 |  |
| InH - Office LOS |  |  | σSF=3.0 | 1<d3D<100m |
| InH - Office NLOS |  |  | σSF=8.03 | 1<d3D<86m |
|  | Optional |  | σSF=8.29 | 1< d3D <86m |
| InH - Shopping mall LOS |  |  | σSF=2.0 | 1<d3D<150m |
| Note 1: d'BP = 4 h'BS h'UT fc/c, where fc is the centre frequency in Hz, c = 3.0108 m/s is the propagation velocity in free space, and h'BS and h'UT are the effective antenna heights at the BS and the UT, respectively. In UMi scenario the effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – 1.0 m, h'UT = hUT–1.0 m, where hBS and hUT are the actual antenna heights, and the effective environment height is assumed to be equal to 1.0 m. In UMa scenario the effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – hE, h'UT = hUT – hE, where hBS and hUT are the actual antenna heights, and the effective environment height hE is a function of the link between a BS and a UT. In the event that the link is determined to be LOS, hE=1m with a probability equal to 1/(1+C(d2D, hUT)) and chosen from a discrete uniform distribution uniform(12,15,…,(hUT-1.5)) otherwise. Note 2: The applicable frequency range of the PL formula in this table is 0.8 < fc < fH GHz, where fH = 30 GHz for RMa and fH = 100 GHz for all the other scenarios. It is noted that RMa path loss model for >7 GHz is validated based on a single measurement campaign conducted at 24 GHz. Note 3: UMa NLOS path loss is from TR36.873 with simplified format and and PLUMa-LOS = Path loss of UMa LOS outdoor scenario. Note 4: PLUMi-LOS = Path loss of UMi-Street Canyon LOS outdoor scenario. Note 5: Break point distance dBP = 2π hBS hUT fc/c, where fc is the centre frequency in Hz, c = 3.0 108 m/s is the propagation velocity in free space, and hBS and hUT are the antenna heights at the BS and the UT, respectively. Note 6: fc denotes the center frequency normalized by 1GHz, all distance related values are normalized by 1m, unless it is stated otherwise. |  |  |  |  |

Table 2.2-2 LOS probability [4]

| Scenario | LOS probability (distance is in meters) |
| --- | --- |
| RMa |  |
| UMi – Street canyon | Outdoor users: Indoor users: Use d2D-out in the formula above instead of d2D |
| UMa | Outdoor users: where and Indoor users: Use d2D-out in the formula above instead of d2D |
| Indoor - Mixed office |  |
| Indoor – Open office |  |
| Note: The LOS probability is derived with assuming antenna heights of 3m for indoor, 10m for UMi, and 25m for UMa |  |

Additionally, the indoor UE fraction is assumed to be 80%, and the actual UE height hUT is calculated according to TR 36.873 as listed below:

General equation: 	hUT=3(nfl – 1) + 1.5

nfl for outdoor UEs:  1

nfl for indoor UEs: 	nfl ~ uniform(1,Nfl), where Nfl ~ uniform(4,8)

Antenna radiation pattern

According to TR38.900, the radiation pattern of each antenna element is shown in Table 2.3-1.

Table 2.3-1: Antenna radiation pattern [4]

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 8 dBi |

Beamforming pattern

The beamforming pattern for BS is based on TR 37.842 as the formula illustrated below , where  is the element pattern defined in Table 2.3-1, and and are array factors defined in Table 2.4-1.

Table 2.4-1 Composite antenna pattern for UE specific beam forming [3]

| Configuration | Multiple columns (NVxNH elements) |
| --- | --- |
| Composite Array radiation pattern in dB | For beam i: the super position vector is given by: the weighting is given by: |
| Antenna array configuration (Row×Column) | 10×1 |
| Horizontal radiating element spacing d/ | 0.5 |
| Vertical radiating element spacing d/ | 0.5 |
| Down-tilt angle (deg) | 12 degrees |

UL power control

The UL power control method for further work can be based on TR 36.942 as the formula illustrated below.

where Pmax is the maximum transmit power, Rmin is the minimum power reduction ratio to prevent UEs with good channels to transmit at very low power level, CL is the path coupling loss defined as max{path loss-G_Tx-G_Rx, MCL}, where path loss is propagation loss plus shadowfading, G_TX is the transmitter antenna gain in the direction of the receiver, G_RX is the receiver antenna gain in the direction of the transmitter and CLx-ile is the x-percentile CL value. With this power control equation, the x percent of UEs that have the highest coupling loss will transmit at Pmax.  Finally, 0<<=1 is the balancing factor for UEs with bad channel and UEs with good channel. The parameter sets are specified in Table2.5-1.

Table 2.5-1 Power control algorithm parameter of LTE coexistence [2]

| Parameter Set | Gamma | CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 40MHz | 10MHz |
| Set 1 | 1 | 112- | 112- |
| Set 2 | 0.8 | 129- | 129- |
| Note:, adjustment parameter related to different carrier frequency point. For fc=2GHz, =0dB. |  |  |  |

However, as the parameter sets above are defined for LTE situations, where the path-loss model and carrier frequency are considered, it may not applicable for the new scenarios in 5G. In addition, considering that the above PC parameters are used for macro cells, we need to investigate whether these parameters are still valid for micro cells.

Considerations on BF accuracy

As BF is a very important aspect in 5G NR, it’s better to settle problems as much as possible in BF part. One of the potential problem need to investigate is whether the BF gain should be maximized towards the target UE in the simulations. The maximum BF gain pointing to the target UE is an ideal assumption which is usually not the case in reality considering BF accuracy. Especially for UEs with mobility, there should be some BF gain loss due to the estimation error of the UE positions. Moreover, if we consider not maximizing the BF gain in all situations, something like BF accuracy needs to be defined for different scenarios. Thus, we propose to consider some BR accuracy so that the BF performance in the evaluation is closer to realistic situations. Similar problem needs to be considered at UE side. And compared with BS, the UE antenna will always change its direction due to the user’s moving, which might introduce additional BF accuracy error.

Proposal: To analyse the influence of the BF accuracy in both BS and UE sides for co-existence study.

Conclusions

In this contribution, we provide our considerations on 5G NR co-existence simulation assumptions in 30GHz. Considering that BF is a very important aspect of 5G, we analyzed the potential impact of BF accuracy on co-existence performance. We propose to further analyze the influence of the BF accuracy in both BS and UE sides for co-existence study.

References

- R4-164528, “WF on WP 5D co-existence study simulation assumptions”, NTT-DOCOMO, INC, RAN4#79.

- TR 36.942, “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) system scenarios”.

- TR37.842, “Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)

- TR38.900, “Study on channel model for frequency spectrum above 6 GHz”.