3GPP TSG-RAN WG4 (Radio) Meeting #80	R4- 165939

Gothenburg, Sweden, 22 – 26 August 2016

Agenda Item:	10.3.1

Source: 	Samsung

Title:	Evaluation results for NR coexistence study in dense urban scenario

Document for:	Discussion

- 1.Introduction

Coexistence study of new radio (NR) access technology is essential to determine appropriate RF parameters. This topic was discussed previously in RAN4#78b and RAN4#79 meetings, specifically, some agreed simulation assumptions were made in [1, 2]. Based on those assumptions, this contribution presents simulation evaluation results of NR coexistence study in dense urban scenario. The results illustrate the impact of beamforming on ACIR requirement.

- 2.Discussion

- 2.1 Coexistence simulation case

The uplink NR eMBB is assumed under synchronized network, where aggressor and victim have the same configuration.

Table 2.1-1 Simulation case of coexistence study for NR

| Case | Operation mode | Aggressor | Victim | Direction |
| --- | --- | --- | --- | --- |
| 1 | TDD | UE of NR eMBB | UE of NR eMBB | Uplink |

- 2.2	Cell layouts

- 2.2.1 NodeB

As shown in Fig. 2.1-1, the network deployment is 19 hexagonal sites (NodeBs), each of which is of 3 sectors. In addition, heterogeneous network deployment with wrap around is also implemented.Uncoordinated macro cellular deployment is assumed.The inter-site distance (ISD) is 500 m.

- 2.2.2 Micro Base Station (BS)

In each sector, 3 micro BSs are randomly dropped with minimum distance 32 m.

Figure 2.2-1: Uncoordinated dense urban cellular deployment.

2.2.3 Aggressor Network

NodeBs and micro BSs in the aggressor network are shifted leftward 288.7 m, which is the radius of a cell.

#### 2.3	Pathloss models [3]

The pathloss model for dense urban are summarized in Table 2.3-1 and the distance definitions are indicated in Figure 2.3-1 and Figure 2.3-2. Distribution of shadow fading is log-normal, and its standard deviation for each scenario is given in Table 2.3-1.

| Figure 2.3-1: Definition of d2D and d3D for outdoor UEs | Figure 2.3-2: Definition of d2D-out, d2D-in and d3D-out, d3D-in for indoor UEs. Note that d3D-out+d3D-in=d2D-out+d2D-in2+hBS-hUT2 |
| --- | --- |

Table 2.3-1: Pathloss models

| Scenario | Pathloss [dB], fc is in GHz and d is in meters | Shadow fading std [dB] | Applicability range, antenna height default values |
| --- | --- | --- | --- |
| Dense Urban LOS |  | σSF=4.0 | 10m <d3D<d'BP1) 1.5m ≦hUT≦ 22.5m hBS= 25 m |
|  |  | σSF=4.0 | d'BP<d3D<5000m 1.5m ≦hUT≦ 22.5m hBS= 25 m |
| Dense Urban NLOS |  | σSF =6 | 10 m <d3D< 5 000 m 1.5 m ≦hUT≦ 22.5 m hBS= 25 m |
| Dense Urban PL |  |  |  |
| Note 1: d'BP= 4 h'BSh'UTfc/c, where fc is the centre frequency in Hz, c = 3.0108 m/s is the propagation velocity in free space, and h'BS and h'UT are the effective antenna heights at the BS and the UT, respectively. In Dense Urban scenario the effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – hE, h'UT = hUT – hE, where hBS and hUT are the actual antenna heights, and the effective environment height hE is a function of the link between a BS and a UT. In the event that the link is determined to be LOS, hE=1m with a probability equal to 1/(1+C(d2D, hUT)) and chosen from a discrete uniform distribution uniform(12,15,…,(hUT-1.5)) otherwise. Note 2: The applicable frequency range of the PL formula in this table is 0.8<fc<100GHz. Note 3: Dense Urban NLOS pathloss is the same with Urban Macro in TR36.873 with simplified format. Note 4: fc denotes the center frequency normalized by 1GHz, all distance related values are normalized by 1m, unless it is stated otherwise. |  |  |  |

#### 2.4	Antenna Gain[4]

- 2.4.1 NodeB

The beamforming antenna is based on an antenna array and consists of a number of identical radiating elements located in the yz-plane with a fixed separation distance (e.g. /2), all elements having identical radiation patterns and “pointing” (having maximum directivity) along the x-axis. A weighting function is used to direct the beam in other directions. Total antenna gain is the sum (logarithmic scale) of the array gain and the element gain. Such a model is described in 3GPP TR 37.842 as follows.

The Advanced Antenna System (AAS) antenna array model is determined by array element pattern, array factor and signals applied to the system. The element pattern and composite antenna pattern are in the following sections. The anddefinition is based on the coordinate system in Section 5.4.4 .1 of 3GPP TR 37.840 and are illustrated in Figure 2.4-1.

Figure 2.4-1: Antenna Model Geometry: : elevation, range from -90 to 90 degree
: Azimuth, range from -180 to 180 degree.

The element pattern and the composite antenna pattern are given in Section 5.3.3 of 3GPP TR 37.842 and are provided Table 2.4-1.

Table 2.4-1: Antenna pattern

(a) Element pattern for antenna array model

| Horizontal Radiation Pattern |  |
| --- | --- |
| Horizontal 3dB bandwidth of single element / deg | 80º |
| Front-to-back ratio | Am = 30dB |
| Vertical Pattern method | , SLAv =30 dB |
| Vertical 3dB bandwidth of single element / deg | 65º |
| Element Pattern |  |
| Element Gain without antenna losses | GE,max= 7.5 dBi |
| Note: *GE,max is obtained from TR37.840 Table 5.4.4.2.1-1. |  |

(b) Composite antenna pattern for NodeB beam forming

| Configuration | Multiple columns (NVxNH elements) |
| --- | --- |
| Composite Array radiation pattern in dB | For beam i: the superposition vector is given by: the weighting is given by: |
| Antenna array configuration (Row×Column) | 16×16 |
| Horizontal radiating element spacing d/ | 0.5 |
| Vertical radiating element spacing d/ | 0.9 |
| Down-tilt angle (deg) | 0 degrees |

- 2.4.2 Micro BS

Micro BSs are assumed to have 1 omnidirectional antenna, with constant antenna gain equal to 6 dBi.

- 2.4.3 UE

UEs are assumed to have 4×4 antennas, with 0 dB element antenna gain.

#### 2.5	Uplink Power control modelling [5]

In 3GPP TR 36.942, the following power control equation is used for the uplink coexistence simulations:

where Pmax is the maximum transmit power, Rmin is the minimum power reduction ratio to prevent UEs with good channels to transmit at very low power level, CL is the coupling loss defined as max{path loss-G_Tx-G_Rx, MCL}, where path loss is propagation loss plus shadow fading, G_TX is the transmitter antenna gain in the direction of the receiver, G_RX is the receiver antenna gain in the direction of the transmitter and CLx-ile is the x-percentile CL value. With this power control equation, the x percent of UEs that have the highest coupling loss will transmit at Pmax. Finally, 0<<=1 is the balancing factor for UEs with bad channel and UEs with good channel.

The parameter sets for power control are listed below.

Table 2.5-1: Power control algorithm parameter (for 30 GHz carrier frequency)

| Parameter set | Gamma | CLx-ile 200 MHz bandwidth |  |
| --- | --- | --- | --- |
| Set 1 | 1 | 100 |

#### 2.6	Other simulation assumptions and parameters

Other simulation assumptions and parameter are summarized in Table 2.6-1 below:

Table 2.6-1: Simulation assumptions and parameters

| Parameter | Value |
| --- | --- |
| Methodology | Static Monte Carlo simulation |
| Deployment scenario | Dense Urban |
| Carrier frequency | 30 GHz |
| Channel Bandwidth | 200 MHz |
| Carrier Spacing/TTI | 75 KHz/0.2 ms |
| NodeB antenna element | Use Table 2.4-1 (a). |
| NodeB Beamforming | 16x16 antenna elements, use Table 2.4-1 (b). |
| Micro BS antenna | 1 Omnidirectional antenna with constant antenna gain 6 dBi. |
| UE antenna element | omnidirectional with 0 dB gain |
| UE Beamforming | 4x4 antenna elements, use Table 2.4-1 (b). |
| SINR to throughput mapping | Adopt scaled Shannon's formula in TR 36.942. |
| Co-existence scenario in terms of interference | UL to UL |
| Traffic | Full buffer |
| ACI leakage model | Flat ACLR |
| Performance metric | Throughput degradation compared to single operator case, i.e. no ACI. Both mean and 5%-tile are considered. |
| Shadowing Correlation | Between cells: 0.5, between sectors: 1.0 |
| Standard Deviation of Shadow Fading | 7.82 |
| MCL | 50 dB |
| Antenna Height | BS:10 m, UE: 1.5 m |
| Transmit Power | BS: 43 dBm, UE: 23 dBm |

# 3.	Simulation results

- 3.1 Mean UE Throughput

The results of mean throughput losses compared to no-ACI case in 30 GHz carrier frequency are given below. Three simulation cases are: without beamforming (wo BF), with beamforming at NodeB only (BF at NodeB), and with beamforming at both NodeB and UE (BF at NodeB and UE).

Table 3.1-1: Mean UE throughput results

| # Case | # 5 dB | # 10dB | # 15dB | # 20dB | # 25dB | # 30dB | # 35dB |
| --- | --- | --- | --- | --- | --- | --- | --- |
| # wo BF | # 37.7 | # 22.5 | # 12.3 | # 6.4 | # 3.3 | # 1.7 | # 0.8 |
| # BF at NodeB | # 31.2 | # 18.5 | # 10.2 | # 5.5 | # 2.9 | # 1.5 | # 0.7 |
| # BF at NodeB and UE | # 26.1 | # 14.6 | # 7.8 | # 4.1 | # 2.2 | # 1.1 | # 0.5 |

Fig. 3.1-1: Mean UE throughput loss.

From Fig. 3.1-1, it is observed that to guarantee a mean throughput loss no more than 5%, ACIR should be no less than 22.5 dB, 20.5 dB, 18 dB for woBF, BF at NodeB, and BF at NodeB and UE, respectively. In addition, by conducting beamforming at BS side or both BS/UE sides, the ACIR requirement can be significantly relaxed since the useful signal strength is enhanced.

Observation 1: Beamforming can effectively relax the ACIR requirement.

- 3.2 5%-tile Edge UE Throughput

The results of 5%-tile throughput losses compared to no-ACI case in 30 GHz carrier frequency are given below. Note that for the cases wo BF and BF at NodeB, 5%-tile edge UE throughput is always zero due to large path loss.

Table 3.2-1: 5%-tile edge UE throughput results

| # Case | # 5 dB | # 10dB | # 15dB | # 20dB | # 25dB | # 30dB | # 35dB |
| --- | --- | --- | --- | --- | --- | --- | --- |
| # BF at NodeB and UE | # 75.8 | # 53.8 | # 32.3 | # 18.3 | # 8.1 | # 4.0 | # 0.7 |

Fig. 3.2-1: 5%-tile edge UE throughput loss.

From Fig. 3.2-1, the required ACIR to guarantee a 5% or less throughput loss in this case is around 28 dB. This is 10 dB higher than that required when mean UE throughput loss is concerned. The main reason is that the received SNR of those 5% edge UEs is low, and the effect of adjacent channel interference becomes more obvious.

Observation 2: Adjacent channel interference has a much severer impact on edge UEs than center UEs.

4.Conslusion

This document presents simulation evaluations for coexistence study in dense urban scenario. Based on the  analysis of throughput loss versus ACIR results, we obtain the following observations.

Observation 1: Beamforming can effectively relax the ACIR requirement.

Observation 2: Adjacent channel interference has a much severer impact on edge UEs than center UEs.

# References

[1]	R4-162879, “WF on RF parameters requested by WP 5D”, NTT DOCOMO, INC., Qualcomm, CMCC, Nokia, Huawei, Ericsson.

[2]	R4-164528, “WF on WP 5D co-existence simulation assumptions”, NTT DOCOMO, INC.

[3]    3GPP TR 38.900 V14.0.0, “Study on channel model for frequency spectrum above 6 GHz”.

[4]   3GPP TR 37.842 V180, “E-UTRA and UTRA; Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)”

[5]	3GPP TR 36.942 v13.0.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) system scenarios”.