3GPP TSG-RAN WG4 (Radio) Meeting #80	R4-165940

Gothenburg, Sweden, 22 – 26 August 2016

Agenda Item:	10.3.1

Source: 	Samsung

Title:	Evaluation results for NR coexistence study in indoor scenario

Document for:	Discussion

1.Introduction

Coexistence study of new radio (NR) access technology is used to determine appropriate RF parameters. This topic was discussed in RAN4#78b and RAN4#79 meetings, in particular, some agreed simulation assumptions were made in [1, 2]. Based on those assumptions, this contribution presents simulation evaluation results of NR coexistence study in indoor scenario.

- 2.Discussion

- 2.1 Coexistence simulation case

The uplink NR eMBB is assumed under synchronized network, where aggressor and victim have the same configuration.

Table 2.1-1 Simulation case of coexistence study for NR

| Cases | Operation mode | Aggressor | Victim | Direction |
| --- | --- | --- | --- | --- |
| 1 | TDD | UE of NR eMBB | UE of NR eMBB | Uplink |

- 2.2	Cell layouts

- Indoor Base Station (BS)

Table 2.2-1: Evaluation parameters for Indoor-Office scenarios[3]

| Parameters |  | Values |  |
| --- | --- | --- | --- |
| Layout | Room size (WxLxH) | 120mx50mx3m |  |
|  | ISD | 20m |  |
| BS antenna height (hBS) |  | 3 m (ceiling) |  |
| UE location | LOS/NLOS | LOS and NLOS | FFS |
|  | Height (hUE) | 1.5 m |  |
| Min. BS - UE distance (2D) |  | 0 m |  |
| UE distribution (horizontal) |  | Uniform |  |
| Total number of UEs in each network |  | 36 |  |

Fig. 2.2-1: Layout of indoor office scenario[3].

#### 2.3	Path loss models [3]

The path loss models are summarized in Table 2.3-1. Distribution of shadow fading is log-normal, and its standard deviation is given in Table 2.3-1.

Table 2.3-1: Path loss models

| Scenario | Path loss [dB], fc is in GHz and d is in meters | Shadow fading std [dB] | Applicability range, antenna height default values |
| --- | --- | --- | --- |
| Indoor Office LOS |  | σSF=3.0 | 1<d3D<100m |
| Indoor Office NLOS |  | σSF=8.03 | 1<d3D<86m |
| Indoor Office |  |  |  |
| Note 1: The applicable frequency range of the PL formula in this table is 0.8<fc<100GHz. Note 2: fc denotes the center frequency normalized by 1GHz, all distance related values are normalized by 1m, unless it is stated otherwise. |  |  |  |

The LOS probability is given in Table 2.3-2.

Table 2.3-2 LOS probability

| Scenario | LOS probability (distance is in meters) |
| --- | --- |
| Indoor – Open office |  |

#### 2.4	Antenna Gain

- 2.4.1 BS

BSs are assumed to have antenna arrays, each antenna element is omnidirectional, with constant gain equal to 6 dBi. The composite antenna pattern is given in Table 2.4-1.

Table 2.4-1 Composite antenna pattern for NodeB beamforming[4]

| Configuration | Multiple columns (NVxNH elements) |
| --- | --- |
| Composite Array radiation pattern in dB | For beam i: the superposition vector is given by: the weighting is given by: |
| Antenna array configuration (Row×Column) | 16×16 |
| Horizontal radiating element spacing d/ | 0.5 |
| Vertical radiating element spacing d/ | 0.9 |
| Down-tilt angle (deg) | 0 degrees |
| Element Gain without antenna losses | GE,max= 6 dBi |

- 2.4.2 UE

UEs are assumed to have 1 antenna with 0 dB antenna gain.

#### 2.5	Power control modelling [5]

In 3GPP TR 36.942, the following power control equation is used for the uplink coexistence simulations:

where Pmax is the maximum transmit power, Rmin is the minimum power reduction ratio to prevent UEs with good channels to transmit at very low power level, CL is the coupling loss defined as max{path loss-G_Tx-G_Rx, MCL}, where path loss is propagation loss plus shadow fading, G_TX is the transmitter antenna gain in the direction of the receiver, G_RX is the receiver antenna gain in the direction of the transmitter and CLx-ile is the x-percentile CL value. With this power control equation, the x percent of UEs that have the highest coupling loss will transmit at Pmax. Finally, 0<<=1 is the balancing factor for UEs with bad channel and UEs with good channel.

The parameter sets for power control are listed below.

Table 2.5-1: Power control algorithm parameter (for 30 GHz carrier frequency)

| Parameter set | Gamma | CLx-ile 200 MHz bandwidth |  |
| --- | --- | --- | --- |
| Set 1 | 1 | 100 |

#### 2.6	Simulation assumptions and parameters

Simulation assumptions and parameter are summarized in Table 2.6-1:

Table 2.6-1: Simulation assumptions and parameters

| Parameter | Value |
| --- | --- |
| Methodology | Static Monte Carlo simulation |
| Deployment scenario | Indoor Office |
| Carrier frequency | 30 GHz |
| Channel Bandwidth | 200 MHz |
| Carrier Spacing/TTI | 75 KHz/0.2 ms |
| BS antenna | 16×16, omnidirectional antenna element with 6 dBi antenna gain |
| UE antenna | 1, omnidirectional with 0 dB antenna gain |
| SINR to throughput mapping | Adopt scaled Shannon's formula in TR 36.942. |
| Co-existence scenario in terms of interference | UL to UL |
| Traffic | Full buffer |
| ACI leakage model | Flat ACLR |
| Performance metric | Throughput degradation compared to single operator case, i.e. no ACI. mean is considered. |
| Shadow Correlation | Between BSs: 0.5; Within BS: 1.0 |
| Standard Deviation of Shadow Fading | 7.82 |
| MCL | 45 dB |
| Transmit Power | BS: 43 dBm, UE: 23 dBm |
| Antenna Height | BS: 3 m, UE: 1.5 m |

3.	Simulation results

We present results of mean throughput loss in two cases: without beamforming at BS and UE (wo BF), with beamforming at BS only (BF at BS). Since 5%-tile edge UE throughput is always zero under either case, the results of 5%-tile edge UE throughput losses are omitted.

Table 3.1-1: Mean UE throughput loss for 30 GHz carrier frequency

| # Case | # 5dB | # 10dB | # 15dB | # 20dB | # 25dB | # 30dB |
| --- | --- | --- | --- | --- | --- | --- |
| # wo BF | # 24.6 | # 14.3 | # 6.8 | # 2.7 | # 1.0 | # 0.3 |
| # BF at BS | # 4.6 | # 1.8 | # 0.6 | # 0.2 | # 0.1 | # 0 |

Fig. 3.1-1: Mean UE throughput loss.

Fig. 3.1-1 shows that 17 dB and 5 dB are enough large ACIR to make mean throughput loss less than 5% in cases without and with BS BF, respectively. It can be observed that the impact of adjacent channel interference can be reduced greatly with BF at BS, since beamforming only improves the received power of useful signal.

Moreover, compared with the ACIR requirement in Urban Macro and Dense Urban scenario (refer to evaluation results in Tdoc R4-165938 and R4-165939), the ACIR requirement in Indoor scenario here is much lower.

The above observations can be summarized as below:

Observation 1: Beamforming can effectively relax the ACIR requirement.

Observation 2: To guarantee the same UE throughput loss, the required ACIR in indoor scenario is smaller than that in Urban Macro or Dense Urban scenarios.

4. Conslusion

This document presents simulation evaluations for coexistence study in indoor network. Since 5%-tile edge UE throughput is always zero in the current setting, only mean UE throughput results are provided. Based on those results, we obtain the following two observations.

Observation 1: Beamforming can effectively relax the ACIR requirement.

Observation 2: To guarantee the same UE throughput loss, the required ACIR in indoor scenario is smaller than that in Urban Macro or Dense Urban scenarios.

# References

[1]	R4-162879, “WF on RF parameters requested by WP 5D”, NTT DOCOMO, INC., Qualcomm, CMCC, Nokia, Huawei, Ericsson.

[2]	R4-164528, “WF on WP 5D co-existence simulation assumptions”, NTT DOCOMO, INC.

[3]    3GPP TR 38.900 V14.0.0, “Study on channel model for frequency spectrum above 6 GHz”.

[4]   3GPP TR 37.842 V180, “E-UTRA and UTRA; Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)”

[5]	3GPP TR 36.942 v13.0.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) system scenarios”.