3GPP TSG-RAN WG4 Meeting #80                                                                   R4-166094

# Gothenburg, Sweden, 22 - 26 Aug, 2016

Title:	Discussion on UE RF of multicarrier operation for V2V with 23 dBm maximum transmit power

Source:	CMCC

Agenda Item:	8.11.3

Document for:	Approval

- Introduction

In RAN4 #79, a reply LS on V2X multicarrier configuration was approved [1]. RAN4 has agreed for multicarrier operation in V2V WI:

Conclusion:

- Analysis and specification of transmission and reception requirements for single component carrier (CC) operation will be completed as a first priority (e.g. for both licensed LTE bands and ITS spectrum at 5.9GHz).

  - The analysis and specification of transmission and reception requirements for MCC V2V operation will be completed in RAN4 as second priority as part of the V2V WI.

    - If the MCC operation for V2V service can’t be completed within V2V WI time line, RAN4 suggests to further study in Rel-14 pending agreement in the RAN plenary. Therefore MCC operation for V2V operation will be supported in Rel-14.

- Note that MCC V2V operation will depend on the V2V UE RF architecture and RAN4 needs further time to study support of MCC V2V operation and LTE operation with intra UE capabilities.

Based on the agreement, we provide our views on how to define general RX and TX requirements for multi-carrier operation in V2V service with 23 dBm maximum transmit power.

- Discussion

  - Channel bandwidths for MCC V2V

According to the agreement [1], RAN4 will define V2V UE RF requirements supporting the single carrier operation as a first priority in the V2V WI, while some operators requested to include con-current operation with V2V ITS spectrum and some licensed LTE bands (e.g Band 3, Band 8, Band 39 and Band 41).

Proposal#1: The MCC V2V service combination of channel bandwidths and operating bands is shown in Table 2.1-1.

The bandwidth classes can be reused from that defined in CA.

Table 2.1-1 inter-band MCC V2V communication channel bandwidth

| E-UTRA inter-band MCC V2V configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA MCC Configuration | Uplink MCC configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| V2V_3A-47A | V2V_3A-47A | 3 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 47 |  |  |  | Yes |  | Yes |  |  |
| V2V_8A-47A | V2V _8A-47A | 1 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 47 |  |  |  | Yes |  | Yes |  |  |
| V2V_39A-47A | V2V _39A-47A | 39 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 47 |  |  |  | Yes |  | Yes |  |  |
| V2V_41A-47A | V2V _41A-47A | 41 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 47 |  |  |  | Yes |  | Yes |  |  |

  - RF requirements for singer carrier transmission operation and multi-carrier reception V2V service

- 2.2.1.Harmonics study

Table 2.2-1 includes the 2nd , 3rd order harmonics products identified for band 3 + band 47 MCC with 1 UL.

Table 2.2-1 Band 3 and Band 47 harmonics products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 5855 | 5925 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 11710 | 11850 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 17565 | 17775 |

Table 2.2-2 includes the 2nd , 3rd , 5rd order harmonics products identified for band 8 + band 47 MCC with 1 UL.

Table 2.2-2 Band 8 and Band 47 harmonics products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 5855 | 5925 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 11710 | 11850 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 17565 | 17775 |
| 5rd harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5rd harmonics frequency limits (MHz) | 4400 | 4575 | 29275 | 29625 |
| 7rd harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7rd harmonics frequency limits (MHz) | 6160 | 6405 | 40985 | 41475 |

Table 2.2-3 includes the 2nd , 3rd order harmonics products identified for band 39 + band 47 MCC with 1 UL.

Table 2.2-3 Band 39 and Band 47 harmonics products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1880 | 1920 | 5855 | 5925 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3760 | 3840 | 11710 | 11850 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5640 | 5760 | 17565 | 17775 |

Table 2.2-4 includes the 2nd , 3rd order harmonics products identified for band 41 + band 47 MCC with 1 UL.

Table 2.2-4 Band 41 and Band 47 harmonics products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 5855 | 5925 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 | 11710 | 11850 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 | 17565 | 17775 |

For inter-band MCC V2V_3A_47A, V2V _8A_47A, V2V _39A_47A and V2V _41A_47A, UE does not have 2nd/3rd/5th order harmonic issues.

- 2.2.2. ΔTIB,c and ΔRIB,c

For inter-band MCC V2V_3A_47A, V2V _8A_47A, V2V _39A_47A and V2V _41A_47A, there is no 2nd/3rd/5th order harmonics with its own DL. The ΔTIB,c,ΔRIB,c values could refer to the agreement for inter-band CA including B46, since B47’s frequency range is inside B46 and is even at the upper bound.

Table 2.2-5: ΔTIB,c

| Inter-band MCC Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| V2V_3A-47A | 3 | 0 |
|  | 47 | 0 |
| V2V_8A-47A | 1 | 0 |
|  | 47 | 0 |
| V2V_39A-47A | 39 | 0 |
|  | 47 | 0 |
| V2V_41A-47A | 41 | 0 |
|  | 7 | 0 |

Table 2.2-6: ΔRIB,c

| Inter-band MCC Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| V2V_3A-47A | 3 | 0 |
|  | 47 | 0 |
| V2V_8A-47A | 1 | 0 |
|  | 47 | 0 |
| V2V_39A-47A | 39 | 0 |
|  | 47 | 0 |
| V2V_41A-47A | 41 | 0 |
|  | 7 | 0 |

  - RF requirements for multi-carrier transmission operation and multi-carrier reception V2V service

- 2.3.1.IMD study

Table 2.3-1 includes the 2nd , 3rd and 5th order IMD products identified for band 3 + band 47 MCC with 2 UL.

Table 2.3-1 Band 3 and Band 47 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 5855 | 5925 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4070 | 4215 | 7565 | 7710 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2505 | 2285 | 9925 | 10140 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9275 | 9495 | 13420 | 13635 |
| Two-tone 5rd order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high - 3*fx_low¦ |
| IMD frequency limits (MHz) | 14355 | 13995 | 6355 | 6720 |
| Two-tone 5rd order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 20985 | 21345 | 16840 | 17205 |
| Two-tone 5rd order IMD products | ¦fx_low - 4*fy_high¦ | ¦fx_high - 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 21990 | 21635 | 1285 | 915 |
| Two-tone 5rd order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 25130 | 25485 | 12695 | 13065 |

Table 2.3-2 includes the 2nd , 3rd and 5th order harmonics and IMD products identified for band 8 + band 47 MCC with 2 UL.

Table 2.3-2 Band 8 and Band 47 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 5855 | 5925 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4940 | 5045 | 6735 | 6840 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4165 | 4025 | 10795 | 10970 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7615 | 7755 | 12590 | 12765 |
| Two-tone 5rd order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high - 3*fx_low¦ |
| IMD frequency limits (MHz) | 16015 | 15735 | 8965 | 9210 |
| Two-tone 5rd order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 19325 | 19605 | 14350 | 14595 |
| Two-tone 5rd order IMD products | ¦fx_low - 4*fy_high¦ | ¦fx_high - 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 22820 | 22505 | 2195 | 2405 |
| Two-tone 5rd order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 24300 | 24615 | 9375 | 9585 |

For inter-band MCC V2V_3A_47A, V2V _8A_47A, V2V _39A_47A and V2V _41A_47A, UE does not have 2nd/3rd/5th order harmonic and intermodulation issues.

- 2.3.2. ΔTIB,c and ΔRIB,c

For inter-band MCC V2V _3A_47A, V2V _8A_47A, V2V _39A_47A, and V2V _41A_47A, since there is no 2nd/3rd/5th order harmonics and intermodulation problem with own DL, ∆TIB,c and ∆RIB,c for 1UL is reused for 2UL.

For two simultaneous UL, the TIB,c and RIB,c values are shown in table 2.3-3 and in table 2.3-4.

Table 2.3-3: ΔTIB,c

| Inter-band MCC Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| V2V _3A-47A | 3 | 0 |
|  | 47 | 0 |
| V2V_8A-47A | 1 | 0 |
|  | 47 | 0 |
| V2V_39A-47A | 39 | 0 |
|  | 47 | 0 |
| V2V_41A-47A | 41 | 0 |
|  | 7 | 0 |

Table 2.3-4: ΔRIB,c

| Inter-band MCC Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| V2V_3A-47A | 3 | 0 |
|  | 47 | 0 |
| V2V_8A-47A | 1 | 0 |
|  | 47 | 0 |
| V2V_39A-47A | 39 | 0 |
|  | 47 | 0 |
| V2V_41A-47A | 41 | 0 |
|  | 7 | 0 |

- Conclusion

There is a text proposal for TR 36.785 V0.2.0 for introduction of UE RF characters of inter-band MCC V2V service.

- Reference

[1] R4-164930, Reply LS on V2V multicarrier configuration, RAN WG4

[2] TR 36.785 V0.2.0

- 3. Text Proposal for TR 36.785v0.2.0

- ---Start of changes---

- 5 Deployment and co-existence studies

- 5.3	Channel bandwidths

- 5.3.1	Channel bandwidths for inter-band MCC V2V service

Table 5.3.1-1 inter-band MCC V2V communication channel bandwidth

| E-UTRA inter-band MCC V2V configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA MCC Configuration | Uplink MCC configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| V2V_3A-47A | V2V_3A-47A | 3 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 47 |  |  |  | Yes |  | Yes |  |  |
| V2V_8A-47A | V2V _8A-47A | 1 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 47 |  |  |  | Yes |  | Yes |  |  |
| V2V_39A-47A | V2V _39A-47A | 39 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 47 |  |  |  | Yes |  | Yes |  |  |
| V2V_41A-47A | V2V _41A-47A | 41 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 47 |  |  |  | Yes |  | Yes |  |  |

- 6	V2V transmitter characteristics

- 6.1	Transmit power

- 6.1.1	 UE maximum output power for V2V service

Table 6.1.1-1: UE Power Class for uplink inter-band MCC V2V service (two bands)

| E-UTRA CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2V_3A-47A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_1A-5A |  |  |  |  | 23 | +2/-3 |  |  |
| V2V _8A-47A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_1A-8A |  |  |  |  | 23 | +2/-3 |  |  |

- 6.1.2	Configured transmitted power

For the UE which supports inter-band MCC V2V service configurations with the uplink assigned to one or two E-UTRA bands the ΔTIB,c is defined for applicable bands in Table 6.1.2-1

Table 6.2.1-1: ΔTIB,c (two bands)

| Inter-band MCC Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| V2V_3A-47A | 3 | 0 |
|  | 47 | 0 |
| V2V_8A-47A | 1 | 0 |
|  | 47 | 0 |
| V2V_39A-47A | 39 | 0 |
|  | 47 | 0 |
| V2V_41A-47A | 41 | 0 |
|  | 7 | 0 |

- 7	V2V receiver characteristics

- 7.1	Reference sensitivity power level

- 7.1.1	Minimum requirements (QPSK)

Table 7.1.1-x: ΔRIB,c (two bands)

| Inter-band MCC Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| V2V_3A-47A | 3 | 0 |
|  | 47 | 0 |
| V2V_8A-47A | 1 | 0 |
|  | 47 | 0 |
| V2V_39A-47A | 39 | 0 |
|  | 47 | 0 |
| V2V_41A-47A | 41 | 0 |
|  | 7 | 0 |

- ---End of changes---