3GPP TSG-WG4 Meeting #80	R4-166488

# Gothenburg, Sweden, 22nd – 26th August 2016

Agenda item: 	6.7.4.1.3

Source: 	Qualcomm Incorporated

Title: 	NPDSCH Demodulation Simulation Results

Document for:	Discussion

# 1	Introduction

For NPDSCH, RAN4 agreed to define the following test cases, and related main configurations are listed as in Table 1 below. Table 2 is the TBS table for NPDSCH.

This paper provides the simulation results and proposes the number of repetitions to satisfy 70% of maximum NPDSCH throughput around the target SNR.

# Table 5 Minimum performance NPDSCH

| Simulation number | Band-width and MCS | Deployment Mode | (ITBS, ISF) (Table 6) | Repetition level | Propagation Condition | Number of NRS ports | Target SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 180KHz QPSK [1/3] | In-band | ([4], [0]) (TBS=56bits) | 1 | [EPA5] | 2 | TBD |
| 2 | 180KHz QPSK [1/3] | In-band | ([4], [0]) (TBS=56bits) | Note 1 | [EPA5] | 2 | [-6] |
| 3 | 180KHz QPSK [1/3] | In-band | ([4], [0]) (TBS=56bits) | Note 1 | [ETU1] | 2 | [-12] |
| 4 | 180KHz QPSK [1/2] | Standalone/Guard-band | ([9], [3]) (TBS=616bits) | Note 1 | [EPA5] | 1 | [-6] |
| 5 | 180KHz QPSK [1/3] | Standalone/Guard-band | ([6], [3]) (TBS=392 bits) | ###### - | [ETU1] | 1 | [-12] |
| Note 1: Choose one repetition level from {1,2,4,8,16,32,64,128,192,256,384,512,768, } to satisfy 70% of the maximum NPDSCH throughput. Note 2: Assumption of In-band configuration: CFI=3 and the number of LTE CRS ports is 4. Demodulation is assumed to be based only NRS. Note 3: Maximum HARQ retransmission is 4. |  |  |  |  |  |  |  |

Table 6	TBS table for NPDSCH

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 |  |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 |  |  |
| 7 | 104 | 224 | 328 | 472 | 584 | 680 |  |  |
| 8 | 120 | 256 | 392 | 536 | 680 |  |  |  |
| 9 | 136 | 296 | 456 | 616 |  |  |  |  |
| 10 | 144 | 328 | 504 | 680 |  |  |  |  |
| 11 | 176 | 376 | 584 |  |  |  |  |  |
| 12 | 208 | 440 | 680 |  |  |  |  |  |

The simulation assumes that NRS is available in every subframe in the frame except subframe 5 which contains NPSS and subframe 9 in every other frame which contains NSSS. This enables channel estimation to be performed using time domain filtering over destaggered NRS tones. A low filter coefficient value is chosen since the channel model considered is a slow fading channel with 1Hz/5Hz doppler. The results shown below include RF margin.

# 2	Simulation Results

The following plot shows the NPDSCH throughput performance with respect to SNR(dB) for simulation number 1 in Table 1.

| Simulation number | Band-width and MCS | Deployment Mode | (ITBS, ISF) (Table 6) | Repetition level | Propagation Condition | Number of NRS ports | Target SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 180KHz QPSK [1/3] | In-band | ([4], [0]) (TBS=56bits) | 1 | [EPA5] | 2 | 4 |

Observation 1: For simulation number 1, target SNR to achieve 70% of max throughput is 4dB.

The following plot shows the NPDSCH throughput performance with respect to SNR(dB) for simulation number 2 in Table 1.

| Simulation number | Band-width and MCS | Deployment Mode | (ITBS, ISF) (Table 6) | Repetition level | Propagation Condition | Number of NRS ports | Target SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 180KHz QPSK [1/3] | In-band | ([4], [0]) (TBS=56bits) | Note 1 | [EPA5] | 2 | [-6] |
| Note 1: Choose one repetition level from {1,2,4,8,16,32,64,128,192,256,384,512,768, } to satisfy 70% of the maximum NPDSCH throughput. Note 2: Assumption of In-band configuration: CFI=3 and the number of LTE CRS ports is 4. Demodulation is assumed to be based only NRS. Note 3: Maximum HARQ retransmission is 4. |  |  |  |  |  |  |  |

Observation 2: For simulation number 2, 16 repetitions are required to achieve 70% of max throughput at -7.5 dB target SNR.

The following plot shows the NPDSCH throughput performance with respect to SNR(dB) for simulation number 3 in Table 1.

| Simulation number | Band-width and MCS | Deployment Mode | (ITBS, ISF) (Table 6) | Repetition level | Propagation Condition | Number of NRS ports | Target SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 180KHz QPSK [1/3] | In-band | ([4], [0]) (TBS=56bits) | Note 1 | [ETU1] | 2 | [-12] |
| Note 1: Choose one repetition level from {1,2,4,8,16,32,64,128,192,256,384,512,768, } to satisfy 70% of the maximum NPDSCH throughput. Note 2: Assumption of In-band configuration: CFI=3 and the number of LTE CRS ports is 4. Demodulation is assumed to be based only NRS. Note 3: Maximum HARQ retransmission is 4. |  |  |  |  |  |  |  |

Observation 3: For simulation number 3, 64 repetitions are required to achieve 70% of max throughput at -12 dB target SNR.

The following plot shows the NPDSCH throughput performance with respect to SNR(dB) for simulation number 4 in Table 1.

| Simulation number | Band-width and MCS | Deployment Mode | (ITBS, ISF) (Table 6) | Repetition level | Propagation Condition | Number of NRS ports | Target SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | 180KHz QPSK [1/2] | Standalone/Guard-band | ([9], [3]) (TBS=616bits) | Note 1 | [EPA5] | 1 | [-6] |
| Note 1: Choose one repetition level from {1,2,4,8,16,32,64,128,192,256,384,512,768, } to satisfy 70% of the maximum NPDSCH throughput. Note 2: Assumption of In-band configuration: CFI=3 and the number of LTE CRS ports is 4. Demodulation is assumed to be based only NRS. Note 3: Maximum HARQ retransmission is 4. |  |  |  |  |  |  |  |

Observation 4: For simulation number 4, 16 repetitions are required to achieve 70% of max throughput at -4.5 dB target SNR.

The following plot shows the NPDSCH throughput performance with respect to SNR(dB) for simulation number 5 in Table 1.

| Simulation number | Band-width and MCS | Deployment Mode | (ITBS, ISF) (Table 6) | Repetition level | Propagation Condition | Number of NRS ports | Target SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 180KHz QPSK [1/3] | Standalone/Guard-band | ([6], [3]) (TBS=392 bits) | ###### - | [ETU1] | 1 | [-12] |
| Note 1: Choose one repetition level from {1,2,4,8,16,32,64,128,192,256,384,512,768, } to satisfy 70% of the maximum NPDSCH throughput. Note 2: Assumption of In-band configuration: CFI=3 and the number of LTE CRS ports is 4. Demodulation is assumed to be based only NRS. Note 3: Maximum HARQ retransmission is 4. |  |  |  |  |  |  |  |

Observation 5: For simulation number 5, 64 repetitions are required to achieve 70% of max throughput at -10.5 dB target SNR.

# 3	Conclusions

This paper contains NPDSCH simulation results for alignment and made the following observations:

Observation 1: For simulation number 1, target SNR to achieve 70% of max throughput is 4.

Observation 2: For simulation number 2, 16 repetitions are required to achieve 70% of max throughput at -7.5 dB target SNR.

Observation 3: For simulation number 3, 64 repetitions are required to achieve 70% of max throughput at -12 dB target SNR.

Observation 4: For simulation number 4, 16 repetitions are required to achieve 70% of max throughput at -4.5 dB target SNR.

Observation 5: For simulation number 5, 64 repetitions are required to achieve 70% of max throughput at -10.5 dB target SNR.

# 4	References

- R4-79AH-0161, “Updated simulation assumptions for NB-IoT UE demodulation”, Huawei, HiSilicon, Ericsson, Qualcomm, Samsung, ZTE