3GPP TSG-RAN WG4 #80	R4-166542

Göteborg, Sweden, 22nd – 26th August 2016

Source: 	Ericsson

Title:  	Details on multi-node tests for Rel-13 LAA

Agenda Item:	7.1.2

Document for:	Discussion

# Introduction

During RAN plenary in June 2016, a new study item has been approved in [1]. Previously, a way forward has been agreed on coexistence tests for Rel-13 LAA in RAN4#78bis [2] and a further way forward have been approved in RAN4#79 in [3].

The coexistence tests are divided in two parts: LBT functionalities tests and multi-node tests. These tests are captured in TS36.141 in RAN4#79 meeting.  In this contribution, we provide more detailed understanding on multi-node tests.

# Multi-node tests

The latest agreements are described in [3].

In this section, we discuss the open issues on multi-node tests.

## Multi-node test setup

An important issue is to decide on how many nodes are connected for the multi-node tests. We propose to include one LAA node, one WiFi AP, one LAA UE and one WiFi STA for the multi-node test setup.

As common understanding, the LAA UE will be connected to LAA BS and WiFi STA will be connected to WiFi AP. The detailed connection setup can vary depending on the implementation of the test setup.

An example setup is shown below in Figure 1.

Figure 1 Example setup for multi-node tests for Rel-13 LAA

The basic setup will be that both LAA node and WiFi AP will operate in best effort traffic, i.e. in priority class 3.

It is worth noting that we only concentrate on DL traffic in this setup. Thus, traffic is only expected in the LAA node and WiFi AP.

## Throughput tests

In throughput tests, it is verified whether a 5GHz device is able to receive certain level of throughput when other 5GHz systems are present in the band.

### Test cases for throughput tests

The test cases are listed below:

|  | Transmitter | Receiver | Comparison | Criteria |
| --- | --- | --- | --- | --- |
| Case 1: | LAA | WiFi | WiFi-WiFi | Impact of LAA transmitter on WiFi throughput: 2nd receiver system achieves certain level of throughput, its WiFi STA in this case. |
| Case 2: | WiFi | LAA | LAA-LAA | Impact of WiFi transmitter on LAA throughput: 2nd receiver system achieves certain level of throughput, its LAA UE in this case. |

LAA-LAA setup is quite well understood, since Rel-13 spec based LAA BS can be used as DUT for LAA-LAA setup. However, it remains to be seen how to define WiFi-WiFi setup. There can be a number of WiFi implementations, thus a number of WiFi-WiFi setups can be used for comparison.

### Test metric for throughput test

WiFi-WiFi benchmark should be defined based on multi-vendor and multi-standard (i.e. multi-generation WiFi systems) example of devices. For example, if there are three WiFi devices representing different vendors, standards and generations of WiFi systems, the throughput performance criteria need be defined based on statistics of throughput performance when WiFi(1)-to-WiFi(2), WiFi(2)-to-WiFi(3), and WiFi(3)-to-WiFi(1), setup is considered.

As an example: If there are several  WiFi-WiFi cases which  achieves a minimum (or average) throughput of TPW1 TPW2 ,…, TPWn, and LAA-WiFi case for corresponding WiFi implementations achieves throughput of TPL1, TPL2, …, TPLn, then the following conditions need to be considered:

- CDF of WiFi-to.WiFi case

- CDF of LAA-to-WiFi case

For both cases above, the criterion can be any of the following:

- within x% of mean,

- within min/max,

- within certain range, etc

## Channel access priority class tests

In R4-162989, RAN4 has agreed to have a maximum of two priority classes. It is worth mentioning here that, from spec point of view, different priority classes are optional. So, it’s not mandatory to implement all priority classes, thus, an LAA node may only implement one priority class (as an example). Thus, it is understood that:

Observation: Tests should only be performed for channel priority class capabilities that are declared by the LAA node.

As a result, if an LAA node is capable of performing only one channel priority class, then the LAA BS will be tested for only one priority class. Similarly, Maximum two priorities for node that can do anything more than one classes.

### Test cases

In addition to previous test cases, another set of tests can be done for LAA BS that also implements priority class 1. The test case can be described as follows:

|  | Transmitter | Receiver | Comparison | Traffic | Criterion |
| --- | --- | --- | --- | --- | --- |
| Case -3 | LAA BS | WiFi AP | WiFi-WiFi | VoIP traffic in all nodes | The criterion should be investigated in more detail. One possibility could be: outage should not be worse in second system receiver when the transmitter is replaced |
| Case- 4 | WiFi AP | LAA BS | LAA-LAA | VoIP traffic in all nodes |  |

### Metric

For channel priority class 1, i.e. for VoIP traffic, the metric will be actually the outage that the victim devices experience. The outage can be defined as VoIP client behavior, such as when does the VoIP client drop the packets and how many percentage of the packets are dropped within a certain time frame?

### Load

In our opinion, best effort traffic model is generally more suitable for the tests. One underlying principle can be that, same level of traffic need to be considered for cases which are compared for performance. This means that, the traffic that is assumed for WiFi-WiFi in case 3 should also be used in LAA-WiFi case in case 3. In this way, the performance comparison will be more reasonable.

## Test signal levels

Test signal levels are an issue which should be carefully thought on. In the multi-node test design, the tests can be performed at least on two different signal levels at which the tested device listens to the channel for other active transmitter(s). These two levels are described below:

- Since LAA threshold is at -72dBm/20MHz, we propose to define one test signal level at above -72dBm/20MHz. This level is used to verify basic functionalities.

- Another test signal level can be defined which is well below the ED threshold. This can be used to verify enhanced below ED functionality. One good value could be to consider a test signal level around WiFi RSSI statistics. This can be discussed further to find a suitable level.

Thus, we describe two different RX signal level for tests, which are shown in Figure 2

Figure 2 Test signal levels for multi-node tests

Δ =10dB could be a good choice. It is worth noting here that:

- The pass/fail criteria should not be the same for below ED test as for the above ED tests. Possibly, it makes sense for the below ED tests to allow a much larger variance.

- For the below ED tests, it is reasonable to have non-full buffer load while performing the tests. As an example: low load could be used for below ED tests.

# Conclusion

In this paper, we have summarized our understanding on some of the issues related to multi-node tests for Rel-13 LAA. We propose RAN4 companies to consider the above the discussions while finalizing the multi-node tests.

# References

RP-161197, New Study Item proposal: Multi-node tests for LAA, Huawei, HiSilicon, Ericsson, Qualcomm, Nokia

R4-162989, “Way Forward on LAA coexistence tests”, Ericsson, Huawei, Qualcomm, Nokia, RAN4 #78bis

R4-164971 WF on LAA co-existence testing, Source: Qualcomm, Ericsson, Huawei, Nokia, RAN4#79.