3GPP TSG-RAN WG4 #80	R4-166544

Goteborg, Sweden, 22th – 26th August 2016

Source: 	Ericsson

Title:  	TP on Scope for Rel-13 TR 36.789: Multi-node tests for LAA

Agenda Item:	7.1.1

Document for:	Approval

# Introduction

LAA multi-node tests will be studied under a new SI, which is approved in [1]. TR36.789 corresponds to documentation from this SI. A skeleton is proposed in [2]. In this contribution, we propose a text for this TR which includes texts related to scope of the report.

# Text proposal

The following text proposal is intended for the introduction and Chapter 1 of TR 36.789.

<<<<< start of TP <<<<<<

# Introduction

Licensed assist access (LAA) in unlicensed spectrum using the Carrier Aggregation Framework of LTE was introduced in Rel-13 as a complementary tool to augment operators’ service offering and solutions. Given the widespread deployment and usage of other technologies in unlicensed spectrum, it is necessary that LAA coexists with existing and future technologies targeted for unlicensed spectrum. Therefore, LBT mechanism was introduced for LAA to coexist with other wireless systems, e.g. Wi-Fi as well as with other LAA systems. As part of the R13 WI, functionality tests for LBT mechanism have been specified for LAA to verify that the LAA Base Stations (BS) have implemented LBT as it is specified in 3GPP specifications.

Meanwhile, there has been request to conduct multi-node tests where two wireless systems share the same unlicensed spectrum and their system performance is to be ensured, e.g. between two LAA systems or between LAA and other wireless systems, e.g. Wi-Fi.   Such multi-node tests are expected to verify that the two systems can coexist when operating in the same unlicensed spectrum. From this perspective, multi-node tests verify coexistence at system level, while it is understood that any LBT functionality verifications of LAA BS are already guaranteed by LBT tests introduced in Rel-13 LAA.

# 1	Scope

The present document describes the multi-node tests for Licensed-Assisted Access (LAA) in unlicensed spectrum. In particular, the report documents how to conduct multi-node tests involving two Rel-13 LAA BSs or one Rel-13 LAA BS and one other wireless system, e.g. Wi-Fi system to make sure that the two systems can coexist in the same unlicensed spectrum.

<<<<< end of TP <<<<<<

# Conclusion

We propose to adopt the above mentioned text proposal for TR 36.789.

# References

RP-161197, New Study Item proposal: Multi-node tests for LAA, Huawei, HiSilicon, Ericsson, Qualcomm, Nokia

R4-166543, Draft TR skeleton for LAA multi-node tests SI (TR 36.789), Ericsson, RAN4#80.