3GPP TSG-RAN WG4 #80	R4-166780

Gothenburg, Sweden, 22 – 26 Aug, 2016

Source: 	Ericsson

Title:	Simulation assumption for LAA PDCCH

Agenda Item:	6.6.4.3

Document for:	Approval

# Introduction

In this paper, simulation assumption for LAA PDCCH  is provided.

# Common parameters for Scell

Table 1 provides all common parameters for Scell. Without special mention, the parameters in Table 1 will be applied for all the following sections.

Table 1: Common parameters for Scell

| Parameters | unit | Value |
| --- | --- | --- |
| MBSFN configuration |  | No MBSFN subframe is configured |
| DRS periodicity | ms | 80 |
| Frequency offset related to primary carrier | Hz | [200] |
| Time offset related to primary carrier | us | TBD |
| Bandwidth | MHz | 20 |
| Number of allocated resource blocks (Note 2) | PRB | 100 |
| Number of control OFDM symbols |  | 2 |
| Signal transmission model |  | Randomness model is used as baseline with the exception: The burst length is {1,3,5,8} subframe |

# Simulation assumption for PDCCH

## PDCCH

The parameters for Scell in PDCCH test is given in Table 2.

Table 2: The parameters for PDCCH (Scells)

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (NOTE 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Propagation condition |  |  | EVA5 |
| Correlation matrix |  |  | 2x2 Low |
| Number of CCE |  |  | 4 |
| Format 2A |  | bits | 48 (not included CRC) (for FDD Pcell) 51 (not included CRC) (for TDD Pcell) |
| PDSCH transmission mode |  |  | 2 |
| NOTE 1: . NOTE 2: PUCCH format 1b with channel selection is used to feedback ACK/NACK for Tests in Table 8.2.1.3.1-4, PUCCH format 3 is used to feedback ACK/NACK for Tests in Table 8.2.1.3.1-6. NOTE 3: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

# Conclusion

In this paper, simulation parameters are provided for PDCCH. We hope the group approves the above parameters for simulation results alignment.