3GPP TSG-RAN WG4 Meeting #80bis                                                                                          R4-167374

Ljubljana, Slovenia, 10 – 14 Oct, 2016

Agenda Item:	6.3.4.2.2

Source: 	Samsung

Title: 	Simulation results for NB-IoT PUSCH format 1

Document for:	Discussion

Introduction

In this contribution, we give simulation results for NB-IoT PUSCH format 1.

Discussion

Evaluation cases were summarized in table below.

| Test Number | Subcarrier Spacing | Number of Tones | Number of RU | Number of Repetition | Propagation Condition | Antenna Configuration | MCS | Modulation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3.75kHz | 1 | 2 | 1, 16, 64 | ETU1 | 1 X 2 | 0 | BPSK |
| 2 | 15kHz | 1 | 2 | 1, 16, 64 | ETU1 | 1 X 2 | 0 | BPSK |
| 3 | 15kHz | 12 | 1 | 2, 16, 64 | ETU1 | 1 X 2 | 9 | QPSK |
| 4 | 15kHz | 6 | 1 | 2, 16, 64 | ETU1 | 1 X 2 | 7 | QPSK |
| 5 | 15kHz | 3 | 1 | 2, 16, 64 | ETU1 | 1 X 2 | 3 | QPSK |

| * Maximum number of transmission is 4 |
| --- |
| * Channel estimation lengths are 16ms for 3.75kHz and 4ms for 15kHz |  |  |
| * Timing/frequency offset estimation is performed |  |

Reference SNR for 70% relative throughput was given in table below.

| Test Number | Number of Repetition | SNR [dB] @ 70% Maximum Throughput |
| --- | --- | --- |
| 1 | 1 | -3.8 |
|  | 16 | -11.06 |
|  | 64 | -13.32 |
| 2 | 1 | -4.36 |
|  | 16 | -11.74 |
|  | 64 | -15.08 |
| 3 | 2 | -2.16 |
|  | 16 | -10.46 |
|  | 64 | -14.39 |
| 4 | 2 | -3.2 |
|  | 16 | -10.99 |
|  | 64 | -14.88 |
| 5 | 2 | -5.63 |
|  | 16 | -12.57 |
|  | 64 | -16.18 |

Conclusion

In this contribution, we give simulation results for NB-IoT PUSCH format 1.