3GPP TSG-RAN WG4 Meeting #80bis	R4-167912

Ljubljana, Slovenia, 10 - 14 October 2016

Agenda Item:	5.2.3.2

Source: 		Ericsson

Title: 	Simulation results for IRC TM2/3/3 test with 4Rx

Document for:	Discussion

# Introduction

In the WF [1],  it was agreed to add new 4Rx testcases for IRC performance in 36.101 Rel-13.

  - Type A IRC,

    - Define new 4Rx tests with TM2/3/3 (8.2.1.2.4 and 8.2.2.2.4)

    - Finalize the tests in maintenance.

# Discussions and Results

During the discussions about antenna connection for legacy requirements with testcases based on 2Rx on a 4Rx UE it was agreed to add new 4Rx testcases for IRC performance. The reason is that the there is only one legacy IRC testcase for demodulation which was needed to be defined for a 4Rx UE. By defining the corresponding testcase for 4Rx there is no need to agree on a way to handle a 2Rx IRC testcase for a 4Rx UE.

Based on this discussion below is the simulation results for both 2Rx and 4Rx performance when adapting the testcase to 4Rx. Simulations are shown for both Low coorelation and the new antenna correlation Medium A.

Figur 1: The simulation results when 8.2.1.2.4 is simulated with 2 and 4Rx

Figur 2: The simulation results when 8.2.1.2.4 with correlation Medium Correlation A is simulated with 2 and 4Rx

Figur 3: The simulation results when 8.2.2.2.4 is simulated with 2 and 4Rx

Figur 4 The simulation results when 8.2.2.2.4 with correlation Medium Correlation A is simulated with 2 and 4Rx

Based on these simulations and after an alignment of simulation results it is proposed to specify a new requirement for 4Rx based on the 2Rx IRC requirement in 8.2.1.2.4 and 8.2.2.2.4 in the 4Rx performance section.

# Proposal for specification

New Table 1 for FDD : Test Parameters for Transmit diversity Performance (FRC) with TM3 interference model

| Parameter |  |  | Unit |  | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB |  | -3 |  | -3 |  | -3 |  |
|  |  |  | dB |  | -3 (NOTE 1) |  | -3 |  | -3 |  |
|  |  |  | dB |  | 0 |  | 0 |  | 0 |  |
| Cell-specific reference signals |  |  |  |  | Antenna ports 0,1 |  | Antenna ports 0,1 |  | Antenna ports 0,1 |  |
| at antenna port |  |  | dBm/15kHz |  | -98 |  | N/A |  | N/A |  |
| DIP (NOTE 2) |  |  | dB |  | N/A |  | -2.23 |  | -8.06 |  |
| BWChannel |  |  | MHz |  | 10 |  | 10 |  | 10 |  |
| Cyclic Prefix |  |  |  |  | Normal |  | Normal |  | Normal |  |
| Cell Id |  |  |  |  | 0 |  | 1 |  | 2 |  |
| Number of control OFDM symbols |  |  |  |  | 2 |  | 2 |  | 2 |  |
| PDSCH transmission mode |  |  |  |  | 2 |  | N/A |  | N/A |  |
| Interference model |  |  |  |  | N/A |  | As specified in clause B.5.2 |  | As specified in clause B.5.2 |  |
| Probability of occurrence of transmission rank in interfering cells |  | Rank 1 | % |  | N/A |  | 80 |  | 80 |  |
|  |  | Rank 2 | % |  | N/A |  | 20 |  | 20 |  |
| Reporting interval |  |  | ms |  | 5 |  | N/A |  | N/A |  |
| Reporting mode |  |  |  |  | PUCCH 1-0 |  | N/A |  | N/A |  |
| Physical channel for CQI reporting |  |  |  |  | PUSCH(Note 5) |  | N/A |  | N/A |  |
| cqi-pmi-ConfigurationIndex |  |  |  |  | 2 |  | N/A |  | N/A |  |
| NOTE 1: NOTE 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. NOTE 3: Cell 1 is the serving cell. Cell 2, 3 are the interfering cells. NOTE 4: Cell 2 transmission is delayed with respect to Cell 1 by 0.33 ms and Cell 3 transmission is delayed with respect to Cell 1 by 0.67 ms. Note 5: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1 and #6 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#5 and #0. |  |  |  |  |  |  |  |  |  |  |

New Table 2 for FDD: Enhanced Performance Requirement Type A, Transmit Diversity (FRC) with TM3 interference model

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions |  |  | Correlation Matrix and Antenna Configuration (NOTE 3) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SINR (dB) (NOTE 2) |  |
| 1 | R.46 FDD | OP.1 FDD | N/A | N/A | EVA70 | EVA70 | EVA70 | 2x4 Low | 70 | TBD | ≥1 |
| NOTE 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. NOTE 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. NOTE 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1, Cell 2 and Cell 3. |  |  |  |  |  |  |  |  |  |  |  |

# Conclusions

Based on these simulation results with alignment with other companies a CR as proposed in [2]

# References

- R4-167008: WF on antenna connection for legacy 2Rx tests for 4Rx UEs,

- R4-167914: CR for IRC TM2/3/3 tests with 4Rx in Rel-13, Ericsson