3GPP TSG-RAN WG4 Meeting #80bis	                                         R4-168317

Ljubljana, Slovenia, 10 - 14 Oct, 2016

Source: 	Huawei, Hisilicon

Title: 	TP for Rel-14 xDL/2UL TR 36.714-00-02: Co-existence and dTib and dRib for CA_3DL_2A-4A-7A_2UL_2A-4A and CA_4DL_2A-4A-7A-7A_2UL_2A-4A

Agenda Item:	7.7.1

Document for:	Approval

# Background

This contribution provides a TP for TR 36.714-00-02 to capture co-existence and ∆TIB and ∆RIB for CA_3DL_2A-4A-7A_2UL_2A-4A and CA_4DL_2A-4A-7A-7A_2UL_2A-4A

# References

[1] RP-16xxxx, “New WID: Basket WI for LTE Advanced Inter-band CA Rel-14 for xDL/2UL”, RAN #7x

# Text Proposal

<Start of Text Proposal for TR 36.714-00-02>

## 6.x	CA_3DL_2A-4A-7A_2UL_2A-4A_BCS0

### 6.x.3 Co-existence issues

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 1710 | 1755 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 3420 | 3510 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 5130 | 5265 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 200 | 95 | 3560 | 3665 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1945 | 2110 | 1510 | 1660 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5410 | 5575 | 5270 | 5420 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3795 | 4020 | 3220 | 3415 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7260 | 7485 | 6980 | 7175 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 190 | 400 | 7120 | 7330 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5170 | 4930 | 5930 | 5645 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8690 | 8930 | 9110 | 9395 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1565 | 1310 | 2310 | 2040 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8830 | 9085 | 8970 | 9240 |

Based on the table above, IMD3 and IMD5 hit on top of own B2 RX and B4 RX, respectively. MSD’s for these cases are already defined in 2UL/2DL specifications for UL CA_2A_4A. None of the IMD fall on top of own B7 RX.

2UL inter-band CA UE can have impacts on other systems such as WiFi, Bluetooth and GNSS due to its harmonics and intermodulations. Table 6.X.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.X.3-2: 2UL B2+B4 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | IMD3, IMD5 |
| Galileo | 1559 | - | 1591 | YES |  | IMD3, IMD5 |
| GLONASS | 1591 | - | 1610 | YES |  | IMD3 |
| GPS | 1563 | - | 1587 | YES |  | IMD3, IMD5 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | NO | US/Europe |  |
|  | 2400 | - | 2494 | NO | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | 3rd harmonic, IMD3, IMD5 |
|  | 5150 | - | 5350 | YES | Europe | 3rd harmonic, IMD3, IMD5 |
|  | 5470 | - | 5725 | YES |  | 3rd harmonic, IMD3, IMD5 |
|  | 5150 | - | 5825 | YES | Asia | 3rd harmonic, IMD3, IMD5 |

### 6.x.4	∆TIB and ∆RIB values

For CA_3DL_2A-4A-7A_2UL_2A-4A, the same ∆TIB and ∆RIB can apply as in respective 1UL-3DL case.

Table 6.X.4-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_2A-4A-7A | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 7 | 0.5 |

Table 6.X.4-2: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_2A-4A-7A | 2 | 0.3 |
|  | 4 | 0.5 |
|  | 7 | 0.5 |

## 6.x	CA_4DL_2A-4A-7A-7A_2UL_2A-4A_BCS0

### 6.x.3 Co-existence issues

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 1710 | 1755 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 3420 | 3510 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 5130 | 5265 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 200 | 95 | 3560 | 3665 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1945 | 2110 | 1510 | 1660 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5410 | 5575 | 5270 | 5420 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3795 | 4020 | 3220 | 3415 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7260 | 7485 | 6980 | 7175 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 190 | 400 | 7120 | 7330 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5170 | 4930 | 5930 | 5645 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8690 | 8930 | 9110 | 9395 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1565 | 1310 | 2310 | 2040 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8830 | 9085 | 8970 | 9240 |

Based on the table above, IMD3 and IMD5 hit on top of own B2 RX and B4 RX, respectively. MSD’s for these cases are already defined in 2UL/2DL specifications for UL CA_2A_4A. None of the IMD fall on top of own B7 RX.

2UL inter-band CA UE can have impacts on other systems such as WiFi, Bluetooth and GNSS due to its harmonics and intermodulations. Table 6.X.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.X.3-2: 2UL B2+B4 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | IMD3, IMD5 |
| Galileo | 1559 | - | 1591 | YES |  | IMD3, IMD5 |
| GLONASS | 1591 | - | 1610 | YES |  | IMD3 |
| GPS | 1563 | - | 1587 | YES |  | IMD3, IMD5 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | NO | US/Europe |  |
|  | 2400 | - | 2494 | NO | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | 3rd harmonic, IMD3, IMD5 |
|  | 5150 | - | 5350 | YES | Europe | 3rd harmonic, IMD3, IMD5 |
|  | 5470 | - | 5725 | YES |  | 3rd harmonic, IMD3, IMD5 |
|  | 5150 | - | 5825 | YES | Asia | 3rd harmonic, IMD3, IMD5 |

### 6.x.4	∆TIB and ∆RIB values

For CA_4DL_2A-4A-7A-7A_2UL_2A-4A, the same ∆TIB and ∆RIB can apply as in respective 1UL-4DL case.

Table 6.X.4-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_2A-4A-7A-7A | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 7 | 0.5 |

Table 6.X.4-2: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_2A-4A-7A-7A | 2 | 0.3 |
|  | 4 | 0.5 |
|  | 7 | 0.5 |

<End of Text Proposal>