3GPP TSG-RAN WG4 Meeting #80bis	R4-168643

Ljubljana, Slovenia, 10 - 14 Oct, 2016

Agenda Item:	8.28.3.1

Source: 	Intel Corporation

Title: 	Discussion on reference receiver for bidirectional HST SFN channels

Document for:	Discussion

- Introduction

The work item (WI) “Performance enhancements for high speed scenario” was approved at RAN plenary meeting #70 [1]. One objective of this WI is to identify downlink demodulation requirements under the identified high speed scenarios. A non-fading “2-path” high speed train (HST) SFN channel model was provided and agreed in high speed scenario study item (SI) phase for the bidirectional SFN with RRH deployment [2]. However, in RAN4#78 Meeting, the agreement on scenario assumptions was made that simulation assumptions agreed in SI are treated as baseline, and other assumptions are not precluded [3]. Moreover, various multi-path (more than two paths) HST SFN channels were proposed in RAN4#78bis Meeting [3]-[5], among which a “4-path” HST SFN channel model was chosen as the baseline multi-path HST SFN channel model in the high speed WI [4].

In RAN4#80 Meeting, a reference receiver was agreed [6] as:

- UE assuming U-shape Doppler spectrum and always covering all paths in Doppler spectrum with AFC on.

And companies are encouraged to provide simulation results of the reference receive based on the baseline “4-path” HST SFN channel assumptions. In this contribution, we first provide simulation results to show the throughput performance under the baseline “4-path” channel assumption, and then share our view on the reference receiver.

- Baseline “4-path” HST SFN channel model and RRH scenarios

Fig. 1 Diagram of “4-path” HST SFN with bidirectional RRHs

Fig.1 shows an example of a “4-path” HST SFN channel model proposed in [4]. Under this assumption, a high speed train can receive four copies of SFN signals from four consecutive RRHs. Table 1 lists the simulation parameters as specified in the Way Forward [7], and the maximum Doppler shift fd is assumed to be +/-875Hz. For the reference receiver, in order to make sure that all paths are covered in the Doppler spectrum with AFC on, we make the maximum Doppler spread to be 1750Hz.

Table 1: Simulation assumptions for UE demodulation performance requirements under SFN scenario (FDD)

| Parameters |  | Unit | Values |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| MCS |  |  | MCS#17 |
| scenario |  |  | Ds=1000, Dmin=50 |
| Propagation condition and correlation matrix | SFN |  | Dynamic SFN channel model: - normalized 4-tap channel model agreed in WF(R4-163419); - Doppler shift, relative time delay and relative power change with time, which is specified in WF (R4-163027) - Static channel matrix as defined in B.1 in 36.101; |
| Velocity |  |  | 350km/h |
| Antenna configuration |  |  | 2x2 |
| Transmission mode |  |  | TM3 |
| Reference receivers |  |  | UE assuming U-shape Doppler spectrum and always covering all paths (e.g. 1750Hz) in Doppler spectrum with AFC on. |
| Noise estimation |  |  | Practical |
| Time and frequency track |  |  | Practical |

- Simulation assumptions and results

Fig.2 illustrates the normalized throughput, relative to the maximum configured throughput for FDD mode.

Fig.2 Normalized throughput for FDD

- Observation: It shows that, for FDD, about 70% of the maximum configured throughput can be achieved by the reference receiver with 1750Hz Doppler spread.

- Proposal: On the reference receiver, it needs further clarification for performance alignment. In order to cover all paths in Doppler spectrum with AFC on, the maximum Doppler spread should be set to twice as large as the maximum Doppler shift, for example, Doppler spread=1750Hz if Doppler shift=+/-875Hz.

- Conclusion

In this contribution, we first provide simulation results to show the throughput performance under the baseline “4-path” channel assumption, and then share our view on the reference receiver.

- Observation: It shows that, for FDD, about 70% of the maximum configured throughput can be achieved by the reference receiver with 1750Hz Doppler spread.

- Proposal: On the reference receiver, it needs further clarification for performance alignment. In order to cover all paths in Doppler spectrum with AFC on, the maximum Doppler spread should be set to twice as large as the maximum Doppler shift, for example, Doppler spread=1750Hz if Doppler shift=+/-875Hz.

- Reference

[1] RP-152263, “New WI proposal: Performance enhancements for high speed scenario”, NTT DoCoMo Inc., Huawei, HiSilicon, Dec. 2015.

[2] R4-151587, “Channel model for SFN deployment with RRH for HST scenario”, Qualcomm, April 2015.

[3] R4-163052, “Way forward on additional channel model for bidirectional SFN scenario”, Huawei, HiSilicon, Intel, MediaTek Inc., NTT DOCOMO, INC, CMCC, ITRI, Qualcomm, Samsung, Ericsson, CATT, April 2016.

[4] R4-162016, “SFN channel model for advanced UE performance requirements”, Huawei, HiSilicon, April 2016.

[5] R4-161632, “SFN channel model in high speed scenarios”, Intel, April 2016.

[6] R4-166805, “Way forward on UE performance enhancement under SFN scenario”, Huawei, HiSilicon, NTT DOCOMO, Qualcomm Incorporated, MediaTek Inc., CMCC, CATT, ZTE, Ericsson, August 2016.

[7] R4-166814, “Simulation assumptions for UE demodulation performance requirements in SFN scenario”, Huawei, HiSilicon, August 2016.