3GPP TSG-RAN WG4 Meeting #80bis 	R4-168840

Ljubliana, Slovenia, 10 – 14 October, 2016

Source:	SoftBank Corp.

Title:	TP for TR36.714-04-01: the support of CA_1A-3A-8A-11A BCS0

Agenda item:	8.4.2

Document for:	Approval

1.  Introduction

B1+B3+B8+B11 was approved in 4DL/1UL basket WID in the last RAN-P [1]. This paper is to evaluate IM/harmonics and relaxation required. This includes the relevant TP to complete this combination.

Note that this 4DL includes an outstanding 2DL of 3A-11A [2] and its relevant 3DLs (1A/8A-3A-11A) [3, 4], which are to be dealt with companion contributions for approval in this meeting.

In R4-168840, two modifications are made in REFSENS exception (7.3.1A-0bD1): (1) 4Rx REFSENS is defined/added in Band 1 and (2) [ ] are removed in Band 3.

2. Basic Configurations

As written in the WID, the following configurations are assumed for B1+B3+B8+B11.

Table 1: 4DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  |  |  | Downlink (DL) band |  |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  |  | Channel BW (MHz) | BS transmit / UE receive |  |  | Channel BW (MHz) |  |
|  |  | FUL_low – FUL_high |  |  |  |  | FDL_low – FDL_high |  |  |  |  |
| CA_1-3-8-11 | 1 | 1920 MHz | – | 1980 MHz |  | 5,10, 15, 20 | 2110 MHz | – | 2170 MHz | 5, 10, 15, 20 | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz |  | 5,10, 15, 20 | 1805 MHz | – | 1880 MHz | 5, 10, 15, 20 |  |
|  | 8 | 880 MHz | – | 915 MHz |  | 5, 10 | 925 MHz | – | 960 MHz | 5, 10 |  |
|  | 11 | 1427.9 MHz | – |  | 1447.9 MHz | 5, 10 | 1475.9 MHz | – | 1495.9 MHz | 5, 10 |  |

Table 2: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_1A-3A-8A-11A | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  | 8 |  |  | Yes | Yes |  |  |  |  |
|  | 11 |  |  | Yes | Yes |  |  |  |  |

3.  Co-existence

The 3rd order IMD products caused in the BS by transmitting of Band 1, Band 3, Band 8 and Band 11 DL carriers (i.e. 4DL CA with one component carrier in each band) are derived in the constituent 3DL and 2DL CAs, which have been already analyzed.

4.  ΔTIB and ΔRIB

The followingΔTIB and ΔRIB are proposed based on its constituent 3DL/1UL CAs:

Table 3: ΔTIB of constituent 3DL/1UL

| ΔTIB | 1+3+8 | 1+3+11 | 1+8+11 | 3+8+11 | Proposed Value |
| --- | --- | --- | --- | --- | --- |
| B1 | 0.3 | 0.3 | 0.3 | - | 0.3 |
| B3 | 0.3 | 0.8 | - | 0.8 | 0.8 |
| B8 | 0.3 | - | 0.3 | 0.3 | 0.3 |
| B11 | - | 0.9 | 0.4 | 0.9 | 0.9 |

Table 4: ΔRIB of constituent 3DL/1UL

| ΔRIB | 1+3+8 | 1+3+11 | 1+8+11 | 3+8+11 | Proposed Value |
| --- | --- | --- | --- | --- | --- |
| B1 | 0 | 0 | 0 | - | 0 |
| B3 | 0 | 0.3 | - | 0.3 | 0.3 |
| B8 | 0 | - | 0 | 0 | 0 |
| B11 | - | 0.5 | 0 | 0.5 | 0.5 |

Based on the tables above, the following relaxations are proposed:

Table 5: Proposed ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_1A-3A-8A-11A | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 8 | 0.3 |
|  | 11 | 0.9 |

Table 6: Proposed ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_1A-3A-8A-11A | 1 | 0 |
|  | 3 | 0.3 |
|  | 8 | 0 |
|  | 11 | 0.5 |

5.  REFSENS

There are two issues about this 4CA. Firstly, B1+B3 CA is subject to REFSENS exception as per Table 7.3.1A-0bA of [5], when lower side of Band 1 is used for Tx and upper side of Band 3 is used for Rx. Then for this combination, it is necessary to add exception information below to Table 7.3.1A-0bD1/D2 of [5]:

Table 7(7.3.1A-0bD1): Reference sensitivity for carrier aggregation QPSK PREFSENS, CA

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-8A-11A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.76 | -99.76 | -97.96 | -96.76 |  |
|  | 34 |  |  | -94 | -91.5 | -90 | -89 |  |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
|  |  |  |  | -99.76 | -96.76 | -94.96 | -93.76 |  |
|  | 8 |  |  | -97 | -94 |  |  |  |
|  | 11 |  |  | -100 | -97 |  |  |  |
| NOTE 4: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 5: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 6: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

Table 8(7.3.1A-0bD2): Uplink configuration for reference sensitivity

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A-8A-11A | 11,2 |  |  | 25 | 25 | 25 | 25 | FDD |
|  | 11,3 |  |  | 25 | 45 | 45 | 45 |  |
| NOTE 1: refers to the UL resource blocks shall be located as close as possible to the downlink channel in Band 3 but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1) in the uplink channel in Band 1. NOTE 2: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz NOTE 3: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. |  |  |  |  |  |  |  |  |

Secondly, B3+B8 is a combination of harmonic fall down in UE but it was agreed that this issue is not specified. Because of that, Band 3 REFSENS is “NA” when Band 8 Tx hits the B3 Rx. Then, the following exception is required to be defined in Table 7.3.1A-0a and 0b of [5]:

Table 9 (7.3.1A-0a): Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-8A-11A4 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.720 | -99.720 | -97.920 | -96.720 |  |
|  | 3 |  |  | N/A | N/A | N/A | N/A |  |
|  | 8 |  |  | -97 | -94 |  |  |  |
|  | 11 |  |  | -100 | -97 |  |  |  |
| NOTE 4: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 20: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

Table 10(7.3.1A-0b): Uplink configuration for the low band (exceptions)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A-8A-11A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |

As mentioned in [2], there is no desense issue foreseen around B3+B11.

6.  Summary

This paper investigates all the necessary elements to complete CA_1A-3A-8A-11A. The relevant TP is shown in Annex.

7.  Reference

[1]      RP-161843   Revised WID: Basket WI for LTE Advanced Inter-band CA Rel-14 for 4DL/1UL, 
Ericsson (Rapporteur).

[2]      R4-167339    TP for TR36.714-02-01: the support of CA_3A-11A BCS0, SoftBank Corp.

[3]      R4-167340    TP for TR36.714-03-01: the support of CA_1A-3A-11A BCS0, SoftBank Corp.

[4]      R4-167341    TP for TR36.714-03-01: the support of CA_3A-8A-11A BCS0, SoftBank Corp.

[5]      TS36.101(v14.1.0)

Annex: TP for TR36.714-04-01 (v0.3.0)

[Unchanged portions skipped]

6.Z	CA_4DL_1A-3A-8A-11A_1UL_BCS0

6.Z.1	Channel bandwidths per operating band for CA

Table 6.Z.1-1: 4DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  |  |  | Downlink (DL) band |  |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  |  | Channel BW (MHz) | BS transmit / UE receive |  |  | Channel BW (MHz) |  |
|  |  | FUL_low – FUL_high |  |  |  |  | FDL_low – FDL_high |  |  |  |  |
| CA_1-3-8-11 | 1 | 1920 MHz | – | 1980 MHz |  | 5,10, 15, 20 | 2110 MHz | – | 2170 MHz | 5, 10, 15, 20 | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz |  | 5, 10, 15, 20 | 1805 MHz | – | 1880 MHz | 5, 10, 15, 20 |  |
|  | 8 | 880 MHz | – | 915 MHz |  | 5,10 | 925 MHz | – | 960 MHz | 5, 10 |  |
|  | 11 | 1427.9 MHz | – |  | 1447.9 MHz | 5, 10 | 1475.9 MHz | – | 1495.9 MHz | 5, 10 |  |

Table 6.Z.1-2: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_1A-3A-8A-11A | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  | 8 |  |  | Yes | Yes |  |  |  |  |
|  | 11 |  |  | Yes | Yes |  |  |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in table 5.6.1-1 of TS 36.101 [4] when operating in single carrier mode.

6.Z.2	Co-existence studies

The 3rd order IMD products caused in the BS by transmitting of Band 1, Band 3, Band 8 and Band 11 DL carriers (i.e. 4DL CA with one component carrier in each band) are derived in the constituent 3DL and 2DL CAs, which have been already analyzed.

6.Z.3	ΔTIB,c and ΔRIB,c values

For the UE which supports inter-band carrier aggregation configurations with uplink assigned to one E-UTRA band the ΔTIB,c and ΔRIB,c  is defined for applicable bands in Tables 6.Z.3-1 and 6.Z.3-2 respectively.

Table 6.Z.3-1: ΔTIB,c for 4DLs aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-8A-11A | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 8 | 0.3 |
|  | 11 | 0.9 |

Table 6.Z.3-2: ΔRIB,c for 4DLs aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-8A-11A | 1 | 0 |
|  | 3 | 0.3 |
|  | 8 | 0 |
|  | 11 | 0.5 |

6.Z.4	REFSENS

There are two issues about this 4CA. Firstly, B1+B3 CA is subject to REFSENS exception when lower side of Band 1 is used for Tx and upper side of Band 3 is used for Rx. Then for this combination, it is necessary to add exception information below:

Table 6.Z.4-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-8A-11A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.76 | -99.76 | -97.96 | -96.76 |  |
|  | 34 |  |  | -94 | -91.5 | -90 | -89 |  |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
|  |  |  |  | -99.76 | -96.76 | -94.96 | -93.76 |  |
|  | 8 |  |  | -97 | -94 |  |  |  |
|  | 11 |  |  | -100 | -97 |  |  |  |
| NOTE 4: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 5: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 6: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

Table 6.Z.4-2: Uplink configuration for reference sensitivity

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A-8A-11A | 11,2 |  |  | 25 | 25 | 25 | 25 | FDD |
|  | 11,3 |  |  | 25 | 45 | 45 | 45 |  |
| NOTE 1: refers to the UL resource blocks shall be located as close as possible to the downlink channel in Band 3 but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1) in the uplink channel in Band 1. NOTE 2: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz NOTE 3: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. |  |  |  |  |  |  |  |  |

Secondly, B3+B8 is a combination of harmonic fall down in UE but it was agreed that this issue is not specified. Because of that, Band 3 REFSENS is “NA” when Band 8 Tx hits the B3 Rx. Then, the following exception is required to be defined also:

Table 6.Z.4-3: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-8A-11A4 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.720 | -99.720 | -97.920 | -96.720 |  |
|  | 3 |  |  | N/A | N/A | N/A | N/A |  |
|  | 8 |  |  | -97 | -94 |  |  |  |
|  | 11 |  |  | -100 | -97 |  |  |  |
| NOTE 4: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 20: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

Table 6.Z.4-4: Uplink configuration for the low band (exceptions)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A-8A-11A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |

[Unchanged portions skipped]

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"