3GPP TSG-RAN WG4#82	R4-1700495

February 13-17, 2017 Athens Greece

Agenda item:	3.3

Source: 	Sprint

Title: 	Way Forward for 5G NR Frequency Bands

Document for:	Approval

# Introduction

This contribution proposes a way forward on the 5G NR bands to include in the 5G NR WID. RAN#71 tasked RAN4 with developing a recommendation for the frequency bands to include in the 5G NR WID.

At RAN4#1 NR–Adhoc companies submitted proposals for bands to be included in Release 15 version of 5G NR [1-14]. [15] .  The results of the submissions are shown in Table 1.

Table 1: Summary of NR Band contributions to 3GPPRAN4-NR#1

| Company | R4-1700xxx | Proposal |
| --- | --- | --- |
| Sprint, Nokia, Ericsson, CTC, C-Spire, China Unicom | 255 | 2496MHz – 2690 MHz |
| China Telecom | 256 | 824- 849MHz/869 – 894MHz, 1920 – 1980MHz/2110 – 2170MHz, 1710 – 1785MHz/1805 – 1880MHz, 2496 – 2600MHz, 3300 - 3400MHz, 3400 – 3600MHz, 4400 - 4500MHz, 4800 - 4990MHz |
| AT&T | 034 | 37-37.6GHz, 37.6-40GHz |
| DISH Network | 193 | 12.2-12.7GHz |
| Orange | 194 | 703-748MHz/758-803MHz,1920-1980MHz/2110-2170MHz,1710-1785MHz/1805-1880MHz,2500-2570MHz/2620MHz-2690MHz,3400-3600MHz,3600-3800MHz,5.925-8.5GHz,24.25-27.5GHz,31.8-33.4GHz |
| KT | 210 | 880 – 915MHz/925 – 960MHz, 1920 – 1980MHz/2110 – 2170MHz, 1710 – 1785MHz/1805 – 1880MHz, 2300 – 2400MHz, 3400-3700MHz, 26.5-29.5GHz, 24.25-27.5GHz, 31.8-33.4GHz, 37-40.5GHz |
| CMCC | 162 | Around 3.5GHz |
| DOCOMO | 139 | 3.3-4.2GHz, 4.4-4.99GHz, 26.5-29.5GHz |
| China Unicom | 241 | 3400 - 3600 MHz, 4800 - 4990 MHz, 4400 – 4500 MHz, 3300 - 3400 MHz, 2496 - 2690 MHz, 24.5 GHz-27.5 GHz, 37 GHz-43.5 GHz |
| Verizon, T-mobile | 244 | 27.5 – 28.35 GHz |
| Etisalat | 267 | 1427 – 1518MHz, 3400 - 3600MHz, 10.7 – 11.7GHz, 24.25- 27.5GHz |
| Huawei, Hisilicon | 124, 267 | 3.3-[3.8GHz~4.2GHz], 4.4-4.99GHz, 24.25-27.5GHz, 26.5-29.5GHz, 37-40GHz, 40.5-43.5GHz, 1427 – 1518MHz, 3400 - 3600MHz, 10.7 – 11.7GHz |
| ZTE | 249 | 3.6-3.6GHz, 24.25-27.5GHz |
| Ericsson | 255, 162 | 2496MHz – 2690MHz, Around 3.5GHz |
| Nokia | 255 | 2496MHz – 2690MHz |

# Discussion

RAN4 does not have to determine the specific bands within each frequency range for the 5G NR WID to be approved.  For purposes of creating the WID the frequency ranges are all that needs to be included in the WID, the upper and lower frequency ranges for a particular band and the number of bands within a frequency range can be determined in the Work Item phase.  The number of bands needed for a given frequency range will depend on the selection of channel bandwidths supported by 5G NR, the regulatory requirements for the band, power amplifier capability and filter requirements needed to meet the needs of the regulatory authority(s).

A couple of other principles should be applied to the selection of 5G NR frequency ranges, one is that the frequency range needs to be supported by several companies (similar to what is done for carrier aggregation proposals) and the other is that every operator that put a proposal into RAN4#1 NR should be able to deploy 5G NR in at least one 5G NR frequency range.

Companies also expressed a concern that the number of bands needed to be limited to meet the Release 15 timeline and to reduce UE complexity.

RAN4 discussed a way forward [15] at the January NR ad-hoc in Spokane , Table 2 shows the frequency ranges proposed in [15] along with the companies supporting the frequency range.

Table 2:  Companies supporting 5G NR frequency Ranges

| Frequency Range | Companies supporting |
| --- | --- |
| 2496 – 2690 MHz | Sprint, Nokia, Ericsson, China Telecom, C-Spire, China Unicom |
| 3.3-4.2 GHz | China Telecom, Huawei, DoCoMo, KT, China Telecom, ZTE, CMCC, Ericsson, Orange |
| 4.4-4.99 GHz | China Telecom, Huawei, DoCoMo, China Telecom, Orange |
| 24.25-27.5 GHz | Orange, KT, Etisalat, Huawei, Hisilicon |
| 26.5-29.5 GHz | KT, DOCOMO, Huawei, Hisilicon |
| 24.25-29.5GHz | China Unicom |
| 27.5 GHz – 28.35 | Verizon, T-Mobile |
| 31.8-33.4 GHz | Orange, KT |
| 37-37.6 GHz | AT&T, KT |
| 37.6-40 GHz | AT&T, KT, Huawei, Hisilicon |

Apply the principles descripted above the proposed Frequency Ranges results in the frequency ranges show in Table 3:

Table 3: Frequency Range

| Frequency Range to use in Release 15 5G NR WID | Companies with contributions within the Frequency Range |
| --- | --- |
| 2496 – 2690 MHz | Sprint, Nokia, Ericsson, China Telecom, C-Spire, China Unicom |
| 3.3-4.2 GHz | China Telecom, Huawei, DoCoMo, KT, China Telecom, ZTE, CMCC, Ericsson, Orange |
| 4.4-4.99 GHz | China Telecom, Huawei, DoCoMo, China Telecom, Orange |
| 24.25-29.5 GHz | DOCOMO, China Unicom, Orange, KT, Etisalat, Huawei, Hisilicon, Verizon, T-Mobile |
| 31.8-33.4 GHz | Orange, KT |
| 37.6-40 GHz | AT&T, KT, Huawei, Hisilicon |

The frequency ranges in Table 3 are for release 15, further bands can and will be added in later release subject to the 3GPP process for adding new bands to 5G NR.

Proposal 1: RAN4 to recommend to RAN plenary that the scope of the frequency ranges in the     5G NR WID be limited to the frequency ranges shown in Table 3.

Proposal 2: Each of the frequency ranges may be subdivided into one or more NR bands

Proposal 3: RAN4 will determine the number of bands to include in 5G NR

# For Approval

Proposal 1: RAN4 to recommend to RAN plenary that the scope of the frequency ranges in the     5G NR WID be limited to the frequency ranges shown in Table 3.

Proposal 2: Each of the frequency ranges may be subdivided into one or more NR bands

Proposal 3: RAN4 will determine the number of bands to include in 5G NR

# Conclusion

The WID doesn’t require RAN4 to come up with a list of specific bands to include in the release 15 5G NR WID.  All that is needed to approve the WID is for RAN to come with a list of frequency ranges.  Historically developing new bands definitions takes a fair amount of effort and given the new challenges with frequency ranges above 6 6GHz there simply isn’t sufficient time left in the study item to come up with 5G NR band definitions. Developing band definitions for frequency ranges that overlap existing LTE bands below 6 GHZ is a much simpler task as the technical landscape is well known.

# Reference

- R4-1700255, 	“Frequency Bands for 5G-RAN”, 	SPRINT Corporation

- R4-1700015, 	“Possible frequency ranges for NR below 6GHz”, 	China Telecom

- R4-1700034, “Frequency bands for the NR Rel-15 Work Item”, AT&T GNS Belgium SPRL

- R4-1700124, “On NR bands”, 	Huawei, Hisilicon

- R4-1700136, 	“Overview on how to organize the NR band related work	Huawei”, Hauwei, Hisilicon

- R4-1700139,	“Potential harmonized bands for NR”,	NTT DOCOMO, INC.

- R4-1700153, 	“Discussion on NR Bands Definition”, 	ZTE, ZTE Microelectronics

- R4-1700162, 	“Dicussion on the proposal on harmonized TDD band plan in the 3.5GHz frequency range for 5G NR”, 	CMCC, Ericsson

- R4-1700193, 	“12 GHz band for NR frequency prioritization in 6-24GHz”, 	Dish Network

- R4-1700194, 	“Frequency bands for New Radio”, 	ORANGE

- R4-1700210, 	“Frequency Ranges Proposal for NR in Rel-15 WI”, 	KT Corporation

- R4-1700244, 	“Adds source T-Mobile”	Verizon, T-Mobile

- R4-1700249,	“Discussion on NR Bands Definition”,	ZTE, ZTE Microelectronics

- R4-1700256,	“Possible frequency ranges for NR below 6GHz”, 	China Telecom

- R4-1700304, 	“WF on NR spectra related work”, 	NTT DOCOMO