3GPP TSG-RAN WG4 Meeting #82	R4-1700707

# Athens, Greece, 13-17 February 2017

Source:	SoftBank Corp.

Title:	TP for TR36.714-03-01: Support of LTE-A Inter-band Carrier Aggregation (3DL/1UL) of Band 8, Band 11 and Band 28

Agenda item:	7.3.2

Document for:	Approval

1.  Introduction

3DL/1UL of 8A-11A-28A was approved in the basket WID [1]. This paper is to propose all the technical elements to complete this 3DL CA, including TP. Since the uplink is limited to Band 8 in the context of 8A-28A [2], this combination might need a new exception table in REFSENS.

2.  Basic Scheme

The bands and BW configurations for the CA are shown in Table 1 and 2.

Table 1: 3DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_8-11-28 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz |  |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |

Table 2: Supported E-UTRA bandwidths per CA configuration for 3DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_8A-11A-28A | 8 |  |  | Yes | Yes |  |  | 40 | 0 |
|  | 11 |  |  | Yes | Yes |  |  |  |  |
|  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE 1: For the UE that signals support of a 3DL CA bandwidth combination set, the UE shall support all single carrier channel bandwidths for the constituent bands as defined in table 5.6.1-1 of TS 36.101 and all the 2DL CA bandwidth combination sets in Table 5.6A.2-1 of TS 36.101 whose channel bandwidths are entirely covered by the signaled 3DL CA bandwidth combination set. |  |  |  |  |  |  |  |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all for the constituent bands as defined in Table 5.6.1-1 of TS 36.101 when operating in single carrier mode.

3.  Co-existence study

The 3rd order IMD products caused in the BS by transmitting of Band 8, Band 11 and Band 28 DL carriers (i.e. 3DL CA with one component carrier in each band) can be calculated as shown in Table 3 below:

Table 3: Band 8, Band 11 and Band 28 DL 3rd order IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high | f3-low | f3-high |
| --- | --- | --- | --- | --- | --- | --- |
| DL frequency (MHz) | 758 | 803 | 925 | 960 | 1475.9 | 1495.9 |
| 3rd order IMD products | ¦f1-high + f2-high – f3-low¦ |  |  | ¦f1-low + f2-low – f3-high¦ |  |  |
| IMD frequency limits (MHz) | 187.1 |  |  | 287.1 |  |  |
| 3rd order IMD products | ¦f1-low + f3-low – f2-high¦ |  |  | ¦f1-high + f3-high – f2-low¦ |  |  |
| IMD frequency limits (MHz) | 1273.9 |  |  | 1373.9 |  |  |
| 3rd order IMD products | ¦f2-low + f3-low – f1-high¦ |  |  | ¦f2-high + f3-high – f1-low¦ |  |  |
| IMD frequency limits (MHz) | 1597.9 |  |  | 1697.9 |  |  |
| 3rd order IMD products | ¦f1-low + f2-low + f3-low¦ |  |  | ¦f1-high + f2-high + f3-high¦ |  |  |
| IMD frequency limits (MHz) | 3158.9 |  |  | 3258.9 |  |  |

It can be seen from Table 3 that in addition to the impacts of harmonics and IMD products from the constituent 2DL CA configurations, the 3rd order IMD products supporting 3 DL CA of Band (8 + 11 + 28) may fall into BS receive band of Bands 24 and 70. However, Bands 24 and 70 are not intended for use in the same geographical area as Bands 8,11 and 28. Therefore, there is no concern foreseen on the 3rd order IMD products falling into 3GPP bands.

Table 4 gives the 2nd and 3rd harmonic for CA with B8+ B11 + B28. It can be seen that distortions caused by 2nd order harmonics of Band 28 located within the pass band of Band 11. It was already identified in previous work and specified in TS 36.101. Except 2nd order harmonics of Band 28, none of the products falls into the B8, B11 or B28 UE receiving bands.

Table 4: UE harmonic table for 3DL/1UL

| UE DL Carriers with B8, B11, B28 | [MHz] |
| --- | --- |
| 2*fx_low to 2*fx_high | 1760 to 1830 |
| 2*fy_low to 2*fy_high | 2855.8 to 2895.8 |
| 2*fz_low to 2*fz_high | 1406 to 1496 |
| 3*fx_low to 3*fx_high | 2640 to 2745 |
| 3*fy_low to 3*fy_high | 4283.7 to 4343.7 |
| 3*fz_low to 3*fz_high | 2109 to 2244 |

4.  ΔTIB and ΔRIB

A specific issue in CA_8A-28A is that the uplink is limited to Band 8 due to antenna performance reason [2]. The same situation goes for this 3CA then B28 as uplink should be out of scope. This means that HTF in Band 28 could be eliminated in 8A-11A-28A to protect B11 Rx from B28 Tx. However, taking 2DL into account, CA_11A-28A still permits Band 28 in uplink and this could be a part of constituent 2DL support. Thus, in this CA, it is proposed to assume that the HTF is still in B28.

Based on the constituent 2DL/1UL values in Table 5 and 6, the following values are proposed. ΔTIB and ΔRIB tables need a note to indicate the limitation.

Table 5: ΔTIB of constituent 2DL/1UL

| ΔTIB | 8+11 | 8+28 | 11+28 | Proposed Value |
| --- | --- | --- | --- | --- |
| B8 | 0.3 | 0.6 | - | 0.6 |
| B11 | 0.4 | - | 0.4 | 0.4 |
| B28 | - | 0.5 | 0.6 | 0.6 |

Table 6: ΔRIB of constituent 2DL/1UL

| ΔRIB | 8+11 | 8+28 | 11+28 | Proposed Value |
| --- | --- | --- | --- | --- |
| B8 | 0 | 0.2 | - | 0.2 |
| B11 | 0 | - | 0 | 0 |
| B28 | - | 0.1 | 0.2 | 0.2 |

Table 7: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_8A-11A-28A | 8 | 0.6 |
|  | 11 | 0.4 |
|  | 28 | 0.6 |
| NOTE 17: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 8 or Band 11. |  |  |

Table 8: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_8A-11A-28A | 8 | 0.2 |
|  | 11 | 0 |
|  | 28 | 0.2 |
| NOTE 16: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 8 or Band 11. |  |  |

5.  REFSENS

As mentioned above, this CA includes a harmonic issue when the uplink is Band 28 but in this CA, the requirements should be excluded for the cases of Band 28 uplink. TP simply mentions this fact.

In terms of REFSENS for CA, since current RAN4 spec (36.101) lists all the “exceptional cases” and the exception of this kind has not been addressed, it seems that we need to have a new table such as:

Table 9 (new): Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to uplink limitation)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_8A-11A-28A[1] | 8 |  |  | NA | NA |  |  | FDD |
|  | 11 |  |  | NA | NA |  |  |  |
|  | 28 |  |  | NA | NA | NA | NA |  |
| [NOTE 1: In this CA, Band 28 uplink is not supported.] |  |  |  |  |  |  |  |  |

Table 10 (New): Uplink configuration for a band (exceptions due to uplink limitation)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_8A-11A-28A | 28 |  |  | NA | NA | NA | NA | FDD |

On the other hand, CA_1-41 has been in the same situation but there has been no special exception written when B41 is (assumed to be) the uplink. So there might be interpretation that notes on ΔTIB and ΔRIB are sufficient. Thus,

[Proposal-1] We need to clarify the cases where the uplink is limited to certain band(s) in terms of REFSENS exception of CA.

6.  Conclusion

This paper is to propose all the necessary items to complete 8A-11A-28A. We need to clarify how to capture REFSENS exception of CA. TP is attached to capture the discussion above.

7.  Reference

[1]     RP-162146	Revision of WI LTE Advanced inter-band CA Rel-14 for 3DL/1UL, Huawei (Rapporteur)

[2]     TR36.714-02-01, section 6.16.7

Annex: TP for TR36.714-03-01 (v0.6.0)

[Unchanged portions skipped]

6.Z 	CA_8A-11A-28A_BCS0

6.Z.1	Operating bands for CA

Table 6.Z.1-1: 3DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_8-11-28 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz |  |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |

6.Z.2	Channel bandwidths per operating band for CA

Table 6.Z.2-1: Supported E-UTRA bandwidths per CA configuration for 3DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_8A-11A-28A | 8 |  |  | Yes | Yes |  |  | 40 | 0 |
|  | 11 |  |  | Yes | Yes |  |  |  |  |
|  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE 1: For the UE that signals support of a 3DL CA bandwidth combination set, the UE shall support all single carrier channel bandwidths for the constituent bands as defined in table 5.6.1-1 of TS 36.101 and all the 2DL CA bandwidth combination sets in Table 5.6A.2-1 of TS 36.101 whose channel bandwidths are entirely covered by the signaled 3DL CA bandwidth combination set. |  |  |  |  |  |  |  |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all for the constituent bands as defined in Table 5.6.1-1 of TS 36.101 when operating in single carrier mode.

6.Z.3 	Harmonics and IMD study

6.Z.3.1	For the BS

The 3rd order IMD products caused in the BS by transmitting of Band 8, Band 11 and Band 28 DL carriers (i.e. 3DL CA with one component carrier in each band) can be calculated as shown in Table 6.Z.3.1-1 below:

Table 6.Z.3.1-1: Band 8, Band 11 and Band 28 DL 3rd order IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high | f3-low | f3-high |
| --- | --- | --- | --- | --- | --- | --- |
| DL frequency (MHz) | 758 | 803 | 925 | 960 | 1475.9 | 1495.9 |
| 3rd order IMD products | ¦f1-high + f2-high – f3-low¦ |  |  | ¦f1-low + f2-low – f3-high¦ |  |  |
| IMD frequency limits (MHz) | 187.1 |  |  | 287.1 |  |  |
| 3rd order IMD products | ¦f1-low + f3-low – f2-high¦ |  |  | ¦f1-high + f3-high – f2-low¦ |  |  |
| IMD frequency limits (MHz) | 1273.9 |  |  | 1373.9 |  |  |
| 3rd order IMD products | ¦f2-low + f3-low – f1-high¦ |  |  | ¦f2-high + f3-high – f1-low¦ |  |  |
| IMD frequency limits (MHz) | 1597.9 |  |  | 1697.9 |  |  |
| 3rd order IMD products | ¦f1-low + f2-low + f3-low¦ |  |  | ¦f1-high + f2-high + f3-high¦ |  |  |
| IMD frequency limits (MHz) | 3158.9 |  |  | 3258.9 |  |  |

It can be seen from Table 6.Z.3.1-1 that in addition to the impacts of harmonics and IMD products from the constituent 2DL CA configurations, the 3rd order IMD products supporting 3 DL CA of Band (8 + 11 + 28) may fall into BS receive band of Bands 24 and 70. However, Bands 24 and 70 are not intended for use in the same geographical area as Bands 8,11 and 28. Therefore, there is no concern foreseen on the 3rd order IMD products falling into 3GPP bands.

6.Z.3.2	For the UE

Table 6.Z.3.2-1 gives the 2nd and 3rd harmonic for CA with B8+ B11 + B28. It can be seen that distortions caused by 2nd order harmonics of Band 28 located within the pass band of Band 11. It was already identified in previous work and specified in TS 36.101. Except 2nd order harmonics of Band 28, none of the products falls into the B8, B11 or B28 UE receiving bands.

Table 6.Z.3.2-1: UE harmonic table for 3DL/1UL

| UE DL Carriers with B8, B11, B28 | [MHz] |
| --- | --- |
| 2*fx_low to 2*fx_high | 1760 to 1830 |
| 2*fy_low to 2*fy_high | 2855.8 to 2895.8 |
| 2*fz_low to 2*fz_high | 1406 to 1496 |
| 3*fx_low to 3*fx_high | 2640 to 2745 |
| 3*fy_low to 3*fy_high | 4283.7 to 4343.7 |
| 3*fz_low to 3*fz_high | 2109 to 2244 |

6.Z.4	ΔTIB,c and ΔRIB,c values

A specific issue in CA_8A-28A is that the uplink is limited to Band 8 due to antenna performance reason. The same situation goes for this 3CA then B28 as uplink should be out of scope. This means that HTF in Band 28 could be eliminated in 8A-11A-28A to protect B11 Rx from B28 Tx. However, taking 2DL into account, CA_11A-28A still permits Band 28 in uplink and this could be a part of constituent 2DL support. Thus, in this CA, it is proposed to assume that the HTF is still in B28.

For the UE which supports inter-band carrier aggregation configurations with uplink assigned to one E-UTRA band the ΔTIB,c is defined for applicable bands in Table 6.Z.4-1.

Table 6.Z.4-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_8A-11A-28A | 8 | 0.6 |
|  | 11 | 0.4 |
|  | 28 | 0.6 |

For the UE which supports inter-band carrier aggregation configuration in Table 6.Z.4-2 with uplink in one E-UTRA band, the minimum requirement for reference sensitivity in Table 7.3.1-1 (of TS 36.101) shall be increased by the amount given in ΔRIB,c in Table 6.Z.4-2 for the applicable E-UTRA bands.

Table 6.Z.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_8A-11A-28A | 1 | 0.2 |
|  | 11 | 0 |
|  | 28 | 0.2 |

6.Z.5	REFSENS

As mentioned above, while this CA includes a harmonic issue when the uplink is Band 28, the requirements should be excluded for the cases of Band 28 uplink due to the limitation in CA_8A-28A.

[Unchanged portions skipped]

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"