3GPP TSG-RAN WG4 #82	R4-1700992

# Athens, Greece, 13th – 17th February 2017

Agenda Item:	7.19.4.2

Source:	Ericsson

Title:	Further discussion on the open issues for V2V demodulation

Document for:	Discussion

# Introduction

In RAN4#80bis meeting, test case list and demodulate purpose for V2V demodulation were discussed. In RAN4#81 meeting, some new WFs were agreed in [4] [5]. There are still some open issues for V2V demodulation. In this paper, we share our view on these open issues.

# General

## Resource allocation modelling in the V2V transmission

Resource allocations modelling was discussed a bit in the past meeting, but there are no formal agreements on the resource allocations modelling up to now. In this section, we further elaborate how to model resource allocation in the V2V transmission.

According to the current RAN4 agreements, in current V2V scope, we will only define requirements for standalone scenarios. For the non-standalone scenario, it will be defined in V2X work item. For the standalone mode, the resource allocation can only be based on mode 4. In Mode 4, Resource allocation is based on distributed scheduling, wherein the scheduling is based on a sensing with semi-persistent transmission based mechanism. The detail description is shown in 14.1.1.4B of 36.213. According to the specification, the resource for PSSCH transmission shall be according to:

If a set of sub-channels in subframe  is determined as the time and frequency resource for PSSCH transmission corresponding to the configured sidelink grant (described in [8]), the same set of sub-channels in subframes  are also determined for PSSCH transmissions corresponding to the same sidelink grant where j=1, 2,…, ,  is the resource reservation interval determined by higher layers, and  is determined by subclause 14.1.5.

When retransmission is configured, one resource is selected using the mechanism defined for the case where a TB is transmitted in one subframe and Selection of another resource is random under some condition. When the resources reselection is triggered, with probability p, the UE keeps the current resource and the counter is reset and with 1-p probability, resource is reselected based on some rule.  In Figure 1, one example resource allocation pattern is given. In Figure 1, the periodicity of new transmission is 100ms and the retransmission is performed with gap k, which is randomly given within (0,15].  The gap is the gap between the initial transmission and retransmission. After Cresel transmission, the subchannel is reselected. A uniform distribute number p is generated, when p is larger than 0.4, the same subchannel is reused, otherwise, a different channel is selected for the new transmission.

Figure 1: Resource allocation for V2V communication

In order to comply the specification, it is unavoidable to explicitly model the signal pattern. One example parameter can be referred in Table 1.

Table 1: Test parameters for V2V resource allocation

| Parameters | Unit | Value |
| --- | --- | --- |
| Resource reservation in SCI |  | 1 |
| SL_RESOURCE_RESELECTION_COUNTER |  | 5 |
| Time gap between initial transmission and retransmission in SCI | ms | Random selection within (0,15] |
| Carrier-specific parameter p for reselection |  | 0.4 |

To reduce the test time, one possible way is to simultaneously model multiple transmission UEs. For example, 50 UEs are modeled in the test. Each UE have the same channel model but with different random seeds to generate the channel model.

Simplified model of automatic resource selection may be needed for demodulation and multiple transmission UEs are modelled with different channel generation random seeds to reduce the test time.

# Single link requirements

For single link test, there are mainly two issues which are open, namely:

- Propagation condition

  - Option 1: EVA180, QPSK/16QAM

  - Option 2: EVA1500, QPSK

  - Option 3: EVA2700, QPSK

  - Option 4: EVA1000, QPSK/16QAM

  - Other options are not precluded and down selection in next meeting

- CFO and Doppler Shift Estimation Algorithm

  - Candidate estimation algorithms

    - Method 1: “Single-DMRS” estimation

    - Method 2: “Cross-DMRS” estimation

  - Performance requirements

    - Option 1: V2V minimum demodulation performance requirements are defined based on Method #1

    - Option 2: V2V minimum demodulation performance requirements are defined based on Method #2

    - Option 3: Different UE capabilities are defined for Methods #1 and #2. Separate V2V minimum demodulation performance requirements are defined for UEs with different capabilities.

For EVA2700, it is corresponding to 500km/h at 5.9GHz. This is the target speed which is set in RAN plenary and it is the working assumption in RAN1 specification development. Thus, at least one test case shall cover this speed. Otherwise, the usage case of LTE V2V specification will be limited.

For the MCS, at least one lower MCS shall be selected to implicitly test PSCCH. Just as analysis in [6], it may be challenge to separate PSCCH test with PSSCH test, it is only feasible to implicitly test PSCCH. To implicitly test PSCCH performance, the target SNR of PSSCH shall be very close to the work point of PSCCH. As one example, the I_TBS could be used for PSSCH simulation. The detail setup is shown in Table 2.

Table 2: PSSCH setup for PSCCH test

| Parameters | Unit | Value | Value |
| --- | --- | --- | --- |
| Channel bandwidth |  | 20 | 10 |
| Number of PRBs |  | 96 | 48 |
| Number of retransmission |  | 2 | 2 |
| Modulation |  | QPSK | QPSK |
| I_TBS |  | 0 | 0 |
| TBS | Bits | 2664 | 1320 |
| Metric |  | Joint performance of PSCCH and PSSCH | Joint performance of PSCCH and PSSCH |

To cover high SNR, in additional to the lower SNR which is covered by lower MCS, 16QAM can be tested in multiple link test. Thus, we can have the setup for each test and its corresponding test purposes as shown in Table 3.

Table 3: Test setup for demodulation requirements and its corresponding test purposes

| Test number | Test configuration | Channel model and MCS selection | Test purpose |
| --- | --- | --- | --- |
| 1 | Single link | EVA180, QPSK | Implicitly test PSCCH performance Test PSSCH performance under lower SNR Test PSSCH performance under lower speed |
| 2 | Single link | EVA2700, QPSK | Test PSSCH performance under high speed |
| 3 | Multiple link | EVA180, 16QAM for one link EVA1500 for QPSK for the second link | Test PSSCH under high SNR Capability to perform simultaneous reception of multiple V2V links with different propagation conditions (time offset, carrier frequency error, delay spread, Doppler spread) Cover medium speed |

Considering Table 3 as a reference for the setup for single link performance test and multiple link performance test

Regarding “Single-DMRS” estimation and “Cross-DMRS” estimation, several contributions have similar observations: there will be error floor for the basic receiver based on “cross-DMRS” scheme [3]. Thus, it seems “single DMRS” is the few choices what we can choose. The complexity of “single DMRS” may be higher than “cross-DMRS”. However, considering the V2V is not battery limited, a complex scheme may be affordable. Thus, RAN4 can target for a receiver to achieve the target high speed (500 km/h).

Regarding how to define performance requirements, it is not clear how to works for option 3. If option 3 is used, it means, in high speed scenario, some UE can handle this scenario and some UE cannot handle this scenario. It further means some pairs can communicate with each other and some pairs cannot communicate with each other. In such scenario, some LTE V2V traffic becomes meaningless. The safety cannot be guaranteed in such system. it hints this specification cannot be used for high speed at all. Thus, it will have very strong limitation on the usage scenario for LTE V2V specification. To make the LTE V2V specification attractive and general, it is better to define performance requirements based on “single-DMRS” estimation.

V2V minimum demodulation performance requirements are defined based on “single-DMRS” estimation

# Multiple link test

## Multiple link performance test

In [1], the group have the following agreements:

- Multi-link PSSCH and PSCCH demodulation performance

  - Capability to perform simultaneous reception of multiple V2V links with different propagation conditions (time offset, carrier frequency error, delay spread, Doppler spread)

  - Capability to perform reception of links with max power imbalance

As discussed in Section 3, multiple-link PSSCH and PSCCH demodulate performance can be used for:

Test PSSCH under high SNR

Capability to perform simultaneous reception of multiple V2V links with different propagation conditions (time offset, carrier frequency error, delay spread, Doppler spread)

Cover medium speed

For multiple link transmission, in RAN1, they have the following agreements:

The number of RBs, and number of transmission subframes depending on the UE absolute speed and UE synchronization source (e.g. GNSS or eNB)

In RAN2, the corresponding signaling are defined in 36.331 as:

SL-PSSCH-TxConfig-r14 ::=		SEQUENCE {

typeTxSync-r14				ENUMERATED {gnss, enb, ue, spare1}		OPTIONAL,	-- Need OR

thresUE-Speed-r14			ENUMERATED {kmph60, kmph80, kmph100, kmph120,

kmph140, kmph160, kmph180, kmph200},

parametersAboveThres-r14	SL-PSSCH-TxParameters-r14,

parametersBelowThres-r14	SL-PSSCH-TxParameters-r14,

...

}

SL-PSSCH-TxParameters-r14 ::=		SEQUENCE {

minMCS-PSSCH-r14			INTEGER (0..31),

maxMCS-PSSCH-r14			INTEGER (0..31),

minRB-NumberPSSCH-r14		INTEGER (1..100),

maxRB-NumberPSSCH-r14		INTEGER (1..100),

allowedRetxNumberPSSCH-r14	ENUMERATED {n0, n1, both, spare1}

}

Based on the signaling, when the UE speed is higher than configured threshold, it will select MCS and number of transmission based on one set, and when the UE speed is lower than configured threshold, it will select MCS and number of transmission based on another set. Based on this procedure, it is beneficial for reception UE to make some performance optimization based on TX speed and Tx Sync type. In order to verify the UE with the right behavior, we can consider the configurations in the multiple link test as Table 4.

Table 4: Test Parameters for the multiple link test

| SL-PSSCH-TxConfig | typeTxSync-r14 |  |  | Gnss |
| --- | --- | --- | --- | --- |
|  | thresUE-Speed-r14 |  |  | kmph100 |
|  | parametersAboveThres | minMCS-PSSCH |  | TBD |
|  |  | maxMCS-PSSCH-r14 |  | TBD |
|  |  | minRB-NumberPSSCH |  | TBD |
|  |  | maxRB-NumberPSSCH |  | TBD |
|  |  | allowedRetxNumberPSSCH |  | n1 |
|  | parametersBelowThres | minMCS-PSSCH |  | TBD |
|  |  | maxMCS-PSSCH-r14 |  | TBD |
|  |  | minRB-NumberPSSCH |  | TBD |
|  |  | maxRB-NumberPSSCH |  | TBD |
|  |  | allowedRetxNumberPSSCH |  | n0 |
| Active Sidelink UE(s) |  |  |  | Sidelink UE 1, Sidelink UE 2 |
| Sidelink UE 1 | Sidelink Transmissions |  |  | PSCCH + PSSCH |
|  | Propagation Channel |  |  | EVA 1500 (corresponding to 270km/h@5.9GHz) |
| Sidelink UE 2 | Sidelink Transmissions |  |  | PSCCH + PSSCH |
|  | Propagation Channel |  |  | EVA 180 (corresponding to 30 km/h@5.9GHz) |

In the test, thresUE-Speed is set 100km/h, when the Tx speed is lower than 100km/h, there will be no retransmission, since the allowedRetxNumberPSSCH is n0 in the parametersBelowThres and when the Tx speed is higher than 100km/h, there will have one retransmission, since the the allowedRetxNumberPSSCH is n1 in the parameter parametersAboveThres. The speed is reflected in the channel model. For the first UE, EVA 1500Hz is used as the channel model and for the second UE, EVA 180Hz is used as the channel model. They are corresponding 30km/h and 270km/h, respectively.

For the TB from the UE 2, UE need soft combining for the two transmissions. For the TB from the UE 1, since one transmission is used, soft combining is not needed. Thus, according to the above configuration, both HARQ and non-HARQ is verified.

In [7], simulation assumptions are provided for different FRC. These FRCs can be reused in multiple link test It is recapped in Table 5. We can use QPSK 1/3 for UE1 and 16QAM ½ for UE2.

Table 5: Proposed FRC for PSSCH transmission

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name |  | FRC.1 | FRC.2 | FRC.3 | FRC.4 |
| Channel bandwidth | MHz | 10 | 20 | 10 | 20 |
| Allocated resource blocks |  | 48 | 96 | 48 | 96 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 |
| TTIs per period |  | 1 | 1 | 1 | 1 |
| Modulation |  | QPSK | QPSK | 16QAM | 16QAM |
| Target Coding Rate |  | 1/3 | 1/3 | 1/2 | 1/2 |
| Transport Block Size |  | 3496 | 6968 | 11448 | 22920 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Sub-Frame |  | 1 | 2 | 1 | 2 |
| Maximum number of HARQ transmissions |  | [1,2] | [1,2] | [1,2] | [1,2] |
| Binary Channel Bits per subframe | Bits | 11520 | 23040 | 23040 | 46080 |
| Max. Throughput averaged over 1 period of 100ms | kbps | 34.96 | 69.68 | 114.48 | 229.20 |
| UE Category |  | ≥ 1 | ≥ 1 |  |  |
| Note 1: 2RBs allocated to SA transmission and 4 symbols allocated to RS. Note 2: Throughput (in kbps) will depend on SA period configuration. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |  |  |

In order to reduce the test time, similar to the single-link test, multiple transmission UEs are modelled. These UEs are divided into two groups, one group UE use the same channel model but different random seeds and the same MCS as UE1 and the other group UE use the same channel model but different random seeds and the same MCS as UE2.

Configure different Doppler in the channel model for different links and configure different number of retransmission, MCS, number of PRBs per the speed in the multiple links.

# Conclusion

In this paper, we provide our view on some open issues for V2V demodulation, we have the following proposals:

Proposal 1	Simplified model of automatic resource selection may be needed for demodulation and multiple transmission UEs are modelled with different channel generation random seeds to reduce the test time.

Proposal 2	Considering Table 3 as a reference for the setup for single link performance test and multiple link performance test

Proposal 3	V2V minimum demodulation performance requirements are defined based on “single-DMRS” estimation

Proposal 4	Configure different Doppler in the channel model for different links and configure different number of retransmission, MCS, number of PRBs per the speed in the multiple links.

# References

- R4-168710, WF on V2V demodulation performance tests, Huawei, Huawei, HiSilicon, LGE, Qualcomm, Ericsson, Intel Corporation, Oct. 2016.

- RP-161603, Revision of WI: Support for V2V services based on LTE sidelink, LG, September, 2016.

- R1-163029, DM-RS Enhancements for V2V PSCCH and PSSCH, Qualcomm Incorporated, April 2016.

- R4-1610866, Way forward on single-link V2V performance requirements, Huawei, HiSilicon, LG Electronics, CATT, Nov. 2016

- R4-1610701, WF on V2V power imbalance tests, LGE, Huawei, HiSilicon, Qualcomm Inc., Ericsson, CATT, Intel, Nov. 2016

- R4-1609180, Discussion on the open issues for V2V demodulation, Ericsson, Nov. 2016.

- R4-1610853, Simulation assumptions for V2V demodulation requirements, Huawei, HiSilicon, LG Electronics, CATT, Nov. 2016.