3GPP TSG-RAN WG4 Meeting #82                                                                  R4-1701068

Athens, Greece, 13 – 17 February, 2017

Agenda Item:	10.2

Source:	Samsung

Title:		Priority bands for NR

Document for:	Approval

- Background

At its Adhoc meeting of RAN4 for NR in January 2017, operators were encouraged to provide initial inputs of possible frequency ranges or bands for NR deployment [1]. There was much argument based on the inputs as shown in Table 1.

Table 1: Summary of inputs to AH meeting from operators on NR bands

| Company | R4-1700xxx | Proposal |
| --- | --- | --- |
| Sprint, CTC, C-Spire, China Unicom | 255 | 2496-2690MHz |
| China Telecom | 256 | 824-849/869-894MHz, 1920-1980/2110 – 2170MHz, 1710-1785/1805-1880MHz, 2496-2690MHz, 3300-3400MHz, 3400-3600MHz, 4400-4500MHz, 4800-4990MHz |
| AT&T | 034 | 37-37.6GHz, 37.6-40GHz |
| DISH Network | 193 | 12.2-12.7GHz |
| Orange | 194 | 703-748/758-803MHz, 832-862/791-821MHz, 1920-1980/2110-2170MHz, 1710-1785/1805-1880MHz, 2500-2570/2620MHz-2690MHz, 3400-3600MHz, 3600-3800MHz, 5.925-8.5GHz, 24.25-27.5GHz, 31.8-33.4GHz |
| KT | 210 | 880-915/925-960MHz, 1920-1980/2110-2170MHz, 1710-1785/1805-1880MHz, 2300-2400MHz, 3400-3700MHz, 26.5-29.5GHz, 24.25-27.5GHz, 31.8-33.4GHz, 37-40.5GHz |
| CMCC | 162 | Around 3.5GHz |
| DOCOMO | 139 | 3.3-4.2GHz, 4.4-4.99GHz, 26.5-29.5GHz. |
| China Unicom | 241 | 3400-3600 MHz, 4800-4990MHz, 4400-4500MHz, 3300-3400MHz, 2496-2690MHz, 24.5-27.5GHz, 37-43.5GHz |
| Verizon, T-Mobile | 244 | 27.5-28.35GHz |
| Etisalat | 267 | 1427-1518MHz, 3400-3600MHz, 10.7-11.7GHz, 24.25-27.5GHz |

In this contribution, we would like to provide our understanding and priority bands for the NR standardization of Rel-15 given that the initial guidelines as described in [2].

- Discussion

As suggested in [2], RAN4 should to select the example band for Rel-15 NR standardization. Therefore, it is highly recommended that RAN4 should take into account having the criteria proposed in [2], i.e., regional plans (government-related), time for commercial deployments (operator-related) and technical feasibility (vendor-related) for each band, in order to satisfy clear market demands at the right time. In the following section corresponding aspects are analyzed respectively for the Rel-15 NR candidate frequency ranges.

Regional plans

For the first stage, the regional spectrum plans for NR deployments are introduced as shown in Figure 1 and 2. All information is narrowed down to the official announcement at the present written in public documents [3, 4].

Figure 1: Candidate bands for NR deployment – below 6GHz

Figure 2: Candidate bands for NR deployment – above 6GHz

Commercial deployment plan

For commercialization aspect, Table 2 shows the initial regional situations, as referred to in [3, 4], with the highly commercial deployment possibility and product availability before or immediately after the Rel-15 NR standardization finalization.

Table 2: Summary of regional situations and target dates

|  | Frequency bands | Time for deployments | Product availability (current stage)* | Remarks (for Rel-15 WI)* |
| --- | --- | --- | --- | --- |
| US | 28GHz: 27.5-28.35GHz | 2018 | [Y] | [Should be for Rel-15] |
|  | 38GHz: 37-40GHz |  | [N] |  |
| KOR | 3.5GHz: 3.4-3.7GHz | 2018 | [Y] | [Should be for Rel-15] |
|  | 28GHz: 26.5-29.5GHz |  | [Y] |  |
| JPN | 3.5GHz: 3.6-4.2GHz | 2020 | [Y] | [3.5GHz and 28GHz can be for Rel-15] [4.5GHz needs to be study, can be for Rel-16] |
|  | 4.5GHz: 4.4-4.9GHz |  | [N] |  |
|  | 28GHz: 27.5-29.5GHz |  | [Y] |  |
| EU | 3.5GHz: 3.4-3.8GHz | 2020 | [Y] | [3.5GHz can be for Rel-15] [26GHz needs to be study, can be for Rel-16] |
|  | 26GHz: 24.25-27.5GHz |  | [N] |  |
| CHN | 3.5GHz: 3.3-3.6GHz | 2020 | [Y] | [3.5GHz can be for Rel-15] [Others needs to be study, can be for Rel-16] |
|  | 4.5GHz: 4.4-4.5/4.8-4.99GHz |  | [N] |  |
|  | [WRC-19: 26GHz, 40GHz] |  | [N] |  |

(*: Examples based on analysis of the current situation in contributor’s point of view)

Therefore, according to the information in figures and a table above, we would like to propose the frequency range around 3.5GHz / 4.5GHz and around 26GHz / 28GHz for the priority bands as the first phase of NR standardization. Detailed analysis of those two bands can be carried out based on the results from the stage above taking into account the technical feasibility combined with further detail commercialization plan.

Technical feasibility - Around 3.5GHz / 4.5GHz

As mentioned in [5], RAN4 should consider the trade-off between benefits of a large bandwidth and an easier or less complex implementation which are summarized in Table 3.

Table 3: Analysis of possible options in 3.5GHz and 4.5GHz band

| Opt | Possible Bands | Relevant region (A:all, P:partial) | Earliest time for deployment | Fractional bandwidth | Remarks (UE aspect) |
| --- | --- | --- | --- | --- | --- |
| 1 | 3.4-3.8GHz | KOR(A), EU(A), JPN(P), CHN(P) | 2018(KOR) | 11.1% | - Highly possible at the present |
| 2 | 3.3-3.8GHz | KOR(A), EU(A), CHN(A), JPN(P) | 2018(KOR) | 14% | - Maybe possible (not now) - Can be considered in phase 2 |
| 3 | 3.3-4.2GHz | KOR(A), EU(A), CHN(P), JPN(A) | 2018(KOR) | 21% | - Impossible |
| 4 | 4.4-4.99GHz | CHN(A), JPN(A) | 2020(CHN, JPN) | 12.8% | - Maybe possible (not now) - Can be considered in phase 2 |

If the option 3 will be adopted, the devices would be required to have at least two filters in order to implement the whole band even for the country whose available spectrum is only a portion of that. Furthermore, concern on PA feasibility to support option3 as a single band is also arisen during the discussion, for which further study is needed.  For option 2 and 4, they may be possible in the near future, however, considering the earliest time plan for each option, they could be taken into account in phase 2 with regards to the UE availability

Accordingly, RAN4 should adopt option 1 as the WI for NR bands of Rel-15.

Proposal 1: For below 6GHz, option 1 (3.4-3.8GHz) should be adopted for NR bands of Rel-15 WI.

Technical feasibility - Around 26GHz / 28GHz

Most of plans in the bands above 6GHz are concentrated on 26GHz and 28GHz for their first stage of NR deployment. In this document, we would like to examine the optimum ranges for both regional plans and technical feasibilities and performance as summarized in Table 4.

Table 4: Analysis of possible options in 26GHz and 28GHz band

| Opt | Possible Bands (GHz) | Relevant region (A:all, P:partial) | Earliest time for deployment | Fractional bandwidth | Remarks (UE aspect) |
| --- | --- | --- | --- | --- | --- |
| 1 | 24.25-27.5 | EU(A), KOR(P) | 2020(EU) | 12.5% | - Maybe possible (not now) - Can be considered in phase 2 |
| 2 | 26.5-29.5 | KOR(A), US(A), JPN(A), EU(P) | 2018(US, KOR) | 10.7% | - Highly possible at the present |
| 3 | 27.5-28.35 | US(A), KOR(P), JPN(P) | 2018(US) | 3% | - Highly possible at the present |
| 4 | 24.25-29.5 | EU(A), JPN(A), KOR(A), US(A) | 2018(US, KOR) | 19.5% | - Impossible |

Option 1 is to be determined by 2020 in EU and no trail plan announced currently, which could be studied in phase 2 in order to discuss the technical feasibility together with the deployment possibility.

Consequently, based on the analysis above, we can have two possible options, i.e., 26.5-29.5GHz and 27.5-28.35GHz, so that two scenarios can be considered for Rel-15 WI as below.

- Scenario 1: to take option 2 for a WI in the frequency range of 28GHz as one band

- Scenario 2: to take both option 2 and 3 for each WI in the frequency range of 28GHz as duplicate bands

Scenario 2 may cause to be cumbersome in terms of efficiency and complexity of UE, and moreover Figure 3 and other results from multiple industries show that there would be no difference between both option 2 and 3 with the same RF spec for FCC requirements. Therefore, RAN4 should take option 2 for a WI of Rel-15 in the frequency range of 28GHz as one band.

Proposal 2: For above 6GHz, option 2 (26.5-29.5GHz) should be considered for NR bands of Rel-15 WI.

- Conclusion

In this contribution, we provide our understanding and priority bands for the NR standardization of Rel-15 based on the regional and time plan, and technical feasibility to choose NR band for Rel-15 WI. The proposals are as below.

Proposal 1: For below 6GHz, option 1 (3.4-3.8GHz) should be adopted for NR bands of Rel-15 WI.

Proposal 2: For above 6GHz, option 2 (26.5-29.5GHz) should be considered for NR bands of Rel-15 WI.

- Reference

[1] R4-1610992, WF on NR Bands

[2] R4-1701069, Proposal on NR band from Rel-15 onwards, Samsung

[3] CEPT Workshop on 5G Mobile Communications, http://www.cept.org/ecc/cept-workshop-on-5g/

[4] K-ICT Spectrum plan, the Ministry of Science, ICT and Future Planning (MSIP)

[5] R4-1700124, Potential harmonized bands for NR, NTT DOCOMO INC.