3GPP TSG RAN4 Meeting #82	R4-1701496

Athens, Greece, 13 – 17 February, 2017

Title: 	MSD for 2UL/2DL CA

Source: 	Huawei, HiSilicon

Agenda item:	7.6.2

Document for:	Other (Approval)

# 1	Introduction

This contribution discusses MSD requirements for 2+66 and 5+66 UL CA.

# 2	Discussion

2+66:

This CA combination has IMD3 and IMD5 falling on top of own B2 and B66 RX, respectively. The testpoints were approved in [1]:

| DL CA | UL CA | IMD |  | UL Fc (MHz) | DL Fc (MHz) | UL BW (MHz) | UL RB # | DL Band | DL Fc (MHz) | IMD center [MHz] | MSD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B2+B66 | B2 | IMD3 | 2*fB2-fB66 | 1855 | 1935 | 5 | 25 | B2 | 1935.0 | 1935.0 | TBD |
|  | B66 |  |  | 1775 | 2175 | 5 | 25 |  |  |  | N/A |
| B2+B66 | B2 | IMD5 | 3*fB2-2*fB66 | 1883.3 | 1963.3 | 5 | 25 | B66 | 2150.0 | 2149.9 | N/A |
|  | B66 |  |  | 1750 | 2150 | 5 | 25 |  |  |  | TBD |

We use the conventional common diplexer based architecture in our analysis with the following component assumptions. B2+B66 is implemented using a quadplexer in diplexer H-branch. Diplexer was not assumed in diversity path.

| Component | IP3 (dBm) | IP5 (dBm) |
| --- | --- | --- |
| Antenna switch | 68 | 53 |  |
| Diplexer | 86 | 53 |  |
| Duplexer | 74 | 53 |  |
| LNA | -6 | -10 |  |

B2 IMD:

B2 PA forward mixing IMD3 at PA output with 60dB isolation    	-22.5

B2 PA forward mixing RFIC Pin2Pin IMD3 at PA output				 -27

B66 IMD:

B2 PA forward mixing IMD5 at PA output with 60dB isolation    	-45.5

B2 PA forward mixing RFIC Pin2Pin IMD5 at PA output				 -54.5

B2+B66 quadplexer TX/RX/x-band isolation							  52

B2 quadplexer TX attenuation at B2 RX (forward)			              46

B2 quadplexer TX attenuation at B66 RX (forward)			              44

B66 quadplexer TX attenuation at B2 RX (forward)			              46

B66 quadplexer TX attenuation at B66 RX (forward)			              45

Antenna isolation 														  10

PCB isolation PA-PA 												   	  60

PCB isolation PAout-RX													  65

Diplexer isolation															  15

B2/B66 FE loss (diplexer+switch+duplexer)						         4.5

RFIC Pout																	    0

RFIC TX pin-pin isolation											     	  40

Based on these parameters we get the following MSD’s. PA forward mixing results are based on measurements with commercial PA’s.  CF for B2 is 1.65dB and CF for B66 is 2.5dB.

B2 IMD3 dominant factors:

B66 IMD5 dominant factors:

The MSD is as follows.

| DL CA | UL CA | IMD |  | UL Fc (MHz) | DL Fc (MHz) | UL BW (MHz) | UL RB # | DL Band | DL Fc (MHz) | IMD center [MHz] | MSD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B2+B66 | B2 | IMD3 | 2*fB2-fB66 | 1855 | 1935 | 5 | 25 | B2 | 1935.0 | 1935.0 | 22.14 |
|  | B66 |  |  | 1775 | 2175 | 5 | 25 |  |  |  | N/A |
| B2+B66 | B2 | IMD5 | 3*fB2-2*fB66 | 1883.3 | 1963.3 | 5 | 25 | B66 | 2150.0 | 2149.9 | N/A |
|  | B66 |  |  | 1750 | 2150 | 5 | 25 |  |  |  | 2.56 |

5+66:

This CA combination has IMD2 and IMD5 falling on top of own B5 RX. The testpoint was suggested in [1]:

| CA | UL CA | IMD |  | UL Fc [MHz] | DL Fc [MHz] | UL BW [MHz] | UL [RB] | DL Band | 3rd DL Fc [MHz] | IMD center [MHz] | DL BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B5+B66 | B5 | IMD2 | fB66-fB5 | 838 | 883 | 5 | 25 | B5 | 883.0 | 883.0 | 5 |
|  | B66 |  |  | 1721 | 2121 | 5 | 25 |  |  |  | 5 |
| B5+B66 | B5 | IMD5 | 2*fB66-3*fB5 | 846.5 | 891.5 | 5 | 25 | B5 | 891.5 | 891.5 | 5 |
|  | B66 |  |  | 1715.5 | 2115.5 | 5 | 25 |  |  |  | 5 |

We use the conventional common diplexer based architecture in our analysis with the following component assumptions. Diplexer was not assumed in diversity path.

| Component | IP2 (dBm) | IP5 (dBm) |
| --- | --- | --- |
| Antenna switch | 112 | 53 |  |
| Diplexer | 115 | 53 |  |
| Duplexer | 100 | 53 |  |
| LNA | 5 | -10 |  |

B5 PA forward mixing IMD2 at PA output with 60dB isolation		-10

B5 PA forward mixing RFIC Pin2Pin IMD2 at PA output				 -14.5

B5 PA forward mixing IMD5 at PA output with 60dB isolation		-39.5

B5 PA forward mixing RFIC Pin2Pin IMD5 at PA output				 -48.5

B5 duplexer TX rejection at B5 RX (forward)						        44

B66 duplexer TX rejection at B5 RB (forward)						  30

B5 TX isolation															  53

B5 RX isolation															  50

Antenna isolation 														  10

PCB isolation PA-PA 												   	  60

PCB isolation PAout-RX													  65

Diplexer isolation															  15

B5 FE loss (diplexer+switch+duplexer)						    	             4.5

RFIC Pout																	    0

RFIC TX pin-pin isolation											     	  40

Based on these parameters we get the following MSD’s. PA forward mixing results are based on measurements with commercial PA’s. CF=0.9dB for IMD2 and 2.5dB for IMD5 in this analysis.

IMD2 dominant factors:

IMD5 dominant factors:

The MSD is as follows.

| DL CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B5+B66 | B5 | IMD2 | fB66 -fB5 | 838 | 5 | 25 | 883 | 5 | 0.9 | 33.68 |
|  | B66 |  |  | 1721 | 5 | 25 |  |  |  |  |
|  | B5 | IMD5 | 2*fB66 -3*fB5 | 846.5 | 5 | 25 | 891.5 | 5 | 1 | 4.00 |
|  | B66 |  |  | 1715.5 | 5 | 25 |  |  |  |  |

In previous meeting we were asked to evaluate options to reduce the IMD2 MSD level. It is highly dominated by the B5 PA FW mixing. The mitigation method could be to add filtering at the PA input to mitigate IMD2 at the PA output. This filtering would basically need to be inside the PA component or inside the module; otherwise the power form B66 PA via PCB would couple to PA input without additional filtering impact.

# 3	Conclusion

MSD analysis for CA_2A-66A was provided with the following results.

| DL CA | UL CA | IMD |  | UL Fc (MHz) | DL Fc (MHz) | UL BW (MHz) | UL RB # | DL Band | DL Fc (MHz) | IMD center [MHz] | MSD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B2+B66 | B2 | IMD3 | 2*fB2-fB66 | 1855 | 1935 | 5 | 25 | B2 | 1935.0 | 1935.0 | 22.14 |
|  | B66 |  |  | 1775 | 2175 | 5 | 25 |  |  |  | N/A |
| B2+B66 | B2 | IMD5 | 3*fB2-2*fB66 | 1883.3 | 1963.3 | 5 | 25 | B66 | 2150.0 | 2149.9 | N/A |
|  | B66 |  |  | 1750 | 2150 | 5 | 25 |  |  |  | 2.56 |

MSD analysis for CA_5A-66A was provided with the following results.

| DL CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B5+B66 | B5 | IMD2 | fB66 -fB5 | 838 | 5 | 25 | 883 | 5 | 0.9 | 33.68 |
|  | B66 |  |  | 1721 | 5 | 25 |  |  |  |  |
|  | B5 | IMD5 | 2*fB66 -3*fB5 | 846.5 | 5 | 25 | 891.5 | 5 | 1 | 4.00 |
|  | B66 |  |  | 1715.5 | 5 | 25 |  |  |  |  |

# 4	References

[1] R4-1610770, “MSD for 2+66 2UL CA”, Huawei, Hisilicon, RAN4#81