3GPP TSG-RAN4#82	R4-1702046

Athens, Greece, 13 – 17 Feb, 2017

# Source:	Intel Corporation

# Title:	TP for TR38.803: Transceiver architecture for mmWave

# Agenda Item:	10.4.2.1

# Document for:	Approval

# 1	Introduction

This contribution is a TP for TR38.803 for UE transceiver architecture for mmWave based on the discussions in previous meetings for UE reference architectures regarding contributions [2] – [5].

This TP is based on aspects taken from [1] for UE reference architecture for mmWave.

# 2	Discussion

This text proposal provides the transceiver architectures including a UE reference architecture for NR for mmWave with a more detailed analysis of the key components in the transmitter and receiver chain, as well as a comparison between digital, analogue and hybrid beamforming.

# 3	Reference

[1] R4-1700604, UE reference architecture for mmWave, Intel

[2] R4-1701333, On UE architecture assumptions, Huawei

[3] R4-1701449, 5G UE reference architecture, LG

[4] R4-165771, UE RF architecture for mmWave, Qualcomm

[5] R4-16700007, UE reference architecture for NR, Sony, Ericsson

# 4	Text proposal for TR 38.803

<<start of change 1>>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP RP-160671: "New SID Proposal: Study on New Radio Access Technology".

[2]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[3]	Recommendation ITU-R SM.328-10, “Spectra and Bandwidth of Emissions”.

[4]	Recommendation ITU-R SM.329-12, “Unwanted emissions in the spurious domain”

[5]	“International Telecommunications Union Radio Regulations”, Edition 2016, Volume 1 – Articles, ITU.

[6]	“Title 47 of the Code of Federal Regulations (CFR)”, Federal Communications Commission.

[7]	Recommendation ITU-R SM.1539-1, “Variation of the boundary between the out-of-band and spurious domains required for the application of Recommendations ITU-R SM.1541 and ITU-R SM.329”.

[8]	Recommendation ITU-R SM.1540, “Unwanted emissions in the out-of-band domain falling into adjacent allocated bands”.

[9]	Recommendation ITU-R SM.1541-6, “Unwanted emissions in the out-of-band domain”.

[10]	Recommendation ITU-R M.2012, “Detailed specifications of the terrestrial radio interfaces of International Mobile Telecommunications-Advanced (IMT-Advanced)”.

[11]	Recommendation ITU-R M.2070, “Generic unwanted emission characteristics of base stations using the terrestrial radio interfaces of IMT-Advanced”.

[12]	Recommendation ITU-R M.2071, “Generic unwanted emission characteristics of mobile stations using the terrestrial radio interfaces of IMT-Advanced”.

[13]	Report ITU-R M.2292, “Characteristics of terrestrial IMT-Advanced systems for frequency sharing/interference analyses”.

[14]	ECC Recommendation (02)05, “Unwanted Emissions”, October 2002, amended March 2012.

[15]	CEPT/ERC/RECOMMENDATION 74-01, “Unwanted Emissions in the Spurious Domain”, Cardiff  2011.

[16]	ETSI EN 301 908, “IMT cellular networks; Harmonized EN covering the essential requirements of article 3.2 of the R&TTE Directive;” (22 parts).

[17]	FCC Report and Order and Further Notice of Proposed Rulemaking, “Use of Spectrum Bands Above 24 GHz For Mobile Radio Services…”, FCC 16-89, July 14, 2016.

[18]	3GPP TR 25.942, Technical Report,”3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Radio Frequency (RF) system scenarios”

[19]	3GPP TR 36.942, Technical Report, ”3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) system scenarios”

[20]	3GPP TR 37.900, Technical Report, “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Radio Frequency (RF) requirements for Multicarrier and Multiple Radio Access Technology (Multi-RAT) Base Station (BS)”

[21]           	TR 37.842, Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)

[22]          	R4-166432, AAS and NR BS requirements, Huawei

[23]	R4-164168, "On PA models", Ericsson

[24]	R4-165901, "Further elaboration on PA models for NR", Ericsson.

[25]	R4-163314, “Realistic power amplifier model for the New Radio evaluation”, Nokia

[26]	R4-167263, “PA model using a Memory Polynomial”, Intel

[27]	ECC PT1 (16)083_A31, Liaison Statement on IMT 2020/ 5G spectrum in Europe

[28]	ECC decision (11)06, Harmonised frequency arrangements for mobile/fixed communications networks (MFCN) operating in the bands 3400-3600 MHz and 3600-3800 MHz

[29]	“Flex5Gware Project, “Deliverable 2.1: Requirements and concepts for the analogue HW in 5G mobile systems”, Dec. 2015.

<<end of change 1>>

<<start of change 2>>

## 6	RF feasibility

Editor’s note: Impacts of RF feasibility on RF requirements are captured in terms of frequencies, physical layer specifications, RF component, and channel bandwidth etc.

### 6.1	Common issues for UE and BS

Editor’s note: Common RF issues for both UE and BS RF requirement feasibility are captured

### 6.2	UE requirements

Editor’s note: UE RF issues and its impacts on specifications are captured such that implementation impact of integrated RF, wider channel bandwidth, wave form, RF component characteristics due to higher frequencies, deployment scenarios(necessity of narrow blocking etc.) on the existing RF requirements.

### 6.2.1	General

#### 6.2.1. X	Transceiver architecture at mmWave

For high frequency broadband systems - especially at mmWave range-, the nonlinearity of analogue components used in RF front ends gives increased challenges in the implementation. The principal impairments at RF front end of the transceiver architecture and the key requirements need to be analysed for achieving high performance, which will be outlined in this subsection.

##### 6.2.1. X.1	Transceiver architectures

Wireless communication requires several stages of signal processing in digital and analogue domain. Once the signal is converted from digital to analogue, it is expose to several analogue impairment sources. Therefore, transceiver architectures selection can influence differently to the performance of the system. The transceiver architecture can be either direct (Homodyne) conversion, as shown in Figure 6.2.1.X.1.1-1 or IF (Heterodyne) conversion, as shown in 6.2.1.X.1.2-1. [29]

###### 6.2.1. X.1.1	Homodyne transceiver architecture

In a direct conversion architecture, the I and Q modulation is carried out in the analogue domain, having as a drawback the deficiency of the modulator; as for example the IQ-imbalance, DC offset and degradation of transmitted signal resulting in degradation of EVM. However, the major advantage of direct conversion architecture is that the image problem is avoided because the IF is zero, and therefore the requirement of a single LO realization is fulfilled. Another disadvantage of this architecture is that there are no other filters than RF-band selection filter, which means that the creation of a DC signal can contribute directly as interference in the band of interest.

Figure 6.2.1.X.1.1-1: Simplified architecture Direct Conversion

###### 6.2.1. X.1.2	Heterodyne transceiver architecture

In the heterodyne architecture, the IF-sampling architecture has the modulation in the digital domain, thus becomes immune to the problems related to the I and Q modulator. Nevertheless, it requires a wide bandwidth DAC, which is more sensible to errors and require more stages of LOs and filters to eliminate the IF spectral images. The heterodyne architecture has an advantage in the phase noise requirements compared to homodyne architecture, since the phase noise is logarithmically proportional to the LO frequency having two sources of LO reduce the phase noise of the system.

The UE reference architecture in Figure 6.2.1.X.1.2-1 with IF conversion is proposed in mmWave, this does not preclude from using direct conversion at a more advance stage in NR.

Figure 6.2.1.X.1.2-1: Simplified architecture one stage IF conversion

##### 6.2.1. X.2	UE Reference architecture

The RF Frontend consist of all components between the antenna and the digital baseband system of a transceiver, which is divided mainly in mixers, RF Phase shifter and power amplifier. The demand of high data rate increases the research on conflicting requirements. Furthermore, high bandwidth requires operation in high frequency RF band and this creates big challenges to achieve high efficiency and good performance. The balance that must be found in size, capacity and cost should be taken into account in NR.

Figure 6.2.1.X.2-1: UE reference architecture

Besides the RF front end, it must be considered that mixers, oscillators, quadrature modulators are also sources of impairments, due to low port isolation, generating imperfections in the output signal.

Mixing the RF signal with the LO realizes frequency conversion within the RF transceivers. The phase of the LO can be non-stationary as a free running oscillator or time varying modelled as a stationary process of a PLL synthesizer. Ideally, the outcome of an LO is a single tone in the frequency domain. However, in reality, the outcome is a modulated tone with a phase shift. Phase noise – which increases with LO frequency - causes significant degradation in the performance and reduces the effective SNR at the receiver, limiting the BER and data rate. The phase noise gives several constraints on the design of oscillators. These are further constraints besides the limitations at the RF frond end – explained in the next subsections - that make the choice of a UE transceiver architecture important.

###### 6.2.1. X.2.1	TX Chain

This subsection discusses the most relevant components in the RF front of end in the transmitter chain, which should be considered as key building blocks for further study in NR. The transmitter is less complex than the receiver, thus a less variety of implementations can be studied.

Power Amplifier

For transmitting large amount of data modulated in complex waveforms, a high linear amplification stage is required to minimize distortion. The drawback of high linear PA is the lack of efficiency. At higher frequency the efficiency and output power decreases. This means that for transmitting a certain amount of signal power, more power is required by the PA compared to the systems operating at carrier frequency under 6 GHz. The major imperfection of the PA is its nonlinear response and memory effects. The nonlinearity problem of the PA becomes evident as requirement of larger bandwidth and higher order modulation schemes are needed to achieve high data rate.

Phase Shifter

Phase shifters are source of imbalance and they can be implemented by power divisor and all – pass filters producing 90 degree phase shift. The accuracy depends on the components so that the 90 degree phase shift is not really exact. There are different phased-array architectures (i.e., RF phase shifters, LO phase shifters) which differ in functionality of phase shifting and signal combining and have as critical factors power consumption, losses and bandwidth. Possible examples of phase shifters (i.e., RF and LO phase shifters) can be seen in Figure 6.2.1.X.2.1-1.

Figure 6.2.1.X.2.1-1: RF and LO Phase Shifters

LO phase shifters - compared to RF phase shifter - has a LO path, which is less sensitive to bandwidth because single tone LO is usually delivered. Bandwidth only translates to conversion gain difference in the mixer while using different LO frequencies and can be compensated pushing up the gain in the LO driver. The penalty for this is the higher power consumption, LO phase shifter has as drawback the high number of mixers. Every element needs a mixer to down convert before phase shifting.

In the case of the RF phase shifter, it needs an RF gain stage prior signal combining for noise reasons due to signal insertion loss. Additionally, it should meet tougher linearity specifications, which increases the losses of RF phase shifter. A way to cancel out the losses is to use an amplifier stage. The bandwidth of RF phase shifter is also challenging and demand more power consumption. The advantage is that requires one single mixer. An example of both RF and LO phase shifters can be seen in Figure 6.2.1.X.2.1-1.

###### 6.2.1. X.2.2	RX Chain

The Rx chain is composed by switch, LNA and phase shifter. This subsection includes the most relevant components in the receiver chain, which should be considered as key building blocks for further study in NR.

Low Noise Amplifier

One of the most important blocks in the receiver is the LNA. It is needed at the input of the receiver to amplify the received signal and suppress the noise contribution of the downconverter. The first stage is the RF block, which begins with the antenna, followed by the switch, the LNA and ends with the phase shifter as explained in the NF will be mainly determined by the LNA. Therefore, the design for the LNA should be optimize for high gain and low NF.

Phase Shifter

In receiver element the phase shifted signal are combined before arriving at the downconverter. The RF phase shifter kind can phase out the interference and then relax power consumption on the blocks down the chain. The advantage of RF phase shifter over the LO phase shifting is that the output signal after RF combiner has a high pattern directivity and can reject an interference before receiving units, maximizing the value of the phased array as a spatial filter. Another advantage is the elimination of LO distribution network resulting in a simple system architecture.

Filter

With the increasing complexity of the transceiver at mmWave, the filter has to satisfy more constraints at the same time, which puts a number of requirements on the filter such as resonance frequency, coupling factor, quality factor, temperature sensitivity, etc. Besides the technical limitation of filter at high frequencies, filters at the RF front end have the disadvantage of including a considerable extra loss to the RF chain. At mmWave is much more difficult than at frequencies below 6 GHz to get a high output power and adding a filter will implies extra loss, which is not easy to compensate. For these reasons FDD will probably remain a main duplex method for lower frequencies and TDD for mmWave range.

###### 6.2.1. X.2.3	Beamforming

Due to high path loss encounter at mmWave, beamforming (BF) assumes an important role to stablish and maintain a robust communication link. There are several discussions about digital and analogue BF implementation. Digital beamforming provides a high flexibility in shaping beams at cost of increasing complexity. The high cost, power consumption and complexity of mmWave mixed signal components constrains the use of fully-digital BF architectures at the UE unlike simpler, lower antenna count more conventionally used in sub-6 GHz systems.  On the other hand, analogue BF has fewer RF chain than antenna elements, which is more cost effective with lower power consumption and therefore, more suitable for the UE at mmWave. The disadvantage of this approach is having less flexibility compared to digital BF, since it can only handle one beam at a time.

The hybrid BF architecture is another approach that consists of reducing the hardware complexity with limited number of RF chains but using digital precoding in order to have similar performance to the digital BF, which enables multi-stream communication and maximizes the sum rate with minimum interference.

<<end of change 2>>