3GPP TSG-RAN WG4 Meeting #82		R4-1702158

Athens, Greece, 13- 17 February 2017

Agenda item:	7.31.2.1

Source:	CATT

Title:	PDSCH demodulation performance of Enhanced CRS-IM

Document for:	Discussion

Introduction

In RAN #73 meeting, the “LTE Enhanced CRS and SU-MIMO Interference Mitigation Performance Requirements” WI was initiated [1]. In the previous RAN4#81 meeting, further agreements on the target scenarios and reference receivers for the Stage 1 investigations were provided in [2]. And it is proposed to provide simulation results to identify target scenarios for requirements definition and confirm reference receiver feasibility. In this contribution, we provide the simulation results based on the agreed simulation assumptions [3] for PDSCH on enhanced CRS-IM. The detailed simulation assumptions are attached in the Annex.

Simulation results

In the way forward [3], the agreed PDSCH test cases are proposed as shown in Table 1.

Table 1.  PDSCH test cases

| Test | TM | CRS pattern | Number of UE RX chains | Number of CRS APs |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Serv. cell | Interf. cell |
| Baseline scenarios |  |  |  |  |  |
| 1 | TM4 | Non Colliding | 2 | 4 | 4 |
| 2 | TM4 | Non Colliding | 4 | 2 | 2 |
| 3 | TM9 | Non Colliding | 4 | 2 | 2 |
| 4 | TM4 | Non Colliding | 4 | 4 | 4 |
| Additional scenarios for analysis |  |  |  |  |  |
| 5 | TM4 | Colliding | 2 | 4 | 4 |
| 6 | TM4 | Colliding | 2 | 4 | 2 |
| 7 | TM4 | Colliding | 2 | 2 | 4 |

Rank + MCS:

| Test cases | TM4 |  | TM9 |
| --- | --- | --- | --- |
|  | 2 CRS APs | 4 CRS APs | 2 CRS APs |
| Rank 1 + 16QAM | R.10-3 | R.14 | R.51-1 |
| Rank 1 + 64QAM | R.35 | R.36 | TBS SFs 1-4, 6-9: 18336 TBS SF 0: 14688 |

In this simulation, the following reference receivers are used for the analysis, the reduced complexity CRS-IM processing receiver may be further evaluated. And because of the marginal gain from the second interference cell cancellation, only 1 cell CRS-IM is evaluated.

- Receiver #1: LMMSE-IRC (baseline)

- Receiver #2: LMMSE-IRC + 1 Cell CRS-IM  (Full complexity two or four ports CRS-IM processing )

Evaluations on Non colliding cases

Test case1: TM4  4 CRS APs + 2 RX chains

Figure 1. throughput performance of TM4  4 CRS APs + 2 RX chains scenario

Test case2: TM4  2 CRS APs + 4 RX chains

Figure 2. throughput performance of TM4  2 CRS APs + 4 RX chains scenario

Test case3: TM9  2 CRS APs + 4 RX chains

Figure 3. throughput performance of TM9  2 CRS APs + 4 RX chains scenario

Test case4: TM4  4 CRS APs + 4 RX chains

Figure 4. throughput performance of TM4  4 CRS APs + 4 RX chains scenario

From the above link-level simulation results of the non colliding scenarios, the performance gains of the LMMSE-IRC+CRS-IM receiver vs. the LMMSE-IRC receiver are shown in table 2.

Table 2. the performance gains of the SNRs to achieve the 70% of the maximum throughput

| Test Case (non colliding) | LMMSE-IRC(dB) | LMMSE-IRC+CRS-IM(dB) | Gains of CRS-IM(dB) |
| --- | --- | --- | --- |
| PDSCH Test1-16QAM | 9.77 | 7.57 | 2.2 |
| PDSCH Test1-64QAM | 14.44 | 12.66 | 1.78 |
| PDSCH Test2-16QAM | 9.73 | 6.00 | 3.73 |
| PDSCH Test2-64QAM | 12.73 | 9.70 | 3.03 |
| PDSCH Test3-16QAM | 7.29 | 5.32 | 1.97 |
| PDSCH Test3-64QAM | 10.78 | 9.62 | 1.16 |
| PDSCH Test4-16QAM | 7.48 | 4.88 | 2.6 |
| PDSCH Test4-64QAM | 11.77 | 9.69 | 2.08 |

Observation 1:

CRS-IM processing can achieve testable performance gain (about 2dB) over baseline receiver under various configurations of non colliding scenarios.

- The largest performance gain ( >3dB ) is observed from TM4  with the configuration of 2CRS APs and 4RX antennas

- For the evaluated scenario TM9 Rank 1 + 64QAM case, only about 1dB performance gain is observed from simulation.

Evaluations on Colliding cases

Test case5: TM4  4 CRS APs + 2 RX chains

Figure 5. throughput performance of TM4  4 CRS APs + 2 RX chains scenario

Test case6: TM4    4/2 CRS APs + 2 RX chains

Figure 6. throughput performance of TM4  4/2 CRS APs + 2 RX chains scenario

Test case7: TM4   2/4 CRS APs + 2 RX chains

From the above simulation results of the colliding scenarios, the performance gain of the LMMSE-IRC+CRS-IM receiver to the LMMSE-IRC receiver is shown in table 3.

Table 3. the performance gain of the SNRs to achieve the 70% of the maximum throughput

| Test Case (Colliding) | LMMSE-IRC(dB) | LMMSE-IRC+CRS-IM(dB) | Gains of CRS-IM (dB) |
| --- | --- | --- | --- |
| PDSCH Test5-16QAM | 9.52 | 8.16 | 1.36 |
| PDSCH Test5-64QAM | 14.39 | 12.89 | 1.5 |
| PDSCH Test6-16QAM | 8.67 | 6.75 | 1.92 |
| PDSCH Test6-64QAM | 13.57 | 11.41 | 2.16 |
| PDSCH Test7-16QAM | 11.50 | 11.35 | 0.15 |
| PDSCH Test7-64QAM | 14.65 | 13.60 | 1.05 |

Observation 2:

CRS-IM processing can achieve testable performance gain (around 1dB) over baseline receiver of colliding scenarios.

Conclusions

In this contribution, we provide our simulation results for PDSCH demodulation on enhanced CRS-IM. And according to the simulation results, the observations are given as below:

Observation 1:

CRS-IM processing can achieve testable performance gain (about 2dB) over baseline receiver under various configurations of non colliding scenarios.

- The largest performance gain ( >3dB ) is observed from TM4  with the configuration of 2CRS APs and 4RX antennas

- For the evaluated scenario TM9 Rank 1 + 64QAM case, only 1dB performance gain is observed from simulation.

Observation 2:

CRS-IM processing can achieve testable performance gain (around 1dB) over baseline receiver of colliding scenarios.

# References

- RP-161877 “New Work Item Proposal: Enhanced CRS and SU-MIMO Interference Mitigation Performance Requirements for LTE”, Intel Corporation, Huawei, LGE, RAN #73, September 2016

- R4-168638 “WF on Enhanced CRS-IM Performance Requirements”, Intel Corporation, LGE, Huawei, HiSilicon, Ericsson, RAN4 #81, November 2016

- R4-168642 “WF on simulation assumptions for Enhanced CRS-IM initial studies”, Intel Corporation, Huawei, HiSilicon, LGE, Ericsson, RAN4 #81, November 2016

- Annex - Simulation assumptions

Table 2 includes a list of the test cases to be used for the initial investigations of the CRS-IM enhancements for PDSCH. The PDSCH simulation assumptions are provided in Table 3.

Table 2. Proposed PDSCH test cases

| Test | TM | CRS pattern | Number of UE RX chains | Number of CRS APs |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Serv. cell | Interf. cell |
| Baseline scenarios |  |  |  |  |  |
| 1 | TM4 | Non Colliding | 2 | 4 | 4 |
| 2 | TM4 | Non Colliding | 4 | 2 | 2 |
| 3 | TM9 | Non Colliding | 4 | 2 | 2 |
| 4 | TM4 | Non Colliding | 4 | 4 | 4 |
| Additional scenarios for analysis |  |  |  |  |  |
| 5 | TM4 | Colliding | 2 | 4 | 4 |
| 6 | TM4 | Colliding | 2 | 4 | 2 |
| 7 | TM4 | Colliding | 2 | 2 | 4 |

Table 3. PDSCH simulation assumptions

| Parameter | Value |
| --- | --- |
| Cell ID pattern | Non-colliding CRS: 0/1/6 Colliding CRS: 0/6/1 |
| Interference power profile | INR1 = 10.45 dB INR2 = 4.6 dB |
| Time offset between cells | Cell 1: 3 µs Cell 2: -1 µs |
| Frequency offset between cells | Cell 1: 300 Hz Cell 2: -100 Hz |
| Channel model | EVA-5Hz for all links |
| Number of control OFDM symbols | 2 |
| HARQ modelling | Maximum 4 HARQ transmissions |
| Transmission parameters | PDSCH is scheduled in SFs 0-4, 6-9 (i.e. except 5) Allocation: - TM4: 50 PRB resource allocation for all SFs - TM9: 50 PRB resource allocation for all SFs 1-4,6-9; 41 PRB resource allocation for all SF 0 PMI - TM4: Reporting mode PUSCH 3-1 - TM9: Random PMI with 1 PRG / 1 TTI granularity Rank + MCS: TM4; TM9. 2 CRS APs; 4 CRS APs; 2 CRS APs. Rank 1 + 16QAM; R.10-3; R.14; R.51-1. Rank 1 + 64QAM; R.35; R.36; TBS SFs 1-4, 6-9: 18336 TBS SF 0: 14688. |
| Interference signal transmission parameters | Rel-13 CRS-IM interference model (B.5.4) Interference loading: 20% 80%/20% rank 1/2 probability |
| Receivers | Baseline: LMMSE-IRC Enhanced reference receiver structures: LMMSE-IRC + 1 cell CRS-IM Enhanced reference receiver for 4 CRS APs - Receiver #1: Full complexity four ports CRS-IM processing - Receiver #2: Reduced complexity CRS-IM processing |