3GPP TSG-RAN WG4 Meeting #82bis 	R4-1702858

# Spokane, US, 3 – 7 April, 2017

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	First simulation results for eLAA PUSCH

Agenda Item:	7.14.7

Document for:	Discussion

# Introduction

The scope of eLAA performance work according to the latest WID is to Specify the necessary UE and base station performance requirements to support UL carrier aggregation for an LAA SCell using Frame Structure type 3 [1]. In the last RAN4 meeting in Athens, a way forward on eLAA demodulation was agreed in [2]. Furthermore, a set of initial simulation assumptions was agreed in [3]. In this contribution we provide simulation results based on the agreed simulation assumptions.

# Simulation assumptions

Simulation assumptions as agreed in [3] are listed in Table 1.

Table 1: Parameter setup for PUSCH Performance requirements

| Parameter | Value |
| --- | --- |
| Carrier frequency | 5 GHz |
| System bandwidth for LAA SCell | 20 MHz |
| Channel model | EPA 5Hz |
| Antenna setup | 1Tx, 2Rx and 4Rx LOW |
| Number of interlaces | 2 |
| Number of PRBs per interlace | 10 |
| PUSCH frequency hopping | No |
| Starting PUSCH symbol configuration | 01 |
| Ending symbol configuration | Up to OFDM symbol 13 |
| MCS | QPSK 1/3, 16QAM ¾, FFS: 64QAM 5/6 |
| CP type | Normal |
| Receiver noise | AWGN |
| Number of user | 1 |
| Resource allocation | continuous |
| Frequency error | 0 |
| Time error | 0 |
| Interference | no |
| Reference Receiver | MRC |
| Performance Metric | SNR values [dB] for which 70% of maximum throughput is achieved |
| Max number of HARQ Transmissions | 4 |
| LBT model | No |

# Simulation results

Simulation results for eLAA PUSCH with 1 and 2 contiguous interlaces are shown in tables 2 and 3.

Table 2: eLAA PUSCH simulation results for 1 interlace allocation.

| 1 interlace, EPA5 |  |  |  |
| --- | --- | --- | --- |
| 70 % TP | 2 Rx | QPSK (MCS 5) | -2.37 |
|  |  | 16QAM (MCS 19) | 10.18 |
|  |  | 64QAM (MCS 27) | 16.73 |
|  | 4 Rx | QPSK (MCS 5) | -5.52 |
|  |  | 16QAM (MCS 19) | 6.08 |
|  |  | 64QAM (MCS 27) | 12.46 |

Table 3: eLAA PUSCH simulation results for 2 interlace allocation (contiguous interlaces).

| 2 interlaces, EPA5 |  |  |  |
| --- | --- | --- | --- |
| 70 % TP | 2 Rx | QPSK (MCS 5) | -2.72 |
|  |  | 16QAM (MCS 19) | 10.15 |
|  |  | 64QAM (MCS 27) | 16.81 |
|  | 4 Rx | QPSK (MCS 5) | -5.85 |
|  |  | 16QAM (MCS 19) | 6.02 |
|  |  | 64QAM (MCS 27) | 12.53 |

As can be seen from the 70 % throughput SNR results, differences for 1 interlace and 2 interlace allocations differ by less than 0.5 dB.

# Summary

In this contribution we have provided first simulation results for eLAA PUSCH based on the simulation assumptions agreed in the Athens meeting.

# References

[1] RP-161832, Work Item on enhanced LAA for LTE, 3GPP TSG RAN Meeting #73, New Orleans, September 2016.

[2] R4-1701086, Way forward on eLAA demodulation performance, 3GPP TSG-RAN WG4 Meeting #82, Athens, Greece, February 2017.

[3] R4-1700990, Simulation assumption for eLAA PUSCH demodulation, Ericsson, 3GPP TSG-RAN WG4 #82, Athens, Greece, February 2017.