3GPP TSG-RAN WG4 Meeting #82bis		R4-1703032

Spokane, USA, 3 - 7 April, 2017

Agenda item:	7.27.2.2

Source:	CATT

Title:	Control channels demodulation performance of Enhanced CRS-IM

Document for:	Discussion

Introduction

In RAN #73 meeting, the “LTE Enhanced CRS and SU-MIMO Interference Mitigation Performance Requirements” WI was initiated [1]. In the previous RAN4#82 meeting, further agreements on the target scenarios and reference receivers for Stage 1 investigations were provided in [2]. And it is proposed to provide simulation results to identify target scenarios for requirements definition and confirm reference receiver feasibility. In this contribution, we provide the simulation results based on the agreed simulation assumptions [3] for downlink control channels on enhanced CRS-IM. The detailed simulation assumptions are attached in the Annex.

Simulation results

In the way forward [3], the agreed control channel test cases are proposed as shown in Table 1.

Table 1. DL control channel test cases

| Test | Physical channel | CRS pattern | Number of UE RX chains | Number of CRS APs |  | Test purpose |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Serv. cell | Interf. cell |  |
| 1 | PDCCH | Non Colliding | 2 | 4 | 4 | 4 CRS APs IM investigation |
| 3 |  | Non Colliding | 4 | 4 | 4 | 4 CRS APs IM + 4 RX CRS IM investigation |
| 1 | PHICH | Non-Colliding | 2 | 4 | 4 | 4 CRS APs IM investigation |
| 3 |  | Non-Colliding | 4 | 4 | 4 | 4 CRS APs IM + 4 RX CRS IM investigation |

And the following reference receivers are used for the analysis:

- Receiver #1: LMMSE-MRC (baseline)

- Receiver #2: LMMSE-IRC

- Receiver #3: LMMSE-IRC + 1 Cell CRS-IM   (Full complexity two or four ports CRS-IM processing)

Simulation results of PDCCH

Test Case 1: PDCCH  4CRS APs + 2RX UE

Figure1. BLER performance of PDCCH Test Case 1

Test Case 3: PDCCH  4 CRS APs + 4 RX UE

- AL=2

- AL=1

Figure2. BLER performance of PDCCH Test Case 3

From the above link-level simulation results of the PDCCH, the performance gains of different receivers are shown in table 2.

Table2. the performance gains of SNR @ 1% BLER performance of PDCCH

| Test Case | MRC(dB) | IRC(dB) | IRC+CRS-IM(dB) | Gains of IRC(dB) | Gains of CRS-IM (dB) |
| --- | --- | --- | --- | --- | --- |
| PDCCH Test Case1 | 14.7 | 14.0 | 11.8 | 0.7 | 2.2 |
| PDCCH Test Case3-1 | 10.6 | 7.4 | 5.5 | 3.2 | 1.9 |
| PDCCH Test Case3-2 | 16.6 | 13.5 | 12.0 | 3.1 | 1.5 |

Observation 1:

For scenarios of 4CRS APs and 2RX chains, the testable performance gain of LMMSE-IRC+CRS-IM receiver (about 3dB) can be achieved over the LMMSE-MRC baseline receiver.

- The gain of CRS-IM processing is more obvious than the gain of IRC processing.

For scenarios of 4CRS APs and 4RX chains, large performance gains of LMMSE-IRC+CRS-IM receiver (about 5dB) can be achieved over the LMMSE-MRC baseline receiver.

- The gain of the IRC processing is more obvious than the gain of CRS-IM processing.

- It can provide a more practical SINR working point for AL=1 case rather than AL=2 case.

Simulation results of PHICH

Test Case 1: PHICH  4 CRS APs + 2 RX UE

Figure3. BLER performance of PHICH Test Case 1

Test Case 3: PHICH  4 CRS APs + 4 RX UE

Figure4. BLER performance of PHICH Test Case 3

Table3. the performance gains of SNR @ 0.1% BLER performance of PHICH

| Test Case | MRC(dB) | IRC(dB) | IRC+CRS-IC(dB) | Gains of IRC(dB) | Gains of CRS-IM (dB) |
| --- | --- | --- | --- | --- | --- |
| PHICH Test Case1 | 13.6 | 13.2 | 11.1 | 0.4 | 2.1 |
| PHICH Test Case3 | 10.7 | 7.6 | 6.8 | 3.1 | 0.8 |

Observation 2:

For scenarios of 4CRS APs and 2RX chains, the testable performance gain of the LMMSE-IRC+CRS-IM receiver (> 2dB) can be achieved over the LMMSE-MRC baseline receiver.

- The gain of CRS-IM processing is more obvious than the gain of IRC processing.

For scenarios of 4CRS APs and 4RX chains, large performance gain of the LMMSE-IRC+CRS-IM receiver (about 4dB) can be achieved over the LMMSE-MRC baseline receiver.

- The performance gain mainly comes from the IRC processing.

Conclusions

In this contribution, we provide our simulation results for DL control channel demodulation on enhanced CRS-IM. And according to the simulation results, the observations are given as below:

Observation 1: ( PDCCH )

For scenarios of 4CRS APs and 2RX chains, the testable performance gain of LMMSE-IRC+CRS-IM receiver (about 3dB) can be achieved over the LMMSE-MRC baseline receiver.

- The gain of CRS-IM processing is more obvious than the gain of IRC processing.

For scenarios of 4CRS APs and 4RX chains, large performance gains of LMMSE-IRC+CRS-IM receiver (about 5dB) can be achieved over the LMMSE-MRC baseline receiver.

- The gain of the IRC processing is more obvious than the gain of CRS-IM processing.

- It can provide a more practical SINR working point for AL=1 case rather than AL=2 case.

Observation 2: ( PHICH )

For scenarios of 4CRS APs and 2RX chains, the testable performance gain of the LMMSE-IRC+CRS-IM receiver (> 2dB) can be achieved over the LMMSE-MRC baseline receiver.

- The gain of CRS-IM processing is more obvious than the gain of IRC processing.

For scenarios of 4CRS APs and 4RX chains, large performance gain of the LMMSE-IRC+CRS-IM receiver (about 4dB) can be achieved over the LMMSE-MRC baseline receiver.

- The performance gain mainly comes from the IRC processing.

- References

- RP-161877 “New Work Item Proposal: Enhanced CRS and SU-MIMO Interference Mitigation Performance Requirements for LTE”, Intel Corporation, Huawei, LGE, RAN #73, September 2016

- R4-1702195 “WF on Enhanced CRS-IM Performance Requirements”, Intel Corporation, LGE, Ericsson, RAN4 #82, February 2017

- R4-1702451 “Simulation assumptions for Enhanced CRS-IM”, Intel Corporation, LGE, RAN4 #82, February 2017

- Annex - Simulation Assumptions

- PDCCH/PCFICH simulation assumptions

PDCCH/PCFICH simulation assumptions Table 6 includes a list of the test cases to be used for the investigations of the CRS-IM enhancements for the PDCCH/PCFICH. The PDCCH/PCFICH simulation assumptions are provided in 4.

Table 4. PDCCH test cases

| Test | Physical channel | CRS pattern | Number of UE RX chains | Number of CRS APs |  | Test purpose |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Serv. cell | Interf. cell |  |
| Scenarios for performance requirements definition |  |  |  |  |  |  |
| 1 | PDCCH | Non Colliding | 2 | 4 | 4 | 4 CRS APs IM investigation |
| Scenarios for feasibility analysis |  |  |  |  |  |  |
| 3 |  | Non Colliding | 4 | 4 | 4 | 4 CRS APs IM + 4 RX CRS IM investigation |

Table 5. PDCCH/PCFICH simulation assumptions

| Parameter | Value |
| --- | --- |
| Cell ID pattern | Non-colliding CRS: 0/1/6 |
| Interference power profile | I1/Noc = 13.91 dB; I2/Noc = 3.34 dB. |
| Time offset between cells | Cell 1: 2 µs Cell 2: 3 µs |
| Frequency offset between cells | Cell 1: 200 Hz Cell 2: 300 Hz |
| Channel model | EPA-5Hz for all links |
| PHICH parameters | Normal PHICH Ng = 1/6 |
| Interference model | Rel-13 CCIM (TS 36.101 B.7) |
| PDCCH/PCFICH parameters | Test #1: PDCCH AL = 2; CFI = 2 Test #3: PDCCH AL = [1,2]; CFI = 2 |
| Receivers | Baseline: LMMSE-MRC. Enhanced reference receiver structures: LMMSE-IRC (for information), LMMSE-IRC + 1 cell CRS-IM. |

Table 6. PDCCH FRC

| PDCCH FRC |  |  |
| --- | --- | --- |
| Parameter | Unit | Value |
| Number of transmitter antennas |  | 4 |
| Channel bandwidth | MHz | 10 |
| Number of OFDM symbols for PDCCH | symbols | 2 |
| Aggregation level | CCE | [1,2] |
| DCI Format |  | 2 |
| Cell ID |  | 0 |
| Payload (without CRC) | Bits | 46 |

- PHICH simulation assumptions

Table 8 includes a list of the test cases to be used for the investigations of the CRS-IM enhancements for the PHICH. The PHICH simulation assumptions are provided in 7.

Table 7. PHICH test cases

| Test | Physical channel | CRS pattern | Number of UE RX chains | Number of CRS APs |  | Test purpose |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Serv. cell | Interf. cell |  |
| Scenarios for performance requirements definition |  |  |  |  |  |  |
| 1 | PHICH | Non-Colliding | 2 | 4 | 4 | 4 CRS APs IM investigation |
| Scenarios for feasibility analysis |  |  |  |  |  |  |
| 3 |  | Non-Colliding | 4 | 4 | 4 | 4 CRS APs IM + 4 RX CRS IM investigation |

Table 8. PHICH simulation assumptions

| Parameter | Value |
| --- | --- |
| Cell ID pattern | Non-colliding CRS: 0/1/6 |
| Interference power profile | I1/Noc = 8.36 dB; I2/Noc = 1.66 dB |
| Time offset between cells | Cell 1: 2 µs Cell 2: 3 µs |
| Frequency offset between cells | Cell 1: 200 Hz Cell 2: 300 Hz |
| Channel model | EPA-5Hz for all links |
| PHICH duration | Extended |
| Interference model | Rel-13 CCIM (TS 36.101 B.7) |
| PDCCH/PCFICH/PHICH parameters | PHICH Ng = 1, FRC: R.19 in the TS 36.101 CFI = 3 |
| Receivers | Baseline: LMMSE-MRC. Enhanced reference receiver structures: LMMSE-IRC (for information), LMMSE-IRC + 1 cell CRS-IM. |

Table 9. PHICH FRC

| PHICH FRC |  |  |
| --- | --- | --- |
| Parameter | Unit | Value |
| Number of transmitter antennas |  | 4 |
| Channel bandwidth | MHz | 10 |
| User roles |  | W I1 I2 |
| Resource allocation |  | (0,0) (0,1) (0,4) |
| Power offsets | dB | -4 0 -3 |
| Payload |  | A R R |