3GPP TSG-RAN WG4 Meeting #82 Bis	R4-1703275

Spokane, USA, 3rd – 7th April 2017

Agenda item:	10.3.2

Source: 	Qualcomm Incorporated

Title: 	Channel Raster and Synchronization Signal Raster Considerations

Document for:	Discussion

# 1. 	Introduction

NR is envisioned to be designed to allow more flexible deployment (e.g. synchronization signal may not locate at the center of NR carrier) and be more energy efficiency (e.g. without always on reference signal similar to CRS in LTE). However, those features targeted by NR design may inevitably complicate the NR initial acquisition. In LTE initial acquisition, always on CRS combined with the fact that LTE SYNC is always at the center of the carrier, provides UE opportunities for simplifying the search procedure, e.g. UE may perform frequency scan to estimate locations of the LTE carrier. Since synchronization signal is always in the center of LTE carrier, UE can search for the carrier center frequency by scanning through targeted frequency bands which can greatly reduce the number of hypothesis of synchronization signal frequency locations. However, as we discussed, NR can face more challenges in initial acquisition, for example

- Reference signals similar to LTE CRS might not be always broadcast, making frequency scan very challenging.

- Potential NR/LTE confusion since both NR and legacy LTE employ OFDM-based waveforms in the downlink.

- Potential “alien waveform” confusion when some mMTC waveforms or some on-demand signals might exist in the frequency band.

- NR is expected to have deployments in some wide frequency bands so frequency scanning using channel raster might not be efficient.

The channel raster and the synchronization signal raster are related since the synchronization signal has to sit on the channel raster. Furthermore, the channel raster should offer enough deployment flexibility for optimal spectral usage.

Discussion on the definition of the center frequency of the synchronization signals is ongoing in RAN1 and some agreements were reached, however, all the details are not finalized.

Working assumptions:

- For a NR cell, the center frequency for the synchronization signal can be different from the center frequency of the NR carrier

  - FFS: Relationship between the center frequency of the NR carrier and the center frequency of synchronization signals and investigate tradeoff between UE complexity and flexibility

The assumption implies that frequency scan based on carrier detection cannot lead to candidates for synchronization signal carrier frequency even after identifying the center of the carrier. To overcome above challenges, it is more efficient to decouple the channel raster and the synchronization signal frequency raster. The synchronization signals and PBCH are transmitted over a frequency defined by the synchronization signal frequency raster. In particular, UE can search for synchronization signals from a known reduced set of frequencies defined by synchronization signal frequency raster which can be coarser than the channel raster. In this contribution, we discuss synchronization signal frequency raster design considerations for both below 6GHz and above 6GHz.

# 2. 	Synchronization Frequency Raster

2.1. Synchronization frequency raster design considerations

From system design and implementation simplicity perspectives, it is beneficial to keep the subcarriers of synchronization signals to be aligned with the subcarriers of data channel. Hence, we discuss following key considerations when designing the synchronization signal frequency raster.

- Let us first assume that the center frequency of the NR carrier is fixed. The question is “what is the frequency of the synchronization signals to make it aligned with the subcarriers of data channel?” The answer is that the synchronization signal frequency raster is an integer multiple of data subcarrier spacing.

- We now fix the frequency of synchronization signals and would like to use any channel raster frequency as the center frequency of the NR carrier. The next question is “what is requirement for the channel raster such that the frequency of synchronization signals can be aligned with subcarriers of data channel?” Its answer is that the channel raster is an integer multiple of data channel subcarrier spacing. Note that LTE channel raster of 100 kHz is not multiple of 15 kHz.

- If we would like to have synchronization raster and channel raster to be multiple of subcarrier spacing, the last question to ask is “what is the maximum of data channel subcarrier spacing to be supported?” In last meeting, RAN4 made the agreements that 15kHz, 30kHz and 60kHz are feasible subcarrier spacing for below 6GHz while 60kHz, 120kHz and 240kHz are potential candidates for subcarrier spacing for above 6GHz [2].

Proposal 1: The frequency of synchronization signals should be aligned with the subcarriers of data channel. More specifically, the synchronization signal frequency raster is an integer multiple of data subcarrier spacing.

2.2. Synchronization signal bandwidth

Below are RAN1 #87 agreements on the synchronization signal frequency raster.

Agreements:

- The raster for NR synchronization signals can be different per frequency range. At least for frequency ranges where NR supports a wider carrier bandwidth and operation in a wider frequency spectrum (e.g. above 6 GHz), the NR synchronization signals raster can be larger than the 100 kHz raster of LTE.

- A joint decision should be made on:

  - the supported minimum carrier bandwidth for a NR carrier

  - the supported bandwidths of synchronization signals for NR

  - the frequency raster for synchronization signals for NR

  - the frequency raster for the center of NR carrier (if applicable)

Furthermore, the following agreement has been reached in RAN1 AdHoc (Jan. 2017 Spokane)

Agreements:

- When the sync bandwidth is smaller than the minimum system bandwidth for a given frequency band, RAN1 strives to make the synchronization signal frequency raster sparser compared to channel raster to reduce UE initial cell selection burden without limiting the NR deployment flexibility

  - FFS If UE is required to search for all the possible synchronization signal frequency locations defined by the synchronization signal frequency raster

- When the sync bandwidth is the same as the minimum system bandwidth for a given frequency band that UE searches, synchronization signal frequency raster is the same as the channel raster

  - FFS If UE is required to search for all the possible synchronization signal frequency locations defined by the synchronization signal frequency raster

Based on the agreements, both supported minimum carrier bandwidth and supported synchronization signal bandwidth should be considered when designing the synchronization signal frequency raster. In [1], we discuss our views on bandwidth requirements for NR. More specifically, the NR frequency bands are classified into two categories:

- Band category A targeted for the deployment of below 6GHz frequency bands

- Band category B targeted for the deployment of above 6GHz frequency bands

For band category A and B, the synchronization signal bandwidth is summarized in Table 1.

Table 1: Synchronization signal bandwidth

| Parameter | Band Category A | Band Category B |
| --- | --- | --- |
| Synchronization signal bandwidth [MHz] | 4.32 | 34.56 |

For band category A, the synchronization signal bandwidth is 4.32 MHz, which introduces more challenges to support minimum system bandwidth of 5MHz considering that large synchronization raster is needed to make the UE initial access complexity/latency manageable. We have the following two proposed solutions

- Option A1: Additional synchronization frequency location can be defined in the RAN4 spec in addition to the large sync raster in the band that 5MHz NR deployment is needed. This solution guarantees that every NR carrier is self discoverable

- Option A2: 5MHz NR carrier can be used as SCell for CA. The actual synchronization signal frequency location in SCell can be indicated/signaled by the PCell. This solution cannot guarantee that synchronization signal on SCell is always self discoverable but does not require RAN4 to exhaust all the possible sync locations for 5MHz deployment.

The numerology of the synchronization signals for different frequency bands and design options are provided in Table 2.

Table 2: Synchronization signal bandwidth and numerology

| Parameter | Band Cat A | Band Cat B |
| --- | --- | --- |
| Synchronization signal bandwidth (MHz) | 4.32 | 34.56 |
| Sub-carrier spacing (kHz) | 30 | 240 |

Proposal 2: Two band categories can be defined for synchronization signals

- Band category A: Synchronization signal has synchronization signal bandwidth 4.32MHz and synchronization signal SCS of 30KHz

- Band category B: Synchronization signal has synchronization signal bandwidth 34.56MHz and synchronization signal SCS of 240KHz

2.3. Channel raster and synchronization frequency raster

As discussed previously, it is desirable to keep the center frequency of synchronization signals to be aligned with the subcarriers of data channel. One requirement is that the channel raster is an integer multiple of data channel subcarrier spacing. In particular, we consider one of the following options for the channel raster (CR) for below 6GHz.

- Channel raster option 1: The channel raster for below 6GHz is 120 kHz which is larger than but closest to LTE CR of 100 kHz.

- Channel raster option 2: The channel raster for below 6GHz is 300 kHz which is a subset of LTE CR.

On the other hand, the proposed channel raster for above 6GHz is [960/1920 kHz].

Proposal 3: The channel raster is either 120 kHz or 300 kHz for below 6GHz, and either 960 kHz or 1.92MHz for above 6GHz.

As discussed in Section 2.1, the synchronization frequency raster and synchronization bandwidth should be selected such that any channel number could be used as the carrier center frequency. In addition, regardless of carrier center frequency, full synchronization signals are transmitted. As a result, for a given synchronization raster and minimum system bandwidth, the synchronization bandwidth () could be upper-bounded by

where is the channel bandwidth discussed in Section 2.2 and is the guard band applied on channel bandwidth. The channel bandwidth and transmission bandwidth configuration for one E-UTRA carrier are defined in [2] and illustrated in Figure 1.

Figure 1. Definition of channel bandwidth and transmission bandwidth configuration for one E-UTRA carrier [2]

From UE perspectives, the larger the synchronization signal frequency raster, the lower the initial search latency. As we discussed before, for the band category A, two different options can be considered for supporting minimum system bandwidth of 5MHz, i.e. Option A1 and Option A2.  Below are synchronization signal frequency raster (SR) proposal for band category A

- SR option 1: If the channel raster is 120 kHz (Channel raster option 1), the synchronization signal frequency raster is 4.68MHz for band category A.

- SR option 2: If the channel raster is 300 kHz (Channel raster option 2), the synchronization signal frequency raster is 4.5MHz for band category A.

For band category B, the synchronization signal frequency raster is proposed to be 36MHz. Table 3 provides computation of the synchronization signal frequency raster given in above SR options.

Table 3: Synchronization raster computation

| Parameter | Band Cat A (120kHz CR) | Band Cat A (300kHz CR) |
| --- | --- | --- |
| Minimum carrier bandwidth (MHz) | 5 |  |
| Synchronization signal bandwidth (MHz) | 4.32 |  |
| Upper bound of sync raster (MHz) | 10 – 10*0.1 – 4.32 = 4.68 |  |
| Synchronization raster | 39*120kHz = 4.68MHz | 15*300kHz = 4.5MHz |

Based on above discussions, the following proposal is made:

Proposal 4: The synchronization signal frequency raster is 36MHz for band category B. In addition, the following options for the synchronization signal frequency raster can be considered for band category A

- SR option 1: If the channel raster is 120 kHz (Channel raster option 1), the synchronization signal frequency raster is 4.68MHz for band category A.

- SR option 2: If the channel raster is 300 kHz (Channel raster option 2), the synchronization signal frequency raster is 4.5MHz for band category A.

Another important thing to consider is the possible frequency location of the synchronization signal and NR carrier, i.e.  sync raster and channel raster. In LTE, the band definition always starts from integer multiple of channel and sync raster (100kHz), hence even though for each band, the channel frequency location is defined relative to the starting frequency of each band, the channel frequency locations are aligned among all the frequency bands. However, based on our discussion on the channel raster and sync raster, this may not be true for NR. Hence we propose that both the channel and sync raster start at 0 Hz for all bands, irrespective of where the band boundaries are.

Proposal 5: Both the channel and sync raster start at 0 Hz for all bands, irrespective of where the band boundaries are.

# 3. 	Conclusion

This contribution presented synchronization raster design for both below 6GHz and above 6GHz. More specifically, it has the following proposals:

Proposal 1: The frequency of synchronization signals should be aligned with the subcarriers of data channel. More specifically, the synchronization signal frequency raster is an integer multiple of data subcarrier spacing.

Proposal 2: Two band categories can be defined for synchronization signals

- Band category A: Synchronization signal has synchronization signal bandwidth 4.32MHz and synchronization signal SCS of 30kHz

- Band category B: Synchronization signal has synchronization signal bandwidth 34.56MHz and synchronization signal SCS of 240kHz

Proposal 3: The channel raster is either 120 kHz or 300 kHz for below 6GHz, and either 960 kHz or 1.92MHz for above 6GHz.

Proposal 4: The synchronization signal frequency raster is 36MHz for band category B. In addition, the following options for the synchronization signal frequency raster can be considered for band category A

- Synchronization signal frequency raster option 1: If the channel raster is 120 kHz, the synchronization signal frequency raster is 4.68MHz for band category A.

- Synchronization signal frequency raster option 2: If the channel raster is 300 kHz, the synchronization signal frequency raster is 4.5MHz for band category A.

Proposal 5: Both the channel and sync raster start at 0 Hz for all bands, irrespective of where the band boundaries are.

# 4. 	References

- R1-1702584, “Synchronization signal bandwidth and multiplexing consideration,” Qualcomm Incorporated.

- R4-1702374, “WF on NR subcarrier spacing and Channel bandwidth”

- TS. 36.101, Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception