3GPP TSG-RAN WG4 #82bis	R4-1703292

# Spokane, USA, 3th – 7th April 2017

Agenda Item:	7.14.7

Source:	Ericsson

Title:	Further discussion on eLAA demodulation

Document for:	Discussion

# Introduction

In RAN4#82 meeting, the eLAA demodulation was further discussed. The WF is agreed in [1]. In this paper, we further discuss the open issues for eLAA demodulation.

# Discussion

## Test purpose

In last meeting, the group discuss the test purposes for eLAA PUSCH. Three possible test purposes are listed as:

- Verify PUSCH new interlace resource structure

- FFS: Verify DM-RS channel estimation performance with contiguous interlace allocation

- FFS: Verify PUSCH demodulation performance with UL LBT model

For the channel estimation, as a general principle, it can be verified by PUSCH performance. If the channel estimation can be verified for the basic unit (one interlace), it is less possible to have channel estimation problem for more interlaces allocation, since the channel estimation algorithm for one interlace is the base for multiple interlaces allocation. Furthermore, in the companion paper [2], the performance for both 2 interlaces and one interlace are given. From the simulation results, we can see that the performance of 2 interlaces is very close to 1 interlace. It is really challenge to check the purpose if we set it. Further, with contiguous resources allocation, channel estimation may be optimized. However, from RAN4 point of view, RAN4 shall target for minimum performance requirements, not for optimization performance. Thus, we don’t think it is necessary to have one test purpose to verify DM-RS channel estimation performance with contiguous interlace allocation.

For UL LBT model, the purpose to model LBT for uplink transmission is to verify whether eNB can handle PUSCH missing due to LBT fail. However, this behaviour is not new. This behaviour is a nature behaviour from Rel-8. From Rel-8, Uplink grant may be missing. When uplink grant is missing, PUSCH will be not transmitted from UE. In this case, eNB must take proper action for the PUSCH missing. Thus, it is not so critical to model LBT or not in the eLAA performance requirements.

Furthermore, as what we have observed in downlink LBT model, it takes tremendous time to converge on the LBT model. For the UL LBT model, it may couple with downlink LBT. It is really challenge to have a good model without extensive discussion. If we can avoid the LBT model, we prefer not to model LBT in uplink performance requirements.

It is not necessary to have dedicated test purposes for DM-RS channel estimation performance with contiguous interlace allocation

For simplicity, UL LBT is not modelled for eLAA PUSCH demodulation performance

## Test configuration

For test configuration, the ending symbol configuration, channel model and MCS are still open. In this section, we share our view on these open issues.

- Ending symbol configuration

- Option 1: Up to OFDM symbol 13

- Option 2: Up to OFDM symbol 12

Per RAN1 agreements, one of two ending symbol positions can be configured. Value 0 indicates PUSCH in a UL subframe is transmitted up to OFDM symbol 13 and value 1 indicates PUSCH is transmitted up to OFDM symbol 12. Up to OFDM symbol 13 or up to OFDM symbol 12, both are treated in legacy system. For example, when sounding signal is configured, the PUSCH format is the same as the format with up to OFDM symbol 12. To simply the test, only one value is selected for the PUSCH transmission, for example, value “0” is used to indicate to simply the test.

PUSCH is transmitted up to OFDM symbol 13

- Channel model

For the channel model, there are two options, one is EPA5 and one is EVA5. We don’t see too much difference for EPA5 and EVA5 to verify eNB behavior. Since in the simulation assumption, we have agreed to use EPA5 for simulation alignment, if there is no big technical problems are identified, we slightly prefer to use EPA5 to reduce the simulation efforts.

EPA5 is preferred if there are no technique issues are identified for EPA5 channel model.

- Modulation

For modulation, we still have QPSK, 16QAM and 64 QAM. We can make down selection for them to reduce the test case

Candidate modulations can be down selected.

# Conclusion

In this paper, we share our view on the open issues for eLAA PUSCH, we have the following proposals:

Proposal 1	It is not necessary to have dedicated test purposes for DM-RS channel estimation performance with contiguous interlace allocation

Proposal 2	For simplicity, UL LBT is not modelled for eLAA PUSCH demodulation performance

Proposal 3	PUSCH is transmitted up to OFDM symbol 13

Proposal 4	EPA5 is preferred if there are no technique issues are identified for EPA5 channel model.

Proposal 5	Candidate modulations can be down selected.

# References

R4-1702146, Way forward on eLAA demodulation performance requirements, Huawei, Ericsson, Nokia, Alcatel-Lucent Shanghai Bell, Meeting, Feb. 2017.

R4-1703293, Preliminary simulation results for eLAA PUSCH, Ericsson, April 2017.