3GPP TSG-RAN WG4 Meeting #82bits		               R4-1703335

Spokane, USA, 3 - 7 April 2017

Source:	CMCC

Title:	Discussion on the directivity for OTA receiver sensitivity

Agenda item:	9.12.2.4

Document for:	Approval

Introduction

According to the WF [1], the formula OTA sensitivity defines the minimum EIS as follows:

    - Minimum EIS = Conducted reference sensitivity – D + L + Off-peak Margin

We know that the antenna GAIN can be calculated form D and L. And almost very vendors supply antenna specifications including, GAIN, HBW, EBW. So in this contribution, we check some proposed formulas for estimating D factor, together with the L factor, to verify how these formulas fulfil the real specifications of antennas. And we found out that none of them can cover all the types of the antennas.

And we propose a potential method to measuring the D factor for accurate evaluation of the AAS system. Also we propose a method to measure the D with no cost or a little more cost.

Discussion

2.1 Formulas to estimate Directivity

According to documentaries, people want to estimate directivity and gain accurately. Many simple formulas are available for estimating antenna gain. And these formals consist of some constants, HBW, EBW, such as:

DB (Directivity Beamwidth Product) is similar to the gain-bandwidth product that is commonly used to characterize circuit devices. Here are some examples shown in table 1[2].

| Case | Antenna Type | Directivity Beamwidth Product |
| --- | --- | --- |
| 1 | No-sidelobe pattern, rectangle beam | 41235 |
| 2 | No-sidelobe pattern, conical beam | 52525 |
| 3 | Uniform rectangle aperture | 32383 |
| 4 | Cosine-uniform rectangle aperture | 35230 |
| 5 | Uniform circular aperture | 33709 |

Table 1 Directivity-beamwidth product for use in directivity estimation

Form the table 1we noticed that every DB has its application range, which again explains why each formula alone cannot satisfy all types of antenna.

For simplification, we choose case 1, 3 and 36000+ model [3] for verification.

36000+ model:	D = 10*log10(36000/( ABW*EBW)+0.3)

2.2 Error analysis for estimate Directivity and fixed L

Here are some commercial antennas specifications for data examples which are described in Table 2 and 3.

The antennas in Table2 and Table3 are divided into two sets according to frequency band. In order to add samples, we also involved some antennas from other contributor [4], and listed in Table4. Columns from 2 to 5 are the measured data of the antennas supplied by vendors. Columns from 6 to 8 are estimated directivity form different formulas.

| SET1 | Freq MHz | Gain dB | HPE degree | HPH degree | Case1 Estimated D dB | Case3 Estimated D dB | 36000+ Estimated D dB | L dB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ant 1 | 2200-2490 | 18.8 | 62 | 4.3 | 20.84 | 21.89 | 21.31 | 2 |
| Ant 2 | 2200-2490 | 16.6 | 60 | 9 | 17.78 | 18.83 | 18.26 | 2 |
| Ant 3 | 2300-2500 | 18.6 | 65 | 5.3 | 19.73 | 20.78 | 20.2 | 2 |
| Ant 4 | 2300-2500 | 18.1 | 62 | 5.5 | 19.77 | 20.82 | 20.25 | 2 |

Table 2 Parameters for SET1 antenna and estimated directivity

| SET2 | Freq MHz | Gain dB | HPE degree | HPH degree | Case1 Estimated D dB | Case3 Estimated D dB | 36000+ Estimated D dB | L dB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ant 5 | 1920-2220 | 18.7 | 65 | 5 | 19.98 | 21.04 | 20.46 | 2 |
| Ant 6 | 1920-2220 | 16.2 | 64 | 10 | 17.04 | 18.09 | 17.5 | 2 |
| Ant 7 | 1920-2180 | 18.3 | 69 | 6.1 | 18.86 | 19.91 | 19.33 | 2 |
| Ant 8 | 1920-2180 | 17.4 | 67 | 6.6 | 18.52 | 19.57 | 18.99 | 2 |

Table 3 Parameters for SET2 antenna and estimated directivity

| SET3 | Freq MHz | Gain dB | HPE degree | HPH degree | Case1 Estimated D dB | Case3 Estimated D dB | 36000+ Estimated D dB | L dB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ant 9 | 1710-1990 | 11 | 67 | 36 | 19.98 | 21.04 | 20.46 | 2 |
| Ant 10 | 1920-2200 | 11.5 | 65 | 31 | 17.04 | 18.09 | 17.5 | 2 |
| Ant11 | 2200-2490 | 12.2 | 60 | 25 | 18.86 | 19.91 | 19.33 | 2 |
| Ant 12 | 2490-2690 | 12.7 | 58 | 25 | 18.52 | 19.57 | 18.99 | 2 |
| Ant13 | 1710-1880 | 20.6 | 65 | 3.7 | 10.33 | 11.28 | 11.83 | 2 |
| Ant14 | 1850-1990 | 21.1 | 62 | 3.5 | 11.11 | 12.06 | 12.59 | 2 |
| Ant15 | 1920-2200 | 21.2 | 60 | 3.3 | 12.39 | 13.34 | 13.86 | 2 |
| Ant16 | 1695-1880 | 18.6 | 43 | 7.8 | 12.54 | 13.49 | 14.00 | 2 |
| Ant17 | 1850-1990 | 19.2 | 39 | 7.3 | 20.34 | 21.29 | 21.76 | 2 |
| Ant18 | 1920-2179 | 19.4 | 38 | 7.1 | 20.79 | 21.74 | 22.21 | 2 |
| Ant19 | 2300-2400 | 19.8 | 36 | 6.4 | 21.19 | 22.14 | 22.60 | 2 |
| Ant20 | 2500-2690 | 19.6 | 33 | 5.8 | 18.9 | 19.85 | 20.32 | 2 |

Table 4 Parameters for SET3 antenna and estimated directivity

After calculate the estimated D, the difference between the real gain and estimated gain can be get below:

The gain errors for SET1,SET2 and SET3 are listed in TABLE 5.

| ERROR | Case 1 model | Case 3 model | 36000+ |
| --- | --- | --- | --- |
| Ant 1 | 0.04 | 1.1 | 0.54 |
| Ant 2 | -0.82 | 0.23 | -0.34 |
| Ant 3 | -0.87 | 0.18 | -0.34 |
| Ant 4 | -0.32 | 0.7 | 0.15 |
| Ant 5 | -0.72 | 0.33 | -0.24 |
| Ant 6 | -1.15 | -0.11 | -0.68 |
| Ant7 | -1.43 | -0.39 | -0.97 |
| Ant 8 | -0.88 | 0.17 | -0.44 |
| Ant 9 | -0.67 | 0.28 | 0.83 |
| Ant 10 | -0.39 | 0.56 | 1.09 |
| Ant 11 | 0.19 | 1.14 | 1.66 |
| Ant 12 | -0.16 | 0.79 | 1.30 |
| Ant13 | -0.26 | 0.69 | 1.16 |
| Ant14 | -0.31 | 0.64 | 1.11 |
| Ant15 | -0.01 | 0.94 | 1.40 |
| Ant16 | 0.29 | 1.25 | 1.72 |
| Ant17 | 0.41 | 1.36 | 1.83 |
| Ant18 | 0.44 | 1.39 | 1.86 |
| Ant19 | 0.73 | 1.68 | 2.15 |
| Ant20 | 1.73 | 2.68 | 3.15 |
| Error range | [-1.43, 1.73] | [-0.39, 2.68] | [-0.97, 3.15] |

Table 5 error calculation for SET1 antennas

According to the gain error results, we can figure out that the calculated gain due to estimated D and fixed L has big error between real values. None of the three formulas can satisfy all the antennas. So it is difficult to use formula to cover all kinds of antenna.

The measurement method of non AAS antenna is complete and mature. For such a accurate measurement system, using formulas to estimate the D leads to unknown errors for different types of antenna. As for AAS, the D factor is depended on the declared ROAOA, which may be irregular and dissymmetric compared to non AAS pattern. So how to make sure the formula shall be chosen in such a way that it will never overestimate the directivity is difficult.

Oberservation1: Using formula to estimated D factor has different errors for different types of non AAS antenna; this is the same for different ROAOA of AAS.

Oberservation2: One uniform formula may involve big error. Consideration should be taken to analysis how much error can be accepted.

2.3 Directivity measurement method

In this contribution, we provide another solution to get the directivity.

The ACLR requirement is defined as TRP measurement of both in channel and co-channel radiated power [5].

This provides a lot of chance to measure different kind of specifications of the AAS system.

When performing ACLR test, the EIRP and the TRP can be obtain at the same time. According to the antenna theory, the directivity of the transmitter antenna can be defined from the formula in dB [6]:

For TDD AAS system which transmitter and receiver have the same connection relationship with antenna, the transmitter and receiver share the same antenna at the same frequency band. Under this condition, the antenna is totally reciprocal. The directivity is the same for both transmitter and receiver.

Proposal 1 For TDD AAS system which transmitter and receiver have the same connection relationship with antenna, the directivity for EIS can be got from transmitter test.

For FDD system the antenna and TDD system which transmitter and receiver have the different connection relationship with antenna, the system is not wholly reciprocal. The receiver directivity can be tested during ALCR measurement.

The method is similar to the transmit antenna directivity measurement. For receive antenna, the radiated power should changed to received power or sensitivity. Of course, the second one will consume a huge of time.

Proposal 2 For FDD system the antenna and TDD system which transmitter and receiver have the different connection relationship with antenna, the directivity for EIS can be tested during transmitter ACLR test by measuring the antenna directivity directly.

Conclusion

In this contribution, we analysed three formulas for estimating D factor of EIS. It can figure out that none of the formula is suitable for all kinds of non AAS antenna; this is the same for the ROAOA of AAS.

Oberservation1: Using formula to estimated D factor has different errors for different types of non AAS antenna; this is the same for different ROAOA of AAS.

Oberservation2: One uniform formula may involve big error. Consideration should be taken to analysis how much error can be accepted.

So we suggest another method to get directive for a potential plan.

Proposal 1 For TDD AAS system which transmitter and receiver have the same connection relationship with antenna, the directivity for EIS can be got from transmitter test.

Proposal 2 For FDD system the antenna and TDD system which transmitter and receiver have the different connection relationship with antenna, the directivity for EIS can be tested during transmitter ACLR test by measuring the antenna directivity directly.

References

- R4-1702271, WF on OTA sensitivity

- Estimating Directivity and Gain of Antennas, WarrenL. Stutzman, IEEE Antennas and Propagation Magazine,Vol. 40, No. 4, August 1998

- R4-1610388, Estimating antenna gain using 3dB beam width, Huawei

- R4-1701725, Directivity value for minimum EIS, Huawei

- R4-1610363,WF on the measurement grid for ACLR, Ericsson, NTT DoCoMo, Huawei

- Antenna Theory and Design, Warren L. Stutzman, Gary A. Thiele.