3GPP TSG-RAN4 Meeting #82bis	R4-1703471

# Spokane, United States, 3rd Apr 2017 - 7th Apr 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.803 | CR | 0003 | rev | - | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TR 38.803: Removing an error of conductive test of beam correspondence |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_newRAT |  |  |  |  |  | Date: |  |  | 2017-03-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-14 (Release 15) Rel-14 (Release 16) |  |
| Reason for change: |  | There is an error in the summary table of the SI outcome (Table 6.2.1-1) regarding beam correspondence which doesn’t require conductive test. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove “At least conductive test is needed.” from beam correnpondence requirement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency of the description still exists. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Unchanged sections omitted>

6.2.1		General

Agreement in SI and issue should be addressed in WI are summarized in Table 6.2.1 -1 for UE RF aspects.

Figure 6.2.1-1: Frequency range 1/2 and the threshold.

Table 6.2.1-1: Summary on Outcome in SI and topic to be addressed in WI

| Requirement |  | Range | Outcome in SI | Topic to be addressed in WI |
| --- | --- | --- | --- | --- |
| Tx | Tx maximum output power | 1 | - At least conductive test is needed. | - Conducted value - Conducted tolerance - Power sharing mechanism with LTE in NSA if specified |
|  |  | 2 | - At least EIRP is used as a metric - Develop requirements for one power class as priority - After requirements are understood for one PC, then, other PCs will be added. - Develop different spatial coverage requirement. Smartphone (i.e. Full sphere) is the baseline of UE types in Rel-15 - For CDF method, RAN4 method for describing spherical coverage of RF parameters is CDF where each point represents equal surface area in sphere surrounding the UE. - To study the advantage of this CDF method. - The other method(s) are not precluded. | - EIRP value - EIRP tolerance - How to categorise the UE type with different spatial coverage - Necessity of TRP considering regulation and/or 3GPP point of view - How to specify different power classes depending on “TRP or EIRP” and band dependency - Necessity of power sharing mechanism with LTE in NSA |
|  | MPR | 1 | - At least conductive test is needed. | - MPR values for both contiguous and non-contiguous resource allocation (MOP and emission requirements need to be defined first) - Granularity of MPR spec table |
|  |  | 2 | - At least EIRP is used as a metric | - Same as range 1 - Necessity of TRP |
|  | A-MPR | 1 | - At least conductive test is needed. | - Whether the same values as LTE are reused or not (MPR requirement is needed first) |
|  |  | 2 | - At least EIRP is used as a metric | - Same as range 1 - Necessity of TRP |
|  | PCMAX | 1 | - At least conductive test is needed. | - Calculation mechanism (e.g. reference SF) - PCMAX tolerance |
|  |  | 2 | - At least EIRP is used as a metric | - Same as range 1 - Necessity of TRP |
|  | Minimum output power | 1 | - At least conductive test is needed. - To specify -40dBm in sub-6GHz | - No open issue |
|  |  | 2 | - At least EIRP is used as a metric | - Whether the same limit (-40 dBm) can be reused in mmWave considering NF, MCL and degradation level of noise floor and system perspective. - Necessity of TRP - How to categorise the UE type with different spatial coverage |
|  | Tx OFF power | 1 | - At least conductive test is needed. - To specify -50dBm/MHz in sub-6GHz | - No open issue |
|  |  | 2 | - TRP is used as a metric | - Whether -50dBm should be used in mmWave considering; - NF of NR UE - MCL between the aggressive and victim UE - Degradation level of noise floor due to interference from aggressive NR UE transmit OFF power |
|  | ON/OFF mask | 1 | - At least conductive test is needed. | - ON/OFF mask value - Whether shorter transient period (20 us) can be reused in sub-6GHz according to possible sub-carrier spacing |
|  |  | 2 | - At least Beam peak is used as a metric | - Same as range 1 - Necessity of TRP - Achievable transient period in mmWave (e.g., 28 GHz) devices assuming dynamic range of 63dB which is starting point |
|  | Power control | 1 | - At least conductive test is needed. | - Power control requirements based on RAN1 agreement |
|  |  | 2 | - At least Beam peak is used as a metric | - Same as range 1 - Necessity of TRP |
|  | Frequency error | 1 | - At least conductive test is needed. - To specify 0.1ppm in sub-6GHz | - No open issue |
|  |  | 2 | - Beam peak is used as a metric | - Frequency error value - Whether the same frequency error (0.1 ppm) can be reused in mmWave considering settling time, etc. |
|  | EVM | 1 | - At least conductive test is needed. - Develop first requirements for the baseline CP-OFDM assuming suitable spectral confinement methods - Similar Transmitter impairments to LTE will be used as baseline for sub-6 and mmWave studies (IQ Image, Carrier leakage, CIM3, Phasenoise) | - EVM value for both average EVM measured over all the allocated PRBs and a few edge PRBs - How to limit sub-carrier spacing for the Rel-15 WID - EVM equalizer spectrum flatness value |
|  |  | 2 | - Beam peak is used as a metric - Same as range 1 | - Same as range 1 |
|  | Carrier leakage | 1 | - At least conductive test is needed. - Develop first requirements for the baseline CP-OFDM assuming suitable spectral confinement methods - Similar Transmitter impairments to LTE will be used as baseline for sub-6 and mmWave studies (IQ Image, Carrier leakage, CIM3, Phasenoise) | - Carrier leakage value - TRx impairment impact to multiple numerology case - How to limit sub-carrier spacing for the Rel-15 WID |
|  |  | 2 | - Beam peak is used as a metric - Same as range 1 | - Same as range 1 |
|  | In-band emissions | 1 | - At least conductive test is needed. - Develop first requirements for the baseline CP-OFDM assuming suitable spectral confinement methods - Similar Transmitter impairments to LTE will be used as baseline for sub-6 and mmWave studies (IQ Image, Carrier leakage, CIM3, Phasenoise) | - In-band emission values - TRx impairment impact to multiple numerology case - How to limit sub-carrier spacing for the Rel-15 WID |
|  |  | 2 | - Beam peak is used as a metric - Same as range 1 | - Same as range 1 |
|  | Occupied BW | 1 | - At least conductive test is needed. | - Occupied BW value |
|  |  | 2 | - TRP is used as a metric | - Same as range 1 - Necessity of EIRP |
|  | SEM | 1 | - At least conductive test is needed. - Assume different numerologies and RB allocations - NR UE shall meet the same SEM limit as that of LTE up to 20 MHz CBW. How to treat larger bandwidth than 20 MHz of NR is FFS. | - SEM value |
|  |  | 2 | - TRP is used as a metric - Same as range 1 | - Same as range 1 - Whether there is any justification not to follow the ITU response |
|  | ACLR | 1 | - At least conductive test is needed. - NR ACLR requirements for UTRA and E-UTRA are to be specified | - ACLR value for UTRA, E-UTRA and NR |
|  |  | 2 | - TRP is used as a metric - NR ACLR requirements for UTRA and E-UTRA are not to be specified | - Same as range 1 |
|  | General spurious | 1 | - At least conductive test is needed. - NR UE shall meet the same spurious limit as that of LTE. How to treat FOOB of larger bandwidth than 20 MHz of NR is FFS. | - General spurious value - Actual required level in mmWave should also be investigated from system point of view (sub-6GHz -> mmWave) |
|  |  | 2 | - TRP is used as a metric - For above 13 GHz transmission, upper frequency limits should be specified as 2nd harmonics of the upper edge of the UL operating band including the full harmonic spectrum. | - Same as range 1 - Whether there is any justification not to follow the ITU response - Feasibility of post PA filtering taking harmonics and other spurious levels into account - Actual required level in mmWave should also be investigated from system point of view (mmWave -> mmWave) - OOB boundary |
|  | Additional spurious | 1 | - At least conductive test is needed. - The same limits are reused in legacy victim bands in sub-6GHz | - How to treat NS applicable bands |
|  |  | 2 | - TRP is used as a metric | - Same as range 1 - Additional limit on top of the ITU response |
|  | UE-to-UE coexistence | 1 | - At least conductive test is needed. - The same limits are reused in legacy victim bands in sub-6GHz | - Actual required level in mmWave should also be investigated from system point of view (sub-6GHz -> mmWave) |
|  |  | 2 | - TRP is used as a metric - -50dBm/MHz (mmWave -> sub-6GHz) | - Actual required level in mmWave should also be investigated from system point of view. (mmWave -> mmWave) |
|  | Tx intermodulation | 1 | - At least conductive test is needed. | - Tx intermodulation value |
|  |  | 2 | - TRP is used as a metric with the blocker from the same direction of transmitted signal. | - Same as range 1 considering probability of being same direction when deciding blocker level |
|  | [New] Beam correspondence | 1 |  |  |
|  |  | 2 | - UE capability is introduced (RAN1 agreement) | - How to define Beam correspondence requirement |
| Rx | REFSENS | 1 | - At least conductive test is needed. | - REFSENS value - MSD impact in NSA of sub-6GHz and mmWave for both 1UL and 2UL. |
|  |  | 2 | - At least EIS is used as a metric - Develop different spatial coverage requirement. Smartphone (i.e. Full sphere) is the baseline of UE types in Rel-15. - For CDF method, RAN4 method for describing spherical coverage of RF parameters is CDF where each point represents equal surface area in sphere surrounding the UE. - To study the advantage of this CDF method. - The other method(s) are not precluded. | - Same as range 1 - How to categorise the UE type with different spatial coverage - Necessity of TRS |
|  | Maximum input level | 1 | - At least conductive test is needed. | - Maximum input level value and the test modulation order |
|  |  | 2 | - At least beam peak is used as a metric | - Same as range 1 |
|  | ACS | 1 | - At least conductive test is needed. | - ACS value |
|  |  | 2 | - Beam peak is used as a metric (to be further investigated) i.e. the blocker from the same direction of wanted signal. | - Same as range 1 considering probability of being same direction when deciding blocker level - Necessity of TRS |
|  | In-band blocking | 1 | - At least conductive test is needed. | - In-band blocking value |
|  |  | 2 | - Beam peak is used as a metric (to be further investigated) i.e. the blocker from the same direction of wanted signal. | - Same as range 1 considering probability of being same direction when deciding blocker level - Necessity of TRS |
|  | Out-of-band blocking | 1 | - At least conductive test is needed. | - Out-of-band blocking value |
|  |  | 2 | - Beam peak is used as a metric (to be further investigated) where OOB blocker is <±FFS% away from the center frequency of the wanted signal | - Blocker frequency offset value - Same as range 1 considering receiver tolerance and possibility of blocker in mmWave. - Necessity of TRS |
|  | Narrow-band blocking | 1 | - At least conductive test is needed. | - Narrow-band blocking value considering applicable bands |
|  |  | 2 |  | - Necessity of this requirement - Same as range 1 |
|  | Spurious response | 1 | - At least conductive test is needed. |  |
|  |  | 2 | - See Out-of-band blocking | - See Out-of-band blocking |
|  | Rx intermodulation | 1 | - At least conductive test is needed. | - Rx intermodulation value |
|  |  | 2 | - Beam peak is used as a metric i.e. the both blockers from the same direction of wanted signal. | - Same as range 1 considering probability of being same direction when deciding blocker level |
|  | Rx spurious emission | 1 | - At least conductive test is needed. | - Rx spurious emission value |
|  |  | 2 | - TRP is used as a metric - For above 13 GHz transmission, upper frequency limits should be specified as 2nd harmonics of the upper edge of the DL operating band including the full harmonic spectrum. | - Same as range 1 |
|  | Receiver image | 1 | - At least conductive test is needed. | - Receiver image value |
|  |  | 2 |  | - Receiver image metric - Same as range 1 |
|  | [New] In-channel selectivity | 1 | - At least conductive test is needed. - Definition in mixed numerology case should follow the same format as uplink, taking the possible power imbalance level between numerologies into consideration | - In-channel selectivity value for different numerologies (15 and 60 kHz SCS) |
|  |  | 2 | - Same as range 1 - Beam peak is used as a metric i.e. the blocker from the same direction of wanted signal. | - Same as range 1 |

<Unchanged sections omitted>