3GPP TSG-RAN WG4 Meeting #82bis	R4-1703990

Spokane, WA, USA, 3rd – 7th April, 2017

Agenda item:	10.4.3.2

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Further discussion on output power accuracy window for NR BS operating above 6 GHz

Document for:	Approval

# 1	Introduction

During the last RAN4 #NR ad hoc meeting, a way-forward on BS output power accuracy value for NR [1] was approved. It captured in [1] that for NR BS operating frequency below 6 GHz, both the conducted and OTA AAS BS output power requirements adopted. However, for frequency above 6 GHz, the following issue needs to be discussed [1]

- If OTA requirement is specified:

  - RAN4 needs to discuss how to decide the accuracy value. Interested companies encourage to present whether the same EIRP accuracy equation with AAS (shown in the next slide) can be reused or not in RAN4#82 meeting. If not, accuracy modelling method would also be provided.

In [2], we initiated discussion on the above issue and, in this contribution, the issue is further discussed with respect to hybrid beamforming architecture [3] which is being proposed for NR operating at frequency above 6 GHz. Then we conclude with our observations and proposals.

# 2	Discussion

It was captured in the way-forward on BS RF requirements for NR [4], the OTA output power accuracy requirement is specified as EIRP.

Now, one of the questions raised in [1] is whether the same EIRP accuracy equation for AAS is applicable to NR BS above 6 GHz. From TR 37.842, the EIRP accuracy is

[dB]																				(1)

where

– is the maximum conducted output power error at the transceiver unit output.								    (1a)

– is the variation in main beam EIRP due to beamforming errors caused by phase error at the transceiver unit output.																														    (1b)

– is the variation due to the error in the passive elements, the RDN, the antenna array gain errors, mismatch errors and insertion losses variations.																					    (1c)

Naturally, equation (1) is valid if NR BS has a similar architectural design and functionality as AAS BS depicted in Figure 1.

Radio

Distribution

Network

(RDN)

Transceiver Unit Array

(TRXUA)

Transceiver Unit #1

#1

TXU/RXU #2

TXU/RXU #NTRXU

#NTABC

Transceiver Array Boundary (TAB)

Antenna Array (AA)

TAB connector

Figure 1: General AAS BS radio architecture (from TR 37.842)

Presently, there is no NR BS architecture specified in 3GPP RAN4. According to [3], RAN1 agreed to study the following three implementation variants of beamforming for NR:

- Analog beamforming

- Digital beamforming

- Hybrid beamforming

For digital beamforming, the NR BS architecture is basically similar to the AAS BS as shown in Figure 1.

Observation 1: For NR BS implementing digital beamforming only, the EIRP accuracy equation for AAS BS can be adopted.

An example of a hybrid beamforming architecture, which is being proposed for mmWave NR BS, is shown in Figure 2. As compared with AAS BS, an analog beamforming block is introduced whereas the other blocks remain functionally equivalent to AAS. Thus, the EIRP accuracy equation of AAS can be adapted for the hybrid beamforming NR BS architecture by taking into account errors introduced by the analog beamforming. The EIRP accuracy of AAS BS can be used as a basis for deriving the EIRP accuracy requirements for hybrid beamforming NR BS.

Observation 2: the EIRP accuracy window of AAS BS can serve as a baseline for the hybrid beamforming NR BS, which is then adapted to derive the EIRP accuracy requirements for hybrid beamforming NR BS.

Digital Beamforming

(Baseband)

TRXU Array

.

.

.

TXU/RXU 2

TXU/RXU 1

Radio Distribution Network

(RDN)

Analog Beamforming

(phase shifters)

Antenna Array

(AA)

.

.

.

TXU/RXU #NTRXU

Figure 2: Example of hybrid beamforming architecture for mmWave NR base station

From equation (1), we can foresee that the analog beamforming mainly contributes to the beamsteering error factor  while the other error factors remain the same. This means, the beamsteering error factor, i.e., equation (1b), is composed of phase error due to the analogue phase shifters and transceiver unit outputs. The former affects the accuracy of analogue beamforming while the latter deals with phase coherency among RF signals generated by transceiver units. Thus, we can extend equation (1b) as

σsteer= σsteer-tx+ σsteer-abf 	[dB]																					(2)

where

σsteer-tx= phase error of RF signals generated by transceiver units

σsteer-abf = phase error due to phase shifters used by analogue beamforming

The definition of  is modified as follows:

– is the variation in main beam EIRP due to beamforming errors caused by phase error at the transceiver unit outputs and the analog phase shifters.

Observation 3: the beamsteering error factor is re-defined as the variation in main beam EIRP due to beamforming errors caused by phase error at the transceiver unit output and of the analogue phase shifters (i.e., σsteer= σsteer-tx+ σsteer-abf).

NR base stations above 6 GHz operate over a wide frequency spectrum, i.e., from 6 GHz to 100 GHz. Preliminary studies using a microwave analog signal generator [5] show that the output power level accuracy window increases with higher frequency bands as shown in Table 1.

| Frequency Bands | -10 dBm < Pout < 0 dBm | 0 dBm < Pout < +10 dBm | +10 dBm < Pout < +16 dBm |
| --- | --- | --- | --- |
| 6 GHz < f < 20 GHz | ±0.8 dB | ±0.8 dB | ±0.8 dB |
| 20 GHz < f < 40 GHz | ±0.9 dB | ±0.9 dB | ±1.0 dB |
| 40 GHz < f < 50 GHz | ±0.9 dB | ±1.3 dB | Not available |

Table 1: Output power accuracy window of an analog signal generator

Observation 4: further studies on the influence of different frequency bands on the EIRP accuracy window are needed.

According to [6], the EIRP accuracy window should capture the amplitude and phase error variations due to temperature fluctuations. By examining equation (1), the amplitude and phase error variations affect  and , respectively.

Observation 5: the amplitude and phase error variations due to temperature fluctuations influence  and .

# 3	Conclusions

In this document, we have further discussed the issue of NR BS output power accuracy. Based on our discussion, we can make the following observations:

Observation 1: For NR BS implementing digital beamforming only, the EIRP accuracy equation for AAS BS can be adopted.

Observation 2: the EIRP accuracy window of AAS BS can serve as a baseline for the hybrid beamforming NR BS, which is then adapted to derive the EIRP accuracy requirements for hybrid beamforming NR BS.

Observation 3: the beamsteering error factor is re-defined as the variation in main beam EIRP due to beamforming errors caused by phase error at the transceiver unit output and of the analog phase shifters (i.e., σsteer= σsteer-tx+ σsteer-abf).

Observation 4: further studies on the influence of different frequency bands on the EIRP accuracy window are needed.

Observation 5: the amplitude and phase error variations due to temperature fluctuations influence  and .

The aforementioned observations lead to the following proposals:

Proposal 1: assuming a hybrid beamforming architecture for mmWave NR base stations, then the EIRP accuracy equation of AAS BS is adopted and σsteer is adapted to reflect the steering error attributed to analogue beamforming as follows:

1(a):  σsteer= σsteer-tx+ σsteer-abf,

where σsteer-tx= phase error of RF signals generated by transceiver units

σsteer-abf = phase error due to analogue phase shifters used by analogue beamforming

1(b): the definition of σsteer is also modified as follows: the variation in main beam EIRP due to beamforming errors caused by phase error at the transceiver unit outputs and the analog phase shifters.

Proposal 2: the EIRP accuracy window (±2.2 dB) of AAS BS can serve as a baseline for the hybrid beamforming NR BS.

Proposal 3: the influence of different frequency bands (from 6 GHz up to 100 GHz) on the EIRP accuracy window is FFS.

Proposal 4: the amplitude and phase error variations due to temperature fluctuation should be included in  and .

# References

- R4-1700265, “WF on BS output power accuracy for NR”, NTT DOCOMO

- R4-1701761, Discussion on output power accuracy value for NR BS operating above 6 GHz, Nokia, Alcatel-Lucent Shanghai Bell

- R1-1700598, Hybrid beamforming for NR, NTT DOCOMO

- R4-1610923, Way forward on BS RF requirements for NR, Nokia, Alcatel-Lucent Shanghai Bell

- Keysight Technologies, E8257D PSG Analog signal generator

- R4-1700173, NR specific new requirements, NTT DOCOMO