3GPP TSG-RAN WG4 Meeting #82bis	R4-1703997

Spokane, WA, USA, 3rd – 7th April, 2017

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	OTA sensitivity

Agenda item:	9.12.2.4

Document for:	Approval

# 1	Introduction

During the RAN4 #82 meeting, a WF on OTA sensitivity [1] was approved. In the WF, the following proposals were captured and are to be approved in the forthcoming meetings:

Proposal 1:

Keep the REL13 Radiated OTA sensitivity requirement and declarations to satisfy the antenna performance parameters captured in that requirement.

Add a new requirement and set of declarations for minimum EIS which replaces the conducted reference sensitivity requirement.

Proposal 2:

- The RoAoA which defines the 3dB contour for the EIS is declared.

  - The 3dB RoAoA is contained within the RoAoA declaration for the REL13 Radiated OTA sensitivity requirement.

- An equation such as the Elliott formula (final choice is FFS) is used to estimate the D value. The equation shall be chosen in such a way that it will never overestimate the directivity. An off peak margin of 3dB is used.

Proposal 3: The L value for wide area BS is a fixed value of 2dB to cover all frequencies and antenna geometries.

Proposal 4: Contributions offering Antenna efficiency and L value for medium and local area AAS BS are encouraged for next meeting.

This document provides Nokia’s views on each of the aforementioned proposals.

# 2	Discussion

From [2], the minimum EIS requirement is calculated as

Minimum EIS=  Conducted reference sensitivity-D+L+ off-peak margin  	[dB]							(1)

where L is a loss factor accounting for antenna loss, distribution loss, integration loss, etc.

Conducted reference sensitivity is the conducted sensitivity level specified in TS 37.105 Rel-13

D is the estimated antenna directivity of a non-AAS BS which has a beam pattern related to the AAS BS range of angles of arrival (that is part of the OSDD declaration)

off-peak margin is to allow coverage for the RoAoA other than just in the peak direction, using the same estimated antenna pattern used to derive D

According to Proposal 2, the minimum antenna directivity D of a non-AAS BS within the range of angles of arrival is computed using an equation is FFS. Potential equations considered for approximating D are given in Table 1.

| Formulae | Assumptions |
| --- | --- |
| Kraus [3]: D=10 log10 41,253Θ1 . Θ2 [dB] (2a) | One narrow (or pencil) beam and negligible minor lobes |
| Elliot [4]: D=10 log10 32,400Θ1 . Θ2 [dB] (2b) | A refinement of equation (2a) for large planar antenna arrays; high estimation accuracy if Θ1 and Θ2 < 30° |
| 36000+[5]: = 10 log1036,000Θ1 . Θ2+0.3 [dB] (2c) | It was derived based on findings in [5] in which the antenna array model in TR 37.840 was used. According to [5], this equation produces the most accurate estimate of D |

Table 1: Formulae for directivity esitmates

In Table 1, Θ1 and Θ2 are half-power beamwidths in one plane and in a plane at right angle to the other, respectively; both are in degrees.

Even though equation (2b) is our preference because it can never result in overestimating D, but this should be validated by computing D from radiation patterns of AAS antennas using equation (3).

D= U(θ,  ϕ)max  14π   02π0πUθ,ϕsinθ dθ dϕ 																							(3)

where U(θ,ϕ)max = maximum radiation intensity

U(θ,ϕ)       = radiation intensity

For Proposal 3,  L=2.0 dB is an estimate based on passive antennas instead of active antenna system. Thus, losses attributed to phase shifters, diplexers and switches are not taken into account. As a result, we propose L=2.2 dB.

For Proposal 4, we propose L=1.8-2.0 dB for medium-range and local area AAS BS.

# 3	Conclusions

This document has provided our views on the proposals captured in [1], which are summarized below:

For Proposal 1:

Keep the REL13 Radiated OTA sensitivity requirement and declarations to satisfy the antenna performance parameters captured in that requirement.

Add a new requirement and set of declarations for minimum EIS which replaces the conducted reference sensitivity requirement.

We agree with this proposal.

For Proposal 2:  we prefer the Elliot equation because it never results in overestimating D. Nevertheless, in order to determine if equation produces the best estimate of D and does not result in overestimation, we propose to compare D that is computed from radiation patterns of AAS antennas using the actual directivity formula.

For Proposal 3: we propose L=2.2 dB taking losses attributed to phase shifters, diplexers and switches.

For Proposal 4: we propose L=1.8-2.0 dB for medium-range and local area AAS BS.

# References

- R4-1702271, Way Forward on OTA sensitivity, Huawei, NEC

- R4-1610807, WF on OTA sensitivity, NEC, Huawei, Ericsson

- J. D. Kraus, Antennas, 2nd Edition, McGraw-Hill, New York, 1988

- R. S. Elliot, Beamwidth and directivity of large scanning arrays, Microwave Journal, January 1964

- R4-1610388, Estimating antenna gain using 3dB beamwidth, Huawei