3GPP TSG-RAN WG4 Meeting #82bis	R4-1703998

Spokane, WA, USA, 3rd – 7th April, 2017

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Definitions of OTA emissions scaling terminology

Agenda item:	9.12.2.6

Document for:	Approval

# 1	Introduction

During the RAN4 #82 meeting, a WF on emissions scaling [1] was approved. One of the open issues captured in the WF is

b.  use the same scaling factor for both hybrid and OTA requirements (NTXU,counted) using the same scaling concept as in Rel-13 AAS but with minor modifications as suggested in the next slide

This document addresses the aforementioned open issue and concludes with our proposals.

# 2	Discussion

In the WF [1], three Rel-13 AAS emissions scaling terms, which need to be adapted for the OTA AAS BS architecture, were identified. Those terms and their definitions suggested in [1] are as follows:

| active transmitter unit | transmitter unit which is ON, and has the ability to send modulated data streams that are parallel and distinct to those sent from other transmitter units to be radiated in the far field |
| --- | --- |
| Ncells | A declared number of corresponding to the minimum number of cells that can be transmitted by an AAS BS in a particular band with transmission on all Transceiver units supporting the operating band. |
| TAB connector TX min cell group | operating band specific declared group of Transceiver units to which TX requirements are applied NOTE: Within this definition, the group corresponds to the group of Transceiver units which are responsible for transmitting a cell when the AAS BS corresponding to the declared minimum number of cells with transmission on all Transceiver units supporting an operating band, but its existence is not limited to that condition. |

Table 1: OTA emissions scaling definitions (from [1])

Based on the definitions in Table 1, we revise the definition as follows:

| active transmitter unit | transmitter unit which is ON, and has the ability to send modulated data streams that are parallel and distinct to those sent from other transmitter units to an antenna array for radiating in the far field |
| --- | --- |
| Ncells | As in Table 1 |
| Transceiver unit TX min cell group | operating band specific declared group of Transceiver units to which TX requirements are applied NOTE: Within this definition, the group corresponds to the group of Transceiver units which are responsible for transmitting a cell when the AAS BS corresponding to the declared minimum number of cells with transmission on all Transceiver units supporting an operating band, but its existence is not limited to that condition. |

Table 2: Proposed definitions

# 3	Conclusions

In this document, we have revised the definition of two emissions scaling terms and propose the following:

Proposal 1:

active transmitter unit: transmitter unit which is ON, and has the ability to send modulated data streams that are parallel and distinct to those sent from other transmitter units to an antenna array for radiating in the far field

Proposal 2:

Ncells 		A declared number of corresponding to the minimum number of cells that can be transmitted by an AAS BS in a particular band with transmission on all Transceiver units supporting the operating band.

Proposal 3:

Transceiver unit TX min cell group: operating band specific declared group of Transceiver units to which TX requirements are applied

NOTE: Within this definition, the group corresponds to the group of Transceiver units which are responsible for transmitting a cell when the AAS BS corresponding to the declared minimum number of cells with transmission on all Transceiver units supporting an operating band, but its existence is not limited to that condition.

# References

- R4-1702506, Way Forward on emissions scaling, Nokia, Alcatel-Lucent Shanghai Bell