3GPP TSG-RAN4 Meeting #82bis	R4-1704066

# Spokane, United States, 3rd Apr 2017 - 7th Apr 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.144 | CR | 0005 | rev | 1 | Current version: | 14.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Finalization of LTE Tablet TRP/TRS for bands 1, 3, 7, 19, 20, 21 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TELECOM ITALIA S.p.A. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_UTRA_TRP_TRS-Core |  |  |  |  |  | Date: |  |  | 2017-03-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-14 (Release 15) Rel-14 (Release 16) |  |
| Reason for change: |  | Introduction of TRP and TRS requirements for LTE tablet devices in Free Space test setup for following bands: 1, 3, 7, 19, 20, 21 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added references to TR 37.902 and TS 36.521-1. Frequency ranges for band 21 added to table 5.4-1. Caption of tables 5.4-1 and 5.5-1 revised in order to address bands for all EUTRA-FDD requirements. Added new tables 6.3.4-1 and B.2.2.4-1 for TRP and new tables 7.3.4-1 and B.3.3.4-1 for TRS requirements for LTE tablet devices, in sections 6.3.4, 7.3.4, B.2.2.4, B.3.3.4 respectively, reflecting approved average values in R4-1702252 and R4-1702524. Minimum values for TRP and maximum values for TRS are defined exploiting the same deltas used for UTRA-FDD tablet requirements, respectively 2 dB for low bands (<1GHz) and 1.5 dB for high bands (>1GHz). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TRP and TRS requirements for LTE tablet devices in Free Space test setup are not specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.4, 6.3.4, 7.3.4, B.2.2.4, B.3.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 37.544 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

##### ---Start of changes---

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

	For a specific reference, subsequent revisions do not apply.

	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 25 101: "User Equipment (UE) radio transmission and reception (FDD)".

[3]	3GPP TS 45.005: "Radio transmission and reception".

[4]	3GPP TS 34.114: "User Equipment (UE) / Mobile Station (MS) Over The Air (OTA) antenna performance; Conformance testing".

[5]	ETSI ETR 273: "Electromagnetic compatibility and Radio spectrum Matters (ERM); Improvement of radiated methods of measurement (using test sites) and evaluation of the corresponding measurement uncertainties; Part 1: Uncertainties in the measurement of mobile radio equipment characteristics; Sub-part 2: Examples and annexes".

[6]	3GPP TR 25.914: ”Measurements of radio performances for UMTS terminals in speech mode”

[7]	3GPP TR 37.977: “Verification of radiated multi-antenna reception performance of User Equipment (UE)”

[8]	3GPP TR 37.902: “Measurements of User Equipment (UE) radio performances for LTE/UMTS terminals; Total Radiated Power (TRP) and Total Radiated Sensitivity (TRS) test methodology”

[9]	3GPP TS 36.521-1: “User Equipment (UE) conformance specification Radio transmission and reception; Part 1: Conformance Testing;”

##### ---Text omitted---

## 5.4	E-UTRA FDD frequency bands

The requirements defined in this specification for E-UTRA FDD apply to the frequency bands defined in Table 5.4-1.

Table 5.4-1 E-UTRA FDD operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
| 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| 4 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz | FDD |
| 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894MHz | FDD |
| 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
| 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
| 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
| 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
| 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
| 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
| 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| 32 |  | N/A |  | 1452 MHz | – | 1496 MHz | FDD1 |
| NOTE 1: Restricted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. |  |  |  |  |  |  |  |

Deployment in other frequency bands is not precluded.

## 5.5	E-UTRA TDD frequency bands

The requirements defined in this specification for E-UTRA TDD apply to the frequency bands defined in Table 5.5-1.

Table 5.5-1 E-UTRA TDD operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
| 39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
| 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
| 41 | 2496 MHz |  | 2690 MHz | 2496 MHz |  | 2690 MHz | TDD |
| 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
| 46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD1,2 |
| NOTE 1: This band is an unlicensed band restricted to licensed-assisted operation using Frame Structure Type 3 NOTE 2: In this version of the specification, restricted to E-UTRA DL operation when carrier aggregation is configured. |  |  |  |  |  |  |  |

Deployment in other frequency bands is not precluded.

##### ---Text omitted---

### 6.3.4	E-UTRA FDD

LEE TRP minimum performance requirements for E-UTRA FDD are defined in Table 6.3.4-1 for Tablet devices.

Table 6.3.4-1: Tablet TRP minimum requirements for E-UTRA FDD in the data transfer position

| Operating band | Power Class 1 | Power Class 2 | Power Class 3 |  | Power Class 4 |
| --- | --- | --- | --- | --- | --- |
|  | Power (dBm) | Power (dBm) | Power (dBm) |  | Power (dBm) |
|  |  |  | Average | Min |  |
| 1 |  |  | 18.5 | 17.0 |  |
| 2 |  |  |  |  |  |
| 3 |  |  | 18.5 | 17.0 |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 7 |  |  | 18.0 | 16.5 |  |
| 8 |  |  |  |  |  |
| 12 |  |  |  |  |  |
| 13 |  |  |  |  |  |
| 19 |  |  | 17.5 | 15.5 |  |
| 20 |  |  | 17.5 | 15.5 |  |
| 21 |  |  | 17.5 | 16.0 |  |
| 28 |  |  |  |  |  |
| NOTE 1: Applicable for multi-mode GSM/UMTS/LTE. NOTE 2: Applicability for devices supporting CDMA or aggregated carriers (e.g. multi-carrier HSPA, LTE Carrier Aggregation) is FFS. |  |  |  |  |  |

##### ---Text omitted---

### 7.3.4	E-UTRA FDD

LEE TRS minimum performance requirements for E-UTRA FDD in data transfer position mode for throughput ≥ 95% of the maximum throughput of the reference measurement channels as specified in section 7.3 of [9] and Annex A of [9] are defined in Table 7.3.4-1 for Tablet devices.

Table 7.3.4-1: Tablet TRS minimum requirements for E-UTRA FDD in data transfer position

| Operating band | Channel bandwidth | Sensitivity (dBm) |  |
| --- | --- | --- | --- |
|  |  | Average | Max |
| 1 | 10 MHz | -93.75 | -92.25 |
| 2 | 10 MHz |  |  |
| 3 | 10 MHz | -95.0 | -93.5 |
| 4 | 10 MHz |  |  |
| 5 | 10 MHz |  |  |
| 7 | 10 MHz | -93.5 | -92.0 |
| 8 | 10 MHz |  |  |
| 12 | 10 MHz |  |  |
| 13 | 10 MHz |  |  |
| 19 | 10 MHz | -91.5 | -89.5 |
| 20 | 10 MHz | -92.5 | -90.5 |
| 21 | 15 MHz | -90.0 | -88.5 |
| 28 | 10 MHz |  |  |
| 32 | 10 MHz |  |  |
| NOTE 1: Applicable for multi-mode GSM/UMTS/LTE. NOTE 2: Applicability for devices supporting CDMA or aggregated carriers (e.g. multi-carrier HSPA, LTE Carrier Aggregation) is FFS. |  |  |  |

##### ---Text omitted---

### B.2.2.4	E-UTRA FDD

Table B.2.2.4-1: Tablet TRP recommended performance for E-UTRA FDD in the data transfer position

| Operating band | Power Class 1 | Power Class 2 | Power Class 3 | Power Class 4 |
| --- | --- | --- | --- | --- |
|  | Power (dBm) | Power (dBm) | Power (dBm) | Power (dBm) |
|  |  |  | Average |  |
| 1 |  |  | 21.0 |  |
| 2 |  |  |  |  |
| 3 |  |  | 20.5 |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 7 |  |  | 20.0 |  |
| 8 |  |  |  |  |
| 12 |  |  |  |  |
| 13 |  |  |  |  |
| 19 |  |  | 20.0 |  |
| 20 |  |  | 19.5 |  |
| 21 |  |  | 20.0 |  |
| 28 |  |  |  |  |
| NOTE 1: Applicable for multi-mode GSM/UMTS/LTE. NOTE 2: Applicability for devices supporting CDMA or aggregated carriers (e.g. multi-carrier HSPA, LTE Carrier Aggregation) is FFS. |  |  |  |  |

##### ---Text omitted---

### B.3.3.4	E-UTRA FDD

Table B.3.3.4-1: Tablet TRS recommended performance for E-UTRA FDD in data transfer position

| Operating band | Channel bandwidth | Sensitivity (dBm) |
| --- | --- | --- |
|  |  | Average |
| 1 | 10 MHz | -96.0 |
| 2 | 10 MHz |  |
| 3 | 10 MHz | -97.0 |
| 4 | 10 MHz |  |
| 5 | 10 MHz |  |
| 7 | 10 MHz | -95.75 |
| 8 | 10 MHz |  |
| 12 | 10 MHz |  |
| 13 | 10 MHz |  |
| 19 | 10 MHz | -94.5 |
| 20 | 10 MHz | -94.5 |
| 21 | 15 MHz | -93.0 |
| 28 | 10 MHz |  |
| 32 | 10 MHz |  |
| NOTE 1: Applicable for multi-mode GSM/UMTS/LTE. NOTE 2: Applicability for devices supporting CDMA or aggregated carriers (e.g. multi-carrier HSPA, LTE Carrier Aggregation) is FFS. |  |  |

##### ---End of changes---