3GPP TSG-RAN WG4 Meeting #82bis		R4-1704166

Spokane, USA, 3- 7 April 2017

Agenda item:	7.27.2.1

Source:	CATT

Title:	PDSCH demodulation performance of Enhanced CRS-IM

Document for:	Discussion

Introduction

In RAN #73 meeting, the “LTE Enhanced CRS and SU-MIMO Interference Mitigation Performance Requirements” WI was initiated [1]. In the previous RAN4#82 meeting, further agreements on the target scenarios and reference receivers for the Stage 1 investigations were provided in [2]. And it is proposed to provide simulation results to identify target scenarios for requirements definition and confirm reference receiver feasibility. In this contribution, we provide the simulation results based on the agreed simulation assumptions [3] for PDSCH on enhanced CRS-IM. The detailed simulation assumptions are attached in the Annex.

Simulation results of non colliding cases

In the way forward [3], the agreed PDSCH non colliding test cases are proposed as shown in Table 1.

Table 1.  PDSCH test cases of non colliding cases

| Test | TM | CRS pattern | Number of UE RX chains | Number of CRS APs |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Serv. cell | Interf. cell |
| Scenarios for performance requirements definition |  |  |  |  |  |
| 1 | TM4 | Non Colliding | 2 | 4 | 4 |
| 2 | TM4 | Non Colliding | 4 | 2 | 2 |
| 3 | TM9 | Non Colliding | 4 | 2 | 2 |
| 4 | TM4 | Non Colliding | 4 | 4 | 4 |

Rank + FRC:

| Test cases | TM4 |  |  | TM9 |
| --- | --- | --- | --- | --- |
|  | 2 CRS APs | 4 CRS APs |  | 2 CRS APs |
| Rank 1 + 64QAM | R.35 | R.36 | R.X | R.Y |

Additional MCS evaluation on Test Case 4

Interference signal transmission parameters:

80%/20% rank 1/2 probability

Option 1: Interference loading = 20%, INR1 = 10.45 dB, INR2 = 4.6 dB

Reference receivers are used for the evaluation:

- Receiver #1: LMMSE-IRC (baseline)

- Receiver #2: LMMSE-IRC + 1 Cell CRS-IM  (Full complexity two or four ports CRS-IM processing)

Test Case1: TM4  4 CRS APs + 2 RX chains

Figure 1. throughput performance of TM4  4 CRS APs + 2 RX chains scenario

Test Case2: TM4  2 CRS APs + 4 RX chains

Figure 2. throughput performance of TM4  2 CRS APs + 4 RX chains scenario

Test Case3: TM9  2 CRS APs + 4 RX chains

Figure 3. throughput performance of TM9  2 CRS APs + 4 RX chains scenario

Test Case4: TM4  4 CRS APs + 4 RX chains

FRC R.36 & FRC R.X:

Normalized throughput of different MCS for TC#4:

Figure 4. throughput performance of TM4  4 CRS APs + 4 RX chains scenario

From the above link-level simulation results of the non colliding scenarios, the performance gains of the LMMSE-IRC+CRS-IM receiver vs. the LMMSE-IRC receiver are shown in table 2.

Table 2. the performance gain of the SNRs  @ 70%  of  the maximum throughput

| Test Case (non colliding) | LMMSE-IRC(dB) | LMMSE-IRC+CRS-IM(dB) | Gains of CRS-IM(dB) |
| --- | --- | --- | --- |
| PDSCH Test1 | 13.6 | 11.9 | 1.7 |
| PDSCH Test2 | 10.41 | 8.85 | 1.56 |
| PDSCH Test3 | 10.78 | 9.62 | 1.16 |
| PDSCH Test4-1 | 9.2 | 6.0（SNR） or -4.825（SINR） | 3.2 |
| PDSCH Test4-2 | 10.4 | 7.9（SNR）or -2.925（SINR） | 2.5 |

Observation 1:

CRS-IM processing can achieve testable performance gain (about 1.5-3dB) over baseline receiver under various configurations of non colliding scenarios.

- Larger performance gain can be achieved from 4 CRS APs rather than 2CRS APs, which shows the necessity of using CRS-IM functionality in the 4CRS APs scenarios.

- For the evaluated scenario TM9 Rank 1 + 64QAM case, only about 1dB performance gain is observed from simulation, not feasible for a test purpose.

- For 4 CRS APs with 4Rx chains scenario, MCS21 can get a more practical SINR working point than MCS19.

Simulation results of colliding cases

In the way forward [3], the agreed PDSCH test cases are proposed as shown in Table 3

Table 3  PDSCH test cases

| Test | TM | CRS pattern | Number of UE RX chains | Number of CRS APs |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Serv. cell | Interf. cell |
| Scenarios for feasibility analysis |  |  |  |  |  |
| 5 | TM4 | Colliding | 2 | 4 | 4 |
| 6 | TM4 | Colliding | 2 | 4 | 2 |

Rank + FRC:

Rank 1 + 64QAM,  R.36

Interference signal transmission parameters:

80%/20% rank 1/2 probability

Option 1: Interference loading = 20%, INR1 = 10.45 dB, INR2 = 4.6 dB

Option 2: Interference loading = 0%, INR1 = 15.8 dB, INR2 = 10.5 dB

Option 3: Low INR values for robustness verification (INR1 = [4.7]dB, INR2=[1.4]dB, loading 20%)

The following reference receivers are used for test case#5:

- Receiver #1: LMMSE-IRC with 1 Cell CRS-IM to improve channel estimation

- Receiver #2: LMMSE-IRC with 1 Cell CRS-IM to improve channel and interference estimation

- Receiver #3: LMMSE-IRC (baseline)

The following reference receivers are used for test case#6:

- Receiver #1: LMMSE-IRC with covariance matrix estimation on serving cell APs 0-3 + 1 Cell CRS-IM

- Receiver #2: LMMSE-IRC with covariance matrix estimation on serving cell APs 2-3+ 1 Cell CRS-IM

- Receiver #3: LMMSE-IRC (baseline)

Test Case5: TM4  4 CRS APs + 2 RX chains

Figure 5. throughput performance of TM4  4 CRS APs + 2 RX chains scenario

Test Case6: TM4    4/2 CRS APs + 2 RX chains

Figure 6. throughput performance of TM4  4/2 CRS APs + 2 RX chains scenario

From the above simulation results of the colliding scenarios, the performance gain of the LMMSE-IRC+CRS-IM receiver to the LMMSE-IRC receiver is shown in table 3.

Table 4 the performance gain of the SNRs @70% of the maximum throughput

| Test Case (Colliding) | LMMSE-IRC(dB) | LMMSE-IRC+CRS-IM Receiver#1(dB) | LMMSE-IRC+CRS-IM Receiver#2(dB) | Gains of CRS-IM of Receiver#1(dB) | Gains of CRS-IM of Receiver#2(dB) |
| --- | --- | --- | --- | --- | --- |
| PDSCH Test5-1 | 13.4 | 12.5 | 11.9 | 0.9 | 1.5 |
| PDSCH Test5-2 | 15.3 | 13.8 | 12.6 | 1.5 | 2.7 |
| PDSCH Test5-3 | 10.7 | 10.3 | 10 | 0.4 | 0.7 |
| PDSCH Test6-1 | 12.35 | 10.61 | 10.7 | 1.74 | 1.65 |
| PDSCH Test6-2 | 14.1 | 10.24 | 10.41 | 3.86 | 3.69 |
| PDSCH Test6-3 | 10.11 | 9.44 | 9.55 | 0.67 | 0.56 |

Observation 2:

CRS-IM processing can achieve certain performance gain over baseline receiver of colliding scenarios.

- For Test Case 5, Receiver#2 can achieve a noticeable better performance than Receiver#1.

- For Test Case 6, Receiver#2 can provide almost no performance gain in all interference configurations which need further look into.

Conclusions

In this contribution, we provide our simulation results for PDSCH demodulation on enhanced CRS-IM. And according to the simulation results, the observations are given as below:

Observation 1:

CRS-IM processing can achieve testable performance gain (about 1.5-5dB) over baseline receiver under various configurations of non colliding scenarios.

- Larger performance gain can be achieved from 4 CRS APs rather than 2CRS APs, which shows the necessity of using the CRS-IM functionality in the 4CRS APs scenarios.

- For the evaluated scenario TM9 Rank 1 + 64QAM case, only about 1dB performance gain is observed from simulation, not feasible for a testable gain achievement.

- For 4 CRS APs with 4Rx chains scenario, MCS21 can get a more practical SINR working point than MCS19.

Observation 2:

CRS-IM processing can achieve certain performance gain over baseline receiver of colliding scenarios.

- For Test Case 5, Receiver#2 can achieve a better performance (>0.5dB) than Receiver#1.

- For Test Case 6, Receiver#2 can provide almost no performance gain in all interference configurations, further evaluation is needed.

# References

- RP-161877 “New Work Item Proposal: Enhanced CRS and SU-MIMO Interference Mitigation Performance Requirements for LTE”, Intel Corporation, Huawei, LGE, RAN #73, September 2016

- R4-1702195 “WF on Enhanced CRS-IM Performance Requirements”, Intel Corporation, LGE, Ericsson, RAN4 #82, February 2017

- R4-1702451 “Simulation assumptions for Enhanced CRS-IM”, Intel Corporation, LGE, RAN4 #82, February 2017

# Annex-simulation assumptions

Table 5 is the list of the test cases to be used for the initial investigations of the CRS-IM enhancements for PDSCH. The PDSCH simulation assumptions are provided in 5.

Table 5. Proposed PDSCH test cases

| Test | TM | CRS pattern | Number of UE RX chains | Number of CRS APs |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Serv. cell | Interf. cell |
| Scenarios for performance requirements definition |  |  |  |  |  |
| 1 | TM4 | Non Colliding | 2 | 4 | 4 |
| 2 | TM4 | Non Colliding | 4 | 2 | 2 |
| 3 | TM9 | Non Colliding | 4 | 2 | 2 |
| 4 | TM4 | Non Colliding | 4 | 4 | 4 |
| Scenarios for feasibility analysis |  |  |  |  |  |
| 5 | TM4 | Colliding | 2 | 4 | 4 |
| 6 | TM4 | Colliding | 2 | 4 | 2 |

Table 6. PDSCH simulation assumptions

| Parameter | Value |
| --- | --- |
| Number of explicitly modelled interference cells | Test 1,2,3,5,6: 2 Test 4: 1 |
| Cell ID pattern | Non-colliding CRS: 0/1/6 Colliding CRS: 0/6/1 |
| Interference signal transmission parameters | Rel-13 CRS-IM interference model (B.5.4) 80%/20% rank 1/2 probability Test 1,2,3,4: Option 1: Interference loading = 20%, INR1 = 10.45 dB, INR2 = 4.6 dB Test 5,6: Option 1: Interference loading = 20%, INR1 = 10.45 dB, INR2 = 4.6 dB Option 2: Interference loading = 0%, INR1 = 15.8 dB, INR2 = 10.5 dB Option 3: Low INR values for robustness verification (INR1 = [4.7]dB, INR2=[1.4]dB, loading 20%) |
| Time offset between cells | Cell 1: 3 µs Cell 2: -1 µs |
| Frequency offset between cells | Cell 1: 300 Hz Cell 2: -100 Hz |
| Channel model | EVA-5Hz for all links |
| Number of control OFDM symbols | 2 |
| HARQ modelling | Maximum 4 HARQ retransmissions |
| CSI-RS configuration for serving cell in TM9 test | Number of CSI-RS ports : 2 CSI-RS periodicity and subframe offset : 5/2 CSI reference signal configuration : 0 Zero-power CSI-RS configuration : 3 /0001000000000000 |
| Transmission parameters | PDSCH is scheduled in SFs 0-4, 6-9 Allocation: - TM4: 50 PRB resource allocation for all SFs - TM9: 50 PRB resource allocation for all SFs 1-4, 6-9; 41 PRB resource allocation for all SF 0 PMI - TM4: Reporting mode PUSCH 3-1 - TM9: Random PMI with 1 PRG / 1 TTI granularity Rank + MCS: Rank 1 + 64QAM |
| Receivers | Baseline: LMMSE-IRC Enhanced reference receiver structures: LMMSE-IRC + 1 cell CRS-IM Enhanced reference receivers for Test #5 - CRS-IM Receiver #1: LMMSE-IRC with 1 Cell CRS-IM to improve channel estimation - CRS-IM Receiver #2: LMMSE-IRC with 1 Cell CRS-IM to improve channel and interference estimation Enhanced reference receivers for Test #6 - CRS-IM Receiver #1: LMMSE-IRC with covariance matrix estimation on serving cell APs 0-3 + 1 Cell CRS-IM - CRS-IM Receiver #2: LMMSE-IRC with covariance matrix estimation on serving cell APs 2-3+ 1 Cell CRS-IM |

Table 7. FRC for TM4 PDSCH test

| TM4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameter | Unit | Value |  |  |
|  |  | 2 CRS APs | 4 CRS APs |  |
| Reference channel |  | R.35 | R.36 | R.X |
| Channel bandwidth | MHz | 10 | 10 | 10 |
| Allocated resource blocks |  | 50 | 50 | 50 |
| Allocated number of PDCCH symbols |  | 2 | 2 | 2 |
| Allocated subframes per Radio Frame |  | 9 | 9 | 9 |
| Modulation |  | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 0.5 | 0.5 | 0.55 |
| Information Bit Payload |  |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 19848 | 18336 | 21384 |
| For Sub-Frame 5 | Bits | n/a | n/a | n/a |
| For Sub-Frame 0 | Bits | 18336 | 18336 | 19848 |
| Number of Code Blocks |  |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 4 | 3 | 4 |
| For Sub-Frame 5 | Bits | n/a | n/a | n/a |
| For Sub-Frame 0 | Bits | 3 | 3 | 4 |
| Binary Channel Bits |  |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 39600 | 38400 | 38400 |
| For Sub-Frame 5 | Bits | n/a | n/a | n/a |
| For Sub-Frame 0 | Bits | 37152 | 36096 | 36096 |
| Max. Throughput averaged over 1 frame | Mbps | 17.712 | 16.502 | 19.092 |
| UE Category |  | ≥ 2 | ≥ 2 | ≥ 2 |
| UE DL Category |  |  |  |  |

Table 8. FRC for TM9 PDSCH test

| TM9 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameter | Unit | Value |  |  |
| Reference channel |  | R.Y |  |  |
| Channel bandwidth | MHz | 10 |  |  |
| Allocated resource blocks |  | 50 (Note 3) |  |  |
| Allocated subframes per Radio Frame |  | 9 |  |  |
| Modulation |  | 64QAM |  |  |
| Target Coding Rate |  | 0.51 |  |  |
| Information Bit Payload |  |  |  |  |
| For Sub-Frames 1,2,3,4 | Bits | 18336 |  |  |
| For Sub-Frames 6,7,8,9 | Bits | 18336 |  |  |
| For Sub-Frame 5 | Bits | n/a |  |  |
| For Sub-Frame 0 | Bits | 14688 |  |  |
| Number of Code Blocks |  |  |  |  |
| For Sub-Frames 1,2,3,4 | Bits | 3 |  |  |
| For Sub-Frames 6,7,8,9 | Bits | 3 |  |  |
| For Sub-Frame 5 | Bits | n/a |  |  |
| For Sub-Frame 0 | Bits | 3 |  |  |
| Binary Channel Bits |  |  |  |  |
| For Sub-Frames 1,4,6,9 | Bits | 36000 |  |  |
| For Sub-Frames 2,7 | Bits | 35400 |  |  |
| For Sub-Frames 3,8 | Bits | 34800 |  |  |
| For Sub-Frame 5 | Bits | n/a |  |  |
| For Sub-Frame 0 | Bits | 29520 |  |  |
| Max. Throughput averaged over 1 frame | Mbps | 16.138 |  |  |
| UE Category |  | ≥ 2 |  |  |
| UE DL Category |  |  |  |  |
| Note 1: 2 symbols allocated to PDCCH. |  |  |  |  |
| Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4]. |  |  |  |  |
| Note 3: 50 resource blocks are allocated in sub-frames 1, 2, 3, 4, 6, 7, 8, 9 and 41 resource blocks |  |  |  |  |
| (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0. |  |  |  |  |