3GPP TSG-RAN WG4 Meeting #82bis	                   	R4-1704331

Spokane, WA, US, 3 - 7 April, 2017

Source: 	Huawei

Title:	TP to TR 37.843: Capturing agreements on the OBUE requirement in OTA setup

Agenda Item:	9.12.2.8

Document for:	Approval

# Introduction

In this contribution we are proposing TP to TR 37.843 [1] capturing RAN4 agreements on the EUTRA OBUE in OTA setup, as agreed during RAN4#81 in [2].

RAN4 agreements on OBUE from [2] are captured below:

- Agreement: OBUE and Spurious emission (not co-location) should be defined as TRP metric.

- Agreement: OTA OBUE limit = Conducted OBUE limit – LO. Here, conducted OBUE limit is the Rel-13 AAS spec., LO is a loss factor

# References

TR 37.843, v. 0.2.0 	Active Antenna System (AAS) Base Station (BS) radiated requirements

R4-1610387		AAS WI: Agenda and meeting minutes for Monday evening ad hoc, RAN4#81

# TP to the TR 37.843

In this section, the TP to the TR 37.843 [1] is proposed. It is proposed to agree on the below TP to the TR 37.843.

- ------------------------------ Modified section ------------------------------

### 5.6.4 	Spectrum emission mask

#### 5.6.4.1	Background information on the conducted requirement

TBD

#### 5.6.4.2	Spectrum emission mask OTA requirement

TBD

- ------------------------------ Next modified section ------------------------------

### 5.6.5 	Operating band unwanted emission

#### 5.6.5.1 	Background information on the conducted requirement

Based on the conducted operating band unwanted emission (OBUE) requirement specified in TS 37.105 [3] for single RAT E-UTRA or for MSR operation, the OBUE limits are defined from 10 MHz below the lowest frequency of each supported downlink operating band to the lower Base Station RF Bandwidth edge located at FBW RF,low and from the upper Base Station RF Bandwidth edge located at FBW RF,high  up to 10 MHz above the highest frequency of each supported downlink operating band.

There is no operating band unwanted emission requirement for a single RAT UTRA FDD or single RAT UTRA TDD AAS BS.

#### 5.6.5.2	Operating band unwanted emissions OTA requirement

Using TRP as a figure of merit to provide equivalent OBUE performance and protection to that of the existing conducted requirement, the conducted requirements can be adopted by translating the test procedure and requirement from TS 36.145-1 [21] subclause 6.6.5 into the spatial domain.

In order that the requirement is measurable the total power OBUE is approximated as the sum of the EIRP at a number of discrete directions on the sphere around the DUT as follows:

,

where EIRPd is the filtered mean power within the desired signal channel bandwidth and p1 and p2 denotes two orthogonal polarizations.

The operating band unwanted emissions level for the OTA requirement can be derived as:

OTA OBUE limit = Conducted OBUE limit – LTX

Where:

  - Conducted OBUE limit is the AAS limit as defined in TS 37.105 [x] specification,

  - Value of the LTX is FFS.

- ------------------------------ End of modified sections ------------------------------