3GPP TSG RAN WG4 Meeting #83	R4-1704578

Hangzhou, China, 15-19 May 2017

# Source:		Rohde & Schwarz

# Title:	Analysis of Harmonization Results

# Document for:	Discussion

# Agenda Item:	7.11.3

# Introduction

This contribution presents an analysis of the harmonization results [1-3] based on the harmonization project plan [4] for the updated HS1 bands [5].

The author would like to thank LG, Samsung, and Google for preparing UEs with the ATF, GTS and Keysight for the support of the test campaign as well as Bluetest and EMITE for support on the harmonization project plan. Special thanks go to CATR for volunteering their labs, personnel, and time to support this harmonization campaign.

# Bands, Devices, and Number of Devices Tested

The HS1 bands were recently updated [5] to include

- TDD: 41, 38

- FDD Low: 5, 13

- FDD High: 3, 7

The devices used for the harmonization campaign are listed in Table 1.

Table 1: UEs used in the harmonization campaign

By the time this contribution was written, RTS and RC+CE methodologies completed different total number of bands as summarized in Table 2.

Table 2: Number of Bands Tested so far

|  | Number of Bands Tested |  |
| --- | --- | --- |
| Band | RTS | RC+CE |
| FDD7 | 8 | 8 |
| FDD3 | 8 | 8 |
| FDD13 | 8 | 6 |
| FDD5 | 7 | 7 |
| TDD38 | 0 | 3 |
| TDD41 | 0 | 3 |

FDD5 testing for RS_6 should be completed shortly so that this band has been tested with 8 devices total which is the current recommended minimum number of bands to judge harmonization [4].

It should be noted that none of the devices failed for any methodologies due to an excess of substitutions for not reaching target TP. The Excel spreadsheet outlines how many P_Mode values had to be substituted for each methodology.

# Cost of Harmonization and Assessed MUs

In the harmonization project plan [4], an updated cost of harmonization definition was proposed that is based on the MUs for the respective methodologies, MPAC, RTS, and RC+CE being assessed by the harmonization lab. The cost of harmonization is illustrated in Figure 1.

Figure 1: Definition of the cost of harmonization utilized in the harmonization analysis

The MU for the three methodologies assessed by CATR [6-8] for their respective systems is summarized in Table 3.

Table 3: Assessed MUs at CATR (harmonization lab)

|  | MPAC | RTS | RC+CE |
| --- | --- | --- | --- |
| Assessed MU, m [dB] | 1.91 | 1.54 | 1.85 |

In the harmonization project plan, it was agreed that the final target for the cost of harmonization is TBD but ≥1.5dB and ≤2dB for the full set of 30 devices. The analysis in this contribution is based on the tightest limit of 1.5dB although up to 2 dB may still be finally agreed. Based on the statistical significance for a limited set of devices for the harmonization campaign [4], it was agreed that harmonization per method is achieved if the cost of harmonization after testing 8 devices is less than or equal to 79% of the target cost of harmonization. Therefore, the target for harmonization based on testing of 8 devices is assumed to be 1.19dB.

# P_Mode Search Algorithms

During RAN4#82bis, non-linear behaviour of TP vs RS EPRE curves for some harmonization devices was pointed out [2]. Separate discussions over email were held following the meeting [9] but no clear conclusions were reached regarding the most suitable P_Mode approach, i.e., whether P_Mode should be defined as the sensitivity where TP reaches the target TP at the highest or lowest DL power. This analysis is presenting data based on the two discussed approaches:

- “lowest”: The lowest RS EPRE at which the raw data is crossing the target TP level with a downward slope is chosen to be P_Mode

- “highest”: The highest RS EPRE at which the raw data is crossing the target TP with a downward slope is chosen to be P_Mode

Figure 2: Definition of “highest” and “lowest” P_Mode

Another P_Mode search approach was chosen due to the differences in minimum recorded TP between the three methodologies:

- MPAC recorded TP down to 60% and 40% max theoretical TP

- RTS recorded TP down to 40% max theoretical TP

- RC+CE recorded TP down to 0% max theoretical TP

Measurement results for the RS_5 device for FDD7 are shown in Figure 3 to outline these differences for the three different methodologies.

(a)                                                                              (b)

(c)

Figure 3: Measurement results showing differences in minimum recorded TP: a) MPAC, b) RTS, c) RC+CE

The RTS results show that for some device rotations, non-linearities were observed after TP dropped below 60% TP. This yielded “lowest” P_Modes which MPAC was not able to pick up because the P_Mode search was stopped once the 60% TP was achieved. Therefore, the “lowest60” P_mode was introduced for this analysis given the differences in minimum recorded TP:

- “lowest60": To level the playing field, this approach only takes the data into account as if the system had stopped the “lowest” P_mode search as soon as TP reached 60%

This approach is illustrated schematically in Figure 4.

(a)

(b)

Figure 4: Illustration of the “lowest60” P_Mode search algorithm applied to the harmonization data. In a) no non-linearity occurs between 70% and 60% TP, while b) shows a non-linearity between 70% and 60% TP.

# Cost of Harmonization Results for “lowest60”

For the “lowest60” P_Mode, a complete set of results is presented here. The results for the other two P_Mode search algorithms can be studied using the Excel spreadsheet (by changing Cell H3 in worksheet ‘Cost of Harmonization’).

The S_Mode results per UE, the optimized offsets (one per band; one for both 70% and 95% target TP levels), and the residual errors after applying the offset are shown in Figure 5 for each of the HS1 bands. It should be noted that the offsets for RTS were chosen to be 0 to show equivalence; the residual errors could be reduced further if non-zero offsets were applied.

(a)

(b)

(c)

(d)

(e)

(f)

Figure 5: Results for S_Mode between MPAC, RTS, RC+CE. Additionally, the optimized offsets for RC+CE as well the residual errors are calculated for a) FDD7, b) FDD3, c) FDD13, d) FDD5, e) TDD38, and f) TDD41. The “lowest60” P_Mode search algorithm was applied for these results.

Together with the assessed MUs from Section 3, the cost of harmonization can be determined for the “lowest60” P_Mode search algorithm, shown in Table 4.

Table 4: Cost of Harmonization for “lowest60” P_Mode search algorithm

These results show that RTS harmonizes with MPAC for the four FDD bands for the reduced limit of 1.19 dB while the cost of harmonization for RC+CE exceeds the 1.5 dB maximum pass/fail limit which is for the upper limit of 30 devices in three bands (FDD7, FDD3, FDD13).

# Robustness Check

The results for the cost of harmonization and the respective optimized offsets are summarized in Figure 6 for the three different P_Mode search algorithms applied to the available harmonization data. For RC+CE, the offsets were optimized for each P_Mode search. Again, no offsets were applied to RTS even though they could further reduce the cost of harmonization.

(a)

(b)

Figure 6: Cost of harmonization and optimized offsets (for RC+CE) for the three different P_Mode search algoritms. Offsets were recalculated for each of the three P_Mode search algorithms. a) Results shown in tabular fashion separately for each band, b) graph of the maximum cost of harmonization for all bands

The results show that for each of the chosen P_Mode search algorithm, RTS harmonizes with MPAC with a maximum cost of harmonization just above 1dB.

Figure 7 shows the impact of cost when fixing the calculation of offsets for one of the P_Mode algorithms (“lowest”) which shows an almost insignificant increase in cost for the “highest” P_Mode algorithm for RC+CE.

(a)

(b)

Figure 7: Cost of harmonization and optimized offsets (for RC+CE) for the three different P_Mode search algoritms. Offsets were calculated once for the “lowest” P_Mode and kept for the other P_Modes. a) Results shown in tabular fashion separately for each band, b) graph of the maximum cost of harmonization for all bands

Additionally, for the “lowest60” P_Mode search algorithm the offset as a function of centre frequency of the tested channels was investigated. The results plotted in Figure 8 show that the offsets for RC+CE do not follow a linear trend which would make a determination of the optimized offsets as a function of frequency/frequency bands impossible. No justification for non-linear offsets with frequency has been proposed and selecting a best fit line would further increase the current cost of harmonzation.

Figure 7: Illustration of the optimized offset as a function of frequency

# Conclusions

Based on the available data, RTS shows it can harmonize with MPAC while RC+CE does not.

# References

- 3GPP R4-1700742, “MIMO OTA harmonization testing results (part 1)”, CATR, 3GPP RAN WG4 #82, February 2017.

- 3GPP R4-1703744, “MIMO OTA harmonization testing results (part 2)”, CATR, 3GPP RAN WG4 #82bis, April 2017.

- 3GPP R4-1704750 (draft shared over reflector), “MIMO OTA harmonization testing results (part 3)”, CATR, 3GPP RAN WG4 #83, May 2017

- R4-164975, “Harmonization Project Plan”, Rohde & Schwarz, Keysight Technologies, Bluetest, CTTC, 3GPP RAN WG4 #79, May 2016.

- R4-1703268, “Proposal to Replace Band 20 in Performance and Harmonization Campaigns with Band 5”, Rohde & Schwarz, Keysight Technologies, CTTC, Bluetest, 3GPP TSG RAN WG4 Meeting #82bis, April 2017.

- R4-1705191, “Measurement uncertainty of the RTS system for harmonization test campaign”, CATR, 3GPP RAN WG4 #83, May 2017

- R4-1705192 “Measurement uncertainty of the MPAC system for harmonization test campaign”, CATR, 3GPP RAN WG4 #83, May 2017

- R4-1705211 “Measurement uncertainty of the RC+CE system for harmonization test campaign”, CATR, 3GPP RAN WG4 #83, May 2017

- R4-1704657, “MIMO OTA email discussion”, Intel Corporation, 3GPP RAN WG4 #83, May 2017