3GPP TSG-RAN WG4 Meeting #83	 R4-1704771

Hangzhou, China, 15-19 May, 2017

Agenda Item:	10.3.1

Source: 	Samsung

Title: 	Consideration on Transmission BW configuration and CHBW in NR UE specification

Document for:	Discussion

Introduction

In RAN4#82bis meeting the RAN4 achieved the agreement on following aspects.

- Minimum channel bandwidth

- Maximum channel bandwidth

- Sub-carrier spacing

And corresponding agreements are summarized in tables below

Table 1: RAN4 agreement on channel bandwidth

| For Rel-15 | Below 6GHz | 6-52.6GHz |
| --- | --- | --- |
| Minimum CHBW | 5MHz | 50MHz |
| Maximum CHBW | 100MHz | 400MHz |

Table 2: RAN4 agreement on sub-carrier spacing on data

|  | SCS for Below 1GHz | SCS for 1GHz~6GHz | SCS for 24GHz-52.6GHz |
| --- | --- | --- | --- |
| data | 15KHz, 30KHz | 15kHz,30kHz, 60kHz | 60 kHz, 120kHz |

Based on above agreements, we share the consideration on how to define Transmission bandwidth configuration and CHBW set for NR bands in UE specification.

Discussion

Flexible channel bandwidth concept

One issue related to channel bandwidth definition is discussion on flexible channel bandwidth and UE requirement scalability accordingly. We can understand the motivation and potential benefit to define UE requirement with flexible manner. However, considering the challenging time line for Rel-15 NR, it is proposed to postpone RAN4 study on such kind of concept. Furthermore, it’s proposed to go directly with limited and fixed channel bandwidth set in first release of NR specification to simplify both standardization and implementation aspect.

Proposal on CBW set definition in NR specification

Several Operators shared initial proposal on channel bandwidth especially for below 6GHz [5][6][7]. Based on those suggestions, legacy LTE channel bandwidth and current agreement on minimum/maximum channel bandwidth, we propose that the following NR channel bandwidth candidates could be considered in Rel-15 phase. These candidate values below can be further down-selected. It’s a tradeoff between standardization/implementation complexity and deployment flexibility.

  - for below 6GHz: 5MHz, 10MHz, 15MHz, 20MHz, 40MHz, 50MHz, 60MHz 80MHz, 100MHz

  - for above 24GHz: 50MHz, 80MHz, 100MHz, 150MHz, 200MHz, 300MHz, 400MHz

And transmission bandwidth configuration could be defined for each frequency range with band agnostic way in specification just as LTE spec. For each NR band, it could be denoted whether specific channel bandwidth would be supported or not as well. RAN4 should study on following aspects:

- The number of PRB for specific combination of Sub-carrier spacing and channel bandwidth, which is relied on spectrum utilization discussion.

- Whether certain combination of Sub-carrier spacing and channel bandwidth would be supported in Rel-15 or not.

- The supported channel bandwidth set for specific NR band

The corresponding templates are shown in tables below. Frequency range on 28GHz, 39GHz, 3.5GHz and 2.6GHz are used as example in table 5 and table 6 for channel bandwidth set. These tables are just shown for initial discussion and it could be further optimized according to RAN4 discussion and operators’ request.

Table 3: Template on Transmission bandwidth configuration NRB for sub-6GHz

| Sub-carrier Spacing(KHz) | Transmission bandwidth configuration NRB/ Channel bandwidth BWchannel(MHz) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 |
| 15 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| 30 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| 60 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |

Note: TBD could be specific number or NA in this table.

Table 4: Template on Transmission bandwidth configuration NRB for 24-52.6GHz

| Sub-carrier Spacing(KHz) | Transmission bandwidth configuration NRB/ Channel bandwidth BWchannel(MHz) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 50 | 80 | 100 | 150 | 200 | 300 | 400 |
| 60 | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| 120 | TBD | TBD | TBD | TBD | TBD | TBD | TBD |

Note: TBD could be specific number or NA in this table.

Table 5: Template on NR Channel Bandwidth for sub-6GHz

| NR Band | SCS(kHz) | Channel bandwidth BWchannel(MHz) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 40 | 50 | 80 | 100 |
| 2.6GHz | 15 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | 30 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | 60 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| 3.5GHz | 15 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | 30 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | 60 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |

Note: TBD could be “Yes” or void in this table.

Table 6: Template on NR Channel Bandwidth for 24-52.6GHz

| NR Band | SCS(kHz) | Channel bandwidth BWchannel(MHz) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 50 | 80 | 100 | 150 | 200 | 300 | 400 |
| 26.5-29.5GHz | 60 | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | 120 | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| 39GHz | 60 | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | 120 | TBD | TBD | TBD | TBD | TBD | TBD | TBD |

Note: TBD could be “Yes” or void in this table.

Conclusion

This contribution provides the preliminary consideration on how to define transmission bandwidth configuration and   channel bandwidth in UE specification. It’s proposed to take template shown in table 3-6 into account in corresponding study. And further decide following aspects:

- The number of PRB for specific combination of Sub-carrier spacing and channel bandwidth, which is relied on spectrum utilization discussion.

- Whether certain combination of Sub-carrier spacing and channel bandwidth would be supported in Rel-15 or not.

- The supported channel bandwidth set for specific NR band

Reference

[1] R4-1703100, LS Reply on NR minimum carrier bandwidth

[2] R4-1704401, WF on Subcarrier Spacing for NR

[3] R4-1704223, LS on SCS for NR

[4] R4-1702091,WF on UE RF requreiemnts scalability for flexible channel bandwidth consideration, MTK

[4] R4-1700631, Proposed NR bandwidth set for UE RF specification for sub-6GHz, Deutsche Telekom, T-Mobile US

[5] R4-1701894, NR UE Tx/Rx bandwidth concept, Vodafone

[6] R4-1704000, NR channel bandwidths, Vodafone

[7] R4-1705240, Discussion on spectrum utilization, Samsung