3GPP TR 36.714-00-02 V0.8.0 (2017-05)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Networks;

LTE Advanced xDL/2UL Carrier Aggregation with x=3,4,5 (Release 14)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners’ Publications Offices.

Keywords

LTE-Radio

3GPP

Postal address

3GPP support office address

650 Route des Lucioles – Sophia Antipolis

Valbonne – FRANCE

Tel. : +33 4 92 94 42 00 Fax : +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2016, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	16

1	Scope	17

2	References	26

3	Definitions, symbols and abbreviations	27

3.1	Definitions	27

3.2	Symbols	27

3.3	Abbreviations	28

4	Background	28

5	xDLs/2UL Carrier Aggregation: General part	28

5.1	UE RF specific	28

5.1.1	Example UE RF architecture	28

5.1.2	General treatment of ∆TIB and ∆RIB values	29

5.1.3	Fallback CA mode and mandatory support of all paired 2UL CA configurations	35

5.1.4	General TX/RX requirements	35

5.1.4.1 Transmitter requirements	36

5.1.4.2 Receiver requirements	36

5.2	RRM specific	51

6	3DLs/2ULs Inter-Band Carrier Aggregation: Specific Band Combination Part	51

6.1	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 7 with 2 ULs	51

6.1.1	List of specific combination issues	51

6.1.1.1	Channel bandwidths per operating band for CA	51

6.1.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-7A	52

6.1.1.3	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-3A-7A	53

6.1.1.4	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_1A-3A-7A	54

6.1.1.5	MSD	55

6.1.1.6	∆TIB and ∆RIB values	55

6.2	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 40 with 2 ULs	55

6.2.1	List of specific combination issues	56

6.2.1.1	Channel bandwidths per operating band for CA	56

6.2.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-40A	56

6.2.1.3	MSD	57

6.2.1.4	∆TIB and ∆RIB values	57

6.3	LTE Advanced Carrier Aggregation: Band 1 and Band 5 and Band 40 with 2 ULs	57

6.3.1	List of specific combination issues	58

6.3.1.1	Channel bandwidths per operating band for CA	58

6.3.1.2	Co-existence studies for LTE-A UL CA_1A-5A and DL CA_1A-5A-40A	58

6.3.1.3	MSD	59

6.3.1.4	∆TIB and ∆RIB values	59

6.4	LTE Advanced Carrier Aggregation: Band 1 and Band 7 and Band 8 with 2 ULs	59

6.4.1	List of specific combination issues	60

6.4.1.1	Channel bandwidths per operating band for CA	60

6.4.1.2	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-7A-8A	60

6.4.1.3	Co-existence studies for LTE-A UL CA_1A-8A and DL CA_1A-7A-8A	61

6.4.1.4	MSD	62

6.4.1.5	∆TIB and ∆RIB values	62

6.5	LTE Advanced Carrier Aggregation: Band 1 and Band 8 and Band 40 with 2 ULs	62

6.5.1	List of specific combination issues	63

6.5.1.1	Channel bandwidths per operating band for CA	63

6.5.1.2	Co-existence studies for LTE-A UL CA_1A-8A and DL CA_1A-8A-40A	63

6.5.1.3	MSD	64

6.5.1.4	∆TIB and ∆RIB values	64

6.6	LTE Advanced Carrier Aggregation: Band 1 and Band 42 and Band 42 with 2 ULs	64

6.6.1	List of specific combination issues	64

6.6.1.1	Channel bandwidths per operating band for CA	64

6.6.1.2	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-42C	65

6.6.1.3	MSD	66

6.6.1.4	∆TIB and ∆RIB values	66

6.7	LTE Advanced Carrier Aggregation: Band 2 and Band 4 and Band 5 with 2 ULs	66

6.7.1	List of specific combination issues	67

6.7.1.1	Channel bandwidths per operating band for CA	67

6.7.1.2	Co-existence studies for LTE-A UL CA_2A-4A and DL CA_2A-4A-5A	67

6.7.1.3	MSD	68

6.7.1.4	∆TIB and ∆RIB values	68

6.8	LTE Advanced Carrier Aggregation: Band 2 and Band 4 and Band 29 with 2 ULs	68

6.8.1	List of specific combination issues	69

6.8.1.1	Channel bandwidths per operating band for CA	69

6.8.1.2	Co-existence studies for LTE-A UL CA_2A-4A and DL CA_2A-4A-29A	69

6.8.1.3	MSD	70

6.8.1.4	∆TIB and ∆RIB values	70

6.9	LTE Advanced Carrier Aggregation: Band 2 and Band 12 and Band 30 with 2 ULs	70

6.9.1	List of specific combination issues	70

6.9.1.1	Channel bandwidths per operating band for CA	70

6.9.1.2	Co-existence studies for LTE-A UL CA_2A-12A and DL CA_2A-12A-30A	71

6.9.1.3	MSD	72

6.9.1.4	∆TIB and ∆RIB values	72

6.10	LTE Advanced Carrier Aggregation: Band 2 and Band 12 and Band 12 with 2 ULs	72

6.10.1	List of specific combination issues	72

6.10.1.1	Channel bandwidths per operating band for CA	72

6.10.1.2	Co-existence studies for LTE-A UL CA_2A-12A and DL CA_2A-12B	73

6.10.1.3	MSD	74

6.10.1.4	∆TIB and ∆RIB values	74

6.11	LTE Advanced Carrier Aggregation: Band 3 and Band 5 and Band 40 with 2 ULs	74

6.11.1	List of specific combination issues	74

6.11.1.1	Channel bandwidths per operating band for CA	74

6.11.1.2	Co-existence studies for LTE-A UL CA_3A-5A and DL CA_3A-5A-40A	75

6.11.1.3	MSD	76

6.11.1.4	∆TIB and ∆RIB values	76

6.12	LTE Advanced Carrier Aggregation: Band 3 and Band 7 and Band 8 with 2 ULs	76

6.12.1	List of specific combination issues	76

6.12.1.1	Channel bandwidths per operating band for CA	76

6.12.1.2	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_3A-7A-8A	76

6.12.1.3	Co-existence studies for LTE-A UL CA_3A-8A and DL CA_3A-7A-8A	78

6.12.1.4	Co-existence studies for LTE-A UL CA_7A-8A and DL CA_3A-7A-8A	79

6.12.1.5	MSD	80

6.12.1.6	∆TIB and ∆RIB values	83

6.13	LTE Advanced Carrier Aggregation: Band 3 and Band 8 and Band 40 with 2 ULs	83

6.13.1	List of specific combination issues	83

6.13.1.1	Channel bandwidths per operating band for CA	83

6.13.1.2	Co-existence studies for LTE-A UL CA_3A-8A and DL CA_3A-8A-40A	83

6.13.1.3	MSD	85

6.13.1.4	∆TIB and ∆RIB values	85

6.14	LTE Advanced Carrier Aggregation: Band 3 and Band 42 and Band 42 with 2 ULs	85

6.14.1	List of specific combination issues	85

6.14.1.1	Channel bandwidths per operating band for CA	85

6.14.1.2	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_3A-42C	85

6.14.1.3	Co-existence studies for LTE-A UL CA_42C and DL CA_3A-42C	87

6.14.1.4	MSD	87

6.14.1.5	∆TIB and ∆RIB values	87

6.15	LTE Advanced Carrier Aggregation: Band 4 and Band 12 and Band 30 with 2 ULs	87

6.15.1	List of specific combination issues	88

6.15.1.1	Channel bandwidths per operating band for CA	88

6.15.1.2	Co-existence studies for LTE-A UL CA_4A-12A and DL CA_4A-12A-30A	88

6.15.1.3	MSD	89

6.15.1.4	∆TIB and ∆RIB values	89

6.16	LTE Advanced Carrier Aggregation: Band 4 and Band 12 and Band 12 with 2 ULs	89

6.16.1	List of specific combination issues	90

6.16.1.1	Channel bandwidths per operating band for CA	90

6.16.1.2	Co-existence studies for LTE-A UL CA_4A-12A and DL CA_4A-12B	90

6.16.1.3	MSD	91

6.16.1.4	∆TIB and ∆RIB values	91

6.17	LTE Advanced Carrier Aggregation: Band 7 and Band 7 and Band 28 with 2 ULs	91

6.17.1	List of specific combination issues	92

6.17.1.1	Channel bandwidths per operating band for CA	92

6.17.1.2	Co-existence studies for LTE-A UL CA_7A-28A and DL CA_7C-28A	92

6.17.1.3	Co-existence studies for LTE-A UL CA_7C and DL CA_7C-28A	93

6.17.1.4	MSD	94

6.17.1.5	∆TIB and ∆RIB values	94

6.18	LTE Advanced Carrier Aggregation: Band 19 and Band 42 and Band 42 with 2 ULs	94

6.18.1	List of specific combination issues	94

6.18.1.1	Channel bandwidths per operating band for CA	94

6.18.1.2	Co-existence studies for LTE-A UL CA_19A-42A and DL CA_19A-42C	95

6.18.1.3	MSD	96

6.18.1.4	∆TIB and ∆RIB values	96

6.19	LTE Advanced Carrier Aggregation: Band 21 and Band 42 and Band 42 with 2 ULs	96

6.19.1	List of specific combination issues	96

6.19.1.1	Channel bandwidths per operating band for CA	96

6.19.1.2	Co-existence studies for LTE-A UL CA_21A-42A and DL CA_21A-42C	97

6.19.1.3	MSD	98

6.19.1.4	∆TIB and ∆RIB values	98

6.20	LTE Advanced Carrier Aggregation: Band 3 and Band 3 and Band 8 with 2 ULs	98

6.20.1	List of specific combination issues	98

6.20.1.1	Channel bandwidths per operating band for CA	98

6.20.1.2	Co-existence studies for LTE-A UL CA_3A-8A and DL CA_3A-3A-8A	99

6.20.1.3	MSD	99

6.20.1.4	∆TIB and ∆RIB values	99

6.21	LTE Advanced Carrier Aggregation: Band 1 and Band 5 and Band 46 with 2 ULs	99

6.21.1	List of specific combination issues	99

6.21.1.1	Channel bandwidths per operating band for CA	99

6.21.1.2	Co-existence studies for LTE-A UL CA_1A-5A and DL CA_1A-5A-46A	99

6.21.1.3	MSD	102

6.21.1.4	∆TIB and ∆RIB values	102

6.22	LTE Advanced Carrier Aggregation: Band 1 and Band 7 and Band 46 with 2 ULs	102

6.22.1	List of specific combination issues	102

6.22.1.1	Channel bandwidths per operating band for CA	102

6.22.1.2	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-7A-46A	102

6.22.1.3	MSD	104

6.22.1.4	∆TIB and ∆RIB values	104

6.23	LTE Advanced Carrier Aggregation: Band 5 and Band 7 and Band 46 with 2 ULs	105

6.23.1	List of specific combination issues	105

6.23.1.1	Channel bandwidths per operating band for CA	105

6.23.1.2	Co-existence studies for LTE-A UL CA_5A-7A and DL CA_5A-7A-46A	105

6.23.1.3	MSD	107

6.23.1.4	∆TIB and ∆RIB values	107

6.24	LTE Advanced Carrier Aggregation: Band 3 and Band 5 and Band 7 with 2 ULs	107

6.24.1	List of specific combination issues	108

6.24.1.1	Channel bandwidths per operating band for CA	108

6.24.1.2	Co-existence studies for LTE-A UL CA_3A-5A and DL CA_3A-5A-7A	108

6.24.1.3	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_3A-5A-7A	109

6.24.1.4	Co-existence studies for LTE-A UL CA_5A-7A and DL CA_3A-5A-7A	110

6.24.1.5	MSD	111

6.24.1.6	∆TIB and ∆RIB values	112

6.25	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 21 with 2 ULs	112

6.25.1	List of specific combination issues	112

6.25.1.1	Channel bandwidths per operating band for CA	112

6.25.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-21A	112

6.25.1.3	Co-existence studies for LTE-A UL CA_1A-21A and DL CA_1A-3A-21A	114

6.25.1.4	Co-existence studies for LTE-A UL CA_3A-21A and DL CA_1A-3A-21A	115

6.25.1.5	MSD	116

6.25.1.6	∆TIB and ∆RIB values	116

6.26	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 42 with 2 ULs	116

6.26.1	List of specific combination issues	116

6.26.1.1	Channel bandwidths per operating band for CA	116

6.26.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-42A	117

6.26.1.3	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-3A-42A	118

6.26.1.4	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_1A-3A-42A	119

6.26.1.5	MSD	121

6.26.1.6	∆TIB and ∆RIB values	121

6.27	LTE Advanced Carrier Aggregation: Band 1 and Band 19 and Band 42 with 2 ULs	121

6.27.1	List of specific combination issues	121

6.27.1.1	Channel bandwidths per operating band for CA	121

6.27.1.2	Co-existence studies for LTE-A UL CA_1A-19A and DL CA_1A-19A-42A	121

6.27.1.3	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-19A-42A	123

6.27.1.4	Co-existence studies for LTE-A UL CA_19A-42A and DL CA_1A-19A-42A	124

6.27.1.5	MSD	125

6.27.1.6	∆TIB and ∆RIB values	125

6.28	LTE Advanced Carrier Aggregation: Band 1 and Band 21 and Band 42 with 2 ULs	125

6.28.1	List of specific combination issues	125

6.28.1.1	Channel bandwidths per operating band for CA	125

6.28.1.2	Co-existence studies for LTE-A UL CA_1A-21A and DL CA_1A-21A-42A	126

6.28.1.3	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-21A-42A	127

6.28.1.4	Co-existence studies for LTE-A UL CA_21A-42A and DL CA_1A-21A-42A	128

6.28.1.5	MSD	130

6.28.1.6	∆TIB and ∆RIB values	130

6.29	LTE Advanced Carrier Aggregation: Band 3 and Band 19 and Band 21 with 2 ULs	130

6.29.1	List of specific combination issues	130

6.29.1.1	Channel bandwidths per operating band for CA	130

6.29.1.2	Co-existence studies for LTE-A UL CA_3A-19A and DL CA_3A-19A-21A	130

6.29.1.3	Co-existence studies for LTE-A UL CA_3A-21A and DL CA_3A-19A-21A	131

6.29.1.4	Co-existence studies for LTE-A UL CA_19A-21A and DL CA_3A-19A-21A	133

6.29.1.5	MSD	134

6.29.1.6	∆TIB and ∆RIB values	134

6.30	LTE Advanced Carrier Aggregation: Band 3 and Band 19 and Band 42 with 2 ULs	134

6.30.1	List of specific combination issues	134

6.30.1.1	Channel bandwidths per operating band for CA	134

6.30.1.2	Co-existence studies for LTE-A UL CA_3A-19A and DL CA_3A-19A-42A	135

6.30.1.3	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_3A-19A-42A	136

6.30.1.4	Co-existence studies for LTE-A UL CA_19A-42A and DL CA_3A-19A-42A	137

6.30.1.5	MSD	138

6.30.1.6	∆TIB and ∆RIB values	139

6.31	LTE Advanced Carrier Aggregation: Band 3 and Band 21 and Band 42 with 2 ULs	139

6.31.1	List of specific combination issues	139

6.31.1.1	Channel bandwidths per operating band for CA	139

6.31.1.2	Co-existence studies for LTE-A UL CA_3A-21A and DL CA_3A-21A-42A	139

6.31.1.3	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_3A-21A-42A	140

6.31.1.4	Co-existence studies for LTE-A UL CA_21A-42A and DL CA_3A-21A-42A	142

6.31.1.5	MSD	143

6.31.1.6	∆TIB and ∆RIB values	143

6.32	LTE Advanced Carrier Aggregation: Band 19 and Band 21 and Band 42 with 2 ULs	143

6.32.1	List of specific combination issues	143

6.32.1.1	Channel bandwidths per operating band for CA	143

6.32.1.2	Co-existence studies for LTE-A UL CA_19A-21A and DL CA_19A-21A-42A	143

6.32.1.3	Co-existence studies for LTE-A UL CA_19A-42A and DL CA_19A-21A-42A	145

6.32.1.4	Co-existence studies for LTE-A UL CA_21A-42A and DL CA_19A-21A-42A	146

6.32.1.5	MSD	147

6.32.1.6	∆TIB and ∆RIB values	147

6.33	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 3 with 2 ULs	147

6.33.1	List of specific combination issues	148

6.33.1.1	Channel bandwidths per operating band for CA	148

6.33.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3C	148

6.33.1.3	Co-existence studies for LTE-A UL CA_3C and DL CA_1A-3C	149

6.33.1.4	MSD	150

6.33.1.5	∆TIB and ∆RIB values	150

6.34	LTE Advanced Carrier Aggregation: Band 3 and Band 3 and Band 8 with 2 ULs	150

6.34.1	List of specific combination issues	150

6.34.1.1	Channel bandwidths per operating band for CA	150

6.34.1.2	Co-existence studies for LTE-A UL CA_3A-8A and DL CA_3C-8A	151

6.34.1.3	Co-existence studies for LTE-A UL CA_3C and DL CA_3C-8A	152

6.34.1.4	MSD	152

6.34.1.5	∆TIB and ∆RIB values	152

6.35	LTE Advanced Carrier Aggregation: Band 41 and Band 42 and Band 42 with 2 ULs	152

6.35.1	List of specific combination issues	153

6.35.1.1	Channel bandwidths per operating band for CA	153

6.35.1.2	Co-existence studies for LTE-A UL CA_41A-42A and DL CA_41A-42C	153

6.35.1.3	Co-existence studies for LTE-A UL CA_42C and DL CA_41A-42C	154

6.35.1.4	MSD	155

6.35.1.5	∆TIB and ∆RIB values	155

6.36	LTE Advanced Carrier Aggregation: Band 41 and Band 41 and Band 42 with 2 ULs	155

6.36.1	List of specific combination issues	155

6.36.1.1	Channel bandwidths per operating band for CA	155

6.36.1.2	Co-existence studies for LTE-A UL CA_41A-42A and DL CA_41C-42A	155

6.36.1.3	Co-existence studies for LTE-A UL CA_41C and DL CA_41C-42A	157

6.36.1.4	MSD	157

6.36.1.5	∆TIB and ∆RIB values	157

6.37	LTE Advanced Carrier Aggregation: Band 2 and Band 4 and Band 7 with 2 ULs	157

6.37.1	List of specific combination issues	158

6.37.1.1	Channel bandwidths per operating band for CA	158

6.37.1.2	Co-existence studies for LTE-A UL CA_2A-4A and DL CA_2A-4A-7A	158

6.37.1.3	MSD	159

6.37.1.4	∆TIB and ∆RIB values	159

6.38	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 28 with 2 ULs	159

6.38.1	List of specific combination issues	160

6.38.1.1	Channel bandwidths per operating band for CA	160

6.38.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-28A	160

6.38.1.3	Co-existence studies for LTE-A UL CA_1A-28A and DL CA_1A-3A-28A	161

6.38.1.4	Co-existence studies for LTE-A UL CA_3A-28A and DL CA_1A-3A-28A	162

6.38.1.5	MSD	163

6.38.1.6	∆TIB and ∆RIB values	163

6.39	LTE Advanced Carrier Aggregation: Band 1 and Band 21 and Band 28 with 2 ULs	164

6.39.1	List of specific combination issues	164

6.39.1.1	Channel bandwidths per operating band for CA	164

6.39.1.2	Co-existence studies for LTE-A UL CA_1A-21A and DL CA_1A-21A-28A	164

6.39.1.3	Co-existence studies for LTE-A UL CA_1A-28A and DL CA_1A-21A-28A	165

6.39.1.4	Co-existence studies for LTE-A UL CA_21A-28A and DL CA_1A-21A-28A	166

6.39.1.5	MSD	168

6.39.1.6	∆TIB and ∆RIB values	168

6.40	LTE Advanced Carrier Aggregation: Band 1 and Band 28 and Band 42 with 2 ULs	168

6.40.1	List of specific combination issues	168

6.40.1.1	Channel bandwidths per operating band for CA	168

6.40.1.2	Co-existence studies for LTE-A UL CA_1A-28A and DL CA_1A-28A-42A	168

6.40.1.3	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-28A-42A	169

6.40.1.4	Co-existence studies for LTE-A UL CA_28A-42A and DL CA_1A-28A-42A	171

6.40.1.5	MSD	172

6.40.1.6	∆TIB and ∆RIB values	172

6.41	LTE Advanced Carrier Aggregation: Band 3 and Band 21 and Band 28 with 2 ULs	172

6.41.1	List of specific combination issues	172

6.41.1.1	Channel bandwidths per operating band for CA	172

6.41.1.2	Co-existence studies for LTE-A UL CA_3A-21A and DL CA_3A-21A-28A	173

6.41.1.3	Co-existence studies for LTE-A UL CA_3A-28A and DL CA_3A-21A-28A	174

6.41.1.4	Co-existence studies for LTE-A UL CA_21A-28A and DL CA_3A-21A-28A	175

6.41.1.5	MSD	176

6.41.1.6	∆TIB and ∆RIB values	176

6.42	LTE Advanced Carrier Aggregation: Band 3 and Band 28 and Band 42 with 2 ULs	176

6.42.1	List of specific combination issues	177

6.42.1.1	Channel bandwidths per operating band for CA	177

6.42.1.2	Co-existence studies for LTE-A UL CA_3A-28A and DL CA_3A-28A-42A	177

6.42.1.3	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_3A-28A-42A	178

6.42.1.4	Co-existence studies for LTE-A UL CA_28A-42A and DL CA_3A-28A-42A	179

6.42.1.5	MSD	180

6.42.1.6	∆TIB and ∆RIB values	181

6.43	LTE Advanced Carrier Aggregation: Band 21 and Band 28 and Band 42 with 2 ULs	181

6.43.1	List of specific combination issues	181

6.43.1.1	Channel bandwidths per operating band for CA	181

6.43.1.2	Co-existence studies for LTE-A UL CA_21A-28A and DL CA_21A-28A-42A	181

6.43.1.3	Co-existence studies for LTE-A UL CA_21A-42A and DL CA_21A-28A-42A	182

6.43.1.4	Co-existence studies for LTE-A UL CA_28A-42A and DL CA_21A-28A-42A	184

6.43.1.5	MSD	185

6.43.1.6	∆TIB and ∆RIB values	185

6.44	LTE Advanced Carrier Aggregation: Band 28 and Band 42 and Band 42 with 2 ULs	185

6.44.1	List of specific combination issues	185

6.44.1.1	Channel bandwidths per operating band for CA	185

6.44.1.2	Co-existence studies for LTE-A UL CA_28A-42A and DL CA_28A-42C	186

6.44.1.3	Co-existence studies for LTE-A UL CA_42C and DL CA_28A-42C	187

6.44.1.4	MSD	187

6.44.1.5	∆TIB and ∆RIB values	187

6.45	LTE Advanced Carrier Aggregation: Band 1 and Band 7 and Band 7 with 2 ULs	187

6.45.1	List of specific combination issues	188

6.45.1.1	Channel bandwidths per operating band for CA	188

6.45.1.2	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-7A-7A	188

6.45.1.3	MSD	189

6.45.1.4	∆TIB and ∆RIB values	189

6.46	LTE Advanced Carrier Aggregation: Band 3 and Band 7 and Band 7 with 2 ULs	190

6.46.1	List of specific combination issues	190

6.46.1.1	Channel bandwidths per operating band for CA	190

6.46.1.2	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_3A-7A-7A	190

6.46.1.3	MSD	191

6.46.1.4	∆TIB and ∆RIB values	192

6.47	LTE Advanced Carrier Aggregation: Band 5 and Band 7 and Band 7 with 2 ULs	192

6.47.1	List of specific combination issues	192

6.47.1.1	Channel bandwidths per operating band for CA	192

6.47.1.2	Co-existence studies for LTE-A UL CA_5A-7A and DL CA_5A-7A-7A	192

6.47.1.3	MSD	193

6.47.1.4	∆TIB and ∆RIB values	194

6.48	LTE Advanced Carrier Aggregation: Band 3 and Band 41 and Band 42 with 2 ULs	194

6.48.1	List of specific combination issues	194

6.48.1.1	Channel bandwidths per operating band for CA	194

6.48.1.2	Co-existence studies for LTE-A UL CA_41A-42A and DL CA_3A_41A-42A	194

6.48.1.3	MSD	195

6.48.1.4	∆TIB and ∆RIB values	196

6.49	LTE Advanced Carrier Aggregation: Band 28 and Band 41 and Band 42 with 2 ULs	196

6.49.1	List of specific combination issues	196

6.49.1.1	Channel bandwidths per operating band for CA	196

6.49.1.2	Co-existence studies for LTE-A UL CA_41A-42A and DL CA_28A_41A-42A	196

6.49.1.3	MSD	197

6.49.1.4	∆TIB and ∆RIB values	197

7	 4DLs/2ULs Inter-Band Carrier Aggregation: Specific Band Combination Part	198

7.1	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 7 and Band 8 with 2 ULs	198

7.1.1	List of specific combination issues	198

7.1.1.1	Channel bandwidths per operating band for CA	198

7.1.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-7A-8A	198

7.1.1.3	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-3A-7A-8A	199

7.1.1.4	Co-existence studies for LTE-A UL CA_1A-8A and DL CA_1A-3A-7A-8A	200

7.1.1.5	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_1A-3A-7A-8A	201

7.1.1.6	Co-existence studies for LTE-A UL CA_3A-8A and DL CA_1A-3A-7A-8A	202

7.1.1.7	MSD	204

7.1.1.8	∆TIB and ∆RIB values	204

7.2	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 5 and Band 40 with 2 ULs	204

7.2.1	List of specific combination issues	204

7.2.1.1	Channel bandwidths per operating band for CA	204

7.2.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-5A-40A	204

7.2.1.3	Co-existence studies for LTE-A UL CA_1A-5A and DL CA_1A-3A-5A-40A	206

7.2.1.4	Co-existence studies for LTE-A UL CA_3A-5A and DL CA_1A-3A-5A-40A	207

7.2.1.5	MSD	208

7.2.1.6	∆TIB and ∆RIB values	208

7.3	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 8 and Band 40 with 2 ULs	209

7.3.1	List of specific combination issues	209

7.3.1.1	Channel bandwidths per operating band for CA	209

7.3.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-8A-40A	209

7.3.1.3	Co-existence studies for LTE-A UL CA_1A-8A and DL CA_1A-3A-8A-40A	210

7.3.1.4	Co-existence studies for LTE-A UL CA_3A-8A and DL CA_1A-3A-8A-40A	212

7.3.1.5	MSD	213

7.3.1.6	∆TIB and ∆RIB values	213

7.4	LTE Advanced Carrier Aggregation: Band 2 and Band 4 and Band 5 and Band 29 with 2 ULs	213

7.4.1	List of specific combination issues	213

7.4.1.1	Channel bandwidths per operating band for CA	213

7.4.1.2	Co-existence studies for LTE-A UL CA_2A-4A and DL CA_2A-4A-5A-29A	214

7.4.1.3	MSD	215

7.4.1.4	∆TIB and ∆RIB values	215

7.5	LTE Advanced Carrier Aggregation: Band 3 and Band 7 and Band 7 and Band 28 with 2 ULs	215

7.5.1	List of specific combination issues	215

7.5.1.1	Channel bandwidths per operating band for CA	215

7.5.1.2	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_3A-7C-28A	216

7.5.1.3	Co-existence studies for LTE-A UL CA_7A-28A and DL CA_3A-7C-28A	217

7.5.1.4	Co-existence studies for LTE-A UL CA_7C and DL CA_3A-7C-28A	218

7.5.1.5	MSD	219

7.5.1.6	∆TIB and ∆RIB values	219

7.6	LTE Advanced Carrier Aggregation: Band 1 and Band 5 and Band 7 and Band 46 with 2 ULs	219

7.6.1	List of specific combination issues	219

7.6.1.1	Channel bandwidths per operating band for CA	219

7.6.1.2	Co-existence studies for LTE-A UL CA_1A-5A and DL CA_1A-5A-7A-46A	219

7.6.1.3	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-5A-7A-46A	221

7.6.1.4	Co-existence studies for LTE-A UL CA_5A-7A and DL CA_1A-5A-7A-46A	223

7.6.1.5	MSD	225

7.6.1.6	∆TIB and ∆RIB values	225

7.7	LTE Advanced Carrier Aggregation: Band 1 and Band 5 and Band 46 and Band 46 with 2 ULs	226

7.7.1	List of specific combination issues	226

7.7.1.1	Channel bandwidths per operating band for CA	226

7.7.1.2	Co-existence studies for LTE-A UL CA_1A-5A and DL CA_1A-5A-46C	226

7.7.1.3	 MSD	228

7.7.1.4	∆TIB and ∆RIB values	228

7.8	LTE Advanced Carrier Aggregation: Band 1 and Band 7 and Band 46 and Band 46 with 2 ULs	228

7.8.1	List of specific combination issues	229

7.8.1.1	Channel bandwidths per operating band for CA	229

7.8.1.2	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-7A-46C	229

7.8.1.3	 MSD	231

7.8.1.4	∆TIB and ∆RIB values	231

7.9	LTE Advanced Carrier Aggregation: Band 5 and Band 7 and Band 46 and Band 46 with 2 ULs	231

7.9.1	List of specific combination issues	231

7.9.1.1	Channel bandwidths per operating band for CA	231

7.9.1.2	Co-existence studies for LTE-A UL CA_5A-7A and DL CA_5A-7A-46C	232

7.9.1.3	 MSD	234

7.9.1.4	∆TIB and ∆RIB values	234

7.10	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 5 and Band 7 with 2 ULs	234

7.10.1	List of specific combination issues	234

7.10.1.1	Channel bandwidths per operating band for CA	234

7.10.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-5A-7A	234

7.10.1.3	Co-existence studies for LTE-A UL CA_1A-5A and DL CA_1A-3A-5A-7A	235

7.10.1.4	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-3A-5A-7A	236

7.10.1.5	Co-existence studies for LTE-A UL CA_3A-5A and DL CA_1A-5A-7A-46A	238

7.10.1.6	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_1A-3A-5A-7A	239

7.10.1.7	Co-existence studies for LTE-A UL CA_5A-7A and DL CA_1A-3A-5A-7A	240

7.10.1.8	MSD	241

7.10.1.9	∆TIB and ∆RIB values	241

7.11	LTE Advanced Carrier Aggregation: Band 1 and Band 19 and Band 21 and Band 42 with 2 ULs	241

7.11.1	List of specific combination issues	242

7.11.1.1	Channel bandwidths per operating band for CA	242

7.11.1.2	Co-existence studies for LTE-A UL CA_1A-19A and DL CA_1A-19A-21A-42A	242

7.11.1.3	Co-existence studies for LTE-A UL CA_1A-21A and DL CA_1A-19A-21A-42A	243

7.11.1.4	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-19A-21A-42A	244

7.11.1.5	Co-existence studies for LTE-A UL CA_19A-21A and DL CA_1A-19A-21A-42A	246

7.11.1.6	Co-existence studies for LTE-A UL CA_19A-42A and DL CA_1A-19A-21A-42A	247

7.11.1.7	Co-existence studies for LTE-A UL CA_21A-42A and DL CA_1A-19A-21A-42A	248

7.11.1.8	MSD	249

7.11.1.9	∆TIB and ∆RIB values	249

7.12	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 19 and Band 42 with 2 ULs	250

7.12.1	List of specific combination issues	250

7.12.1.1	Channel bandwidths per operating band for CA	250

7.12.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-19A-42A	250

7.12.1.3	Co-existence studies for LTE-A UL CA_1A-19A and DL CA_1A-3A-19A-42A	251

7.12.1.4	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-3A-19A-42A	252

7.12.1.5	Co-existence studies for LTE-A UL CA_3A-19A and DL CA_1A-3A-19A-42A	254

7.12.1.6	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_1A-3A-19A-42A	255

7.12.1.7	Co-existence studies for LTE-A UL CA_19A-42A and DL CA_1A-3A-19A-42A	256

7.12.1.8	MSD	257

7.12.1.9	∆TIB and ∆RIB values	258

7.13	LTE Advanced Carrier Aggregation: Band 1 and Band 19 and Band 42 and Band 42 with 2 ULs	258

7.13.1	List of specific combination issues	258

7.13.1.1	Channel bandwidths per operating band for CA	258

7.13.1.2	Co-existence studies for LTE-A UL CA_1A-19A and DL CA_1A-19A-42C	258

7.13.1.3	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-19A-42C	260

7.13.1.4	Co-existence studies for LTE-A UL CA_19A-42A and DL CA_1A-19A-42C	261

7.13.1.5	 MSD	262

7.13.1.6	∆TIB and ∆RIB values	262

7.14	LTE Advanced Carrier Aggregation: Band 1 and Band 21 and Band 42 and Band 42 with 2 ULs	262

7.14.1	List of specific combination issues	263

7.14.1.1	Channel bandwidths per operating band for CA	263

7.14.1.2	Co-existence studies for LTE-A UL CA_1A-21A and DL CA_1A-21A-42C	263

7.14.1.3	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-21A-42C	264

7.14.1.4	Co-existence studies for LTE-A UL CA_21A-42A and DL CA_1A-21A-42C	266

7.14.1.5	 MSD	267

7.14.1.6	∆TIB and ∆RIB values	267

7.15	LTE Advanced Carrier Aggregation: Band 19 and Band 21 and Band 42 and Band 42 with 2 ULs	267

7.15.1	List of specific combination issues	267

7.15.1.1	Channel bandwidths per operating band for CA	267

7.15.1.2	Co-existence studies for LTE-A UL CA_19A-21A and DL CA_19A-21A-42C	268

7.15.1.3	Co-existence studies for LTE-A UL CA_19A-42A and DL CA_19A-21A-42C	269

7.15.1.4	Co-existence studies for LTE-A UL CA_21A-42A and DL CA_19A-21A-42C	270

7.15.1.5	 MSD	271

7.15.1.6	∆TIB and ∆RIB values	271

7.16	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 42 and Band 42 with 2 ULs	272

7.16.1	List of specific combination issues	272

7.16.1.1	Channel bandwidths per operating band for CA	272

7.16.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-42C	272

7.16.1.3	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-3A-42C	274

7.16.1.4	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_1A-3A-42C	275

7.16.1.5	 MSD	276

7.16.1.6	∆TIB and ∆RIB values	277

7.17	LTE Advanced Carrier Aggregation: Band 3 and Band 19 and Band 42 and Band 42 with 2 ULs	278

7.17.1	List of specific combination issues	278

7.17.1.1	Channel bandwidths per operating band for CA	278

7.17.1.2	Co-existence studies for LTE-A UL CA_3A-19A and DL CA_3A-19A-42C	278

7.17.1.3	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_3A-19A-42C	279

7.17.1.4	Co-existence studies for LTE-A UL CA_19A-42A and DL CA_3A-19A-42C	281

7.17.1.5	 MSD	282

7.17.1.6	∆TIB and ∆RIB values	282

7.18	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 3 and Band 8 with 2 ULs	282

7.18.1	List of specific combination issues	282

7.18.1.1	Channel bandwidths per operating band for CA	282

7.18.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3C-8A	283

7.18.1.3	Co-existence studies for LTE-A UL CA_1A-8A and DL CA_1A-3C-8A	284

7.18.1.4	Co-existence studies for LTE-A UL CA_3A-8A and DL CA_1A-3C-8A	285

7.18.1.5	Co-existence studies for LTE-A UL CA_3C and DL CA_1A-3C-8A	286

7.18.1.6	 MSD	287

7.18.1.7	∆TIB and ∆RIB values	287

7.19	LTE Advanced Carrier Aggregation: Band 41 and Band 41 and Band 42 and Band 42 with 2 ULs	287

7.19.1	List of specific combination issues	287

7.19.1.1	Channel bandwidths per operating band for CA	287

7.19.1.2	Co-existence studies for LTE-A UL CA_41A-42A and DL CA_41C-42C	288

7.19.1.3	Co-existence studies for LTE-A UL CA_42C and DL CA_41C-42C	289

7.19.1.4	Co-existence studies for LTE-A UL CA_41C and DL CA_41C-42C	289

7.19.1.5	MSD	290

7.19.1.6	∆TIB and ∆RIB values	290

7.20	LTE Advanced Carrier Aggregation: Band 2 and Band 4 and Band 7 and Band 7 with 2 ULs	290

7.20.1	List of specific combination issues	290

7.20.1.1	Channel bandwidths per operating band for CA	290

7.20.1.2	Co-existence studies for LTE-A UL CA_2A-4A and DL CA_2A-4A-7A-7A	290

7.20.1.3	MSD	292

7.20.1.4	∆TIB and ∆RIB values	292

7.21	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 19 and Band 21 with 2 ULs	292

7.21.1	List of specific combination issues	292

7.21.1.1	Channel bandwidths per operating band for CA	292

7.21.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-19A-21A	292

7.21.1.3	Co-existence studies for LTE-A UL CA_1A-19A and DL CA_1A-3A-19A-21A	293

7.21.1.4	Co-existence studies for LTE-A UL CA_1A-21A and DL CA_1A-3A-19A-21A	295

7.21.1.5	Co-existence studies for LTE-A UL CA_3A-19A and DL CA_1A-3A-19A-21A	296

7.21.1.6	Co-existence studies for LTE-A UL CA_3A-21A and DL CA_1A-3A-19A-21A	297

7.21.1.7	Co-existence studies for LTE-A UL CA_19A-21A and DL CA_1A-3A-19A-21A	298

7.21.1.8	MSD	299

7.21.1.9	∆TIB and ∆RIB values	300

7.22	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 21 and Band 28 with 2 ULs	300

7.22.1	List of specific combination issues	300

7.22.1.1	Channel bandwidths per operating band for CA	300

7.22.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-21A-28A	300

7.22.1.3	Co-existence studies for LTE-A UL CA_1A-21A and DL CA_1A-3A-21A-28A	301

7.22.1.4	Co-existence studies for LTE-A UL CA_1A-28A and DL CA_1A-3A-21A-28A	303

7.22.1.5	Co-existence studies for LTE-A UL CA_3A-21A and DL CA_1A-3A-21A-28A	304

7.22.1.6	Co-existence studies for LTE-A UL CA_3A-28A and DL CA_1A-3A-21A-28A	305

7.22.1.7	Co-existence studies for LTE-A UL CA_21A-28A and DL CA_1A-3A-21A-28A	306

7.22.1.8	MSD	308

7.22.1.9	∆TIB and ∆RIB values	308

7.23	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 21 and Band 42 with 2 ULs	308

7.23.1	List of specific combination issues	308

7.23.1.1	Channel bandwidths per operating band for CA	308

7.23.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-21A-42A	309

7.23.1.3	Co-existence studies for LTE-A UL CA_1A-21A and DL CA_1A-3A-21A-42A	310

7.23.1.4	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-3A-21A-42A	311

7.23.1.5	Co-existence studies for LTE-A UL CA_3A-21A and DL CA_1A-3A-21A-42A	312

7.23.1.6	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_1A-3A-21A-42A	313

7.23.1.7	Co-existence studies for LTE-A UL CA_21A-42A and DL CA_1A-3A-21A-42A	315

7.23.1.8	MSD	316

7.23.1.9	∆TIB and ∆RIB values	316

7.24	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 28 and Band 42 with 2 ULs	316

7.24.1	List of specific combination issues	317

7.24.1.1	Channel bandwidths per operating band for CA	317

7.24.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-28A-42A	317

7.24.1.3	Co-existence studies for LTE-A UL CA_1A-28A and DL CA_1A-3A-28A-42A	318

7.24.1.4	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-3A-28A-42A	319

7.24.1.5	Co-existence studies for LTE-A UL CA_3A-28A and DL CA_1A-3A-28A-42A	320

7.24.1.6	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_1A-3A-28A-42A	322

7.24.1.7	Co-existence studies for LTE-A UL CA_28A-42A and DL CA_1A-3A-21A-42A	323

7.24.1.8	MSD	324

7.24.1.9	∆TIB and ∆RIB values	324

7.25	LTE Advanced Carrier Aggregation: Band 1 and Band 21 and Band 28 and Band 42 with 2 ULs	325

7.25.1	List of specific combination issues	325

7.25.1.1	Channel bandwidths per operating band for CA	325

7.25.1.2	Co-existence studies for LTE-A UL CA_1A-21A and DL CA_1A-21A-28A-42A	325

7.25.1.3	Co-existence studies for LTE-A UL CA_1A-28A and DL CA_1A-21A-28A-42A	326

7.25.1.4	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-21A-28A-42A	327

7.25.1.5	Co-existence studies for LTE-A UL CA_21A-28A and DL CA_1A-21A-28A-42A	329

7.25.1.6	Co-existence studies for LTE-A UL CA_21A-42A and DL CA_1A-21A-28A-42A	330

7.25.1.7	Co-existence studies for LTE-A UL CA_28A-42A and DL CA_1A-21A-28A-42A	331

7.25.1.8	MSD	332

7.25.1.9	∆TIB and ∆RIB values	333

7.26	LTE Advanced Carrier Aggregation: Band 1 and Band 28 and Band 42 and Band 42 with 2 ULs	333

7.26.1	List of specific combination issues	333

7.26.1.1	Channel bandwidths per operating band for CA	333

7.26.1.2	Co-existence studies for LTE-A UL CA_1A-28A and DL CA_1A-28A-42C	334

7.26.1.3	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-28A-42C	335

7.26.1.4	Co-existence studies for LTE-A UL CA_28A-42A and DL CA_1A-28A-42C	336

7.26.1.5	MSD	337

7.26.1.6	∆TIB and ∆RIB values	337

7.27	LTE Advanced Carrier Aggregation: Band 3 and Band 21 and Band 42 and Band 42 with 2 ULs	337

7.27.1	List of specific combination issues	338

7.27.1.1	Channel bandwidths per operating band for CA	338

7.27.1.2	Co-existence studies for LTE-A UL CA_3A-21A and DL CA_3A-21A-42C	338

7.27.1.3	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_3A-21A-42C	339

7.27.1.4	Co-existence studies for LTE-A UL CA_21A-42A and DL CA_3A-21A-42C	341

7.27.1.5	MSD	342

7.27.1.6	∆TIB and ∆RIB values	342

7.28	LTE Advanced Carrier Aggregation: Band 3 and Band 28 and Band 42 and Band 42 with 2 ULs	342

7.28.1	List of specific combination issues	342

7.28.1.1	Channel bandwidths per operating band for CA	342

7.28.1.2	Co-existence studies for LTE-A UL CA_3A-28A and DL CA_3A-28A-42C	343

7.28.1.3	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_3A-28A-42C	344

7.28.1.4	Co-existence studies for LTE-A UL CA_28A-42A and DL CA_3A-28A-42C	345

7.28.1.5	Co-existence studies for LTE-A UL CA_42C and DL CA_3A-28A-42C	346

7.28.1.6	MSD	347

7.28.1.7	∆TIB and ∆RIB values	347

7.29	LTE Advanced Carrier Aggregation: Band 21 and Band 28 and Band 42 and Band 42 with 2 ULs	347

7.29.1	List of specific combination issues	347

7.29.1.1	Channel bandwidths per operating band for CA	347

7.29.1.2	Co-existence studies for LTE-A UL CA_21A-28A and DL CA_21A-28A-42C	348

7.29.1.3	Co-existence studies for LTE-A UL CA_21A-42A and DL CA_21A-28A-42C	349

7.29.1.4	Co-existence studies for LTE-A UL CA_28A-42A and DL CA_21A-28A-42C	350

7.29.1.5	MSD	351

7.29.1.6	∆TIB and ∆RIB values	351

7.30	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 7 and Band 7 with 2 ULs	352

7.30.1	List of specific combination issues	352

7.30.1.1	Channel bandwidths per operating band for CA	352

7.30.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-7A-7A	352

7.30.1.3	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-3A-7A-7A	353

7.30.1.4	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_1A-3A-7A-7A	354

7.30.1.5	MSD	356

7.30.1.6	∆TIB and ∆RIB values	356

7.31	LTE Advanced Carrier Aggregation: Band 1 and Band 5 and Band 7 and Band 7 with 2 ULs	356

7.31.1	List of specific combination issues	356

7.31.1.1	Channel bandwidths per operating band for CA	356

7.31.1.2	Co-existence studies for LTE-A UL CA_1A-5A and DL CA_1A-5A-7A-7A	357

7.31.1.3	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-5A-7A-7A	358

7.31.1.4	Co-existence studies for LTE-A UL CA_5A-7A and DL CA_1A-5A-7A-7A	359

7.31.1.5	MSD	360

7.31.1.6	∆TIB and ∆RIB values	360

7.32	LTE Advanced Carrier Aggregation: Band 3 and Band 5 and Band 7 and Band 7 with 2 ULs	360

7.32.1	List of specific combination issues	361

7.32.1.1	Channel bandwidths per operating band for CA	361

7.32.1.2	Co-existence studies for LTE-A UL CA_3A-5A and DL CA_3A-5A-7A-7A	361

7.32.1.3	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_3A-5A-7A-7A	362

7.32.1.4	Co-existence studies for LTE-A UL CA_5A-7A and DL CA_3A-5A-7A-7A	364

7.32.1.5	MSD	365

7.32.1.6	∆TIB and ∆RIB values	365

7.33	LTE Advanced Carrier Aggregation: Band 3 and Band 3 and Band 7 and Band 7 with 2 ULs	365

7.33.1	List of specific combination issues	365

7.33.1.1	Channel bandwidths per operating band for CA	365

7.33.1.2	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_3C-7C	366

7.33.1.3	MSD	367

7.33.1.4	∆TIB and ∆RIB values	367

7.34	LTE Advanced Carrier Aggregation: Band 3 and Band 41 and Band 42 and Band 42 with 2 ULs	368

7.34.1	List of specific combination issues	368

7.34.1.1	Channel bandwidths per operating band for CA	368

7.34.1.2	Co-existence studies for LTE-A UL CA_41A-42A and DL CA_3A_41A-42C	368

7.34.1.3	Co-existence studies for LTE-A UL CA_42C and DL CA_3A-41A-42C	369

7.34.1.4	MSD	370

7.34.1.5	∆TIB and ∆RIB values	370

7.35	LTE Advanced Carrier Aggregation: Band 3 and Band 41 and Band 41 and Band 42 with 2 ULs	370

7.35.1	List of specific combination issues	370

7.35.1.1	Channel bandwidths per operating band for CA	370

7.35.1.2	Co-existence studies for LTE-A UL CA_41A-42A and DL CA_3A_41C-42A	371

7.35.1.3	 MSD	372

7.35.1.4	∆TIB and ∆RIB values	372

7.36	LTE Advanced Carrier Aggregation: Band 28 and Band 41 and Band 42 and Band 42 with 2 ULs	372

7.36.1	List of specific combination issues	372

7.36.1.1	Channel bandwidths per operating band for CA	372

7.36.1.2	Co-existence studies for LTE-A UL CA_41A-42A and DL CA_28A_41A-42C	373

7.36.1.3	Co-existence studies for LTE-A UL CA_42C and DL CA_28A-41A-42C	374

7.36.1.4	MSD	375

7.36.1.5	∆TIB and ∆RIB values	375

7.37	LTE Advanced Carrier Aggregation: Band 28 and Band 41 and Band 41 and Band 42 with 2 ULs	375

7.37.1	List of specific combination issues	375

7.37.1.1	Channel bandwidths per operating band for CA	375

7.37.1.2	Co-existence studies for LTE-A UL CA_41A-42A and DL CA_28A_41C-42A	376

7.37.1.3	 MSD	377

7.37.1.4	∆TIB and ∆RIB values	377

7.38	LTE Advanced Carrier Aggregation: Band 3 and Band 28 and Band 41 and Band 42 with 2 ULs	377

7.38.1	List of specific combination issues	377

7.38.1.1	Channel bandwidths per operating band for CA	377

7.38.1.2	Co-existence studies for LTE-A UL CA_41A-42A and DL CA_3A-28A-41A-42A	378

7.38.1.3	 MSD	379

7.38.1.4	∆TIB and ∆RIB values	379

8	 5DLs/2ULs Inter-Band Carrier Aggregation: Specific Band Combination Part	379

8.1	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 19 and Band 42 and Band 42 with 2 ULs	379

8.1.1	List of specific combination issues	379

8.1.1.1	Channel bandwidths per operating band for CA	379

8.1.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-19A-42A-42A	380

8.1.1.3	Co-existence studies for LTE-A UL CA_1A-19A and DL CA_1A-3A-19A-42A-42A	381

8.1.1.4	Co-existence studies for LTE-A UL CA_ 1A-42A and DL CA_1A-3A-19A-42A-42A	382

8.1.1.5	Co-existence studies for LTE-A UL CA_ 3A-19A and DL CA_1A-3A-19A-42A-42A	383

8.1.1.6	Co-existence studies for LTE-A UL CA_ 3A-42A and DL CA_1A-3A-19A-42A-42A	385

8.1.1.7	Co-existence studies for LTE-A UL CA_ 19A-42A and DL CA_1A-3A-19A-42A-42A	386

8.1.1.8	MSD	387

8.1.1.9	∆TIB and ∆RIB values	388

8.2	LTE Advanced Carrier Aggregation: Band 1 and Band 19 and Band 21 and Band 42 and Band 42 with 2 ULs	388

8.2.1	List of specific combination issues	388

8.2.1.1	Channel bandwidths per operating band for CA	388

8.2.1.2	Co-existence studies for LTE-A UL CA_1A-19A and DL CA_1A-19A-21A-42C	388

8.2.1.3	Co-existence studies for LTE-A UL CA_1A-21A and DL CA_1A-19A-21A-42C	390

8.2.1.4	Co-existence studies for LTE-A UL CA_ 1A-42A and DL CA_1A-19A-21A-42C	391

8.2.1.5	Co-existence studies for LTE-A UL CA_ 19A-21A and DL CA_1A-19A-21A-42C	392

8.2.1.6	Co-existence studies for LTE-A UL CA_ 19A-42A and DL CA_1A-19A-21A-42C	393

8.2.1.7	Co-existence studies for LTE-A UL CA_ 21A-42A and DL CA_1A-19A-21A-42C	394

8.2.1.8	MSD	396

8.2.1.9	∆TIB and ∆RIB values	396

8.3	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 21 and Band 42 and Band 42 with 2 ULs	397

8.3.1	List of specific combination issues	397

8.3.1.1	Channel bandwidths per operating band for CA	397

8.3.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-21A-42A-42A	397

8.3.1.3	Co-existence studies for LTE-A UL CA_1A-21A and DL CA_1A-3A-21A-42A-42A	398

8.3.1.4	Co-existence studies for LTE-A UL CA_ 1A-42A and DL CA_1A-3A-21A-42A-42A	400

8.3.1.5	Co-existence studies for LTE-A UL CA_ 3A-21A and DL CA_1A-3A-21A-42A-42A	401

8.3.1.6	Co-existence studies for LTE-A UL CA_ 3A-42A and DL CA_1A-3A-21A-42A-42A	402

8.3.1.7	Co-existence studies for LTE-A UL CA_21A-42A and DL CA_1A-3A-21A-42A-42A	403

8.3.1.8	MSD	404

8.3.1.9	∆TIB and ∆RIB values	405

8.4	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 28 and Band 42 and Band 42 with 2 ULs	405

8.4.1	List of specific combination issues	405

8.4.1.1	Channel bandwidths per operating band for CA	405

8.4.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-28A-42A-42A	406

8.4.1.3	Co-existence studies for LTE-A UL CA_1A-28A and DL CA_1A-3A-28A-42A-42A	407

8.4.1.4	Co-existence studies for LTE-A UL CA_ 1A-42A and DL CA_1A-3A-28A-42A-42A	408

8.4.1.5	Co-existence studies for LTE-A UL CA_ 3A-28A and DL CA_1A-3A-28A-42A-42A	409

8.4.1.6	Co-existence studies for LTE-A UL CA_ 3A-42A and DL CA_1A-3A-28A-42A-42A	410

8.4.1.7	Co-existence studies for LTE-A UL CA_28A-42A and DL CA_1A-3A-28A-42A-42A	412

8.4.1.8	MSD	413

8.4.1.9	∆TIB and ∆RIB values	413

8.5	LTE Advanced Carrier Aggregation: Band 1 and Band 21 and Band 28 and Band 42 and Band 42 with 2 ULs	413

8.5.1	List of specific combination issues	413

8.5.1.1	Channel bandwidths per operating band for CA	413

8.5.1.2	Co-existence studies for LTE-A UL CA_1A-21A and DL CA_1A-21A-28A-42A-42A	414

8.5.1.3	Co-existence studies for LTE-A UL CA_1A-28A and DL CA_1A-21A-28A-42A-42A	415

8.5.1.4	Co-existence studies for LTE-A UL CA_ 1A-42A and DL CA_1A-21A-28A-42A-42A	416

8.5.1.5	Co-existence studies for LTE-A UL CA_ 21A-28A and DL CA_1A-21A-28A-42A-42A	418

8.5.1.6	Co-existence studies for LTE-A UL CA_ 21A-42A and DL CA_1A-21A-28A-42A-42A	419

8.5.1.7	Co-existence studies for LTE-A UL CA_28A-42A and DL CA_1A-21A-28A-42A-42A	420

8.5.1.8	MSD	421

8.5.1.9	∆TIB and ∆RIB values	422

8.6	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 5 and Band 7 and Band 7 with 2 ULs	422

8.6.1	List of specific combination issues	422

8.6.1.1	Channel bandwidths per operating band for CA	422

8.6.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-5A-7A-7A	423

8.6.1.3	Co-existence studies for LTE-A UL CA_1A-5A and DL CA_1A-3A-5A-7A-7A	424

8.6.1.4	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-3A-5A-7A-7A	425

8.6.1.5	Co-existence studies for LTE-A UL CA_3A-5A and DL CA_1A-3A-5A-7A-7A	426

8.6.1.6	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_1A-3A-5A-7A-7A	427

8.6.1.7	Co-existence studies for LTE-A UL CA_5A-7A and DL CA_1A-3A-5A-7A-7A	429

8.6.1.8	MSD	430

8.6.1.9	∆TIB and ∆RIB values	430

Annex A:	431

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

Y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

1	Scope

The present document is a technical report for LTE Advanced 2ULs and xDLs Carrier Aggregation with x=3,4,5 under Rel-14 time frame. The purpose is to gather the relevant background information and studies in order to address 2ULs and xDLs Carrier Aggregation requirements.

This TR covers relevant background information and studies core & performance requirements for the Rel-14 band combinations in Table 1-1, 1-10 and 1-11.

Table 1-1: 3DL/2UL CA configurations part of the WI

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz |  | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3C | CA_1A-3A or CA_3C | 1 |  |  | Yes | Yes |  | Yes | Yes | 60 | 0 |
|  |  | 3 | See CA_3C BCS0 in Table 1-6 |  |  |  |  |  |  |  |  |
| CA_1A-3A-7A | CA_1A-3A or CA_1A-7A or CA_3A-7A | 1 |  |  | Yes | Yes |  | Yes | Yes | 60 | 0 |
|  |  | 3 |  |  | Yes | Yes |  | Yes | Yes |  |  |
|  |  | 7 |  |  |  | Yes |  | Yes | Yes |  |  |
| CA_1A-3A-21A | CA_1A-3A or CA_1A-21A or CA_3A-21A | 1 |  |  | Yes | Yes |  | Yes | Yes | 55 | 0 |
|  |  | 3 |  |  | Yes | Yes |  | Yes | Yes |  |  |
|  |  | 21 |  |  | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-28A | CA_1A-3A, CA_1A-28A, CA_3A-28A | 1 |  |  | Yes | Yes |  | Yes | Yes | 50 | 0 |
|  |  | 3 |  |  | Yes | Yes |  | Yes | Yes |  |  |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |  |
| CA_1A-3A-40A | CA_1A-3A | 1 |  |  | Yes | Yes |  | Yes | Yes | 60 | 0 |
|  |  | 3 |  |  | Yes | Yes |  | Yes | Yes |  |  |
|  |  | 40 |  |  | Yes | Yes |  | Yes | Yes |  |  |
| CA_1A-3A-42A | CA_1A-3A or CA_1A-42A or CA_3A-42A | 1 |  |  | Yes | Yes |  | Yes | Yes | 60 | 0 |
|  |  | 3 |  |  | Yes | Yes |  | Yes | Yes |  |  |
|  |  | 42 |  |  | Yes | Yes |  | Yes | Yes |  |  |
| CA_1A-5A-40A | CA_1A-5A | 1 |  |  | Yes | Yes |  | Yes | Yes | 50 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 40 |  |  |  | Yes |  | Yes | Yes |  |  |
| CA_1A-5A-46A | CA_1A-5A | 1 |  |  | Yes | Yes |  | Yes | Yes | 50 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 46 |  |  |  |  |  |  | Yes |  |  |
| CA_1A-7A-7A | CA_1A-7A | 1 |  |  | Yes | Yes |  | Yes | Yes | 60 | 0 |
|  |  | 7 | See CA_7A-7A BCS3 in Table 1-9 |  |  |  |  |  |  |  |  |
| CA_1A-7A-8A | CA_1A-7A or CA_1A-8A | 1 |  |  | Yes | Yes |  | Yes | Yes | 50 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes | Yes |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |  |
| CA_1A-7A-46A | CA_1A-7A | 1 |  |  | Yes | Yes |  | Yes | Yes | 60 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes | Yes |  |  |
|  |  | 46 |  |  |  |  |  |  | Yes |  |  |
| CA_1A-8A-40A | CA_1A-8A | 1 |  |  | Yes | Yes |  | Yes | Yes | 50 | 0 |
|  |  | 8 |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 40 |  |  | Yes | Yes |  | Yes | Yes |  |  |
| CA_1A-19A-42A | CA_1A-19A or CA_1A-42A or CA_19A-42A | 1 |  |  | Yes | Yes |  | Yes | Yes | 55 | 0 |
|  |  | 19 |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 42 |  |  | Yes | Yes |  | Yes | Yes |  |  |
| CA_1A-21A-28A | CA_1A-21A, CA_1A-28A, CA_21A-28A | 1 |  |  | Yes | Yes |  | Yes | Yes | 45 | 0 |
|  |  | 21 |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |  |
| CA_1A-21A-42A | CA_1A-21A or CA_1A-42A or CA_21A-42A | 1 |  |  | Yes | Yes |  | Yes | Yes | 55 | 0 |
|  |  | 21 |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 42 |  |  | Yes | Yes |  | Yes | Yes |  |  |
| CA_1A-28A-42A | CA_1A-28A, CA_1A-42A, CA_28A-42A | 1 |  |  | Yes | Yes |  | Yes | Yes | 50 | 0 |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 42 |  |  | Yes | Yes |  | Yes | Yes |  |  |
| CA_1A-42C | CA_1A-42A | 1 |  |  | Yes | Yes |  | Yes | Yes | 60 | 0 |
|  |  | CA_42C | See CA_42C BCS0 in Table 1-4 |  |  |  |  |  |  |  |  |
| CA_2A-4A-5A | CA_2A-4A | 2 |  |  | Yes | Yes |  | Yes | Yes | 50 | 0 |
|  |  | 4 |  |  | Yes | Yes |  | Yes | Yes |  |  |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |  |
| CA_2A-4A-7A | .CA_2A-4A | 2 |  |  | Yes | Yes |  | Yes | Yes | 60 | 0 |
|  |  | 4 |  |  | Yes | Yes |  | Yes | Yes |  |  |
|  |  | 7 |  |  | Yes | Yes |  | Yes | Yes |  |  |
| CA_2A-4A-29A | CA_2A-4A | 2 |  |  | Yes | Yes |  | Yes | Yes | 50 | 0 |
|  |  | 4 |  |  | Yes | Yes |  | Yes | Yes |  |  |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |  |
| CA_2A-12A-30A | CA_2A-12A | 2 |  |  | Yes | Yes |  | Yes | Yes | 40 | 0 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |  |
| CA_2A-12B | CA_2A-12A | 2 |  |  | Yes | Yes |  | Yes | Yes | 35 | 0 |
|  |  | CA_12B | See CA_12B BCS0 in Table 1-2 |  |  |  |  |  |  |  |  |
| CA_3A-3A-8A | CA_3A-8A | CA_3A-3A | See CA_3A-3A BCS0 in Table 1-5 |  |  |  |  |  |  | 50 | 0 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | CA_3A-3A | See CA_3A-3A BCS1 in Table 1-5 |  |  |  |  |  |  | 40 | 1 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |  |
| CA_3A-5A-7A | CA_3A-5A or CA_3A-7A or CA_5A-7A | 3 |  |  |  | Yes |  | Yes | Yes | 50 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 7 |  |  |  | Yes |  | Yes | Yes |  |  |
| CA_3A-5A-40A | CA_3A-5A | 3 |  |  | Yes | Yes |  | Yes | Yes | 50 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 40 |  |  |  | Yes |  | Yes | Yes |  |  |
| CA_3A-7A-7A | CA_3A-7A | 3 |  |  | Yes | Yes |  | Yes | Yes | 60 | 0 |
|  |  | 7 | See CA_7A-7A BCS3 in Table 1-9 |  |  |  |  |  |  |  |  |
| CA_3A-7A-8A | CA_3A-7A or CA_3A-8A or CA_7A-8A | 3 |  |  | Yes | Yes |  | Yes |  | 40 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 3 |  |  | Yes | Yes |  | Yes | Yes | 50 | 1 |
|  |  | 7 |  |  |  | Yes |  | Yes | Yes |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |  |
| CA_3C-8A | CA_3A-8A or CA-3C | 3 | See CA_3C BCS0 in Table 1-6 |  |  |  |  |  |  | 50 | 0 |
|  |  | 8 |  | Yes | Yes | Yes |  |  |  |  |  |
| CA_3A-8A-40A | CA_3A-8A | 3 |  |  | Yes | Yes |  | Yes | Yes | 50 | 0 |
|  |  | 8 |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 40 |  |  | Yes | Yes |  | Yes | Yes |  |  |
| CA_3A-19A-21A | CA_3A-19A or CA_3A-21A or CA_19A-21A | 3 |  |  | Yes | Yes |  | Yes | Yes | 50 | 0 |
|  |  | 19 |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 21 |  |  | Yes | Yes |  | Yes |  |  |  |
| CA_3A-19A-42A | CA_3A-19A or CA_3A-42A or CA_19A-42A | 3 |  |  | Yes | Yes |  | Yes | Yes | 55 | 0 |
|  |  | 19 |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 42 |  |  | Yes | Yes |  | Yes | Yes |  |  |
| CA_3A-21A-28A | CA_3A-21A, CA_3A-28A, CA_21A-28A | 3 |  |  | Yes | Yes |  | Yes | Yes | 45 | 0 |
|  |  | 21 |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |  |
| CA_3A-21A-42A | CA_3A-21A or CA_3A-42A or CA_21A-42A | 3 |  |  | Yes | Yes |  | Yes | Yes | 55 | 0 |
|  |  | 21 |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 42 |  |  | Yes | Yes |  | Yes | Yes |  |  |
| CA_3A-28A-42A | CA_3A-28A, CA_3A-42A, CA_28A-42A | 3 |  |  | Yes | Yes |  | Yes | Yes | 50 | 0 |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 42 |  |  | Yes | Yes |  | Yes | Yes |  |  |
| CA_3A-41A-42A | CA_41A-42A | 3 |  |  | Yes | Yes |  | Yes | Yes | 60 | 0 |
|  |  | 41 |  |  |  | Yes |  | Yes | Yes |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes | Yes |  |  |
| CA_3A-42C | CA_3A-42A, CA_42C | 3 |  |  | Yes | Yes |  | Yes | Yes | 60 | 0 |
|  |  | CA_42C | See CA_42C BCS0 in Table 1-4 |  |  |  |  |  |  |  |  |
| CA_4A-12A-30A | CA_4A-12A | 4 |  |  | Yes | Yes |  | Yes | Yes | 40 | 0 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |  |
| CA_4A-12B | CA_4A-12A | 4 |  |  | Yes | Yes |  | Yes | Yes | 35 | 0 |
|  |  | CA_12B | See CA_12B BCS0 in Table 1-2 |  |  |  |  |  |  |  |  |
| CA_5A-7A-7A | CA_5A-7A | 5 |  |  | Yes | Yes |  |  |  | 50 | 0 |
|  |  | 7 | See CA_7A-7A BCS3 in Table 1-9 |  |  |  |  |  |  |  |  |
| CA_5A-7A-46A | CA_5A-7A | 5 |  |  | Yes | Yes |  |  |  | 50 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes | Yes |  |  |
|  |  | 46 |  |  |  |  |  |  | Yes |  |  |
| CA_7C-28A | CA_7C or CA_7A-28A | 7 | See CA_7C BCS2 in Table 1-3 |  |  |  |  |  |  | 60 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes | Yes |  |  |
| CA_19A-21A-42A | CA_19A-21A or CA_19A-42A or CA_21A-42A | 19 |  |  | Yes | Yes |  | Yes |  | 50 | 0 |
|  |  | 21 |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 42 |  |  | Yes | Yes |  | Yes | Yes |  |  |
| CA_19A-42C | CA_19A-42A | 19 |  |  | Yes | Yes |  | Yes |  | 55 | 0 |
|  |  | CA_42C | See CA_42C BCS0 in Table 1-4 |  |  |  |  |  |  |  |  |
| CA_21A-42C | CA_21A-42A | 21 |  |  | Yes | Yes |  | Yes |  | 55 | 0 |
|  |  | CA_42C | See CA_42C BCS0 in Table 1-4 |  |  |  |  |  |  |  |  |
| CA_21A-28A-42A | CA_21A-28A, CA_21A-42A, CA_28A-42A | 21 |  |  | Yes | Yes |  | Yes |  | 45 | 0 |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 42 |  |  | Yes | Yes |  | Yes | Yes |  |  |
| CA_28A-41A-42A | CA_41A-42A | 28 |  |  | Yes | Yes |  |  |  | 50 | 0 |
|  |  | 41 |  |  |  | Yes |  | Yes | Yes |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes | Yes |  |  |
| CA_28A-42C | CA_28A-42A, CA_42C | 28 |  |  | Yes | Yes |  |  |  | 50 | 0 |
|  |  | 42 | See CA_42C BCS0 in Table 1-4 |  |  |  |  |  |  |  |  |
| CA_41A-42C | CA_41A-42A or CA_42C | 41 |  |  |  | Yes |  | Yes | Yes | 60 | 0 |
|  |  | 42 | See CA_42C BCS1 in Table 1-4 |  |  |  |  |  |  |  |  |
| CA_41C-42A | CA_41A-42A or CA_41C | 41 | See CA_41C BCS0 in Table 1-7 |  |  |  |  |  |  | 60 | 0 |
|  |  | 42 |  |  |  | Yes |  | Yes | Yes |  |  |
| CA_xA-yA-zA | CA_xA-yA or CA_yA-zA or CA_xA-zA | X |  |  |  |  |  |  |  | Up to 60 | 0 |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |  |  |  |

Table 1-2: E-UTRA CA_12B configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_12B | 5 | 5, 10 | 15 | 0 |

Table 1-3: E-UTRA CA_7C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_7C | CA_7C | 15 | 15 | 40 | 0 |
|  |  | 20 | 20 |  |  |
|  |  | 10 | 20 | 40 | 1 |
|  |  | 15 | 15, 20 |  |  |
|  |  | 20 | 10, 15, 20 |  |  |
|  |  | 15 | 10, 15 | 40 | 2 |
|  |  | 20 | 15, 20 |  |  |

Table 1-4: E-UTRA CA_42C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_42C | 5, 10, 15, 20 | 20 | 40 | 0 |
|  | 20 | 5, 10, 15 |  |  |
|  | 10, 15, 20 | 20 | 40 | 1 |
|  | 20 | 10, 15 |  |  |

Table 1-5: E-UTRA CA_3A-3A configurations and bandwidth combination sets defined for intra-band CA

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRACA configuration | Uplink CA configurations (NOTE 1) | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_3A-3A | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 40 | 0 |
|  |  | 5, 10 | 5, 10, 15, 20 |  |  | 30 | 1 |

Table 1-6: E-UTRA CA_3C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_3C | 5, 10, 15 | 20 | 40 | 0 |
|  | 20 | 5, 10, 15, 20 |  |  |

Table 1-7: E-UTRA CA_41C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_41C | 10 | 20 | 40 | 0 |
|  | 15 | 15, 20 |  |  |
|  | 20 | 10, 15, 20 |  |  |

Table 1-8: E-UTRA CA_46C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_46C | 20 | 20 | 40 | 0 |

Table 1-9: E-UTRA CA_7A-7A configurations and bandwidth combination sets defined for intra-band CA

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRACA configuration | Uplink CA configurations (NOTE 1) | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_7A-7A | - | 5 | 15 |  |  | 40 | 0 |
|  |  | 10 | 10, 15 |  |  |  |  |
|  |  | 15 | 15, 20 |  |  |  |  |
|  |  | 20 | 20 |  |  |  |  |
|  |  | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 40 | 1 |
|  |  | 5, 10, 15, 20 | 5, 10 |  |  | 30 | 2 |
|  |  | 10, 15, 20 | 10, 15, 20 |  |  | 40 | 3 |

Table 1-10: 4DL/2UL CA configurations part of the WI

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-5A-7A | CA_1A-3A or CA_1A-5A or CA_1A-7A or CA_3A-5A or CA_3A-7A or CA_5A-7A | 1 |  |  | Yes | Yes | Yes | Yes |  | 70 | 0 |
|  |  | 3 |  |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |  |
| CA_1A-3A-7A-7A | CA_1A-3A or CA_1A-7A or CA_3A-7A | 1 |  |  | Yes | Yes | Yes | Yes |  | 80 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |  |
|  |  | 7 | See CA_7A-7A BCS3 in Table 1-9 |  |  |  |  |  |  |  |  |
| CA_1A-3A-8A-40A | CA_1A-3A, CA_1A-8A, CA_3A-8A | 1 |  |  | Yes | Yes | Yes | Yes |  | 70 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |  |
|  |  | 8 |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes |  |  |  |
| CA_1A-3A-5A-40A | CA_1A-3A, CA_1A-5A, CA_3A-5A | 1 |  |  | Yes | Yes | Yes | Yes |  | 70 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |  |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 40 |  |  |  | Yes | Yes | Yes |  |  |  |
| CA_1A-3A-7A-8A | CA_1A-3A, CA_1A-7A, CA_1A-8A, CA_3A-7A, CA_3A-8A | 1 |  |  | Yes | Yes | Yes | Yes |  | 70 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |  |
| CA_1A-3C-8A | CA_1A-3A or CA_1A-8A or CA_3A-8A or CA_3C | 1 |  |  | Yes | Yes | Yes | Yes |  | 70 | 0 |
|  |  | 3 | See CA_3C BCS0 in Table 1-6 |  |  |  |  |  |  |  |  |
|  |  | 8 |  | Yes | Yes | Yes |  |  |  |  |  |
| CA_1A-3A-19A-21A | CA_1A-3A, CA_1A-19A, CA_1A-21A, CA_3A-19A, CA_3A-21A, CA_19A-21A | 1 |  |  | Yes | Yes | Yes | Yes |  | 70 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |  |
|  |  | 19 |  |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |  |
| CA_1A-3A-19A-42A | CA_1A-3A or CA_1A-19A or CA_1A-42A or CA_3A-19A or CA_3A-42A or CA_19A-42A | 1 |  |  | Yes | Yes | Yes | Yes |  | 75 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |  |
|  |  | 19 |  |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |  |
| CA_1A-3A-21A-28A | CA_1A-3A, CA_1A-21A, CA_1A-28A, CA_3A-21A, CA_3A-28A, CA_21A-28A | 1 |  |  | Yes | Yes | Yes | Yes |  | 65 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |  |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |  |
| CA_1A-3A-21A-42A | CA_1A-3A, CA_1A-21A, CA_1A-42A, CA_3A-21A, CA_3A-42A, CA_21A-42A | 1 |  |  | Yes | Yes | Yes | Yes |  | 75 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |  |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |  |
| CA_1A-3A-28A-42A | CA_1A-3A, CA_1A-28A, CA_1A-42A, CA_3A-28A, CA_3A-42A, CA_28A-42A | 1 |  |  | Yes | Yes | Yes | Yes |  | 70 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |  |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |  |
| CA_1A-3A-42C | CA_1A-3A or CA_1A-42A or CA_3A-42A | 1 |  |  | Yes | Yes | Yes | Yes |  | 80 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 1-4 |  |  |  |  |  |  |  |  |
| CA_1A-5A-7A-7A | CA_1A-5A or CA_1A-7A or CA_5A-7A | 1 |  |  | Yes | Yes | Yes |  | Yes | 70 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 7 | See CA_7A-7A BCS3 in Table 1-9 |  |  |  |  |  |  |  |  |
| CA_1A-5A-7A-46A | CA_1A-5A or CA_1A-7A or CA_5A-7A | 1 |  |  | Yes | Yes | Yes | Yes |  | 70 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 46 |  |  |  |  |  | Yes |  |  |  |
| CA_1A-5A-46C | CA_1A-5A | 1 |  |  | Yes | Yes | Yes | Yes |  | 70 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 46 | See the CA_46C BCS0 in Table 1-8 |  |  |  |  |  |  |  |  |
| CA_1A-7A-46C | CA_1A-7A | 1 |  |  | Yes | Yes | Yes | Yes |  | 80 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 46 | See the CA_46C BCS0 in Table 1-8 |  |  |  |  |  |  |  |  |
| CA_1A-19A-21A-42A | CA_1A-19A or CA_1A-21A or CA_1A-42A or CA_19A-21A or CA_19A-42A or CA_21A-42A | 1 |  |  | Yes | Yes | Yes | Yes |  | 70 | 0 |
|  |  | 19 |  |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |  |
| CA_1A-19A-42C | CA_1A-19A or CA_1A-42A or CA_19A-42A | 1 |  |  | Yes | Yes | Yes | Yes |  | 75 | 0 |
|  |  | 19 |  |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 1-4 |  |  |  |  |  |  |  |  |
| CA_1A-21A-28A-42A | CA_1A-21A, CA_1A-28A, CA_1A-42A, CA_21A-28A, CA_21A-42A, CA_28A-42A | 1 |  |  | Yes | Yes | Yes | Yes |  | 65 | 0 |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |  |
| CA_1A-21A-42C | CA_1A-21A or CA_1A-42A or CA_21A-42A | 1 |  |  | Yes | Yes | Yes | Yes |  | 75 | 0 |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 1-4 |  |  |  |  |  |  |  |  |
| CA_1A-28A-42C | CA_1A-28A, CA_1A-42A, CA_28A-42A | 1 |  |  | Yes | Yes | Yes | Yes |  | 70 | 0 |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 42 | See CA_42C BCS0 in Table 1-4 |  |  |  |  |  |  |  |  |
| CA_2A-4A-5A-29A | CA_2A-4A | 2 |  |  | Yes | Yes | Yes | Yes |  | 60 | 0 |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |  |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |  |
| CA_2A-4A-7A-7A | CA_2A-4A | 2 |  |  | Yes | Yes | Yes | Yes |  | 80 | 0 |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |  |
|  |  | 7 | See the CA_7A-7A BCS1 in Table 1-9 |  |  |  |  |  |  |  |  |
| CA_3A-5A-7A-7A | CA_3A-5A or CA_3A-7A or CA_5A-7A | 3 |  |  |  | Yes | Yes |  | Yes | 70 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 7 | See CA_7A-7A BCS3 in Table 1-9 |  |  |  |  |  |  |  |  |
| CA_3C-7C | CA_3A-7A | 3 | See CA_3C Bandwidth Combination Set 0 in Table 1-6 |  |  |  |  |  |  | 80 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 1-3 |  |  |  |  |  |  |  |  |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 1-6 |  |  |  |  |  |  | 80 | 1 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 1-3 |  |  |  |  |  |  |  |  |
| CA_3A-7C-28A | CA_3A-7A, CA_7C, CA_7A-28A | 3 |  |  |  | Yes | Yes | Yes |  | 80 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 1-3 |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes | Yes | Yes |  |  |  |
| CA_3A-19A-42C | CA_3A-19A or CA_3A-42A or CA_19A-42A | 3 |  |  | Yes | Yes | Yes | Yes |  | 75 | 0 |
|  |  | 19 |  |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 1-4 |  |  |  |  |  |  |  |  |
| CA_3A-21A-42C | CA_3A-21A, CA_3A-42A, CA_21A-42A | 3 |  |  | Yes | Yes | Yes | Yes |  | 75 | 0 |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 42 | See CA_42C BCS0 in Table 1-4 |  |  |  |  |  |  |  |  |
| CA_3A-28A-41A-42A | CA_41A-42A | 3 |  |  | Yes | Yes | Yes |  | Yes | 70 | 0 |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 41 |  |  |  | Yes | Yes |  | Yes |  |  |
|  |  | 42 |  |  |  | Yes | Yes |  | Yes |  |  |
| CA_3A-28A-42C | CA_3A-28A, CA_3A-42A, CA_28A-42A, CA_42C | 3 |  |  | Yes | Yes | Yes | Yes |  | 70 | 0 |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 42 | See CA_42C BCS0 in Table 1-4 |  |  |  |  |  |  |  |  |
| CA_3A-41A-42C | CA_42C, CA_41A-42A | 3 |  |  | Yes | Yes | Yes |  | Yes | 80 | 0 |
|  |  | 41 |  |  |  | Yes | Yes |  | Yes |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 0 in Table 1-4 |  |  |  |  |  |  |  |  |
| CA_3A-41C-42A | CA_41A-42A | 3 |  |  | Yes | Yes | Yes |  | Yes | 80 | 0 |
|  |  | 41 | See CA_41C Bandwidth combination set 0 in Table 1-7 |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes | Yes |  | Yes |  |  |
| CA_5A-7A-46C | CA_5A-7A | 5 |  |  | Yes | Yes |  |  |  | 70 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 46 | See the CA_46C BCS0 in Table 1-8 |  |  |  |  |  |  |  |  |
| CA_19A-21A-42C | CA_19A-21A or CA_19A-42A or CA_21A-42A | 19 |  |  | Yes | Yes | Yes |  |  | 70 | 0 |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 1-4 |  |  |  |  |  |  |  |  |
| CA_21A-28A-42C | CA_21A-28A, CA_21A-42A, CA_28A-42A | 21 |  |  | Yes | Yes | Yes |  |  | 65 | 0 |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 42 | See CA_42C BCS0 in Table 1-4 |  |  |  |  |  |  |  |  |
| CA_28A-41A-42C | CA_42C, CA_41A-42A | 28 |  |  | Yes | Yes |  |  |  | 70 | 0 |
|  |  | 41 |  |  |  | Yes | Yes |  | Yes |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 0 in Table 1-4 |  |  |  |  |  |  |  |  |
| CA_28A-41C-42A | CA_41A-42A | 28 |  |  | Yes | Yes |  |  |  | 70 | 0 |
|  |  | 41 | See CA_41C Bandwidth combination set 0 in Table 1-7 |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes | Yes |  | Yes |  |  |
| CA_41C-42C | CA_41A-42A or CA_42C or CA_41C | 41 | See CA_41C BCS0 in Table 1-7 |  |  |  |  |  |  | 80 | 0 |
|  |  | 42 | See CA_42C BCS1 in Table 1-4 |  |  |  |  |  |  |  |  |
| CA_wA-xA-yA-zA | CA_wA-xA, CA_wA-yA, CA_wA-zA, CA_xA-yA, CA_xA-zA, CA_yA-zA | W |  |  |  |  |  |  |  | Up to 80 |  |
|  |  | X |  |  |  |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |  |  |  |

Table 1-11: 5DL/2UL CA configurations part of the WI

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz |  | 5 MHz | 10 MHz | 15 MHz |  | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-5A-7A-7A | CA_1A-3A or CA_1A-5A or CA_1A-7A or CA_3A-5A or CA_3A-7A or CA_5A-7A | 1 |  |  |  | Yes | Yes | Yes |  | Yes | 90 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes |  | Yes |  |  |
|  |  | 5 |  |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 7 | See CA_7A-7A BCS3 in Table 1-9 |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-19A-42C | CA_1A-3A or CA_1A-19A or CA_1A-42A or CA_3A-19A or CA_3A-42A or CA_19A-42A | 1 |  |  |  | Yes | Yes | Yes |  | Yes | 95 | 0 |
|  |  | 3 |  |  |  | Yes | Yes | Yes |  | Yes |  |  |
|  |  | 19 |  |  |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 1-4 |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-21A-42C | CA_1A-3A, CA_1A-21A, CA_1A-42A, CA_3A-21A, CA_3A-42A, CA_21A-42A | 1 |  |  |  | Yes | Yes | Yes |  | Yes | 95 | 0 |
|  |  | 3 |  |  |  | Yes | Yes | Yes |  | Yes |  |  |
|  |  | 21 |  |  |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 42 | See CA_42C BCS0 in Table 1-4 |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-28A-42C | CA_1A-3A, CA_1A-28A, CA_1A-42A, CA_3A-28A, CA_3A-42A, CA_28A-42A | 1 |  |  |  | Yes | Yes | Yes |  | Yes | 90 | 0 |
|  |  | 3 |  |  |  | Yes | Yes | Yes |  | Yes |  |  |
|  |  | 28 |  |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 42 | See CA_42C BCS0 in Table 1-4 |  |  |  |  |  |  |  |  |  |
| CA_1A-19A-21A-42C | CA_1A-19A or CA_1A-21A or CA_1A-42A or CA_19A-21A or CA_19A-42A or CA_21A-42A | 1 |  |  |  | Yes | Yes | Yes |  | Yes | 90 | 0 |
|  |  | 19 |  |  |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 21 |  |  |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 1-4 |  |  |  |  |  |  |  |  |  |
| CA_1A-21A-28A-42C | CA_1A-21A, CA_1A-28A, CA_1A-42A, CA_21A-28A, CA_21A-42A, CA_28A-42A | 1 |  |  |  | Yes | Yes | Yes |  | Yes | 85 | 0 |
|  |  | 21 |  |  |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 28 |  |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 42 | See CA_42C BCS0 in Table 1-4 |  |  |  |  |  |  |  |  |  |
| CA_vA-wA-xA-yA-zA | CA_vA-wA or CA_vA-xA or CA_vA-yA or CA_vA-zA or CA_wA-xA or CA_wA-yA or CA_wA-zA or CA_xA-yA or CA_xA-zA or CA_yA-zA | V |  |  |  |  |  |  |  |  | Up to 100 | 0 |
|  |  | W |  |  |  |  |  |  |  |  |  |  |
|  |  | X |  |  |  |  |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |  |  |  |  |

This TR contains a general part and band specific combination part. The actual requirements are added to the corresponding technical specifications.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: “Vocabulary for 3GPP Specifications”.

[2]	3GPP TR 30.007: “Guideline on WI/SI for new Operating Bands”

[3]	3GPP TS 36.101: “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception”

[4]		RP-152246, “New WID: LTE Advanced inter-band CA Rel-14 for xDL/2UL with x=3,4,5”, LG Electronics”

[5]	R4-153754, “LS on Out-of-band blocking for 2UL CA”, NTT DoCoMo

[6] 	R4-168231, “WF on MSD due to IMD problems for xDL/2UL CA with Band 46”,   LG Electronics

[7]	R4-1702446, ““WF on the MSD test principles for xDL/2UL CA”, LG Electronics

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Carrier aggregation: Aggregation of two or more component carriers in order to support wider transmission bandwidths.

Channel bandwidth: The RF bandwidth supporting a single E-UTRA RF carrier with the transmission bandwidth configured in the uplink or downlink of a cell. The channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements.

Inter-band carrier aggregation: Carrier aggregation of component carriers in different operating bands.

Mixed Intra-band and inter-band carrier aggregation: Carrier aggregation of component carriers in same operating band and different operating bands.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

4	Background

The present document is a technical report for LTE Advanced 2UL and xDL Carrier Aggregation under Rel-14 time frame. It covers both the UE and BS side. The document is divided in two different parts:

- 	General part: this part covers BS and UE specific which is band combination independent.

- 	Specific band combination part:  this part covers each band combination and its specific issues independently from each other (i.e. one subclause is defined per band combination)

The specific band combination parts are independent and therefore, the working speed also differs

5	xDLs/2UL Carrier Aggregation: General part

5.1	UE RF specific

5.1.1	Example UE RF architecture

RAN4 consider xDL/1UL CA UE RF architectures including TDD-FDD CA based on the each CA band combinations.

Figure 5.1.1-1 UE example RF architecture using cascaded-Diplexer

Figure 5.1.1-1 is an example UE RF architecture using cascaded diplexer for 2UL/xDL inter-band CA. RAN4 do not preclude dual Antenna architecture for xDL/2UL CA UE. However, the UE RF requirements could be derived in the single antenna UE RF architecture.

5.1.2	General treatment of ∆TIB and ∆RIB values

Following Table 5.1.2-2 is proposed the additional insertion loss for xDL/2UL CA band combinations. We can used agreed additional insersion loss in xDL/1UL CA UE when the new RF components are not introduce for the xDL/2UL CA band combination.

Table 5.1.2-2: CA band combinations for xDL/2UL inter-band CA

| CA | DL Band Combination | UL Band Combination | Frequency | Additional ILs [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | TIB,c |  | RIB,c |  |
| 3DL/2UL Inter-band CA | CA_1A-3A-7A | CA_1A-3A or CA_1A-7A or CA_3A-7A | 2.1G+1.8G+2.6G | B1 | 0.6 | B1 | 0.0 |
|  |  |  |  | B3 | 0.6 | B3 | 0.0 |
|  |  |  |  | B7 | 0.6 | B7 | 0.0 |
|  | CA_1A-3A-40A | CA_1A-3A | 2.1G+1.8G+2.3G | B1 | 0.5 | B1 | 0.0 |
|  |  |  |  | B3 | 0.5 | B3 | 0.0 |
|  |  |  |  | B40 | 0.5 | B40 | 0.0 |
|  | CA_1A-5A-40A | CA_1A-5A | 2.1G+800M+2.3G | B1 | 0.5 | B1 | 0.0 |
|  |  |  |  | B5 | 0.3 | B5 | 0.0 |
|  |  |  |  | B40 | 0.5 | B40 | 0.0 |
|  | CA_1A-7A-8A | CA_1A-7A or CA_1A-8A | 2.1G+2.6G+900M | B1 | 0.5 | B1 | 0.0 |
|  |  |  |  | B7 | 0.6 | B7 | 0.0 |
|  |  |  |  | B8 | 0.6 | B8 | 0.2 |
|  | CA_1A-8A-40A | CA_1A-8A | 2.1G+900M+2.3G | B1 | 0.5 | B1 | 0.0 |
|  |  |  |  | B8 | 0.3 | B8 | 0.0 |
|  |  |  |  | B40 | 0.5 | B40 | 0.0 |
|  | CA_1A-42C | CA_1A-42A | 2.1G+3.5G | B1 | 0.3 | B1 | 0.0 |
|  |  |  |  | B42 | 0.8 | B42 | 0.5 |
|  | CA_2A-4A-5A | CA_2A-4A | 1.9G+2.1G+800M | B2 | 0.5 | B2 | 0.3 |
|  |  |  |  | B4 | 0.5 | B4 | 0.3 |
|  |  |  |  | B5 | 0.3 | B5 | 0.0 |
|  | CA_2A-4A-29A | CA_2A-4A | 1..9G+2.1G+700M | B2 | 0.5 | B2 | 0.4 |
|  |  |  |  | B4 | 0.5 | B4 | 0.4 |
|  |  |  |  | B29 | N/A | B29 | 0.0 |
|  | CA_2A-12A-30A | CA_2A-12A | 1.9G+700M+2.3G | B2 | 0.5 | B2 | 0.4 |
|  |  |  |  | B12 | 0.3 | B12 | 0.0 |
|  |  |  |  | B30 | 0.3 | B30 | 0.5 |
|  | CA_2A-12B | CA_2A-12A | 1.9G+700M | B2 | 0.3 | B2 | 0.0 |
|  |  |  |  | B12 | 0.3 | B12 | 0.0 |
|  | CA_3A-3A-8A | CA_3A-8A | 1.8G+900M | B3 | 0.3 | B3 | 0.0 |
|  |  |  |  | B8 | 0.3 | B8 | 0.0 |
|  | CA_3A-5A-40A | CA_3A-5A | 1.8G+2.6G+700M | B3 | 0.5 | B3 | 0.0 |
|  |  |  |  | B5 | 0.3 | B5 | 0.0 |
|  |  |  |  | B40 | 0.5 | B40 | 0.0 |
|  | CA_3A-7A-8A | CA_3A-7A or CA_3A-8A or CA_7A-8A | 1.8G+2.6G+900M | B3 | 0.5 | B3 | 0.0 |
|  |  |  |  | B7 | 0.5 | B7 | 0.0 |
|  |  |  |  | B8 | 0.6 | B8 | 0.2 |
|  | CA_3A-8A-40A | CA_3A-8A | 1.8G+900M+2.3G | B3 | 0.5 | B3 | 0.0 |
|  |  |  |  | B8 | 0.3 | B8 | 0.0 |
|  |  |  |  | B40 | 0.5 | B40 | 0.0 |
|  | CA_3A-42C | CA_3A-42A | 1.8G+3.5G | B3 | 0.6 | B3 | 0.2 |
|  |  |  |  | B42 | 0.8 | B42 | 0.5 |
|  | CA_4A-12A-30A | CA_4A-12A | 2.1G+700M+2.3G | B4 | 0.5 | B4 | 0.4 |
|  |  |  |  | B12 | 0.8 | B12 | 0.5 |
|  |  |  |  | B30 | 0.3 | B30 | 0.5 |
|  | CA_4A-12B | CA_4A-12A | 2.1G+700M | B4 | 0.3 | B4 | 0.0 |
|  |  |  |  | B12 | 0.8 | B12 | 0.5 |
|  | CA_7C-28A | CA_7A-28A or CA_7C | 2.6G+700M | B7 | 0.3 | B7 | 0.0 |
|  |  |  |  | B28 | 0.3 | B28 | 0.0 |
|  | CA_19A-42C | CA_19A-42A | 800M+3.5G | B19 | 0.3 | B19 | 0.0 |
|  |  |  |  | B42 | 0.8 | B42 | 0.5 |
|  | CA_21A-42C | CA_21A-42A | 1.5G+3.5G | B21 | 0.5 | B21 | 0.2 |
|  |  |  |  | B42 | 0.8 | B42 | 0.5 |
|  | CA_1A-3A-21A | CA_1A-3A or CA_1A-21A or CA_3A-21A | 2.1G+1.8G1.5G | B1 | 0.3 | B1 | 0.0 |
|  |  |  |  | B3 | 0.8 | B3 | 0.3 |
|  |  |  |  | B21 | 0.9 | B21 | 0.5 |
|  | CA_1A-3A-42A | CA_1A-3A or CA_1A-42A or CA_3A-42A | 2.1G+1.8G+3.5G | B1 | 0.6 | B1 | 0.2 |
|  |  |  |  | B3 | 0.6 | B3 | 0.2 |
|  |  |  |  | B42 | 0.8 | B42 | 0.5 |
|  | CA_1A-19A-42A | CA_1A-19A or CA_1A-42A or CA_19A-42A | 2.1G+800M+1.5G | B1 | 0.3 | B1 | 0.0 |
|  |  |  |  | B19 | 0.3 | B19 | 0.0 |
|  |  |  |  | B42 | 0.8 | B42 | 0.5 |
|  | CA_1A-21A-42A | CA_1A-21A or CA_1A-42A or CA_21A-42A | 2.1G+1.5G+3.5G | B1 | 0.3 | B1 | 0.0 |
|  |  |  |  | B21 | 0.4 | B21 | 0.0 |
|  |  |  |  | B42 | 0.8 | B42 | 0.5 |
|  | CA_3A-5A-7A | CA_3A-5A or CA_3A-7A or CA_5A-7A | 1.8G+800M+2.6G | B3 | 0.5 | B3 | 0.0 |
|  |  |  |  | B5 | 0.3 | B5 | 0.0 |
|  |  |  |  | B7 | 0.5 | B7 | 0.0 |
|  | CA_3A-19A-21A | CA_3A-19A or CA_3A-21A or CA_19A-21A | 1.8G+800M+1.5G | B3 | 0.8 | B3 | 0.3 |
|  |  |  |  | B19 | 0.3 | B19 | 0.0 |
|  |  |  |  | B21 | 0.9 | B21 | 0.5 |
|  | CA_3A-19A-42A | CA_3A-19A or CA_3A-42A or CA_19A-42A | 1.8G+800M+3.5G | B3 | 0.6 | B3 | 0.2 |
|  |  |  |  | B19 | 0.3 | B19 | 0.0 |
|  |  |  |  | B42 | 0.8 | B42 | 0.5 |
|  | CA_3A-21A-42A | CA_3A-21A or CA_3A-42A or CA_21A-42A | 1.8G+1.5G+3.5G | B3 | 0.8 | B3 | 0.3 |
|  |  |  |  | B21 | 0.9 | B21 | 0.5 |
|  |  |  |  | B42 | 0.8 | B42 | 0.5 |
|  | CA_19A-21A-42A | CA_19A-21A or CA_19A-42A or CA_21A-42A | 800M+1.5G+3.5G | B19 | 0.3 | B19 | 0.0 |
|  |  |  |  | B21 | 0.4 | B21 | 0.0 |
|  |  |  |  | B42 | 0.8 | B42 | 0.5 |
|  | CA_1A-3C | CA_1A-3A or CA_3C | 2.1G+1.8G | B1 | 0.3 | B1 | 0.0 |
|  |  |  |  | B3 | 0.3 | B3 | 0.0 |
|  | CA_3C-8A | CA_3A-8A or CA-3C | 1.8G+900M | B3 | 0.3 | B3 | 0.0 |
|  |  |  |  | B8 | 0.3 | B8 | 0.0 |
|  | CA_41A-42C | CA_41A-42A or CA_42C | 2.6G+3.5G | B41 | 0 | B41 | 0.4 |
|  |  |  |  | B42 | 0.5 | B42 | 0.5 |
|  | CA_41C-42A | CA_41A-42A or CA_41C | 2.6G+3.5G | B41 | 0 | B41 | 0.4 |
|  |  |  |  | B42 | 0.5 | B42 | 0.5 |
|  | CA_1A-5A-46A | CA_1A-5A | 2.1G+800M+5G | B1 | 0.3 | B1 | 0.0 |
|  |  |  |  | B5 | 0.3 | B5 | 0.0 |
|  |  |  |  | B46 | N/A | B46 | N/A |
|  | CA_1A-7A-46A | CA_1A-7A | 2.1G+2.6G+5G | B1 | 0.5 | B1 | 0.0 |
|  |  |  |  | B7 | 0.6 | B7 | 0.0 |
|  |  |  |  | B46 | N/A | B46 | N/A |
|  | CA_5A-7A-46A | CA_5A-7A | 800M+2.6G+5G | B5 | 0.3 | B5 | 0.0 |
|  |  |  |  | B7 | 0.3 | B7 | 0.0 |
|  |  |  |  | B46 | N/A | B46 | N/A |
|  | CA_2A-4A-7A | .CA_2A-4A | 1.9G+2.1G+2.6G | B2 | 0.5 | B2 | 0.3 |
|  |  |  |  | B4 | 0.5 | B4 | 0.5 |
|  |  |  |  | B7 | 0.5 | B7 | 0.5 |
|  | CA_1A-3A-28A | CA_1A-3A, CA_1A-28A, CA_3A-28A | 700M+1.8G+2.1G | B1 | 0.3 | B1 | 0 |
|  |  |  |  | B3 | 0.3 | B3 | 0 |
|  |  |  |  | B28 | 0.6 | B28 | 0.2 |
|  | CA_1A-21A-28A | CA_1A-21A, CA_1A-28A, CA_21A-28A | 700M+1.5G+2.1G | B1 | 0.3 | B1 | 0 |
|  |  |  |  | B21 | 0.4 | B21 | 0 |
|  |  |  |  | B28 | 0.6 | B28 | 0.2 |
|  | CA_1A-28A-42A | CA_1A-28A, CA_1A-42A, CA_28A-42A | 700M+2.1G+3.5G | B1 | 0.3 | B1 | 0 |
|  |  |  |  | B28 | 0.6 | B28 | 0.2 |
|  |  |  |  | B42 | 0.8 | B42 | 0.5 |
|  | CA_3A-21A-28A | CA_3A-21A, CA_3A-28A, CA_21A-28A | 700M+1.5G+1.8G | B3 | 0.8 | B3 | 0.3 |
|  |  |  |  | B21 | 0.9 | B21 | 0.5 |
|  |  |  |  | B28 | 0.3 | B28 | 0 |
|  | CA_3A-28A-42A | CA_3A-28A, CA_3A-42A, CA_28A-42A | 700M+1.8G+3.5G | B3 | 0.6 | B3 | 0.2 |
|  |  |  |  | B28 | 0.5 | B28 | 0.2 |
|  |  |  |  | B42 | 0.8 | B42 | 0.5 |
|  | CA_21A-28A-42A | CA_21A-28A, CA_21A-42A, CA_28A-42A | 700M+1.5G+3.5G | B21 | 0.4 | B21 | 0 |
|  |  |  |  | B28 | 0.5 | B28 | 0.2 |
|  |  |  |  | B42 | 0.8 | B42 | 0.5 |
|  | CA_28A-42C | CA_28A-42A, CA_42C | 700M+3.5G | B28 | 0.5 | B28 | 0.2 |
|  |  |  |  | B42 | 0.8 | B42 | 0.5 |
|  | CA_1A-7A-7A | CA_1A-7A | 2.1G+2.6G | B1 | 0.5 | B1 | 0 |
|  |  |  |  | B7 | 0.6 | B7 | 0 |
|  | CA_3A-7A-7A | CA_3A-7A | 1.8G+2.6G | B3 | 0.5 | B3 | 0 |
|  |  |  |  | B7 | 0.5 | B7 | 0 |
|  | CA_5A-7A-7A | CA_5A-7A | 800M+2.6G | B5 | 0.3 | B5 | 0 |
|  |  |  |  | B7 | 0.3 | B7 | 0 |
|  | CA_3A-42C | CA_42C | 1.8G+3.5G | B3 | TBD | B3 | TBD |
|  |  |  |  | B42 | TBD | B42 | TBD |
|  | CA_3A-41A-42A | CA_41A-42A | 1.8G+2.6G+3.5G | B3 | TBD | B3 | TBD |
|  |  |  |  | B41 | TBD | B41 | TBD |
|  |  |  |  | B42 | TBD | B42 | TBD |
|  | CA_28A-41A-42A | CA_41A-42A | 700M+2.6G+3.5G | B28 | TBD | B28 | TBD |
|  |  |  |  | B41 | TBD | B41 | TBD |
|  |  |  |  | B42 | TBD | B42 | TBD |
| 4DL/2UL inter-band CA | CA_1A-3A-5A-40A | CA_1A-3A or CA_1A-5A or CA_3A-5A | 2.1G+1.8G+800M+2.3G | B1 | 0.5 | B1 | 0.0 |
|  |  |  |  | B3 | 0.5 | B3 | 0.0 |
|  |  |  |  | B5 | 0.3 | B5 | 0.0 |
|  |  |  |  | B40 | 0.5 | B40 | 0.0 |
|  | CA_1A-3A-8A-40A | CA_1A-3A or CA_1A-8A or CA_3A-8A | 2.1G+1.8G+900M+2.3G | B1 | 0.5 | B1 | 0.0 |
|  |  |  |  | B3 | 0.5 | B3 | 0.0 |
|  |  |  |  | B8 | 0.3 | B8 | 0.0 |
|  |  |  |  | B40 | 0.5 | B40 | 0.0 |
|  | CA_1A-3A-7A-8A | CA_1A-3A or CA_1A-7A or CA_1A-8A or CA_3A-7A or CA_3A-8A | 2.1G+1.8G+2.6G+900M | B1 | 0.6 | B1 | 0.0 |
|  |  |  |  | B3 | 0.6 | B3 | 0.0 |
|  |  |  |  | B7 | 0.6 | B7 | 0.0 |
|  |  |  |  | B8 | 0.6 | B8 | 0.2 |
|  | CA_2A-4A-5A-29A | CA_2A-4A | 1.9G+2.1G+800M+700M | B2 | 0.5 | B2 | 0.3 |
|  |  |  |  | B4 | 0.5 | B4 | 0.3 |
|  |  |  |  | B5 | 0.5 | B5 | 0.0 |
|  |  |  |  | B29 | N/A | B29 | 0.0 |
|  | CA_3A-7C-28A | CA_3A-7A or CA_7A-28A or CA_7C | 1.8G+2.6G+700M | B3 | 0.5 | B3 | 0.0 |
|  |  |  |  | B7 | 0.5 | B7 | 0.0 |
|  |  |  |  | B28 | 0.3 | B28 | 0.0 |
|  | CA_1A-3A-5A-7A | CA_1A-3A or CA_1A-5A or CA_1A-7A or CA_3A-5A or CA_3A-7A or CA_5A-7A | 2.1G+1.8G+800M+2.6G | B1 | 0.6 | B1 | 0.0 |
|  |  |  |  | B3 | 0.6 | B3 | 0.0 |
|  |  |  |  | B5 | 0.3 | B5 | 0.0 |
|  |  |  |  | B7 | 0.6 | B7 | 0.0 |
|  | CA_1A-3A-19A-42A | CA_1A-3A or CA_1A-19A or CA_1A-42A or CA_3A-19A or CA_3A-42A or CA_19A-42A | 2.1G+1.8G+800M+3.5G | B1 | 0.6 | B1 | 0.2 |
|  |  |  |  | B3 | 0.6 | B3 | 0.2 |
|  |  |  |  | B19 | 0.3 | B19 | 0.0 |
|  |  |  |  | B42 | 0.8 | B42 | 0.5 |
|  | CA_1A-19A-21A-42A | CA_1A-19A or CA_1A-21A or CA_1A-42A or CA_19A-21A or CA_19A-42A or CA_21A-42A | 2.1G+800M+1.5G+3.5G | B1 | 0.3 | B1 | 0.0 |
|  |  |  |  | B19 | 0.3 | B19 | 0.0 |
|  |  |  |  | B21 | 0.4 | B21 | 0.0 |
|  |  |  |  | B42 | 0.8 | B42 | 0.5 |
|  | CA_1A-3A-42C | CA_1A-3A or CA_1A-42A or CA_3A-42A | 2.1G+1.8G+3.5G | B1 | 0.6 | B1 | 0.2 |
|  |  |  |  | B3 | 0.6 | B3 | 0.2 |
|  |  |  |  | B42 | 0.8 | B42 | 0.5 |
|  | CA_1A-19A-42C | CA_1A-19A or CA_1A-42A or CA_19A-42A | 2.1G+800M+3.5G | B1 | 0.3 | B1 | 0.0 |
|  |  |  |  | B19 | 0.3 | B19 | 0.0 |
|  |  |  |  | B42 | 0.8 | B42 | 0.5 |
|  | CA_1A-21A-42C | CA_1A-21A or CA_1A-42A or CA_21A-42A | 2.1G+1.5G+3.5G | B1 | 0.3 | B1 | 0.0 |
|  |  |  |  | B21 | 0.4 | B21 | 0.0 |
|  |  |  |  | B42 | 0.8 | B42 | 0.5 |
|  | CA_3A-19A-42C | CA_3A-19A or CA_3A-42A or CA_19A-42A | 1.8G+800M+3.5G | B3 | 0.6 | B3 | 0.2 |
|  |  |  |  | B19 | 0.3 | B19 | 0.0 |
|  |  |  |  | B42 | 0.8 | B42 | 0.5 |
|  | CA_19A-21A-42C | CA_19A-21A or CA_19A-42A or CA_21A-42A | 800M+1.5G+3.5G | B19 | 0.3 | B19 | 0.0 |
|  |  |  |  | B21 | 0.4 | B21 | 0.0 |
|  |  |  |  | B42 | 0.8 | B42 | 0.5 |
|  | CA_1A-3C-8A | CA_1A-3A or CA_1A-8A or CA_3A-8A or CA_3C | 2.1G+1.8G+900M | B1 | 0.3 | B1 | 0.0 |
|  |  |  |  | B3 | 0.3 | B3 | 0.0 |
|  |  |  |  | B8 | 0.3 | B8 | 0.0 |
|  | CA_41C-42C | CA_41A-42A | 2.6G+3.5G | B41 | 0 | B41 | 0.4 |
|  |  |  |  | B42 | 0.5 | B42 | 0.5 |
|  | CA_1A-5A-7A-46A | CA_1A-5A or CA_1A-7A or CA_5A-7A | 2.1G+800M+2.6G+5G | B1 | 0.5 | B1 | 0.0 |
|  |  |  |  | B5 | 0.3 | B5 | 0.0 |
|  |  |  |  | B7 | 0.6 | B7 | 0.0 |
|  |  |  |  | B46 | N/A | B46 | N/A |
|  | CA_1A-5A-46C | CA_1A-5A | 2.1G+800M+5G | B1 | 0.3 | B1 | 0.0 |
|  |  |  |  | B5 | 0.3 | B5 | 0.0 |
|  |  |  |  | B46 | N/A | B46 | N/A |
|  | CA_1A-7A-46C | CA_1A-7A | 2.1G+2.6G+5G | B1 | 0.5 | B1 | 0.0 |
|  |  |  |  | B7 | 0.6 | B7 | 0.0 |
|  |  |  |  | B46 | N/A | B46 | N/A |
|  | CA_5A-7A-46C | CA_5A-7A | 800M+2.6G+5G | B5 | 0.3 | B5 | 0.0 |
|  |  |  |  | B7 | 0.3 | B7 | 0.0 |
|  |  |  |  | B46 | N/A | B46 | N/A |
|  | CA_2A-4A-7A-7A | CA_2A-4A | 1.9G+2.1G+2.6G+2.6G | B2 | 0.5 | B2 | 0.3 |
|  |  |  |  | B4 | 0.5 | B4 | 0.5 |
|  |  |  |  | B7 | 0.5 | B7 | 0.5 |
|  | CA_1A-3A-19A-21A | CA_1A-3A, CA_1A-19A, CA_1A-21A, CA_3A-19A, CA_3A-21A, CA_19A-21A | 800M+1.5G+1.8G+2.1G | B1 | 0.3 | B1 | 0 |
|  |  |  |  | B3 | 0.8 | B3 | 0.3 |
|  |  |  |  | B19 | 0.3 | B19 | 0 |
|  |  |  |  | B21 | 0.9 | B21 | 0.5 |
|  | CA_1A-3A-21A-28A | CA_1A-3A, CA_1A-21A, CA_1A-28A, CA_3A-21A, CA_3A-28A, CA_21A-28A | 700M+1.5G+1.8G+2.1G | B1 | 0.3 | B1 | 0 |
|  |  |  |  | B3 | 0.8 | B3 | 0.3 |
|  |  |  |  | B21 | 0.9 | B21 | 0.5 |
|  |  |  |  | B28 | 0.6 | B28 | 0.2 |
|  | CA_1A-3A-21A-42A | CA_1A-3A, CA_1A-21A, CA_1A-42A, CA_3A-21A, CA_3A-42A, CA_21A-42A | 1.5G+1.8G+2.1G+3.5G | B1 | 0.6 | B1 | 0.2 |
|  |  |  |  | B3 | 0.8 | B3 | 0.3 |
|  |  |  |  | B21 | 0.9 | B21 | 0.5 |
|  |  |  |  | B42 | 0.8 | B42 | 0.5 |
|  | CA_1A-3A-28A-42A | CA_1A-3A, CA_1A-28A, CA_1A-42A, CA_3A-28A, CA_3A-42A, CA_28A-42A | 700M+1.8G+2.1G+3.5G | B1 | 0.6 | B1 | 0.2 |
|  |  |  |  | B3 | 0.6 | B3 | 0.2 |
|  |  |  |  | B28 | 0.6 | B28 | 0.2 |
|  |  |  |  | B42 | 0.8 | B42 | 0.5 |
|  | CA_1A-21A-28A-42A | CA_1A-21A, CA_1A-28A, CA_1A-42A, CA_21A-28A, CA_21A-42A, CA_28A-42A | 700M+1.5G+2.1G+3.5G | B1 | 0.3 | B1 | 0 |
|  |  |  |  | B21 | 0.4 | B21 | 0 |
|  |  |  |  | B28 | 0.6 | B28 | 0.2 |
|  |  |  |  | B42 | 0.8 | B42 | 0.5 |
|  | CA_1A-28A-42C | CA_1A-28A, CA_1A-42A, CA_28A-42A | 700M+2.1G+3.5G | B1 | 0.3 | B1 | 0 |
|  |  |  |  | B28 | 0.6 | B28 | 0.2 |
|  |  |  |  | B42 | 0.8 | B42 | 0.5 |
|  | CA_3A-21A-42C | CA_3A-21A, CA_3A-42A, CA_21A-42A | 1.5G+1.8G+3.5G | B3 | 0.8 | B3 | 0.3 |
|  |  |  |  | B21 | 0.9 | B21 | 0.5 |
|  |  |  |  | B42 | 0.8 | B42 | 0.5 |
|  | CA_3A-28A-42C | CA_3A-28A, CA_3A-42A, CA_28A-42A | 700M+1.8G+3.5G | B3 | 0.6 | B3 | 0.2 |
|  |  |  |  | B28 | 0.5 | B28 | 0.2 |
|  |  |  |  | B42 | 0.8 | B42 | 0.5 |
|  | CA_21A-28A-42C | CA_21A-28A, CA_21A-42A, CA_28A-42A | 700M+1.5G+3.5G | B21 | 0.4 | B21 | 0 |
|  |  |  |  | B28 | 0.5 | B28 | 0.2 |
|  |  |  |  | B42 | 0.8 | B42 | 0.5 |
|  | CA_1A-3A-7A-7A | CA_1A-3A, CA_1A-7A, CA_3A-7A | 1.8G+2.1G+2.6G | B1 | 0.6 | B1 | 0 |
|  |  |  |  | B3 | 0.6 | B3 | 0 |
|  |  |  |  | B7 | 0.6 | B7 | 0 |
|  | CA_1A-5A-7A-7A | CA_1A-5A, CA_1A-7A, CA_5A-7A | 800M+2.1G+2.6G | B1 | 0.5 | B1 | 0 |
|  |  |  |  | B5 | 0.3 | B5 | 0 |
|  |  |  |  | B7 | 0.6 | B7 | 0 |
|  | CA_3A-5A-7A-7A | CA_3A-5A, CA_3A-7A, CA_5A-7A | 800M+ 1.8G+2.6G | B3 | 0.5 | B3 | 0 |
|  |  |  |  | B5 | 0.3 | B5 | 0 |
|  |  |  |  | B7 | 0.5 | B7 | 0 |
|  | CA_3C-7C | CA_3A-7A | 1.8G+2.6G | B3 | TBD | B3 | TBD |
|  |  |  |  | B7 | TBD | B7 | TBD |
|  | CA_3A-28A-41A-42A | CA_41A-42A | 700M+1.8G+2.6G+3.5G | B3 | TBD | B3 | TBD |
|  |  |  |  | B28 | TBD | B28 | TBD |
|  |  |  |  | B41 | TBD | B41 | TBD |
|  |  |  |  | B42 | TBD | B42 | TBD |
|  | CA_3A-28A-42C | CA_42C | 700M+1.8G+3.5G | B3 | TBD | B3 | TBD |
|  |  |  |  | B28 | TBD | B28 | TBD |
|  |  |  |  | B42 | TBD | B42 | TBD |
|  | CA_3A-41A-42C | CA_42C, CA_41A-42A | 1.8G+2.6G+3.5G | B3 | TBD | B3 | TBD |
|  |  |  |  | B41 | TBD | B41 | TBD |
|  |  |  |  | B42 | TBD | B42 | TBD |
|  | CA_3A-41C-42A | CA_41A-42A | 1.8G+2.6G+3.5G | B3 | TBD | B3 | TBD |
|  |  |  |  | B41 | TBD | B41 | TBD |
|  |  |  |  | B42 | TBD | B42 | TBD |
|  | CA_28A-41A-42C | CA_42C, CA_41A-42A | 700M+2.6G+3.5G | B28 | TBD | B28 | TBD |
|  |  |  |  | B41 | TBD | B41 | TBD |
|  |  |  |  | B42 | TBD | B42 | TBD |
|  | CA_28A-41C-42A | CA_41A-42A | 700M+2.6G+3.5G | B28 | TBD | B28 | TBD |
|  |  |  |  | B41 | TBD | B41 | TBD |
|  |  |  |  | B42 | TBD | B42 | TBD |
| 5DL/2UL inter-band CA | CA_1A-19A-21A-42C | CA_1A-19A or CA_1A-21A or CA_1A-42A or CA_19A-21A or CA_19A-42A or CA_21A-42A | 2.1G+800M+1.5G+3.5G | B1 | 0.3 | B1 | 0 |
|  |  |  |  | B19 | 0.3 | B19 | 0 |
|  |  |  |  | B21 | 0.4 | B21 | 0 |
|  |  |  |  | B42 | 0.8 | B42 | 0.5 |
|  | CA_1A-3A-19A-42C | CA_1A-3A or CA_1A-19A or CA_1A-42A or CA_3A-19A or CA_3A-42A or CA_19A-42A | 2.1G+1.8G+800M+3.5G | B1 | 0.6 | B1 | 0.2 |
|  |  |  |  | B3 | 0.6 | B3 | 0.2 |
|  |  |  |  | B19 | 0.3 | B19 | 0 |
|  |  |  |  | B42 | 0.8 | B42 | 0.5 |
|  | CA_1A-3A-21A-42C | CA_1A-3A, CA_1A-21A, CA_1A-42A, CA_3A-21A, CA_3A-42A, CA_21A-42A | 1.5G+1.8G+2.1G+3.5G | B1 | 0.6 | B1 | 0.2 |
|  |  |  |  | B3 | 0.8 | B3 | 0.3 |
|  |  |  |  | B21 | 0.9 | B21 | 0.5 |
|  |  |  |  | B42 | 0.8 | B42 | 0.5 |
|  | CA_1A-3A-28A-42C | CA_1A-3A, CA_1A-28A, CA_1A-42A, CA_3A-28A, CA_3A-42A, CA_28A-42A | 700M+1.8G+2.1G+3.5G | B1 | 0.6 | B1 | 0.2 |
|  |  |  |  | B3 | 0.6 | B3 | 0.2 |
|  |  |  |  | B28 | 0.6 | B28 | 0.2 |
|  |  |  |  | B42 | 0.8 | B42 | 0.5 |
|  | CA_1A-21A-28A-42C | CA_1A-21A, CA_1A-28A, CA_1A-42A, CA_21A-28A, CA_21A-42A, CA_28A-42A | 700M+1.5G+2.1G+3.5G | B1 | 0.3 | B1 | 0 |
|  |  |  |  | B21 | 0.4 | B21 | 0 |
|  |  |  |  | B28 | 0.6 | B28 | 0.2 |
|  |  |  |  | B42 | 0.8 | B42 | 0.5 |
|  | CA_1A-3A-5A-7A-7A | CA_1A-3A, CA_1A-5A, CA_1A-7A, CA_3A-5A, CA_3A-7A, CA_5A-7A | 800M+1.8G+2.1G+2.6G | B1 | 0.6 | B1 | 0 |
|  |  |  |  | B3 | 0.6 | B3 | 0 |
|  |  |  |  | B5 | 0.3 | B5 | 0 |
|  |  |  |  | B7 | 0.6 | B7 | 0 |

5.1.3	Fallback CA mode and mandatory support of all paired 2UL CA configurations

Based on the WF (R4-168853), RAN4 agreed to revised Note 6 in Table 5.6A.1-2a of TS36.101 as below,

- Note 6 wording improved:

  - Current: If the UE supports uplink CA for corresponding downlink CA it shall support this uplink CA configuration

  - New: If the UE supports any uplink CA configuration for corresponding downlink CA configuration it shall support this uplink CA configuration

- The Note 6 shall apply also for 4DL/2UL (Table 5.6A.1-2b of TS36.101) and 5DL/2UL (once specified). However the Note 6 is not applied for 2DL/2UL (Table 5.6A.1-2) and shall be removed.

5.1.4	General TX/RX requirements

5.1.4.1 Transmitter requirements

Dual uplink Transmitter requirements already analysed in TR36.860 for dual uplink inter-band CA. The xDL/2UL CA UE follow the legacy UE transmitter requirements in Table 5.1.4.1-1.

Table 5.1.4.1-1: Common UE RF Tx requirements for dual uplink inter-band CA configuration

| Section / Clause in TS36.101 | Description |
| --- | --- |
| 5.6A.1 | Channel bandwidths per operating band for CA |
| 6.2.2A | UE maximum output power for CA |
| 6.2.5A | Configured transmitted Power for CA |
| 6.3.2A | UE Minimum output power for CA |
| 6.3.3A | UE Transmit OFF power for CA |
| 6.3.4A | ON/OFF time mask for CA |
| 6.3.5A | Power control for CA |
| 6.5.1A | Frequency error for CA |
| 6.5.2A | Transmit modulation quality for CA |
| 6.6.1A | Occupied bandwidth for CA |
| 6.6.2.1A | Spectrum emission mask for CA |
| 6.6.2.3 | Adjacent Channel Leakage Ratio |
| 6.6.3.1A | Spurious Emission for CA |
| 6.6.3.2A | Spurious emission band UE co-existence for CA |
| 6.7.1A | Transmit intermodulation for CA |

5.1.4.2 Receiver requirements

Dual uplink receiver requirements already analysed in TR36.860 for dual uplink inter-band CA according to the self-interference or not.

Additional UE RF receiver requirements for xDL/2UL CA UE are listed in the Table 5.1.4.2-1.

The xDL/2UL CA band combinations without self-interference issues can follow the legacy UE Rx requirements in xDL/1UL CA RF Rx requirements. Since RAN4 already concluded that test coverage of Out-of-band blocking and spurious response for 2DL/2UL are covered by 1UL test (i.e. single carrier LTE and 1UL/2DL CA) [5]. Hence only REFSENS will be tested xDL/2UL CA UE with all downlink component carriers active and both of the uplink carriers active.

Furthermore, RAN4 should define the new maximum sensitivity degradation (MSD) for REFSENS requirements of each CA band combinations when the xDL/2UL CA band combinations has self-interference problems.

Table 5.1.4.2-1: Common UE RF Rx requirements for dual uplink inter-band CA configuration

| Section / Clause in TS36.101 | Description | Comments |
| --- | --- | --- |
| 7.3.1A | Reference sensitivity for CA | Applied to all xDL/2UL CA band combinations |
| 7.6.2.1A | Out-of-band blocking for CA | No need [5] |
| 7.7.1A | Spurious response for CA | No need [5] |

### 5.1.5	Summary of interference studies

Table 5.1.5-1 summarizes the CA band combinations with self-interference problems for CA_3DL/2UL CA.

Table 5.1.5-1: Summary of Self-interference analysis for 2UL/3DL CA

| downlink CA configuration | uplink CA Configuration | Harmonic relation to 3rd band without uplink | intermodulation to 3rd band without uplink | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| CA_1A-3A-40A | CA_1A-3A | - | 5th order | Yes | 8.0 Table 7.3.1A-0bA for CA_1A-3A [TS36.101] |
| CA_1A-5A-40A | CA_1A-5A | - | 4th order | - | 9.0 |
| CA_1A-3A-7A | CA_1A-3A | - | - | Yes | N/A |
|  | CA_1A-7A | - | - |  | Table 7.3.1A-0bA for CA_1A-3A [TS36.101] |
|  | CA_3A-7A | - | - |  | Table 7.3.1A-0bA for CA_1A-3A [TS36.101] |
| CA_1A-7A-8A | CA_1A-7A | - | 5th order | - | No Need The IMD5 is fairly small and the overlapped region is quite small portion |
|  | CA_1A-8A | B8 3rd harmonic |  | - | Table 7.3.1A-0a for CA_1A-7A-8A [TS36.101] |
| CA_1A-8A-40A | CA_1A-8A | - | - | - | N/A |
| CA_1A-42C | CA_1A-42A | - | 4th order | - | No impact when consider a fixed Tx-Rx separation of 190MHz in Band 1 own Rx band. |
| CA_2A-4A-5A | CA_2A-4A | - | - | - | N/A |
| CA_2A-4A-29A | CA_2A-4A |  |  |  | N/A |
| CA_2A-12B | CA_2A-12A | - | - | - | N/A |
| CA_2A-12A-30A | CA_2A-12A | - | 4th order | - | 12.0 |
| CA_3A-5A-40A | CA_3A-5A | - | - | - | N/A |
| CA_3A-7A-8A | CA_3A-7A |  | 3rd order | - | TBD |
|  | CA_3A-8A | B8 3rd harmonic | 2nd & 3rd order | - | Table 7.3.1A-0a for CA_3A-7A-8A [TS36.101] TBD |
|  | CA_7A-8A | B8 2nd harmonics | - |  | Harmonics issues already completed in 2DL/1UL CA_3A-8A |
| CA_3A-8A-40A | CA_3A-8A | - | - | - | N/A |
| CA_3A-42C | CA_3A-42A | - | - | - | Studied in 2UL/2DL CA in Rel-14. N/A |
|  | CA_42C | - | - | - | N/A |
| CA_4A-12B | CA_4A-12A | - | - | - | N/A |
| CA_4A-12A-30A | CA_4A-12A | - | - | - | N/A |
| CA_7C-28A | CA_7C | - | - | - | N/A |
|  | CA_7A-28A | - | - | - | N/A |
| CA_19A-42C | CA_19A-42A | - | 4th order | - | Studied in 2UL/2DL CA in Rel-14. Consider specific spectrum of the operator N/A |
| CA_21A-42C | CA_21A-42A | - | - | - | N/A |
| CA_1A-5A-46A | CA_1A-5A | B1 3rd harmonic B5 7th harmonics | 4th & 5th order | - | - Harmonics issues already completed in 2DL/1UL CA_1A-46A & CA_5A-46A. 1.4 |
| CA_1A-7A-46A | CA_1A-7A | B1 3rd harmonic | 4th & 5th order | - | - Harmonics issues already completed in 2DL/1UL CA_1A-46A. 4.4 |
| CA_5A-7A-46A | CA_5A-7A | B5 7th harmonics | 3rd & 5th order | - | - Harmonics issues already completed in 2DL/1UL CA_5A-46A. 12.5 |
| CA_1A-3A-21A | CA_1A-3A | - | 3rd & 5th order | Yes | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_1A-21A | - | - | - | N/A |
|  | CA_3A-21A | - | 3rd order | - | - No MSD due to IMD to be specified according to actural spectrum holdings. |
| CA_1A-3A-42A | CA_1A-3A | B3 2nd harmonics | 4th order | Yes | - Harmonics issues already completed in 2DL/1UL CA_3A-42A. 13.3 |
|  | CA_1A-42A | - | - | - | N/A |
|  | CA_3A-42A | - | - | - | N/A |
| CA_1A-19A-42A | CA_1A-19A | - | 3rd order | - | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_1A-42A | - | 5th order | - | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_19A-42A | - | - | - | N/A |
| CA_1A-21A-42A | CA_1A-21A | - | 2nd order | - | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_1A-42A | - | 2nd order | - | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_21A-42A | - | 2nd order | - | - No MSD due to IMD to be specified according to actural spectrum holdings. |
| CA_3A-5A-7A | CA_3A-5A | - | 2nd & 3rd order | - | 30.0 for 2nd IMD 19.5 for 3rd IMD |
|  | CA_3A-7A | - | 3rd order | - | 19.2 |
|  | CA_5A-7A | - | - | - | N/A |
| CA_3A-19A-21A | CA_3A-19A | - | - | - | N/A |
|  | CA_3A-21A | - | 5th order | - | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_19A-21A | - | 5th order | - | 4.0 |
| CA_3A-19A-42A | CA_3A-19A | B3 2nd harmonics | 3rd & 5th order | - | - Harmonics issues already completed in 2DL/1UL CA_3A-42A - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_3A-42A | - | - | - | N/A |
|  | CA_19A-42A | - | 3rd order | - | - No MSD since the overlapped region (0.1MHz) is quite small portion. |
| CA_3A-21A-42A | CA_3A-21A | B3 2nd harmonics | - | - | - Harmonics issues already completed in 2DL/1UL CA_3A-42A |
|  | CA_3A-42A | - | 5th order | - | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_21A-42A | - | - | - | N/A |
| CA_19A-21A-42A | CA_19A-21A | - | 4th order | - | 12.9 |
|  | CA_19A-42A | - | - | - | N/A |
|  | CA_21A-42A | - | 4th order | - | 12.5 |
| CA_1A-3C | CA_1A-3A | - | - | Yes | Table 7.3.1A-0bA for CA_1A-3A |
|  | CA_3C | - | - | - | N/A |
| CA_3C-8A | CA_3C | - | - | - | N/A |
|  | CA_3A-8A | B8 2nd harmonics | 4th & 5th order |  | Harmonics issues already completed in 2DL/1UL CA_3A-8A IMD issues already covered in 2UL/2DL CA_3A-8A |
| CA_2A-4A-7A | CA_2A-4A | - | - | - | N/A |
| CA_41A-42C | CA_41A-42A | - | - | - | N/A |
| CA_41C-42A | CA_41A-42A | - | - | - | N/A |
|  | CA_41C | - | - | - | N/A |
| CA_1A-3A-28A | CA_1A-3A | - | - | Yes | Table 7.3.1A-0bA for CA_1A-3A |
|  | CA_1A-28A | - | 5th order |  | 4.3 |
|  | CA_3A-28A | B28 3rd harmonics | 4th order |  | Harmonics issues already completed in 2DL/1UL CA_1A-28A 10.7 |
| CA_1A-21A-28A | CA_1A-21A | - | - | - | N/A |
|  | CA_1A-28A | B28 2nd harmonics | - | - | No MSD due to harmonics by considering of actural spectrum holdings. |
|  | CA_21A-28A | B28 3rd harmonics | 2nd & 3rd order | - | Harmonics issues already completed in 2DL/1UL CA_1A-28A No MSD due to IMD problems by considering of actural spectrum holdings. |
| CA_1A-28A-42A | CA_1A-28A | - | 3rd order | - | 15.4 |
|  | CA_1A-42A | - | - | - | N/A |
|  | CA_28A-42A | B28 3rd harmonics | 3rd order | - | Harmonics issues already completed in 2DL/1UL CA_1A-28A 11.3 |
| CA_3A-21A-28A | CA_3A-21A | - | 5th order | - | 2.9 |
|  | CA_3A-28A | B28 2nd harmonics | - | - | No MSD due to harmonics by considering of actural spectrum holdings. |
|  | CA_21A-28A | - | - | - | N/A |
| CA_3A-28A-42A | CA_3A-28A | B3 2nd harmonics | - | - | Harmonics issues already completed in 2DL/1UL CA_3A-42A |
|  | CA_3A-42A | - | - | - | N/A |
|  | CA_28A-42A | - | - | - | N/A |
| CA_21A-28A-42A | CA_21A-28A | - | 3rd & 4th order | - | No MSD due to IMD by considering of actural spectrum holdings. |
|  | CA_21A-42A | - | 4th order | - | No MSD due to IMD by considering of actural spectrum holdings. |
|  | CA_28A-42A | B28 2nd harmonics | - | - | No MSD due to harmonics by considering of actural spectrum holdings. |
| CA_28A-42C | CA_28A-42A | - | - | - | N/A |
|  | CA_42C | - | - | - | N/A |
| CA_1A-7A-7A | CA_1A-7A | - | - | - | N/A |
| CA_3A-7A-7A | CA_3A-7A | - | 4th order |  | 4th IMD was covered in 2DL/2UL CA_3A-7A |
| CA_5A-7A-7A | CA_5A-7A | - | 5th order | - | 5th IMD was covered in 2DL/2UL CA_5A-7A |
| CA_3A-41A-42A | 41A-42A | - | 3rd & 4th order | - | - TBD for 3rd IMD - TBD for 4th IMD |
| CA_28A-41A-42A | 41A-42A | - | 2nd & 5th order | - | - TBD for 2nd IMD - TBD for 5th IMD |

Table 5.1.5-2 summarizes the CA_ band combinations with self-interference problems for CA_4DL/2UL CA.

Table 5.1.5-2: Summary of Self-interference analysis for 2UL/4DL CA

| downlink CA configuration | uplink CA Configuration | Harmonic relation to 3rd and 4th band without uplink | intermodulation to 3rd and 4th band without uplink | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| CA_1A-3A-8A-40A | CA_1A-3A | - | 5th order into B40 | Yes | No need (already covered 3DL CA_1A-3A-40A with 2UL CA_1A-3A) |
|  | CA_1A-8A | 2nd harmonic into B3 | - |  | Table 7.3.1A-0a for CA_1A-3A-8A-40A [TS36.101] Table 7.3.1A-0bA for CA_1A-3A [TS36.101] |
|  | CA_3A-8A | - | - |  | N/A |
| CA_1A-3A-5A-40A | CA_1A-3A | - | 5th order into B40 | Yes | No need (already covered 3DL CA_1A-3A-40A with 2UL CA_1A-3A) |
|  | CA_1A-5A | - | 4th order into B40 |  | No need (already covered 3DL CA_1A-5A-40A with 2UL CA_1A-5A) |
|  | CA_3A-5A | - | - |  | Table 7.3.1A-0bA for CA_1A-3A [TS36.101] |
| CA_1A-3A-7A-8A | CA_1A-3A | - | - | Yes | N/A |
|  | CA_1A-7A | - | 5th order into B8 |  | No need (already covered 3DL CA_1A-7A-8A with 2UL CA_1A-7A) |
|  | CA_1A-8A | 2nd harmonic into B3 & | - |  | Table 7.3.1A-0a for CA_1A-3A-7A-8A [TS36.101] Table 7.3.1A-0bA for CA_1A-3A [TS36.101] |
|  |  | 3rd harmonic into B7 | - |  | Table 7.3.1A-0a for CA_1A-3A-7A-8A [TS36.101] |
|  | CA_3A-7A |  | 3rd order into B8 |  | No need (already covered 3DL CA_3A-7A-8A with 2UL CA_3A-7A) |
|  | CA_3A-8A | 3rd harmonic into B7 | 2nd & 3rd order into B7 |  | Table 7.3.1A-0a for CA_1A-3A-7A-8A [TS36.101] No need (already covered 3DL CA_3A-7A-8A with 2UL CA_3A-8A) |
| CA_2A-4A-5A-29A | CA_2A-4A | - | - | - | N/A |
| CA_3A-7C-28A | CA_3A-7A | - | 2nd order into B28 | - | No need (already covered 3DL CA_3A-7A-28A with 2UL CA_3A-7A) |
|  | CA_7A-28A | - | 2nd order into B3 | - | No need (already covered 3DL CA_3A-7A-28A with 2UL CA_7A-28A) |
|  | CA_7C | - | - | - | N/A |
| CA_1A-5A-7A-46A | CA_1A-5A | B1 3rd harmonic B5 7th harmonics | 4th & 5th order into B46 | - | - Harmonics issues already completed in 2DL/1UL CA_1A-46A & CA_5A-46A. - No need for IMD (it will covered 3DL CA_1A-5A-46A with 2UL CA_1A-5A) |
|  | CA_1A-7A | B1 3rd harmonic | 4th & 5th order into B46 5th order into B5 | - | - Harmonics issues already completed in 2DL/1UL CA_1A-46A. - No need for IMD (it will covered 3DL CA_1A-7A-46A with 2UL CA_1A-7A) - No need for IMD (it was already covered 3DL CA_1A-5A-7A with 2UL_CA_1A-7A |
|  | CA_5A-7A | B5 7th harmonics | 3rd & 5th order into B46 | - | - Harmonics issues already completed in 2DL/1UL CA_5A-46A. - No need for IMD (it will covered 3DL CA_5A-7A-46A with 2UL CA_5A-7A) |
| CA_1A-3A-5A-7A | CA_1A-3A | - | - | - | N/A |
|  | CA_1A-5A | - | - | - | N/A |
|  | CA_1A-7A | - | 5th order into B5 | - | - No need for IMD (it was already covered 3DL CA_1A-5A-7A with 2UL CA_1A-7A) |
|  | CA_3A-5A | - | 2nd & 3rd order into B7 | - | - No need for IMD (it will covered 3DL CA_3A-5A-7A with 2UL CA_3A-5A) |
|  | CA_3A-7A | - | 3rd order into B5 |  | - No need for IMD (it will covered 3DL CA_3A-5A-7A with 2UL CA_3A-7A) |
|  | CA_5A-7A | - | - | - | N/A |
| CA_1A-5A-46C | CA_1A-5A | B1 3rd harmonic B5 7th harmonics | 4th & 5th order | - | - Harmonics issues already completed in 2DL/1UL CA_1A-46A & CA_5A-46A. - No need for IMD (it will covered 3DL CA_1A-5A-46A with 2UL CA_1A-5A) |
| CA_1A-7A-46C | CA_1A-7A | B1 3rd harmonic | 4th & 5th order | - | - Harmonics issues already completed in 2DL/1UL CA_1A-46A. - No need for IMD (it will covered 3DL CA_1A-7A-46A with 2UL CA_1A-7A) |
| CA_5A-7A-46C | CA_5A-7A | B5 7th harmonics | 3rd & 5th order | - | - Harmonics issues already completed in 2DL/1UL CA_5A-46A. - No need for IMD (it will covered 3DL CA_5A-7A-46A with 2UL CA_5A-7A) |
| CA_1A-19A-21A-42A | CA_1A-19A | - | 3rd order into B42 | - | No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_1A-21A | - | 2nd order into B42 | - | No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_1A-42A | - | 2nd order into B21 5th order into B19 | - | No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_19A-21A | - | 4th order into B42 | - | - No need for IMD (it will covered 3DL CA_19A-21A-42A with 2UL CA_19A-21A) |
|  | CA_19A-42A | - | - | - | N/A |
|  | CA_21A-42A | - | 2nd order into B1 4th order into B19 | - | - No need for IMD (it will covered 3DL CA_19A-21A-42A with 2UL CA_21A-42A) |
| CA_1A-3A-19A-42A | CA_1A-3A | B3 2nd harmonics into B42 | 4th order into B42 | Yes | - No need for IMD (it will covered 3DL CA_1A-3A-42A with 2UL CA_1A-3A) |
|  | CA_1A-19A | - | 3rd order into B42 | - | No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_1A-42A | - | 5th order into B19 | - | No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_3A-19A | B3 2nd harmonics into B42 | 3rd & 5th order into B42 | - | No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_3A-42A | - | - | - | N/A |
|  | CA_19A-42A | - | 3rd order into B3 | - | No need for IMD (it will covered 3DL CA_3A-19A-42A with 2UL CA_19A-42A) |
| CA_1A-3C-8A | CA_1A-3A | - | - | Yes | -Table 7.3.1A-0bA for CA_1A-3A [TS36.101] |
|  | CA_1A-8A | B8 2nd harmonics into B3 | - | - | - Harmonics issues already completed in 2DL/1UL CA_3A-8A |
|  | CA_3A-8A | B8 2nd harmonics into B3 | 4th & 5th order into B3 | - | - Harmonics/IMD issues already completed in 2DL/2UL CA_3A-8A |
|  | CA_3C | - | - | - | N/A |
| CA_1A-3A-42C | CA_1A-3A | B3 2nd harmonics into B42 | 4th order | Yes | - Harmonics issues already completed in 2DL/1UL CA_3A-42A. No need for IMD (it will covered 3DL CA_1A-3A-42A with 2UL CA_1A-3A) |
|  | CA_1A-42A | - | - | - | - N/A |
|  | CA_3A-42A | B3 2nd harmonics into B42 | 4th & 5th order into B42- | - | - Harmonics issues already completed in 2DL/1UL CA_3A-42A. - No need for IMD (it was already covered 2DL/2UL CA_3A-42A) |
| CA_1A-19A-42C | CA_1A-19A | - | 3rd order into B42 | - | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_1A-42A | - | 5th order into B19 | - | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_19A-42A | - | - | - | - N/A |
| CA_1A-21A-42C | CA_1A-21A | - | 2nd order into B42 | - | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_1A-42A | - | 2nd order into B21 | - | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_21A-42A | - | 2nd order into B1 | - | - No MSD due to IMD to be specified according to actural spectrum holdings. |
| CA_3A-19A-42C | CA_3A-19A | B3 2nd harmonic into B42 | 3rd & 5th order into B42 | - | - Harmonics issues already completed in 2DL/1UL CA_3A-42A - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_3A-42A | B3 2nd harmonic into B42 | 4th & 5th order into B42 | - | - Harmonics issues already completed in 2DL/1UL CA_3A-42A No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_19A-42A | - | 3rd order into B3 | - | No MSD due to IMD to be specified according to actural spectrum holdings. |
| CA_19A-21A-42C | CA_19A-21A | - | 4th order into B42 | - | - No need for IMD (it will covered 3DL CA_19A-21A-42A with 2UL CA_19A-21A) |
|  | CA_19A-42A | - | - | - | N/A |
|  | CA_21A-42A | - | 4th order into B19 | - | - No need for IMD (it will covered 3DL CA_19A-21A-42A with 2UL CA_21A-42A) |
| CA_2A-4A-7A-7A | CA_2A-4A | - | - | - | N/A |
| CA_41C-42C | CA_41A-42A | - | - | - | N/A |
|  | CA_41C | - | - | - | N/A |
|  | CA_42C | - | - | - | N/A |
| CA_1A-3A-19A-21A | CA_1A-3A | - | 3rd & 5th order into B21 | Yes | No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_1A-19A | - | - |  | N/A |
|  | CA_1A-21A | - | - |  | N/A |
|  | CA_3A-19A | - | - |  | N/A |
|  | CA_3A-21A | - | 3rd order into B1, 5th order into B19 |  | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_19A-21A | - | 5th order into B3 |  | No need for IMD (it covered 3DL CA_3A-19A-21A with 2UL CA_19A-21A) |
| CA_1A-3A-21A-28A | CA_1A-3A | - | 3rd & 5th order into B21 | Yes | No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_1A-21A | - | - |  | N/A |
|  | CA_1A-28A | B28 2nd harmonics into B21 | 5th order into B3 |  | No MSD due to harmonics to be specified according to actural spectrum holdings. No need for IMD (it covered 3DL CA_1A-3A-28A with 2UL CA_1A-28A) |
|  | CA_3A-21A |  | 3rd order into B1, 5th order into B28 |  | - No MSD due to IMD to be specified according to actural spectrum holdings. - No need for IMD (it covered 3DL CA_3A-21A-28A with 2UL CA_3A-21A) |
|  | CA_3A-28A | B28 2nd harmonics into B21 B28 3rd harmonics into B1 | 4th order into B1 |  | - No MSD due to harmonics to be specified according to actural spectrum holdings. - No need for IMD (it covered 3DL CA_1A-3A-28A with 2UL CA_3A-28A) |
|  | CA_21A-28A | B28 3rd harmonics into B1 | 2nd & 3rd order into B1 |  | - Harmonics issues already completed in 2DL/1UL CA_1A-28A - No MSD due to IMD problems by considering of actural spectrum holdings. |
| CA_1A-3A-21A-42A | CA_1A-3A | B3 2nd harmonics into B42 | 3rd & 5th order into B21, 4th order into B42 | Yes | - Harmonics issues already completed in 2DL/1UL CA_3A-42A. - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_1A-21A | - | 2nd order into B42 |  | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_1A-42A | - | 2nd order into B21 |  | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_3A-21A | B3 2nd harmonics into B42 | 3rd order into B1, |  | - Harmonics issues already completed in 2DL/1UL CA_3A-42A. - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_3A-42A | - | 5th order into B21 |  | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_21A-42A | - | 2nd order into B1 |  | - No MSD due to IMD to be specified according to actural spectrum holdings. |
| CA_1A-3A-28A-42A | CA_1A-3A | B3 2nd harmonics into B42 | 4th order into B42 | Yes | - Harmonics issues already completed in 2DL/1UL CA_3A-42A. - No need for IMD (it covered 3DL CA_1A-3A-42A with 2UL CA_1A-3A) |
|  | CA_1A-28A | - | 5th order into B3, 3rd order into B42 |  | - No need for IMD (it covered 3DL CA_1A-3A-28A with 2UL CA_1A-28A) - No need for IMD (it covered 3DL CA_1A-28A-42A with 2UL CA_1A-28A) |
|  | CA_1A-42A | - | - |  | N/A |
|  | CA_3A-28A | B28 3rd harmonics into B1 B3 2nd harmonics into B42 | 4th order into B1 |  | - Harmonics issues already completed in 2DL/1UL CA_1A-28A & CA_3A-42A - No need for IMD (it covered 3DL CA_1A-3A-28A with 2UL CA_3A-28A) |
|  | CA_3A-42A | - | - |  | N/A |
|  | CA_28A-42A | B28 3rd harmonics into B1 | 3rd order into B1 |  | - Harmonics issues already completed in 2DL/1UL CA_1A-28A - No need for IMD (it covered 3DL CA_1A-28A-42A with 2UL CA_28A-42A) |
| CA_1A-21A-28A-42A | CA_1A-21A | - | 2nd order into B42 | - | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_1A-28A | B28 2nd harmonics | 3rd order into B42 | - | - No MSD due to harmonics to be specified according to actural spectrum holdings. - No need for IMD (it covered 3DL CA_1A-28A-42A with 2UL CA_1A-28A) |
|  | CA_1A-42A | - | 2nd order into B21 | - | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_21A-28A | B28 3rd harmonics into B1 | 2nd & 3rd order into B1 3rd & 4th order into B42 | - | - No MSD due to harmonics/IMD to be specified according to actural spectrum holdings. |
|  | CA_21A-42A | - | 2nd order into B1, 4th order into B28 | - | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_28A-42A | B28 3rd harmonics into B1 B28 2nd harmonics into B21 | 3rd order into B1 | - | - Harmonics issues already completed in 2DL/1UL CA_1A-28A and CA_21A-28A - No need for IMD (it covered 3DL CA_1A-28A-42A with 2UL CA_28A-42A) |
| CA_1A-28A-42C | CA_1A-28A | - | 3rd order into B42 | - | - No need for IMD (it covered 3DL CA_1A-28A-42A with 2UL CA_1A-28A) |
|  | CA_1A-42A | - | - | - | N/A |
|  | CA_28A-42A | B28 3rd harmonics into B1 | 3rd order into B1 | - | - Harmonics issues already completed in 2DL/1UL CA_1A-28A - No need for IMD (it covered 3DL CA_1A-28A-42A with 2UL CA_28A-42A) |
| CA_3A-21A-42C | CA_3A-21A | B3 2nd harmonics into B42 | - | - | - Harmonics issues already completed in 2DL/1UL CA_3A-42A |
|  | CA_3A-42A | - | 5th order into B21 | - | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_21A-42A | - | - | - | N/A |
| CA_3A-28A-42C | CA_3A-28A | B3 2nd harmonics into B42 | - | - | - Harmonics issues already completed in 2DL/1UL CA_3A-42A |
|  | CA_3A-42A | B3 2nd harmonics into B42 | 4th & 5th order into B42 | - | - Harmonics/IMD issues already completed in 2DL/2UL CA_3A-42A |
|  | CA_28A-42A | - | - | - | N/A |
|  | CA_42C | - | - | - | N/A |
| CA_21A-28A-42C | CA_21A-28A | - | 3rd & 4th order into B42 | - | No MSD due to IMD by considering of actural spectrum holdings. |
|  | CA_21A-42A | - | 4th order into B28 | - | No MSD due to IMD by considering of actural spectrum holdings. |
|  | CA_28A-42A | B28 2nd harmonics | - | - | No MSD due to harmonics by considering of actural spectrum holdings. |
| CA_1A-3A-7A-7A | CA_1A-3A | - | - | Yes | N/A |
|  | CA_1A-7A | - | - |  | Table 7.3.1A-0bA for CA_1A-3A [TS36.101] |
|  | CA_3A-7A | - | 4th order into B7 |  | Table 7.3.1A-0bA for CA_1A-3A [TS36.101] - IMD issues already completed in 2DL/2UL CA_3A-7A |
| CA_1A-5A-7A-7A | CA_1A-5A | - | - | - | N/A |
|  | CA_1A-7A | - | 5th order into B5 | - | - No need for IMD (it covered 3DL CA_1A-5A-7A with 2UL CA_1A-7A) |
|  | CA_5A-7A | - | 5th order into B7 | - | - No need for IMD (it already completed in 2DL/2UL CA_5A-7A |
| CA_3A-5A-7A-7A | CA_3A-5A | - | 2nd & 3rd order into B7 | - | - No need for IMD (it covered 3DL CA_3A-5A-7A with 2UL CA_3A-5A) |
|  | CA_3A-7A | - | 3rd order into B5, 4th order into B7 | - | - No need for IMD (it covered 3DL CA_3A-5A-7A with 2UL CA_3A-7A) - No need for IMD (it already completed in 2DL/2UL CA_3A-7A |
|  | CA_5A-7A | - | 5th order into B7 | - | - No need for IMD (it already completed in 2DL/2UL CA_5A-7A |
| CA_3C-7C | CA_3A-7A | - | 4th order into B7 | - | - No need for IMD (it already completed in 2DL/2UL CA_3A-7A |
| CA_3A-41A-42C | CA_41A-42A | - | 3rd & 4th order into B3 | - | - No need for IMD (it covered 3DL CA_3A-41A-42A with 2UL CA_41A-42A) |
|  | CA_42C | - | - | - | N/A |
| CA_3A-41C-42A | CA_41A-42A | - | 3rd & 4th order into B3 | - | - No need for IMD (it covered 3DL CA_3A-41A-42A with 2UL CA_41A-42A) |
| CA_28A-41A-42C | CA_42A-42A | - | 2nd & 5th order into B28 | - | - No need for IMD (it covered 3DL CA_28A-41A-42A with 2UL CA_41A-42A) |
|  | CA_42C | - | - | - | N/A |
| CA_28A-41C-42A | CA_41A-42A | - | 2nd & 5th order into B28 | - | - No need for IMD (it covered 3DL CA_28A-41A-42A with 2UL CA_41A-42A) |
| CA_3A-28A-41A-42A | CA_41A-42A | - | 3rd & 4th order into B3 2nd & 5th order into B28 | - | - No need for IMD (it covered 3DL CA_3A-41A-42A with 2UL CA_41A-42A) - No need for IMD (it covered 3DL CA_28A-41A-42A with 2UL CA_41A-42A) |

Table 5.1.5-3 summarizes the CA_ band combinations with self-interference problems for CA_5DL/2UL CA.

Table 5.1.5-3: Summary of Self-interference analysis for 2UL/5DL CA

| downlink CA configuration | uplink CA Configuration | Harmonic relation to 3rd, 4th and 5th band without uplink | intermodulation to 3rd, 4th and 5th band without uplink | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| CA_1A-19A-21A-42C | CA_1A-19A | - | 3rd order into B42 | - | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_1A-21A | - | 2nd order into B42 |  | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_1A-42A | - | 2nd order into B21 5th order into B19 |  | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_19A-21A | - | 4th order into B42 |  | - No need for IMD (it will covered 3DL CA_19A-21A-42A with 2UL CA_19A-21A) |
|  | CA_19A-42A | - | - |  | N/A |
|  | CA_21A-42A | - | 2nd order into B1 4th order into B19 |  | - No MSD due to IMD to be specified according to actural spectrum holdings. - No need for IMD (it will covered 3DL CA_19A-21A-42A with 2UL CA_21A-42A) |
| CA_1A-3A-19A-42C | CA_1A-3A | B3 2nd harmonics into B42 | 4th order into B42 | Yes | - No MSD due to IMD to be specified according to actural spectrum holdings. - No need for IMD (it will covered 3DL CA_1A-3A-42A with 2UL CA_1A-3A) |
|  | CA_1A-19A | - | 3rd order into B42 |  | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_1A-42A | - | 5th order into B19 |  | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_3A-19A | B3 2nd harmonics into B42 | 3rd & 5th order into B42 |  | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_3A-42A | B3 2nd harmonics into B42 | 4th & 5th order into B42 |  | - No MSD due to IMD to be specified according to actural spectrum holdings. - No need for IMD (it was already covered 2DL/2UL CA_3A-42A |
|  | CA_19A-42A | - | 3rd order into B3 |  | - No MSD due to IMD to be specified according to actural spectrum holdings. |
| CA_1A-3A-21A-42C | CA_1A-3A | B3 2nd harmonics into B42 | 3rd & 5th order into B21, 4th order into B42 | Yes | - Harmonics issues already completed in 2DL/1UL CA_3A-42A. - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_1A-21A | - | 2nd order into B42 |  | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_1A-42A | - | 2nd order into B21 |  | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_3A-21A | B3 2nd harmonics into B42 | 3rd order into B1, |  | - Harmonics issues already completed in 2DL/1UL CA_3A-42A. - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_3A-42A | B3 2nd harmonics into B42 | 4th order into B42 5th order into B21 |  | - Harmonics/IMD issues already completed in 2DL/2UL CA_3A-42A. - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_21A-42A | - | 2nd order into B1 |  | - No MSD due to IMD to be specified according to actural spectrum holdings. |
| CA_1A-3A-28A-42C | CA_1A-3A | B3 2nd harmonics into B42 | 4th order into B42 | Yes | - Harmonics issues already completed in 2DL/1UL CA_3A-42A. - No need for IMD (it covered 3DL CA_1A-3A-42A with 2UL CA_1A-3A) |
|  | CA_1A-28A | - | 5th order into B3, 3rd order into B42 |  | - No need for IMD (it covered 3DL CA_1A-3A-28A with 2UL CA_1A-28A) - No need for IMD (it covered 3DL CA_1A-28A-42A with 2UL CA_1A-28A) |
|  | CA_1A-42A | - | - |  | N/A |
|  | CA_3A-28A | B28 3rd harmonics into B1 B3 2nd harmonics into B42 | 4th order into B1 |  | - Harmonics issues already completed in 2DL/1UL CA_1A-28A & CA_3A-42A - No need for IMD (it covered 3DL CA_1A-3A-28A with 2UL CA_3A-28A) |
|  | CA_3A-42A | B3 2nd harmonics into B42 | 4th order into B42 |  | - Harmonics/IMD issues already completed in 2DL/2UL CA_3A-42A |
|  | CA_28A-42A | B28 3rd harmonics into B1 | 3rd order into B1 |  | - Harmonics issues already completed in 2DL/1UL CA_1A-28A - No need for IMD (it covered 3DL CA_1A-28A-42A with 2UL CA_28A-42A) |
| CA_1A-21A-28A-42C | CA_1A-21A | - | 2nd order into B42 | - | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_1A-28A | B28 2nd harmonics into B21 | 3rd order into B42 | - | - No MSD due to harmonics to be specified according to actural spectrum holdings. - No need for IMD (it covered 3DL CA_1A-28A-42A with 2UL CA_1A-28A) |
|  | CA_1A-42A | - | 2nd order into B21 | - | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_21A-28A | B28 3rd harmonics into B1 | 2nd & 3rd order into B1 3rd & 4th order into B42 | - | - No MSD due to harmonics/IMD to be specified according to actural spectrum holdings. |
|  | CA_21A-42A | - | 2nd order into B1, 4th order into B28 | - | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_28A-42A | B28 3rd harmonics into B1 B28 2nd harmonics into B21 | 3rd order into B1 | - | - Harmonics issues already completed in 2DL/1UL CA_1A-28A and CA_21A-28A - No need for IMD (it covered 3DL CA_1A-28A-42A with 2UL CA_28A-42A) |
| CA_1A-3A-5A-7A-7A | CA_1A-3A | - | - | - | N/A |
|  | CA_1A-5A | - | - | - | N/A |
|  | CA_1A-7A | - | 5th order into B5 | - | - No need for IMD (it was already covered 3DL CA_1A-5A-7A with 2UL CA_1A-7A) |
|  | CA_3A-5A | - | 2nd & 3rd order into B7 | - | - No need for IMD (it will covered 3DL CA_3A-5A-7A with 2UL CA_3A-5A) |
|  | CA_3A-7A | - | 3rd order into B5, 4th order into B7 | - | - No need for IMD (it covered 3DL CA_3A-5A-7A with 2UL CA_3A-7A) - No need for IMD (it already completed in 2DL/2UL CA_3A-7A |
|  | CA_5A-7A | - | 5th order into B7 | - | - No need for IMD (it already completed in 2DL/2UL CA_5A-7A |

Based on the MSD test configuration as summarized in Table 5.1.5.1, 5.1.5.2 & 5.1.5.3, it is proposed to define the following REFSENS exceptions and test configurations in the core specifications for 3DL/2UL inter-band CA with IMD problem as shown in Table 5.1.5.4.

Table 5.1.5.4: Proposed 3DL/2UL inter-band CA REFSENS exceptions and test configurations

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | 3rd DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode |
| CA_1A-3A-40A | CA_1A-3A | 1 | 1950 | 5 | 25 | 2140 | 5 | N/A | TDD-FDD |
|  |  | 3 | 1735 | 5 | 25 | 1830 | 5 |  |  |
|  |  | 40 |  |  |  | 2380 | 5 | 8.0 |  |
| CA_1A-5A-40A | CA_1A-5A | 1 | 1977.5 | 5 | 25 | 2167.5 | 5 | N/A | TDD-FDD |
|  |  | 5 | 826.5 | 5 | 25 | 871.5 | 5 |  |  |
|  |  | 40 |  |  |  | 2305 | 10 | 9.0 |  |
| CA_2A-12A-30A | CA_2A-12A | 2 | 1885 | 5 | 25 | 1965 | 5 | N/A | FDD |
|  |  | 12 | 708.5 | 5 | 25 | 738.5 | 5 |  |  |
|  |  | 30 | 2308 | 5 | 25 | 2353 | 5 | 12.0 |  |
| CA_3A-7A-8A | CA_3A-7A | 3 | 1735 | 5 | 25 | 1830 | 5 | N/A | FDD |
|  |  | 7 | 2530 | 10 | 50 | 2650 | 10 |  |  |
|  |  | 8 | 895 | 5 | 25 | 940 | 5 | TBD |  |
|  | CA_3A-8A | 3 | 1780 | 5 | 25 | 1875 | 5 | N/A | FDD |
|  |  | 8 | 890 | 5 | 25 | 935 | 5 |  |  |
|  |  | 7 | 2550 | 10 | 50 | 2670 | 10 | TBD |  |
| CA_1A-5A-46A | CA_1A-5A | 1 | 1950 | 5 | 25 | 2140 | 5 | N/A | TDD-FDD |
|  |  | 5 | 830 | 5 | 25 | 875 | 5 |  |  |
|  |  | 46 |  |  |  | 5560 | 20 | 1.4 |  |
| CA_1A-7A-46A | CA_1A-7A | 1 | 1950 | 5 | 25 | 2140 | 5 | N/A | TDD-FDD |
|  |  | 7 | 2510 | 10 | 50 | 2630 | 10 |  |  |
|  |  | 46 |  |  |  | 5580 | 20 | 4.4 |  |
| CA_5A-7A-46A | CA_5A-7A | 5 | 845 | 5 | 25 | 890 | 5 | N/A | TDD-FDD |
|  |  | 7 | 2510 | 10 | 50 | 2630 | 10 |  |  |
|  |  | 46 |  |  |  | 5865 | 20 | 12.5 |  |
| CA_1A-3A-42A | CA_1A-3A | 1 | 1922.5 | 5 | 25 | 2112.5 | 5 | N/A | TDD-FDD |
|  |  | 3 | 1782.5 | 5 | 25 | 1877.5 | 5 |  |  |
|  |  | 42 |  |  |  | 3425 | 5 | 13.3 |  |
| CA_3A-5A-7A | CA_3A-5A | 3 | 1780 | 10 | 50 | 1875 | 10 | N/A | FDD |
|  |  | 5 | 845 | 5 | 25 | 890 | 5 |  |  |
|  |  | 7 | 2505 | 10 | 50 | 2625 | 10 | 30.0 |  |
|  | CA_3A-7A | 3 | 1725 | 10 | 50 | 1820 | 10 | N/A | FDD |
|  |  | 7 | 2565 | 10 | 50 | 2685 | 10 |  |  |
|  |  | 5 | 840 | 5 | 25 | 885 | 5 | 19.2 |  |
| CA_3A-19A-21A | CA_19A-21A | 19 | 832.5 | 5 | 25 | 877.5 | 5 | N/A | FDD |
|  |  | 21 | 1460.4 | 5 | 25 | 1508.4 | 5 |  |  |
|  |  | 3 | 1774.6 | 5 | 25 | 1869.6 | 5 | 4.0 |  |
| CA_3A-19A-42A | CA_19A-42A | 19 | 832.5 | 5 | 25 | 877.5 | 5 | N/A | TDD-FDD |
|  |  | 42 | 3517.5 | 5 | 25 | 3517.5 | 5 |  |  |
|  |  | 3 | 1767.4 | 5 | 25 | 1862.4 | 5 | 0 |  |
| CA_19A-21A-42A | CA_19A-21A | 19 | 842.5 | 5 | 25 | 887.5 | 5 | N/A | TDD-FDD |
|  |  | 21 | 1450.4 | 5 | 25 | 1498.4 | 5 |  |  |
|  |  | 42 | 3508.7 | 5 | 25 | 3508.7 | 5 | 12.9 |  |
|  | CA_21A-42A | 21 | 1460.4 | 5 | 25 | 1508.4 | 5 | N/A | TDD-FDD |
|  |  | 42 | 3500 | 5 | 25 | 3500 | 5 |  |  |
|  |  | 19 | 836.2 | 5 | 25 | 881.2 | 5 | 12.5 |  |
| CA_1A-3A-28A | CA_1A-28A | 1 | 1975 | 5 | 25 | 2165 | 5 | N/A | FDD |
|  |  | 28 | 710.5 | 5 | 25 | 765.5 | 5 |  |  |
|  |  | 3 | 1723.5 | 5 | 25 | 1818.5 | 5 | 4.3 |  |
|  | CA_3A-28A | 3 | 1780 | 5 | 25 | 1875 | 5 | N/A | FDD |
|  |  | 28 | 710.5 | 5 | 25 | 765.5 | 5 |  |  |
|  |  | 1 | 1949 | 5 | 25 | 2139 | 5 | 10.7 |  |
| CA_1A-28A-42A | CA_1A-28A | 1 | 1955 | 5 | 25 | 2145 | 5 | N/A | TDD-FDD |
|  |  | 28 | 735 | 5 | 25 | 790 | 5 |  |  |
|  |  | 42 | 3425 | 5 | 25 | 3425 | 5 | 15.4 |  |
|  | CA_28A-42A | 28 | 710.5 | 5 | 25 | 765.5 | 5 | N/A | TDD-FDD |
|  |  | 42 | 3560 | 5 | 25 | 3560 | 5 |  |  |
|  |  | 1 | 1949 | 5 | 25 | 2139 | 5 | 11.3 |  |
| CA_3A-21A-28A | CA_3A-21A | 3 | 1782 | 5 | 25 | 1877 | 5 | N/A | FDD |
|  |  | 21 | 1451 | 5 | 25 | 1499 | 5 |  |  |
|  |  | 28 | 734 | 5 | 25 | 789 | 5 | 2.9 |  |
| CA_3A-41A-42A | CA_41A-42A | 41 | 2640 | 10 | 50 | 2640 | 10 | N/A | FDD-TDD |
|  |  | 42 | 3425 | 10 | 50 | 3425 | 10 |  |  |
|  |  | 3 | 1760 | 5 | 25 | 1855 | 5 | TBD |  |
| CA_28A-41A-42A | CA_41A-42A | 41 | 2672 | 10 | 50 | 2672 | 10 | N/A | FDD-TDD |
|  |  | 42 | 3460 | 10 | 50 | 3460 | 10 |  |  |
|  |  | 28 | 733 | 5 | 25 | 788 | 5 | TBD |  |

The required MSD levels are derived by taking average values of proposed MSD levels from interested companies. If there are outliers, the sources for differences will be discussed and applicability for averaging considered case by case.

5.2	RRM specific

[Editor Note] It will be updated in future

6	3DLs/2ULs Inter-Band Carrier Aggregation: Specific Band Combination Part

6.1	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 7 with 2 ULs

6.1.1	List of specific combination issues

6.1.1.1	Channel bandwidths per operating band for CA

Table 6.1.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-3A-7A | CA_1A-3A, CA_1A-7A, CA_3A-7A | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |

6.1.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-7A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.1.2-1

Table 6.1.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 270 | 135 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2055 | 2250 | 1440 | 1650 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5550 | 5745 | 5340 | 5550 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3975 | 4230 | 3150 | 3435 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7470 | 7725 | 7050 | 7335 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 270 | 540 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5220 | 4860 | 6210 | 5895 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8760 | 9120 | 9390 | 9705 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1515 | 1170 | 2520 | 2190 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 | 9315 | 9180 | 9510 |

There are no harmonic or IMD products from band 1 and band 3 uplinks which would desensitize band 7 downlink.

6.1.1.3	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-3A-7A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.1.3-1

Table 6.1.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 520 | 650 | 4420 | 4550 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1270 | 1460 | 3020 | 3220 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6340 | 6530 | 6920 | 7120 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3190 | 3440 | 5520 | 5790 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8260 | 8510 | 9420 | 9690 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1300 | 1040 | 8840 | 9100 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8360 | 8020 | 5420 | 5110 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11920 | 12260 | 10180 | 10490 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3870 | 3540 | 940 | 620 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11340 | 11670 | 10760 | 11080 |

There are no harmonic or IMD products from band 1 and band 7 uplinks which would desensitize band 3 downlink.

6.1.1.4	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_1A-3A-7A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.1.4-1

Table 6.1.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 715 | 860 | 4210 | 4355 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 850 | 1070 | 3215 | 3430 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5920 | 6140 | 6710 | 6925 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2560 | 2855 | 5715 | 6000 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7630 | 7925 | 9210 | 9495 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1720 | 1430 | 8420 | 8710 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8570 | 8215 | 4640 | 4270 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11710 | 12065 | 9340 | 9710 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4290 | 3930 | 355 | 10 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10920 | 11280 | 10130 | 10495 |

There are no harmonic or IMD products from band 3 and band 7 uplinks which would desensitize band 1 downlink.

6.1.1.5	MSD

When uplink CA configurations CA_1A-3A, CA_1A-7A and CA_3A-7A are paired with downlink CA configuration CA_1A-3A-5A there are no interference components from 2 uplink operation which would interfere the downlink of the third band.

6.1.1.6	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.2	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 40 with 2 ULs

6.2.1	List of specific combination issues

6.2.1.1	Channel bandwidths per operating band for CA

Table 6.2.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-3A-40A | CA_1A-3A | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes |  |  |

6.2.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-40A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.2.1.2-1

Table 6.2.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 135 | 270 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1440 | 1650 | 2055 | 2250 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5340 | 5550 | 5550 | 5745 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3150 | 3435 | 3975 | 4230 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7050 | 7335 | 7470 | 7725 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 540 | 270 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6210 | 5895 | 5220 | 4860 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9390 | 9705 | 8760 | 9120 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2520 | 2190 | 1515 | 1170 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9180 | 9510 | 8970 | 9315 |

The 3rd IMD will interfere own Rx frequency of Band 1. But this issue has covered in 2UL/2DL CA in rel-12.

The 5th IMD will interfere own Rx frequency of Band 40. It will be analysed to slove the self- interference problem.

6.2.1.3	MSD

When uplink CA configurations CA_1A-3A is paired with downlink CA configuration CA_1A-3A-40A there is self interference problem from 2 uplink operation which would interfere the downlink of the Band 40.

6.2.1.4	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.3	LTE Advanced Carrier Aggregation: Band 1 and Band 5 and Band 40 with 2 ULs

6.3.1	List of specific combination issues

6.3.1.1	Channel bandwidths per operating band for CA

Table 6.3.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-5A-40A | CA_1A-5A | 1 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 40 |  |  |  | Yes | Yes | Yes |  |  |

6.3.1.2	Co-existence studies for LTE-A UL CA_1A-5A and DL CA_1A-5A-40A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.3.1.2-1

Table 6.3.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1071 | 1156 | 2744 | 2829 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 332 | 222 | 2991 | 3136 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3568 | 3678 | 4664 | 4809 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 492 | 627 | 4911 | 5116 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4392 | 4527 | 6584 | 6789 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2312 | 2142 | 5488 | 5658 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7096 | 6831 | 1476 | 1316 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8504 | 8769 | 5216 | 5376 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4292 | 4062 | 1293 | 1488 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7408 | 7638 | 6312 | 6507 |

The 4th IMD fall into the own Rx frequency of Band 1 and Band 40. But the IMD problem of Band 1 has covered in 2UL/2DL CA_1A-5A in rel-12.

The 4th IMD impacts of Band 40 will be analysed to slove the self- interference problem.

6.3.1.3	MSD

When uplink CA configurations CA_1A-5A is paired with downlink CA configuration CA_1A-5A-40A there is self interference problem from 2 uplink operation which would interfere the downlink of the Band 40.

6.3.1.4	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.4	LTE Advanced Carrier Aggregation: Band 1 and Band 7 and Band 8 with 2 ULs

6.4.1	List of specific combination issues

6.4.1.1	Channel bandwidths per operating band for CA

Table 6.4.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-7A-8A | CA_1A-7A or CA_1A-8A | 1 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |

6.4.1.2	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-7A-8A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.4.1.2-1

Table 6.4.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 520 | 650 | 4420 | 4550 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1270 | 1460 | 3020 | 3220 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6340 | 6530 | 6920 | 7120 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3190 | 3440 | 5520 | 5790 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8260 | 8510 | 9420 | 9690 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1300 | 1040 | 8840 | 9100 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8360 | 8020 | 5420 | 5110 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11920 | 12260 | 10180 | 10490 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3870 | 3540 | 940 | 620 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11340 | 11670 | 10760 | 11080 |

The 5th IMD fall into the own Rx frequency of Band 8. It will be analysed to slove the self- interference problem.

6.4.1.3	Co-existence studies for LTE-A UL CA_1A-8A and DL CA_1A-7A-8A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.4.1.3-1

Table 6.4.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1005 | 1100 | 2800 | 2895 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 220 | 90 | 2925 | 3080 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3680 | 3810 | 4720 | 4875 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 660 | 825 | 4845 | 5060 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4560 | 4725 | 6640 | 6855 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2200 | 2010 | 5600 | 5790 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7040 | 6765 | 1740 | 1540 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8560 | 8835 | 5440 | 5640 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4180 | 3930 | 1095 | 1320 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7520 | 7770 | 6480 | 6705 |

The 4th IMD fall into the own Rx frequency of Band 8. But this issue has covered 2UL/2DL CA in Rel-12.

The 3rd Harmonics will interfere own Rx frequency of Band 7. It will be analysed to slove the self- interference problem.

6.4.1.4	MSD

When uplink CA configurations CA_1A-7A is paired with downlink CA configuration CA_1A-7A-8A there is self interference problem from 2 uplink operation which would interfere the downlink of the Band 8.

And also when uplink CA configurations CA_1A-8A is paired with downlink CA configuration CA_1A-7A-8A there is self interference problem from 2 uplink operation which would interfere the downlink of the Band 7.

6.4.1.5	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.5	LTE Advanced Carrier Aggregation: Band 1 and Band 8 and Band 40 with 2 ULs

6.5.1	List of specific combination issues

6.5.1.1	Channel bandwidths per operating band for CA

Table 6.5.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-8A-40A | CA_1A-8A | 1 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 8 |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes |  |  |

6.5.1.2	Co-existence studies for LTE-A UL CA_1A-8A and DL CA_1A-8A-40A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.5.1.2-1

Table 6.5.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1005 | 1100 | 2800 | 2895 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 220 | 90 | 2925 | 3080 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3680 | 3810 | 4720 | 4875 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 660 | 825 | 4845 | 5060 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4560 | 4725 | 6640 | 6855 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2200 | 2010 | 5600 | 5790 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7040 | 6765 | 1740 | 1540 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8560 | 8835 | 5440 | 5640 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4180 | 3930 | 1095 | 1320 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7520 | 7770 | 6480 | 6705 |

The 4th IMD fall into the own Rx frequency of Band 8. But this issue has covered 2UL/2DL CA in Rel-12.

There are no harmonic or IMD products from Band 1 and Band 8 uplinks which would desensitize Band 40 downlink.

6.5.1.3	MSD

When uplink CA configurations CA_1A-8A is paired with downlink CA configuration CA_1A-8A-40A there is no interference components from 2 uplink operation which would interfere the downlink of the third band.

6.5.1.4	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.6	LTE Advanced Carrier Aggregation: Band 1 and Band 42 and Band 42 with 2 ULs

6.6.1	List of specific combination issues

6.6.1.1	Channel bandwidths per operating band for CA

Table 6.6.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-42C | CA_1A-42A | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | CA_42C | See CA_42C BCS0 in Table 6.6.1.1-2 |  |  |  |  |  |  |  |

Table 6.6.1.1-2: E-UTRA CA_42C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_42C | 5, 10, 15, 20 | 20 | 40 | 0 |
|  | 20 | 5, 10, 15 |  |  |

6.6.1.2	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-42C

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.6.1.2-1

Table 6.6.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1420 | 1680 | 5320 | 5580 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 240 | 560 | 4820 | 5280 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7240 | 7560 | 8720 | 9180 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2160 | 2540 | 8220 | 8880 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9160 | 9540 | 12120 | 12780 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3360 | 2840 | 10640 | 11160 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12480 | 11620 | 4520 | 4080 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15520 | 16380 | 11080 | 11520 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6960 | 6240 | 860 | 1440 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14040 | 14760 | 12560 | 13140 |

The 4th IMD fall into the own Rx frequency of Band 1. In rel-12, the IMD impact is not study since there was no RB allocation in the 1920~1925MHz to protect PHS band. But RAN4 has new A-MPR requirements for Band 1 to protect PHS band without any RB restriction in 1920~1925MHz. So the 4th IMD impact can be analysed when A-MPR is applied for CA_1A-42A. However, when assuming the Tx-Rx separation of 190MHz is always kept in band 1, the 4th IMD never impacts the received signal of band 1. Therefore, it is concluded that there is no IMD and harmonic issue for own Rx frequency range as well as CA_1A-42A case.

6.6.1.3	MSD

When uplink CA configurations CA_1A-42A is paired with downlink CA configuration CA_1A-42C there is no interference components from 2 uplink operation which would interfere the downlink of the Band 1.

6.6.1.4	∆TIB and ∆RIB values

[[Editor Note] It will be updated in future

6.7	LTE Advanced Carrier Aggregation: Band 2 and Band 4 and Band 5 with 2 ULs

6.7.1	List of specific combination issues

6.7.1.1	Channel bandwidths per operating band for CA

Table 6.7.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_2A-4A-5A | CA_2A-4A | 2 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |

6.7.1.2	Co-existence studies for LTE-A UL CA_2A-4A and DL CA_2A-4A-5A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.7.1.2-1

Table 6.7.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1755 | 1850 | 1910 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3510 | 3700 | 3820 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5265 | 5550 | 5730 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 95 | 200 | 3560 | 3665 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1510 | 1660 | 1945 | 2110 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5270 | 5420 | 5410 | 5575 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3220 | 3415 | 3795 | 4020 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6980 | 7175 | 7260 | 7485 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 400 | 190 | 7120 | 7330 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5930 | 5645 | 5170 | 4930 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9110 | 9395 | 8690 | 8930 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2310 | 2040 | 1565 | 1310 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 | 9240 | 8830 | 9085 |

The 3rd & 5th IMDs fall into the own Rx frequency of Band 2 and Band 4. But these impacts already covered in 2UL/2DL CA in Rel-12.

There are no harmonic or IMD products from Band 2 and Band 4 uplinks which would desensitize Band 5 downlink.

6.7.1.3	MSD

When uplink CA configurations CA_2A-4A is paired with downlink CA configuration CA_2A-4A-5A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 5.

6.7.1.4	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.8	LTE Advanced Carrier Aggregation: Band 2 and Band 4 and Band 29 with 2 ULs

6.8.1	List of specific combination issues

6.8.1.1	Channel bandwidths per operating band for CA

Table 6.8.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_2A-4A-29A | CA_2A-4A | 2 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |

6.8.1.2	Co-existence studies for LTE-A UL CA_2A-4A and DL CA_2A-4A-29A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.8.1.2-1

Table 6.8.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1755 | 1850 | 1910 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3510 | 3700 | 3820 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5265 | 5550 | 5730 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 95 | 200 | 3560 | 3665 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1510 | 1660 | 1945 | 2110 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5270 | 5420 | 5410 | 5575 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3220 | 3415 | 3795 | 4020 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6980 | 7175 | 7260 | 7485 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 400 | 190 | 7120 | 7330 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5930 | 5645 | 5170 | 4930 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9110 | 9395 | 8690 | 8930 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2310 | 2040 | 1565 | 1310 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 | 9240 | 8830 | 9085 |

The 3rd & 5th IMDs fall into the own Rx frequency of Band 2 and Band 4. But these impacts already covered in 2UL/2DL CA in Rel-12.

There are no harmonic or IMD products from Band 2 and Band 4 uplinks which would desensitize Band 29 downlink.

6.8.1.3	MSD

When uplink CA configurations CA_2A-4A is paired with downlink CA configuration CA_2A-4A-29A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 29.

6.8.1.4	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.9	LTE Advanced Carrier Aggregation: Band 2 and Band 12 and Band 30 with 2 ULs

6.9.1	List of specific combination issues

6.9.1.1	Channel bandwidths per operating band for CA

Table 6.9.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_2A-12A-30A | CA_2A-12A | 2 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |

6.9.1.2	Co-existence studies for LTE-A UL CA_2A-12A and DL CA_2A-12A-30A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.9.1.2-1

Table 6.9.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 698 | 716 | 1850 | 1910 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1396 | 1432 | 3700 | 3820 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2094 | 2148 | 5550 | 5730 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1134 | 1212 | 2548 | 2626 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 514 | 418 | 2984 | 3122 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3246 | 3342 | 4398 | 4536 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 184 | 298 | 4834 | 5032 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3944 | 4058 | 6248 | 6446 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2424 | 2268 | 5096 | 5252 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6942 | 6684 | 1014 | 882 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8098 | 8356 | 4642 | 4774 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4334 | 4118 | 1552 | 1726 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6946 | 7162 | 5794 | 5968 |

The 4th IMD will interfere own Rx frequency of Band 30. It will be analysed to slove the self- interference problem.

6.9.1.3	MSD

When uplink CA configurations CA_2A-12A is paired with downlink CA configuration CA_2A-12A-30A there is interference components from 2 uplink operation which would interfere the downlink of the Band 30.

6.9.1.4	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.10	LTE Advanced Carrier Aggregation: Band 2 and Band 12 and Band 12 with 2 ULs

6.10.1	List of specific combination issues

6.10.1.1	Channel bandwidths per operating band for CA

Table 6.10.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_2A-12B | CA_2A-12A | 2 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  |  | CA_12B | See CA_12B BCS0 in Table 6.10.1.1-2 |  |  |  |  |  |  |  |

Table 6.10.1.1-2: E-UTRA CA_12B configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_12B | 5 | 5, 10 | 15 | 0 |

6.10.1.2	Co-existence studies for LTE-A UL CA_2A-12A and DL CA_2A-12B

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.10.1.2-1

Table 6.10.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 698 | 716 | 1850 | 1910 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1396 | 1432 | 3700 | 3820 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2094 | 2148 | 5550 | 5730 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1134 | 1212 | 2548 | 2626 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 514 | 418 | 2984 | 3122 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3246 | 3342 | 4398 | 4536 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 184 | 298 | 4834 | 5032 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3944 | 4058 | 6248 | 6446 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2424 | 2268 | 5096 | 5252 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6942 | 6684 | 1014 | 882 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8098 | 8356 | 4642 | 4774 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4334 | 4118 | 1552 | 1726 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6946 | 7162 | 5794 | 5968 |

There are no harmonic or IMD products from Band 2 and Band 12 uplinks which would desensitize Band 12 downlink.

6.10.1.3	MSD

When uplink CA configurations CA_2A-12A is paired with downlink CA configuration CA_2A-12B there is no interference components from 2 uplink operation which would interfere the downlink of the Band 12.

6.10.1.4	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.11	LTE Advanced Carrier Aggregation: Band 3 and Band 5 and Band 40 with 2 ULs

6.11.1	List of specific combination issues

6.11.1.1	Channel bandwidths per operating band for CA

Table 6.11.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A-5A-40A | CA_3A-5A | 3 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 40 |  |  |  | Yes | Yes | Yes |  |  |

6.11.1.2	Co-existence studies for LTE-A UL CA_3A-5A and DL CA_3A-5A-40A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.11.1.2-1

Table 6.11.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 861 | 961 | 2534 | 2634 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 137 | 12 | 2571 | 2746 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3358 | 3483 | 4244 | 4419 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 687 | 837 | 4281 | 4531 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4182 | 4332 | 5954 | 6204 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1922 | 1722 | 5068 | 5268 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6316 | 5991 | 1686 | 1511 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7664 | 7989 | 5006 | 5181 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3707 | 3432 | 873 | 1098 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6778 | 7053 | 5892 | 6117 |

The 2nd &5th IMDs fall into the own Rx frequency of Band 5. And also the 4th IMD fall into the own Rx frequency of Band 3. But these impacts already covered in 2UL/2DL CA in Rel-12.

There are no harmonic or IMD products from Band 3 and Band 5 uplinks which would desensitize Band 40 downlink.

6.11.1.3	MSD

When uplink CA configurations CA_3A-5A is paired with downlink CA configuration CA_3A-5A-40A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 40.

6.11.1.4	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.12	LTE Advanced Carrier Aggregation: Band 3 and Band 7 and Band 8 with 2 ULs

6.12.1	List of specific combination issues

6.12.1.1	Channel bandwidths per operating band for CA

Table 6.12.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A-7A-8A | CA_3A-7A or CA_3A-8A or CA_7A-8A | 3 |  |  | Yes | Yes | Yes |  | 40 | 0 |
|  |  | 7 |  |  |  | Yes | Yes |  |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes | 50 | 1 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes | 50 | 2 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |

6.12.1.2	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_3A-7A-8A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.12.1.2-1

Table 6.12.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 715 | 860 | 4210 | 4355 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 850 | 1070 | 3215 | 3430 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5920 | 6140 | 6710 | 6925 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2560 | 2855 | 5715 | 6000 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7630 | 7925 | 9210 | 9495 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1720 | 1430 | 8420 | 8710 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8570 | 8215 | 4640 | 4270 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11710 | 12065 | 9340 | 9710 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4290 | 3930 | 355 | 10 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10920 | 11280 | 10130 | 10495 |

The 4th IMD fall into the own Rx frequency of Band 7. But this impact already covered in 2UL/2DL CA in Rel-12.

The 3rd IMD will interfere own Rx frequency of Band 8. It will be analysed to slove the self- interference problem.

6.12.1.3	Co-existence studies for LTE-A UL CA_3A-8A and DL CA_3A-7A-8A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.12.1.3-1

Table 6.12.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 795 | 905 | 2590 | 2700 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 25 | 120 | 2505 | 2690 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3470 | 3615 | 4300 | 4485 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 855 | 1035 | 4215 | 4475 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4350 | 4530 | 6010 | 6270 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1810 | 1590 | 5180 | 5400 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6260 | 5925 | 1950 | 1735 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7720 | 8055 | 5230 | 5445 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3595 | 3300 | 675 | 930 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6890 | 7185 | 6060 | 6315 |

The 2nd harmonics of Band 8, 4th & 5th IMDs fall into the own Rx frequency of Band 3. And also the 4th & 5th IMDs fall into the own Rx frequency of Band 8. But these impacts already covered in 2UL/2DL CA in Rel-12 using operator specific own spectrum allocation in this example is 1735 - 1755 / 1830 - 1850 MHz in Band 3 and 905 - 915 / 950 - 960 MHz in Band 8. Hence only 4th IMD problem in Band 3 has covered in rel-12 specification.

And also, the 3rd harmonics of Band 8 and 2nd &3rd IMDs will interfere own Rx frequency of Band 7. It will be analysed to slove the self- interference problem.

6.12.1.4	Co-existence studies for LTE-A UL CA_7A-8A and DL CA_3A-7A-8A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.12.1.4-1

Table 6.12.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1585 | 1690 | 3380 | 3485 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 810 | 670 | 4085 | 4260 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4260 | 4400 | 5880 | 6055 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 70 | 245 | 6585 | 6830 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5140 | 5315 | 8380 | 8625 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3380 | 3170 | 6760 | 6970 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9400 | 9085 | 1160 | 950 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10880 | 11195 | 6020 | 6230 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5950 | 5670 | 2255 | 2500 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9260 | 9540 | 7640 | 7885 |

The 2nd harmonics of Band 8 fall into the own Rx frequency of Band 3. But these impacts already covered in 2DL/1UL CA_3A-8A. Also the 3rd harmonics of Band 8 impacted to the own Rx frequency of Band 7. Also the 5th IMD fall into the own Rx frequency of Band 8. However this impact already covered in 2DL/2UL CA_7A-8A. Hence there was no self-interference issues.

6.12.1.5	MSD

When uplink CA configurations CA_3A-7A is paired with downlink CA configuration CA_3A-7A-8A there is interference components from 2 uplink operation which would interfere the downlink of the Band 8.

When uplink CA configurations CA_3A-8A is paired with downlink CA configuration CA_3A-7A-8A there is interference components from 2 uplink operation which would interfere the downlink of the Band 7.

And also when uplink CA configurations CA_7A-8A is paired with downlink CA configuration CA_3A-7A-8A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 3.

Figure 6.12.1.5-1shows the RF front-end architecture for the MSD analysis. Another alternative would be common triplexer based architecture. There is a harmonic trap filter assumed in B8 path for this CA case because harmonic trap was assumed for B8+B7 1UL/2DL CA and for B3+B7+B8 1UL/3DL CA.

Figure 6.12.1.5-1 Common diplexer architecture for CA_3A-7A-8A 3DL with CA_3A-7A and CA_3A-8A 2UL CA

Table 6.12.1.5-1shows the MSD analysis for IMD3 interference landing on band 8 DL in CA_3A-7A-8A 3DL with CA_3A-7A 2UL CA.

Table 6.12.1.5-1: MSD analysis for IMD3 of CA_3A-7A-8A 3DL with CA_3A-7A 2UL

| Reference | UL CA | IMD |  | UL Fc [MHz] | UL BW [MHz] | UL [RB] | DL Band | IMD center [MHz] | DL BW [MHz] | CF [dB] | MSD [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R4-168860 by Huawei, Hisilicon | B3 | IMD3 | 2*fB3 - fB7 | 1735 | 5 | 25 | B8 | 940.0 | 5 | 0.62 | 21.6 |
|  | B7 |  |  | 2530 | 10 | 50 |  |  |  |  |  |
| R4-165757 by LGE | B3 | IMD3 | 2*fB3 - fB7 | 1735 | 5 | 25 | B8 | 940.0 | 5 | 1.2 | 14.1 |
|  | B7 |  |  | 2530 | 10 | 50 |  |  |  |  |  |

Table 6.12.1.5-2, 6.12.1.5-3, 6.12.1.5-4 are the MSD analyses for IMD2, IMD3 and the overlapped IMD2+IMD3 interference landing on band 7 DL in CA_3A-7A-8A 3DL with CA_3A-8A 2UL CA. Note that for the overlapped IMD2+IMD3 case, the B8 third harmonic will also fall on top of the B7 DL block, but the impact of the third harmonic is negligible because it is more than 20dB below the MSD due to IMD2 and IMD3. Thus “H3” is not included in the MSD table.

Table 6.12.1.5-2: MSD analysis for IMD2 of CA_3A-7A-8A 3DL with CA_3A-8A 2UL

| Reference | UL CA | IMD |  | UL Fc [MHz] | UL BW [MHz] | UL [RB] | DL Band | IMD center [MHz] | DL BW [MHz] | CF [dB] | MSD [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R4-168860 by Huawei, Hisilicon | B3 | IMD2 | fB3 + fB8 | 1735 | 5 | 25 | B7 | 2635.0 | 10 | 0 | 32.0 |
|  | B8 |  |  | 900 | 5 | 25 |  |  |  |  |  |
| R4-165757 by LGE | B3 | IMD2 | fB3 + fB8 | 1735 | 5 | 25 | B7 | 2635.0 | 10 | 1.0 | 25.1 |
|  | B8 |  |  | 900 | 5 | 25 |  |  |  |  |  |

Table 6.12.1.5-3: MSD analysis for IMD3 of CA_3A-7A-8A 3DL with CA_3A-8A 2UL

| Reference | UL CA | IMD |  | UL Fc [MHz] | UL BW [MHz] | UL [RB] | DL Band | IMD center [MHz] | DL BW [MHz] | CF [dB] | MSD [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R4-168860 by Huawei, Hisilicon | B3 | IMD3 | 2*fB3 - fB8 | 1772.5 | 5 | 25 | B7 | 2635.0 | 10 | 0 | 22.1 |
|  | B8 |  |  | 910 | 5 | 25 |  |  |  |  |  |
| R4-165757 by LGE | B3 | IMD3 | 2*fB3 - fB8 | 1772.5 | 5 | 25 | B7 | 2635.0 | 10 | 1.8 | 12.9 |
|  | B8 |  |  | 910 | 5 | 25 |  |  |  |  |  |

Table 6.12.1.5-4: MSD analysis for overlapped IMD2+IMD3 of CA_3A-7A-8A 3DL with CA_3A-8A 2UL

| Reference | UL CA | IMD |  | UL Fc [MHz] | UL BW [MHz] | UL [RB] | DL Band | IMD center [MHz] | DL BW [MHz] | CF [dB] | MSD [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R4-168860 by Huawei, Hisilicon | B3 | IMD2+ IMD3 | fB3 + fB8, 2*fB3 - fB8 | 1780 | 5 | 25 | B7 | 2670 | 10 | 0 | 32.5 |
|  | B8 |  |  | 890 | 5 | 25 |  |  |  |  |  |

For this combination, there exists an additional scenario where IMD2 and IMD3 from B3+B8 both hit on top of B7 DL but they do not overlap. The MSD analyses are shown in table 6.12.1.5-5. Note that this is an example calculation with UL RB allocations wider than in respective single band RFESENS and these IMD’s don’t include CF’s that will be fairly significant in this case.

Table 6.12.1.5-5: MSD analysis for non-overlapped IMD2+IMD3 of CA_3A-7A-8A 3DL with CA_3A-8A 2UL from R4-168800 (proposed by Huawei, Hisilicon)

| DL CA | UL CA | IMD |  | UL Fc [MHz] | UL BW [MHz] | UL [RB] | DL Band | IMD center [MHz] | DL BW [MHz] | MSD [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3+7+8 | B3 | IMD2 | fB3 + fB8 | 1762.5 | 15 | 75 | B7 | 2650.0 | 20 | 29.0 |
|  | B8 |  |  | 900 | 10 | 50 |  |  |  |  |
|  | B3 | IMD3 | 2*fB3 - fB8 | 1762.5 | 15 | 75 | B7 | 2650.0 | 20 | 19.2 |
|  | B8 |  |  | 900 | 10 | 50 |  |  |  |  |
|  | B3 | IMD2+ IMD3 | fB3 + fB8, 2*fB3 - fB8 | 1762.5 | 15 | 75 | B7 | 2650 | 20 | 29.5 |
|  | B8 |  |  | 900 | 10 | 50 |  |  |  |  |

When considering which test point should be defined in the standard, it was agreed to define the IMD3 case for the CA_3A-7A-8A 3DL paired with CA_3A-7A 2UL CA, and define the overlapped IMD2 + IMD3 case for the CA_3A-7A-8A 3DL paired with CA_3A-8A 2UL CA. Table 6.12.1.5-6 shows the merged MSD result from the interested companies, and table 6.12.1.5-7 is the MSD summery for this combination.

Table 6.12.1.5-6: merged MSD results from interested companies

| DL CA | UL CA | IMD |  | Qualcomm | LGE | Huawei | Avg. MSD [dB] | Proposed MSD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B3+B7+B8 | B3 | IMD3 | 2*fB3 - fB7 | TBD | 14.1 | 21.6 | TBD | TBD |
|  | B7 |  |  |  |  |  |  |  |
|  | B3 | IMD2+IMD3 | fB3 + fB8, 2*fB3 - fB8 | TBD | 25.5 | 32.5 | TBD | TBD |
|  | B8 |  |  |  |  |  |  |  |

Table 6.12.1.5-7: MSD summary for CA_3A-7A-8A 3DL/2UL CA

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | 3rd DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode |
| CA_3A-7A-8A | CA_3A-7A | 3 | 1735 | 5 | 25 | 1830 | 5 | N/A | FDD |
|  |  | 7 | 2530 | 10 | 50 | 2650 | 10 |  |  |
|  |  | 8 | 895 | 5 | 25 | 940 | 5 | TBD |  |
|  | CA_3A-8A | 3 | 1780 | 5 | 25 | 1875 | 5 | N/A | FDD |
|  |  | 8 | 890 | 5 | 25 | 935 | 5 |  |  |
|  |  | 7 | 2550 | 10 | 50 | 2670 | 10 | TBD |  |

6.12.1.6	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.13	LTE Advanced Carrier Aggregation: Band 3 and Band 8 and Band 40 with 2 ULs

6.13.1	List of specific combination issues

6.13.1.1	Channel bandwidths per operating band for CA

Table 6.13.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A-8A-40A | CA_3A-8A | 3 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 8 |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes |  |  |

6.13.1.2	Co-existence studies for LTE-A UL CA_3A-8A and DL CA_3A-8A-40A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.13.1.2-1

Table 6.13.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 795 | 905 | 2590 | 2700 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 25 | 120 | 2505 | 2690 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3470 | 3615 | 4300 | 4485 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 855 | 1035 | 4215 | 4475 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4350 | 4530 | 6010 | 6270 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1810 | 1590 | 5180 | 5400 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6260 | 5925 | 1950 | 1735 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7720 | 8055 | 5230 | 5445 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3595 | 3300 | 675 | 930 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6890 | 7185 | 6060 | 6315 |

The 2nd harmonics of Band 8, 4th & 5th IMDs fall into the own Rx frequency of Band 3. And also the 4th & 5th IMDs fall into the own Rx frequency of Band 8. But these impacts already covered in 2UL/2DL CA in Rel-12 using operator specific own spectrum allocation in this example is 1735 - 1755 / 1830 - 1850 MHz in Band 3 and 905 - 915 / 950 - 960 MHz in Band 8. Hence only 4th IMD problem in Band 3 has covered in rel-12 specification.

There are no harmonics or IMD products from Band 3 and Band 8 uplinks which would desensitize Band 40 downlink.

6.13.1.3	MSD

When uplink CA configurations CA_3A-8A is paired with downlink CA configuration CA_3A-8A-40A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 40.

6.13.1.4	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.14	LTE Advanced Carrier Aggregation: Band 3 and Band 42 and Band 42 with 2 ULs

6.14.1	List of specific combination issues

6.14.1.1	Channel bandwidths per operating band for CA

Table 6.14.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A-42C | CA_3A-42A, CA_42C | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | CA_42C | See CA_42C BCS0 in Table 6.14.1.1-2 |  |  |  |  |  |  |  |

Table 6.14.1.1-2: E-UTRA CA_42C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_42C | 5, 10, 15, 20 | 20 | 40 | 0 |
|  | 20 | 5, 10, 15 |  |  |

6.14.1.2	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_3A-42C

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.14.1.2-1

Table 6.14.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1615 | 1890 | 5110 | 5385 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 180 | 170 | 5015 | 5490 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6820 | 7170 | 8510 | 8985 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1530 | 1955 | 8415 | 9090 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8530 | 8955 | 11910 | 12585 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3780 | 3230 | 10220 | 10770 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12690 | 11815 | 3740 | 3240 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15310 | 16185 | 10240 | 10740 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7380 | 6630 | 1445 | 2070 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13620 | 14370 | 11930 | 12555 |

The 2nd harmonics of Band 3falls into the own Rx frequency of Band 42. And also the 2nd, 4th & 5th IMDs fall into the own Rx frequency of Band 3. In addition, 2nd and 3rd order IMD and harmonics may also fall into Rx frequencies of bands 9, 22, 35, 39 and 43. But these impacts are already covered in 2UL/2DL CA_3A-42A in Rel-14 TR36.714-02-02.  Impacts for ISM and GNSS bands are also already captured in Rel-14 TR 36.714-02-02.

There are no harmonics or IMD products from Band 3 and Band 42 uplinks which would desensitize Band 42 downlink.

6.14.1.3	Co-existence studies for LTE-A UL CA_42C and DL CA_3A-42C

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.14.1.3-1

Table 6.14.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high |
| --- | --- | --- |
| UL frequency (MHz) | 3400 | 3600 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 6800 | 7200 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 10200 | 10800 |

From the harmonics analysis table, there was no harmonics from Band 42 uplinks which would desensitize the 3rd own Rx frequency of Band 3. Frequency separation of band 3 and 42 is 1520 MHz therefore there is no impact from Band 42 uplink CA to Band 3 reception evenif RAN4 consider asynchronization scenario.

6.14.1.4	MSD

When uplink CA configurations CA_3A-42A is paired with downlink CA configuration CA_3A-42C there is no interference components from 2 uplink operation which would interfere the additional (second) downlink of the Band 42.

When uplink CA configurations CA_42C is paired with downlink CA configuration CA_3A-42C there is no interference components from 2 uplink operation which would interfere the 3rd downlink of the Band 3.

6.14.1.5	∆TIB and ∆RIB values

For inter-band CA_3-42, same requirements on values for ∆TIB and ∆RIB can apply with 2DL/1UL and 2DL/2UL cases. Therefore, for two simultaneous UL, the TIB,c and RIB,c values are shown in table 6.14.1.5-1 and in table 6.14.1.5-2 as already captured in Table 5.1.2-2 in this TR:

Table 6.14.1.5-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_3-42 | 3 | 0.6 |
|  | 42 | 0.8 |

Table 6.14.1.5-2: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_3-42 | 3 | 0.2 |
|  | 42 | 0.5 |

6.15	LTE Advanced Carrier Aggregation: Band 4 and Band 12 and Band 30 with 2 ULs

6.15.1	List of specific combination issues

6.15.1.1	Channel bandwidths per operating band for CA

Table 6.15.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_4A-12A-30A | CA_4A-12A | 4 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |

6.15.1.2	Co-existence studies for LTE-A UL CA_4A-12A and DL CA_4A-12A-30A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.15.1.2-1

Table 6.15.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 698 | 716 | 1710 | 1755 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1396 | 1432 | 3420 | 3510 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2094 | 2148 | 5130 | 5265 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 994 | 1057 | 2408 | 2471 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 359 | 278 | 2704 | 2812 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3106 | 3187 | 4118 | 4226 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 339 | 438 | 4414 | 4567 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3804 | 3903 | 5828 | 5981 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2114 | 1988 | 4816 | 4942 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6322 | 6124 | 1154 | 1037 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7538 | 7736 | 4502 | 4619 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3869 | 3698 | 1272 | 1416 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6526 | 6697 | 5514 | 5658 |

The 3rd harmonics of Band 12, 4th IMD fall into the own Rx frequency of Band 4. But these impacts already covered in 2UL/2DL CA in Rel-12.

There are no harmonics or IMD products from Band 4 and Band 12 uplinks which would desensitize Band 30 downlink.

6.15.1.3	MSD

When uplink CA configurations CA_4A-12A is paired with downlink CA configuration CA_4A-12A-30A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 30.

6.15.1.4	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.16	LTE Advanced Carrier Aggregation: Band 4 and Band 12 and Band 12 with 2 ULs

6.16.1	List of specific combination issues

6.16.1.1	Channel bandwidths per operating band for CA

Table 6.16.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_4A-12B | CA_4A-12A | 4 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  |  | CA_12B | See CA_12B BCS0 in Table 6.16.1.1-2 |  |  |  |  |  |  |  |

Table 6.16.1.1-2: E-UTRA CA_12B configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_12B | 5 | 5, 10 | 15 | 0 |

6.16.1.2	Co-existence studies for LTE-A UL CA_4A-12A and DL CA_4A-12B

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.16.1.2-1

Table 6.16.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 698 | 716 | 1710 | 1755 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1396 | 1432 | 3420 | 3510 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2094 | 2148 | 5130 | 5265 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 994 | 1057 | 2408 | 2471 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 359 | 278 | 2704 | 2812 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3106 | 3187 | 4118 | 4226 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 339 | 438 | 4414 | 4567 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3804 | 3903 | 5828 | 5981 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2114 | 1988 | 4816 | 4942 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6322 | 6124 | 1154 | 1037 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7538 | 7736 | 4502 | 4619 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3869 | 3698 | 1272 | 1416 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6526 | 6697 | 5514 | 5658 |

The 3rd harmonics of Band 12, 4th IMD fall into the own Rx frequency of Band 4. But these impacts already covered in 2UL/2DL CA in Rel-12.

There are no harmonics or IMD products from Band 4 and Band 12 uplinks which would desensitize Band 12 downlink.

6.16.1.3	MSD

When uplink CA configurations CA_4A-12A is paired with downlink CA configuration CA_4A-12B there is no interference components from 2 uplink operation which would interfere the downlink of the Band 12.

6.16.1.4	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.17	LTE Advanced Carrier Aggregation: Band 7 and Band 7 and Band 28 with 2 ULs

6.17.1	List of specific combination issues

6.17.1.1	Channel bandwidths per operating band for CA

Table 6.17.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_7C-28A | CA_7C or CA_7A-28A | 7 | See CA_7C BCS2 in Table 6.17.1.1-2 |  |  |  |  |  | 60 | 0 |
|  |  | 28 |  |  |  | Yes | Yes | Yes |  |  |

Table 6.17.1.1-2: E-UTRA CA_7C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_7C | CA_7C | 15 | 15 | 40 | 0 |
|  |  | 20 | 20 |  |  |
|  |  | 10 | 20 | 40 | 1 |
|  |  | 15 | 15, 20 |  |  |
|  |  | 20 | 10, 15, 20 |  |  |
|  |  | 15 | 10, 15 | 40 | 2 |
|  |  | 20 | 15, 20 |  |  |

6.17.1.2	Co-existence studies for LTE-A UL CA_7A-28A and DL CA_7C-28A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.17.1.2-1

Table 6.17.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1752 | 1867 | 3203 | 3318 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1164 | 1004 | 4252 | 4437 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3906 | 4066 | 5703 | 5888 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 461 | 256 | 6752 | 7007 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4609 | 4814 | 8203 | 8458 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3734 | 3504 | 6406 | 6636 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9577 | 9252 | 492 | 242 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10703 | 11028 | 5312 | 5562 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6304 | 6004 | 2756 | 3031 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8906 | 9206 | 7109 | 7384 |

There are no harmonics or IMD products from Band 7 and Band 28 uplinks which would desensitize Band 7 downlink.

6.17.1.3	Co-existence studies for LTE-A UL CA_7C and DL CA_7C-28A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.17.1.3-1

Table 6.17.1.3-1: Harmonic analysis

| UE UL carriers | fy_low | fy_high |
| --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 |

There is no harmonics from Band 7uplinks which would desensitize Band 28 downlink. Frequency separation of band 28 downlink and 7 uplink is 1697 MHz therefore there is no impact from Band 7 uplink CA to band 28 reception.

6.17.1.4	MSD

When uplink CA configurations CA_7A-28A is paired with downlink CA configuration CA_7C-28A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 7.

And also when uplink CA configurations CA_7C is paired with downlink CA configuration CA_7C-28A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 28.

6.17.1.5	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.18	LTE Advanced Carrier Aggregation: Band 19 and Band 42 and Band 42 with 2 ULs

6.18.1	List of specific combination issues

6.18.1.1	Channel bandwidths per operating band for CA

Table 6.18.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_19A-42C | CA_19A-42A | 19 |  |  | Yes | Yes | Yes |  | 55 | 0 |
|  |  | CA_42C | See CA_42C BCS0 in Table 6.18.1.1-2 |  |  |  |  |  |  |  |

Table 6.18.1.1-2: E-UTRA CA_42C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_42C | 5, 10, 15, 20 | 20 | 40 | 0 |
|  | 20 | 5, 10, 15 |  |  |

6.18.1.2	Co-existence studies for LTE-A UL CA_19A-42A and DL CA_19A-42C

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.18.1.2-1

Table 6.18.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 830 | 845 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1660 | 1690 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2490 | 2535 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2555 | 2770 | 4230 | 4445 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1940 | 1710 | 5955 | 6370 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5060 | 5290 | 7630 | 8045 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1110 | 865 | 9355 | 9970 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5890 | 6135 | 11030 | 11645 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 5540 | 5110 | 8460 | 8890 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13570 | 12755 | 20 | 280 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14430 | 15245 | 6720 | 6980 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9140 | 8510 | 4265 | 4710 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11860 | 12490 | 9290 | 9735 |

The 4th IMD fall into the own Rx frequency of Band 19. But these impacts already decide not to make MSD requirements since RAN4 consider the specific operator spectrum in Band 19 in 2UL/2DL CA in Rel-14

There are no harmonics or IMD products from Band 19 and Band 42 uplinks which would desensitize Band 42 downlink.

6.18.1.3	MSD

When uplink CA configurations CA_19A-42A is paired with downlink CA configuration CA_19A-42C there is no interference components from 2 uplink operation which would interfere the downlink of the Band 42.

6.18.1.4	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.19	LTE Advanced Carrier Aggregation: Band 21 and Band 42 and Band 42 with 2 ULs

6.19.1	List of specific combination issues

6.19.1.1	Channel bandwidths per operating band for CA

Table 6.19.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_21A-42C | CA_21A-42A | 21 |  |  | Yes | Yes | Yes |  | 55 | 0 |
|  |  | CA_42C | See CA_42C BCS0 in Table 6.19.1.1-2 |  |  |  |  |  |  |  |

Table 6.19.1.1-2: E-UTRA CA_42C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_42C | 5, 10, 15, 20 | 20 | 40 | 0 |
|  | 20 | 5, 10, 15 |  |  |

6.19.1.2	Co-existence studies for LTE-A UL CA_21A-42A and DL CA_21A-42C

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.19.1.2-1

Table 6.19.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1937.1 | 2152.1 | 4847.9 | 5062.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 704.2 | 474.2 | 5337.1 | 5752.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6295.8 | 6525.8 | 8247.9 | 8662.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 743.7 | 988.7 | 8737.1 | 9352.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7743.7 | 7988.7 | 11647.9 | 12262.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 4304.2 | 3874.2 | 9695.8 | 10125.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12952.1 | 12137.1 | 2451.6 | 2191.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15047.9 | 15862.9 | 9191.6 | 9451.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7904.2 | 7274.2 | 2411.3 | 2856.3 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13095.8 | 13725.8 | 11143.7 | 11588.7 |

There are no harmonics or IMD products from Band 21 and Band 42 uplinks which would desensitize Band 42 downlink.

6.19.1.3	MSD

When uplink CA configurations CA_21A-42A is paired with downlink CA configuration CA_21A-42C there is no interference components from 2 uplink operation which would interfere the downlink of the Band 42.

6.19.1.4	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.20	LTE Advanced Carrier Aggregation: Band 3 and Band 3 and Band 8 with 2 ULs

6.20.1	List of specific combination issues

6.20.1.1	Channel bandwidths per operating band for CA

Table 6.20.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A-3A-8A | CA_3A-8A | CA_3A-3A | See CA_3A-3A BCS0 in Table 6.20.1.1-2 |  |  |  |  |  | 50 | 0 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | CA_3A-3A | See CA_3A-3A BCS1 in Table 6.20.1.1-2 |  |  |  |  |  | 40 | 1 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |

Table 6.20.1.1-2: E-UTRA CA_3A-3A configurations and bandwidth combination sets for intra-band CA

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRACA configuration | Uplink CA configurations (NOTE 1) | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_3A-3A | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 40 | 0 |
|  |  | 5, 10 | 5, 10, 15, 20 |  |  | 30 | 1 |

6.20.1.2	Co-existence studies for LTE-A UL CA_3A-8A and DL CA_3A-3A-8A

[Editor Note] It will be updated in future

6.20.1.3	MSD

[Editor Note] It will be updated in future

6.20.1.4	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.21	LTE Advanced Carrier Aggregation: Band 1 and Band 5 and Band 46 with 2 ULs

6.21.1	List of specific combination issues

6.21.1.1	Channel bandwidths per operating band for CA

Table 6.21.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-5A-46A | CA_1A-5A | 1 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 46 |  |  |  |  |  | Yes |  |  |

6.21.1.2	Co-existence studies for LTE-A UL CA_1A-5A and DL CA_1A-5A-46A

For 2UL / 3DL own receiver desensitization study up to 8th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.21.1.2-1 and 6.21.1.2-2.

Table 6.21.1.2-1: Harmonic analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 5760 | 5940 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 7680 | 7920 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 9600 | 9900 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 4944 | 5094 | 11520 | 11880 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 5768 | 5943 | 13440 | 13860 |
| 8th harmonics frequency limits | 8*fx_low | 8*fx_high | 8* fy_low | 8* fy_high |
| 8th harmonics frequency limits (MHz) | 6592 | 6792 | 15360 | 15840 |

Table 6.21.1.2-2: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1071 | 1156 | 2744 | 2829 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 332 | 222 | 2991 | 3136 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3568 | 3678 | 4664 | 4809 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 492 | 627 | 4911 | 5116 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4392 | 4527 | 6584 | 6789 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2312 | 2142 | 5488 | 5658 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7096 | 6831 | 1476 | 1316 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8504 | 8769 | 5216 | 5376 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4292 | 4062 | 1293 | 1488 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7408 | 7638 | 6312 | 6507 |

From the harmonics analysis table, the 3rd harmonics by Band 1 fall into the Band 46 own RX ranges.And also the 7th harmonics by Band 5 fall into the Band 46 own RX frequency ranges. But the harmonics problems of each interference band has covered in 2DL/1UL CA_1A-46A in rel-13 and 5A-46A in rel-14 with additional guard band to ganruntee 0dBi MSD.

The 4th IMD fall into the own Rx frequency of Band 1. But this impact has covered in 2DL/2UL CA_1A-5A.

The 4th & 5th IMDs by Band 1 and Band 5 fall into the own Rx frequency of Band 46. It will be analysed to slove the self- interference problem.

6.21.1.3	MSD

When uplink CA configurations CA_1A-5A is paired with downlink CA configuration CA_1A-5A-46A there is interference components from 2 uplink operation which would interfere the downlink of the Band 46.

Based on the WF [6], the merged MSD level was provided from interested companies as below Table 6.21.1.3-1.

Table 6.21.1.3-1: MSD values provided by companies

| DL CA | UL CA | IMD |  | LGE | Qualcomm | Huawei | Avg. MSD [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B1+B5+B46 | B1 | IMD4 | 2*fB1 +2*fB5 | 0.88 | 3.27 | 0 | 1.4 |
|  | B5 |  |  |  |  |  |  |
|  | B1 | IMD5 | fB1 +4*fB5 |  | 0 | 0 | 0 |
|  | B5 |  |  |  |  |  |  |

6.21.1.4	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.22	LTE Advanced Carrier Aggregation: Band 1 and Band 7 and Band 46 with 2 ULs

6.22.1	List of specific combination issues

6.22.1.1	Channel bandwidths per operating band for CA

Table 6.22.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-7A-46A | CA_1A-7A | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 46 |  |  |  |  |  | Yes |  |  |

6.22.1.2	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-7A-46A

For 2UL / 3DL own receiver desensitization study up to 8th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.22.1.2-1 and 6.22.1.2-2.

Table 6.22.1.2-1: Harmonic analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 7500 | 7710 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 10000 | 10280 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 12500 | 12850 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 11520 | 11880 | 15000 | 15420 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 13440 | 13860 | 17500 | 17990 |
| 8th harmonics frequency limits | 8*fx_low | 8*fx_high | 8* fy_low | 8* fy_high |
| 8th harmonics frequency limits (MHz) | 15360 | 15840 | 20000 | 20560 |

Table 6.22.1.2-2: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2500 | 2570 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 520 | 650 | 4420 | 4550 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1270 | 1460 | 3020 | 3220 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6340 | 6530 | 6920 | 7120 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3190 | 3440 | 5520 | 5790 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8260 | 8510 | 9420 | 9690 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1300 | 1040 | 8840 | 9100 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8360 | 8020 | 5420 | 5110 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11920 | 12260 | 10180 | 10490 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3870 | 3540 | 940 | 620 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11340 | 11670 | 10760 | 11080 |

From the harmonics analysis table, the 3rd harmonics by Band 1 fall into the Band 46 own RX ranges. But the harmonics problem by Band 1 transmission has covered in 2DL/1UL CA_1A-46A in rel-13 with additional guard band to ganruntee 0dBi MSD.

The 4th & 5th IMDs by Band 1 and Band 7 fall into the own Rx frequency of Band 46. It will be analysed to slove the self- interference problem.

6.22.1.3	MSD

When uplink CA configurations CA_1A-7A is paired with downlink CA configuration CA_1A-7A-46A there is interference components from 2 uplink operation which would interfere the downlink of the Band 46.

Based on the WF [6], the merged MSD level was provided from interested companies as below Table 6.22.1.3-1.

Table 6.22.1.3-1: MSD values provided by companies

| DL CA | UL CA | IMD |  | LGE | Qualcomm | Huawei | Avg. MSD [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B1+B7+B46 | B1 | IMD4 | 3*fB7 -fB1 | 4.20 | 3.6 | 5.3 | 4.4 |
|  | B7 |  |  |  |  |  |  |
|  | B1 | IMD5 | 4*fB1 -1*fB7 |  | 4.7 | 0 | 2.4 |
|  | B7 |  |  |  |  |  |  |

6.22.1.4	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.23	LTE Advanced Carrier Aggregation: Band 5 and Band 7 and Band 46 with 2 ULs

6.23.1	List of specific combination issues

6.23.1.1	Channel bandwidths per operating band for CA

Table 6.23.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_5A-7A-46A | CA_5A-7A | 5 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 46 |  |  |  |  |  | Yes |  |  |

6.23.1.2	Co-existence studies for LTE-A UL CA_5A-7A and DL CA_5A-7A-46A

For 2UL / 3DL own receiver desensitization study up to 8th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.23.1.2-1 and 6.23.1.2-2.

Table 6.23.1.2-1: Harmonic analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 7500 | 7710 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 10000 | 10280 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 12500 | 12850 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 4944 | 5094 | 15000 | 15420 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 5768 | 5943 | 17500 | 17990 |
| 8th harmonics frequency limits | 8*fx_low | 8*fx_high | 8* fy_low | 8* fy_high |
| 8th harmonics frequency limits (MHz) | 6592 | 6792 | 20000 | 20560 |

Table 6.23.1.2-2: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1651 | 1746 | 3324 | 3419 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 922 | 802 | 4151 | 4316 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4148 | 4268 | 5824 | 5989 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 98 | 47 | 6651 | 6886 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4972 | 5117 | 8324 | 8559 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3492 | 3302 | 6648 | 6838 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9456 | 9151 | 896 | 726 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10824 | 11129 | 5796 | 5966 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6062 | 5802 | 2453 | 2668 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9148 | 9408 | 7472 | 7687 |

From the harmonics analysis table, the 7th harmonics by Band 5 fall into the Band 46 own RX ranges. But the harmonics problem by Band 5 transmission has covered in 2DL/1UL CA_5A-46A in rel-14 with additional guard band to ganruntee 0dBi MSD.

The 3rd & 5th IMDs fall into the own own Rx frequency of Band 5. Also the 5th IMD fall into the own Rx frequency of Band 7. But these impacts were already covered in 2DL/2UL CA_5A-7A.

The 3rd & 5th IMDs by Band 5 and Band 7 fall into the own Rx frequency of Band 46. It will be analysed to slove the self- interference problem.

6.23.1.3	MSD

When uplink CA configurations CA_5A-7A is paired with downlink CA configuration CA_5A-7A-46A there is interference components from 2 uplink operation which would interfere the downlink of the Band 46.

Based on the WF [6], the merged MSD level was provided from interested companies as below Table 6.23.1.3-1.

Table 6.23.1.3-1: MSD values provided by companies

| DL CA | UL CA | IMD |  | LGE | Qualcomm | Huawei | Avg. MSD [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B5+B7+B46 | B5 | IMD3 | 2*fB7 +*fB5 | 6.84 | 15.1 | 15.4 | 12.5 |
|  | B7 |  |  |  |  |  |  |
|  | B5 | IMD5 | 3*fB7 -2*fB5 |  | 0 | 0 | 0 |
|  | B7 |  |  |  |  |  |  |

6.23.1.4	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.24	LTE Advanced Carrier Aggregation: Band 3 and Band 5 and Band 7 with 2 ULs

6.24.1	List of specific combination issues

6.24.1.1	Channel bandwidths per operating band for CA

Table 6.24.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A-5A-7A | CA_3A-5A or CA_3A-7A or CA_5A-7A | 3 |  |  |  | Yes | Yes | Yes | 50 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |

6.24.1.2	Co-existence studies for LTE-A UL CA_3A-5A and DL CA_3A-5A-7A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.24.1.2-1

Table 6.24.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 861 | 961 | 2534 | 2634 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 137 | 12 | 2571 | 2746 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3358 | 3483 | 4244 | 4419 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 687 | 837 | 4281 | 4531 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4182 | 4332 | 5954 | 6204 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1922 | 1722 | 5068 | 5268 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6316 | 5991 | 1686 | 1511 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7664 | 7989 | 5006 | 5181 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3707 | 3432 | 873 | 1098 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6778 | 7053 | 5892 | 6117 |

The 2nd & 5th IMDs fall into the own Rx Band 5. Also the 4th IMD fll into the own Rx Band 3. But these impacts already has covered in 2DL/2UL CA_3A-5A.

The 2nd & 3rd IMDs fall into the own Rx frequency of Band 7. It will be analysed to slove the self- interference problem.

6.24.1.3	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_3A-5A-7A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.24.1.3-1

Table 6.24.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 715 | 860 | 4210 | 4355 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 850 | 1070 | 3215 | 3430 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5920 | 6140 | 6710 | 6925 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2560 | 2855 | 5715 | 6000 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7630 | 7925 | 9210 | 9495 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1720 | 1430 | 8420 | 8710 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8570 | 8215 | 4640 | 4270 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11710 | 12065 | 9340 | 9710 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4290 | 3930 | 355 | 10 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10920 | 11280 | 10130 | 10495 |

The 4th IMD fall into the own Rx Band 7. But this impact already has covered in 2DL/2UL CA_3A-7A.

The 3rd IMD fall into the own Rx frequency of Band 5. It will be analysed to slove the self- interference problem.

6.24.1.4	Co-existence studies for LTE-A UL CA_5A-7A and DL CA_3A-5A-7A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.24.1.4-1

Table 6.24.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1651 | 1746 | 3324 | 3419 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 922 | 802 | 4151 | 4316 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4148 | 4268 | 5824 | 5989 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 98 | 47 | 6651 | 6886 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4972 | 5117 | 8324 | 8559 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3492 | 3302 | 6648 | 6838 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9456 | 9151 | 896 | 726 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10824 | 11129 | 5796 | 5966 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6062 | 5802 | 2453 | 2668 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9148 | 9408 | 7472 | 7687 |

The 3rd & 5th IMDs fall into own Rx frequency of Band 5. Also the 5th IMD fall into the own Rx frequency of Band 7. But these impacts already has covered in 2DL/2UL CA_5A-7A.

The harmonics/ IMD do not impact to the 3rd own Rx frequency of Band 3.

6.24.1.5	MSD

When uplink CA configurations CA_3A-5A is paired with downlink CA configuration CA_3A-5A-7A there is interference components from 2 uplink operation which would interfere the downlink of the Band 7.

When uplink CA configurations CA_3A-7A is paired with downlink CA configuration CA_3A-5A-7A there is interference components from 2 uplink operation which would interfere the downlink of the Band 5.

When uplink CA configurations CA_5A-7A is paired with downlink CA configuration CA_3A-5A-7A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 3.

The merged MSD level was provided from interested companies as below Table 6.24.1.5-1.

Table 6.24.1.5-1: MSD values provided by companies

| DL CA | UL CA | IMD |  | LGE | Huawei | Qualcomm | Avg. MSD [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B3+B5+B7 | B3 | IMD2 | fB3 +fB5 | 26.35 | [33.6] | 29.9 | 30.0 |
|  | B5 |  |  |  |  |  |  |
|  | B3 | IMD3 | 2*fB3 -fB5 | N/A | [17.8] | 21.3 | 19.5 |
|  | B5 |  |  |  |  |  |  |
|  | B3 | IMD3 | 2*fB3 -fB7 | 15.63 | [21.7] | 20.4 | 19.2 |
|  | B7 |  |  |  |  |  |  |

6.24.1.6	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.25	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 21 with 2 ULs

6.25.1	List of specific combination issues

6.25.1.1	Channel bandwidths per operating band for CA

Table 6.25.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-3A-21A | CA_1A-3A or CA_1A-21A or CA_3A-21A | 1 |  |  | Yes | Yes | Yes | Yes | 55 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |

6.25.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-21A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.25.1.2-1 based on a certain operater’s actual spectrum holdings.

Table 6.25.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1764.9 | 1784.9 | 1940 | 1960 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3529.8 | 3569.8 | 3880 | 3920 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5294.7 | 5354.7 | 5820 | 5880 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 155.1 | 195.1 | 3704.9 | 3744.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1569.8 | 1629.8 | 2095.1 | 2155.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5469.8 | 5529.8 | 5644.9 | 5704.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3334.7 | 3414.7 | 4035.1 | 4115.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7234.7 | 7314.7 | 7584.9 | 7664.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 390.2 | 310.2 | 7409.8 | 7489.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6075.1 | 5975.1 | 5199.6 | 5099.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9524.9 | 9624.9 | 8999.6 | 9099.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2350.2 | 2250.2 | 1474.7 | 1374.7 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9349.8 | 9449.8 | 9174.7 | 9274.7 |

The 3rd IMD fall into the own Rx Band 1. But this impact already has covered in 2DL/2UL CA_1A-3A.

The 3rd & 5th IMDs fall into the own Rx frequency of Band 21. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21. Then, there is no IMD problems to the  own Rx frequency of Band 21.

6.25.1.3	Co-existence studies for LTE-A UL CA_1A-21A and DL CA_1A-3A-21A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.25.1.3-1 based on a certain operater’s actual spectrum holdings.

Table 6.25.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 1940 | 1960 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 3880 | 3920 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 5820 | 5880 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 477.1 | 512.1 | 3387.9 | 3422.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 935.8 | 985.8 | 2417.1 | 2472.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4835.8 | 4885.8 | 5327.9 | 5382.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2383.7 | 2448.7 | 4357.1 | 4432.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6283.7 | 6348.7 | 7267.9 | 7342.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1024.2 | 954.2 | 6775.8 | 6845.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6392.1 | 6297.1 | 3911.6 | 3831.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9207.9 | 9302.9 | 7731.6 | 7811.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2984.2 | 2894.2 | 508.7 | 423.7 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8715.8 | 8805.8 | 8223.7 | 8308.7 |

There was no harmonics/IMDs impact to the 3rd own Rx frequency Band 3.

6.25.1.4	Co-existence studies for LTE-A UL CA_3A-21A and DL CA_1A-3A-21A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.25.1.4-1 based on a certain operater’s actual spectrum holdings.

Table 6.25.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 1764.9 | 1784.9 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 3529.8 | 3569.8 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 5294.7 | 5354.7 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 302 | 337 | 3212.8 | 3247.8 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1110.9 | 1160.9 | 2066.9 | 2121.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4660.7 | 4710.7 | 4977.7 | 5032.7 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2558.8 | 2623.8 | 3831.8 | 3906.8 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6108.6 | 6173.6 | 6742.6 | 6817.6 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 674 | 604 | 6425.6 | 6495.6 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5691.7 | 5596.7 | 4086.7 | 4006.7 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8507.5 | 8602.5 | 7556.5 | 7636.5 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2458.9 | 2368.9 | 858.9 | 773.9 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8190.5 | 8280.5 | 7873.5 | 7958.5 |

The 3rd IMD fall into the 3rd own Rx frequency of Band 1. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21. Also RAN4 can consider fixed Tx-Rx separation in the operator specific frequency in Band 1. Then, there was no harmonics/IMDs impact to the 3rd own Rx frequency of Band 1.

6.25.1.5	MSD

When uplink CA configurations CA_1A-3A is paired with downlink CA configuration CA_1A-3A-21A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 21.

When uplink CA configurations CA_1A-21A is paired with downlink CA configuration CA_1A-3A-21A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 3.

When uplink CA configurations CA_3A-21A is paired with downlink CA configuration CA_1A-3A-21A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 1.

6.25.1.6	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.26	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 42 with 2 ULs

6.26.1	List of specific combination issues

6.26.1.1	Channel bandwidths per operating band for CA

Table 6.26.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-3A-42A | CA_1A-3A or CA_1A-42A or CA_3A-42A | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |

6.26.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-42A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.26.1.2-1

Table 6.26.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 135 | 270 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1440 | 1650 | 2055 | 2250 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5340 | 5550 | 5550 | 5745 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3150 | 3435 | 3975 | 4230 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7050 | 7335 | 7470 | 7725 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 540 | 270 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6210 | 5895 | 5220 | 4860 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9390 | 9705 | 8760 | 9120 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2520 | 2190 | 1515 | 1170 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9180 | 9510 | 8970 | 9315 |

The 2nd harmonic by Band 3 and 3rd IMD fall into the own Rx Band 1. But this impact already has covered in 2DL/2UL CA_1A-3A.

The 4th IMD fall into the 3rd own Rx frequency of Band 42. It will be analysed to slove the self- interference problem.

6.26.1.3	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-3A-42A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.26.1.3-1

Table 6.26.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1420 | 1680 | 5320 | 5580 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 240 | 560 | 4820 | 5280 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7240 | 7560 | 8720 | 9180 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2160 | 2540 | 8220 | 8880 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9160 | 9540 | 12120 | 12780 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3360 | 2840 | 10640 | 11160 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12480 | 11620 | 4520 | 4080 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15520 | 16380 | 11080 | 11520 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6960 | 6240 | 860 | 1440 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14040 | 14760 | 12560 | 13140 |

The 3rd IMD fall into the own Rx Band 1. But this impact already has covered in 2DL/2UL CA_1A-42A.

Hence, there was no harmonics/IMDs impact to the 3rd own Rx frequency Band 3.

6.26.1.4	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_1A-3A-42A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.26.1.4-1

Table 6.26.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1615 | 1890 | 5110 | 5385 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 180 | 170 | 5015 | 5490 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6820 | 7170 | 8510 | 8985 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1530 | 1955 | 8415 | 9090 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8530 | 8955 | 11910 | 12585 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3780 | 3230 | 10220 | 10770 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12690 | 11815 | 3740 | 3240 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15310 | 16185 | 10240 | 10740 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7380 | 6630 | 1445 | 2070 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13620 | 14370 | 11930 | 12555 |

The 2nd , 4th & 5th IMDs fall into the own Rx frequency of Band 3. Also the 2nd harmonics from Band 3, 4th & 5th IMDs fall into the own Rx frequency of Band 42. But these impact already has covered in 2DL/2UL CA_3A-42A.

Hence, there was no harmonics/IMDs impact to the 3rd own Rx frequency of Band 1.

6.26.1.5	MSD

When uplink CA configurations CA_1A-3A is paired with downlink CA configuration CA_1A-3A-42A there is interference components from 2 uplink operation which would interfere the downlink of the Band 42.

When uplink CA configurations CA_1A-42A is paired with downlink CA configuration CA_1A-3A-42A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 3.

When uplink CA configurations CA_3A-42A is paired with downlink CA configuration CA_1A-3A-42A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 1.

The merged MSD level was provided from interested companies as below Table 6.26.1.5-1.

Table 6.26.1.5-1: MSD values provided by companies

| DL CA | UL CA | IMD |  | LGE | Huawei | Qualcomm | Avg. MSD [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B1+B3+B42 | B1 | IMD4 | 3*fB3 -fB1 | 8.47 | 14.1 | 17.3 | 13.3 |
|  | B3 |  |  |  |  |  |  |

6.26.1.6	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.27	LTE Advanced Carrier Aggregation: Band 1 and Band 19 and Band 42 with 2 ULs

6.27.1	List of specific combination issues

6.27.1.1	Channel bandwidths per operating band for CA

Table 6.27.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-19A-42A | CA_1A-19A or CA_1A-42A or CA_19A-42A | 1 |  |  | Yes | Yes | Yes | Yes | 55 | 0 |
|  |  | 19 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |

6.27.1.2	Co-existence studies for LTE-A UL CA_1A-19A and DL CA_1A-19A-42A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.27.1.2-1 based on a certain operater’s actual spectrum holdings.

Table 6.27.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 830 | 845 | 1940 | 1960 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1660 | 1690 | 3880 | 3920 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2490 | 2535 | 5820 | 5880 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1095 | 1130 | 2770 | 2805 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 300 | 250 | 3035 | 3090 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3600 | 3650 | 4710 | 4765 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 530 | 595 | 4975 | 5050 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4430 | 4495 | 6650 | 6725 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2260 | 2190 | 5540 | 5610 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7010 | 6915 | 1440 | 1360 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8590 | 8685 | 5260 | 5340 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4220 | 4130 | 1345 | 1430 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7480 | 7570 | 6370 | 6455 |

The 4th IMD fall into the own Rx Band 1. But this impact already has covered in 2DL/2UL CA_1A-19A.

The 3rd IMD fall into the 3rd own Rx frequency of Band 42. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 19. Then, there is no IMD problems to the 3rd own Rx frequency of Band 42.

6.27.1.3	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-19A-42A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.27.1.3-1 based on a certain operater’s actual spectrum holdings.

Table 6.27.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1940 | 1960 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3880 | 3920 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5820 | 5880 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1520 | 1580 | 5420 | 5480 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 360 | 440 | 5000 | 5100 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7360 | 7440 | 8900 | 9000 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2300 | 2400 | 8480 | 8620 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9300 | 9400 | 12380 | 12520 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3160 | 3040 | 10840 | 10960 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12140 | 11960 | 4360 | 4240 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15860 | 16040 | 11240 | 11360 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6680 | 6520 | 1080 | 1220 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14320 | 14480 | 12780 | 12920 |

The 4th IMD fall into the own Rx Band 1. But this impact already has covered in 2DL/2UL CA_1A-42A.

The 5th IMD fall into the 3rd own Rx frequency of Band 19. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 19. Then, there is no IMD problems to the 3rd own Rx frequency of Band 19.

6.27.1.4	Co-existence studies for LTE-A UL CA_19A-42A and DL CA_1A-19A-42A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.27.1.4-1 based on a certain operater’s actual spectrum holdings.

Table 6.27.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 830 | 845 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1660 | 1690 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2490 | 2535 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2635 | 2690 | 4310 | 4365 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1860 | 1790 | 6115 | 6210 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5140 | 5210 | 7790 | 7885 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1030 | 945 | 9595 | 9730 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5970 | 6055 | 11270 | 11405 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 5380 | 5270 | 8620 | 8730 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13250 | 13075 | 100 | 200 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14750 | 14925 | 6800 | 6900 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 8900 | 8750 | 4425 | 4550 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 12100 | 12250 | 9450 | 9575 |

The 4th IMD fall into the own Rx frequency of Band 19. But this impact already has covered in 2DL/2UL CA_19A-42A.

Hence, there was no harmonics/IMDs impact to the 3rd own Rx frequency of Band 1.

6.27.1.5	MSD

When uplink CA configurations CA_1A-19A is paired with downlink CA configuration CA_1A-19A-42A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 42.

When uplink CA configurations CA_1A-42A is paired with downlink CA configuration CA_1A-19A-42A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 19.

When uplink CA configurations CA_19A-42A is paired with downlink CA configuration CA_1A-19A-42A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 1.

6.27.1.6	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.28	LTE Advanced Carrier Aggregation: Band 1 and Band 21 and Band 42 with 2 ULs

6.28.1	List of specific combination issues

6.28.1.1	Channel bandwidths per operating band for CA

Table 6.28.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-21A-42A | CA_1A-21A or CA_1A-42A or CA_21A-42A | 1 |  |  | Yes | Yes | Yes | Yes | 55 | 0 |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |

6.28.1.2	Co-existence studies for LTE-A UL CA_1A-21A and DL CA_1A-21A-42A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.28.1.2-1 based on a certain operater’s actual spectrum holdings.

Table 6.28.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 1940 | 1960 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 3880 | 3920 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 5820 | 5880 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 477.1 | 512.1 | 3387.9 | 3422.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 935.8 | 985.8 | 2417.1 | 2472.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4835.8 | 4885.8 | 5327.9 | 5382.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2383.7 | 2448.7 | 4357.1 | 4432.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6283.7 | 6348.7 | 7267.9 | 7342.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1024.2 | 954.2 | 6775.8 | 6845.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6392.1 | 6297.1 | 3911.6 | 3831.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9207.9 | 9302.9 | 7731.6 | 7811.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2984.2 | 2894.2 | 508.7 | 423.7 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8715.8 | 8805.8 | 8223.7 | 8308.7 |

The 2nd IMD fall into the 3rd own Rx frequency of Band 42. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21. Then, there is no IMD problems to the 3rd own Rx frequency of Band 42.

6.28.1.3	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-21A-42A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.28.1.3-1 based on a certain operater’s actual spectrum holdings.

Table 6.28.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1940 | 1960 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3880 | 3920 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5820 | 5880 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1520 | 1580 | 5420 | 5480 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 360 | 440 | 5000 | 5100 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7360 | 7440 | 8900 | 9000 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2300 | 2400 | 8480 | 8620 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9300 | 9400 | 12380 | 12520 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3160 | 3040 | 10840 | 10960 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12140 | 11960 | 4360 | 4240 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15860 | 16040 | 11240 | 11360 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6680 | 6520 | 1080 | 1220 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14320 | 14480 | 12780 | 12920 |

The 4th IMD fall into the own Rx Band 1. But this impact already has covered in 2DL/2UL CA_1A-42A.

The 2nd IMD fall into the 3rd own Rx frequency of Band 21. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21. Then, there is no IMD problems to the 3rd own Rx frequency of Band 21.

6.28.1.4	Co-existence studies for LTE-A UL CA_21A-42A and DL CA_1A-21A-42A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.28.1.4-1 based on a certain operater’s actual spectrum holdings.

Table 6.28.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2017.1 | 2072.1 | 4927.9 | 4982.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 624.2 | 554.2 | 5497.1 | 5592.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6375.8 | 6445.8 | 8407.9 | 8502.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 823.7 | 908.7 | 8977.1 | 9112.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7823.7 | 7908.7 | 11887.9 | 12022.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 4144.2 | 4034.2 | 9855.8 | 9965.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12632.1 | 12457.1 | 2371.6 | 2271.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15367.9 | 15542.9 | 9271.6 | 9371.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7664.2 | 7514.2 | 2571.3 | 2696.3 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13335.8 | 13485.8 | 11303.7 | 11428.7 |

The 2nd IMD fall into the 3rd own Rx frequency of Band 1. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21. Then, there is no IMD problems to the 3rd own Rx frequency of Band 1.

6.28.1.5	MSD

When uplink CA configurations CA_1A-21A is paired with downlink CA configuration CA_1A-21A-42A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 42.

When uplink CA configurations CA_1A-42A is paired with downlink CA configuration CA_1A-21A-42A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 21.

When uplink CA configurations CA_21A-42A is paired with downlink CA configuration CA_1A-21A-42A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 1.

6.28.1.6	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.29	LTE Advanced Carrier Aggregation: Band 3 and Band 19 and Band 21 with 2 ULs

6.29.1	List of specific combination issues

6.29.1.1	Channel bandwidths per operating band for CA

Table 6.29.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A-19A-21A | CA_3A-19A or CA_3A-21A or CA_19A-21A | 3 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 19 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |

6.29.1.2	Co-existence studies for LTE-A UL CA_3A-19A and DL CA_3A-19A-21A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.29.1.2-1 based on a certain operater’s actual spectrum holdings.

Table 6.29.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 830 | 845 | 1764.9 | 1784.9 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1660 | 1690 | 3529.8 | 3569.8 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2490 | 2535 | 5294.7 | 5354.7 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 919.9 | 954.9 | 2594.9 | 2629.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 124.9 | 74.9 | 2684.8 | 2739.8 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3424.9 | 3474.9 | 4359.8 | 4414.8 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 705.1 | 770.1 | 4449.7 | 4524.7 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4254.9 | 4319.9 | 6124.7 | 6199.7 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1909.8 | 1839.8 | 5189.8 | 5259.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6309.6 | 6214.6 | 1615.1 | 1535.1 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7889.6 | 7984.6 | 5084.9 | 5164.9 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3694.7 | 3604.7 | 994.8 | 1079.8 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6954.7 | 7044.7 | 6019.8 | 6104.8 |

The 2nd & 5th IMDs fall into the own Rx frequency of Band 19. Also the 4th IMD fall into the own Rx frequency of Band 3. But theses impacts already has covered in 2DL/2UL CA_3A-19A.

Hence, there was no harmonics/IMDs impact to the 3rd own Rx frequency of Band 21.

6.29.1.3	Co-existence studies for LTE-A UL CA_3A-21A and DL CA_3A-19A-21A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.29.1.3-1 based on a certain operater’s actual spectrum holdings.

Table 6.29.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 1764.9 | 1784.9 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 3529.8 | 3569.8 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 5294.7 | 5354.7 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 302 | 337 | 3212.8 | 3247.8 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1110.9 | 1160.9 | 2066.9 | 2121.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4660.7 | 4710.7 | 4977.7 | 5032.7 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2558.8 | 2623.8 | 3831.8 | 3906.8 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6108.6 | 6173.6 | 6742.6 | 6817.6 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 674 | 604 | 6425.6 | 6495.6 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5691.7 | 5596.7 | 4086.7 | 4006.7 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8507.5 | 8602.5 | 7556.5 | 7636.5 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2458.9 | 2368.9 | 858.9 | 773.9 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8190.5 | 8280.5 | 7873.5 | 7958.5 |

The 5th IMD fall into the 3rd own Rx frequency of Band 19. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 19 and Band 21. Then, there is no IMD problems to the 3rd own Rx frequency of Band 19.

6.29.1.4	Co-existence studies for LTE-A UL CA_19A-21A and DL CA_3A-19A-21A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.29.1.4-1

Table 6.29.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 830 | 845 | 1447.9 | 1462.9 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1660 | 1690 | 2895.8 | 2925.8 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2490 | 2535 | 4343.7 | 4388.7 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 602.9 | 632.9 | 2277.9 | 2307.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 197.1 | 242.1 | 2050.8 | 2095.8 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3107.9 | 3152.9 | 3725.8 | 3770.8 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1027.1 | 1087.1 | 3498.7 | 3558.7 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3937.9 | 3997.9 | 5173.7 | 5233.7 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1265.8 | 1205.8 | 4555.8 | 4615.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5021.6 | 4946.6 | 1932.1 | 1857.1 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 6621.6 | 6696.6 | 4767.9 | 4842.9 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2728.7 | 2653.7 | 360.8 | 435.8 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6003.7 | 6078.7 | 5385.8 | 5460.8 |

The 5th IMD fall into the 3rd own Rx frequency of Band 3. This issue have same impact when RAN4 consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 19 and Band 21. It will be analysed to slove the self- interference problem.

6.29.1.5	MSD

When uplink CA configurations CA_3A-19A is paired with downlink CA configuration CA_3A-19A-21A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 21.

When uplink CA configurations CA_3A-21A is paired with downlink CA configuration CA_3A-19A-21A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 19.

When uplink CA configurations CA_19A-21A is paired with downlink CA configuration CA_3A-19A-21A there is interference components from 2 uplink operation which would interfere the downlink of the Band 3.

The merged MSD level was provided from interested companies as below Table 6.29.1.5-1.

Table 6.29.1.5-1: MSD values provided by companies

| DL CA | UL CA | IMD |  | LGE | Huawei | Qualcomm | Avg. MSD [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B3+B19+B21 | B19 | IMD5 | 4*fB19 – fB21 | 3.75 | 4.4 | 3.96 | 4.0 |
|  | B21 |  |  |  |  |  |  |

6.29.1.6	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.30	LTE Advanced Carrier Aggregation: Band 3 and Band 19 and Band 42 with 2 ULs

6.30.1	List of specific combination issues

6.30.1.1	Channel bandwidths per operating band for CA

Table 6.30.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A-19A-42A | CA_3A-19A or CA_3A-42A or CA_19A-42A | 3 |  |  | Yes | Yes | Yes | Yes | 55 | 0 |
|  |  | 19 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |

6.30.1.2	Co-existence studies for LTE-A UL CA_3A-19A and DL CA_3A-19A-42A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.30.1.2-1 based on a certain operater’s actual spectrum holdings.

Table 6.30.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 830 | 845 | 1764.9 | 1784.9 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1660 | 1690 | 3529.8 | 3569.8 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2490 | 2535 | 5294.7 | 5354.7 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 919.9 | 954.9 | 2594.9 | 2629.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 124.9 | 74.9 | 2684.8 | 2739.8 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3424.9 | 3474.9 | 4359.8 | 4414.8 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 705.1 | 770.1 | 4449.7 | 4524.7 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4254.9 | 4319.9 | 6124.7 | 6199.7 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1909.8 | 1839.8 | 5189.8 | 5259.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6309.6 | 6214.6 | 1615.1 | 1535.1 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7889.6 | 7984.6 | 5084.9 | 5164.9 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3694.7 | 3604.7 | 994.8 | 1079.8 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6954.7 | 7044.7 | 6019.8 | 6104.8 |

The 2nd & 5th IMDs fall into the own Rx frequency of Band 19. Also the 4th IMD fall into the own Rx frequency of Band 3. But theses impacts already has covered in 2DL/2UL CA_3A-19A.

The 2nd harmonics from Band 3 transmission will be interference to the Band 42. But this harmonics issue already has covered in 2DL/1UL CA_3A-42A.

The 3rd & 5th IMDs fall into the 3rd own Rx frequency of Band 42. However, RAN4 consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 19 and Band 21.

Then, there was no harmonics/IMDs problem to the 3rd own Rx frequency of Band 42.

6.30.1.3	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_3A-19A-42A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.30.1.3-1 based on a certain operater’s actual spectrum holdings.

Table 6.30.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1764.9 | 1784.9 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3529.8 | 3569.8 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5294.7 | 5354.7 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1695.1 | 1755.1 | 5244.9 | 5304.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 9.8 | 89.8 | 5175.1 | 5275.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7009.8 | 7089.8 | 8724.9 | 8824.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1774.7 | 1874.7 | 8655.1 | 8795.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8774.7 | 8874.7 | 12204.9 | 12344.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3510.2 | 3390.2 | 10489.8 | 10609.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12315.1 | 12135.1 | 3659.6 | 3539.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15684.9 | 15864.9 | 10539.6 | 10659.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7030.2 | 6870.2 | 1605.3 | 1745.3 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13969.8 | 14129.8 | 12254.7 | 12394.7 |

The 2nd , 4th & 5th IMDs fall into the own Rx frequency of Band 3. Also the 2nd harmonics from Band 3, 4th & 5th IMDs fall into the own Rx frequency of Band 42. But these impact already has covered in 2DL/2UL CA_3A-42A.

Hence, there was no harmonics/IMDs impact to the 3rd own Rx frequency of Band 19.

6.30.1.4	Co-existence studies for LTE-A UL CA_19A-42A and DL CA_3A-19A-42A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.30.1.4-1 based on a certain operater’s actual spectrum holdings.

Table 6.30.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 830 | 845 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1660 | 1690 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2490 | 2535 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2635 | 2690 | 4310 | 4365 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1860 | 1790 | 6115 | 6210 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5140 | 5210 | 7790 | 7885 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1030 | 945 | 9595 | 9730 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5970 | 6055 | 11270 | 11405 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 5380 | 5270 | 8620 | 8730 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13250 | 13075 | 100 | 200 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14750 | 14925 | 6800 | 6900 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 8900 | 8750 | 4425 | 4550 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 12100 | 12250 | 9450 | 9575 |

The 4th IMD fall into the own Rx frequency of Band 19. But this impact already has covered in 2DL/2UL CA_19A-42A.

The 3rd IMD fall into the 3rd own Rx frequency of Band 3. The overlap range is 0.1MHz that is no critical impact to the sensitivity of Band 3 when RAN4 consider operator specific frequency ranges.

6.30.1.5	MSD

When uplink CA configurations CA_3A-19A is paired with downlink CA configuration CA_3A-19A-42A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 42.

When uplink CA configurations CA_3A-42A is paired with downlink CA configuration CA_3A-19A-42A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 19.

When uplink CA configurations CA_19A-42A is paired with downlink CA configuration CA_3A-19A-42A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 3.

6.30.1.6	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.31	LTE Advanced Carrier Aggregation: Band 3 and Band 21 and Band 42 with 2 ULs

6.31.1	List of specific combination issues

6.31.1.1	Channel bandwidths per operating band for CA

Table 6.31.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A-21A-42A | CA_3A-21A or CA_3A-42A or CA_21A-42A | 3 |  |  | Yes | Yes | Yes | Yes | 55 | 0 |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |

6.31.1.2	Co-existence studies for LTE-A UL CA_3A-21A and DL CA_3A-21A-42A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.31.1.2-1 based on a certain operater’s actual spectrum holdings.

Table 6.31.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 1764.9 | 1784.9 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 3529.8 | 3569.8 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 5294.7 | 5354.7 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 302 | 337 | 3212.8 | 3247.8 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1110.9 | 1160.9 | 2066.9 | 2121.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4660.7 | 4710.7 | 4977.7 | 5032.7 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2558.8 | 2623.8 | 3831.8 | 3906.8 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6108.6 | 6173.6 | 6742.6 | 6817.6 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 674 | 604 | 6425.6 | 6495.6 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5691.7 | 5596.7 | 4086.7 | 4006.7 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8507.5 | 8602.5 | 7556.5 | 7636.5 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2458.9 | 2368.9 | 858.9 | 773.9 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8190.5 | 8280.5 | 7873.5 | 7958.5 |

The 2nd harmonics from Band 3 transmission will be interference to the Band 42. But this harmonics issue already has covered in 2DL/1UL CA_3A-42A.

Hence, there was no harmonics/IMDs impact to the 3rd own Rx frequency of Band 42.

6.31.1.3	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_3A-21A-42A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.31.1.3-1 based on a certain operater’s actual spectrum holdings.

Table 6.31.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1764.9 | 1784.9 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3529.8 | 3569.8 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5294.7 | 5354.7 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1695.1 | 1755.1 | 5244.9 | 5304.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 9.8 | 89.8 | 5175.1 | 5275.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7009.8 | 7089.8 | 8724.9 | 8824.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1774.7 | 1874.7 | 8655.1 | 8795.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8774.7 | 8874.7 | 12204.9 | 12344.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3510.2 | 3390.2 | 10489.8 | 10609.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12315.1 | 12135.1 | 3659.6 | 3539.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15684.9 | 15864.9 | 10539.6 | 10659.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7030.2 | 6870.2 | 1605.3 | 1745.3 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13969.8 | 14129.8 | 12254.7 | 12394.7 |

The 2nd , 4th & 5th IMDs fall into the own Rx frequency of Band 3. Also the 2nd harmonics from Band 3, 4th & 5th IMDs fall into the own Rx frequency of Band 42. But these impact already has covered in 2DL/2UL CA_3A-42A.

The 5th IMD fall into the 3rd own Rx frequency of Band 21. However, RAN4 consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21.

Then, there was no harmonics/IMDs problem to the 3rd own Rx frequency of Band 21.

6.31.1.4	Co-existence studies for LTE-A UL CA_21A-42A and DL CA_3A-21A-42A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.31.1.4-1 based on a certain operater’s actual spectrum holdings.

Table 6.31.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2017.1 | 2072.1 | 4927.9 | 4982.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 624.2 | 554.2 | 5497.1 | 5592.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6375.8 | 6445.8 | 8407.9 | 8502.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 823.7 | 908.7 | 8977.1 | 9112.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7823.7 | 7908.7 | 11887.9 | 12022.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 4144.2 | 4034.2 | 9855.8 | 9965.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12632.1 | 12457.1 | 2371.6 | 2271.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15367.9 | 15542.9 | 9271.6 | 9371.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7664.2 | 7514.2 | 2571.3 | 2696.3 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13335.8 | 13485.8 | 11303.7 | 11428.7 |

There was no harmonics/IMDs impact to the 3rd own Rx frequency of Band 3.

6.31.1.5	MSD

When uplink CA configurations CA_3A-21A is paired with downlink CA configuration CA_3A-21A-42A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 42.

When uplink CA configurations CA_3A-42A is paired with downlink CA configuration CA_3A-21A-42A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 21.

When uplink CA configurations CA_21A-42A is paired with downlink CA configuration CA_3A-21A-42A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 3.

6.31.1.6	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.32	LTE Advanced Carrier Aggregation: Band 19 and Band 21 and Band 42 with 2 ULs

6.32.1	List of specific combination issues

6.32.1.1	Channel bandwidths per operating band for CA

Table 6.32.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_19A-21A-42A | CA_19A-21A or CA_19A-42A or CA_21A-42A | 19 |  |  | Yes | Yes | Yes |  | 50 | 0 |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |

6.32.1.2	Co-existence studies for LTE-A UL CA_19A-21A and DL CA_19A-21A-42A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.32.1.2-1

Table 6.32.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 830 | 845 | 1447.9 | 1462.9 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1660 | 1690 | 2895.8 | 2925.8 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2490 | 2535 | 4343.7 | 4388.7 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 602.9 | 632.9 | 2277.9 | 2307.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 197.1 | 242.1 | 2050.8 | 2095.8 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3107.9 | 3152.9 | 3725.8 | 3770.8 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1027.1 | 1087.1 | 3498.7 | 3558.7 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3937.9 | 3997.9 | 5173.7 | 5233.7 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1265.8 | 1205.8 | 4555.8 | 4615.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5021.6 | 4946.6 | 1932.1 | 1857.1 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 6621.6 | 6696.6 | 4767.9 | 4842.9 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2728.7 | 2653.7 | 360.8 | 435.8 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6003.7 | 6078.7 | 5385.8 | 5460.8 |

The 4th IMD fall into the 3rd own Rx frequency of Band 42. It will be analysed to slove the self- interference problem.

6.32.1.3	Co-existence studies for LTE-A UL CA_19A-42A and DL CA_19A-21A-42A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.32.1.3-1 based on a certain operater’s actual spectrum holdings.

Table 6.32.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 830 | 845 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1660 | 1690 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2490 | 2535 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2635 | 2690 | 4310 | 4365 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1860 | 1790 | 6115 | 6210 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5140 | 5210 | 7790 | 7885 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1030 | 945 | 9595 | 9730 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5970 | 6055 | 11270 | 11405 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 5380 | 5270 | 8620 | 8730 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13250 | 13075 | 100 | 200 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14750 | 14925 | 6800 | 6900 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 8900 | 8750 | 4425 | 4550 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 12100 | 12250 | 9450 | 9575 |

The 4th IMD fall into the own Rx frequency of Band 19. But this impact already has covered in 2DL/2UL CA_19A-42A.

Hence, there was no harmonics/IMDs impact to the 3rd own Rx frequency of Band 21.

6.32.1.4	Co-existence studies for LTE-A UL CA_21A-42A and DL CA_19A-21A-42A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.32.1.4-1 based on a certain operater’s actual spectrum holdings.

Table 6.32.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2017.1 | 2072.1 | 4927.9 | 4982.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 624.2 | 554.2 | 5497.1 | 5592.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6375.8 | 6445.8 | 8407.9 | 8502.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 823.7 | 908.7 | 8977.1 | 9112.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7823.7 | 7908.7 | 11887.9 | 12022.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 4144.2 | 4034.2 | 9855.8 | 9965.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12632.1 | 12457.1 | 2371.6 | 2271.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15367.9 | 15542.9 | 9271.6 | 9371.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7664.2 | 7514.2 | 2571.3 | 2696.3 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13335.8 | 13485.8 | 11303.7 | 11428.7 |

The 4th IMD fall into the 3rd own Rx frequency of Band 19 when RAN4 consider operator specific frequency range in the related bands. It will be analysed to slove the self- interference problem.

6.32.1.5	MSD

When uplink CA configurations CA_19A-21A is paired with downlink CA configuration CA_19A-21A-42A there is interference components from 2 uplink operation which would interfere the downlink of the Band 42.

When uplink CA configurations CA_19A-42A is paired with downlink CA configuration CA_19A-21A-42A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 21.

When uplink CA configurations CA_21A-42A is paired with downlink CA configuration CA_19A-21A-42A there is interference components from 2 uplink operation which would interfere the downlink of the Band 19.

The merged MSD level was provided from interested companies as below Table 6.32.1.5-1.

Table 6.32.1.5-1: MSD values provided by companies

| DL CA | UL CA | IMD |  | LGE | Huawei | Qualcomm | Avg. MSD [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B19+B21+B42 | B19 | IMD4 | 3*fB21 - fB19 | 8.71 | 14.2 | 15.88 | 12.9 |
|  | B21 |  |  |  |  |  |  |
|  | B21 | IMD4 | 3*fB21 - fB42 | 12.85 | 12.4 | 12.34 | 12.5 |
|  | B42 |  |  |  |  |  |  |

6.32.1.6	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.33	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 3 with 2 ULs

6.33.1	List of specific combination issues

6.33.1.1	Channel bandwidths per operating band for CA

Table 6.33.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-3C | CA_1A-3A or CA_3C | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 3 | See CA_3C BCS0 in Table 6.33.1.1-2 |  |  |  |  |  |  |  |

Table 6.33.1.1-2: E-UTRA CA_3C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_3C | 5, 10, 15 | 20 | 40 | 0 |
|  | 20 | 5, 10, 15, 20 |  |  |

6.33.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3C

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.33.1.2-1

Table 6.33.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 135 | 270 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1440 | 1650 | 2055 | 2250 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5340 | 5550 | 5550 | 5745 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3150 | 3435 | 3975 | 4230 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7050 | 7335 | 7470 | 7725 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 540 | 270 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6210 | 5895 | 5220 | 4860 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9390 | 9705 | 8760 | 9120 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2520 | 2190 | 1515 | 1170 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9180 | 9510 | 8970 | 9315 |

The 3rd IMD fall into the own Rx frequency of Band 1. But this impact aleady has covered in 2DL/2UL CA_1A-3A.

Hence, there was no harmonics/IMDs impact to the 3rd own Rx frequency of Band 3.

6.33.1.3	Co-existence studies for LTE-A UL CA_3C and DL CA_1A-3C

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.33.1.3-1

Table 6.33.1.3-1: Harmonic analysis

| UE UL carriers | fx_low | fx_high |
| --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 |

From the harmonic analysis table, there was no harmonics from Band 3 uplinks which would desensitize the 3rd own Rx frequency of Band 1. Frequency separation of band 1 downlink and 3 uplink is 325 MHz therefore there is no impact from Band 3 uplink CA to Band 1 reception.

6.33.1.4	MSD

When uplink CA configurations CA_1A-3A is paired with downlink CA configuration CA_1A-3C there is no interference components from 2 uplink operation which would interfere the downlink of the Band 3.

When uplink CA configurations CA_3C is paired with downlink CA configuration CA_1A-3C there is no interference components from 2 uplink operation which would interfere the downlink of the Band 1.

6.33.1.5	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.34	LTE Advanced Carrier Aggregation: Band 3 and Band 3 and Band 8 with 2 ULs

6.34.1	List of specific combination issues

6.34.1.1	Channel bandwidths per operating band for CA

Table 6.34.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3C-8A | CA_3A-8A or CA_3C | 3 | See CA_3C BCS0 in Table 6.34.1.1-2 |  |  |  |  |  | 50 | 0 |
|  |  | 8 |  | Yes | Yes | Yes |  |  |  |  |

Table 6.34.1.1-2: E-UTRA CA_3C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_3C | 5, 10, 15 | 20 | 40 | 0 |
|  | 20 | 5, 10, 15, 20 |  |  |

6.34.1.2	Co-existence studies for LTE-A UL CA_3A-8A and DL CA_3C-8A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.34.1.2-1

Table 6.34.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 795 | 905 | 2590 | 2700 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 25 | 120 | 2505 | 2690 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3470 | 3615 | 4300 | 4485 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 855 | 1035 | 4215 | 4475 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4350 | 4530 | 6010 | 6270 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1810 | 1590 | 5180 | 5400 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6260 | 5925 | 1950 | 1735 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7720 | 8055 | 5230 | 5445 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3595 | 3300 | 675 | 930 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6890 | 7185 | 6060 | 6315 |

The 2nd harmonics from Band 8 and the 4th & 5th IMDs fall into the own Rx frequency of Band 3. Also the 4th & 5th IMDs fall into the own Rx frequency of Band 8. But these impacts aleady has covered in 2DL/2UL CA_3A-8A.

Hence, there was no harmonics/IMDs impact to the 3rd own Rx frequency of Band 3.

6.34.1.3	Co-existence studies for LTE-A UL CA_3C and DL CA_3C-8A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.34.1.3-1

Table 6.34.1.3-1: Harmonic analysis

| UE UL carriers | fx_low | fx_high |
| --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 |

From the harmonic analysis table, there was no harmonics from Band 3 uplinks which would desensitize the 3rd own Rx frequency of Band 1. Frequency separation of band 1 downlink and 3 uplink is 325 MHz therefore there is no impact from Band 3 uplink CA to Band 1 reception.

6.34.1.4	MSD

When uplink CA configurations CA_3A-8A is paired with downlink CA configuration CA_3C-8A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 3.

When uplink CA configurations CA_3C is paired with downlink CA configuration CA_3C-8A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 8.

6.34.1.5	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.35	LTE Advanced Carrier Aggregation: Band 41 and Band 42 and Band 42 with 2 ULs

6.35.1	List of specific combination issues

6.35.1.1	Channel bandwidths per operating band for CA

Table 6.35.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_41A-42C | CA_41A-42A or CA_42C | 41 |  |  |  | Yes | Yes | Yes | 60 | 0 |
|  |  | 42 | See CA_42C BCS1 in Table 6.35.1.1-2 |  |  |  |  |  |  |  |

Table 6.35.1.1-2: E-UTRA CA_42C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_42C | 5, 10, 15, 20 | 20 | 40 | 0 |
|  | 20 | 5, 10, 15 |  |  |
|  | 10, 15, 20 | 20 | 40 | 1 |
|  | 20 | 10, 15 |  |  |

6.35.1.2	Co-existence studies for LTE-A UL CA_41A-42A and DL CA_41A-42C

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.35.1.2-1

Table 6.35.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 710 | 1104 | 5896 | 6290 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1392 | 1980 | 4110 | 4704 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8392 | 8980 | 9296 | 9890 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3888 | 4670 | 7510 | 8304 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10888 | 11670 | 12696 | 13490 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2208 | 1420 | 11792 | 12580 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11904 | 10910 | 7360 | 6384 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 16096 | 17090 | 13384 | 14360 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5808 | 4820 | 1270 | 288 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 15192 | 16180 | 14288 | 15270 |

From the harmonics/IMDs analysis table, there was no harmonics/IMDs impact to the 3rd own Rx frequency of Band 42.

6.35.1.3	Co-existence studies for LTE-A UL CA_42C and DL CA_41A-42C

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.35.1.3-1

Table 6.35.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high |
| --- | --- | --- |
| UL frequency (MHz) | 3400 | 3600 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 6800 | 7200 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 10200 | 10800 |

From the harmonics analysis table, there was no harmonics from Band 42 uplinks which would desensitize the 3rd own Rx frequency of Band 41. Frequency separation of band 41 and 42 is 710 MHz therefore there is no impact from Band 42 uplink CA to Band 41 reception evenif RAN4 consider asynchronization scenario.

6.35.1.4	MSD

When uplink CA configurations CA_41A-42A is paired with downlink CA configuration CA_41A-42C there is no interference components from 2 uplink operation which would interfere the downlink of the Band 42.

When uplink CA configurations CA_42C is paired with downlink CA configuration CA_41A-42C there is no interference components from 2 uplink operation which would interfere the downlink of the Band 41.

6.35.1.5	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.36	LTE Advanced Carrier Aggregation: Band 41 and Band 41 and Band 42 with 2 ULs

6.36.1	List of specific combination issues

6.36.1.1	Channel bandwidths per operating band for CA

Table 6.36.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_41C-42A | CA_41A-42A, CA_41C | 41 | See CA_41C BCS0 in Table 6.36.1.1-2 |  |  |  |  |  | 60 | 0 |
|  |  | 42 |  |  |  | Yes | Yes | Yes |  |  |

Table 6.36.1.1-2: E-UTRA CA_41C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_41C | 10 | 20 | 40 | 0 |
|  | 15 | 15, 20 |  |  |
|  | 20 | 10, 15, 20 |  |  |

6.36.1.2	Co-existence studies for LTE-A UL CA_41A-42A and DL CA_41C-42A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.36.1.2-1

Table 6.36.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 710 | 1104 | 5896 | 6290 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1392 | 1980 | 4110 | 4704 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8392 | 8980 | 9296 | 9890 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3888 | 4670 | 7510 | 8304 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10888 | 11670 | 12696 | 13490 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2208 | 1420 | 11792 | 12580 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11904 | 10910 | 7360 | 6384 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 16096 | 17090 | 13384 | 14360 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5808 | 4820 | 1270 | 288 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 15192 | 16180 | 14288 | 15270 |

From the harmonics/IMDs analysis table, there was no harmonics/IMDs impact to the 3rd own Rx frequency of Band 41.

6.36.1.3	Co-existence studies for LTE-A UL CA_41C and DL CA_41C-42A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.36.1.3-1

Table 6.36.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high |
| --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 |

From the harmonics analysis table, there was no harmonics from Band 41 uplinks which would desensitize the 3rd own Rx frequency of Band 42. Frequency separation of band 41 and 42 is 710 MHz therefore there is no impact from Band 41 uplink CA to Band 42 reception evenif RAN4 consider asynchronization scenario.

6.36.1.4	MSD

When uplink CA configurations CA_41A-42A is paired with downlink CA configuration CA_41C-42A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 41.

When uplink CA configurations CA_41C is paired with downlink CA configuration CA_41C-42A there is no interference components from 2 uplink operation which would interfere the 3rd downlink of the Band 42.

6.36.1.5	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.37	LTE Advanced Carrier Aggregation: Band 2 and Band 4 and Band 7 with 2 ULs

6.37.1	List of specific combination issues

6.37.1.1	Channel bandwidths per operating band for CA

Table 6.37.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_2A-4A-7A | CA_2A-4A | 2 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |

6.37.1.2	Co-existence studies for LTE-A UL CA_2A-4A and DL CA_2A-4A-7A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.37.1.2-1

Table 6.37.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1755 | 1850 | 1910 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3510 | 3700 | 3820 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5265 | 5550 | 5730 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 95 | 200 | 3560 | 3665 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1510 | 1660 | 1945 | 2110 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5270 | 5420 | 5410 | 5575 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3220 | 3415 | 3795 | 4020 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6980 | 7175 | 7260 | 7485 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 400 | 190 | 7120 | 7330 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5930 | 5645 | 5170 | 4930 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9110 | 9395 | 8690 | 8930 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2310 | 2040 | 1565 | 1310 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 | 9240 | 8830 | 9085 |

The 3rd & 5th IMDs fall into the own Rx frequency of Band 2 and Band 4. But these impacts already covered in 2UL/2DL CA_2A-4A in Rel-12.

Hence, there was no harmonic or IMD products from Band 2 and Band 4 uplinks which would desensitize Band 7 downlink.

6.37.1.3	MSD

When uplink CA configurations CA_2A-4A is paired with downlink CA configuration CA_2A-4A-7A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 7.

6.37.1.4	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.38	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 28 with 2 ULs

6.38.1	List of specific combination issues

6.38.1.1	Channel bandwidths per operating band for CA

Table 6.38.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-3A-28A | CA_1A-3A or CA_1A-28A or CA_3A-28A | 1 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |

6.38.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-28A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.38.1.2-1.

Table 6.38.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 135 | 270 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1440 | 1650 | 2055 | 2250 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5340 | 5550 | 5550 | 5745 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3150 | 3435 | 3975 | 4230 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7050 | 7335 | 7470 | 7725 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 540 | 270 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6210 | 5895 | 5220 | 4860 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9390 | 9705 | 8760 | 9120 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2520 | 2190 | 1515 | 1170 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9180 | 9510 | 8970 | 9315 |

The 3rd IMD fall into the own Rx Band 1. But this impact already has covered in 2DL/2UL CA_1A-3A.

There was no harmonics/IMDs impact to the 3rd own Rx frequency Band 28.

6.38.1.3	Co-existence studies for LTE-A UL CA_1A-28A and DL CA_1A-3A-28A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.38.1.3-1.

Table 6.38.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1172 | 1277 | 2623 | 2728 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 574 | 424 | 3092 | 3257 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3326 | 3476 | 4543 | 4708 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 129 | 324 | 5012 | 5237 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4029 | 4224 | 6463 | 6688 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2554 | 2344 | 5246 | 5456 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7217 | 6932 | 1072 | 832 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8383 | 8668 | 4732 | 4972 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4534 | 4264 | 1596 | 1851 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7166 | 7436 | 5949 | 6204 |

The 5th IMDs fall into the own Rx frequency of Band 3. It will be analysed to slove the self- interference problem.

6.38.1.4	Co-existence studies for LTE-A UL CA_3A-28A and DL CA_1A-3A-28A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.38.1.4-1.

Table 6.38.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 962 | 1082 | 2413 | 2533 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 379 | 214 | 2672 | 2867 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3116 | 3281 | 4123 | 4318 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 324 | 534 | 4382 | 4652 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3819 | 4029 | 5833 | 6103 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2164 | 1924 | 4826 | 5066 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6437 | 6092 | 1282 | 1027 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7543 | 7888 | 4522 | 4777 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3949 | 3634 | 1176 | 1461 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6536 | 6851 | 5529 | 5814 |

The 4th IMD fall into the 3rd own Rx frequency of Band 1. It will be analysed to slove the self- interference problem.

6.38.1.5	MSD

When uplink CA configurations CA_1A-3A is paired with downlink CA configuration CA_1A-3A-28A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 21.

When uplink CA configurations CA_1A-28A is paired with downlink CA configuration CA_1A-3A-28A there is interference components from 2 uplink operation which would interfere the downlink of the Band 3.

When uplink CA configurations CA_3A-28A is paired with downlink CA configuration CA_1A-3A-28A there is interference components from 2 uplink operation which would interfere the downlink of the Band 1.

6.38.1.6	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.39	LTE Advanced Carrier Aggregation: Band 1 and Band 21 and Band 28 with 2 ULs

6.39.1	List of specific combination issues

6.39.1.1	Channel bandwidths per operating band for CA

Table 6.39.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-21A-28A | CA_1A-21A or CA_1A-28A or CA_21A-28A | 1 |  |  | Yes | Yes | Yes | Yes | 45 | 0 |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |

6.39.1.2	Co-existence studies for LTE-A UL CA_1A-21A and DL CA_1A-21A-28A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.39.1.2-1 based on a certain operater’s actual spectrum holdings.

Table 6.39.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 1940 | 1960 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 3880 | 3920 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 5820 | 5880 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 477.1 | 512.1 | 3387.9 | 3422.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 935.8 | 985.8 | 2417.1 | 2472.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4835.8 | 4885.8 | 5327.9 | 5382.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2383.7 | 2448.7 | 4357.1 | 4432.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6283.7 | 6348.7 | 7267.9 | 7342.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1024.2 | 954.2 | 6775.8 | 6845.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6392.1 | 6297.1 | 3911.6 | 3831.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9207.9 | 9302.9 | 7731.6 | 7811.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2984.2 | 2894.2 | 508.7 | 423.7 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8715.8 | 8805.8 | 8223.7 | 8308.7 |

There is no IMD problems to the 3rd own Rx frequency of Band 28 when consider specicific holding frequency bands including Band 21.

6.39.1.3	Co-existence studies for LTE-A UL CA_1A-28A and DL CA_1A-21A-28A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.39.1.3-1 based on a certain operater’s actual spectrum holdings.

Table 6.39.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 728 | 738 | 1940 | 1960 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1456 | 1476 | 3880 | 3920 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2184 | 2214 | 5820 | 5880 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1202 | 1232 | 2668 | 2698 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 504 | 464 | 3142 | 3192 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3396 | 3436 | 4608 | 4658 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 224 | 274 | 5082 | 5152 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4124 | 4174 | 6548 | 6618 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2464 | 2404 | 5336 | 5396 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7112 | 7022 | 1012 | 952 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8488 | 8578 | 4852 | 4912 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4424 | 4344 | 1666 | 1736 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7276 | 7356 | 6064 | 6134 |

There is no IMD problems to the 3rd own Rx frequency of Band 21 when consider specicific holding frequency bands including Band 21.

6.39.1.4	Co-existence studies for LTE-A UL CA_21A-28A and DL CA_1A-21A-28A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.39.1.4-1 based on a certain operater’s actual spectrum holdings.

Table 6.39.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 728 | 738 | 1447.9 | 1462.9 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1456 | 1476 | 2895.8 | 2925.8 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2184 | 2214 | 4343.7 | 4388.7 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 709.9 | 734.9 | 2175.9 | 2200.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 6.9 | 28.1 | 2157.8 | 2197.8 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2903.9 | 2938.9 | 3623.8 | 3663.8 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 721.1 | 766.1 | 3605.7 | 3660.7 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3631.9 | 3676.9 | 5071.7 | 5126.7 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1469.8 | 1419.8 | 4351.8 | 4401.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5123.6 | 5053.6 | 1504.1 | 1449.1 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 6519.6 | 6589.6 | 4359.9 | 4414.9 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2932.7 | 2867.7 | 681.8 | 741.8 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5799.7 | 5864.7 | 5079.8 | 5139.8 |

The 2nd & 3rd IMD fall into the 3rd own Rx frequency of Band 1. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21. Then, there is no IMD problems to the own Rx frequency of Band 1.

6.39.1.5	MSD

When uplink CA configurations CA_1A-21A is paired with downlink CA configuration CA_1A-21A-28A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 28.

When uplink CA configurations CA_1A-28A is paired with downlink CA configuration CA_1A-21A-28A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 21.

When uplink CA configurations CA_21A-28A is paired with downlink CA configuration CA_1A-21A-28A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 1.

6.39.1.6	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.40	LTE Advanced Carrier Aggregation: Band 1 and Band 28 and Band 42 with 2 ULs

6.40.1	List of specific combination issues

6.40.1.1	Channel bandwidths per operating band for CA

Table 6.40.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-28A-42A | CA_1A-28A or CA_1A-42A or CA_28A-42A | 1 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |

6.40.1.2	Co-existence studies for LTE-A UL CA_1A-28A and DL CA_1A-28A-42A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.40.1.2-1.

Table 6.40.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1172 | 1277 | 2623 | 2728 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 574 | 424 | 3092 | 3257 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3326 | 3476 | 4543 | 4708 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 129 | 324 | 5012 | 5237 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4029 | 4224 | 6463 | 6688 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2554 | 2344 | 5246 | 5456 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7217 | 6932 | 1072 | 832 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8383 | 8668 | 4732 | 4972 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4534 | 4264 | 1596 | 1851 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7166 | 7436 | 5949 | 6204 |

The 3rd IMD fall into the 3rd own Rx frequency of Band 42. It will be analysed to slove the self- interference problem.

6.40.1.3	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-28A-42A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.40.1.3-1.

Table 6.40.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1420 | 1680 | 5320 | 5580 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 240 | 560 | 4820 | 5280 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7240 | 7560 | 8720 | 9180 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2160 | 2540 | 8220 | 8880 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9160 | 9540 | 12120 | 12780 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3360 | 2840 | 10640 | 11160 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12480 | 11620 | 4520 | 4080 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15520 | 16380 | 11080 | 11520 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6960 | 6240 | 860 | 1440 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14040 | 14760 | 12560 | 13140 |

The 4th IMD fall into the own Rx Band 1. But this impact already has covered in 2DL/2UL CA_1A-42A.

There is no IMD problems to the 3rd own Rx frequency of Band 28.

6.40.1.4	Co-existence studies for LTE-A UL CA_28A-42A and DL CA_1A-28A-42A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.40.1.4-1.

Table 6.40.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2652 | 2897 | 4103 | 4348 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2194 | 1904 | 6052 | 6497 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4806 | 5096 | 7503 | 7948 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1491 | 1156 | 9452 | 10097 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5509 | 5844 | 10903 | 11548 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 5794 | 5304 | 8206 | 8696 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13697 | 12852 | 408 | 788 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14303 | 15148 | 6212 | 6592 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9394 | 8704 | 4556 | 5091 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11606 | 12296 | 8909 | 9444 |

The 3rd IMD fall into the 3rd own Rx frequency of Band 1. It will be analysed to slove the self- interference problem.

6.40.1.5	MSD

When uplink CA configurations CA_1A-28A is paired with downlink CA configuration CA_1A-28A-42A there is interference components from 2 uplink operation which would interfere the downlink of the Band 42.

When uplink CA configurations CA_1A-42A is paired with downlink CA configuration CA_1A-28A-42A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 28.

When uplink CA configurations CA_28A-42A is paired with downlink CA configuration CA_1A-28A-42A there is interference components from 2 uplink operation which would interfere the downlink of the Band 1.

6.40.1.6	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.41	LTE Advanced Carrier Aggregation: Band 3 and Band 21 and Band 28 with 2 ULs

6.41.1	List of specific combination issues

6.41.1.1	Channel bandwidths per operating band for CA

Table 6.41.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A-21A-28A | CA_3A-21A or CA_3A-28A or CA_21A-28A | 3 |  |  | Yes | Yes | Yes | Yes | 45 | 0 |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |

6.41.1.2	Co-existence studies for LTE-A UL CA_3A-21A and DL CA_3A-21A-28A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.39.1.2-1 based on a certain operater’s actual spectrum holdings.

Table 6.41.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 1764.9 | 1784.9 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 3529.8 | 3569.8 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 5294.7 | 5354.7 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 302 | 337 | 3212.8 | 3247.8 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1110.9 | 1160.9 | 2066.9 | 2121.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4660.7 | 4710.7 | 4977.7 | 5032.7 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2558.8 | 2623.8 | 3831.8 | 3906.8 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6108.6 | 6173.6 | 6742.6 | 6817.6 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 674 | 604 | 6425.6 | 6495.6 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5691.7 | 5596.7 | 4086.7 | 4006.7 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8507.5 | 8602.5 | 7556.5 | 7636.5 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2458.9 | 2368.9 | 858.9 | 773.9 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8190.5 | 8280.5 | 7873.5 | 7958.5 |

The 5th IMD fall into the 3rd own Rx frequency of Band 28. It will be analysed to slove the self- interference problem.

6.41.1.3	Co-existence studies for LTE-A UL CA_3A-28A and DL CA_3A-21A-28A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.41.1.3-1 based on a certain operater’s actual spectrum holdings.

Table 6.41.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 728 | 738 | 1764.9 | 1784.9 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1456 | 1476 | 3529.8 | 3569.8 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2184 | 2214 | 5294.7 | 5354.7 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1026.9 | 1056.9 | 2492.9 | 2522.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 328.9 | 288.9 | 2791.8 | 2841.8 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3220.9 | 3260.9 | 4257.8 | 4307.8 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 399.1 | 449.1 | 4556.7 | 4626.7 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3948.9 | 3998.9 | 6022.7 | 6092.7 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2113.8 | 2053.8 | 4985.8 | 5045.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6411.6 | 6321.6 | 1187.1 | 1127.1 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7787.6 | 7877.6 | 4676.9 | 4736.9 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3898.7 | 3818.7 | 1315.8 | 1385.8 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6750.7 | 6830.7 | 5713.8 | 5783.8 |

The 2nd harmonic by Band 28 fall into the own Rx Band 21. But this impact already has covered in 2DL/2UL CA_21A-28A.

There is no IMD problems to the 3rd own Rx frequency of Band 21 when consider specicific holding frequency bands including Band 21.

6.41.1.4	Co-existence studies for LTE-A UL CA_21A-28A and DL CA_3A-21A-28A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.41.1.4-1 based on a certain operater’s actual spectrum holdings.

Table 6.41.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 728 | 738 | 1447.9 | 1462.9 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1456 | 1476 | 2895.8 | 2925.8 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2184 | 2214 | 4343.7 | 4388.7 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 709.9 | 734.9 | 2175.9 | 2200.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 6.9 | 28.1 | 2157.8 | 2197.8 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2903.9 | 2938.9 | 3623.8 | 3663.8 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 721.1 | 766.1 | 3605.7 | 3660.7 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3631.9 | 3676.9 | 5071.7 | 5126.7 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1469.8 | 1419.8 | 4351.8 | 4401.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5123.6 | 5053.6 | 1504.1 | 1449.1 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 6519.6 | 6589.6 | 4359.9 | 4414.9 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2932.7 | 2867.7 | 681.8 | 741.8 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5799.7 | 5864.7 | 5079.8 | 5139.8 |

There is no IMD problems to the own Rx frequency of Band 3 when consider specicific holding frequency bands including Band 21.

6.41.1.5	MSD

When uplink CA configurations CA_3A-21A is paired with downlink CA configuration CA_3A-21A-28A there is nterference components from 2 uplink operation which would interfere the downlink of the Band 28.

When uplink CA configurations CA_3A-28A is paired with downlink CA configuration CA_3A-21A-28A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 21.

When uplink CA configurations CA_21A-28A is paired with downlink CA configuration CA_3A-21A-28A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 3.

6.41.1.6	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.42	LTE Advanced Carrier Aggregation: Band 3 and Band 28 and Band 42 with 2 ULs

6.42.1	List of specific combination issues

6.42.1.1	Channel bandwidths per operating band for CA

Table 6.42.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A-28A-42A | CA_3A-28A or CA_3A-42A or CA_28A-42A | 3 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |

6.42.1.2	Co-existence studies for LTE-A UL CA_3A-28A and DL CA_3A-28A-42A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.42.1.2-1.

Table 6.42.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 962 | 1082 | 2413 | 2533 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 379 | 214 | 2672 | 2867 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3116 | 3281 | 4123 | 4318 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 324 | 534 | 4382 | 4652 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3819 | 4029 | 5833 | 6103 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2164 | 1924 | 4826 | 5066 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6437 | 6092 | 1282 | 1027 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7543 | 7888 | 4522 | 4777 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3949 | 3634 | 1176 | 1461 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6536 | 6851 | 5529 | 5814 |

The 2nd harmonic by Band 3 fall into the own Rx Band 42. But this impact already has covered in 2DL/2UL CA_3A-42A.

There is no IMD problems to the 3rd own Rx frequency of Band 21 when consider specicific holding frequency bands including Band 21.

Hence, there is no harmonics/IMD problems to the own Rx frequency of Band 42.

6.42.1.3	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_3A-28A-42A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.42.1.3-1.

Table 6.42.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1615 | 1890 | 5110 | 5385 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 180 | 170 | 5015 | 5490 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6820 | 7170 | 8510 | 8985 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1530 | 1955 | 8415 | 9090 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8530 | 8955 | 11910 | 12585 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3780 | 3230 | 10220 | 10770 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12690 | 11815 | 3740 | 3240 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15310 | 16185 | 10240 | 10740 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7380 | 6630 | 1445 | 2070 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13620 | 14370 | 11930 | 12555 |

From the harmonics/IMD analysis table, a number of harmonics/IMDs fall into the own Rx Band 3 and Band 42. But these issues already covered in 2DL/2UL CA_3A-42A.

There is no IMD problems to the 3rd own Rx frequency of Band 28.

6.42.1.4	Co-existence studies for LTE-A UL CA_28A-42A and DL CA_3A-28A-42A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.42.1.4-1.

Table 6.42.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2652 | 2897 | 4103 | 4348 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2194 | 1904 | 6052 | 6497 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4806 | 5096 | 7503 | 7948 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1491 | 1156 | 9452 | 10097 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5509 | 5844 | 10903 | 11548 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 5794 | 5304 | 8206 | 8696 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13697 | 12852 | 408 | 788 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14303 | 15148 | 6212 | 6592 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9394 | 8704 | 4556 | 5091 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11606 | 12296 | 8909 | 9444 |

There is no IMD problems to the 3rd own Rx frequency of Band 3.

6.42.1.5	MSD

When uplink CA configurations CA_3A-28A is paired with downlink CA configuration CA_3A-28A-42A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 42.

When uplink CA configurations CA_3A-42A is paired with downlink CA configuration CA_3A-28A-42A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 28.

When uplink CA configurations CA_28A-42A is paired with downlink CA configuration CA_3A-28A-42A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 3.

6.42.1.6	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.43	LTE Advanced Carrier Aggregation: Band 21 and Band 28 and Band 42 with 2 ULs

6.43.1	List of specific combination issues

6.43.1.1	Channel bandwidths per operating band for CA

Table 6.43.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_21A-28A-42A | CA_21A-28A or CA_21A-42A or CA_28A-42A | 21 |  |  | Yes | Yes | Yes |  | 45 | 0 |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |

6.43.1.2	Co-existence studies for LTE-A UL CA_21A-28A and DL CA_21A-28A-42A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.43.1.2-1 based on a certain operater’s actual spectrum holdings.

Table 6.43.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 728 | 738 | 1447.9 | 1462.9 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1456 | 1476 | 2895.8 | 2925.8 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2184 | 2214 | 4343.7 | 4388.7 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 709.9 | 734.9 | 2175.9 | 2200.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 6.9 | 28.1 | 2157.8 | 2197.8 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2903.9 | 2938.9 | 3623.8 | 3663.8 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 721.1 | 766.1 | 3605.7 | 3660.7 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3631.9 | 3676.9 | 5071.7 | 5126.7 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1469.8 | 1419.8 | 4351.8 | 4401.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5123.6 | 5053.6 | 1504.1 | 1449.1 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 6519.6 | 6589.6 | 4359.9 | 4414.9 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2932.7 | 2867.7 | 681.8 | 741.8 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5799.7 | 5864.7 | 5079.8 | 5139.8 |

The 3rd & 4th IMDs fall into the 3rd own Rx frequency of Band 42. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21. Then, there is no IMD problems to the own Rx frequency of Band 42.

6.43.1.3	Co-existence studies for LTE-A UL CA_21A-42A and DL CA_21A-28A-42A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.43.1.3-1 based on a certain operater’s actual spectrum holdings.

Table 6.43.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2017.1 | 2072.1 | 4927.9 | 4982.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 624.2 | 554.2 | 5497.1 | 5592.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6375.8 | 6445.8 | 8407.9 | 8502.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 823.7 | 908.7 | 8977.1 | 9112.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7823.7 | 7908.7 | 11887.9 | 12022.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 4144.2 | 4034.2 | 9855.8 | 9965.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12632.1 | 12457.1 | 2371.6 | 2271.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15367.9 | 15542.9 | 9271.6 | 9371.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7664.2 | 7514.2 | 2571.3 | 2696.3 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13335.8 | 13485.8 | 11303.7 | 11428.7 |

The 4th IMD fall into the own Rx Band 28. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21. Then, there is no IMD problems to the own Rx frequency of Band 28.

6.43.1.4	Co-existence studies for LTE-A UL CA_28A-42A and DL CA_21A-28A-42A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.43.1.4-1 based on a certain operater’s actual spectrum holdings.

Table 6.43.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 728 | 738 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1456 | 1476 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2184 | 2214 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2742 | 2792 | 4208 | 4258 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2064 | 2004 | 6222 | 6312 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4936 | 4996 | 7688 | 7778 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1336 | 1266 | 9702 | 9832 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5664 | 5734 | 11168 | 11298 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 5584 | 5484 | 8416 | 8516 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13352 | 13182 | 528 | 608 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14648 | 14818 | 6392 | 6472 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9104 | 8964 | 4746 | 4856 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11896 | 12036 | 9144 | 9254 |

The 2nd harmonic by Band 28 fall into the own Rx Band 21. But this impact already has covered in 2DL/2UL CA_21A-28A.

There is no IMD problems to the 3rd own Rx frequency of Band 21, when consider specicific holding frequency bands including Band 21.

6.43.1.5	MSD

When uplink CA configurations CA_21A-28A is paired with downlink CA configuration CA_21A-28A-42A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 42.

When uplink CA configurations CA_21A-42A is paired with downlink CA configuration CA_21A-28A-42A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 28.

When uplink CA configurations CA_28A-42A is paired with downlink CA configuration CA_21A-28A-42A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 21.

6.43.1.6	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.44	LTE Advanced Carrier Aggregation: Band 28 and Band 42 and Band 42 with 2 ULs

6.44.1	List of specific combination issues

6.44.1.1	Channel bandwidths per operating band for CA

Table 6.44.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_28A-42C | CA_28A-42A, CA_42C | 28 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 42 | See CA_42C BCS0 in Table 6.44.1.1-2 |  |  |  |  |  |  |  |

Table 6.44.1.1-2: E-UTRA CA_42C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_42C | 5, 10, 15, 20 | 20 | 40 | 0 |
|  | 20 | 5, 10, 15 |  |  |
|  | 10, 15, 20 | 20 | 40 | 1 |
|  | 20 | 10, 15 |  |  |

6.44.1.2	Co-existence studies for LTE-A UL CA_28A-42A and DL CA_28A-42C

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.44.1.2-1

Table 6.44.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2652 | 2897 | 4103 | 4348 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2194 | 1904 | 6052 | 6497 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4806 | 5096 | 7503 | 7948 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1491 | 1156 | 9452 | 10097 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5509 | 5844 | 10903 | 11548 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 5794 | 5304 | 8206 | 8696 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13697 | 12852 | 408 | 788 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14303 | 15148 | 6212 | 6592 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9394 | 8704 | 4556 | 5091 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11606 | 12296 | 8909 | 9444 |

From the harmonics/IMDs analysis table, there was no harmonics/IMDs impact to the 3rd own Rx frequency of Band 42.

6.44.1.3	Co-existence studies for LTE-A UL CA_42C and DL CA_28A-42C

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.44.1.3-1

Table 6.44.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high |
| --- | --- | --- |
| UL frequency (MHz) | 3400 | 3600 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 6800 | 7200 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 10200 | 10800 |

From the harmonics analysis table, there was no harmonics from Band 42 uplinks which would desensitize the 3rd own Rx frequency of Band 3. Frequency separation of band 3 and 42 is 1520 MHz therefore there is no impact from Band 42 uplink CA to Band 3 reception evenif RAN4 consider asynchronization scenario.

6.44.1.4	MSD

When uplink CA configurations CA_28A-42A is paired with downlink CA configuration CA_28A-42C there is no interference components from 2 uplink operation which would interfere the downlink of the Band 42.

When uplink CA configurations CA_42C is paired with downlink CA configuration CA_28A-42C there is no interference components from 2 uplink operation which would interfere the 3rd downlink of the Band 28.

6.44.1.5	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.45	LTE Advanced Carrier Aggregation: Band 1 and Band 7 and Band 7 with 2 ULs

6.45.1	List of specific combination issues

6.45.1.1	Channel bandwidths per operating band for CA

Table 6.45.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-7A-7A | CA_1A-7A | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 3 in Table 6.45.1.1-2 |  |  |  |  |  |  |  |

Table 6.45.1.1-2: E-UTRA CA_7A-7A configurations and bandwidth combination sets for intra-band CA

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRACA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_7A-7A | - | 5 | 15 |  |  | 40 | 0 |
|  |  | 10 | 10, 15 |  |  |  |  |
|  |  | 15 | 15, 20 |  |  |  |  |
|  |  | 20 | 20 |  |  |  |  |
|  |  | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 40 | 1 |
|  |  | 5, 10, 15, 20 | 5, 10 |  |  | 30 | 2 |
|  |  | 10, 15, 20 | 10, 15, 20 |  |  | 40 | 3 |

6.45.1.2	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-7A-7A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.45.1.2-1

Table 6.45.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 520 | 650 | 4420 | 4550 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1270 | 1460 | 3020 | 3220 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6340 | 6530 | 6920 | 7120 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3190 | 3440 | 5520 | 5790 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8260 | 8510 | 9420 | 9690 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1300 | 1040 | 8840 | 9100 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8360 | 8020 | 5420 | 5110 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11920 | 12260 | 10180 | 10490 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3870 | 3540 | 940 | 620 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11340 | 11670 | 10760 | 11080 |

There are no harmonic or IMD products from band 1 and band 7 uplinks which would desensitize band 7 downlink.

6.45.1.3	MSD

When uplink CA configurations CA_1A-7A is paired with downlink CA configuration CA_1A-7A-7A there is no interference components from 2 uplink operation which would interfere the downlink of the third band.

6.45.1.4	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.46	LTE Advanced Carrier Aggregation: Band 3 and Band 7 and Band 7 with 2 ULs

6.46.1	List of specific combination issues

6.46.1.1	Channel bandwidths per operating band for CA

Table 6.46.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A-7A-7A | CA_3A-7A | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 3 in Table 6.46.1.1-2 |  |  |  |  |  |  |  |

Table 6.46.1.1-2: E-UTRA CA_7A-7A configurations and bandwidth combination sets for intra-band CA

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRACA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_7A-7A | - | 5 | 15 |  |  | 40 | 0 |
|  |  | 10 | 10, 15 |  |  |  |  |
|  |  | 15 | 15, 20 |  |  |  |  |
|  |  | 20 | 20 |  |  |  |  |
|  |  | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 40 | 1 |
|  |  | 5, 10, 15, 20 | 5, 10 |  |  | 30 | 2 |
|  |  | 10, 15, 20 | 10, 15, 20 |  |  | 40 | 3 |

6.46.1.2	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_3A-7A-7A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.46.1.2-1

Table 6.46.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 715 | 860 | 4210 | 4355 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 850 | 1070 | 3215 | 3430 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5920 | 6140 | 6710 | 6925 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2560 | 2855 | 5715 | 6000 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7630 | 7925 | 9210 | 9495 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1720 | 1430 | 8420 | 8710 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8570 | 8215 | 4640 | 4270 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11710 | 12065 | 9340 | 9710 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4290 | 3930 | 355 | 10 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10920 | 11280 | 10130 | 10495 |

The 4th IMD fall into the 3rd own Rx band 7. But this impact already covered in 2UL/2DL CA_3A-7A. So there is no harmonic or IMD products from band 3 and band 7 uplinks which would desensitize band 7 downlink.

6.46.1.3	MSD

When uplink CA configurations CA_3A-7A is paired with downlink CA configuration CA_3A-7A-7A there is no interference components from 2 uplink operation which would interfere the downlink of the third band.

6.46.1.4	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.47	LTE Advanced Carrier Aggregation: Band 5 and Band 7 and Band 7 with 2 ULs

6.47.1	List of specific combination issues

6.47.1.1	Channel bandwidths per operating band for CA

Table 6.47.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_5A-7A-7A | CA_5A-7A | 5 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 3 in Table 6.47.1.1-2 |  |  |  |  |  |  |  |

Table 6.47.1.1-2: E-UTRA CA_7A-7A configurations and bandwidth combination sets for intra-band CA

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRACA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_7A-7A | - | 5 | 15 |  |  | 40 | 0 |
|  |  | 10 | 10, 15 |  |  |  |  |
|  |  | 15 | 15, 20 |  |  |  |  |
|  |  | 20 | 20 |  |  |  |  |
|  |  | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 40 | 1 |
|  |  | 5, 10, 15, 20 | 5, 10 |  |  | 30 | 2 |
|  |  | 10, 15, 20 | 10, 15, 20 |  |  | 40 | 3 |

6.47.1.2	Co-existence studies for LTE-A UL CA_5A-7A and DL CA_5A-7A-7A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.47.1.2-1

Table 6.47.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1651 | 1746 | 3324 | 3419 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 922 | 802 | 4151 | 4316 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4148 | 4268 | 5824 | 5989 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 98 | 47 | 6651 | 6886 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4972 | 5117 | 8324 | 8559 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3492 | 3302 | 6648 | 6838 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9456 | 9151 | 896 | 726 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10824 | 11129 | 5796 | 5966 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6062 | 5802 | 2453 | 2668 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9148 | 9408 | 7472 | 7687 |

The 5th IMD fall into the 3rd own Rx band 7. But this impact already covered in 2UL/2DL CA_5A-7A. So there is no harmonic or IMD products from band 5 and band 7 uplinks which would desensitize band 7 downlink.

6.47.1.3	MSD

When uplink CA configurations CA_5A-7A is paired with downlink CA configuration CA_5A-7A-7A there is no interference components from 2 uplink operation which would interfere the downlink of the third band.

6.47.1.4	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.48	LTE Advanced Carrier Aggregation: Band 3 and Band 41 and Band 42 with 2 ULs

6.48.1	List of specific combination issues

6.48.1.1	Channel bandwidths per operating band for CA

Table 6.48.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A_41A-42A | CA_41A-42A | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 41 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 42 |  |  |  | Yes | Yes | Yes |  |  |

6.48.1.2	Co-existence studies for LTE-A UL CA_41A-42A and DL CA_3A_41A-42A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.48.1.2-1

Table 6.48.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 710 | 1104 | 5896 | 6290 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1392 | 1980 | 4110 | 4704 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8392 | 8980 | 9296 | 9890 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3888 | 4670 | 7510 | 8304 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10888 | 11670 | 12696 | 13490 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2208 | 1420 | 11792 | 12580 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11904 | 10910 | 7360 | 6384 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 16096 | 17090 | 13384 | 14360 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5808 | 4820 | 1270 | 288 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 15192 | 16180 | 14288 | 15270 |

From the harmonics/IMDs analysis table, there were 3rd & 4th IMDs impact to the 3rd own Rx frequency of Band 3.

It will be analysed to slove the self- interference problem.

6.48.1.3	MSD

When uplink CA configurations CA_41A-42A is paired with downlink CA configuration CA_3A_41A-42A there is interference components from 2 uplink operation which would interfere the downlink of the Band 3.

6.48.1.4	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.49	LTE Advanced Carrier Aggregation: Band 28 and Band 41 and Band 42 with 2 ULs

6.49.1	List of specific combination issues

6.49.1.1	Channel bandwidths per operating band for CA

Table 6.49.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_28A-41A-42A | CA_41A-42A | 28 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 41 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 42 |  |  |  | Yes | Yes | Yes |  |  |

6.49.1.2	Co-existence studies for LTE-A UL CA_41A-42A and DL CA_28A_41A-42A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.49.1.2-1

Table 6.49.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 710 | 1104 | 5896 | 6290 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1392 | 1980 | 4110 | 4704 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8392 | 8980 | 9296 | 9890 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3888 | 4670 | 7510 | 8304 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10888 | 11670 | 12696 | 13490 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2208 | 1420 | 11792 | 12580 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11904 | 10910 | 7360 | 6384 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 16096 | 17090 | 13384 | 14360 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5808 | 4820 | 1270 | 288 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 15192 | 16180 | 14288 | 15270 |

From the harmonics/IMDs analysis table, there were 2nd & 5th IMDs impact to the 3rd own Rx frequency of Band 28.

It will be analysed to slove the self- interference problem.

6.49.1.3	MSD

When uplink CA configurations CA_41A-42A is paired with downlink CA configuration CA_28A_41A-42A there is interference components from 2 uplink operation which would interfere the downlink of the Band 28.

6.49.1.4	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7		4DLs/2ULs Inter-Band Carrier Aggregation: Specific Band Combination Part

7.1	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 7 and Band 8 with 2 ULs

7.1.1	List of specific combination issues

7.1.1.1	Channel bandwidths per operating band for CA

Table 7.1.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-7A-8A | CA_1A-3A, CA_1A-7A, CA_1A-8A, CA_3A-7A, CA_3A-8A | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |

7.1.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-7A-8A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.1.1.2-1

Table 7.1.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 270 | 135 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2055 | 2250 | 1440 | 1650 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5550 | 5745 | 5340 | 5550 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3975 | 4230 | 3150 | 3435 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7470 | 7725 | 7050 | 7335 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 270 | 540 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5220 | 4860 | 6210 | 5895 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8760 | 9120 | 9390 | 9705 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1515 | 1170 | 2520 | 2190 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 | 9315 | 9180 | 9510 |

There are no harmonic or IMD products from band 1 and band 3 uplinks which would desensitize band 7 and/or 8 downlink.

7.1.1.3	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-3A-7A-8A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.1.1.3-1

Table 7.1.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 520 | 650 | 4420 | 4550 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1270 | 1460 | 3020 | 3220 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6340 | 6530 | 6920 | 7120 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3190 | 3440 | 5520 | 5790 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8260 | 8510 | 9420 | 9690 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1300 | 1040 | 8840 | 9100 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8360 | 8020 | 5420 | 5110 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11920 | 12260 | 10180 | 10490 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3870 | 3540 | 940 | 620 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11340 | 11670 | 10760 | 11080 |

There are no harmonic or IMD products from band 1 and band 7 uplinks which would desensitize band 3 downlink.

But, 5th IMD product of band 1 and band 7 transmissions will interfere the own band 8 reception.

7.1.1.4	Co-existence studies for LTE-A UL CA_1A-8A and DL CA_1A-3A-7A-8A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.1.1.4-1

Table 7.1.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 2640 | 2745 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1100 | 1005 | 2800 | 2895 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2925 | 3080 | 220 | 90 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4720 | 4875 | 3680 | 3810 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4845 | 5060 | 660 | 825 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6640 | 6855 | 4560 | 4725 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2010 | 2200 | 5600 | 5790 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1740 | 1540 | 7040 | 6765 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5440 | 5640 | 8560 | 8835 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1095 | 1320 | 4180 | 3930 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6480 | 6705 | 7520 | 7770 |

The 2nd harmonics of band 8 transmissions will interfere the own band 3 reception. And aslo 3rd harmonics of band 8 transmissions will interfere the own band 7 reception.

7.1.1.5	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_1A-3A-7A-8A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.1.1.5-1

Table 7.1.1.5-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 715 | 860 | 4210 | 4355 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 850 | 1070 | 3215 | 3430 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5920 | 6140 | 6710 | 6925 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2560 | 2855 | 5715 | 6000 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7630 | 7925 | 9210 | 9495 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1720 | 1430 | 8420 | 8710 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8570 | 8215 | 4640 | 4270 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11710 | 12065 | 9340 | 9710 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4290 | 3930 | 355 | 10 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10920 | 11280 | 10130 | 10495 |

There are no harmonic or IMD products from band 3 and band 7 uplinks which would desensitize band 1 downlink.

The 3rd order IMD product of band 3 and band 7 transmissions will interfere the own band 8 reception.

7.1.1.6	Co-existence studies for LTE-A UL CA_3A-8A and DL CA_1A-3A-7A-8A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.1.1.6-1

Table 7.1.1.6-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 2640 | 2745 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 905 | 795 | 2590 | 2700 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2505 | 2690 | 25 | 120 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4300 | 4485 | 3470 | 3615 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4215 | 4475 | 855 | 1035 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6010 | 6270 | 4350 | 4530 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1590 | 1810 | 5180 | 5400 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1950 | 1735 | 6260 | 5925 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5230 | 5445 | 7720 | 8055 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 675 | 930 | 3595 | 3300 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6060 | 6315 | 6890 | 7185 |

There are no harmonic or IMD products from band 3 and band 8 uplinks which would desensitize band 1 downlink.

The 2nd order & 3rd order IMD product of band 3 and band 8 transmissions will interfere the own band 7 reception. And also 3rd harmonics of band 8 transmissions will interfere the own band 7 reception.

7.1.1.7	MSD

When uplink CA configurations CA_1A-3A is paired with downlink CA configuration CA_1A-3A-7A-8A there are no interference components from 2 uplink operation which would interfere the downlink of the band 7 and band 8.

However when uplink CA configurations CA_1A-7A is paired with downlink CA configuration CA_1A-3A-7A-8A there are interference components from 2 uplink operation which would interfere the downlink of the band 8.

When uplink CA configurations CA_1A-8A is paired with downlink CA configuration CA_1A-3A-7A-8A there are interference components from 2 uplink operation which would interfere the downlink of the band 3 and band 7.

When uplink CA configurations CA_3A-7A is paired with downlink CA configuration CA_1A-3A-7A-8A there are interference components from 2 uplink operation which would interfere the downlink of the band 8.

When uplink CA configurations CA_3A-8A is paired with downlink CA configuration CA_1A-3A-7A-8A there are interference components from 2 uplink operation which would interfere the downlink of the band 7.

7.1.1.8	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7.2	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 5 and Band 40 with 2 ULs

7.2.1	List of specific combination issues

7.2.1.1	Channel bandwidths per operating band for CA

Table 7.2.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-5A-40A | CA_1A-3A or CA_1A-5A or CA_3A-5A | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 40 |  |  |  | Yes | Yes | Yes |  |  |

7.2.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-5A-40A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.2.1.2-1

Table 7.2.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 135 | 270 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1440 | 1650 | 2055 | 2250 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5340 | 5550 | 5550 | 5745 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3150 | 3435 | 3975 | 4230 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7050 | 7335 | 7470 | 7725 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 540 | 270 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6210 | 5895 | 5220 | 4860 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9390 | 9705 | 8760 | 9120 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2520 | 2190 | 1515 | 1170 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9180 | 9510 | 8970 | 9315 |

There are no harmonic or IMD products from band 1 and band 3 uplinks which would desensitize band 5 downlink.

The 5th IMD product of band 1 and band 3 transmissions will interfere the own band 40 reception.

7.2.1.3	Co-existence studies for LTE-A UL CA_1A-5A and DL CA_1A-3A-5A-40A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.2.1.3-1

Table 7.2.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1071 | 1156 | 2744 | 2829 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 332 | 222 | 2991 | 3136 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3568 | 3678 | 4664 | 4809 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 492 | 627 | 4911 | 5116 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4392 | 4527 | 6584 | 6789 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2312 | 2142 | 5488 | 5658 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7096 | 6831 | 1476 | 1316 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8504 | 8769 | 5216 | 5376 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4292 | 4062 | 1293 | 1488 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7408 | 7638 | 6312 | 6507 |

There are no harmonic or IMD products from band 1 and band 5 uplinks which would desensitize band 3 downlink.

But, 4th IMD product of band 1 and band 5 transmissions will interfere the own band 40 reception.

7.2.1.4	Co-existence studies for LTE-A UL CA_3A-5A and DL CA_1A-3A-5A-40A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.2.1.4-1

Table 7.2.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 861 | 961 | 2534 | 2634 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 137 | 12 | 2571 | 2746 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3358 | 3483 | 4244 | 4419 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 687 | 837 | 4281 | 4531 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4182 | 4332 | 5954 | 6204 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1922 | 1722 | 5068 | 5268 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6316 | 5991 | 1686 | 1511 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7664 | 7989 | 5006 | 5181 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3707 | 3432 | 873 | 1098 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6778 | 7053 | 5892 | 6117 |

There are no harmonic or IMD products from band 3 and band 5 uplinks which would desensitize band 1 downlink and/or band 40 downlink.

7.2.1.5	MSD

When uplink CA configurations CA_1A-3A is paired with downlink CA configuration CA_1A-3A-5A-40A there is no interference components from 2 uplink operation which would interfere the downlink of the band 5.

However, when uplink CA configurations CA_1A-3A is paired with downlink CA configuration CA_1A-3A-5A-40A there are interference components from 2 uplink operation which would interfere the downlink of the band 40.

When uplink CA configurations CA_1A-5A is paired with downlink CA configuration CA_1A-3A-5A-40A there is no interference components from 2 uplink operation which would interfere the downlink of the band 3.

However, when uplink CA configurations CA_1A-5A is paired with downlink CA configuration CA_1A-3A-5A-40A there are interference components from 2 uplink operation which would interfere the downlink of the band 40.

When uplink CA configurations CA_3A-5A is paired with downlink CA configuration CA_1A-3A-5A-40A there is no interference components from 2 uplink operation which would interfere the downlink of the band 1 and band 40.

7.2.1.6	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7.3	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 8 and Band 40 with 2 ULs

7.3.1	List of specific combination issues

7.3.1.1	Channel bandwidths per operating band for CA

Table 7.3.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-8A-40A | CA_1A-3A or CA_1A-8A or CA_3A-8A | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes |  |  |

7.3.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-8A-40A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.3.1.2-1

Table 7.3.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 135 | 270 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1440 | 1650 | 2055 | 2250 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5340 | 5550 | 5550 | 5745 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3150 | 3435 | 3975 | 4230 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7050 | 7335 | 7470 | 7725 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 540 | 270 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6210 | 5895 | 5220 | 4860 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9390 | 9705 | 8760 | 9120 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2520 | 2190 | 1515 | 1170 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9180 | 9510 | 8970 | 9315 |

There are no harmonic or IMD products from band 1 and band 3 uplinks which would desensitize band 8 downlink.

The 5th IMD product of band 1 and band 3 transmissions will interfere the own band 40 reception.

7.3.1.3	Co-existence studies for LTE-A UL CA_1A-8A and DL CA_1A-3A-8A-40A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.3.1.3-1

Table 7.3.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1005 | 1100 | 2800 | 2895 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 220 | 90 | 2925 | 3080 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3680 | 3810 | 4720 | 4875 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 660 | 825 | 4845 | 5060 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4560 | 4725 | 6640 | 6855 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2200 | 2010 | 5600 | 5790 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7040 | 6765 | 1740 | 1540 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8560 | 8835 | 5440 | 5640 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4180 | 3930 | 1095 | 1320 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7520 | 7770 | 6480 | 6705 |

The 2nd harmonics of band 8 transmissions will interfere the own band 3 reception.

There are no harmonic or IMD products from band 1 and band 8 uplinks which would desensitize band 40 downlink.

7.3.1.4	Co-existence studies for LTE-A UL CA_3A-8A and DL CA_1A-3A-8A-40A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.3.1.4-1

Table 7.3.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 795 | 905 | 2590 | 2700 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 25 | 120 | 2505 | 2690 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3470 | 3615 | 4300 | 4485 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 855 | 1035 | 4215 | 4475 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4350 | 4530 | 6010 | 6270 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1810 | 1590 | 5180 | 5400 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6260 | 5925 | 1950 | 1735 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7720 | 8055 | 5230 | 5445 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3595 | 3300 | 675 | 930 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6890 | 7185 | 6060 | 6315 |

There are no harmonic or IMD products from band 3 and band 8 uplinks which would desensitize band 1 downlink and/or band 40 downlink.

7.3.1.5	MSD

When uplink CA configurations CA_1A-3A is paired with downlink CA configuration CA_1A-3A-8A-40A there is no interference components from 2 uplink operation which would interfere the downlink of the band 8.

However, when uplink CA configurations CA_1A-3A is paired with downlink CA configuration CA_1A-3A-8A-40A there are interference components from 2 uplink operation which would interfere the downlink of the band 40.

When uplink CA configurations CA_1A-8A is paired with downlink CA configuration CA_1A-3A-8A-40A there is no interference components from 2 uplink operation which would interfere the downlink of the band 40.

However, when uplink CA configurations CA_1A-8A is paired with downlink CA configuration CA_1A-3A-8A-40A there are interference components from 2 uplink operation which would interfere the downlink of the band 3.

When uplink CA configurations CA_3A-8A is paired with downlink CA configuration CA_1A-3A-8A-40A there is no interference components from 2 uplink operation which would interfere the downlink of the band 1 and band 40.

7.3.1.6	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7.4	LTE Advanced Carrier Aggregation: Band 2 and Band 4 and Band 5 and Band 29 with 2 ULs

7.4.1	List of specific combination issues

7.4.1.1	Channel bandwidths per operating band for CA

Table 7.4.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_2A-4A-5A-29A | CA_2A-4A | 2 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |

7.4.1.2	Co-existence studies for LTE-A UL CA_2A-4A and DL CA_2A-4A-5A-29A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.4.1.2-1

Table 7.4.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1755 | 1850 | 1910 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3510 | 3700 | 3820 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5265 | 5550 | 5730 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 95 | 200 | 3560 | 3665 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1510 | 1660 | 1945 | 2110 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5270 | 5420 | 5410 | 5575 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3220 | 3415 | 3795 | 4020 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6980 | 7175 | 7260 | 7485 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 400 | 190 | 7120 | 7330 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5930 | 5645 | 5170 | 4930 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9110 | 9395 | 8690 | 8930 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2310 | 2040 | 1565 | 1310 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 | 9240 | 8830 | 9085 |

There are no harmonic or IMD products from band 2 and band 4 uplinks which would desensitize band 5 downlink and band 29 downlink.

7.4.1.3	MSD

When uplink CA configurations CA_2A-4A is paired with downlink CA configuration CA_2A-4A-5A-29A there are no interference components from 2 uplink operation which would interfere the downlink of these band 5 and band 29.

7.4.1.4	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7.5	LTE Advanced Carrier Aggregation: Band 3 and Band 7 and Band 7 and Band 28 with 2 ULs

7.5.1	List of specific combination issues

7.5.1.1	Channel bandwidths per operating band for CA

Table 7.5.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-7C-28A | CA_3A-7A or CA_7A-28A or CA_7C | 3 |  |  |  | Yes | Yes | Yes | 80 | 0 |
|  |  | 7 | See CA_7C BCS2 in Table 7.5.1.1-2 |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes | Yes | Yes |  |  |

Table 7.5.1.1-2: E-UTRA CA_7C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_7C | CA_7C | 15 | 15 | 40 | 0 |
|  |  | 20 | 20 |  |  |
|  |  | 10 | 20 | 40 | 1 |
|  |  | 15 | 15, 20 |  |  |
|  |  | 20 | 10, 15, 20 |  |  |
|  |  | 15 | 10, 15 | 40 | 2 |
|  |  | 20 | 15, 20 |  |  |

7.5.1.2	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_3A-7C-28A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.5.1.2-1

Table 7.5.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 715 | 860 | 4210 | 4355 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 850 | 1070 | 3215 | 3430 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5920 | 6140 | 6710 | 6925 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2560 | 2855 | 5715 | 6000 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7630 | 7925 | 9210 | 9495 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1720 | 1430 | 8420 | 8710 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8570 | 8215 | 4640 | 4270 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11710 | 12065 | 9340 | 9710 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4290 | 3930 | 355 | 10 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10920 | 11280 | 10130 | 10495 |

There are no harmonic or IMD products from band 3 and band 7 uplinks which would desensitize band 7 downlink.

The 2nd IMD product of band 3 and band 7 transmissions will interfere the own band 28 reception.

7.5.1.3	Co-existence studies for LTE-A UL CA_7A-28A and DL CA_3A-7C-28A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.5.1.3-1

Table 7.5.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1752 | 1867 | 3203 | 3318 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1164 | 1004 | 4252 | 4437 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3906 | 4066 | 5703 | 5888 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 461 | 256 | 6752 | 7007 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4609 | 4814 | 8203 | 8458 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3734 | 3504 | 6406 | 6636 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9577 | 9252 | 492 | 242 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10703 | 11028 | 5312 | 5562 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6304 | 6004 | 2756 | 3031 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8906 | 9206 | 7109 | 7384 |

The 2nd IMD products from band 7 and band 28 transmissions will interfere the own band 3 reception.

There are no harmonic or IMD products from band 7 and band 28 uplinks which would desensitize band 7 downlink.

7.5.1.4	Co-existence studies for LTE-A UL CA_7C and DL CA_3A-7C-28A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics were calculated and presented in Table 7.5.1.4-1

Table 7.5.1.4-1: Harmonic analysis

| UE UL carriers | fy_low | fy_high |
| --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 |

There are no harmonic of band 7 uplinks which would desensitize band 3 downlink and/or band 28 downlink. Frequency separation of band 28 downlink and 7 uplink is 1697 MHz and the frequency separation of band 3 downlink and 7 uplink is 680 MHz, therefore there are no impact from Band 7 uplink CA to these band 3 and band 28 reception.

7.5.1.5	MSD

When uplink CA configurations CA_3A-7A is paired with downlink CA configuration CA_3A-7C-28A there is no interference components from 2 uplink operation which would interfere the downlink of the band 7.

However, when uplink CA configurations CA_3A-7A is paired with downlink CA configuration CA_3A-7C-28A there are interference components from 2 uplink operation which would interfere the downlink of the band 28.

When uplink CA configurations CA_7A-28A is paired with downlink CA configuration CA_3A-7C-28A there is no interference components from 2 uplink operation which would interfere the downlink of the band 7.

However, when uplink CA configurations CA_7A-28A is paired with downlink CA configuration CA_3A-7C-28A there are interference components from 2 uplink operation which would interfere the downlink of the band 3.

When uplink CA configurations CA_7C is paired with downlink CA configuration CA_3A-7C-28A there are no interference components from 2 uplink operation which would interfere the downlink of the band 3 and band 28.

7.5.1.6	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7.6	LTE Advanced Carrier Aggregation: Band 1 and Band 5 and Band 7 and Band 46 with 2 ULs

7.6.1	List of specific combination issues

7.6.1.1	Channel bandwidths per operating band for CA

Table 7.6.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-5A-7A-46A | CA_1A-5A or CA_1A-7A or CA_5A-7A | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 46 |  |  |  |  |  | Yes |  |  |

7.6.1.2	Co-existence studies for LTE-A UL CA_1A-5A and DL CA_1A-5A-7A-46A

For 2UL / 4DL own receiver desensitization study up to 8th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.6.1.2-1 and 7.6.1.2-2

Table 7.6.1.2-1: Harmonic analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 5760 | 5940 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 7680 | 7920 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 9600 | 9900 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 4944 | 5094 | 11520 | 11880 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 5768 | 5943 | 13440 | 13860 |
| 8th harmonics frequency limits | 8*fx_low | 8*fx_high | 8* fy_low | 8* fy_high |
| 8th harmonics frequency limits (MHz) | 6592 | 6792 | 15360 | 15840 |

Table 7.6.1.2-2: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1071 | 1156 | 2744 | 2829 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 332 | 222 | 2991 | 3136 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3568 | 3678 | 4664 | 4809 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 492 | 627 | 4911 | 5116 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4392 | 4527 | 6584 | 6789 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2312 | 2142 | 5488 | 5658 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7096 | 6831 | 1476 | 1316 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8504 | 8769 | 5216 | 5376 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4292 | 4062 | 1293 | 1488 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7408 | 7638 | 6312 | 6507 |

From the harmonics analysis table, the 3rd harmonics by Band 1 fall into the Band 46 own RX ranges.And also the 7th harmonics by Band 5 fall into the Band 46 own RX frequency ranges. But the harmonics problems of each interference band has covered in 2DL/1UL CA_1A-46A in rel-13 and 5A-46A in rel-14 with additional guard band to ganruntee 0dB MSD.

The 4th IMD fall into the own Rx frequency of Band 1. But this impact has covered in 2DL/2UL CA_1A-5A.

The 4th & 5th IMDs by Band 1 and Band 5 fall into the own Rx frequency of Band 46. But these IMD problems also will be covered in 3DL_CA_1A-5A-46A with 2UL_CA_1A-5A.

Hence, there was no interference to the own 3rd and 4th Rx frequency range.

7.6.1.3	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-5A-7A-46A

For 2UL / 4DL own receiver desensitization study up to 8th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.6.1.3-1 and 7.6.1.3-2

Table 7.6.1.3-1: Harmonic analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 7500 | 7710 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 10000 | 10280 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 12500 | 12850 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 11520 | 11880 | 15000 | 15420 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 13440 | 13860 | 17500 | 17990 |
| 8th harmonics frequency limits | 8*fx_low | 8*fx_high | 8* fy_low | 8* fy_high |
| 8th harmonics frequency limits (MHz) | 15360 | 15840 | 20000 | 20560 |

Table 7.6.1.3-2: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2500 | 2570 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 520 | 650 | 4420 | 4550 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1270 | 1460 | 3020 | 3220 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6340 | 6530 | 6920 | 7120 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3190 | 3440 | 5520 | 5790 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8260 | 8510 | 9420 | 9690 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1300 | 1040 | 8840 | 9100 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8360 | 8020 | 5420 | 5110 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11920 | 12260 | 10180 | 10490 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3870 | 3540 | 940 | 620 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11340 | 11670 | 10760 | 11080 |

From the harmonics analysis table, the 3rd harmonics by Band 1 fall into the Band 46 own RX ranges. But the harmonics problem by Band 1 transmission has covered in 2DL/1UL CA_1A-46A in rel-13 with additional guard band to ganruntee 0dB MSD.

The 4th & 5th IMDs fall into the own Rx frequency of Band 46. But these IMD problems also will be covered in 3DL_CA_1A-7A-46A with 2UL_CA_1A-7A.

Hence, there was no interference to the own 3rd and 4th Rx frequency range.

7.6.1.4	Co-existence studies for LTE-A UL CA_5A-7A and DL CA_1A-5A-7A-46A

For 2UL / 4DL own receiver desensitization study up to 8th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.6.1.4-1 and 7.6.1.4-2.

Table 7.6.1.4-1: Harmonic analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 7500 | 7710 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 10000 | 10280 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 12500 | 12850 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 4944 | 5094 | 15000 | 15420 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 5768 | 5943 | 17500 | 17990 |
| 8th harmonics frequency limits | 8*fx_low | 8*fx_high | 8* fy_low | 8* fy_high |
| 8th harmonics frequency limits (MHz) | 6592 | 6792 | 20000 | 20560 |

Table 7.6.1.4-2: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1651 | 1746 | 3324 | 3419 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 922 | 802 | 4151 | 4316 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4148 | 4268 | 5824 | 5989 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 98 | 47 | 6651 | 6886 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4972 | 5117 | 8324 | 8559 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3492 | 3302 | 6648 | 6838 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9456 | 9151 | 896 | 726 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10824 | 11129 | 5796 | 5966 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6062 | 5802 | 2453 | 2668 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9148 | 9408 | 7472 | 7687 |

From the harmonics analysis table, the 7th harmonics by Band 5 fall into the Band 46 own RX ranges. But the harmonics problem by Band 5 transmission has covered in 2DL/1UL CA_5A-46A in rel-14 with additional guard band to ganruntee 0dB MSD.

The 3rd & 5th IMDs fall into the own own Rx frequency of Band 5. Also the 5th IMD fall into the own Rx frequency of Band 7. But these impacts were already covered in 2DL/2UL CA_5A-7A.

The 3rd & 5th IMDs by Band 5 and Band 7 fall into the own Rx frequency of Band 46. But these IMD problems also will be covered in 3DL_CA_5A-7A-46A with 2UL_CA_5A-7A.

Hence, there was no interference to the own 3rd and 4th Rx frequency range.

7.6.1.5	MSD

When uplink CA configurations CA_1A-5A is paired with downlink CA configuration CA_1A-5A-7A-46A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 7 and Band 46.

When uplink CA configurations CA_1A-7A is paired with downlink CA configuration CA_1A-5A-7A-46A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 5 and Band 46.

When uplink CA configurations CA_5A-7A is paired with downlink CA configuration CA_1A-5A-7A-46A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 1 and Band 46.

7.6.1.6	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7.7	LTE Advanced Carrier Aggregation: Band 1 and Band 5 and Band 46 and Band 46 with 2 ULs

7.7.1	List of specific combination issues

7.7.1.1	Channel bandwidths per operating band for CA

Table 7.7.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-5A-46C | CA_1A-5A | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 46 | See the CA_46C BCS0 in Table 7.7.1.1-2 |  |  |  |  |  |  |  |

Table 7.7.1.1-2: E-UTRA CA_46C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_46C | 20 | 20 | 40 | 0 |

7.7.1.2	Co-existence studies for LTE-A UL CA_1A-5A and DL CA_1A-5A-46C

For 2UL / 4DL own receiver desensitization study up to 8th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.7.1.2-1 and 7.7.1.2-2

Table 7.7.1.2-1: Harmonic analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 5760 | 5940 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 7680 | 7920 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 9600 | 9900 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 4944 | 5094 | 11520 | 11880 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 5768 | 5943 | 13440 | 13860 |
| 8th harmonics frequency limits | 8*fx_low | 8*fx_high | 8* fy_low | 8* fy_high |
| 8th harmonics frequency limits (MHz) | 6592 | 6792 | 15360 | 15840 |

Table 7.7.1.2-2: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1071 | 1156 | 2744 | 2829 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 332 | 222 | 2991 | 3136 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3568 | 3678 | 4664 | 4809 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 492 | 627 | 4911 | 5116 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4392 | 4527 | 6584 | 6789 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2312 | 2142 | 5488 | 5658 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7096 | 6831 | 1476 | 1316 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8504 | 8769 | 5216 | 5376 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4292 | 4062 | 1293 | 1488 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7408 | 7638 | 6312 | 6507 |

From the harmonics analysis table, the 3rd harmonics by Band 1 fall into the Band 46 own RX ranges.And also the 7th harmonics by Band 5 fall into the Band 46 own RX frequency ranges. But the harmonics problems of each interference band has covered in 2DL/1UL CA_1A-46A in rel-13 and 5A-46A in rel-14 with additional guard band to ganruntee 0dB MSD.

The 4th IMD fall into the own Rx frequency of Band 1. But this impact has covered in 2DL/2UL CA_1A-5A.

The 4th & 5th IMDs by Band 1 and Band 5 fall into the own Rx frequency of Band 46. But these IMD problems also will be covered in 3DL_CA_1A-5A-46A with 2UL_CA_1A-5A.

Hence, there was no interference to the own 3rd and 4th Rx frequency range.

7.7.1.3		MSD

When uplink CA configurations CA_1A-5A is paired with downlink CA configuration CA_1A-5A-46C there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 46 and Band 46.

7.7.1.4	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7.8	LTE Advanced Carrier Aggregation: Band 1 and Band 7 and Band 46 and Band 46 with 2 ULs

7.8.1	List of specific combination issues

7.8.1.1	Channel bandwidths per operating band for CA

Table 7.8.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-7A-46C | CA_1A-7A | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 46 | See the CA_46C BCS0 in Table 7.8.1.1-2 |  |  |  |  |  |  |  |

Table 7.8.1.1-2: E-UTRA CA_46C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_46C | 20 | 20 | 40 | 0 |

7.8.1.2	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-7A-46C

For 2UL / 4DL own receiver desensitization study up to 8th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.8.1.2-1 and 7.8.1.2-2

Table 7.8.1.2-1: Harmonic analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 7500 | 7710 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 10000 | 10280 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 12500 | 12850 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 11520 | 11880 | 15000 | 15420 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 13440 | 13860 | 17500 | 17990 |
| 8th harmonics frequency limits | 8*fx_low | 8*fx_high | 8* fy_low | 8* fy_high |
| 8th harmonics frequency limits (MHz) | 15360 | 15840 | 20000 | 20560 |

Table 7.8.1.2-2: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2500 | 2570 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 520 | 650 | 4420 | 4550 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1270 | 1460 | 3020 | 3220 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6340 | 6530 | 6920 | 7120 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3190 | 3440 | 5520 | 5790 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8260 | 8510 | 9420 | 9690 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1300 | 1040 | 8840 | 9100 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8360 | 8020 | 5420 | 5110 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11920 | 12260 | 10180 | 10490 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3870 | 3540 | 940 | 620 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11340 | 11670 | 10760 | 11080 |

From the harmonics analysis table, the 3rd harmonics by Band 1 fall into the Band 46 own RX ranges. But the harmonics problem by Band 1 transmission has covered in 2DL/1UL CA_1A-46A in rel-13 with additional guard band to ganruntee 0dB MSD.

The 4th & 5th IMDs fall into the own Rx frequency of Band 46. But these IMD problems also will be covered in 3DL_CA_1A-7A-46A with 2UL_CA_1A-7A.

Hence, there was no interference to the own 3rd and 4th Rx frequency range.

7.8.1.3		MSD

When uplink CA configurations CA_1A-7A is paired with downlink CA configuration CA_1A-7A-46C there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 46 and Band 46.

7.8.1.4	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7.9	LTE Advanced Carrier Aggregation: Band 5 and Band 7 and Band 46 and Band 46 with 2 ULs

7.9.1	List of specific combination issues

7.9.1.1	Channel bandwidths per operating band for CA

Table 7.9.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_5A-7A-46C | CA_5A-7A | 5 |  |  | Yes | Yes |  |  | 70 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 46 | See the CA_46C BCS0 in Table 7.9.1.1-2 |  |  |  |  |  |  |  |

Table 7.9.1.1-2: E-UTRA CA_46C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_46C | 20 | 20 | 40 | 0 |

7.9.1.2	Co-existence studies for LTE-A UL CA_5A-7A and DL CA_5A-7A-46C

For 2UL / 4DL own receiver desensitization study up to 8th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.9.1.2-1 and 7.9.1.2-2

Table 7.9.1.2-1: Harmonic analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 7500 | 7710 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 10000 | 10280 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 12500 | 12850 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 4944 | 5094 | 15000 | 15420 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 5768 | 5943 | 17500 | 17990 |
| 8th harmonics frequency limits | 8*fx_low | 8*fx_high | 8* fy_low | 8* fy_high |
| 8th harmonics frequency limits (MHz) | 6592 | 6792 | 20000 | 20560 |

Table 7.9.1.2-2: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1651 | 1746 | 3324 | 3419 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 922 | 802 | 4151 | 4316 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4148 | 4268 | 5824 | 5989 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 98 | 47 | 6651 | 6886 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4972 | 5117 | 8324 | 8559 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3492 | 3302 | 6648 | 6838 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9456 | 9151 | 896 | 726 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10824 | 11129 | 5796 | 5966 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6062 | 5802 | 2453 | 2668 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9148 | 9408 | 7472 | 7687 |

From the harmonics analysis table, the 7th harmonics by Band 5 fall into the Band 46 own RX ranges. But the harmonics problem by Band 5 transmission has covered in 2DL/1UL CA_5A-46A in rel-14 with additional guard band to ganruntee 0dB MSD.

The 3rd & 5th IMDs fall into the own own Rx frequency of Band 5. Also the 5th IMD fall into the own Rx frequency of Band 7. But these impacts were already covered in 2DL/2UL CA_5A-7A.

The 3rd & 5th IMDs by Band 5 and Band 7 fall into the own Rx frequency of Band 46. But these IMD problems also will be covered in 3DL_CA_5A-7A-46A with 2UL_CA_5A-7A.

Hence, there was no interference to the own 3rd and 4th Rx frequency range.

7.9.1.3		MSD

When uplink CA configurations CA_5A-7A is paired with downlink CA configuration CA_5A-7A-46C there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 46 and Band 46.

7.9.1.4	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7.10	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 5 and Band 7 with 2 ULs

7.10.1	List of specific combination issues

7.10.1.1	Channel bandwidths per operating band for CA

Table 7.10.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-3A-5A-7A | CA_1A-3A or CA_1A-5A or CA_1A-7A or CA_3A-5A or CA_3A-7A or CA_5A-7A | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 3 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |

7.10.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-5A-7A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.10.1.2-1

Table 7.10.1.2-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 135 | 270 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1440 | 1650 | 2055 | 2250 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5340 | 5550 | 5550 | 5745 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3150 | 3435 | 3975 | 4230 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7050 | 7335 | 7470 | 7725 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 540 | 270 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6210 | 5895 | 5220 | 4860 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9390 | 9705 | 8760 | 9120 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2520 | 2190 | 1515 | 1170 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9180 | 9510 | 8970 | 9315 |

From the harmonics/IMDs analysis table, there was no interference to the own 3rd and 4th Rx frequency range.

7.10.1.3	Co-existence studies for LTE-A UL CA_1A-5A and DL CA_1A-3A-5A-7A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.10.1.3-1

Table 7.10.1.3-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1071 | 1156 | 2744 | 2829 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 332 | 222 | 2991 | 3136 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3568 | 3678 | 4664 | 4809 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 492 | 627 | 4911 | 5116 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4392 | 4527 | 6584 | 6789 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2312 | 2142 | 5488 | 5658 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7096 | 6831 | 1476 | 1316 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8504 | 8769 | 5216 | 5376 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4292 | 4062 | 1293 | 1488 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7408 | 7638 | 6312 | 6507 |

From the harmonics/IMDs analysis table, there was no interference to the own 3rd and 4th Rx frequency range.

7.10.1.4	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-3A-5A-7A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.10.1.4-1

Table 7.10.1.4-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 520 | 650 | 4420 | 4550 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1270 | 1460 | 3020 | 3220 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6340 | 6530 | 6920 | 7120 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3190 | 3440 | 5520 | 5790 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8260 | 8510 | 9420 | 9690 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1300 | 1040 | 8840 | 9100 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8360 | 8020 | 5420 | 5110 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11920 | 12260 | 10180 | 10490 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3870 | 3540 | 940 | 620 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11340 | 11670 | 10760 | 11080 |

The 5th IMDs fall into the own Rx frequency of Band 5. But these IMD problems also will be covered in 3DL_CA_1A-5A-7A with 2UL_CA_1A-7A.

Hence, there was no interference to the own 3rd and 4th Rx frequency range.

7.10.1.5	Co-existence studies for LTE-A UL CA_3A-5A and DL CA_1A-5A-7A-46A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.10.1.5-1.

Table 7.10.1.5-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 861 | 961 | 2534 | 2634 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 137 | 12 | 2571 | 2746 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3358 | 3483 | 4244 | 4419 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 687 | 837 | 4281 | 4531 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4182 | 4332 | 5954 | 6204 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1922 | 1722 | 5068 | 5268 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6316 | 5991 | 1686 | 1511 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7664 | 7989 | 5006 | 5181 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3707 | 3432 | 873 | 1098 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6778 | 7053 | 5892 | 6117 |

The 2nd & 3rd IMDs fall into the own Rx frequency of Band 7. But these IMD impacts will be covered in 3DL_CA_3A-5A-7A with 2UL CA_3A-5A.

Hence, there was no interference to the own 3rd and 4th Rx frequency range.

7.10.1.6	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_1A-3A-5A-7A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.10.1.6-1.

Table 7.10.1.6-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 715 | 860 | 4210 | 4355 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 850 | 1070 | 3215 | 3430 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5920 | 6140 | 6710 | 6925 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2560 | 2855 | 5715 | 6000 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7630 | 7925 | 9210 | 9495 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1720 | 1430 | 8420 | 8710 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8570 | 8215 | 4640 | 4270 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11710 | 12065 | 9340 | 9710 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4290 | 3930 | 355 | 10 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10920 | 11280 | 10130 | 10495 |

The 3rd IMD fall into the own Rx frequency of Band 5. But the IMD problem will be covered in 3DL_CA_3A-5A-7A with 2UL_CA_3A-7A.

Hence, there was no interference to the own 3rd and 4th Rx frequency range.

7.10.1.7	Co-existence studies for LTE-A UL CA_5A-7A and DL CA_1A-3A-5A-7A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.10.1.7-1.

Table 7.10.1.7-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1651 | 1746 | 3324 | 3419 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 922 | 802 | 4151 | 4316 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4148 | 4268 | 5824 | 5989 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 98 | 47 | 6651 | 6886 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4972 | 5117 | 8324 | 8559 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3492 | 3302 | 6648 | 6838 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9456 | 9151 | 896 | 726 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10824 | 11129 | 5796 | 5966 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6062 | 5802 | 2453 | 2668 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9148 | 9408 | 7472 | 7687 |

From the harmonics/IMDs analysis table, there was no interference to the own 3rd and 4th Rx frequency range.

7.10.1.8	MSD

When uplink CA configurations CA_1A-3A is paired with downlink CA configuration CA_1A-3A-5A-7A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 5 and Band 7.

When uplink CA configurations CA_1A-5A is paired with downlink CA configuration CA_1A-3A-5A-7A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 3 and Band 7.

When uplink CA configurations CA_1A-7A is paired with downlink CA configuration CA_1A-3A-5A-7A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 3 and Band 5.

When uplink CA configurations CA_3A-5A is paired with downlink CA configuration CA_1A-3A-5A-7A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 1 and Band 7.

When uplink CA configurations CA_3A-7A is paired with downlink CA configuration CA_1A-3A-5A-7A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 1 and Band 5.

When uplink CA configurations CA_5A-7A is paired with downlink CA configuration CA_1A-3A-5A-7A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 1 and Band 3.

7.10.1.9	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7.11	LTE Advanced Carrier Aggregation: Band 1 and Band 19 and Band 21 and Band 42 with 2 ULs

7.11.1	List of specific combination issues

7.11.1.1	Channel bandwidths per operating band for CA

Table 7.11.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-19A-21A-42A | CA_1A-19A or CA_1A-21A or CA_1A-42A or CA_19A-21A or CA_19A-42A or CA_21A-42A | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |  |
|  |  | 19 |  |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |  |

7.11.1.2	Co-existence studies for LTE-A UL CA_1A-19A and DL CA_1A-19A-21A-42A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.11.1.2-1 based on a certain operater’s actual spectrum holdings.

Table 7.11.1.2-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 830 | 845 | 1940 | 1960 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1660 | 1690 | 3880 | 3920 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2490 | 2535 | 5820 | 5880 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1095 | 1130 | 2770 | 2805 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 300 | 250 | 3035 | 3090 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3600 | 3650 | 4710 | 4765 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 530 | 595 | 4975 | 5050 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4430 | 4495 | 6650 | 6725 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2260 | 2190 | 5540 | 5610 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7010 | 6915 | 1440 | 1360 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8590 | 8685 | 5260 | 5340 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4220 | 4130 | 1345 | 1430 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7480 | 7570 | 6370 | 6455 |

The 4th IMD fall into the own Rx Band 1. But this impact already has covered in 2DL/2UL CA_1A-19A.

The 3rd IMD fall into the 4th own Rx frequency of Band 42. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 19. Then, there is no IMD problems to the 3rd & 4th own Rx frequency of Band 21 and Band 42.

7.11.1.3	Co-existence studies for LTE-A UL CA_1A-21A and DL CA_1A-19A-21A-42A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.11.1.3-1 based on a certain operater’s actual spectrum holdings.

Table 7.11.1.3-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 1940 | 1960 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 3880 | 3920 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 5820 | 5880 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 477.1 | 512.1 | 3387.9 | 3422.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 935.8 | 985.8 | 2417.1 | 2472.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4835.8 | 4885.8 | 5327.9 | 5382.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2383.7 | 2448.7 | 4357.1 | 4432.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6283.7 | 6348.7 | 7267.9 | 7342.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1024.2 | 954.2 | 6775.8 | 6845.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6392.1 | 6297.1 | 3911.6 | 3831.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9207.9 | 9302.9 | 7731.6 | 7811.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2984.2 | 2894.2 | 508.7 | 423.7 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8715.8 | 8805.8 | 8223.7 | 8308.7 |

The 2nd IMD fall into the 4th own Rx frequency of Band 42. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21. Then, there is no IMD problems to the 3rd & 4th own Rx frequency of Band 19 and Band 42.

7.11.1.4	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-19A-21A-42A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.11.1.4-1 based on a certain operater’s actual spectrum holdings.

Table 7.11.1.4-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1940 | 1960 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3880 | 3920 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5820 | 5880 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1520 | 1580 | 5420 | 5480 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 360 | 440 | 5000 | 5100 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7360 | 7440 | 8900 | 9000 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2300 | 2400 | 8480 | 8620 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9300 | 9400 | 12380 | 12520 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3160 | 3040 | 10840 | 10960 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12140 | 11960 | 4360 | 4240 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15860 | 16040 | 11240 | 11360 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6680 | 6520 | 1080 | 1220 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14320 | 14480 | 12780 | 12920 |

The 4th IMD fall into the own Rx Band 1. But this impact already has covered in 2DL/2UL CA_1A-42A.

The 2nd IMD fall into the 4th own Rx frequency of Band 21. And also the 5th IMD fall into the 3rd own Rx frequency of Band 19. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 19 and Band 21. Then, there is no IMD problems to the 3rd & 4th own Rx frequency of Band 19 and Band 21.

7.11.1.5	Co-existence studies for LTE-A UL CA_19A-21A and DL CA_1A-19A-21A-42A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.11.1.5-1 based on a certain operater’s actual spectrum holdings.

Table 7.11.1.5-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 830 | 845 | 1447.9 | 1462.9 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1660 | 1690 | 2895.8 | 2925.8 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2490 | 2535 | 4343.7 | 4388.7 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 602.9 | 632.9 | 2277.9 | 2307.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 197.1 | 242.1 | 2050.8 | 2095.8 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3107.9 | 3152.9 | 3725.8 | 3770.8 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1027.1 | 1087.1 | 3498.7 | 3558.7 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3937.9 | 3997.9 | 5173.7 | 5233.7 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1265.8 | 1205.8 | 4555.8 | 4615.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5021.6 | 4946.6 | 1932.1 | 1857.1 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 6621.6 | 6696.6 | 4767.9 | 4842.9 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2728.7 | 2653.7 | 360.8 | 435.8 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6003.7 | 6078.7 | 5385.8 | 5460.8 |

From the harmonics and IMD analysis table, the 4th IMD fall into the 4th own Rx frequency of Band 42. But this IMD problem will be covered in 3DL_CA_19A-21A-42A with 2UL_CA_19A-21A.

7.11.1.6	Co-existence studies for LTE-A UL CA_19A-42A and DL CA_1A-19A-21A-42A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.11.1.6-1 based on a certain operater’s actual spectrum holdings.

Table 7.11.1.6-1: Harmonic and IMDs analysis

| UE U+N5:R30L carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 830 | 845 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1660 | 1690 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2490 | 2535 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2635 | 2690 | 4310 | 4365 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1860 | 1790 | 6115 | 6210 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5140 | 5210 | 7790 | 7885 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1030 | 945 | 9595 | 9730 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5970 | 6055 | 11270 | 11405 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 5380 | 5270 | 8620 | 8730 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13250 | 13075 | 100 | 200 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14750 | 14925 | 6800 | 6900 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 8900 | 8750 | 4425 | 4550 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 12100 | 12250 | 9450 | 9575 |

From the harmonics/IMDs analysis table, there was no interference to the own 3rd and 4th Rx frequency range.

7.11.1.7	Co-existence studies for LTE-A UL CA_21A-42A and DL CA_1A-19A-21A-42A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.11.1.7-1 based on a certain operater’s actual spectrum holdings.

Table 7.11.1.7-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2017.1 | 2072.1 | 4927.9 | 4982.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 624.2 | 554.2 | 5497.1 | 5592.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6375.8 | 6445.8 | 8407.9 | 8502.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 823.7 | 908.7 | 8977.1 | 9112.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7823.7 | 7908.7 | 11887.9 | 12022.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 4144.2 | 4034.2 | 9855.8 | 9965.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12632.1 | 12457.1 | 2371.6 | 2271.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15367.9 | 15542.9 | 9271.6 | 9371.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7664.2 | 7514.2 | 2571.3 | 2696.3 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13335.8 | 13485.8 | 11303.7 | 11428.7 |

The 2nd IMD fall into the 3rd own Rx frequency of Band 1. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21. Then, there is no IMD problems to the 3rd own Rx frequency of Band 1.

The 4th IMD fall into the 3rd own Rx frequency of Band 19 evenif RAN4 consider operator specific frequency range in the related bands. But it also will be covered in 3DL_CA_19A-21A-42A with 2UL_CA_21A-42A.

Hence, there was no interference to the own 3rd and 4th Rx frequency range of Band 1 and Band 19.

7.11.1.8	MSD

When uplink CA configurations CA_1A-19A is paired with downlink CA configuration CA_1A-19A-21A-42A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 21 and Band 42.

When uplink CA configurations CA_1A-21A is paired with downlink CA configuration CA_1A-19A-21A-42A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 19 and Band 42.

When uplink CA configurations CA_1A-42A is paired with downlink CA configuration CA_1A-19A-21A-42A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 19 and Band 21.

When uplink CA configurations CA_19A-21A is paired with downlink CA configuration CA_1A-19A-21A-42A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 1 and Band 42.

When uplink CA configurations CA_19A-42A is paired with downlink CA configuration CA_1A-19A-21A-42A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 1 and Band 21.

When uplink CA configurations CA_21A-42A is paired with downlink CA configuration CA_1A-19A-21A-42A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 1 and Band 19.

7.11.1.9	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7.12	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 19 and Band 42 with 2 ULs

7.12.1	List of specific combination issues

7.12.1.1	Channel bandwidths per operating band for CA

Table 7.12.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-3A-19A-42A | CA_1A-3A or CA_1A-19A or CA_1A-42A or CA_3A-19A or CA_3A-42A or CA_19A-42A | 1 |  |  | Yes | Yes | Yes | Yes | 75 | 0 |  |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |  |
|  |  | 19 |  |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |  |

7.12.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-19A-42A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.12.1.2-1 based on a certain operater’s actual spectrum holdings.

Table 7.12.1.2-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1764.9 | 1784.9 | 1940 | 1960 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3529.8 | 3569.8 | 3880 | 3920 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5294.7 | 5354.7 | 5820 | 5880 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 155.1 | 195.1 | 3704.9 | 3744.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1569.8 | 1629.8 | 2095.1 | 2155.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5469.8 | 5529.8 | 5644.9 | 5704.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3334.7 | 3414.7 | 4035.1 | 4115.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7234.7 | 7314.7 | 7584.9 | 7664.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 390.2 | 310.2 | 7409.8 | 7489.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6075.1 | 5975.1 | 5199.6 | 5099.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9524.9 | 9624.9 | 8999.6 | 9099.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2350.2 | 2250.2 | 1474.7 | 1374.7 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9349.8 | 9449.8 | 9174.7 | 9274.7 |

The 2nd harmonic by Band 3 and 3rd IMD fall into the own Rx Band 1. But this impact already has covered in 2DL/2UL CA_1A-3A.

The 4th IMD fall into the 4th own Rx frequency of Band 42. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 19. Then, there is no IMD problems to the 3rd & 4th own Rx frequency of Band 19 and Band42.

7.12.1.3	Co-existence studies for LTE-A UL CA_1A-19A and DL CA_1A-3A-19A-42A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.12.1.3-1 based on a certain operater’s actual spectrum holdings.

Table 7.12.1.3-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 830 | 845 | 1940 | 1960 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1660 | 1690 | 3880 | 3920 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2490 | 2535 | 5820 | 5880 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1095 | 1130 | 2770 | 2805 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 300 | 250 | 3035 | 3090 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3600 | 3650 | 4710 | 4765 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 530 | 595 | 4975 | 5050 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4430 | 4495 | 6650 | 6725 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2260 | 2190 | 5540 | 5610 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7010 | 6915 | 1440 | 1360 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8590 | 8685 | 5260 | 5340 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4220 | 4130 | 1345 | 1430 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7480 | 7570 | 6370 | 6455 |

The 4th IMD fall into the own Rx Band 1. But this impact already has covered in 2DL/2UL CA_1A-19A.

The 3rd IMD fall into the 4th own Rx frequency of Band 42. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 19. Then, there is no IMD problems to the 3rd & 4th own Rx frequency of Band 3 and Band 42.

7.12.1.4	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-3A-19A-42A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.12.1.4-1 based on a certain operater’s actual spectrum holdings.

Table 7.12.1.4-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1940 | 1960 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3880 | 3920 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5820 | 5880 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1520 | 1580 | 5420 | 5480 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 360 | 440 | 5000 | 5100 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7360 | 7440 | 8900 | 9000 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2300 | 2400 | 8480 | 8620 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9300 | 9400 | 12380 | 12520 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3160 | 3040 | 10840 | 10960 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12140 | 11960 | 4360 | 4240 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15860 | 16040 | 11240 | 11360 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6680 | 6520 | 1080 | 1220 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14320 | 14480 | 12780 | 12920 |

The 4th IMD fall into the own Rx Band 1. But this impact already has covered in 2DL/2UL CA_1A-42A.

The 5th IMD fall into the 4th own Rx frequency of Band 19. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 19. Then, there is no IMD problems to the 3rd & 4th own Rx frequency of Band 3 and Band 19.

7.12.1.5	Co-existence studies for LTE-A UL CA_3A-19A and DL CA_1A-3A-19A-42A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.12.1.5-1 based on a certain operater’s actual spectrum holdings..

Table 7.12.1.5-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 830 | 845 | 1764.9 | 1784.9 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1660 | 1690 | 3529.8 | 3569.8 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2490 | 2535 | 5294.7 | 5354.7 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 919.9 | 954.9 | 2594.9 | 2629.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 124.9 | 74.9 | 2684.8 | 2739.8 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3424.9 | 3474.9 | 4359.8 | 4414.8 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 705.1 | 770.1 | 4449.7 | 4524.7 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4254.9 | 4319.9 | 6124.7 | 6199.7 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1909.8 | 1839.8 | 5189.8 | 5259.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6309.6 | 6214.6 | 1615.1 | 1535.1 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7889.6 | 7984.6 | 5084.9 | 5164.9 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3694.7 | 3604.7 | 994.8 | 1079.8 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6954.7 | 7044.7 | 6019.8 | 6104.8 |

The 2nd & 5th IMDs fall into the own Rx frequency of Band 19. Also the 4th IMD fall into the own Rx frequency of Band 3. But theses impacts already has covered in 2DL/2UL CA_3A-19A.

The 2nd harmonics from Band 3 transmission will be interference to the Band 42. But this harmonics issue already has covered in 2DL/1UL CA_3A-42A.

The 3rd & 5th IMDs fall into the 4th own Rx frequency of Band 42. However, RAN4 consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 19.

Then, there was no harmonics/IMDs problem to the 3rd & 4th own Rx frequency of Band 1 and Band 42.

7.12.1.6	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_1A-3A-19A-42A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.12.1.6-1 based on a certain operater’s actual spectrum holdings.

Table 7.12.1.6-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1764.9 | 1784.9 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3529.8 | 3569.8 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5294.7 | 5354.7 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1695.1 | 1755.1 | 5244.9 | 5304.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 9.8 | 89.8 | 5175.1 | 5275.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7009.8 | 7089.8 | 8724.9 | 8824.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1774.7 | 1874.7 | 8655.1 | 8795.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8774.7 | 8874.7 | 12204.9 | 12344.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3510.2 | 3390.2 | 10489.8 | 10609.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12315.1 | 12135.1 | 3659.6 | 3539.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15684.9 | 15864.9 | 10539.6 | 10659.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7030.2 | 6870.2 | 1605.3 | 1745.3 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13969.8 | 14129.8 | 12254.7 | 12394.7 |

The 2nd , 4th & 5th IMDs fall into the own Rx frequency of Band 3. Also the 2nd harmonics from Band 3, 4th & 5th IMDs fall into the own Rx frequency of Band 42. But these impact already has covered in 2DL/2UL CA_3A-42A.

Hence, there was no harmonics/IMDs impact to the 3rd & 4th own Rx frequency of Band 1 and Band 19.

7.12.1.7	Co-existence studies for LTE-A UL CA_19A-42A and DL CA_1A-3A-19A-42A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.12.1.7-1 based on a certain operater’s actual spectrum holdings.

Table 7.12.1.7-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 830 | 845 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1660 | 1690 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2490 | 2535 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2635 | 2690 | 4310 | 4365 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1860 | 1790 | 6115 | 6210 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5140 | 5210 | 7790 | 7885 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1030 | 945 | 9595 | 9730 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5970 | 6055 | 11270 | 11405 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 5380 | 5270 | 8620 | 8730 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13250 | 13075 | 100 | 200 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14750 | 14925 | 6800 | 6900 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 8900 | 8750 | 4425 | 4550 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 12100 | 12250 | 9450 | 9575 |

The 4th IMD fall into the own Rx frequency of Band 19. But this impact already has covered in 2DL/2UL CA_19A-42A.

The 3rd IMD fall into the 4th own Rx frequency of Band 3. The overlap range is 0.1MHz that is no critical impact to the sensitivity of Band 3 when RAN4 consider operator specific frequency ranges.

Hence, there was no harmonics/IMDs impact to the 3rd & 4th own Rx frequency of Band 1 and Band 3.

7.12.1.8	MSD

When uplink CA configurations CA_1A-19A is paired with downlink CA configuration CA_1A-19A-21A-42A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 21 and Band 42.

When uplink CA configurations CA_1A-21A is paired with downlink CA configuration CA_1A-19A-21A-42A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 19 and Band 42.

When uplink CA configurations CA_1A-42A is paired with downlink CA configuration CA_1A-19A-21A-42A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 19 and Band 21.

When uplink CA configurations CA_19A-21A is paired with downlink CA configuration CA_1A-19A-21A-42A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 1 and Band 42.

When uplink CA configurations CA_19A-42A is paired with downlink CA configuration CA_1A-19A-21A-42A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 1 and Band 21.

When uplink CA configurations CA_21A-42A is paired with downlink CA configuration CA_1A-19A-21A-42A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 1 and Band 19.

7.12.1.9	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7.13	LTE Advanced Carrier Aggregation: Band 1 and Band 19 and Band 42 and Band 42 with 2 ULs

7.13.1	List of specific combination issues

7.13.1.1	Channel bandwidths per operating band for CA

Table 7.13.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-19A-42C | CA_1A-19A or CA_1A-42A or CA_19A-42A | 1 |  |  | Yes | Yes | Yes | Yes | 75 | 0 |  |
|  |  | 19 |  |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 42C | See CA_42C Bandwidth Combination Set 0 in Table 7.13.1.1-2 |  |  |  |  |  |  |  |  |

Table 7.13.1.1-2: E-UTRA CA_42C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_42C | 5, 10, 15, 20 | 20 | 40 | 0 |
|  | 20 | 5, 10, 15 |  |  |
|  | 10, 15, 20 | 20 | 40 | 1 |
|  | 20 | 10, 15 |  |  |

7.13.1.2	Co-existence studies for LTE-A UL CA_1A-19A and DL CA_1A-19A-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.13.1.2-1 based on a certain operater’s actual spectrum holdings.

Table 7.13.1.2-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 830 | 845 | 1940 | 1960 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1660 | 1690 | 3880 | 3920 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2490 | 2535 | 5820 | 5880 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1095 | 1130 | 2770 | 2805 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 300 | 250 | 3035 | 3090 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3600 | 3650 | 4710 | 4765 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 530 | 595 | 4975 | 5050 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4430 | 4495 | 6650 | 6725 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2260 | 2190 | 5540 | 5610 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7010 | 6915 | 1440 | 1360 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8590 | 8685 | 5260 | 5340 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4220 | 4130 | 1345 | 1430 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7480 | 7570 | 6370 | 6455 |

The 4th IMD fall into the own Rx Band 1. But this impact already has covered in 2DL/2UL CA_1A-19A.

The 3rd IMD fall into the 3rd own Rx frequency of Band 42. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 19. Then, there is no IMD problems to the 3rd & 4th own Rx frequency of Band 42.

7.13.1.3	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-19A-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.13.1.3-1 based on a certain operater’s actual spectrum holdings.

Table 7.13.1.3-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1940 | 1960 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3880 | 3920 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5820 | 5880 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1520 | 1580 | 5420 | 5480 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 360 | 440 | 5000 | 5100 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7360 | 7440 | 8900 | 9000 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2300 | 2400 | 8480 | 8620 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9300 | 9400 | 12380 | 12520 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3160 | 3040 | 10840 | 10960 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12140 | 11960 | 4360 | 4240 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15860 | 16040 | 11240 | 11360 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6680 | 6520 | 1080 | 1220 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14320 | 14480 | 12780 | 12920 |

The 4th IMD fall into the own Rx Band 1. But this impact already has covered in 2DL/2UL CA_1A-42A.

The 5th IMD fall into the 3rd own Rx frequency of Band 19. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 19. Then, there is no IMD problems to the 3rd & 4th own Rx frequency of Band 19 and Band 42.

7.13.1.4	Co-existence studies for LTE-A UL CA_19A-42A and DL CA_1A-19A-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.13.1.4-1 based on a certain operater’s actual spectrum holdings.

Table 7.13.1.4-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 830 | 845 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1660 | 1690 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2490 | 2535 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2635 | 2690 | 4310 | 4365 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1860 | 1790 | 6115 | 6210 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5140 | 5210 | 7790 | 7885 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1030 | 945 | 9595 | 9730 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5970 | 6055 | 11270 | 11405 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 5380 | 5270 | 8620 | 8730 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13250 | 13075 | 100 | 200 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14750 | 14925 | 6800 | 6900 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 8900 | 8750 | 4425 | 4550 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 12100 | 12250 | 9450 | 9575 |

The 4th IMD fall into the own Rx frequency of Band 19. But this impact already has covered in 2DL/2UL CA_19A-42A.

Hence, there was no harmonics/IMDs impact to the 3rd & 4th own Rx frequency of Band 1 and Band 42.

7.13.1.5		MSD

When uplink CA configurations CA_1A-19A is paired with downlink CA configuration CA_1A-19A-42C there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 42 and Band 42.

When uplink CA configurations CA_1A-42A is paired with downlink CA configuration CA_1A-19A-42C there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 19 and Band 42.

When uplink CA configurations CA_19A-42A is paired with downlink CA configuration CA_1A-19A-42C there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 1 and Band 42.

7.13.1.6	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7.14	LTE Advanced Carrier Aggregation: Band 1 and Band 21 and Band 42 and Band 42 with 2 ULs

7.14.1	List of specific combination issues

7.14.1.1	Channel bandwidths per operating band for CA

Table 7.14.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-21A-42C | CA_1A-21A or CA_1A-42A or CA_21A-42A | 1 |  |  | Yes | Yes | Yes | Yes | 75 | 0 |  |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 42C | See CA_42C Bandwidth Combination Set 0 in Table 7.14.1.1-2 |  |  |  |  |  |  |  |  |

Table 7.14.1.1-2: E-UTRA CA_42C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_42C | 5, 10, 15, 20 | 20 | 40 | 0 |
|  | 20 | 5, 10, 15 |  |  |
|  | 10, 15, 20 | 20 | 40 | 1 |
|  | 20 | 10, 15 |  |  |

7.14.1.2	Co-existence studies for LTE-A UL CA_1A-21A and DL CA_1A-21A-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.14.1.2-1 based on a certain operater’s actual spectrum holdings.

Table 7.14.1.2-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 1940 | 1960 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 3880 | 3920 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 5820 | 5880 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 477.1 | 512.1 | 3387.9 | 3422.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 935.8 | 985.8 | 2417.1 | 2472.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4835.8 | 4885.8 | 5327.9 | 5382.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2383.7 | 2448.7 | 4357.1 | 4432.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6283.7 | 6348.7 | 7267.9 | 7342.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1024.2 | 954.2 | 6775.8 | 6845.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6392.1 | 6297.1 | 3911.6 | 3831.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9207.9 | 9302.9 | 7731.6 | 7811.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2984.2 | 2894.2 | 508.7 | 423.7 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8715.8 | 8805.8 | 8223.7 | 8308.7 |

The 2nd IMD fall into the 3rd own Rx frequency of Band 42. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21. Then, there is no IMD problems to the 3rd & 4th own Rx frequency of Band 42.

7.14.1.3	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-21A-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.14.1.3-1 based on a certain operater’s actual spectrum holdings.

Table 7.14.1.3-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1940 | 1960 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3880 | 3920 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5820 | 5880 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1520 | 1580 | 5420 | 5480 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 360 | 440 | 5000 | 5100 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7360 | 7440 | 8900 | 9000 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2300 | 2400 | 8480 | 8620 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9300 | 9400 | 12380 | 12520 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3160 | 3040 | 10840 | 10960 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12140 | 11960 | 4360 | 4240 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15860 | 16040 | 11240 | 11360 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6680 | 6520 | 1080 | 1220 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14320 | 14480 | 12780 | 12920 |

The 4th IMD fall into the own Rx Band 1. But this impact already has covered in 2DL/2UL CA_1A-42A.

The 2nd IMD fall into the 3rd own Rx frequency of Band 21. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21. Then, there is no IMD problems to the 3rd & 4th own Rx frequency of Band 21 and Band 42.

7.14.1.4	Co-existence studies for LTE-A UL CA_21A-42A and DL CA_1A-21A-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.14.1.4-1 based on a certain operater’s actual spectrum holdings.

Table 7.14.1.4-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2017.1 | 2072.1 | 4927.9 | 4982.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 624.2 | 554.2 | 5497.1 | 5592.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6375.8 | 6445.8 | 8407.9 | 8502.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 823.7 | 908.7 | 8977.1 | 9112.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7823.7 | 7908.7 | 11887.9 | 12022.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 4144.2 | 4034.2 | 9855.8 | 9965.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12632.1 | 12457.1 | 2371.6 | 2271.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15367.9 | 15542.9 | 9271.6 | 9371.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7664.2 | 7514.2 | 2571.3 | 2696.3 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13335.8 | 13485.8 | 11303.7 | 11428.7 |

The 2nd IMD fall into the 3rd own Rx frequency of Band 1. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21. Then, there is no IMD problems to the 3rd & 4th own Rx frequency of Band 1 and Band 42.

7.14.1.5		MSD

When uplink CA configurations CA_1A-21A is paired with downlink CA configuration CA_1A-21A-42C there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 42 and Band 42.

When uplink CA configurations CA_1A-42A is paired with downlink CA configuration CA_1A-21A-42C there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 21 and Band 42.

When uplink CA configurations CA_21A-42A is paired with downlink CA configuration CA_1A-21A-42C there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 1 and Band 42.

7.14.1.6	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7.15	LTE Advanced Carrier Aggregation: Band 19 and Band 21 and Band 42 and Band 42 with 2 ULs

7.15.1	List of specific combination issues

7.15.1.1	Channel bandwidths per operating band for CA

Table 7.15.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_19A-21A-42C | CA_19A-21A or CA_19A-42A or CA_21A-42A | 19 |  |  | Yes | Yes | Yes |  | 70 | 0 |  |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 42C | See CA_42C Bandwidth Combination Set 0 in Table 7.15.1.1-2 |  |  |  |  |  |  |  |  |

Table 7.15.1.1-2: E-UTRA CA_42C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_42C | 5, 10, 15, 20 | 20 | 40 | 0 |
|  | 20 | 5, 10, 15 |  |  |
|  | 10, 15, 20 | 20 | 40 | 1 |
|  | 20 | 10, 15 |  |  |

7.15.1.2	Co-existence studies for LTE-A UL CA_19A-21A and DL CA_19A-21A-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.15.1.2-1.

Table 7.15.1.2-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 830 | 845 | 1447.9 | 1462.9 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1660 | 1690 | 2895.8 | 2925.8 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2490 | 2535 | 4343.7 | 4388.7 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 602.9 | 632.9 | 2277.9 | 2307.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 197.1 | 242.1 | 2050.8 | 2095.8 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3107.9 | 3152.9 | 3725.8 | 3770.8 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1027.1 | 1087.1 | 3498.7 | 3558.7 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3937.9 | 3997.9 | 5173.7 | 5233.7 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1265.8 | 1205.8 | 4555.8 | 4615.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5021.6 | 4946.6 | 1932.1 | 1857.1 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 6621.6 | 6696.6 | 4767.9 | 4842.9 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2728.7 | 2653.7 | 360.8 | 435.8 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6003.7 | 6078.7 | 5385.8 | 5460.8 |

The 2nd IMD fall into the 3rd own Rx frequency of Band 42. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 19 and Band 21. Also, the IMD impact will be covered 3DL_CA_19A-21A-42A with 2UL_CA_19A-21A.

Hence, there is no IMD problems to the 3rd & 4th own Rx frequency of Band 42.

7.15.1.3	Co-existence studies for LTE-A UL CA_19A-42A and DL CA_19A-21A-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.15.1.3-1 based on a certain operater’s actual spectrum holdings.

Table 7.15.1.3-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 830 | 845 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1660 | 1690 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2490 | 2535 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2635 | 2690 | 4310 | 4365 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1860 | 1790 | 6115 | 6210 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5140 | 5210 | 7790 | 7885 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1030 | 945 | 9595 | 9730 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5970 | 6055 | 11270 | 11405 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 5380 | 5270 | 8620 | 8730 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13250 | 13075 | 100 | 200 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14750 | 14925 | 6800 | 6900 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 8900 | 8750 | 4425 | 4550 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 12100 | 12250 | 9450 | 9575 |

The 4th IMD fall into the own Rx frequency of Band 19. But this impact already has covered in 2DL/2UL CA_19A-42A.

Hence, there was no harmonics/IMDs impact to the 3rd & 4th own Rx frequency of Band 21 and Band 42.

7.15.1.4	Co-existence studies for LTE-A UL CA_21A-42A and DL CA_19A-21A-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.15.1.4-1 based on a certain operater’s actual spectrum holdings.

Table 7.15.1.4-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2017.1 | 2072.1 | 4927.9 | 4982.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 624.2 | 554.2 | 5497.1 | 5592.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6375.8 | 6445.8 | 8407.9 | 8502.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 823.7 | 908.7 | 8977.1 | 9112.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7823.7 | 7908.7 | 11887.9 | 12022.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 4144.2 | 4034.2 | 9855.8 | 9965.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12632.1 | 12457.1 | 2371.6 | 2271.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15367.9 | 15542.9 | 9271.6 | 9371.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7664.2 | 7514.2 | 2571.3 | 2696.3 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13335.8 | 13485.8 | 11303.7 | 11428.7 |

The 4th IMD fall into the 3rd own Rx frequency of Band 19 when RAN4 consider operator specific frequency range in the related bands. But, the IMD impact will be covered 3DL_CA_19A-21A-42A with 2UL_CA_21A-42A.

Hence, there is no IMD problems to the 3rd & 4th own Rx frequency of Band 19 and Band 42.

7.15.1.5		MSD

When uplink CA configurations CA_19A-21A is paired with downlink CA configuration CA_19A-21A-42C there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 42 and Band 42.

When uplink CA configurations CA_19A-42A is paired with downlink CA configuration CA_19A-21A-42C there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 21 and Band 42.

When uplink CA configurations CA_21A-42A is paired with downlink CA configuration CA_19A-21A-42C there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 19 and Band 42.

7.15.1.6	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7.16	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 42 and Band 42 with 2 ULs

7.16.1	List of specific combination issues

7.16.1.1	Channel bandwidths per operating band for CA

Table 7.16.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-3A-42C | CA_1A-3A or CA_1A-42A or CA_3A-42A | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |  |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |  |
|  |  | 42C | See CA_42C Bandwidth Combination Set 0 in Table 7.16.1.1-2 |  |  |  |  |  |  |  |  |

Table 7.16.1.1-2: E-UTRA CA_42C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_42C | 5, 10, 15, 20 | 20 | 40 | 0 |
|  | 20 | 5, 10, 15 |  |  |
|  | 10, 15, 20 | 20 | 40 | 1 |
|  | 20 | 10, 15 |  |  |

7.16.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.16.1.2-1.

Table 7.16.1.2-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 135 | 270 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1440 | 1650 | 2055 | 2250 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5340 | 5550 | 5550 | 5745 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3150 | 3435 | 3975 | 4230 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7050 | 7335 | 7470 | 7725 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 540 | 270 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6210 | 5895 | 5220 | 4860 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9390 | 9705 | 8760 | 9120 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2520 | 2190 | 1515 | 1170 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9180 | 9510 | 8970 | 9315 |

The 2nd harmonic by Band 3 and 3rd IMD fall into the own Rx Band 1. But this impact already has covered in 2DL/2UL CA_1A-3A.

The 4th IMD fall into the 3rd own Rx frequency of Band 42. But, the IMD impact will be covered 3DL_CA_1A-3A-42A with 2UL_CA_1A-3A.

Hence, there is no IMD problems to the 3rd & 4th own Rx frequency of Band 42.

7.16.1.3	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-3A-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.16.1.3-1.

Table 7.16.1.3-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1420 | 1680 | 5320 | 5580 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 240 | 560 | 4820 | 5280 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7240 | 7560 | 8720 | 9180 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2160 | 2540 | 8220 | 8880 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9160 | 9540 | 12120 | 12780 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3360 | 2840 | 10640 | 11160 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12480 | 11620 | 4520 | 4080 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15520 | 16380 | 11080 | 11520 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6960 | 6240 | 860 | 1440 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14040 | 14760 | 12560 | 13140 |

The 3rd IMD fall into the own Rx Band 1. But this impact already has covered in 2DL/2UL CA_1A-42A.

Hence, there was no harmonics/IMDs impact to the 3rd & 4th own Rx frequency of Band 3 and Band 42.

7.16.1.4	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_1A-3A-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.16.1.4-1.

Table 7.16.1.4-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1615 | 1890 | 5110 | 5385 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 180 | 170 | 5015 | 5490 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6820 | 7170 | 8510 | 8985 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1530 | 1955 | 8415 | 9090 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8530 | 8955 | 11910 | 12585 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3780 | 3230 | 10220 | 10770 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12690 | 11815 | 3740 | 3240 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15310 | 16185 | 10240 | 10740 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7380 | 6630 | 1445 | 2070 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13620 | 14370 | 11930 | 12555 |

The 2nd , 4th & 5th IMDs fall into the own Rx frequency of Band 3. Also the 2nd harmonics from Band 3, 4th & 5th IMDs fall into the own Rx frequency of Band 42. But these impact already has covered in 2DL/2UL CA_3A-42A.

Hence, there was no harmonics/IMDs impact to the 3rd & 4th own Rx frequency of Band 1 and Band 42.

7.16.1.5		MSD

When uplink CA configurations CA_1A-3A is paired with downlink CA configuration CA_1A-3A-42C there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 42 and Band 42.

When uplink CA configurations CA_1A-42A is paired with downlink CA configuration CA_1A-3A-42C there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 3 and Band 42.

When uplink CA configurations CA_3A-42A is paired with downlink CA configuration CA_1A-3A-42C there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 1 and Band 42.

7.16.1.6	∆TIB and ∆RIB values

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 830 | 845 | 1447.9 | 1462.9 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1660 | 1690 | 2895.8 | 2925.8 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2490 | 2535 | 4343.7 | 4388.7 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 602.9 | 632.9 | 2277.9 | 2307.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 197.1 | 242.1 | 2050.8 | 2095.8 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3107.9 | 3152.9 | 3725.8 | 3770.8 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1027.1 | 1087.1 | 3498.7 | 3558.7 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3937.9 | 3997.9 | 5173.7 | 5233.7 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1265.8 | 1205.8 | 4555.8 | 4615.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5021.6 | 4946.6 | 1932.1 | 1857.1 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 6621.6 | 6696.6 | 4767.9 | 4842.9 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2728.7 | 2653.7 | 360.8 | 435.8 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6003.7 | 6078.7 | 5385.8 | 5460.8 |

[Editor Note] It will be updated in future

7.17	LTE Advanced Carrier Aggregation: Band 3 and Band 19 and Band 42 and Band 42 with 2 ULs

7.17.1	List of specific combination issues

7.17.1.1	Channel bandwidths per operating band for CA

Table 7.17.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A-19A-42C | CA_3A-19A or CA_3A-42A or CA_19A-42A | 3 |  |  | Yes | Yes | Yes | Yes | 75 | 0 |  |
|  |  | 19 |  |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 42C | See CA_42C Bandwidth Combination Set 0 in Table 7.17.1.1-2 |  |  |  |  |  |  |  |  |

Table 7.17.1.1-2: E-UTRA CA_42C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_42C | 5, 10, 15, 20 | 20 | 40 | 0 |
|  | 20 | 5, 10, 15 |  |  |
|  | 10, 15, 20 | 20 | 40 | 1 |
|  | 20 | 10, 15 |  |  |

7.17.1.2	Co-existence studies for LTE-A UL CA_3A-19A and DL CA_3A-19A-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.17.1.2-1 based on a certain operater’s actual spectrum holdings.

Table 7.17.1.2-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 830 | 845 | 1764.9 | 1784.9 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1660 | 1690 | 3529.8 | 3569.8 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2490 | 2535 | 5294.7 | 5354.7 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 919.9 | 954.9 | 2594.9 | 2629.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 124.9 | 74.9 | 2684.8 | 2739.8 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3424.9 | 3474.9 | 4359.8 | 4414.8 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 705.1 | 770.1 | 4449.7 | 4524.7 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4254.9 | 4319.9 | 6124.7 | 6199.7 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1909.8 | 1839.8 | 5189.8 | 5259.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6309.6 | 6214.6 | 1615.1 | 1535.1 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7889.6 | 7984.6 | 5084.9 | 5164.9 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3694.7 | 3604.7 | 994.8 | 1079.8 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6954.7 | 7044.7 | 6019.8 | 6104.8 |

The 2nd & 5th IMDs fall into the own Rx frequency of Band 19. Also the 4th IMD fall into the own Rx frequency of Band 3. But theses impacts already has covered in 2DL/2UL CA_3A-19A.

The 2nd harmonics from Band 3 transmission will be interference to the Band 42. But this harmonics issue already has covered in 2DL/1UL CA_3A-42A.

The 3rd & 5th IMDs fall into the 3rd own Rx frequency of Band 42. However, RAN4 consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 19.

Then, there was no harmonics/IMDs problem to the 3rd & 4th own Rx frequency of Band 42.

7.17.1.3	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_3A-19A-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.17.1.3-1 based on a certain operater’s actual spectrum holdings.

Table 7.17.1.3-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1764.9 | 1784.9 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3529.8 | 3569.8 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5294.7 | 5354.7 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1695.1 | 1755.1 | 5244.9 | 5304.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 9.8 | 89.8 | 5175.1 | 5275.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7009.8 | 7089.8 | 8724.9 | 8824.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1774.7 | 1874.7 | 8655.1 | 8795.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8774.7 | 8874.7 | 12204.9 | 12344.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3510.2 | 3390.2 | 10489.8 | 10609.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12315.1 | 12135.1 | 3659.6 | 3539.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15684.9 | 15864.9 | 10539.6 | 10659.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7030.2 | 6870.2 | 1605.3 | 1745.3 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13969.8 | 14129.8 | 12254.7 | 12394.7 |

The 2nd , 4th & 5th IMDs fall into the own Rx frequency of Band 3. Also the 2nd harmonics from Band 3, 4th & 5th IMDs fall into the own Rx frequency of Band 42. But these impact already has covered in 2DL/2UL CA_3A-42A.

Hence, there was no harmonics/IMDs impact to the 3rd & 4th own Rx frequency of Band 19 and Band 42

7.17.1.4	Co-existence studies for LTE-A UL CA_19A-42A and DL CA_3A-19A-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.17.1.4-1 based on a certain operater’s actual spectrum holdings.

Table 7.17.1.4-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 830 | 845 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1660 | 1690 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2490 | 2535 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2635 | 2690 | 4310 | 4365 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1860 | 1790 | 6115 | 6210 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5140 | 5210 | 7790 | 7885 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1030 | 945 | 9595 | 9730 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5970 | 6055 | 11270 | 11405 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 5380 | 5270 | 8620 | 8730 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13250 | 13075 | 100 | 200 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14750 | 14925 | 6800 | 6900 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 8900 | 8750 | 4425 | 4550 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 12100 | 12250 | 9450 | 9575 |

The 4th IMD fall into the own Rx frequency of Band 19. But this impact already has covered in 2DL/2UL CA_19A-42A.

The 3rd IMD fall into the 3rd own Rx frequency of Band 3. The overlap range is 0.1MHz that is no critical impact to the sensitivity of Band 3 when RAN4 consider operator specific frequency ranges.

7.17.1.5		MSD

When uplink CA configurations CA_3A-19A is paired with downlink CA configuration CA_3A-19A-42C there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 42 and Band 42.

When uplink CA configurations CA_3A-42A is paired with downlink CA configuration CA_3A-19A-42C there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 19 and Band 42.

When uplink CA configurations CA_19A-42A is paired with downlink CA configuration CA_3A-19A-42C there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 3 and Band 42.

7.17.1.6	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7.18	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 3 and Band 8 with 2 ULs

7.18.1	List of specific combination issues

7.18.1.1	Channel bandwidths per operating band for CA

Table 7.18.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz |  | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-3C-8A | CA_1A-3A or CA_1A-8A or CA_3A-8A or CA_3C | 1 |  |  | Yes | Yes | Yes |  | Yes | 70 | 0 |
|  |  | 3 | See CA_3C BCS0 in Table 7.18.1.1-2 |  |  |  |  |  |  |  |  |
|  |  | 8 |  | Yes | Yes | Yes |  |  |  |  |  |

Table 7-18.1.1-2: E-UTRA CA_3C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_3C | 5, 10, 15 | 20 | 40 | 0 |
|  | 20 | 5, 10, 15, 20 |  |  |

7.18.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3C-8A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.18.1.2-1.

Table 7.18.1.2-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 135 | 270 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1440 | 1650 | 2055 | 2250 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5340 | 5550 | 5550 | 5745 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3150 | 3435 | 3975 | 4230 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7050 | 7335 | 7470 | 7725 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 540 | 270 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6210 | 5895 | 5220 | 4860 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9390 | 9705 | 8760 | 9120 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2520 | 2190 | 1515 | 1170 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9180 | 9510 | 8970 | 9315 |

The 3rd IMD fall into the own Rx Band 1. But this impact already has covered in 2DL/2UL CA_1A-3A.

Hence, there is no harmonics/IMDs impact to the 3rd & 4th own Rx frequency of Band 3 and Band 8

7.18.1.3	Co-existence studies for LTE-A UL CA_1A-8A and DL CA_1A-3C-8A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.18.1.3-1.

Table 7.18.1.3-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1005 | 1100 | 2800 | 2895 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 220 | 90 | 2925 | 3080 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3680 | 3810 | 4720 | 4875 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 660 | 825 | 4845 | 5060 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4560 | 4725 | 6640 | 6855 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2200 | 2010 | 5600 | 5790 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7040 | 6765 | 1740 | 1540 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8560 | 8835 | 5440 | 5640 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4180 | 3930 | 1095 | 1320 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7520 | 7770 | 6480 | 6705 |

The 2nd harmonics by Band 8 fall into the own Rx Band 3. But this impact already has covered in 2DL/1UL CA_3A-8A.And 4th IMD impact to the Band 1. But the IMD impact already has covered in 2DL/2UL CA_1A-8A.

Hence, there is no self-interference problems to the 3rd & 4th own Rx frequency of Band 3 and Band 3.

7.18.1.4	Co-existence studies for LTE-A UL CA_3A-8A and DL CA_1A-3C-8A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.18.1.4-1.

Table 7.18.1.4-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 795 | 905 | 2590 | 2700 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 25 | 120 | 2505 | 2690 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3470 | 3615 | 4300 | 4485 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 855 | 1035 | 4215 | 4475 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4350 | 4530 | 6010 | 6270 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1810 | 1590 | 5180 | 5400 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6260 | 5925 | 1950 | 1735 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7720 | 8055 | 5230 | 5445 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3595 | 3300 | 675 | 930 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6890 | 7185 | 6060 | 6315 |

The 2nd harmonics by Band 8 fall into the own Rx Band 3. But this impact already has covered in 2DL/1UL CA_3A-8A.And 4th & 5th IMD impacts to the Band 3. But the IMD impact already has covered in 2DL/2UL CA_3A-8A.

Hence, there is no self-interference problems to the 3rd & 4th own Rx frequency of Band 1 and Band 3.

7.18.1.5	Co-existence studies for LTE-A UL CA_3C and DL CA_1A-3C-8A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.18.1.5-1.

Table 7.18.1.5-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high |
| --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 |

From the harmonic analysis table, there was no harmonics from Band 3 uplinks which would desensitize the 3rd own Rx frequency of Band 1 and Band 8. Frequency separation of band 1 downlink and 3 uplink is 325 MHz and Frequency separation of Band 8 downlink and 3 uplink is 750 MHz therefore there is no impact from Band 3 uplink CA to Band 1 and Band 8 reception.

7.18.1.6		MSD

When uplink CA configurations CA_1A-3A is paired with downlink CA configuration CA_1A-3C-8A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 3 and Band 8.

When uplink CA configurations CA_1A-8A is paired with downlink CA configuration CA_1A-3C-8A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 3 and Band 3.

When uplink CA configurations CA_3A-8A is paired with downlink CA configuration CA_1A-3C-8A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 1 and Band 3.

When uplink CA configurations CA_3C is paired with downlink CA configuration CA_1A-3C-8A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 1 and Band 8.

7.18.1.7	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7.19	LTE Advanced Carrier Aggregation: Band 41 and Band 41 and Band 42 and Band 42 with 2 ULs

7.19.1	List of specific combination issues

7.19.1.1	Channel bandwidths per operating band for CA

Table 7.19.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_41C-42C | CA_41A-42A, CA_41C, CA_42C | 41 | See CA_41C BCS0 in Table 7.19.1.1-2 |  |  |  |  |  | 80 | 0 |
|  |  | 42 | See CA_42C BCS1 in Table 7.19.1.1-3 |  |  |  |  |  |  |  |

Table 7-19.1.1-2: E-UTRA CA_41C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_41C | 10 | 20 | 40 | 0 |
|  | 15 | 15, 20 |  |  |
|  | 20 | 10, 15, 20 |  |  |

Table 7.19.1.1-3: E-UTRA CA_42C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_42C | 5, 10, 15, 20 | 20 | 40 | 0 |
|  | 20 | 5, 10, 15 |  |  |
|  | 10, 15, 20 | 20 | 40 | 1 |
|  | 20 | 10, 15 |  |  |

7.19.1.2	Co-existence studies for LTE-A UL CA_41A-42A and DL CA_41C-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.19.1.2-1

Table 7.19.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 710 | 1104 | 5896 | 6290 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1392 | 1980 | 4110 | 4704 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8392 | 8980 | 9296 | 9890 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3888 | 4670 | 7510 | 8304 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10888 | 11670 | 12696 | 13490 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2208 | 1420 | 11792 | 12580 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11904 | 10910 | 7360 | 6384 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 16096 | 17090 | 13384 | 14360 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5808 | 4820 | 1270 | 288 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 15192 | 16180 | 14288 | 15270 |

From the harmonics/IMDs analysis table, there is no harmonics/IMDs impact to the 3rd & 4th own Rx frequency of Band 41 and Band 42.

7.19.1.3	Co-existence studies for LTE-A UL CA_42C and DL CA_41C-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.19.1.3-1

Table 7.19.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high |
| --- | --- | --- |
| UL frequency (MHz) | 3400 | 3600 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 6800 | 7200 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 10200 | 10800 |

From the harmonics analysis table, there was no harmonics from Band 42 uplinks which would desensitize the 3rd own Rx frequency of Band 41. Frequency separation of band 41 and 42 is 710 MHz therefore there is no impact from Band 42 uplink CA to Band 41 reception evenif RAN4 consider asynchronization scenario.

7.19.1.4	Co-existence studies for LTE-A UL CA_41C and DL CA_41C-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.19.1.4-1

Table 7.19.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high |
| --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 |

From the harmonics analysis table, there was no harmonics from Band 41 uplinks which would desensitize the 3rd own Rx frequency of Band 42. Frequency separation of band 41 and 42 is 710 MHz therefore there is no impact from Band 41 uplink CA to Band 42 reception evenif RAN4 consider asynchronization scenario.

7.19.1.5	MSD

When uplink CA configurations CA_41A-42A is paired with downlink CA configuration CA_41C-42C there is no interference components from 2 uplink operation which would interfere the downlink of the Band 41 and Band 42.

When uplink CA configurations CA_42C is paired with downlink CA configuration CA_41C-42C there is no interference components from 2 uplink operation which would interfere the downlink of the Band 41 and Band 41.

When uplink CA configurations CA_41C is paired with downlink CA configuration CA_41C-42C there is no interference components from 2 uplink operation which would interfere the downlink of the Band 42 and Band 42.

7.19.1.6	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7.20	LTE Advanced Carrier Aggregation: Band 2 and Band 4 and Band 7 and Band 7 with 2 ULs

7.20.1	List of specific combination issues

7.20.1.1	Channel bandwidths per operating band for CA

Table 7.20.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_2A-4A-7A | CA_2A-4A | 2 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 7 | See the CA_7A-7A BCS1 in Table 7.20.1.1-2 |  |  |  |  |  |  |  |

7.20.1.2	Co-existence studies for LTE-A UL CA_2A-4A and DL CA_2A-4A-7A-7A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.20.1.2-1

Table 7.20.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1755 | 1850 | 1910 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3510 | 3700 | 3820 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5265 | 5550 | 5730 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 95 | 200 | 3560 | 3665 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1510 | 1660 | 1945 | 2110 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5270 | 5420 | 5410 | 5575 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3220 | 3415 | 3795 | 4020 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6980 | 7175 | 7260 | 7485 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 400 | 190 | 7120 | 7330 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5930 | 5645 | 5170 | 4930 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9110 | 9395 | 8690 | 8930 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2310 | 2040 | 1565 | 1310 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 | 9240 | 8830 | 9085 |

The 3rd & 5th IMDs fall into the own Rx frequency of Band 2 and Band 4. But these impacts already covered in 2UL/2DL CA_2A-4A in Rel-12.

Hence, there was no harmonic or IMD products from Band 2 and Band 4 uplinks which would desensitize Band 7 downlink.

7.20.1.3	MSD

When uplink CA configurations CA_2A-4A is paired with downlink CA configuration CA_2A-4A-7A-7A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 7.

7.20.1.4	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7.21	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 19 and Band 21 with 2 ULs

7.21.1	List of specific combination issues

7.21.1.1	Channel bandwidths per operating band for CA

Table 7.21.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-3A-19A-21A | CA_1A-3A or CA_1A-19A or CA_1A-21A or CA_3A-19A or CA_3A-21A or CA_19A-21A | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |  |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |  |
|  |  | 19 |  |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |  |

7.21.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-19A-21A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.21.1.2-1 based on a certain operater’s actual spectrum holdings.

Table 7.21.1.2-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1764.9 | 1784.9 | 1940 | 1960 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3529.8 | 3569.8 | 3880 | 3920 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5294.7 | 5354.7 | 5820 | 5880 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 155.1 | 195.1 | 3704.9 | 3744.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1569.8 | 1629.8 | 2095.1 | 2155.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5469.8 | 5529.8 | 5644.9 | 5704.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3334.7 | 3414.7 | 4035.1 | 4115.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7234.7 | 7314.7 | 7584.9 | 7664.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 390.2 | 310.2 | 7409.8 | 7489.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6075.1 | 5975.1 | 5199.6 | 5099.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9524.9 | 9624.9 | 8999.6 | 9099.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2350.2 | 2250.2 | 1474.7 | 1374.7 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9349.8 | 9449.8 | 9174.7 | 9274.7 |

The 3rd & 5th IMDs fall into the 4th own Rx frequency of Band 21. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 19 and Band 21. Then, there is no IMD problems to the 3rd & 4th own Rx frequency of Band 19 and Band21.

7.21.1.3	Co-existence studies for LTE-A UL CA_1A-19A and DL CA_1A-3A-19A-21A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.21.1.3-1 based on a certain operater’s actual spectrum holdings.

Table 7.21.1.3-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 830 | 845 | 1940 | 1960 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1660 | 1690 | 3880 | 3920 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2490 | 2535 | 5820 | 5880 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1095 | 1130 | 2770 | 2805 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 300 | 250 | 3035 | 3090 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3600 | 3650 | 4710 | 4765 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 530 | 595 | 4975 | 5050 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4430 | 4495 | 6650 | 6725 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2260 | 2190 | 5540 | 5610 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7010 | 6915 | 1440 | 1360 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8590 | 8685 | 5260 | 5340 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4220 | 4130 | 1345 | 1430 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7480 | 7570 | 6370 | 6455 |

The 4th IMD fall into the own Rx Band 1. But this impact already has covered in 2DL/2UL CA_1A-19A.

There is no harmonics/IMD problems to the 3rd & 4th own Rx frequency of Band 3 and Band 21 when RAN4 consider specicific holding frequency bands.

7.21.1.4	Co-existence studies for LTE-A UL CA_1A-21A and DL CA_1A-3A-19A-21A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.21.1.4-1 based on a certain operater’s actual spectrum holdings.

Table 7.21.1.4-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 1940 | 1960 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 3880 | 3920 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 5820 | 5880 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 477.1 | 512.1 | 3387.9 | 3422.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 935.8 | 985.8 | 2417.1 | 2472.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4835.8 | 4885.8 | 5327.9 | 5382.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2383.7 | 2448.7 | 4357.1 | 4432.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6283.7 | 6348.7 | 7267.9 | 7342.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1024.2 | 954.2 | 6775.8 | 6845.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6392.1 | 6297.1 | 3911.6 | 3831.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9207.9 | 9302.9 | 7731.6 | 7811.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2984.2 | 2894.2 | 508.7 | 423.7 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8715.8 | 8805.8 | 8223.7 | 8308.7 |

From the harmonics/IMD analysis table, there is no IMD problems to the 3rd & 4th own Rx frequency of Band 3 and Band 19 when RAN4 consider specicific holding frequency bands.

7.21.1.5	Co-existence studies for LTE-A UL CA_3A-19A and DL CA_1A-3A-19A-21A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.21.1.5-1 based on a certain operater’s actual spectrum holdings..

Table 7.21.1.5-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 830 | 845 | 1764.9 | 1784.9 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1660 | 1690 | 3529.8 | 3569.8 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2490 | 2535 | 5294.7 | 5354.7 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 919.9 | 954.9 | 2594.9 | 2629.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 124.9 | 74.9 | 2684.8 | 2739.8 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3424.9 | 3474.9 | 4359.8 | 4414.8 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 705.1 | 770.1 | 4449.7 | 4524.7 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4254.9 | 4319.9 | 6124.7 | 6199.7 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1909.8 | 1839.8 | 5189.8 | 5259.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6309.6 | 6214.6 | 1615.1 | 1535.1 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7889.6 | 7984.6 | 5084.9 | 5164.9 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3694.7 | 3604.7 | 994.8 | 1079.8 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6954.7 | 7044.7 | 6019.8 | 6104.8 |

The 2nd & 5th IMDs fall into the own Rx frequency of Band 19. Also the 4th IMD fall into the own Rx frequency of Band 3. But theses impacts already has covered in 2DL/2UL CA_3A-19A.

Hence, there was no harmonics/IMDs problem to the 3rd & 4th own Rx frequency of Band 1 and Band 21 when RAN4 consider specicific holding frequency bands.

7.21.1.6	Co-existence studies for LTE-A UL CA_3A-21A and DL CA_1A-3A-19A-21A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.21.1.6-1 based on a certain operater’s actual spectrum holdings.

Table 7.21.1.6-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 1764.9 | 1784.9 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 3529.8 | 3569.8 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 5294.7 | 5354.7 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 302 | 337 | 3212.8 | 3247.8 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1110.9 | 1160.9 | 2066.9 | 2121.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4660.7 | 4710.7 | 4977.7 | 5032.7 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2558.8 | 2623.8 | 3831.8 | 3906.8 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6108.6 | 6173.6 | 6742.6 | 6817.6 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 674 | 604 | 6425.6 | 6495.6 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5691.7 | 5596.7 | 4086.7 | 4006.7 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8507.5 | 8602.5 | 7556.5 | 7636.5 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2458.9 | 2368.9 | 858.9 | 773.9 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8190.5 | 8280.5 | 7873.5 | 7958.5 |

The 3rd IMD fall into the own Rx frequency of Band 1. Also the 5th IMD fall into the own Rx frequency of Band 19. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 19 and Band 21. Then, there is no IMD problems to the 3rd & 4th own Rx frequency of Band 1 and Band 19.

Hence, there was no harmonics/IMDs impact to the 3rd & 4th own Rx frequency of Band 1 and Band 19.

7.21.1.7	Co-existence studies for LTE-A UL CA_19A-21A and DL CA_1A-3A-19A-21A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.21.1.7-1 based on a certain operater’s actual spectrum holdings.

Table 7.21.1.7-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 830 | 845 | 1447.9 | 1462.9 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1660 | 1690 | 2895.8 | 2925.8 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2490 | 2535 | 4343.7 | 4388.7 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 602.9 | 632.9 | 2277.9 | 2307.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 197.1 | 242.1 | 2050.8 | 2095.8 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3107.9 | 3152.9 | 3725.8 | 3770.8 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1027.1 | 1087.1 | 3498.7 | 3558.7 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3937.9 | 3997.9 | 5173.7 | 5233.7 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1265.8 | 1205.8 | 4555.8 | 4615.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5021.6 | 4946.6 | 1932.1 | 1857.1 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 6621.6 | 6696.6 | 4767.9 | 4842.9 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2728.7 | 2653.7 | 360.8 | 435.8 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6003.7 | 6078.7 | 5385.8 | 5460.8 |

The 5th IMD fall into the 4th own Rx frequency of Band 3. But this impact already covered in 3DL CA_3A-19A-21A with 2UL CA_19A-21A.

Hence, there was no harmonics/IMDs impact to the 3rd & 4th own Rx frequency of Band 1 and Band 3.

7.21.1.8	MSD

When uplink CA configurations CA_1A-3A is paired with downlink CA configuration CA_1A-3A-19A-21A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 19 and Band 21.

When uplink CA configurations CA_1A-19A is paired with downlink CA configuration CA_1A-3A-19A-21A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 3 and Band 21.

When uplink CA configurations CA_1A-21A is paired with downlink CA configuration CA_1A-3A-19A-21A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 3 and Band 19.

When uplink CA configurations CA_3A-19A is paired with downlink CA configuration CA_1A-3A-19A-21A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 1 and Band 21.

When uplink CA configurations CA_3A-21A is paired with downlink CA configuration CA_1A-3A-19A-21A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 1 and Band 19.

When uplink CA configurations CA_19A-21A is paired with downlink CA configuration CA_1A-3A-19A-21A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 1 and Band 3.

7.21.1.9	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7.22	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 21 and Band 28 with 2 ULs

7.22.1	List of specific combination issues

7.22.1.1	Channel bandwidths per operating band for CA

Table 7.22.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-3A-21A-28A | CA_1A-3A or CA_1A-21A or CA_1A-28A or CA_3A-21A or CA_3A-28A or CA_21A-28A | 1 |  |  | Yes | Yes | Yes | Yes | 65 | 0 |  |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |  |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |  |

7.22.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-21A-28A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.22.1.2-1 based on a certain operater’s actual spectrum holdings.

Table 7.22.1.2-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1764.9 | 1784.9 | 1940 | 1960 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3529.8 | 3569.8 | 3880 | 3920 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5294.7 | 5354.7 | 5820 | 5880 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 155.1 | 195.1 | 3704.9 | 3744.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1569.8 | 1629.8 | 2095.1 | 2155.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5469.8 | 5529.8 | 5644.9 | 5704.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3334.7 | 3414.7 | 4035.1 | 4115.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7234.7 | 7314.7 | 7584.9 | 7664.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 390.2 | 310.2 | 7409.8 | 7489.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6075.1 | 5975.1 | 5199.6 | 5099.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9524.9 | 9624.9 | 8999.6 | 9099.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2350.2 | 2250.2 | 1474.7 | 1374.7 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9349.8 | 9449.8 | 9174.7 | 9274.7 |

The 3th & 5th IMDs fall into the 4th own Rx frequency of Band 21. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21. Then, there is no IMD problems to the 3rd & 4th own Rx frequency of Band 21 and Band 28.

7.22.1.3	Co-existence studies for LTE-A UL CA_1A-21A and DL CA_1A-3A-21A-28A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.22.1.3-1 based on a certain operater’s actual spectrum holdings.

Table 7.22.1.3-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 1940 | 1960 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 3880 | 3920 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 5820 | 5880 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 477.1 | 512.1 | 3387.9 | 3422.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 935.8 | 985.8 | 2417.1 | 2472.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4835.8 | 4885.8 | 5327.9 | 5382.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2383.7 | 2448.7 | 4357.1 | 4432.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6283.7 | 6348.7 | 7267.9 | 7342.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1024.2 | 954.2 | 6775.8 | 6845.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6392.1 | 6297.1 | 3911.6 | 3831.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9207.9 | 9302.9 | 7731.6 | 7811.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2984.2 | 2894.2 | 508.7 | 423.7 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8715.8 | 8805.8 | 8223.7 | 8308.7 |

There is no harmonics/IMD problems to the 3rd & 4th own Rx frequency of Band 3 and Band 28 when RAN4 consider specicific holding frequency bands.

7.22.1.4	Co-existence studies for LTE-A UL CA_1A-28A and DL CA_1A-3A-21A-28A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.22.1.4-1 based on a certain operater’s actual spectrum holdings.

Table 7.22.1.4-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 728 | 738 | 1940 | 1960 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1456 | 1476 | 3880 | 3920 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2184 | 2214 | 5820 | 5880 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1202 | 1232 | 2668 | 2698 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 504 | 464 | 3142 | 3192 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3396 | 3436 | 4608 | 4658 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 224 | 274 | 5082 | 5152 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4124 | 4174 | 6548 | 6618 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2464 | 2404 | 5336 | 5396 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7112 | 7022 | 1012 | 952 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8488 | 8578 | 4852 | 4912 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4424 | 4344 | 1666 | 1736 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7276 | 7356 | 6064 | 6134 |

The 2nd harmonics from Band 28 fall into the own Rx Band 21. But this impact already has covered in 2DL/2UL CA_21A-28A.

Also the 5th IMD fall into the own Rx Band 3. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21. Then, there is no IMD problems to the 3rd & 4th own Rx frequency of Band 3 and Band 21.

7.22.1.5	Co-existence studies for LTE-A UL CA_3A-21A and DL CA_1A-3A-21A-28A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.22.1.5-1 based on a certain operater’s actual spectrum holdings..

Table 7.22.1.5-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 1764.9 | 1784.9 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 3529.8 | 3569.8 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 5294.7 | 5354.7 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 302 | 337 | 3212.8 | 3247.8 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1110.9 | 1160.9 | 2066.9 | 2121.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4660.7 | 4710.7 | 4977.7 | 5032.7 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2558.8 | 2623.8 | 3831.8 | 3906.8 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6108.6 | 6173.6 | 6742.6 | 6817.6 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 674 | 604 | 6425.6 | 6495.6 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5691.7 | 5596.7 | 4086.7 | 4006.7 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8507.5 | 8602.5 | 7556.5 | 7636.5 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2458.9 | 2368.9 | 858.9 | 773.9 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8190.5 | 8280.5 | 7873.5 | 7958.5 |

The 3rd IMD fall into the own Rx frequency of Band 1. However RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21. Then, there is no IMD problems to the 3rd own Rx frequency of Band 1.

Also the 5th IMD fall into the own Rx frequency of Band 28. But theses impacts already has covered in 3DL CA_3A-21A-28A with 2UL_CA_3A-21A.

Then, there was no harmonics/IMDs problem to the 3rd & 4th own Rx frequency of Band 1 and Band 28 when RAN4 consider specicific holding frequency bands.

7.22.1.6	Co-existence studies for LTE-A UL CA_3A-28A and DL CA_1A-3A-21A-28A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.22.1.6-1 based on a certain operater’s actual spectrum holdings.

Table 7.22.1.6-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 728 | 738 | 1764.9 | 1784.9 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1456 | 1476 | 3529.8 | 3569.8 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2184 | 2214 | 5294.7 | 5354.7 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1026.9 | 1056.9 | 2492.9 | 2522.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 328.9 | 288.9 | 2791.8 | 2841.8 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3220.9 | 3260.9 | 4257.8 | 4307.8 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 399.1 | 449.1 | 4556.7 | 4626.7 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3948.9 | 3998.9 | 6022.7 | 6092.7 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2113.8 | 2053.8 | 4985.8 | 5045.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6411.6 | 6321.6 | 1187.1 | 1127.1 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7787.6 | 7877.6 | 4676.9 | 4736.9 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3898.7 | 3818.7 | 1315.8 | 1385.8 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6750.7 | 6830.7 | 5713.8 | 5783.8 |

The 2nd harmonics from Band 28 fall into the own Rx band 21. Also the 3rd harmonics from Band 28 fall into the own Rx band Band 1. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21. Then, there is no harmonics problems to the 3rd & 4th own Rx frequency of Band 1 and Band 21.

Also, the 4th IMD fall into the own Rx Band 1. But this impact already covered in 3DL CA_1A-3A-28A with 2UL CA_3A-28A.

Hence, there was no harmonics/IMDs impact to the 3rd & 4th own Rx frequency of Band 1 and Band 21.

7.22.1.7	Co-existence studies for LTE-A UL CA_21A-28A and DL CA_1A-3A-21A-28A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.22.1.7-1 based on a certain operater’s actual spectrum holdings.

Table 7.22.1.7-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 728 | 738 | 1447.9 | 1462.9 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1456 | 1476 | 2895.8 | 2925.8 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2184 | 2214 | 4343.7 | 4388.7 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 709.9 | 734.9 | 2175.9 | 2200.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 6.9 | 28.1 | 2157.8 | 2197.8 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2903.9 | 2938.9 | 3623.8 | 3663.8 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 721.1 | 766.1 | 3605.7 | 3660.7 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3631.9 | 3676.9 | 5071.7 | 5126.7 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1469.8 | 1419.8 | 4351.8 | 4401.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5123.6 | 5053.6 | 1504.1 | 1449.1 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 6519.6 | 6589.6 | 4359.9 | 4414.9 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2932.7 | 2867.7 | 681.8 | 741.8 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5799.7 | 5864.7 | 5079.8 | 5139.8 |

The 3rd harmonics from Band 28 fall into the own Rx band Band 1. Also, the 2nd & 3rd IMDs fall into the own Rx Band 1. However, RAN4 can consider specicific holding frequency bands when study the harmonics/IMD analysis for the related CA band combinations including Band 21.

Then, there is no harmonics problems to the 3rd & 4th own Rx frequency of Band 1 and Band 3.

7.22.1.8	MSD

When uplink CA configurations CA_1A-3A is paired with downlink CA configuration CA_1A-3A-21A-28A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 21 and Band 28.

When uplink CA configurations CA_1A-21A is paired with downlink CA configuration CA_1A-3A-21A-28A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 3 and Band 28.

When uplink CA configurations CA_1A-28A is paired with downlink CA configuration CA_1A-3A-21A-28A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 3 and Band 21.

When uplink CA configurations CA_3A-21A is paired with downlink CA configuration CA_1A-3A-21A-28A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 1 and Band 28.

When uplink CA configurations CA_3A-28A is paired with downlink CA configuration CA_1A-3A-21A-28A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 1 and Band 21.

When uplink CA configurations CA_21A-28A is paired with downlink CA configuration CA_1A-3A-21A-28A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 1 and Band 3.

7.22.1.9	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7.23	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 21 and Band 42 with 2 ULs

7.23.1	List of specific combination issues

7.23.1.1	Channel bandwidths per operating band for CA

Table 7.23.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-3A-21A-42A | CA_1A-3A, CA_1A-21A, CA_1A-42A, CA_3A-21A, CA_3A-42A, CA_21A-42A | 1 |  |  | Yes | Yes | Yes | Yes | 75 | 0 |  |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |  |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |  |

7.23.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-21A-42A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.23.1.2-1 based on a certain operater’s actual spectrum holdings.

Table 7.23.1.2-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1764.9 | 1784.9 | 1940 | 1960 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3529.8 | 3569.8 | 3880 | 3920 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5294.7 | 5354.7 | 5820 | 5880 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 155.1 | 195.1 | 3704.9 | 3744.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1569.8 | 1629.8 | 2095.1 | 2155.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5469.8 | 5529.8 | 5644.9 | 5704.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3334.7 | 3414.7 | 4035.1 | 4115.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7234.7 | 7314.7 | 7584.9 | 7664.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 390.2 | 310.2 | 7409.8 | 7489.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6075.1 | 5975.1 | 5199.6 | 5099.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9524.9 | 9624.9 | 8999.6 | 9099.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2350.2 | 2250.2 | 1474.7 | 1374.7 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9349.8 | 9449.8 | 9174.7 | 9274.7 |

The 2nd harmonic by Band 3 fall into Band 42. But this impact already has covered in 2DL/1UL CA_3A-42A.

The 3th & 5th IMDs fall into the 4th own Rx frequency of Band 21. Also the 4th IMD fll into the own Rx Band 42. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21. Then, there is no IMD problems to the 3rd & 4th own Rx frequency of Band 21 and Band 42.

7.23.1.3	Co-existence studies for LTE-A UL CA_1A-21A and DL CA_1A-3A-21A-42A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.23.1.3-1 based on a certain operater’s actual spectrum holdings.

Table 7.23.1.3-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 1940 | 1960 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 3880 | 3920 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 5820 | 5880 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 477.1 | 512.1 | 3387.9 | 3422.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 935.8 | 985.8 | 2417.1 | 2472.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4835.8 | 4885.8 | 5327.9 | 5382.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2383.7 | 2448.7 | 4357.1 | 4432.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6283.7 | 6348.7 | 7267.9 | 7342.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1024.2 | 954.2 | 6775.8 | 6845.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6392.1 | 6297.1 | 3911.6 | 3831.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9207.9 | 9302.9 | 7731.6 | 7811.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2984.2 | 2894.2 | 508.7 | 423.7 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8715.8 | 8805.8 | 8223.7 | 8308.7 |

The 2nd IMD fall into the own Rx band 42. However, RAN4 can consider specicific holding frequency bands when study the harmonics/IMDs analysis for the related CA band combinations including Band 21. Then, there is no harmonics/IMD problems to the 3rd & 4th own Rx frequency of Band 3 and Band 42.

7.23.1.4	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-3A-21A-42A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.23.1.4-1 based on a certain operater’s actual spectrum holdings.

Table 7.23.1.4-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1940 | 1960 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3880 | 3920 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5820 | 5880 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1520 | 1580 | 5420 | 5480 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 360 | 440 | 5000 | 5100 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7360 | 7440 | 8900 | 9000 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2300 | 2400 | 8480 | 8620 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9300 | 9400 | 12380 | 12520 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3160 | 3040 | 10840 | 10960 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12140 | 11960 | 4360 | 4240 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15860 | 16040 | 11240 | 11360 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6680 | 6520 | 1080 | 1220 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14320 | 14480 | 12780 | 12920 |

The 2nd IMD fall into the own Rx Band 21. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21. Then, there is no IMD problems to the 3rd & 4th own Rx frequency of Band 3 and Band 21.

7.23.1.5	Co-existence studies for LTE-A UL CA_3A-21A and DL CA_1A-3A-21A-42A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.23.1.5-1 based on a certain operater’s actual spectrum holdings.

Table 7.23.1.5-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 1764.9 | 1784.9 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 3529.8 | 3569.8 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 5294.7 | 5354.7 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 302 | 337 | 3212.8 | 3247.8 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1110.9 | 1160.9 | 2066.9 | 2121.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4660.7 | 4710.7 | 4977.7 | 5032.7 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2558.8 | 2623.8 | 3831.8 | 3906.8 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6108.6 | 6173.6 | 6742.6 | 6817.6 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 674 | 604 | 6425.6 | 6495.6 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5691.7 | 5596.7 | 4086.7 | 4006.7 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8507.5 | 8602.5 | 7556.5 | 7636.5 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2458.9 | 2368.9 | 858.9 | 773.9 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8190.5 | 8280.5 | 7873.5 | 7958.5 |

The 2nd harmonic by Band 3 fall into Band 42. But this impact already has covered in 2DL/1UL CA_3A-42A.

The 3rd IMD fall into the own Rx frequency of Band 1. However RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21. Then, there is no IMD problems to the 3rd own Rx frequency of Band 1.

So, there was no harmonics/IMDs problem to the 3rd & 4th own Rx frequency of Band 1 and Band 42 when RAN4 consider specicific holding frequency bands.

7.23.1.6	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_1A-3A-21A-42A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.23.1.6-1 based on a certain operater’s actual spectrum holdings.

Table 7.23.1.6-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1764.9 | 1784.9 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3529.8 | 3569.8 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5294.7 | 5354.7 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1695.1 | 1755.1 | 5244.9 | 5304.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 9.8 | 89.8 | 5175.1 | 5275.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7009.8 | 7089.8 | 8724.9 | 8824.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1774.7 | 1874.7 | 8655.1 | 8795.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8774.7 | 8874.7 | 12204.9 | 12344.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3510.2 | 3390.2 | 10489.8 | 10609.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12315.1 | 12135.1 | 3659.6 | 3539.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15684.9 | 15864.9 | 10539.6 | 10659.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7030.2 | 6870.2 | 1605.3 | 1745.3 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13969.8 | 14129.8 | 12254.7 | 12394.7 |

From the harmonics/IMD analysis table, a number of harmonics/IMDs fall into the own Rx Band 3 and Band 42. But these issues already covered in 2DL/2UL CA_3A-42A.

The 5th IMD fall into the own Rx band 21. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21. Then, there is no harmonics/IMD problems to the 3rd & 4th own Rx frequency of Band 1 and Band 21.

7.23.1.7	Co-existence studies for LTE-A UL CA_21A-42A and DL CA_1A-3A-21A-42A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.23.1.7-1 based on a certain operater’s actual spectrum holdings.

Table 7.23.1.7-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2017.1 | 2072.1 | 4927.9 | 4982.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 624.2 | 554.2 | 5497.1 | 5592.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6375.8 | 6445.8 | 8407.9 | 8502.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 823.7 | 908.7 | 8977.1 | 9112.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7823.7 | 7908.7 | 11887.9 | 12022.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 4144.2 | 4034.2 | 9855.8 | 9965.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12632.1 | 12457.1 | 2371.6 | 2271.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15367.9 | 15542.9 | 9271.6 | 9371.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7664.2 | 7514.2 | 2571.3 | 2696.3 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13335.8 | 13485.8 | 11303.7 | 11428.7 |

The 2nd IMD fall into the own Rx Band 1. However, RAN4 can consider specicific holding frequency bands when study the harmonics/IMD analysis for the related CA band combinations including Band 21.

Then, there is no harmonics problems to the 3rd & 4th own Rx frequency of Band 1 and Band 3.

7.23.1.8	MSD

When uplink CA configurations CA_1A-3A is paired with downlink CA configuration CA_1A-3A-21A-42A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 21 and Band 42.

When uplink CA configurations CA_1A-21A is paired with downlink CA configuration CA_1A-3A-21A-42A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 3 and Band 42.

When uplink CA configurations CA_1A-42A is paired with downlink CA configuration CA_1A-3A-21A-42A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 3 and Band 21.

When uplink CA configurations CA_3A-21A is paired with downlink CA configuration CA_1A-3A-21A-42A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 1 and Band 42.

When uplink CA configurations CA_3A-42A is paired with downlink CA configuration CA_1A-3A-21A-42A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 1 and Band 21.

When uplink CA configurations CA_21A-42A is paired with downlink CA configuration CA_1A-3A-21A-42A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 1 and Band 3.

7.23.1.9	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7.24	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 28 and Band 42 with 2 ULs

7.24.1	List of specific combination issues

7.24.1.1	Channel bandwidths per operating band for CA

Table 7.24.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-3A-28A-42A | CA_1A-3A, CA_1A-28A, CA_1A-42A, CA_3A-28A, CA_3A-42A, CA_28A-42A | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |  |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |  |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |  |

7.24.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-28A-42A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.24.1.2-1.

Table 7.24.1.2-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 135 | 270 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1440 | 1650 | 2055 | 2250 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5340 | 5550 | 5550 | 5745 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3150 | 3435 | 3975 | 4230 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7050 | 7335 | 7470 | 7725 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 540 | 270 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6210 | 5895 | 5220 | 4860 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9390 | 9705 | 8760 | 9120 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2520 | 2190 | 1515 | 1170 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9180 | 9510 | 8970 | 9315 |

The 2nd harmonic by Band 3 fall into Band 42. But this impact already has covered in 2DL/1UL CA_3A-42A.

The 4th IMD fall into the 4th own Rx frequency of Band 42. However, this IMD problem already covered in 3DL CA_1A-3A-42A with 2UL CA_1A-3A. Hence, there is no IMD problems to the 3rd & 4th own Rx frequency of Band 28 and Band 42.

7.24.1.3	Co-existence studies for LTE-A UL CA_1A-28A and DL CA_1A-3A-28A-42A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.24.1.3-1.

Table 7.24.1.3-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1172 | 1277 | 2623 | 2728 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 574 | 424 | 3092 | 3257 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3326 | 3476 | 4543 | 4708 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 129 | 324 | 5012 | 5237 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4029 | 4224 | 6463 | 6688 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2554 | 2344 | 5246 | 5456 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7217 | 6932 | 1072 | 832 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8383 | 8668 | 4732 | 4972 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4534 | 4264 | 1596 | 1851 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7166 | 7436 | 5949 | 6204 |

The 5th IMD fall into the own Rx band 3. However, the IMD problems already covered in 3DL CA_1A-3A-28A with 2UL CA_1A-28A. And also the 3rd IMD fall into the own Rx Band 42. This IMD issues also covered in 3DL CA_1A-28A-42A with 2UL CA_1A-28A.

Hence, there were no harmonics/IMD problems to the 3rd & 4th own Rx frequency of Band 3 and Band 42.

7.24.1.4	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-3A-28A-42A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.24.1.4-1.

Table 7.24.1.4-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1420 | 1680 | 5320 | 5580 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 240 | 560 | 4820 | 5280 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7240 | 7560 | 8720 | 9180 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2160 | 2540 | 8220 | 8880 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9160 | 9540 | 12120 | 12780 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3360 | 2840 | 10640 | 11160 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12480 | 11620 | 4520 | 4080 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15520 | 16380 | 11080 | 11520 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6960 | 6240 | 860 | 1440 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14040 | 14760 | 12560 | 13140 |

From the harmonics/IMD analysis table, there were no harmonics/IMDs problems to the 3rd & 4th own Rx frequency of Band 3 and Band 28.

7.24.1.5	Co-existence studies for LTE-A UL CA_3A-28A and DL CA_1A-3A-28A-42A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.24.1.5-1.

Table 7.24.1.5-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 962 | 1082 | 2413 | 2533 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 379 | 214 | 2672 | 2867 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3116 | 3281 | 4123 | 4318 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 324 | 534 | 4382 | 4652 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3819 | 4029 | 5833 | 6103 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2164 | 1924 | 4826 | 5066 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6437 | 6092 | 1282 | 1027 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7543 | 7888 | 4522 | 4777 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3949 | 3634 | 1176 | 1461 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6536 | 6851 | 5529 | 5814 |

The 3rd harmonic by Band 28 fall into Band 1. But this impact already has covered in 2DL/1UL CA_1A-28A. And also the 2nd harmonics by Band 3 fall into Band 42. This harmonics impact already has covered in 2DL/1UL CA_3A-42A.

The 4th IMD fall into the own Rx frequency of Band 1. But this IMD problems already has covered in 3DL CA_1A-3A-28A with 2UL CA_3A-28A.

Hence, there were no harmonics/IMDs problems to the 3rd & 4th own Rx frequency of Band 1 and Band 42.

7.24.1.6	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_1A-3A-28A-42A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.24.1.6-1.

Table 7.24.1.6-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1615 | 1890 | 5110 | 5385 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 180 | 170 | 5015 | 5490 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6820 | 7170 | 8510 | 8985 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1530 | 1955 | 8415 | 9090 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8530 | 8955 | 11910 | 12585 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3780 | 3230 | 10220 | 10770 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12690 | 11815 | 3740 | 3240 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15310 | 16185 | 10240 | 10740 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7380 | 6630 | 1445 | 2070 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13620 | 14370 | 11930 | 12555 |

From the harmonics/IMD analysis table, a number of harmonics/IMDs fall into the own Rx Band 3 and Band 42. But these issues already covered in 2DL/2UL CA_3A-42A.

Hence, there were no harmonics/IMD problems to the 3rd & 4th own Rx frequency of Band 1 and Band 28.

7.24.1.7	Co-existence studies for LTE-A UL CA_28A-42A and DL CA_1A-3A-21A-42A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.24.1.7-1.

Table 7.24.1.7-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2652 | 2897 | 4103 | 4348 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2194 | 1904 | 6052 | 6497 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4806 | 5096 | 7503 | 7948 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1491 | 1156 | 9452 | 10097 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5509 | 5844 | 10903 | 11548 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 5794 | 5304 | 8206 | 8696 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13697 | 12852 | 408 | 788 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14303 | 15148 | 6212 | 6592 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9394 | 8704 | 4556 | 5091 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11606 | 12296 | 8909 | 9444 |

The 3rd harmonics by Band 28 fall into the own Rx Band 1. But this impact already covered in 2DL/1UL CA_1A-28A._Also 3rd IMD fall into the own Rx Band 1. However, the IMD problems already covered in 3DL CA_1A-28A-42A with 2UL CA_28A-42A.

Hence, there is no harmonics/IMDs problems to the 3rd & 4th own Rx frequency of Band 1 and Band 3.

7.24.1.8	MSD

When uplink CA configurations CA_1A-3A is paired with downlink CA configuration CA_1A-3A-28A-42A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 28 and Band 42.

When uplink CA configurations CA_1A-28A is paired with downlink CA configuration CA_1A-3A-28A-42A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 3 and Band 42.

When uplink CA configurations CA_1A-42A is paired with downlink CA configuration CA_1A-3A-28A-42A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 3 and Band 28.

When uplink CA configurations CA_3A-28A is paired with downlink CA configuration CA_1A-3A-28A-42A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 1 and Band 42.

When uplink CA configurations CA_3A-42A is paired with downlink CA configuration CA_1A-3A-28A-42A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 1 and Band 28.

When uplink CA configurations CA_28A-42A is paired with downlink CA configuration CA_1A-3A-28A-42A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 1 and Band 3.

7.24.1.9	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7.25	LTE Advanced Carrier Aggregation: Band 1 and Band 21 and Band 28 and Band 42 with 2 ULs

7.25.1	List of specific combination issues

7.25.1.1	Channel bandwidths per operating band for CA

Table 7.25.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-3A-21A-42A | CA_1A-21A, CA_1A-28A, CA_1A-42A, CA_21A-28A, CA_21A-42A, CA_28A-42A | 1 |  |  | Yes | Yes | Yes | Yes | 65 | 0 |  |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |  |

7.25.1.2	Co-existence studies for LTE-A UL CA_1A-21A and DL CA_1A-21A-28A-42A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.25.1.2-1 based on a certain operater’s actual spectrum holdings.

Table 7.25.1.2-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 1940 | 1960 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 3880 | 3920 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 5820 | 5880 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 477.1 | 512.1 | 3387.9 | 3422.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 935.8 | 985.8 | 2417.1 | 2472.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4835.8 | 4885.8 | 5327.9 | 5382.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2383.7 | 2448.7 | 4357.1 | 4432.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6283.7 | 6348.7 | 7267.9 | 7342.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1024.2 | 954.2 | 6775.8 | 6845.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6392.1 | 6297.1 | 3911.6 | 3831.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9207.9 | 9302.9 | 7731.6 | 7811.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2984.2 | 2894.2 | 508.7 | 423.7 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8715.8 | 8805.8 | 8223.7 | 8308.7 |

The 2nd IMD fall into the 4th own Rx frequency of Band 42. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21. Then, there is no IMD/harmonics problems to the 3rd & 4th own Rx frequency of Band 28 and Band 42.

7.25.1.3	Co-existence studies for LTE-A UL CA_1A-28A and DL CA_1A-21A-28A-42A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.25.1.3-1 based on a certain operater’s actual spectrum holdings.

Table 7.25.1.3-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 728 | 738 | 1940 | 1960 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1456 | 1476 | 3880 | 3920 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2184 | 2214 | 5820 | 5880 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1202 | 1232 | 2668 | 2698 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 504 | 464 | 3142 | 3192 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3396 | 3436 | 4608 | 4658 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 224 | 274 | 5082 | 5152 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4124 | 4174 | 6548 | 6618 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2464 | 2404 | 5336 | 5396 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7112 | 7022 | 1012 | 952 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8488 | 8578 | 4852 | 4912 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4424 | 4344 | 1666 | 1736 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7276 | 7356 | 6064 | 6134 |

The 2nd harmonics by Band 28 fall into the own Rx band 21. Also the 3rd IMD fall into the own Rx band 42. However, RAN4 can consider specicific holding frequency bands when study the harmonics/IMDs analysis for the related CA band combinations including Band 21. Also the IMD problem is already covered in 3DL CA_1A-28A-42A with 2UL CA_1A-28A.

Hence, there were no harmonics/IMD problems to the 3rd & 4th own Rx frequency of Band 21 and Band 42.

7.25.1.4	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-21A-28A-42A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.25.1.4-1 based on a certain operater’s actual spectrum holdings.

Table 7.25.1.4-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1940 | 1960 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3880 | 3920 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5820 | 5880 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1520 | 1580 | 5420 | 5480 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 360 | 440 | 5000 | 5100 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7360 | 7440 | 8900 | 9000 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2300 | 2400 | 8480 | 8620 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9300 | 9400 | 12380 | 12520 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3160 | 3040 | 10840 | 10960 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12140 | 11960 | 4360 | 4240 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15860 | 16040 | 11240 | 11360 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6680 | 6520 | 1080 | 1220 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14320 | 14480 | 12780 | 12920 |

The 2nd IMD fall into the own Rx Band 21. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21. Then, there is no IMD problems to the 3rd & 4th own Rx frequency of Band 21 and Band 28.

7.25.1.5	Co-existence studies for LTE-A UL CA_21A-28A and DL CA_1A-21A-28A-42A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.25.1.5-1 based on a certain operater’s actual spectrum holdings.

Table 7.25.1.5-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 728 | 738 | 1447.9 | 1462.9 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1456 | 1476 | 2895.8 | 2925.8 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2184 | 2214 | 4343.7 | 4388.7 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 709.9 | 734.9 | 2175.9 | 2200.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 6.9 | 28.1 | 2157.8 | 2197.8 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2903.9 | 2938.9 | 3623.8 | 3663.8 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 721.1 | 766.1 | 3605.7 | 3660.7 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3631.9 | 3676.9 | 5071.7 | 5126.7 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1469.8 | 1419.8 | 4351.8 | 4401.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5123.6 | 5053.6 | 1504.1 | 1449.1 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 6519.6 | 6589.6 | 4359.9 | 4414.9 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2932.7 | 2867.7 | 681.8 | 741.8 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5799.7 | 5864.7 | 5079.8 | 5139.8 |

The 3rd harmonic by Band 28 fall into Band 1. But this impact already has covered in 2DL/1UL CA_1A-28A.

The 2nd & 3rd IMDs fall into the own Rx frequency of Band 1. Also the 3rd & 4th IMD fall into the own Rx Band 42. However RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21. Then, there is no IMD problems to the 3rd & 4th own Rx frequency of Band 1 and Band 42.

So, there was no harmonics/IMDs problem to the 3rd & 4th own Rx frequency of Band 1 and Band 42 when RAN4 consider specicific holding frequency bands.

7.25.1.6	Co-existence studies for LTE-A UL CA_21A-42A and DL CA_1A-21A-28A-42A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.25.1.6-1 based on a certain operater’s actual spectrum holdings.

Table 7.25.1.6-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2017.1 | 2072.1 | 4927.9 | 4982.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 624.2 | 554.2 | 5497.1 | 5592.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6375.8 | 6445.8 | 8407.9 | 8502.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 823.7 | 908.7 | 8977.1 | 9112.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7823.7 | 7908.7 | 11887.9 | 12022.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 4144.2 | 4034.2 | 9855.8 | 9965.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12632.1 | 12457.1 | 2371.6 | 2271.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15367.9 | 15542.9 | 9271.6 | 9371.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7664.2 | 7514.2 | 2571.3 | 2696.3 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13335.8 | 13485.8 | 11303.7 | 11428.7 |

The 2nd IMD fall into the own Rx Band 1. Also the 4th IMD fall into the own Rx Band 28. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21. Hence, there were no harmonics/IMD problems to the 3rd & 4th own Rx frequency of Band 1 and Band 28.

7.25.1.7	Co-existence studies for LTE-A UL CA_28A-42A and DL CA_1A-21A-28A-42A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.25.1.7-1 based on a certain operater’s actual spectrum holdings.

Table 7.25.1.7-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 728 | 738 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1456 | 1476 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2184 | 2214 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2742 | 2792 | 4208 | 4258 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2064 | 2004 | 6222 | 6312 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4936 | 4996 | 7688 | 7778 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1336 | 1266 | 9702 | 9832 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5664 | 5734 | 11168 | 11298 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 5584 | 5484 | 8416 | 8516 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13352 | 13182 | 528 | 608 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14648 | 14818 | 6392 | 6472 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9104 | 8964 | 4746 | 4856 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11896 | 12036 | 9144 | 9254 |

The 2nd harmonics by Band 28 fall into the own Rx Band 21. Also the 3rd harmonics by Band 28 fall into the own Rx Band 1. These harmonics issues already covered in 2DL/1UL CA_1A-28A and 2DL/1UL CA_21A-28A

Also the 3rd IMD fall into the own Rx Band 1. This IMD issues also covered in 3DL CA_1A-28A-42A with 2UL CA_28A-42A.

Hence, there were no harmonics/IMDs problems to the 3rd & 4th own Rx frequency of Band 1 and Band 21.

7.25.1.8	MSD

When uplink CA configurations CA_1A-21A is paired with downlink CA configuration CA_1A-21A-28A-42A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 28 and Band 42.

When uplink CA configurations CA_1A-28A is paired with downlink CA configuration CA_1A-21A-28A-42A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 21 and Band 42.

When uplink CA configurations CA_1A-42A is paired with downlink CA configuration CA_1A-21A-28A-42A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 21 and Band 28.

When uplink CA configurations CA_21A-28A is paired with downlink CA configuration CA_1A-21A-28A-42A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 1 and Band 42.

When uplink CA configurations CA_21A-42A is paired with downlink CA configuration CA_1A-21A-28A-42A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 1 and Band 28.

When uplink CA configurations CA_28A-42A is paired with downlink CA configuration CA_1A-21A-28A-42A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 1 and Band 21.

7.25.1.9	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7.26	LTE Advanced Carrier Aggregation: Band 1 and Band 28 and Band 42 and Band 42 with 2 ULs

7.26.1	List of specific combination issues

7.26.1.1	Channel bandwidths per operating band for CA

Table 7.26.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-28A-42C | CA_1A-28A or CA_1A-42A or CA_28A-42A | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 7.26.1.1-2 |  |  |  |  |  |  |  |

Table 7.26.1.1-2: E-UTRA CA_42C configurations and bandwidth combination sets for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_42C | 5, 10, 15, 20 | 20 | 40 | 0 |
|  | 20 | 5, 10, 15 |  |  |
|  | 10, 15, 20 | 20 | 40 | 1 |
|  | 20 | 10, 15 |  |  |

7.26.1.2	Co-existence studies for LTE-A UL CA_1A-28A and DL CA_1A-28A-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.26.1.2-1.

Table 7.26.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 728 | 738 | 1940 | 1960 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1456 | 1476 | 3880 | 3920 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2184 | 2214 | 5820 | 5880 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1202 | 1232 | 2668 | 2698 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 504 | 464 | 3142 | 3192 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3396 | 3436 | 4608 | 4658 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 224 | 274 | 5082 | 5152 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4124 | 4174 | 6548 | 6618 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2464 | 2404 | 5336 | 5396 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7112 | 7022 | 1012 | 952 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8488 | 8578 | 4852 | 4912 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4424 | 4344 | 1666 | 1736 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7276 | 7356 | 6064 | 6134 |

The 3rd IMD fall into the 3rd own Rx frequency of Band 42. The IMD issue is covered in 3DL CA_1A-28A-42A with 2UL CA_1A-28A. Hence, there were no harmonics/IMDs problems to the 3rd & 4th own Rx frequency of Band 42 and Band 42.

7.26.1.3	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-28A-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.26.1.3-1.

Table 7.26.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1420 | 1680 | 5320 | 5580 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 240 | 560 | 4820 | 5280 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7240 | 7560 | 8720 | 9180 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2160 | 2540 | 8220 | 8880 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9160 | 9540 | 12120 | 12780 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3360 | 2840 | 10640 | 11160 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12480 | 11620 | 4520 | 4080 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15520 | 16380 | 11080 | 11520 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6960 | 6240 | 860 | 1440 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14040 | 14760 | 12560 | 13140 |

The 4th IMD fall into the own Rx Band 1. But this impact already has covered in 2DL/2UL CA_1A-42A.

There is no harmonics/IMD problems to the 3rd and 4th own Rx frequency of Band 28 and Band 42.

7.26.1.4	Co-existence studies for LTE-A UL CA_28A-42A and DL CA_1A-28A-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.26.1.4-1.

Table 7.26.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2652 | 2897 | 4103 | 4348 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2194 | 1904 | 6052 | 6497 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4806 | 5096 | 7503 | 7948 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1491 | 1156 | 9452 | 10097 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5509 | 5844 | 10903 | 11548 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 5794 | 5304 | 8206 | 8696 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13697 | 12852 | 408 | 788 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14303 | 15148 | 6212 | 6592 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9394 | 8704 | 4556 | 5091 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11606 | 12296 | 8909 | 9444 |

The 3rd harmonics by Band 28 fall into the own Rx Band 1. But the harmonics problem was already covered in 2DL/1UL CA_1A-28A. Also the 3rd IMD fall into the own Rx Band 1. The IMD issues also covered in 3DL CA_1A-28A-42A with 2UL CA_28A-42A.

Hence, there were no harmonics/IMD problems to the own 3rd & 4th Rx Band 1 and Band 42.

7.26.1.5	MSD

When uplink CA configurations CA_1A-28A is paired with downlink CA configuration CA_1A-28A-42C there is no interference components from 2 uplink operation which would interfere the downlink of the Band 42 and Band 42.

When uplink CA configurations CA_1A-42A is paired with downlink CA configuration CA_1A-28A-42C there is no interference components from 2 uplink operation which would interfere the downlink of the Band 28 and Band 42.

When uplink CA configurations CA_28A-42A is paired with downlink CA configuration CA_1A-28A-42C there is no interference components from 2 uplink operation which would interfere the downlink of the Band 1 and Band 42.

7.26.1.6	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7.27	LTE Advanced Carrier Aggregation: Band 3 and Band 21 and Band 42 and Band 42 with 2 ULs

7.27.1	List of specific combination issues

7.27.1.1	Channel bandwidths per operating band for CA

Table 7.27.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A-21A-42C | CA_3A-21A or CA_3A-42A or CA_21A-42A | 3 |  |  | Yes | Yes | Yes | Yes | 75 | 0 |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 7.27.1.1-2 |  |  |  |  |  |  |  |

Table 7.27.1.1-2: E-UTRA CA_42C configurations and bandwidth combination sets for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_42C | 5, 10, 15, 20 | 20 | 40 | 0 |
|  | 20 | 5, 10, 15 |  |  |
|  | 10, 15, 20 | 20 | 40 | 1 |
|  | 20 | 10, 15 |  |  |

7.27.1.2	Co-existence studies for LTE-A UL CA_3A-21A and DL CA_3A-21A-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.27.1.2-1 based on a certain operater’s actual spectrum holdings.

Table 7.27.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 1764.9 | 1784.9 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 3529.8 | 3569.8 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 5294.7 | 5354.7 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 302 | 337 | 3212.8 | 3247.8 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1110.9 | 1160.9 | 2066.9 | 2121.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4660.7 | 4710.7 | 4977.7 | 5032.7 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2558.8 | 2623.8 | 3831.8 | 3906.8 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6108.6 | 6173.6 | 6742.6 | 6817.6 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 674 | 604 | 6425.6 | 6495.6 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5691.7 | 5596.7 | 4086.7 | 4006.7 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8507.5 | 8602.5 | 7556.5 | 7636.5 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2458.9 | 2368.9 | 858.9 | 773.9 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8190.5 | 8280.5 | 7873.5 | 7958.5 |

The 2nd harmonics from Band 3 transmission will be interference to the Band 42. But this harmonics issue already has covered in 2DL/1UL CA_3A-42A.

Hence, there was no harmonics/IMDs impact to the 3rd & 4th own Rx frequency of Band 42 and Band 42.

7.27.1.3	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_3A-21A-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.27.1.3-1 based on a certain operater’s actual spectrum holdings.

Table 7.27.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1764.9 | 1784.9 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3529.8 | 3569.8 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5294.7 | 5354.7 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1695.1 | 1755.1 | 5244.9 | 5304.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 9.8 | 89.8 | 5175.1 | 5275.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7009.8 | 7089.8 | 8724.9 | 8824.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1774.7 | 1874.7 | 8655.1 | 8795.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8774.7 | 8874.7 | 12204.9 | 12344.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3510.2 | 3390.2 | 10489.8 | 10609.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12315.1 | 12135.1 | 3659.6 | 3539.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15684.9 | 15864.9 | 10539.6 | 10659.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7030.2 | 6870.2 | 1605.3 | 1745.3 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13969.8 | 14129.8 | 12254.7 | 12394.7 |

The 2nd, 4th & 5th IMDs fall into the own Rx frequency of Band 3. Also the 2nd harmonics from Band 3, 4th & 5th IMDs fall into the own Rx frequency of Band 42. But these impact already has covered in 2DL/2UL CA_3A-42A.

The 5th IMD fall into the 3rd own Rx frequency of Band 21. However, RAN4 consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21.

Hence, there was no harmonics/IMDs problem to the 3rd & 4th own Rx frequency of Band 21 and Band 42.

7.27.1.4	Co-existence studies for LTE-A UL CA_21A-42A and DL CA_3A-21A-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.27.1.4-1 based on a certain operater’s actual spectrum holdings.

Table 7.27.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2017.1 | 2072.1 | 4927.9 | 4982.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 624.2 | 554.2 | 5497.1 | 5592.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6375.8 | 6445.8 | 8407.9 | 8502.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 823.7 | 908.7 | 8977.1 | 9112.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7823.7 | 7908.7 | 11887.9 | 12022.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 4144.2 | 4034.2 | 9855.8 | 9965.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12632.1 | 12457.1 | 2371.6 | 2271.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15367.9 | 15542.9 | 9271.6 | 9371.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7664.2 | 7514.2 | 2571.3 | 2696.3 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13335.8 | 13485.8 | 11303.7 | 11428.7 |

From the harmonics/IMD analysis table, there was no harmonics/IMDs impact to the 3rd & 4th own Rx frequency of Band 3 and Band 42.

7.27.1.5	MSD

When uplink CA configurations CA_3A-21A is paired with downlink CA configuration CA_3A-21A-42C there is no interference components from 2 uplink operation which would interfere the downlink of the Band 42 and Band 42.

When uplink CA configurations CA_3A-42A is paired with downlink CA configuration CA_3A-21A-42C there is no interference components from 2 uplink operation which would interfere the downlink of the Band 21 and Band 42.

When uplink CA configurations CA_21A-42A is paired with downlink CA configuration CA_3A-21A-42C there is no interference components from 2 uplink operation which would interfere the downlink of the Band 3 and Band 42.

7.27.1.6	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7.28	LTE Advanced Carrier Aggregation: Band 3 and Band 28 and Band 42 and Band 42 with 2 ULs

7.28.1	List of specific combination issues

7.28.1.1	Channel bandwidths per operating band for CA

Table 7.28.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A-28A-42C | CA_3A-28A or CA_3A-42A or CA_28A-42A or CA_42C | 3 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 7.28.1.1-2 |  |  |  |  |  |  |  |

Table 7.28.1.1-2: E-UTRA CA_42C configurations and bandwidth combination sets for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_42C | 5, 10, 15, 20 | 20 | 40 | 0 |
|  | 20 | 5, 10, 15 |  |  |
|  | 10, 15, 20 | 20 | 40 | 1 |
|  | 20 | 10, 15 |  |  |

7.28.1.2	Co-existence studies for LTE-A UL CA_3A-28A and DL CA_3A-28A-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.28.1.2-1.

Table 7.28.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 962 | 1082 | 2413 | 2533 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 379 | 214 | 2672 | 2867 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3116 | 3281 | 4123 | 4318 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 324 | 534 | 4382 | 4652 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3819 | 4029 | 5833 | 6103 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2164 | 1924 | 4826 | 5066 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6437 | 6092 | 1282 | 1027 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7543 | 7888 | 4522 | 4777 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3949 | 3634 | 1176 | 1461 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6536 | 6851 | 5529 | 5814 |

The 2nd harmonic by Band 3 fall into the own Rx Band 42. But this impact already has covered in 2DL/1UL CA_3A-42A.

There was no harmonics/IMD problems to the 3rd & 4th own Rx frequency of Band 42.

7.28.1.3	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_3A-28A-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.28.1.3-1.

Table 7.28.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1615 | 1890 | 5110 | 5385 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 180 | 170 | 5015 | 5490 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6820 | 7170 | 8510 | 8985 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1530 | 1955 | 8415 | 9090 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8530 | 8955 | 11910 | 12585 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3780 | 3230 | 10220 | 10770 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12690 | 11815 | 3740 | 3240 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15310 | 16185 | 10240 | 10740 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7380 | 6630 | 1445 | 2070 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13620 | 14370 | 11930 | 12555 |

The 2nd harmonics by Band 3 fall into the own Rx Band 42. Also the 4th & 5th IMDs fall into the own Rx band 42. Further more 2nd, 4th and 5th IMDs fall into the own Rx Band 3. But these harmonics/IMD problems already covered in 2DL/2UL CA_3A-42A.

Hence, there is no harmonics/IMD problems to the 3rd & 4th own Rx frequency of Band 28 and Band 42.

7.28.1.4	Co-existence studies for LTE-A UL CA_28A-42A and DL CA_3A-28A-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.28.1.4-1.

Table 7.28.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2652 | 2897 | 4103 | 4348 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2194 | 1904 | 6052 | 6497 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4806 | 5096 | 7503 | 7948 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1491 | 1156 | 9452 | 10097 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5509 | 5844 | 10903 | 11548 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 5794 | 5304 | 8206 | 8696 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13697 | 12852 | 408 | 788 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14303 | 15148 | 6212 | 6592 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9394 | 8704 | 4556 | 5091 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11606 | 12296 | 8909 | 9444 |

From the harmonics/IMD analysis table, there is no harmonics/IMDs problems to the 3rd & 4th own Rx frequency of Band 3 and Band 42.

7.28.1.5	Co-existence studies for LTE-A UL CA_42C and DL CA_3A-28A-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.28.1.5-1

Table 7.28.1.5-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high |
| --- | --- | --- |
| UL frequency (MHz) | 3400 | 3600 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 6800 | 7200 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 10200 | 10800 |

From the harmonics analysis table, there was no harmonics from Band 42 uplinks which would desensitize the 3rd & 4th own Rx frequency of Band 3 and Band 28. The worst frequency separation of band 3 and 42 is 1520 MHz therefore there is no impact from Band 42 uplink CA to Band 3 reception evenif RAN4 consider asynchronization scenario.

7.28.1.6	MSD

When uplink CA configurations CA_3A-28A is paired with downlink CA configuration CA_3A-28A-42C there is no interference components from 2 uplink operation which would interfere the downlink of the Band 42 and Band 42.

When uplink CA configurations CA_3A-42A is paired with downlink CA configuration CA_3A-28A-42C there is no interference components from 2 uplink operation which would interfere the downlink of the Band 28 and Band 42.

When uplink CA configurations CA_28A-42A is paired with downlink CA configuration CA_3A-28A-42C there is no interference components from 2 uplink operation which would interfere the downlink of the Band 3 and Band 42.

When uplink CA configurations CA_42C is paired with downlink CA configuration CA_3A-28A-42C there is no interference components from 2 uplink operation which would interfere the downlink of the Band 3 and Band 28.

7.28.1.7	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7.29	LTE Advanced Carrier Aggregation: Band 21 and Band 28 and Band 42 and Band 42 with 2 ULs

7.29.1	List of specific combination issues

7.29.1.1	Channel bandwidths per operating band for CA

Table 7.29.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_21A-28A-42C | CA_21A-28A or CA_21A-42A or CA_28A-42A | 21 |  |  | Yes | Yes | Yes |  | 65 | 0 |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 7.29.1.1-2 |  |  |  |  |  |  |  |

Table 7.29.1.1-2: E-UTRA CA_42C configurations and bandwidth combination sets for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_42C | 5, 10, 15, 20 | 20 | 40 | 0 |
|  | 20 | 5, 10, 15 |  |  |
|  | 10, 15, 20 | 20 | 40 | 1 |
|  | 20 | 10, 15 |  |  |

7.29.1.2	Co-existence studies for LTE-A UL CA_21A-28A and DL CA_21A-28A-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.29.1.2-1 based on a certain operater’s actual spectrum holdings.

Table 7.29.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 728 | 738 | 1447.9 | 1462.9 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1456 | 1476 | 2895.8 | 2925.8 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2184 | 2214 | 4343.7 | 4388.7 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 709.9 | 734.9 | 2175.9 | 2200.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 6.9 | 28.1 | 2157.8 | 2197.8 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2903.9 | 2938.9 | 3623.8 | 3663.8 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 721.1 | 766.1 | 3605.7 | 3660.7 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3631.9 | 3676.9 | 5071.7 | 5126.7 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1469.8 | 1419.8 | 4351.8 | 4401.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5123.6 | 5053.6 | 1504.1 | 1449.1 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 6519.6 | 6589.6 | 4359.9 | 4414.9 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2932.7 | 2867.7 | 681.8 | 741.8 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5799.7 | 5864.7 | 5079.8 | 5139.8 |

The 3rd & 4th IMDs fall into the 3rd own Rx frequency of Band 42. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21. Then, there is no IMD problems to the own Rx frequency of Band 42.

Hence, there was no harmonics/IMD problems to the own 3rd & 4th Rx Band 42 and Band 42.

7.29.1.3	Co-existence studies for LTE-A UL CA_21A-42A and DL CA_21A-28A-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.29.1.3-1 based on a certain operater’s actual spectrum holdings.

Table 7.29.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2017.1 | 2072.1 | 4927.9 | 4982.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 624.2 | 554.2 | 5497.1 | 5592.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6375.8 | 6445.8 | 8407.9 | 8502.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 823.7 | 908.7 | 8977.1 | 9112.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7823.7 | 7908.7 | 11887.9 | 12022.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 4144.2 | 4034.2 | 9855.8 | 9965.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12632.1 | 12457.1 | 2371.6 | 2271.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15367.9 | 15542.9 | 9271.6 | 9371.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7664.2 | 7514.2 | 2571.3 | 2696.3 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13335.8 | 13485.8 | 11303.7 | 11428.7 |

The 4th IMD fall into the own Rx Band 28. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21. Then, there is no IMD problems to the own Rx frequency of Band 28.

Hence, there was no harmonics/IMD problems to the own 3rd & 4th Rx Band 28 and Band 42.

7.29.1.4	Co-existence studies for LTE-A UL CA_28A-42A and DL CA_21A-28A-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.29.1.4-1 based on a certain operater’s actual spectrum holdings.

Table 7.29.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 728 | 738 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1456 | 1476 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2184 | 2214 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2742 | 2792 | 4208 | 4258 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2064 | 2004 | 6222 | 6312 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4936 | 4996 | 7688 | 7778 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1336 | 1266 | 9702 | 9832 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5664 | 5734 | 11168 | 11298 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 5584 | 5484 | 8416 | 8516 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13352 | 13182 | 528 | 608 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14648 | 14818 | 6392 | 6472 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9104 | 8964 | 4746 | 4856 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11896 | 12036 | 9144 | 9254 |

The 2nd harmonic by Band 28 fall into the own Rx Band 21. But this impact already has covered in 2DL/1UL CA_21A-28A.

There is no IMD problems to the 3rd own Rx frequency of Band 21, when consider specicific holding frequency bands including Band 21.

Hence, there was no harmonics/IMD problems to the own 3rd & 4th Rx Band 21 and Band 42.

7.29.1.5	MSD

When uplink CA configurations CA_21A-28A is paired with downlink CA configuration CA_21A-28A-42C there is no interference components from 2 uplink operation which would interfere the downlink of the Band 42 and Band 42.

When uplink CA configurations CA_21A-42A is paired with downlink CA configuration CA_21A-28A-42C there is no interference components from 2 uplink operation which would interfere the downlink of the Band 28 and Band 42.

When uplink CA configurations CA_28A-42A is paired with downlink CA configuration CA_21A-28A-42C there is no interference components from 2 uplink operation which would interfere the downlink of the Band 21 and Band 42.

7.29.1.6	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7.30	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 7 and Band 7 with 2 ULs

7.30.1	List of specific combination issues

7.30.1.1	Channel bandwidths per operating band for CA

Table 7.30.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-7A-8A | CA_1A-3A, CA_1A-7A, CA_3A-7A | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 3 in Table 7.30.1.1-2 |  |  |  |  |  |  |  |

Table 7.30.1.1-2: E-UTRA CA_7A-7A configurations and bandwidth combination sets for intra-band CA

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRACA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_7A-7A | - | 5 | 15 |  |  | 40 | 0 |
|  |  | 10 | 10, 15 |  |  |  |  |
|  |  | 15 | 15, 20 |  |  |  |  |
|  |  | 20 | 20 |  |  |  |  |
|  |  | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 40 | 1 |
|  |  | 5, 10, 15, 20 | 5, 10 |  |  | 30 | 2 |
|  |  | 10, 15, 20 | 10, 15, 20 |  |  | 40 | 3 |

7.30.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-7A-7A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.30.1.2-1

Table 7.30.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 270 | 135 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2055 | 2250 | 1440 | 1650 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5550 | 5745 | 5340 | 5550 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3975 | 4230 | 3150 | 3435 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7470 | 7725 | 7050 | 7335 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 270 | 540 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5220 | 4860 | 6210 | 5895 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8760 | 9120 | 9390 | 9705 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1515 | 1170 | 2520 | 2190 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 | 9315 | 9180 | 9510 |

There were no harmonic/IMD problems from band 1 and band 3 uplinks which would desensitize the own Rx Band 7 and Band 7.

7.30.1.3	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-3A-7A-7A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.30.1.3-1

Table 7.30.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 520 | 650 | 4420 | 4550 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1270 | 1460 | 3020 | 3220 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6340 | 6530 | 6920 | 7120 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3190 | 3440 | 5520 | 5790 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8260 | 8510 | 9420 | 9690 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1300 | 1040 | 8840 | 9100 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8360 | 8020 | 5420 | 5110 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11920 | 12260 | 10180 | 10490 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3870 | 3540 | 940 | 620 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11340 | 11670 | 10760 | 11080 |

There were no harmonic/IMD problems from band 1 and band 7 uplinks which would desensitize the own Rx Band 3 and Band 7.

7.30.1.4	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_1A-3A-7A-7A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.30.1.4-1

Table 7.30.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 715 | 860 | 4210 | 4355 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 850 | 1070 | 3215 | 3430 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5920 | 6140 | 6710 | 6925 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2560 | 2855 | 5715 | 6000 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7630 | 7925 | 9210 | 9495 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1720 | 1430 | 8420 | 8710 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8570 | 8215 | 4640 | 4270 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11710 | 12065 | 9340 | 9710 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4290 | 3930 | 355 | 10 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10920 | 11280 | 10130 | 10495 |

The 4th IMD fall into the own Rx Band 7. But the IMD problems already covered in 2DL/2UL CA_3A-7A. Hence, there are no harmonic/IMD problems from band 3 and band 7 uplinks which would desensitize the own Rx Band 1 and Band 7.

7.30.1.5	MSD

When uplink CA configurations CA_1A-3A is paired with downlink CA configuration CA_1A-3A-7A-7A there are no interference components from 2 uplink operation which would interfere the downlink of the Band 7 and Band 7.

When uplink CA configurations CA_1A-7A is paired with downlink CA configuration CA_1A-3A-7A-7A there are no interference components from 2 uplink operation which would interfere the downlink of the Band 3 and Band 7.

When uplink CA configurations CA_3A-7A is paired with downlink CA configuration CA_1A-3A-7A-7A there are no interference components from 2 uplink operation which would interfere the downlink of the Band 7 and Band 8.

7.30.1.6	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7.31	LTE Advanced Carrier Aggregation: Band 1 and Band 5 and Band 7 and Band 7 with 2 ULs

7.31.1	List of specific combination issues

7.31.1.1	Channel bandwidths per operating band for CA

Table 7.31.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-5A-7A-7A | CA_1A-5A or CA_1A-7A or CA_5A-7A | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 3 in Table 7.31.1.1-2 |  |  |  |  |  |  |  |

Table 7.31.1.1-2: E-UTRA CA_7A-7A configurations and bandwidth combination sets for intra-band CA

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRACA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_7A-7A | - | 5 | 15 |  |  | 40 | 0 |
|  |  | 10 | 10, 15 |  |  |  |  |
|  |  | 15 | 15, 20 |  |  |  |  |
|  |  | 20 | 20 |  |  |  |  |
|  |  | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 40 | 1 |
|  |  | 5, 10, 15, 20 | 5, 10 |  |  | 30 | 2 |
|  |  | 10, 15, 20 | 10, 15, 20 |  |  | 40 | 3 |

7.31.1.2	Co-existence studies for LTE-A UL CA_1A-5A and DL CA_1A-5A-7A-7A

For 2UL / 4DL own receiver desensitization study up to 3th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.31.1.2-1

Table 7.31.1.2-1: Harmonic/IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1071 | 1156 | 2744 | 2829 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 332 | 222 | 2991 | 3136 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3568 | 3678 | 4664 | 4809 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 492 | 627 | 4911 | 5116 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4392 | 4527 | 6584 | 6789 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2312 | 2142 | 5488 | 5658 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7096 | 6831 | 1476 | 1316 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8504 | 8769 | 5216 | 5376 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4292 | 4062 | 1293 | 1488 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7408 | 7638 | 6312 | 6507 |

The 4th IMD fall into the own Rx frequency of Band 1. But this impact has covered in 2DL/2UL CA_1A-5A.

Hence, there was no harmonics/IMD problems to the own 3rd and 4th Rx Band 7 and Band 7.

7.31.1.3	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-5A-7A-7A

For 2UL / 4DL own receiver desensitization study up to 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.31.1.3-1

Table 7.31.1.3-1: Harmonic/IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 520 | 650 | 4420 | 4550 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1270 | 1460 | 3020 | 3220 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6340 | 6530 | 6920 | 7120 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3190 | 3440 | 5520 | 5790 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8260 | 8510 | 9420 | 9690 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1300 | 1040 | 8840 | 9100 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8360 | 8020 | 5420 | 5110 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11920 | 12260 | 10180 | 10490 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3870 | 3540 | 940 | 620 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11340 | 11670 | 10760 | 11080 |

The 5th IMD fall into the own Rx frequency of Band 5. But the IMD problems was covered in 3DL CA_1A-5A-7A with 2UL CA_1A-7A.

Hence, there was no harmonics.IMD problems to the own 3rd and 4th Rx Band 5 and Band 7.

7.31.1.4	Co-existence studies for LTE-A UL CA_5A-7A and DL CA_1A-5A-7A-7A

For 2UL / 4DL own receiver desensitization study up to 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.31.1.4-1.

Table 7.31.1.4-1: Harmonic/IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 839 | 849 | 2500 | 2540 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1678 | 1698 | 5000 | 5080 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2517 | 2547 | 7500 | 7620 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1651 | 1701 | 3339 | 3389 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 862 | 802 | 4151 | 4241 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4178 | 4238 | 5839 | 5929 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 23 | 47 | 6651 | 6781 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5017 | 5087 | 8339 | 8469 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3402 | 3302 | 6678 | 6778 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9321 | 9151 | 896 | 816 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10839 | 11009 | 5856 | 5936 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5942 | 5802 | 2453 | 2563 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9178 | 9318 | 7517 | 7627 |

The 5th IMD fall into the own Rx frequency of Band 7. Also the 3rd and 5th IMD fall into the own Rx frequency of Band 5. But these impacts were already covered in 2DL/2UL CA_5A-7A.

Hence, there was no harmonics/IMDs problems to the own 3rd and 4th Rx Band 1 and Band 7.

7.31.1.5	MSD

When uplink CA configurations CA_1A-5A is paired with downlink CA configuration CA_1A-5A-7A-7A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 7 and Band 7.

When uplink CA configurations CA_1A-7A is paired with downlink CA configuration CA_1A-5A-7A-7A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 5 and Band 7.

When uplink CA configurations CA_5A-7A is paired with downlink CA configuration CA_1A-5A-7A-7A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 1 and Band 7.

7.31.1.6	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7.32	LTE Advanced Carrier Aggregation: Band 3 and Band 5 and Band 7 and Band 7 with 2 ULs

7.32.1	List of specific combination issues

7.32.1.1	Channel bandwidths per operating band for CA

Table 7.32.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A-5A-7A-7A | CA_3A-5A or CA_3A-7A or CA_5A-7A | 3 |  |  |  | Yes | Yes | Yes | 70 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 3 in Table 7.32.1.1-2 |  |  |  |  |  |  |  |

Table 7.32.1.1-2: E-UTRA CA_7A-7A configurations and bandwidth combination sets for intra-band CA

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRACA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_7A-7A | - | 5 | 15 |  |  | 40 | 0 |
|  |  | 10 | 10, 15 |  |  |  |  |
|  |  | 15 | 15, 20 |  |  |  |  |
|  |  | 20 | 20 |  |  |  |  |
|  |  | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 40 | 1 |
|  |  | 5, 10, 15, 20 | 5, 10 |  |  | 30 | 2 |
|  |  | 10, 15, 20 | 10, 15, 20 |  |  | 40 | 3 |

7.32.1.2	Co-existence studies for LTE-A UL CA_3A-5A and DL CA_3A-5A-7A-7A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.32.1.2-1

Table 7.32.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 861 | 961 | 2534 | 2634 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 137 | 12 | 2571 | 2746 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3358 | 3483 | 4244 | 4419 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 687 | 837 | 4281 | 4531 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4182 | 4332 | 5954 | 6204 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1922 | 1722 | 5068 | 5268 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6316 | 5991 | 1686 | 1511 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7664 | 7989 | 5006 | 5181 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3707 | 3432 | 873 | 1098 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6778 | 7053 | 5892 | 6117 |

The 2nd & 3rd IMDs fall into the own Rx Band 7. But these impacts already has covered in 3DL CA_3A-5A-7A with 2UL CA_3A-5A.

Hence, there were no harmonics/IMDs problems to the own Rx Band 7 and Band 7.

7.32.1.3	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_3A-5A-7A-7A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.32.1.3-1

Table 7.32.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 715 | 860 | 4210 | 4355 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 850 | 1070 | 3215 | 3430 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5920 | 6140 | 6710 | 6925 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2560 | 2855 | 5715 | 6000 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7630 | 7925 | 9210 | 9495 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1720 | 1430 | 8420 | 8710 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8570 | 8215 | 4640 | 4270 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11710 | 12065 | 9340 | 9710 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4290 | 3930 | 355 | 10 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10920 | 11280 | 10130 | 10495 |

The 4th IMD fall into the own Rx Band 7. But this impact already has covered in 2DL/2UL CA_3A-7A.

The 3rd IMD fall into the own Rx frequency of Band 5. It was covered in 3DL CA_3A-5A-7A with 2UL CA_3A-7A.

Hence, there were no harmonics/IMDs problems to the own Rx Band 5 and Band 7.

7.32.1.4	Co-existence studies for LTE-A UL CA_5A-7A and DL CA_3A-5A-7A-7A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.32.1.4-1

Table 7.32.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1651 | 1746 | 3324 | 3419 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 922 | 802 | 4151 | 4316 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4148 | 4268 | 5824 | 5989 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 98 | 47 | 6651 | 6886 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4972 | 5117 | 8324 | 8559 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3492 | 3302 | 6648 | 6838 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9456 | 9151 | 896 | 726 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10824 | 11129 | 5796 | 5966 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6062 | 5802 | 2453 | 2668 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9148 | 9408 | 7472 | 7687 |

The 3rd & 5th IMDs fall into own Rx frequency of Band 5. Also the 5th IMD fall into the own Rx frequency of Band 7. But these impacts already has covered in 2DL/2UL CA_5A-7A.

The harmonics/ IMD do not impact to the 3rd & 4th own Rx frequency of Band 3 and Band 7.

7.32.1.5	MSD

When uplink CA configurations CA_3A-5A is paired with downlink CA configuration CA_3A-5A-7A-7A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 7 and Band 7.

When uplink CA configurations CA_3A-7A is paired with downlink CA configuration CA_3A-5A-7A-7A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 5 and Band 7.

When uplink CA configurations CA_5A-7A is paired with downlink CA configuration CA_3A-5A-7A-7A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 3 and Band 7.

7.32.1.6	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7.33	LTE Advanced Carrier Aggregation: Band 3 and Band 3 and Band 7 and Band 7 with 2 ULs

7.33.1	List of specific combination issues

7.33.1.1	Channel bandwidths per operating band for CA

Table 7.33.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3C-7C | CA_3A-7A | 3 | See CA_3C Bandwidth Combination Set 0 in Table 7.33.1.1-2 |  |  |  |  |  | 80 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 7.33.1.1-3 |  |  |  |  |  |  |  |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 7.33.1.1-2 |  |  |  |  |  | 80 | 1 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 7.33.1.1-3 |  |  |  |  |  |  |  |

Table 7.33.1.1-2: E-UTRA CA_3C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_3C | 5, 10, 15 | 20 | 40 | 0 |
|  | 20 | 5, 10, 15, 20 |  |  |

Table 7.33.1.1-3: E-UTRA CA_7C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_7C | CA_7C | 15 | 15 | 40 | 0 |
|  |  | 20 | 20 |  |  |
|  |  | 10 | 20 | 40 | 1 |
|  |  | 15 | 15, 20 |  |  |
|  |  | 20 | 10, 15, 20 |  |  |
|  |  | 15 | 10, 15 | 40 | 2 |
|  |  | 20 | 15, 20 |  |  |

7.33.1.2	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_3C-7C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.33.1.2-1

Table 7.33.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 715 | 860 | 4210 | 4355 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 850 | 1070 | 3215 | 3430 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5920 | 6140 | 6710 | 6925 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2560 | 2855 | 5715 | 6000 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7630 | 7925 | 9210 | 9495 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1720 | 1430 | 8420 | 8710 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8570 | 8215 | 4640 | 4270 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11710 | 12065 | 9340 | 9710 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4290 | 3930 | 355 | 10 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10920 | 11280 | 10130 | 10495 |

The 4th IMD fall into the own Rx frequency of Band 7. But this impact was already covered in 2DL/2UL in Rel-12.  Impacts for ISM and GNSS bands were also already captured in Rel-12 TR 36.860-12.

There are no harmonics or IMD products from Band 3 and Band 7 uplinks which would desensitize Band 3 downlink.

7.33.1.3	MSD

When uplink CA configurations CA_3A-7A is paired with downlink CA configuration CA_3C-7C there is no interference components from 2 uplink operation which would interfere downlink of the Band 3 and there is interference component from 2 uplink operation which would interfere downlink of the Band 7, but this impact has been covered in 2DL/2UL CA_3A-7A in36.101 Table 7.3.1A-0f, no further MSD studies for this combination.

7.33.1.4	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7.34	LTE Advanced Carrier Aggregation: Band 3 and Band 41 and Band 42 and Band 42 with 2 ULs

7.34.1	List of specific combination issues

7.34.1.1	Channel bandwidths per operating band for CA

Table 7.34.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A_41A-42C | CA_41A-42A, CA_42C | 3 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 41 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 0 in Table 7.34.1.1-2 |  |  |  |  |  |  |  |

Table 7.34.1.1-2: E-UTRA CA_42C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_42C | 5, 10, 15, 20 | 20 | 40 | 0 |
|  | 20 | 5, 10, 15 |  |  |
|  | 10, 15, 20 | 20 | 40 | 1 |
|  | 20 | 10, 15 |  |  |

7.34.1.2	Co-existence studies for LTE-A UL CA_41A-42A and DL CA_3A_41A-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.34.1.2-1

Table 7.34.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 710 | 1104 | 5896 | 6290 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1392 | 1980 | 4110 | 4704 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8392 | 8980 | 9296 | 9890 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3888 | 4670 | 7510 | 8304 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10888 | 11670 | 12696 | 13490 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2208 | 1420 | 11792 | 12580 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11904 | 10910 | 7360 | 6384 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 16096 | 17090 | 13384 | 14360 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5808 | 4820 | 1270 | 288 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 15192 | 16180 | 14288 | 15270 |

From the harmonics/IMDs analysis table, there were 3rd & 4th IMDs impact to the 3rd own Rx frequency of Band 3.

It will be covered in inter-band 3DL_CA_3A-41A-42A with 2UL_CA_41A-42A. Hence there was no self interference issues.

7.34.1.3	Co-existence studies for LTE-A UL CA_42C and DL CA_3A-41A-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.34.1.3-1

Table 7.34.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high |
| --- | --- | --- |
| UL frequency (MHz) | 3400 | 3600 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 6800 | 7200 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 10200 | 10800 |

From the harmonics analysis table, there was no harmonics from Band 42 uplinks which would desensitize the 3rd & 4th own Rx frequency of Band 3 and Band 41. The worst frequency separation of band 41 and 42 is 710 MHz therefore there is no impact from Band 42 uplink CA to Band 41 reception evenif RAN4 consider asynchronization scenario.

7.34.1.4	MSD

When uplink CA configurations CA_41A-42A is paired with downlink CA configuration CA_3A_41A-42C there is no interference components from 2 uplink operation which would interfere the downlink of the Band 3 and Band 42.

When uplink CA configurations CA_42C is paired with downlink CA configuration CA_3A_41A-42C there is no interference components from 2 uplink operation which would interfere the downlink of the Band 3 and Band 41.

7.34.1.5	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7.35	LTE Advanced Carrier Aggregation: Band 3 and Band 41 and Band 41 and Band 42 with 2 ULs

7.35.1	List of specific combination issues

7.35.1.1	Channel bandwidths per operating band for CA

Table 7.35.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A_41C-42A | CA_41A-42A | 3 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 41 | See CA_41C Bandwidth combination set 0 in Table 7.35.1.1-2 |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes | Yes | Yes |  |  |

Table 7.35.1.1-2: E-UTRA CA_41C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_41C | 10 | 20 | 40 | 0 |
|  | 15 | 15, 20 |  |  |
|  | 20 | 10, 15, 20 |  |  |

7.35.1.2	Co-existence studies for LTE-A UL CA_41A-42A and DL CA_3A_41C-42A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.35.1.2-1

Table 7.35.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 710 | 1104 | 5896 | 6290 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1392 | 1980 | 4110 | 4704 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8392 | 8980 | 9296 | 9890 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3888 | 4670 | 7510 | 8304 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10888 | 11670 | 12696 | 13490 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2208 | 1420 | 11792 | 12580 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11904 | 10910 | 7360 | 6384 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 16096 | 17090 | 13384 | 14360 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5808 | 4820 | 1270 | 288 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 15192 | 16180 | 14288 | 15270 |

From the harmonics/IMDs analysis table, there were 3rd & 4th IMDs impact to the 3rd own Rx frequency of Band 3.

It will be covered in inter-band 3DL_CA_3A-41A-42A with 2UL_CA_41A-42A. Hence there was no self interference issues.

7.35.1.3		MSD

When uplink CA configurations CA_41A-42A is paired with downlink CA configuration CA_3A_41C-42A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 3 and Band 41.

7.35.1.4	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7.36	LTE Advanced Carrier Aggregation: Band 28 and Band 41 and Band 42 and Band 42 with 2 ULs

7.36.1	List of specific combination issues

7.36.1.1	Channel bandwidths per operating band for CA

Table 7.36.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_28A_41A-42C | CA_41A-42A, CA_42C | 28 |  |  | Yes | Yes |  |  | 70 | 0 |
|  |  | 41 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 0 in Table 7.36.1.1-2 |  |  |  |  |  |  |  |

Table 7.36.1.1-2: E-UTRA CA_42C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_42C | 5, 10, 15, 20 | 20 | 40 | 0 |
|  | 20 | 5, 10, 15 |  |  |
|  | 10, 15, 20 | 20 | 40 | 1 |
|  | 20 | 10, 15 |  |  |

7.36.1.2	Co-existence studies for LTE-A UL CA_41A-42A and DL CA_28A_41A-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.36.1.2-1

Table 7.36.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 710 | 1104 | 5896 | 6290 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1392 | 1980 | 4110 | 4704 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8392 | 8980 | 9296 | 9890 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3888 | 4670 | 7510 | 8304 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10888 | 11670 | 12696 | 13490 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2208 | 1420 | 11792 | 12580 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11904 | 10910 | 7360 | 6384 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 16096 | 17090 | 13384 | 14360 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5808 | 4820 | 1270 | 288 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 15192 | 16180 | 14288 | 15270 |

From the harmonics/IMDs analysis table, there were 2nd & 5th IMDs impact to the 3rd own Rx frequency of Band 28.

It will be covered in inter-band 3DL_CA_28A-41A-42A with 2UL_CA_41A-42A. Hence there was no self interference issues.

7.36.1.3	Co-existence studies for LTE-A UL CA_42C and DL CA_28A-41A-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.36.1.3-1

Table 7.36.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high |
| --- | --- | --- |
| UL frequency (MHz) | 3400 | 3600 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 6800 | 7200 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 10200 | 10800 |

From the harmonics analysis table, there was no harmonics from Band 42 uplinks which would desensitize the 3rd & 4th own Rx frequency of Band 28 and Band 41. The worst frequency separation of band 41 and 42 is 710 MHz therefore there is no impact from Band 42 uplink CA to Band 41 reception evenif RAN4 consider asynchronization scenario.

7.36.1.4	MSD

When uplink CA configurations CA_41A-42A is paired with downlink CA configuration CA_28A_41A-42C there is no interference components from 2 uplink operation which would interfere the downlink of the Band 28 and Band 42.

When uplink CA configurations CA_42C is paired with downlink CA configuration CA_28A_41A-42C there is no interference components from 2 uplink operation which would interfere the downlink of the Band 28 and Band 41.

7.36.1.5	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7.37	LTE Advanced Carrier Aggregation: Band 28 and Band 41 and Band 41 and Band 42 with 2 ULs

7.37.1	List of specific combination issues

7.37.1.1	Channel bandwidths per operating band for CA

Table 7.37.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_28A_41C-42A | CA_41A-42A | 28 |  |  | Yes | Yes |  |  | 70 | 0 |
|  |  | 41 | See CA_41C Bandwidth combination set 0 in Table 7.37.1.1-2 |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes | Yes | Yes |  |  |

Table 7.37.1.1-2: E-UTRA CA_41C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_41C | 10 | 20 | 40 | 0 |
|  | 15 | 15, 20 |  |  |
|  | 20 | 10, 15, 20 |  |  |

7.37.1.2	Co-existence studies for LTE-A UL CA_41A-42A and DL CA_28A_41C-42A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.37.1.2-1

Table 7.37.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 710 | 1104 | 5896 | 6290 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1392 | 1980 | 4110 | 4704 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8392 | 8980 | 9296 | 9890 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3888 | 4670 | 7510 | 8304 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10888 | 11670 | 12696 | 13490 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2208 | 1420 | 11792 | 12580 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11904 | 10910 | 7360 | 6384 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 16096 | 17090 | 13384 | 14360 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5808 | 4820 | 1270 | 288 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 15192 | 16180 | 14288 | 15270 |

From the harmonics/IMDs analysis table, there were 2nd & 5th IMDs impact to the 3rd own Rx frequency of Band 28.

It will be covered in inter-band 3DL_CA_28A-41A-42A with 2UL_CA_41A-42A. Hence there was no self interference issues.

7.37.1.3		MSD

When uplink CA configurations CA_41A-42A is paired with downlink CA configuration CA_28A_41C-42A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 28 and Band 41.

7.37.1.4	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

7.38	LTE Advanced Carrier Aggregation: Band 3 and Band 28 and Band 41 and Band 42 with 2 ULs

7.38.1	List of specific combination issues

7.38.1.1	Channel bandwidths per operating band for CA

Table 7.38.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A_28A-41A-42A | CA_41A-42A | 3 |  |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 28 |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 41 |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 42 |  |  |  |  | Yes | Yes | Yes |  |  |

7.38.1.2	Co-existence studies for LTE-A UL CA_41A-42A and DL CA_3A-28A-41A-42A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.38.1.2-1

Table 7.38.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 710 | 1104 | 5896 | 6290 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1392 | 1980 | 4110 | 4704 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8392 | 8980 | 9296 | 9890 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3888 | 4670 | 7510 | 8304 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10888 | 11670 | 12696 | 13490 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2208 | 1420 | 11792 | 12580 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11904 | 10910 | 7360 | 6384 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 16096 | 17090 | 13384 | 14360 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5808 | 4820 | 1270 | 288 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 15192 | 16180 | 14288 | 15270 |

From the harmonics/IMDs analysis table, there were 3rd & 4th IMDs impact to the 3rd own Rx frequency of Band 3. Also the 2nd & 5th IMDs fall into the 4th own Rx frequency of Band 28. However, the impacts will be covered in inter-band 3DL_CA_3A-41A-42A with 2UL_CA_41A-42A and 3DL_CA_28A-41A-42A with 2UL_CA_41A-42A. Hence there was no self interference issues.

7.38.1.3		MSD

When uplink CA configurations CA_41A-42A is paired with downlink CA configuration CA_3A-28A-41A-42A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 3 and Band 28.

7.38.1.4	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

8		5DLs/2ULs Inter-Band Carrier Aggregation: Specific Band Combination Part

8.1	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 19 and Band 42 and Band 42 with 2 ULs

8.1.1	List of specific combination issues

8.1.1.1	Channel bandwidths per operating band for CA

Table 8.1.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-3A-19A-42C | CA_1A-3A or CA_1A-19A or CA_1A-42A or CA_3A-19A or CA_3A-42A or CA_19A-42A | 1 |  |  | Yes | Yes | Yes | Yes | 95 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 19 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 8.1.1.1-2 |  |  |  |  |  |  |  |

Table 8.1.1.1-2: E-UTRA CA_42C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_42C | 5, 10, 15, 20 | 20 | 40 | 0 |
|  | 20 | 5, 10, 15 |  |  |
|  | 10, 15, 20 | 20 | 40 | 1 |
|  | 20 | 10, 15 |  |  |

8.1.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-19A-42A-42A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.1.1.2-1 based on a certain operater’s actual spectrum holdings.

Table 8.1.1.2-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1764.9 | 1784.9 | 1940 | 1960 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3529.8 | 3569.8 | 3880 | 3920 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5294.7 | 5354.7 | 5820 | 5880 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 155.1 | 195.1 | 3704.9 | 3744.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1569.8 | 1629.8 | 2095.1 | 2155.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5469.8 | 5529.8 | 5644.9 | 5704.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3334.7 | 3414.7 | 4035.1 | 4115.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7234.7 | 7314.7 | 7584.9 | 7664.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 390.2 | 310.2 | 7409.8 | 7489.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6075.1 | 5975.1 | 5199.6 | 5099.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9524.9 | 9624.9 | 8999.6 | 9099.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2350.2 | 2250.2 | 1474.7 | 1374.7 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9349.8 | 9449.8 | 9174.7 | 9274.7 |

The 2nd harmonic by Band 3 and 3rd IMD fall into the own Rx Band 1. But this impact already has covered in 2DL/2UL CA_1A-3A.

The 4th IMD fall into the 4th own Rx frequency of Band 42. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 19. Then, there is no IMD problems to the 3rd, 4th & 5th own Rx frequency of Band 19, Band 42 and Band 42.

8.1.1.3	Co-existence studies for LTE-A UL CA_1A-19A and DL CA_1A-3A-19A-42A-42A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.1.1.3-1 based on a certain operater’s actual spectrum holdings.

Table 8.1.1.3-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 830 | 845 | 1940 | 1960 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1660 | 1690 | 3880 | 3920 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2490 | 2535 | 5820 | 5880 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1095 | 1130 | 2770 | 2805 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 300 | 250 | 3035 | 3090 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3600 | 3650 | 4710 | 4765 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 530 | 595 | 4975 | 5050 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4430 | 4495 | 6650 | 6725 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2260 | 2190 | 5540 | 5610 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7010 | 6915 | 1440 | 1360 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8590 | 8685 | 5260 | 5340 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4220 | 4130 | 1345 | 1430 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7480 | 7570 | 6370 | 6455 |

The 4th IMD fall into the own Rx Band 1. But this impact already has covered in 2DL/2UL CA_1A-19A.

The 3rd IMD fall into the 4th own Rx frequency of Band 42. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 19. Then, there is no IMD problems to the 3rd, 4th & 5th own Rx frequency of Band 3, Band 42 and Band 42.

8.1.1.4	Co-existence studies for LTE-A UL CA_ 1A-42A and DL CA_1A-3A-19A-42A-42A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.1.1.4-1 based on a certain operater’s actual spectrum holdings.

Table 8.1.1.4-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1940 | 1960 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3880 | 3920 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5820 | 5880 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1520 | 1580 | 5420 | 5480 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 360 | 440 | 5000 | 5100 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7360 | 7440 | 8900 | 9000 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2300 | 2400 | 8480 | 8620 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9300 | 9400 | 12380 | 12520 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3160 | 3040 | 10840 | 10960 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12140 | 11960 | 4360 | 4240 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15860 | 16040 | 11240 | 11360 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6680 | 6520 | 1080 | 1220 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14320 | 14480 | 12780 | 12920 |

The 4th IMD fall into the own Rx Band 1. But this impact already has covered in 2DL/2UL CA_1A-42A.

The 5th IMD fall into the 4th own Rx frequency of Band 19. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 19. Then, there is no IMD problems to the 3rd, 4th & 5th own Rx frequency of Band 3, Band 19 and Band 42.

8.1.1.5	Co-existence studies for LTE-A UL CA_ 3A-19A and DL CA_1A-3A-19A-42A-42A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.1.1.5-1 based on a certain operater’s actual spectrum holdings.

Table 8.1.1.5-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 830 | 845 | 1764.9 | 1784.9 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1660 | 1690 | 3529.8 | 3569.8 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2490 | 2535 | 5294.7 | 5354.7 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 919.9 | 954.9 | 2594.9 | 2629.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 124.9 | 74.9 | 2684.8 | 2739.8 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3424.9 | 3474.9 | 4359.8 | 4414.8 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 705.1 | 770.1 | 4449.7 | 4524.7 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4254.9 | 4319.9 | 6124.7 | 6199.7 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1909.8 | 1839.8 | 5189.8 | 5259.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6309.6 | 6214.6 | 1615.1 | 1535.1 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7889.6 | 7984.6 | 5084.9 | 5164.9 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3694.7 | 3604.7 | 994.8 | 1079.8 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6954.7 | 7044.7 | 6019.8 | 6104.8 |

The 2nd & 5th IMDs fall into the own Rx frequency of Band 19. Also the 4th IMD fall into the own Rx frequency of Band 3. But theses impacts already has covered in 2DL/2UL CA_3A-19A.

The 2nd harmonics from Band 3 transmission will be interference to the Band 42. But this harmonics issue already has covered in 2DL/1UL CA_3A-42A.

The 3rd & 5th IMDs fall into the 4th own Rx frequency of Band 42. However, RAN4 consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 19.

Then, there was no harmonics/IMDs problem to the 3rd, 4th & 5th own Rx frequency of Band 1, Band 42 and Band 42.

8.1.1.6	Co-existence studies for LTE-A UL CA_ 3A-42A and DL CA_1A-3A-19A-42A-42A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.1.1.6-1 based on a certain operater’s actual spectrum holdings.

Table 8.1.1.6-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1764.9 | 1784.9 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3529.8 | 3569.8 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5294.7 | 5354.7 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1695.1 | 1755.1 | 5244.9 | 5304.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 9.8 | 89.8 | 5175.1 | 5275.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7009.8 | 7089.8 | 8724.9 | 8824.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1774.7 | 1874.7 | 8655.1 | 8795.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8774.7 | 8874.7 | 12204.9 | 12344.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3510.2 | 3390.2 | 10489.8 | 10609.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12315.1 | 12135.1 | 3659.6 | 3539.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15684.9 | 15864.9 | 10539.6 | 10659.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7030.2 | 6870.2 | 1605.3 | 1745.3 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13969.8 | 14129.8 | 12254.7 | 12394.7 |

The 2nd , 4th & 5th IMDs fall into the own Rx frequency of Band 3. Also the 2nd harmonics from Band 3, 4th & 5th IMDs fall into the own Rx frequency of Band 42. But these impact already has covered in 2DL/2UL CA_3A-42A.

Hence, there was no harmonics/IMDs impact to the 3rd, 4th & 5th own Rx frequency of Band 1, Band 19 and Band 42.

8.1.1.7	Co-existence studies for LTE-A UL CA_ 19A-42A and DL CA_1A-3A-19A-42A-42A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.1.1.7-1 based on a certain operater’s actual spectrum holdings.

Table 8.1.1.7-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 830 | 845 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1660 | 1690 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2490 | 2535 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2635 | 2690 | 4310 | 4365 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1860 | 1790 | 6115 | 6210 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5140 | 5210 | 7790 | 7885 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1030 | 945 | 9595 | 9730 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5970 | 6055 | 11270 | 11405 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 5380 | 5270 | 8620 | 8730 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13250 | 13075 | 100 | 200 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14750 | 14925 | 6800 | 6900 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 8900 | 8750 | 4425 | 4550 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 12100 | 12250 | 9450 | 9575 |

The 4th IMD fall into the own Rx frequency of Band 19. But this impact already has covered in 2DL/2UL CA_19A-42A.

The 3rd IMD fall into the 4th own Rx frequency of Band 3. The overlap range is 0.1MHz that is no critical impact to the sensitivity of Band 3 when RAN4 consider operator specific frequency ranges.

Hence, there was no harmonics/IMDs impact to the 3rd, 4th & 5th own Rx frequency of Band 1, Band 3 and Band 42.

8.1.1.8	MSD

When uplink CA configurations CA_1A-3A is paired with downlink CA configuration CA_1A-3A-19A-42C there is no interference components from 2 uplink operation which would interfere the 3rd, 4th & 5th downlink of the Band 19 and Band 42 and Band 42.

When uplink CA configurations CA_1A-19A is paired with downlink CA configuration CA_1A-3A-19A-42C there is no interference components from 2 uplink operation which would interfere the 3rd, 4th & 5th downlink of the Band 3 and Band 42 and Band 42.

When uplink CA configurations CA_1A-42A is paired with downlink CA configuration CA_1A-3A-19A-42C there is no interference components from 2 uplink operation which would interfere the 3rd, 4th & 5th downlink of the Band 3 and Band 19 and Band 42.

When uplink CA configurations CA_3A-19A is paired with downlink CA configuration CA_1A-3A-19A-42C there is no interference components from 2 uplink operation which would interfere the 3rd, 4th & 5th downlink of the Band 1 and Band 42 and Band 42.

When uplink CA configurations CA_3A-42A is paired with downlink CA configuration CA_1A-3A-19A-42C there is no interference components from 2 uplink operation which would interfere the 3rd, 4th & 5th downlink of the Band 1 and Band 19 and Band 42.

When uplink CA configurations CA_19A-42A is paired with downlink CA configuration CA_1A-3A-19A-42C there is no interference components from 2 uplink operation which would interfere the 3rd, 4th & 5th downlink of the Band 1 and Band 3 and Band 42.

8.1.1.9	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

8.2	LTE Advanced Carrier Aggregation: Band 1 and Band 19 and Band 21 and Band 42 and Band 42 with 2 ULs

8.2.1	List of specific combination issues

8.2.1.1	Channel bandwidths per operating band for CA

Table 8.2.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-19A-21A-42C | CA_1A-19A or CA_1A-21A or CA_1A-42A or CA_19A-21A or CA_19A-42A or CA_21A-42A | 1 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 19 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 8.2.1.1-2 |  |  |  |  |  |  |  |

Table 8.2.1.1-2: E-UTRA CA_42C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_42C | 5, 10, 15, 20 | 20 | 40 | 0 |
|  | 20 | 5, 10, 15 |  |  |
|  | 10, 15, 20 | 20 | 40 | 1 |
|  | 20 | 10, 15 |  |  |

8.2.1.2	Co-existence studies for LTE-A UL CA_1A-19A and DL CA_1A-19A-21A-42C

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.2.1.2-1 based on a certain operater’s actual spectrum holdings.

Table 8.2.1.2-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 830 | 845 | 1940 | 1960 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1660 | 1690 | 3880 | 3920 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2490 | 2535 | 5820 | 5880 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1095 | 1130 | 2770 | 2805 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 300 | 250 | 3035 | 3090 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3600 | 3650 | 4710 | 4765 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 530 | 595 | 4975 | 5050 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4430 | 4495 | 6650 | 6725 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2260 | 2190 | 5540 | 5610 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7010 | 6915 | 1440 | 1360 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8590 | 8685 | 5260 | 5340 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4220 | 4130 | 1345 | 1430 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7480 | 7570 | 6370 | 6455 |

The 4th IMD fall into the own Rx Band 1. But this impact already has covered in 2DL/2UL CA_1A-19A.

The 3rd IMD fall into the 4th own Rx frequency of Band 42. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 19 and Band 21. Then, there is no IMD problems to the 3rd, 4th & 5th own Rx frequency of Band 21 and Band 42 and Band 42.

8.2.1.3	Co-existence studies for LTE-A UL CA_1A-21A and DL CA_1A-19A-21A-42C

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.2.1.3-1 based on a certain operater’s actual spectrum holdings.

Table 8.2.1.3-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 1940 | 1960 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 3880 | 3920 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 5820 | 5880 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 477.1 | 512.1 | 3387.9 | 3422.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 935.8 | 985.8 | 2417.1 | 2472.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4835.8 | 4885.8 | 5327.9 | 5382.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2383.7 | 2448.7 | 4357.1 | 4432.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6283.7 | 6348.7 | 7267.9 | 7342.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1024.2 | 954.2 | 6775.8 | 6845.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6392.1 | 6297.1 | 3911.6 | 3831.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9207.9 | 9302.9 | 7731.6 | 7811.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2984.2 | 2894.2 | 508.7 | 423.7 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8715.8 | 8805.8 | 8223.7 | 8308.7 |

The 2nd IMD fall into the 4th own Rx frequency of Band 42. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 19 and Band 21. Then, there is no IMD problems to the 3rd, 4th & 5th own Rx frequency of Band 19 and Band 42 and Band 42.

8.2.1.4	Co-existence studies for LTE-A UL CA_ 1A-42A and DL CA_1A-19A-21A-42C

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.2.1.4-1 based on a certain operater’s actual spectrum holdings.

Table 8.2.1.4-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1940 | 1960 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3880 | 3920 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5820 | 5880 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1520 | 1580 | 5420 | 5480 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 360 | 440 | 5000 | 5100 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7360 | 7440 | 8900 | 9000 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2300 | 2400 | 8480 | 8620 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9300 | 9400 | 12380 | 12520 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3160 | 3040 | 10840 | 10960 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12140 | 11960 | 4360 | 4240 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15860 | 16040 | 11240 | 11360 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6680 | 6520 | 1080 | 1220 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14320 | 14480 | 12780 | 12920 |

The 4th IMD fall into the own Rx Band 1. But this impact already has covered in 2DL/2UL CA_1A-42A.

The 2nd IMD fall into the 4th own Rx frequency of Band 21. And also the 5th IMD fall into the 3rd own Rx frequency of Band 19. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 19 and Band 21. Then, there is no IMD problems to the 3rd, 4th & 5th own Rx frequency of Band 19 and Band 21 and Band 42.

8.2.1.5	Co-existence studies for LTE-A UL CA_ 19A-21A and DL CA_1A-19A-21A-42C

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.2.1.5-1.

Table 8.2.1.5-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 830 | 845 | 1447.9 | 1462.9 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1660 | 1690 | 2895.8 | 2925.8 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2490 | 2535 | 4343.7 | 4388.7 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 602.9 | 632.9 | 2277.9 | 2307.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 197.1 | 242.1 | 2050.8 | 2095.8 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3107.9 | 3152.9 | 3725.8 | 3770.8 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1027.1 | 1087.1 | 3498.7 | 3558.7 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3937.9 | 3997.9 | 5173.7 | 5233.7 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1265.8 | 1205.8 | 4555.8 | 4615.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5021.6 | 4946.6 | 1932.1 | 1857.1 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 6621.6 | 6696.6 | 4767.9 | 4842.9 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2728.7 | 2653.7 | 360.8 | 435.8 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6003.7 | 6078.7 | 5385.8 | 5460.8 |

From the harmonics and IMD analysis table, the 4th IMD fall into the 4th own Rx frequency of Band 42. But this IMD problem will be covered in 3DL_CA_19A-21A-42A with 2UL_CA_19A-21A.

8.2.1.6	Co-existence studies for LTE-A UL CA_ 19A-42A and DL CA_1A-19A-21A-42C

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.2.1.6-1 based on a certain operater’s actual spectrum holdings.

Table 8.2.1.6-1: Harmonic and IMDs analysis

| UE U+N5:R30L carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 830 | 845 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1660 | 1690 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2490 | 2535 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2635 | 2690 | 4310 | 4365 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1860 | 1790 | 6115 | 6210 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5140 | 5210 | 7790 | 7885 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1030 | 945 | 9595 | 9730 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5970 | 6055 | 11270 | 11405 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 5380 | 5270 | 8620 | 8730 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13250 | 13075 | 100 | 200 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14750 | 14925 | 6800 | 6900 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 8900 | 8750 | 4425 | 4550 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 12100 | 12250 | 9450 | 9575 |

From the harmonics/IMDs analysis table, there was no interference to the own 3rd, 4th and 5th Rx frequency range.

8.2.1.7	Co-existence studies for LTE-A UL CA_ 21A-42A and DL CA_1A-19A-21A-42C

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.2.1.7-1 based on a certain operater’s actual spectrum holdings.

Table 8.2.1.7-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2017.1 | 2072.1 | 4927.9 | 4982.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 624.2 | 554.2 | 5497.1 | 5592.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6375.8 | 6445.8 | 8407.9 | 8502.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 823.7 | 908.7 | 8977.1 | 9112.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7823.7 | 7908.7 | 11887.9 | 12022.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 4144.2 | 4034.2 | 9855.8 | 9965.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12632.1 | 12457.1 | 2371.6 | 2271.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15367.9 | 15542.9 | 9271.6 | 9371.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7664.2 | 7514.2 | 2571.3 | 2696.3 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13335.8 | 13485.8 | 11303.7 | 11428.7 |

The 2nd IMD fall into the 3rd own Rx frequency of Band 1. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 19 and Band 21. Then, there is no IMD problems to the 3rd own Rx frequency of Band 1.

The 4th IMD fall into the 3rd own Rx frequency of Band 19 evenif RAN4 consider operator specific frequency range in the related bands. But it also will be covered in 3DL_CA_19A-21A-42A with 2UL_CA_21A-42A.

Hence, there was no interference to the own 3rd, 4th and 5th Rx frequency range of Band 1 and Band 19 and Band 42.

8.2.1.8	MSD

When uplink CA configurations CA_1A-19A is paired with downlink CA configuration CA_1A-19A-21A-42C there is no interference components from 2 uplink operation which would interfere the 3rd, 4th & 5th downlink of the Band 21 and Band 42 and Band 42.

When uplink CA configurations CA_1A-21A is paired with downlink CA configuration CA_1A-19A-21A-42C there is no interference components from 2 uplink operation which would interfere the 3rd, 4th & 5th downlink of the Band 19 and Band 42 and Band 42.

When uplink CA configurations CA_1A-42A is paired with downlink CA configuration CA_1A-19A-21A-42C there is no interference components from 2 uplink operation which would interfere the 3rd, 4th & 5th downlink of the Band 19 and Band 21 and Band 42.

When uplink CA configurations CA_19A-21A is paired with downlink CA configuration CA_1A-19A-21A-42C there is no interference components from 2 uplink operation which would interfere the 3rd, 4th & 5th downlink of the Band 1 and Band 42 and Band 42.

When uplink CA configurations CA_19A-42A is paired with downlink CA configuration CA_1A-19A-21A-42C there is no interference components from 2 uplink operation which would interfere the 3rd, 4th & 5th downlink of the Band 1 and Band 21 and Band 42.

When uplink CA configurations CA_21A-42A is paired with downlink CA configuration CA_1A-19A-21A-42C there is no interference components from 2 uplink operation which would interfere the 3rd, 4th & 5th downlink of the Band 1 and Band 19 and Band 42.

8.2.1.9	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

8.3	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 21 and Band 42 and Band 42 with 2 ULs

8.3.1	List of specific combination issues

8.3.1.1	Channel bandwidths per operating band for CA

Table 8.3.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-3A-21A-42C | CA_1A-3A, CA_1A-21A, CA_1A-42A, CA_3A-21A, CA_3A-42A, CA_21A-42A | 1 |  |  | Yes | Yes | Yes | Yes | 95 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 8.3.1.1-2 |  |  |  |  |  |  |  |

Table 8.3.1.1-2: E-UTRA CA_42C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_42C | 5, 10, 15, 20 | 20 | 40 | 0 |
|  | 20 | 5, 10, 15 |  |  |
|  | 10, 15, 20 | 20 | 40 | 1 |
|  | 20 | 10, 15 |  |  |

8.3.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-21A-42A-42A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.3.1.2-1 based on a certain operater’s actual spectrum holdings.

Table 8.3.1.2-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1764.9 | 1784.9 | 1940 | 1960 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3529.8 | 3569.8 | 3880 | 3920 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5294.7 | 5354.7 | 5820 | 5880 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 155.1 | 195.1 | 3704.9 | 3744.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1569.8 | 1629.8 | 2095.1 | 2155.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5469.8 | 5529.8 | 5644.9 | 5704.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3334.7 | 3414.7 | 4035.1 | 4115.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7234.7 | 7314.7 | 7584.9 | 7664.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 390.2 | 310.2 | 7409.8 | 7489.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6075.1 | 5975.1 | 5199.6 | 5099.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9524.9 | 9624.9 | 8999.6 | 9099.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2350.2 | 2250.2 | 1474.7 | 1374.7 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9349.8 | 9449.8 | 9174.7 | 9274.7 |

The 2nd harmonic by Band 3 fall into the own Rx band 42. But this impact already has covered in 2DL/1UL CA_3A-42A.

The 3rd & 5th IMDs fall into the own 3rd Rx Band 21. Aslo the 4th IMD fall into the own 4th Rx frequency of Band 42. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21. Then, there is no IMD problems to the 3rd, 4th & 5th own Rx frequency of Band 21, Band 42 and Band 42.

8.3.1.3	Co-existence studies for LTE-A UL CA_1A-21A and DL CA_1A-3A-21A-42A-42A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.3.1.3-1 based on a certain operater’s actual spectrum holdings.

Table 8.3.1.3-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 1940 | 1960 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 3880 | 3920 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 5820 | 5880 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 477.1 | 512.1 | 3387.9 | 3422.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 935.8 | 985.8 | 2417.1 | 2472.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4835.8 | 4885.8 | 5327.9 | 5382.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2383.7 | 2448.7 | 4357.1 | 4432.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6283.7 | 6348.7 | 7267.9 | 7342.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1024.2 | 954.2 | 6775.8 | 6845.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6392.1 | 6297.1 | 3911.6 | 3831.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9207.9 | 9302.9 | 7731.6 | 7811.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2984.2 | 2894.2 | 508.7 | 423.7 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8715.8 | 8805.8 | 8223.7 | 8308.7 |

The 2nd IMD fall into the 4th own Rx frequency of Band 42. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21. Then, there is no harmonics/IMD problems to the 3rd, 4th & 5th own Rx frequency of Band 3, Band 42 and Band 42.

8.3.1.4	Co-existence studies for LTE-A UL CA_ 1A-42A and DL CA_1A-3A-21A-42A-42A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.3.1.4-1 based on a certain operater’s actual spectrum holdings.

Table 8.3.1.4-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1940 | 1960 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3880 | 3920 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5820 | 5880 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1520 | 1580 | 5420 | 5480 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 360 | 440 | 5000 | 5100 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7360 | 7440 | 8900 | 9000 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2300 | 2400 | 8480 | 8620 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9300 | 9400 | 12380 | 12520 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3160 | 3040 | 10840 | 10960 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12140 | 11960 | 4360 | 4240 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15860 | 16040 | 11240 | 11360 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6680 | 6520 | 1080 | 1220 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14320 | 14480 | 12780 | 12920 |

The 2nd IMD fall into the 3rd own Rx frequency of Band 21. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21. Then, there is no harmonics/IMD problems to the 3rd, 4th & 5th own Rx frequency of Band 3, Band 21 and Band 42.

8.3.1.5	Co-existence studies for LTE-A UL CA_ 3A-21A and DL CA_1A-3A-21A-42A-42A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.3.1.5-1 based on a certain operater’s actual spectrum holdings.

Table 8.3.1.5-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 1764.9 | 1784.9 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 3529.8 | 3569.8 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 5294.7 | 5354.7 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 302 | 337 | 3212.8 | 3247.8 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1110.9 | 1160.9 | 2066.9 | 2121.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4660.7 | 4710.7 | 4977.7 | 5032.7 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2558.8 | 2623.8 | 3831.8 | 3906.8 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6108.6 | 6173.6 | 6742.6 | 6817.6 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 674 | 604 | 6425.6 | 6495.6 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5691.7 | 5596.7 | 4086.7 | 4006.7 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8507.5 | 8602.5 | 7556.5 | 7636.5 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2458.9 | 2368.9 | 858.9 | 773.9 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8190.5 | 8280.5 | 7873.5 | 7958.5 |

The 2nd harmonics from Band 3 transmission will be interference to the Band 42. But this harmonics issue already has covered in 2DL/1UL CA_3A-42A.

The 3rd IMD fall into the 3rd own Rx frequency of Band 1. However, RAN4 consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21.

Hence, there was no harmonics/IMDs problem to the 3rd, 4th & 5th own Rx frequency of Band 1, Band 42 and Band 42.

8.3.1.6	Co-existence studies for LTE-A UL CA_ 3A-42A and DL CA_1A-3A-21A-42A-42A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.3.1.6-1 based on a certain operater’s actual spectrum holdings.

Table 8.3.1.6-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1764.9 | 1784.9 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3529.8 | 3569.8 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5294.7 | 5354.7 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1695.1 | 1755.1 | 5244.9 | 5304.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 9.8 | 89.8 | 5175.1 | 5275.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7009.8 | 7089.8 | 8724.9 | 8824.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1774.7 | 1874.7 | 8655.1 | 8795.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8774.7 | 8874.7 | 12204.9 | 12344.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3510.2 | 3390.2 | 10489.8 | 10609.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12315.1 | 12135.1 | 3659.6 | 3539.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15684.9 | 15864.9 | 10539.6 | 10659.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7030.2 | 6870.2 | 1605.3 | 1745.3 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13969.8 | 14129.8 | 12254.7 | 12394.7 |

The 2nd harmonics from Band 3, 4th and 5th IMDs fall into the own Rx frequency of Band 42. But these impact already has covered in 2DL/2UL CA_3A-42A.

The 5th IMD fall into the own Rx frequency of Band 21. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21. Then, there is no harmonics/IMD problems to the 3rd, 4th & 5th own Rx frequency of Band 1, Band 21 and Band 42.

8.3.1.7	Co-existence studies for LTE-A UL CA_21A-42A and DL CA_1A-3A-21A-42A-42A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.3.1.7-1 based on a certain operater’s actual spectrum holdings.

Table 8.3.1.7-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2017.1 | 2072.1 | 4927.9 | 4982.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 624.2 | 554.2 | 5497.1 | 5592.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6375.8 | 6445.8 | 8407.9 | 8502.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 823.7 | 908.7 | 8977.1 | 9112.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7823.7 | 7908.7 | 11887.9 | 12022.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 4144.2 | 4034.2 | 9855.8 | 9965.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12632.1 | 12457.1 | 2371.6 | 2271.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15367.9 | 15542.9 | 9271.6 | 9371.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7664.2 | 7514.2 | 2571.3 | 2696.3 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13335.8 | 13485.8 | 11303.7 | 11428.7 |

The 2nd IMD fall into the 3rd own Rx frequency of Band 1. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21. Then, there is no harmonics/IMD problems to the 3rd, 4th & 5th own Rx frequency of Band 1, Band 3 and Band 42.

8.3.1.8	MSD

When uplink CA configurations CA_1A-3A is paired with downlink CA configuration CA_1A-3A-21A-42C there is no interference components from 2 uplink operation which would interfere the 3rd, 4th & 5th downlink of the Band 21 and Band 42 and Band 42.

When uplink CA configurations CA_1A-21A is paired with downlink CA configuration CA_1A-3A-21A-42C there is no interference components from 2 uplink operation which would interfere the 3rd, 4th & 5th downlink of the Band 3 and Band 42 and Band 42.

When uplink CA configurations CA_1A-42A is paired with downlink CA configuration CA_1A-3A-21A-42C there is no interference components from 2 uplink operation which would interfere the 3rd, 4th & 5th downlink of the Band 3 and Band 21 and Band 42.

When uplink CA configurations CA_3A-21A is paired with downlink CA configuration CA_1A-3A-21A-42C there is no interference components from 2 uplink operation which would interfere the 3rd, 4th & 5th downlink of the Band 1 and Band 42 and Band 42.

When uplink CA configurations CA_3A-42A is paired with downlink CA configuration CA_1A-3A-21A-42C there is no interference components from 2 uplink operation which would interfere the 3rd, 4th & 5th downlink of the Band 1 and Band 21 and Band 42.

When uplink CA configurations CA_21A-42A is paired with downlink CA configuration CA_1A-3A-21A-42C there is no interference components from 2 uplink operation which would interfere the 3rd, 4th & 5th downlink of the Band 1 and Band 3 and Band 42.

8.3.1.9	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

8.4	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 28 and Band 42 and Band 42 with 2 ULs

8.4.1	List of specific combination issues

8.4.1.1	Channel bandwidths per operating band for CA

Table 8.4.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-3A-28A-42C | CA_1A-3A, CA_1A-28A, CA_1A-42A, CA_3A-28A, CA_3A-42A, CA_28A-42A | 1 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 8.4.1.1-2 |  |  |  |  |  |  |  |

Table 8.4.1.1-2: E-UTRA CA_42C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_42C | 5, 10, 15, 20 | 20 | 40 | 0 |
|  | 20 | 5, 10, 15 |  |  |
|  | 10, 15, 20 | 20 | 40 | 1 |
|  | 20 | 10, 15 |  |  |

8.4.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-28A-42A-42A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.4.1.2-1.

Table 8.4.1.2-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 135 | 270 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1440 | 1650 | 2055 | 2250 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5340 | 5550 | 5550 | 5745 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3150 | 3435 | 3975 | 4230 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7050 | 7335 | 7470 | 7725 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 540 | 270 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6210 | 5895 | 5220 | 4860 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9390 | 9705 | 8760 | 9120 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2520 | 2190 | 1515 | 1170 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9180 | 9510 | 8970 | 9315 |

The 2nd harmonic by Band 3 fall into the own Rx band 42. But this impact already has covered in 2DL/1UL CA_3A-42A. The 4th IMD fall into the own 4th Rx Band 42. But this impact already covered in 3DL CA_1A-3A-42A with 2UL CA_1A-3A.

Hence, there were no harmonics/IMD problems to the 3rd, 4th & 5th own Rx frequency of Band 28, Band 42 and Band 42.

8.4.1.3	Co-existence studies for LTE-A UL CA_1A-28A and DL CA_1A-3A-28A-42A-42A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.4.1.3-1.

Table 8.4.1.3-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1172 | 1277 | 2623 | 2728 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 574 | 424 | 3092 | 3257 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3326 | 3476 | 4543 | 4708 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 129 | 324 | 5012 | 5237 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4029 | 4224 | 6463 | 6688 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2554 | 2344 | 5246 | 5456 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7217 | 6932 | 1072 | 832 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8383 | 8668 | 4732 | 4972 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4534 | 4264 | 1596 | 1851 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7166 | 7436 | 5949 | 6204 |

The 3rd IMD fall into the 4th own Rx frequency of Band 42. But this impact already covered in 3DL CA_1A-28A-42A with 2UL CA_1A-28A. Also the 5th IMD fall into the own Rx Band 3. The IMD impact also covered in 3DL CA_1A-3A-28A with 2UL CA_1A-28A.

Hence, there were no harmonics/IMD problems to the 3rd, 4th & 5th own Rx frequency of Band 3, Band 42 and Band 42.

8.4.1.4	Co-existence studies for LTE-A UL CA_ 1A-42A and DL CA_1A-3A-28A-42A-42A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.4.1.4-1.

Table 8.4.1.4-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1420 | 1680 | 5320 | 5580 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 240 | 560 | 4820 | 5280 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7240 | 7560 | 8720 | 9180 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2160 | 2540 | 8220 | 8880 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9160 | 9540 | 12120 | 12780 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3360 | 2840 | 10640 | 11160 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12480 | 11620 | 4520 | 4080 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15520 | 16380 | 11080 | 11520 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6960 | 6240 | 860 | 1440 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14040 | 14760 | 12560 | 13140 |

From the harmonics/IMD analysis table, there were no harmonics/IMD problems to the 3rd, 4th & 5th own Rx frequency of Band 3, Band 28 and Band 42.

8.4.1.5	Co-existence studies for LTE-A UL CA_ 3A-28A and DL CA_1A-3A-28A-42A-42A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.4.1.5-1.

Table 8.4.1.5-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 962 | 1082 | 2413 | 2533 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 379 | 214 | 2672 | 2867 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3116 | 3281 | 4123 | 4318 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 324 | 534 | 4382 | 4652 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3819 | 4029 | 5833 | 6103 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2164 | 1924 | 4826 | 5066 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6437 | 6092 | 1282 | 1027 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7543 | 7888 | 4522 | 4777 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3949 | 3634 | 1176 | 1461 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6536 | 6851 | 5529 | 5814 |

The 2nd harmonics from Band 3 transmission will be interference to the Band 42. But this harmonics issue already has covered in 2DL/1UL CA_3A-42A. The 3rd harmonics from Band 28 transmission will be interference to the Band 1. But this harmonics issue already has covered in 2DL/1UL CA_1A-28A

The 4th IMD fall into the 3rd own Rx frequency of Band 1. But, the IMD impact already covered in 3DL CA_1A-3A-28A with 2UL CA_3A-28A.

Hence, there was no harmonics/IMDs problem to the 3rd, 4th & 5th own Rx frequency of Band 1, Band 42 and Band 42.

8.4.1.6	Co-existence studies for LTE-A UL CA_ 3A-42A and DL CA_1A-3A-28A-42A-42A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.4.1.6-1.

Table 8.4.1.6-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1615 | 1890 | 5110 | 5385 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 180 | 170 | 5015 | 5490 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6820 | 7170 | 8510 | 8985 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1530 | 1955 | 8415 | 9090 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8530 | 8955 | 11910 | 12585 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3780 | 3230 | 10220 | 10770 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12690 | 11815 | 3740 | 3240 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15310 | 16185 | 10240 | 10740 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7380 | 6630 | 1445 | 2070 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13620 | 14370 | 11930 | 12555 |

The 2nd harmonics from Band 3, 4th and 5th IMDs fall into the own Rx frequency of Band 42. Also 2nd, 4th and 5th IMDs fall into the own Rx frequency of Band 3. But these impact already has covered in 2DL/2UL CA_3A-42A.

Hence, there were no harmonics/IMD problems to the 3rd, 4th & 5th own Rx frequency of Band 1, Band 28 and Band 42.

8.4.1.7	Co-existence studies for LTE-A UL CA_28A-42A and DL CA_1A-3A-28A-42A-42A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.4.1.7-1.

Table 8.4.1.7-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2652 | 2897 | 4103 | 4348 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2194 | 1904 | 6052 | 6497 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4806 | 5096 | 7503 | 7948 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1491 | 1156 | 9452 | 10097 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5509 | 5844 | 10903 | 11548 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 5794 | 5304 | 8206 | 8696 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13697 | 12852 | 408 | 788 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14303 | 15148 | 6212 | 6592 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9394 | 8704 | 4556 | 5091 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11606 | 12296 | 8909 | 9444 |

The 3rd harmonics by Band 28 fall into the own Rx Band 1. But the harmonics impact already covered in 2DL/1UL CA_1A-28A. The 3rd IMD fall into the 3rd own Rx frequency of Band 1. But the impact also covered in 3DL CA_1A-28A-42A with 2UL CA_28A-42A.

Hence, there were no harmonics/IMD problems to the 3rd, 4th & 5th own Rx frequency of Band 1, Band 3 and Band 42.

8.4.1.8	MSD

When uplink CA configurations CA_1A-3A is paired with downlink CA configuration CA_1A-3A-28A-42C there is no interference components from 2 uplink operation which would interfere the 3rd, 4th & 5th downlink of the Band 21 and Band 42 and Band 42.

When uplink CA configurations CA_1A-28A is paired with downlink CA configuration CA_1A-3A-28A-42C there is no interference components from 2 uplink operation which would interfere the 3rd, 4th & 5th downlink of the Band 3 and Band 42 and Band 42.

When uplink CA configurations CA_1A-42A is paired with downlink CA configuration CA_1A-3A-28A-42C there is no interference components from 2 uplink operation which would interfere the 3rd, 4th & 5th downlink of the Band 3 and Band 28 and Band 42.

When uplink CA configurations CA_3A-28A is paired with downlink CA configuration CA_1A-3A-28A-42C there is no interference components from 2 uplink operation which would interfere the 3rd, 4th & 5th downlink of the Band 1 and Band 42 and Band 42.

When uplink CA configurations CA_3A-42A is paired with downlink CA configuration CA_1A-3A-28A-42C there is no interference components from 2 uplink operation which would interfere the 3rd, 4th & 5th downlink of the Band 1 and Band 28 and Band 42.

When uplink CA configurations CA_28A-42A is paired with downlink CA configuration CA_1A-3A-28A-42C there is no interference components from 2 uplink operation which would interfere the 3rd, 4th & 5th downlink of the Band 1 and Band 3 and Band 42.

8.4.1.9	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

8.5	LTE Advanced Carrier Aggregation: Band 1 and Band 21 and Band 28 and Band 42 and Band 42 with 2 ULs

8.5.1	List of specific combination issues

8.5.1.1	Channel bandwidths per operating band for CA

Table 8.5.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-21A-28A-42C | CA_1A-21A, CA_1A-28A, CA_1A-42A, CA_21A-28A, CA_21A-42A, CA_28A-42A | 1 |  |  | Yes | Yes | Yes | Yes | 85 | 0 |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 8.5.1.1-2 |  |  |  |  |  |  |  |

Table 8.5.1.1-2: E-UTRA CA_42C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_42C | 5, 10, 15, 20 | 20 | 40 | 0 |
|  | 20 | 5, 10, 15 |  |  |
|  | 10, 15, 20 | 20 | 40 | 1 |
|  | 20 | 10, 15 |  |  |

8.5.1.2	Co-existence studies for LTE-A UL CA_1A-21A and DL CA_1A-21A-28A-42A-42A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.5.1.2-1 based on a certain operater’s actual spectrum holdings .

Table 8.5.1.2-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 1940 | 1960 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 3880 | 3920 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 5820 | 5880 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 477.1 | 512.1 | 3387.9 | 3422.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 935.8 | 985.8 | 2417.1 | 2472.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4835.8 | 4885.8 | 5327.9 | 5382.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2383.7 | 2448.7 | 4357.1 | 4432.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6283.7 | 6348.7 | 7267.9 | 7342.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1024.2 | 954.2 | 6775.8 | 6845.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6392.1 | 6297.1 | 3911.6 | 3831.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9207.9 | 9302.9 | 7731.6 | 7811.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2984.2 | 2894.2 | 508.7 | 423.7 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8715.8 | 8805.8 | 8223.7 | 8308.7 |

The 2nd IMD fall into the own 4th Rx Band 42. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21. Then, there is no IMD problems to the 3rd, 4th & 5th own Rx frequency of Band 28, Band 42 and Band 42.

Hence, there were no harmonics/IMD problems to the 3rd, 4th & 5th own Rx frequency of Band 28, Band 42 and Band 42.

8.5.1.3	Co-existence studies for LTE-A UL CA_1A-28A and DL CA_1A-21A-28A-42A-42A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.5.1.3-1 based on a certain operater’s actual spectrum holdings.

Table 8.5.1.3-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 728 | 738 | 1940 | 1960 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1456 | 1476 | 3880 | 3920 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2184 | 2214 | 5820 | 5880 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1202 | 1232 | 2668 | 2698 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 504 | 464 | 3142 | 3192 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3396 | 3436 | 4608 | 4658 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 224 | 274 | 5082 | 5152 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4124 | 4174 | 6548 | 6618 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2464 | 2404 | 5336 | 5396 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7112 | 7022 | 1012 | 952 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8488 | 8578 | 4852 | 4912 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4424 | 4344 | 1666 | 1736 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7276 | 7356 | 6064 | 6134 |

The 2nd harmonics fall into the own Rx Band 21. However, RAN4 can consider specicific holding frequency bands when study the harmonics analysis for the related CA band combinations including Band 21. Also the 3rd IMD fall into the 4th own Rx frequency of Band 42. But the IMD impact already covered in 3DL CA_1A-28A-42A with 2UL CA_1A-28A.

Hence, there were no harmonics/IMD problems to the 3rd, 4th & 5th own Rx frequency of Band 21, Band 42 and Band 42.

8.5.1.4	Co-existence studies for LTE-A UL CA_ 1A-42A and DL CA_1A-21A-28A-42A-42A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.5.1.4-1 based on a certain operater’s actual spectrum holdings.

Table 8.5.1.4-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1940 | 1960 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3880 | 3920 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5820 | 5880 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1520 | 1580 | 5420 | 5480 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 360 | 440 | 5000 | 5100 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7360 | 7440 | 8900 | 9000 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2300 | 2400 | 8480 | 8620 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9300 | 9400 | 12380 | 12520 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3160 | 3040 | 10840 | 10960 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12140 | 11960 | 4360 | 4240 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15860 | 16040 | 11240 | 11360 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6680 | 6520 | 1080 | 1220 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14320 | 14480 | 12780 | 12920 |

The 2nd IMD fall into the own Rx Band 21. However, RAN4 can consider specicific holding frequency bands when study the IMD analysis for the related CA band combinations including Band 21.

Hence, there were no harmonics/IMD problems to the 3rd, 4th & 5th own Rx frequency of Band 21, Band 28 and Band 42.

8.5.1.5	Co-existence studies for LTE-A UL CA_ 21A-28A and DL CA_1A-21A-28A-42A-42A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.5.1.5-1 based on a certain operater’s actual spectrum holdings.

Table 8.5.1.5-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 728 | 738 | 1447.9 | 1462.9 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1456 | 1476 | 2895.8 | 2925.8 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2184 | 2214 | 4343.7 | 4388.7 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 709.9 | 734.9 | 2175.9 | 2200.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 6.9 | 28.1 | 2157.8 | 2197.8 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2903.9 | 2938.9 | 3623.8 | 3663.8 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 721.1 | 766.1 | 3605.7 | 3660.7 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3631.9 | 3676.9 | 5071.7 | 5126.7 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1469.8 | 1419.8 | 4351.8 | 4401.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5123.6 | 5053.6 | 1504.1 | 1449.1 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 6519.6 | 6589.6 | 4359.9 | 4414.9 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2932.7 | 2867.7 | 681.8 | 741.8 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5799.7 | 5864.7 | 5079.8 | 5139.8 |

The 3rd harmonics from Band 28 transmission will be interference to the Band 1. Also the 2nd & 3rd IMDs fall into the 3rd own Rx frequency of Band 1. However, RAN4 can consider specicific holding frequency bands when study the IMD/harmonics analysis for the related CA band combinations including Band 21.

Hence, there was no harmonics/IMDs problem to the 3rd, 4th & 5th own Rx frequency of Band 1, Band 42 and Band 42.

8.5.1.6	Co-existence studies for LTE-A UL CA_ 21A-42A and DL CA_1A-21A-28A-42A-42A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.5.1.6-1 based on a certain operater’s actual spectrum holdings.

Table 8.5.1.6-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2017.1 | 2072.1 | 4927.9 | 4982.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 624.2 | 554.2 | 5497.1 | 5592.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6375.8 | 6445.8 | 8407.9 | 8502.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 823.7 | 908.7 | 8977.1 | 9112.1 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7823.7 | 7908.7 | 11887.9 | 12022.9 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 4144.2 | 4034.2 | 9855.8 | 9965.8 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12632.1 | 12457.1 | 2371.6 | 2271.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15367.9 | 15542.9 | 9271.6 | 9371.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7664.2 | 7514.2 | 2571.3 | 2696.3 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13335.8 | 13485.8 | 11303.7 | 11428.7 |

The 2nd IMD fall into the own Rx frequency of Band 1. However, RAN4 can consider specicific holding frequency bands when study the harmonics/IMDs analysis for the related CA band combinations including Band 21.

Hence, there were no harmonics/IMD problems to the 3rd, 4th & 5th own Rx frequency of Band 1, Band 28 and Band 42.

8.5.1.7	Co-existence studies for LTE-A UL CA_28A-42A and DL CA_1A-21A-28A-42A-42A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.5.1.7-1 based on a certain operater’s actual spectrum holdings.

Table 8.5.1.7-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 728 | 738 | 3480 | 3520 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1456 | 1476 | 6960 | 7040 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2184 | 2214 | 10440 | 10560 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2742 | 2792 | 4208 | 4258 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2064 | 2004 | 6222 | 6312 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4936 | 4996 | 7688 | 7778 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1336 | 1266 | 9702 | 9832 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5664 | 5734 | 11168 | 11298 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 5584 | 5484 | 8416 | 8516 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13352 | 13182 | 528 | 608 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14648 | 14818 | 6392 | 6472 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9104 | 8964 | 4746 | 4856 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11896 | 12036 | 9144 | 9254 |

The 3rd harmonics by Band 28 fall into the own Rx Band 1. Aslo the 2nd harmonics by Band 28 fall into the own Rx Band 21. But the harmonics impact already covered in 2DL/1UL CA_1A-28A and 2DL/1UL CA_21A-28A. The 3rd IMD fall into the 3rd own Rx frequency of Band 1. But the impact also covered in 3DL CA_1A-28A-42A with 2UL CA_28A-42A.

Hence, there were no harmonics/IMD problems to the 3rd, 4th & 5th own Rx frequency of Band 1, Band 21 and Band 42.

8.5.1.8	MSD

When uplink CA configurations CA_1A-21A is paired with downlink CA configuration CA_1A-21A-28A-42C there is no interference components from 2 uplink operation which would interfere the 3rd, 4th & 5th downlink of the Band 28 and Band 42 and Band 42.

When uplink CA configurations CA_1A-28A is paired with downlink CA configuration CA_1A-21A-28A-42C there is no interference components from 2 uplink operation which would interfere the 3rd, 4th & 5th downlink of the Band 21 and Band 42 and Band 42.

When uplink CA configurations CA_1A-42A is paired with downlink CA configuration CA_1A-21A-28A-42C there is no interference components from 2 uplink operation which would interfere the 3rd, 4th & 5th downlink of the Band 21 and Band 28 and Band 42.

When uplink CA configurations CA_21A-28A is paired with downlink CA configuration CA_1A-21A-28A-42C there is no interference components from 2 uplink operation which would interfere the 3rd, 4th & 5th downlink of the Band 1 and Band 42 and Band 42.

When uplink CA configurations CA_21A-42A is paired with downlink CA configuration CA_1A-21A-28A-42C there is no interference components from 2 uplink operation which would interfere the 3rd, 4th & 5th downlink of the Band 1 and Band 28 and Band 42.

When uplink CA configurations CA_28A-42A is paired with downlink CA configuration CA_1A-21A-28A-42C there is no interference components from 2 uplink operation which would interfere the 3rd, 4th & 5th downlink of the Band 1 and Band 21 and Band 42.

8.5.1.9	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

8.6	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 5 and Band 7 and Band 7 with 2 ULs

8.6.1	List of specific combination issues

8.6.1.1	Channel bandwidths per operating band for CA

Table 8.6.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-3A-5A-7A-7A | CA_1A-3A or CA_1A-5A or CA_1A-7A or CA_3A-5A or CA_3A-7A or CA_5A-7A | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 3 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 3 in Table 8.6.1.1-2 |  |  |  |  |  |  |  |

Table 8.6.1.1-2: E-UTRA CA_7A-7A configurations and bandwidth combination sets for intra-band CA

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRACA configuration | Uplink CA configurations (NOTE 1) | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_7A-7A | - | 5 | 15 |  |  | 40 | 0 |
|  |  | 10 | 10, 15 |  |  |  |  |
|  |  | 15 | 15, 20 |  |  |  |  |
|  |  | 20 | 20 |  |  |  |  |
|  |  | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 40 | 1 |
|  |  | 5, 10, 15, 20 | 5, 10 |  |  | 30 | 2 |
|  |  | 10, 15, 20 | 10, 15, 20 |  |  | 40 | 3 |

8.6.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-5A-7A-7A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.6.1.2-1

Table 8.6.1.2-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 135 | 270 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1440 | 1650 | 2055 | 2250 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5340 | 5550 | 5550 | 5745 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3150 | 3435 | 3975 | 4230 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7050 | 7335 | 7470 | 7725 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 540 | 270 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6210 | 5895 | 5220 | 4860 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9390 | 9705 | 8760 | 9120 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2520 | 2190 | 1515 | 1170 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9180 | 9510 | 8970 | 9315 |

From the harmonics/IMDs analysis table, there was no interference to the own 3rd, 4th and 5th Rx Band 5, Band 7 and Band 7.

8.6.1.3	Co-existence studies for LTE-A UL CA_1A-5A and DL CA_1A-3A-5A-7A-7A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.6.1.3-1

Table 8.6.1.3-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1071 | 1156 | 2744 | 2829 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 332 | 222 | 2991 | 3136 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3568 | 3678 | 4664 | 4809 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 492 | 627 | 4911 | 5116 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4392 | 4527 | 6584 | 6789 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2312 | 2142 | 5488 | 5658 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7096 | 6831 | 1476 | 1316 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8504 | 8769 | 5216 | 5376 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4292 | 4062 | 1293 | 1488 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7408 | 7638 | 6312 | 6507 |

From the harmonics/IMDs analysis table, there was no interference to the own 3rd, 4th and 5th Rx Band 3, Band 7 and Band 7.

8.6.1.4	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-3A-5A-7A-7A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.6.1.4-1

Table 8.6.1.4-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 520 | 650 | 4420 | 4550 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1270 | 1460 | 3020 | 3220 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6340 | 6530 | 6920 | 7120 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3190 | 3440 | 5520 | 5790 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8260 | 8510 | 9420 | 9690 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1300 | 1040 | 8840 | 9100 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8360 | 8020 | 5420 | 5110 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11920 | 12260 | 10180 | 10490 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3870 | 3540 | 940 | 620 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11340 | 11670 | 10760 | 11080 |

The 5th IMDs fall into the own Rx frequency of Band 5. But these IMD problems also will be covered in 3DL_CA_1A-5A-7A with 2UL_CA_1A-7A.

Hence, there was no interference to the own 3rd, 4th and 5th Rx Band 3, Band 5 and Band 7.

8.6.1.5	Co-existence studies for LTE-A UL CA_3A-5A and DL CA_1A-3A-5A-7A-7A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.6.1.5-1.

Table 8.6.1.5-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 861 | 961 | 2534 | 2634 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 137 | 12 | 2571 | 2746 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3358 | 3483 | 4244 | 4419 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 687 | 837 | 4281 | 4531 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4182 | 4332 | 5954 | 6204 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1922 | 1722 | 5068 | 5268 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6316 | 5991 | 1686 | 1511 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7664 | 7989 | 5006 | 5181 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3707 | 3432 | 873 | 1098 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6778 | 7053 | 5892 | 6117 |

The 2nd & 3rd IMDs fall into the own Rx frequency of Band 7. But these IMD impacts will be covered in 3DL_CA_3A-5A-7A with 2UL CA_3A-5A.

Hence, there was no interference to the own 3rd, 4th and 5th Rx Band 1, Band 7 and Band 7.

8.6.1.6	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_1A-3A-5A-7A-7A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.6.1.6-1.

Table 8.6.1.6-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 715 | 860 | 4210 | 4355 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 850 | 1070 | 3215 | 3430 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5920 | 6140 | 6710 | 6925 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2560 | 2855 | 5715 | 6000 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7630 | 7925 | 9210 | 9495 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1720 | 1430 | 8420 | 8710 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8570 | 8215 | 4640 | 4270 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11710 | 12065 | 9340 | 9710 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4290 | 3930 | 355 | 10 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10920 | 11280 | 10130 | 10495 |

The 3rd IMD fall into the own Rx frequency of Band 5. But the IMD problem was covered in 3DL_CA_3A-5A-7A with 2UL_CA_3A-7A. Also the 4th IMD fall into the own Rx frequency of Band 7. But the IMD problem was covered in 2DL/2UL_CA_3A-7A.

Hence, there was no interference to the own 3rd, 4th and 5th Rx Band 1, Band 5 and Band 7.

8.6.1.7	Co-existence studies for LTE-A UL CA_5A-7A and DL CA_1A-3A-5A-7A-7A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.6.1.7-1.

Table 8.6.1.7-1: Harmonic and IMDs analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1651 | 1746 | 3324 | 3419 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 922 | 802 | 4151 | 4316 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4148 | 4268 | 5824 | 5989 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 98 | 47 | 6651 | 6886 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4972 | 5117 | 8324 | 8559 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3492 | 3302 | 6648 | 6838 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9456 | 9151 | 896 | 726 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10824 | 11129 | 5796 | 5966 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6062 | 5802 | 2453 | 2668 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9148 | 9408 | 7472 | 7687 |

The 5th IMD fall into the own Rx frequency of Band 7. But the IMD problem was covered in 2DL/2UL_CA_5A-7A.

Hence, there was no interference to the own 3rd, 4th and 5th Rx Band 1, Band 3 and Band 7.

8.6.1.8	MSD

When uplink CA configurations CA_1A-3A is paired with downlink CA configuration CA_1A-3A-5A-7A-7A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 5 and Band 7.

When uplink CA configurations CA_1A-5A is paired with downlink CA configuration CA_1A-3A-5A-7A-7A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 3 and Band 7 and Band 7.

When uplink CA configurations CA_1A-7A is paired with downlink CA configuration CA_1A-3A-5A-7A-7A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 3 and Band 5 and Band 7.

When uplink CA configurations CA_3A-5A is paired with downlink CA configuration CA_1A-3A-5A-7A-7A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 1 and Band 7 and Band 7.

When uplink CA configurations CA_3A-7A is paired with downlink CA configuration CA_1A-3A-5A-7A-7A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 1 and Band 5 and Band 7.

When uplink CA configurations CA_5A-7A is paired with downlink CA configuration CA_1A-3A-5A-7A-7A there is no interference components from 2 uplink operation which would interfere the 3rd & 4th downlink of the Band 1 and Band 3 and Band 7.

8.6.1.9	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

Annex A:

Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2016-02 | 3GPP RAN4#78 | R4-161111 |  |  | TR36.714-00-02 Skeleton for xDL/2UL CA | N/A | 0.0.1 |
| 2016-04 | 3GPP RAN4#78BIS | R4-161955 |  |  | Approved Text proposal in RAN4 #78: R4-160557, “TP on the Harminic/IMD analysis table for xDL/2UL CA band combinations” R4-160524, “Work plan for xDL/2UL CA” | 0.0.1 | 0.1.0 |
| 2016-05 | 3GPP RAN4#79 | R4-163679 |  |  | Approved Text proposal in RAN4 #78BIS: R4-161956, “TP for general part for xDL/2UL CA” R4-162946, “TP on the operating band and coexistence analysis for 3DL/2UL inter-band CA in rel-14” R4-162947, “TP on the operating band and coexistence analysis for 4DL/2UL inter-band CA in rel-14” R4-161853, “TP for TR 36.714-00-02: LTE Advanced Carrier Aggregation of Band Combination (1+42: 3DL/2UL)” R4-161878, “TP for TR 36.714-00-02: LTE Advanced Carrier Aggregation of Band Combination (19+42: 3DL/2UL)“ R4-161879, “TP for TR 36.714-00-02: LTE Advanced Carrier Aggregation of Band Combination (21+42: 3DL/2UL)“ R4-162945, “TP on the self-desense analysis results for xDL/2UL CA band combinations” | 0.1.0 | 0.2.0 |
| 2016-08 | 3GPP RAN4#80 | R4-165756 |  |  | Approved Text proposal in RAN4 #79: R4-164698, “MSD test configurations for xDL/2UL CA” RP-160748, “Revised WID for xDL/UL inter-band CA with x=3,4,5 in Rel-14” | 0.2.0 | 0.3.0 |
| 2016-10 | 3GPP RAN4#80BIS | R4-168183 |  |  | Approved Text proposal in RAN4 #80: R4-166912, "TP for MSD level for 3DLs/2ULs inter-band CA in Rel-14," R4-165238, "TP for TR 36.714-00-02 CA_3A-42C," R4-167135, "Next level fallback configurations for xDL 2UL," | 0.3.0 | 0.4.0 |
| 2016-11 | 3GPP RAN4#81 | R4-1609783 |  |  | Approved Text Proposals in RAN4 #80BIS R4-168185, “TP on harmonics/IMD analysis for new CA band combinations for 3DL/2UL CA,” R4-168189, “TP on harmonics/IMD analysis for new CA band combinations for 4DL/2UL CA,” R4-168190, “TP on harmonics/IMD analysis for new CA band combinations for 5DL/2UL CA,” R4-168191, “Self-desense analysis for xDL/2UL CA band combinations in Rel-14,” R4-168857, “Test configuration for new CA band combinations in xDL/2UL CA basket WI,” R4-168858, “TP on the general part for new xDL/2UL CA,” R4-168231, “WF on MSD due to IMD problems for xDL/2UL CA with Band 46,” R4-168860, “TP for TR36.714-00-02 on MSD for 3+7+8 3DL 2UL CA,” R4-168853, “WF on Note6 in the uplink CA,” | 0.4.0 | 0.5.0 |
| 2017-02 | 3GPP RAN4#82 | R4-1701195 |  |  | Approved Text Proposals in RAN4 #81 & RAN plenary #74 R4-1610771, “TP on MSD results for xDL/2UL CA with B46” R4-1609969, “TP for TR36.714-00-02: xDL/2UL CA analysis results” R4-1610007, “TP for TR 36.714-00-02: MSD for CA_3A-7A-8A 3DL with CA_3A-7A and CA_3A-8A 2UL CA” RP-162036, “Revised WID for xDL/UL inter-band CA with x=3,4,5 in Rel-14” | 0.5.0 | 0.6.0 |
| 2017-04 | 3GPP RAN4#82BIS | R4-1703282 |  |  | Approved Text Proposals in RAN4 #82 & RAN plenary #75 meetings R4-1701197, “TP on operating band and coexistence analysis for xDL/2UL CA” R4-1701200, “MSD test configuration for xDL/2UL CA with self-desense problems in Rel-14” R4-1702446, “WF on the MSD test principles for xDL/2UL CA” R4-1701198, “TP on self-desnese analysis results” R4-1700931, “TP for TR 36.714-00-02: Coexistence studies and MSD for CA_3A-3A-8A 3DL with CA_3A-8A 2UL” R4-1702248, “TP on summary of MSD for xDL/2UL CA” RP-170128, “Revised WID for xDL/2UL inter-band CA with x=3,4,5 in Rel-14” | 0.6.0 | 0.7.0 |
| 2017-05 | 3GPP RAN4#83 | R4-1704820 |  |  | Approved Text Proposals in RAN4 #82BIS meeting R4-1704048, “TP on operating band and coexistence analysis for xDL/2UL CA” R4-1703284 “TP on self-desnese analysis results” R4-1703285, “MSD test configuration for xDL/2UL CA with self-desense problems in Rel-14” R4-1703549, “TP for TR 36.714-00-02: operating bands, channel bandwidths for CA_4DL_3C-7C_2UL_3A-7A” R4-1703550, “TP for TR 36.714-00-02: Coexistence studies UE requirements relaxation for CA_4DL_3C-7C_2UL_3A-7A” | 0.7.0 | 0.8.0 |