3GPP TS G-RAN WG4 Meeting #83		R4-1704923

# Hangzhou, China, 15 - 19 May, 2017

Agenda item:	7.27.2.1

Source:	CATT

Title:	PDSCH demodulation performance of Enhanced CRS-IM

Document for:	Discussion

Introduction

In RAN #73 meeting, the “LTE Enhanced CRS and SU-MIMO Interference Mitigation Performance Requirements” WI was initiated [1]. In the previous RAN4#82bis meeting, further agreements on the target scenarios and reference receivers for the Stage 1 investigations were provided in [2]. And it is proposed to provide simulation results to identify target scenarios for requirements definition and confirm reference receiver feasibility. In this contribution, we provide the simulation results based on the agreed simulation assumptions [3] for PDSCH on enhanced CRS-IM for both FDD and TDD. The detailed simulation assumptions are attached in the Annex.

Simulation results of PDSCH for FDD

In the way forward [3], the agreed PDSCH non colliding test cases are proposed as shown in Table 1.

| Test | Duplexing | TM | CRS pattern | Number of UE RX chains | Number of CRS APs |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Serv. cell | Interf. cells |
| 1a | FDD | TM4 | Non Colliding | 2 | 4 | 4 |
| 2a |  | TM4 | Non Colliding | 4 | 2 | 2 |
| 3a |  | TM9 | Non Colliding | 4 | 2 | 2 |
| 4a |  | TM4 | Non Colliding | 4 | 4 | 4 |

Table 1.  PDSCH test cases of FDD mode

Rank + FRC:

| Test cases | TM4 |  | TM9 |
| --- | --- | --- | --- |
|  | 2 CRS APs | 4 CRS APs | 2 CRS APs |
| Rank 1 + 64QAM | R.35 | R.X | R.Y |

Interference signal transmission parameters:

80%/20% rank 1/2 probability

Option 1: Interference loading = 20%, INR1 = 10.45 dB, INR2 = 4.6 dB

Reference receivers are used for the evaluation:

- Receiver #1: LMMSE-IRC (baseline)

- Receiver #2: LMMSE-IRC + 1 Cell CRS-IM  (Full complexity two or four ports CRS-IM processing)

Test Case1a: TM4  4 CRS APs + 2 RX chains

Figure 1. throughput performance of TM4  4 CRS APs + 2 RX chains scenario

Test Case2a: TM4  2 CRS APs + 4 RX chains

Figure 2. throughput performance of TM4  2 CRS APs + 4 RX chains scenario

Test Case3a: TM9  2 CRS APs + 4 RX chains

Figure 3. throughput performance of TM9  2 CRS APs + 4 RX chains scenario

Test Case4a: TM4  4 CRS APs + 4 RX chains

Figure 4. throughput performance of TM4  4 CRS APs + 4 RX chains scenario

From the above link-level simulation results of the non colliding scenarios, the performance gains of the LMMSE-IRC+CRS-IM receiver vs. the LMMSE-IRC receiver are shown in table 2&3.

Table 2. the alignment result of the performance gain of the SNRs  @ 70%  of  the maximum throughput (FDD)

| Test Case (non colliding) | LMMSE-IRC(dB) | LMMSE-IRC+CRS-IM(dB) | Gains of CRS-IM(dB) |
| --- | --- | --- | --- |
| PDSCH Test1a | 13.6 | 11.9 | 1.7 |
| PDSCH Test2a | 10.41 | 8.85 | 1.56 |
| PDSCH Test3a | 10.78 | 9.62 | 1.16 |
| PDSCH Test4a | 10.4 | 7.9 | 2.5 |

Table 3. the impairment result of the performance gain of the SNRs  @ 70%  of  the maximum throughput (FDD)

| Test Case (non colliding) | LMMSE-IRC(dB) | LMMSE-IRC+CRS-IM(dB) | Gains of CRS-IM(dB) |
| --- | --- | --- | --- |
| PDSCH Test1a | 15.6 | 13.9 | 1.7 |
| PDSCH Test2a | 12.41 | 10.85 | 1.56 |
| PDSCH Test3a | 12.78 | 11.62 | 1.16 |
| PDSCH Test4a | 12.4 | 9.9 | 2.5 |

Observation 1:

CRS-IM processing can achieve testable performance gain (about 1.2-3dB) over baseline receiver under various scenarios for FDD mode.

Simulation results of PDSCH for TDD

In the way forward [3], the agreed PDSCH test cases are proposed as shown in Table 4

| Test | Duplexing | TM | CRS pattern | Number of UE RX chains | Number of CRS APs |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Serv. cell | Interf. cells |
| 1b | TDD | TM4 | Non Colliding | 2 | 4 | 4 |
| 2b |  | TM4 | Non Colliding | 4 | 2 | 2 |
| 3b |  | TM9 | Non Colliding | 4 | 2 | 2 |
| 4b |  | TM4 | Non Colliding | 4 | 4 | 4 |

Table 4.  PDSCH test cases of TDD mode

Rank + FRC:

| Test cases | TM4 |  | TM9 |
| --- | --- | --- | --- |
|  | 2 CRS APs | 4 CRS APs | 2 CRS APs |
| Rank 1 + 64QAM | R.35 | R.X-TDD | R.Y-TDD |

Interference signal transmission parameters:

80%/20% rank 1/2 probability

Option 1: Interference loading = 20%, INR1 = 10.45 dB, INR2 = 4.6 dB

Reference receivers are used for the evaluation:

- Receiver #1: LMMSE-IRC (baseline)

- Receiver #2: LMMSE-IRC + 1 Cell CRS-IM  (Full complexity two or four ports CRS-IM processing)

Test Case1b: TM4  4 CRS APs + 2 RX chains

Figure 5. throughput performance of TM4  4 CRS APs + 2 RX chains scenario

Test Case2b: TM4  2 CRS APs + 4 RX chains

Figure 6. throughput performance of TM4  2 CRS APs + 4 RX chains scenario

Test Case3b: TM9  2 CRS APs + 4 RX chains

Figure 7. throughput performance of TM9  2 CRS APs + 4 RX chains scenario

Test Case4b: TM4  4 CRS APs + 4 RX chains

Figure 8. throughput performance of TM4  4 CRS APs + 4 RX chains scenario

From the above link-level simulation results of the non colliding scenarios, the performance gains of the LMMSE-IRC+CRS-IM receiver vs. the LMMSE-IRC receiver are shown in table 5&6.

Table 5. the alignment result of the performance gain of the SNRs  @ 70%  of  the maximum throughput (TDD)

| Test Case | LMMSE-IRC(dB) | LMMSE-IRC+CRS-IM(dB) | Gains of CRS-IM(dB) |
| --- | --- | --- | --- |
| PDSCH Test1b | 11.96 | 13.91 | 1.95 |
| PDSCH Test2b | 8.91 | 10.52 | 1.61 |
| PDSCH Test3b | 9.49 | 11.07 | 1.58 |
| PDSCH Test4b | 7.59 | 10.52 | 2.93 |

Table 6. the impairment result of the performance gain of the SNRs  @ 70%  of  the maximum throughput (TDD)

| Test Case | LMMSE-IRC(dB) | LMMSE-IRC+CRS-IM(dB) | Gains of CRS-IM(dB) |
| --- | --- | --- | --- |
| PDSCH Test1b | 13.96 | 15.91 | 1.95 |
| PDSCH Test2b | 10.91 | 12.52 | 1.61 |
| PDSCH Test3b | 11.49 | 13.07 | 1.58 |
| PDSCH Test4b | 9.59 | 12.52 | 2.93 |

Observation 2:

CRS-IM processing can achieve testable performance gain (about 1.5-3dB) over baseline receiver under various scenarios for TDD mode.

Conclusions

In this contribution, we provide our simulation results (including the alignment and the impairment results) for PDSCH demodulation on enhanced CRS-IM. And according to the simulation results, the observations are given as below:

Observation 1:

CRS-IM processing can achieve testable performance gain (about 1.2-3dB) over baseline receiver under various scenarios for FDD mode.

Observation 2:

CRS-IM processing can achieve testable performance gain (about 1.5-3dB) over baseline receiver under various scenarios for TDD mode.

# References

- RP-161877 “New Work Item Proposal: Enhanced CRS and SU-MIMO Interference Mitigation Performance Requirements for LTE”, Intel Corporation, Huawei, LGE, RAN #73, September 2016

- R4-1704230 “WF on Enhanced CRS-IM Performance Requirements”, Intel Corporation, LGE, CATT, RAN4 #82bis, April  2017

- R4-1704231 “Simulation assumptions for Enhanced CRS-IM”, Intel Corporation, LGE, CATT, RAN4 #82bis, April  2017

# Annex-simulation assumptions

Table7 includes a list of the test cases to be used for performance requirements definition of the CRS-IM enhancements for PDSCH. The PDSCH simulation assumptions are provided in Table 8. In Table 9, 10, 11 and 12 FRCs for PDSCH tests are presented.

Table 7. PDSCH test cases

| Test | Duplexing | TM | CRS pattern | Number of UE RX chains | Number of CRS APs |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Serv. cell | Interf. cells |
| 1a | FDD | TM4 | Non Colliding | 2 | 4 | 4 |
| 2a |  | TM4 | Non Colliding | 4 | 2 | 2 |
| 3a |  | TM9 | Non Colliding | 4 | 2 | 2 |
| 4a |  | TM4 | Non Colliding | 4 | 4 | 4 |
| 1b | TDD | TM4 | Non Colliding | 2 | 4 | 4 |
| 2b |  | TM4 | Non Colliding | 4 | 2 | 2 |
| 3b |  | TM9 | Non Colliding | 4 | 2 | 2 |
| 4b |  | TM4 | Non Colliding | 4 | 4 | 4 |

Table 8. PDSCH simulation assumptions

| Parameter |  |  | Value |  |
| --- | --- | --- | --- | --- |
| Duplexing |  |  | FDD, TDD |  |
| System bandwidth |  |  | 10 MHz |  |
| Tx EVM |  |  | 6% |  |
| Antenna models |  |  | Tests 1: 4x2 low correlation Tests 2, 3: 2x4 low correlation Test 4: 4x4 low correlation |  |
| Cell ID pattern |  |  | Non-colliding CRS: - Serving cell: 0 - Interference cell 1: 1 - Interference cell 2: 6 |  |
| Interference power profile |  |  | Interference cell 1: I1/Noc = 10.45 dB Interference cell 2: I2/Noc = 4.6 dB |  |
| Network synchronization in time |  |  | All cells are synchronous |  |
|  |  |  | Time-delay wrt. serving cell |  |
|  |  |  | 1st interfering cell | 2nd interfering cell |
|  |  |  | 3 µs | -1 µs |
| Network synchronization in frequency |  |  | Frequency shift wrt. serving cell |  |
|  |  |  | 1st interfering cell | 2nd interfering cell |
|  |  |  | 300 Hz | -100 Hz |
| Channel model |  |  | EVA-5Hz for all links. Use different channel seed for between cells. |  |
| Number of control region OFDM symbols (CFI) |  |  | CFI = 2. Same value in all cells |  |
| HARQ modelling |  |  | Maximum 4 HARQ retransmissions |  |
| TDD parameters |  |  | Uplink downlink Configuration: 1 Special subframe configuration: 4 |  |
| CSI-RS configuration for serving cell (for Test 3) |  |  | FDD: Number of CSI-RS ports : 2 CSI-RS periodicity and subframe offset : 5/2 CSI reference signal configuration : 0 Zero-power CSI-RS configuration : 3 /0001000000000000 TDD: Number of CSI-RS ports : 2 CSI-RS periodicity and subframe offset : 5/4 CSI reference signal configuration : 8 Zero-power CSI-RS configuration : 4 /0010000000000000 |  |
| Desired PDSCH transmission parameters | Resource allocation |  | PDSCH is scheduled in subframes - 0-4, 6-9 for FDD - 0, 1, 4, 6, 9 for TDD TM4: 50 PRB for all subframes TM9: - FDD: 50 PRB resource allocation for all SFs 1-4, 6-9; 41 PRB resource allocation for all SF 0 - TDD: 50 PRB resource allocation for all SFs 4, 9; 41 PRB resource allocation for all SF 0 |  |
|  | PMI |  | TM4: Follow PMI, reporting mode PUSCH 3-1 TM9: Random PMI with 1 PRG / 1 TTI granularity |  |
| Physical channels and signal explicitly transmitted in interfering cells |  |  | CRS PDSCH PSS/SSS/PBCH |  |
| Interference PDSCH transmission parameters |  | Resource allocation | Random full band (50PRB) on/off model, proportional to the average resource utilization in the interfering cells. Average resource utilization in the interfering cells: 20% |  |
|  |  | Rank | Randomly changing rank per allocated subband from subframe to subframe: 80% rank-1, 20% rank-2 |  |
|  |  | Interference model | Based on TS 36.101 B.5.3 and B.5.4 |  |
|  |  | TM | Same as serving cell TM |  |
| Receivers |  |  | Baseline: LMMSE-IRC Enhanced reference receiver structures: LMMSE-IRC + 1 cell CRS-IM |  |

Table 9. FDD FRCs for TM4 PDSCH tests

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
|  |  | Test 1a | Test 2a | Test 4a |
| Reference channel |  | R.36 | R.35 | R.X |
| Channel bandwidth | MHz | 10 | 10 | 10 |
| Allocated resource blocks |  | 50 | 50 | 50 |
| Allocated number of PDCCH symbols |  | 2 | 2 | 2 |
| Allocated subframes per Radio Frame |  | 9 | 9 | 9 |
| Modulation |  | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 0.5 | 0.5 | 0.55 |
| Information Bit Payload |  |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 18336 | 19848 | 21384 |
| For Sub-Frame 5 | Bits | n/a | n/a | n/a |
| For Sub-Frame 0 | Bits | 18336 | 18336 | 19848 |
| Number of Code Blocks |  |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 3 | 4 | 4 |
| For Sub-Frame 5 | Bits | n/a | n/a | n/a |
| For Sub-Frame 0 | Bits | 3 | 3 | 4 |
| Binary Channel Bits |  |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 38400 | 39600 | 38400 |
| For Sub-Frame 5 | Bits | n/a | n/a | n/a |
| For Sub-Frame 0 | Bits | 36096 | 37152 | 36096 |
| Max. Throughput averaged over 1 frame | Mbps | 16.502 | 17.712 | 19.092 |
| UE Category |  | ≥ 2 | ≥ 2 | ≥ 2 |

Table 10. TDD FRCs for TM4 PDSCH tests

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
|  |  | Test 1b | Test 2b | Test 4b |
| Reference channel |  | R.36-TDD | R.35-TDD | R.X-TDD |
| Channel bandwidth | MHz | 10 | 10 | 10 |
| Allocated resource blocks |  | 50 | 50 | 50 |
| Uplink-Downlink Configuration |  | 1 | 1 | 1 |
| Allocated subframes per Radio Frame |  | 2+2 | 2+2 | 2+2 |
| Modulation |  | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 0.5 | 0.5 | 0.55 |
| Information Bit Payload |  |  |  |  |
| For Sub-Frames 4,9 | Bits | 18336 | 19848 | 21384 |
| For Sub-Frames 1,6 | Bits | 15840 | 15840 | 16992 |
| For Sub-Frame 5 | Bits | n/a | n/a | n/a |
| For Sub-Frame 0 | Bits | n/a | n/a | n/a |
| Number of Code Blocks |  |  |  |  |
| For Sub-Frames 4,9 | Bits | 3 | 4 | 4 |
| For Sub-Frames 1,6 | Bits | 3 | 3 | 3 |
| For Sub-Frame 5 | Bits | n/a | n/a | n/a |
| For Sub-Frame 0 | Bits | n/a | n/a | n/a |
| Binary Channel Bits |  |  |  |  |
| For Sub-Frames 4,9 | Bits | 38400 | 39600 | 38400 |
| For Sub-Frames 1,6 | Bits | 30768 | 31968 | 30768 |
| For Sub-Frame 5 | Bits | n/a | n/a | n/a |
| For Sub-Frame 0 | Bits | n/a | n/a | n/a |
| Max. Throughput averaged over 1 frame | Mbps | 6.835 | 7.138 | 7.675 |
| UE Category |  | ≥ 2 | ≥ 2 | ≥ 2 |

Table 11. FDD FRC for TM9 PDSCH test

| Parameter | Unit | Value |
| --- | --- | --- |
|  |  | Test 3a |
| Reference channel |  | R.Y |
| Channel bandwidth | MHz | 10 |
| Allocated resource blocks |  | 50 (Note 3) |
| Allocated subframes per Radio Frame |  | 9 |
| Modulation |  | 64QAM |
| Target Coding Rate |  | 0,51 |
| Information Bit Payload |  |  |
| For Sub-Frames 1,2,3,4 | Bits | 18336 |
| For Sub-Frames 6,7,8,9 | Bits | 18336 |
| For Sub-Frame 5 | Bits | n/a |
| For Sub-Frame 0 | Bits | 14688 |
| Number of Code Blocks |  |  |
| For Sub-Frames 1,2,3,4 | Bits | 3 |
| For Sub-Frames 6,7,8,9 | Bits | 3 |
| For Sub-Frame 5 | Bits | n/a |
| For Sub-Frame 0 | Bits | 3 |
| Binary Channel Bits |  |  |
| For Sub-Frames 1,4,6,9 | Bits | 36000 |
| For Sub-Frames 2,7 | Bits | 35400 |
| For Sub-Frames 3,8 | Bits | 34800 |
| For Sub-Frame 5 | Bits | n/a |
| For Sub-Frame 0 | Bits | 29520 |
| Max. Throughput averaged over 1 frame | Mbps | 16,138 |
| UE Category |  | ≥ 2 |

Table12. TDD FRC for TM9 PDSCH test

| Parameter | Unit | Value |
| --- | --- | --- |
|  |  | Test 3b |
| Reference channel |  | R.Y-TDD |
| Channel bandwidth | MHz | 10 |
| Allocated resource blocks |  | 50 (Note 3) |
| Uplink-Downlink Configuration |  | 1 |
| Allocated subframes per Radio Frame |  | 3+2 |
| Modulation |  | 64QAM |
| Target Coding Rate |  | 0.51 |
| Information Bit Payload |  |  |
| For Sub-Frames 4,9 (non CSI-RS subframe) | Bits | n/a |
| For Sub-Frames 4,9 | Bits | 18336 |
| For Sub-Frames 1,6 | Bits | 11832 |
| For Sub-Frame 5 | Bits | n/a |
| For Sub-Frame 0 | Bits | 14688 |
| Number of Code Blocks |  |  |
| For Sub-Frames 4,9 (non CSI-RS subframe) | Bits | n/a |
| For Sub-Frames 4,9 | Bits |  |
| For Sub-Frames 1,6 | Bits |  |
| For Sub-Frame 5 | Bits | n/a |
| For Sub-Frame 0 | Bits |  |
| Binary Channel Bits |  |  |
| For Sub-Frames 4,9 (non CSI-RS subframe) | Bits | n/a |
| For Sub-Frames 4,9 | Bits | 34200 |
| For Sub-Frames 1,6 | Bits | 23616 |
| For Sub-Frame 5 | Bits | n/a |
| For Sub-Frame 0 | Bits | 29520 |
| Max. Throughput averaged over 1frame | Mbps | 7.5024 |
| UE Category |  | ≥ 2 |