3GPP TS G-RAN WG4 Meeting #83		R4-1704924

# Hangzhou, China, 15 - 19 May, 2017

Agenda item:	7.27.2.2

Source:	CATT

Title:	PDCCH demodulation performance of Enhanced CRS-IM

Document for:	Discussion

Introduction

In RAN #73 meeting, the “LTE Enhanced CRS and SU-MIMO Interference Mitigation Performance Requirements” WI was initiated [1]. In the previous RAN4#82bis meeting, further agreements on the target scenarios and reference receivers for Stage 1 investigations were provided in [2], and PHICH test cases are deprioritized. In this contribution, we provide the simulation results based on the agreed simulation assumptions [3] for PDCCH on enhanced CRS-IM for both FDD and TDD. The detailed simulation assumptions are attached in the Annex.

Simulation results of PDCCH for FDD

In the way forward [3], the agreed PDCCH test cases are proposed as shown in Table 1.

Table 1. PDCCH  test cases

| Test | Physical channel | CRS pattern | Number of UE RX chains | Number of CRS APs |  | Test purpose |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Serv. cell | Interf. cell |  |
| 1 | PDCCH | Non Colliding | 2 | 4 | 4 | 4 CRS APs IM verification |
| 3 |  | Non Colliding | 4 | 4 | 4 | 4 CRS APs IM + 4 RX CRS IM verification |

Channel model:

EPA-5Hz, low correlation

EPA-5Hz, medium correlation

And the following reference receivers are used for the analysis:

- Receiver #1: LMMSE-MRC (baseline)

- Receiver #2: LMMSE-IRC

- Receiver #3: LMMSE-IRC + 1 Cell CRS-IM   (Full complexity two or four ports CRS-IM processing)

Test Case 1: PDCCH  4CRS APs + 2RX UE

Figure1. BLER performance of PDCCH Test Case 1

Test Case 3: PDCCH  4 CRS APs + 4 RX UE

- AL=2 , EPA-5Hz, low correlation

- AL=2 , EPA-5Hz, medium correlation

- AL=1 , EPA-5Hz, low correlation

- AL=1 , EPA-5Hz, medium correlation

Figure2. BLER performances of PDCCH Test Case 3

From the above link-level simulation results of the PDCCH, the performance gains of different receivers are shown in table 2&3.

Table2. the alignment result of the performance gains of SNR @ 1% BLER performance of PDCCH for FDD

| Test Case | MRC(dB) | IRC(dB) | IRC+CRS-IM(dB) | Gains of IRC(dB) | Gains of CRS-IM (dB) |
| --- | --- | --- | --- | --- | --- |
| PDCCH Test Case1 | 14.7 | 14.0 | 11.8 | 0.7 | 2.2 |
| PDCCH Test Case3 AL=2&Low | 10.6 | 7.4 | SNR=5.5 SINR= - 8.933 | 3.2 | 1.9 |
| PDCCH Test Case3 AL=2&Medium | 20.34 | 14.85 | SNR=13.55 SINR= - 0.884 | 5.5 | 1.30 |
| PDCCH Test Case3 AL=1&Low | 16.6 | 13.5 | SNR=12.0 SINR= - 2.434 | 3.1 | 1.5 |
| PDCCH Test Case3 AL=1&Medium | 27.50 | 21.56 | SNR=20.42 SINR= 5.986 | 5.94 | 1.15 |

Table3. the impairment result of the performance gains of SNR @ 1% BLER performance of PDCCH for FDD

| Test Case | MRC(dB) | IRC(dB) | IRC+CRS-IM(dB) | Gains of IRC(dB) | Gains of CRS-IM (dB) |
| --- | --- | --- | --- | --- | --- |
| PDCCH Test Case1 | 15.7 | 15.0 | 12.8 | 0.7 | 2.2 |
| PDCCH Test Case3 AL=2&Low | 11.6 | 8.4 | SNR=6.5 SINR= - 7.933 | 3.2 | 1.9 |
| PDCCH Test Case3 AL=2&Medium | 21.34 | 15.85 | SNR=14.55 SINR= 0.116 | 5.5 | 1.30 |
| PDCCH Test Case3 AL=1&Low | 17.6 | 14.5 | SNR=13.0 SINR= - 1.434 | 3.1 | 1.5 |
| PDCCH Test Case3 AL=1&Medium | 28.5 | 22.56 | SNR=21.42 SINR= 6.986 | 5.94 | 1.15 |

Observation 1:

For scenarios of 4CRS APs and 2RX chains, the testable performance gain of LMMSE-IRC+CRS-IM receiver (about 3dB) can be achieved over the LMMSE-MRC baseline receiver.

- The gain of CRS-IM processing is much higher than that of IRC processing.

For scenarios of 4CRS APs and 4RX chains, large performance gains of LMMSE-IRC+CRS-IM receiver (about 5dB) can be achieved over the LMMSE-MRC baseline receiver.

- The gain of the IRC processing is much higher in medium correlation channel model than that of CRS-IM processing in low correlation channel model

- For PDCCH AL=2 with low correlation channel model, the working SINR is too low for a practical test case requirement

- The gain of the IRC processing with AL=1 in low correlation channel scenario can get comparable testable gain.

Proposal 1:

For scenario of 4CRS APs and 4RX chains, AL1 with low correlation channel model test case is proposed based on LMMSE-IRC receiver with CRS-IM for FDD mode.

Simulation results of PDCCH for TDD

In the way forward [3], the agreed control channel test cases are proposed as shown in Table 4.

Table 4. PDCCH test cases

| Test | Physical channel | CRS pattern | Number of UE RX chains | Number of CRS APs |  | Test purpose |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Serv. cell | Interf. cell |  |
| 1 | PDCCH | Non Colliding | 2 | 4 | 4 | 4 CRS APs IM investigation |
| 3 |  | Non Colliding | 4 | 4 | 4 | 4 CRS APs IM + 4 RX CRS IM investigation |

Channel model:

EPA-5Hz, low correlation

EPA-5Hz, medium correlation

And the following reference receivers are used for the analysis:

- Receiver #1: LMMSE-MRC (baseline)

- Receiver #2: LMMSE-IRC

- Receiver #3: LMMSE-IRC + 1 Cell CRS-IM   (Full complexity two or four ports CRS-IM processing)

Test Case 1: PDCCH  4CRS APs + 2RX UE

Figure3. BLER performance of PDCCH Test Case 1

Test Case 3: PDCCH  4 CRS APs + 4 RX UE

- AL=2 , EPA-5Hz, low correlation

- AL=2 , EPA-5Hz, medium correlation

- AL=1 , EPA-5Hz, low correlation

- AL=1 , EPA-5Hz, medium correlation

Figure4. BLER performances of PDCCH Test Case 3

From the above link-level simulation results of the PDCCH, the performance gains of different receivers are shown in table 5&6.

Table5. the alignment result of the performance gains of SNR @ 1% BLER performance of PDCCH for TDD

| Test Case | MRC(dB) | IRC(dB) | IRC+CRS-IM(dB) | Gains of IRC(dB) | Gains of CRS-IM (dB) |
| --- | --- | --- | --- | --- | --- |
| PDCCH Test Case1 | 15.23 | 14.44 | 12.35 | 0.79 | 2.09 |
| PDCCH Test Case3 AL=2&Low | 11.14 | 7.91 | SNR=6.09 SINR= - 8.35 | 3.23 | 1.83 |
| PDCCH Test Case3 AL=2&Medium | 20.65 | 15.15 | SNR=13.90 SINR= - 0.533 | 5.5 | 1.24 |
| PDCCH Test Case3 AL=1&Low | 17.57 | 14.43 | SNR=13.02 SINR= - 1.414 | 3.14 | 1.41 |
| PDCCH Test Case3 AL=1&Medium | 28.68 | 22.54 | SNR=21.71 SINR= 7.276 | 6.14 | 0.84 |

Table 6. the impairment result of the performance gains of SNR @ 1% BLER performance of PDCCH for TDD

| Test Case | MRC(dB) | IRC(dB) | IRC+CRS-IM(dB) | Gains of IRC(dB) | Gains of CRS-IM (dB) |
| --- | --- | --- | --- | --- | --- |
| PDCCH Test Case1 | 16.23 | 15.44 | 13.35 | 0.79 | 2.09 |
| PDCCH Test Case3 AL=2&Low | 12.14 | 8.91 | SNR=7.09 SINR= - 7.35 | 3.23 | 1.83 |
| PDCCH Test Case3 AL=2&Medium | 21.65 | 16.15 | SNR=14.90 SINR= 0.467 | 5.5 | 1.24 |
| PDCCH Test Case3 AL=1&Low | 18.57 | 15.43 | SNR=14.02 SINR= - 0.414 | 3.14 | 1.41 |
| PDCCH Test Case3 AL=1&Medium | 29.68 | 23.54 | SNR=22.71 SINR= 8.276 | 6.14 | 0.84 |

Observation 2:

For scenarios of 4CRS APs and 2RX chains, the testable performance gain of LMMSE-IRC+CRS-IM receiver (about 3dB) can be achieved over the LMMSE-MRC baseline receiver.

- The gain of CRS-IM processing is more obvious than the gain of IRC processing.

For scenarios of 4CRS APs and 4RX chains, large performance gains of LMMSE-IRC+CRS-IM receiver (about 5dB) can be achieved over the LMMSE-MRC baseline receiver.

- The gain of the IRC processing is more obvious than the gain of CRS-IM processing.

- For PDCCH AL=2 with low correlation channel model, the working SINR is too low for a practical test case requirement

- The gain of the IRC processing with AL=1 in low correlation channel scenario can get comparable testable gain.

Proposal 2:

For scenario of 4CRS APs and 4RX chains, AL1 with low correlation channel model test case is proposed based on LMMSE-IRC receiver with CRS-IM for TDD mode.

Conclusions

In this contribution, we provide our simulation results for DL control channel demodulation on enhanced CRS-IM. And according to the simulation results, the observations are given as below:

Observation 1:

For scenarios of 4CRS APs and 2RX chains, the testable performance gain of LMMSE-IRC+CRS-IM receiver (about 3dB) can be achieved over the LMMSE-MRC baseline receiver.

- The gain of CRS-IM processing is much higher than that of IRC processing.

For scenarios of 4CRS APs and 4RX chains, large performance gains of LMMSE-IRC+CRS-IM receiver (about 5dB) can be achieved over the LMMSE-MRC baseline receiver.

- The gain of the IRC processing is much higher in medium correlation channel model than that of CRS-IM processing in low correlation channel model

- For PDCCH AL=2 with low correlation channel model, the working SINR is too low for a practical test case requirement

- The gain of the IRC processing with AL=1 in low correlation channel scenario can get comparable testable gain.

Proposal 1:

For scenario of 4CRS APs and 4RX chains, AL1 with low correlation channel model test case is proposed based on LMMSE-IRC receiver with CRS-IM for FDD mode.

Observation 2:

For scenarios of 4CRS APs and 2RX chains, the testable performance gain of LMMSE-IRC+CRS-IM receiver (about 3dB) can be achieved over the LMMSE-MRC baseline receiver.

- The gain of CRS-IM processing is more obvious than the gain of IRC processing.

For scenarios of 4CRS APs and 4RX chains, large performance gains of LMMSE-IRC+CRS-IM receiver (about 5dB) can be achieved over the LMMSE-MRC baseline receiver.

- The gain of the IRC processing is more obvious than the gain of CRS-IM processing.

- For PDCCH AL=2 with low correlation channel model, the working SINR is too low for a practical test case requirement

- The gain of the IRC processing with AL=1 in low correlation channel scenario can get comparable testable gain.

Proposal 2:

For scenario of 4CRS APs and 4RX chains, AL1 with low correlation channel model test case is proposed based on LMMSE-IRC receiver with CRS-IM for TDD mode.

- References

- RP-161877 “New Work Item Proposal: Enhanced CRS and SU-MIMO Interference Mitigation Performance Requirements for LTE”, Intel Corporation, Huawei, LGE, RAN #73, September 2016

- R4-1704230 “WF on Enhanced CRS-IM Performance Requirements”, Intel Corporation, LGE, CATT, RAN4 #82bis, April  2017

- R4-1704231 “Simulation assumptions for Enhanced CRS-IM”, Intel Corporation, LGE, CATT, RAN4 #82bis, April  2017

- Annex - Simulation Assumptions

- PDCCH/PCFICH simulation assumptions

Table 7 includes a list of the test cases to be used for performance requirements definition of the CRS-IM enhancements for the PDCCH/PCFICH. The PDCCH/PCFICH simulation assumptions are provided in Table 8. In Table 9 and Table 10 FRCs for PDCCH tests are presented.

Table 7. PDCCH test cases

| Test | Physical channel | CRS pattern | Number of UE RX chains | Number of CRS APs |  | Test purpose |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Serv. cell | Interf. cell |  |
| 1 | PDCCH | Non Colliding | 2 | 4 | 4 | 4 CRS APs IM verification |
| 3 |  | Non Colliding | 4 | 4 | 4 | 4 CRS APs IM + 4 RX CRS IM verification |

Table 8. PDCCH/PCFICH simulation assumptions

| Parameter | Value |  |
| --- | --- | --- |
| Duplexing | FDD, TDD |  |
| System bandwidth | 10 MHz |  |
| Tx EVM | 6% |  |
| Antenna models | Test 1: 2x4 low correlation Test 3: 4x4 low correlation |  |
| Cell ID pattern | Non-colliding CRS: - Serving cell: 0 - Interference cell 1: 1 - Interference cell 2: 6 |  |
| Interference power profile | Interference cell 1: I1/Noc = 13.91 dB Interference cell 2: I2/Noc = 3.34 dB |  |
| Network synchronization in time | All cells are synchronous |  |
|  | Time-delay wrt. serving cell |  |
|  | 1st interfering cell | 2nd interfering cell |
|  | 2 µs | 3 µs |
| Network synchronization in frequency | Frequency shift wrt. serving cell |  |
|  | 1st interfering cell | 2nd interfering cell |
|  | 200 Hz | 300 Hz |
| Channel model | EPA-5Hz for all links. Use different channel seed for between cells. |  |
| PHICH parameters | Normal PHICH Ng = 1/6 |  |
| Interference model | TS 36.101 B.7 (Rel-13 CCIM) |  |
| PDCCH/PCFICH parameters | Test #1: PDCCH AL = 2; CFI = 2 Test #3: PDCCH AL = [1,2]; CFI = 2 |  |
| Receivers | Baseline: LMMSE-MRC. Enhanced reference receiver structures: LMMSE-IRC (for information), LMMSE-IRC + 1 cell CRS-IM. |  |

Table 9. FDD FRCs for PDCCH

| Parameter | Unit | Value |
| --- | --- | --- |
|  |  | Test 1,3 |
| Reference channel |  | R.Z FDD |
| Number of transmitter antennas |  | 4 |
| Channel bandwidth | MHz | 10 |
| Number of OFDM symbols for PDCCH | symbols | 2 |
| Aggregation level | CCE | 2 |
| DCI Format |  | 2 |
| Cell ID |  | 0 |
| Payload (without CRC) | Bits | 46 |

Table10. TDD FRCs for PDCCH

| Parameter | Unit | Value |
| --- | --- | --- |
|  |  | Test 1,3 |
| Reference channel |  | R.Z TDD |
| Number of transmitter antennas |  | 4 |
| Channel bandwidth | MHz | 10 |
| Number of OFDM symbols for PDCCH | symbols | 2 |
| Aggregation level | CCE | 2 |
| DCI Format |  | 2 |
| Cell ID |  | 0 |
| Payload (without CRC) | Bits | 49 |