3GPP TS G-RAN WG4 Meeting #83		R4-1704925

# Hangzhou, China, 15 - 19 May, 2017

Agenda item:	7.27.3

Source:	CATT

Title:	PDSCH demodulation performance of Enhanced SU-MIMO IM

Document for:	Discussion

Introduction

In RAN #73 meeting, the “LTE Enhanced CRS and SU-MIMO Interference Mitigation Performance Requirements” WI was initiated [1]. In the previous RAN4#82bis meeting, further agreements on the target scenarios and reference receivers for the Stage 1 investigations were provided in [2]. And it is proposed to provide both alignment and impairment simulation results to identify target scenarios for requirements definition and confirm reference receiver feasibility. In this contribution, we provide both the alignment and the impairment simulation results based on the agreed simulation assumptions provided in [3] for PDSCH on enhanced SU-MIMO IM. The detailed simulation assumptions are attached in the Annex.

Simulation assumptions

In the simulation summary [3], the agreed PDSCH test cases for performance requirements are proposed as shown in Table 1.

Table 1.  Test cases for performance requirements：（FDD&TDD）

| Test scenario | Rank | Descriptions of other parameters |
| --- | --- | --- |
| TC#1 | Rank 2 | TM4 16QAM 1/2 ETU70 2x4 ULA Medium |
| TC#2 |  | TM9 16QAM 1/2 EPA5 2X4 ULA Medium |
| TC#3 | Rank 3 | TM3 16QAM 1/2 EVA70 4x4 ULA Medium A |
| TS #1 | Rank 2 | TM4 64QAM 1/2 EPA5 4x4 ULA Medium / ULA Medium A |
| TS #5 | Rank 4 | TM4 16QAM 0.57 EPA5 4x4 XPOL Medium A |
| TS #6 |  | TM9 16QAM 0.57 EPA5 4x4 XPOL Medium A |

Reference receivers are used for the evaluation:

- Receiver #1: LMMSE(baseline)

- Receiver #2: R-ML

Simulation results for FDD

The simulation results of the test cases TC#1-TC#3 for FDD are given below:

Test Case 1 of MIMO Rank-2 layers TM4 16QAM:

Figure 1. throughput performance of TC#1 (FDD)

Test Case 2 of MIMO Rank-2 layers TM9:

Figure 2. throughput performance of TC#2 (FDD)

Test Case 3 of MIMO Rank-3 layers TM3 (updated) :

Figure 3. throughput performance of TC#3 (FDD)

Test Scenario 1 of MIMO Rank-2 layers TM4 64QAM:

- TM4 64QAM 1/2 EPA5 4x4 ULA Medium

- TM4 64QAM 1/2 EPA5 4x4 ULA Medium A

Figure 4. throughput performance of Test Scenario#1 TM4-64QAM (FDD)

From the above link-level simulation results, the performance gains of the R-ML receiver vs. the LMMSE receiver are shown in table 2&3.

Table 2. the alignment result of the  performance gain of the SNRs  @ 70%  of  the maximum throughput

| Test Case | LMMSE(dB) | R-ML(dB) | Gains over LMMSE(dB) |
| --- | --- | --- | --- |
| TC#1 | 17.48 | 14.82 | 2.66 |
| TC#2 | 17.49 | 14.37 | 3.12 |
| TC#3 | 25.65 | 19.42 | 6.23 |
| TS #1 TM4 ULA Medium | 20.79 | 19.12 | 1.67 |
| TS #1 TM4 ULA Medium A | 12.84 | 12.44 | 0.4 |

Table 3. the impairment result of the  performance gain of the SNRs  @ 70%  of  the maximum throughput

| Test Case | LMMSE(dB) | R-ML(dB) | Gains over LMMSE(dB) |
| --- | --- | --- | --- |
| TC#1 | 18.98 | 16.32 | 2.66 |
| TC#2 | 18.99 | 15.87 | 3.12 |
| TC#3 | 27.15 | 20.92 | 6.23 |

Observation 1:

Testable performance gain of the R-ML receiver can be achieved over the LMMSE receiver under the 3 scenarios for performance requirements for FDD.

Observation 2:

For the RANK2 test case with 64QAM, the performance gain is only testable in Medium correlation channel.

Proposal 1:

Keep TC#1 with 16QAM for requirement definition from test feasibility point of view

Simulation results for TDD

The simulation results of the test cases TC#1-TC#3 for TDD are given below:

Test Case 1 of MIMO Rank-2 layers TM4:

Figure 5. throughput performance of TC#1 (TDD)

Test Case 2 of MIMO Rank-2 layers TM9:

Figure 6. throughput performance of TC#2 (TDD)

Test Case 3 of MIMO Rank-3 layers TM3 (updated) :

Figure 7. throughput performance of TC#3 (TDD)

From the above link-level simulation results, the performance gains of the R-ML receiver vs. the LMMSE receiver are shown in table 4&5.

Table 4. the alignment result of  the performance gain of the SNRs  @ 70%  of  the maximum throughput

| Test Case | LMMSE(dB) | R-ML(dB) | Gains over LMMSE(dB) |
| --- | --- | --- | --- |
| TC#1 | 16.67 | 14.19 | 2.48 |
| TC#2 | 16.88 | 13.88 | 3.0 |
| TC#3 | 25.14 | 19.32 | 5.82 |

Table 5. the impairment result of  the performance gain of the SNRs  @ 70%  of  the maximum throughput

| Test Case | LMMSE(dB) | R-ML(dB) | Gains over LMMSE(dB) |
| --- | --- | --- | --- |
| TC#1 | 18.17 | 15.69 | 2.48 |
| TC#2 | 18.38 | 15.38 | 3.0 |
| TC#3 | 26.64 | 20.82 | 5.82 |

Observation 3:

Testable performance gain of the R-ML receiver can be achieved over the LMMSE receiver under the 3 scenarios for performance requirements for TDD, the performance gains are comparable with these of FDD.

Simulation results for MIMO Rank-4 scenarios

Test Scenario #5:

TM4 16QAM 0.57 EPA5 4x4 XPOL Medium A

Figure 8. throughput performance of Test Scenario#5 TM4-16QAM

Test Scenario #6:

TM9 16QAM 0.57 EPA5 4x4 XPOL Medium A

Figure 9. throughput performance of Test Scenario#6 TM9-16QAM

From the above simulation results of the colliding scenarios, the performance gain of the R-ML receiver to the LMMSE receiver is shown in Table 6.

Table 6 the performance gain of the SNRs @70% of the maximum throughput

| Test Scenario | LMMSE(dB) | R-ML(dB) | Gain over LMMSE(dB) |
| --- | --- | --- | --- |
| TS #5 TM4 XPOL Medium A | 14.16 | 12.96 | 1.2 |
| TS #6 TM9 XPOL Medium A | 19.05 | 16.23 | 2.82 |

Observation 4:

For TM9 with 16QAM scenario, the R-ML receiver can provide testable gain under the XPOL Medium A channel condition.

Proposal 2:

Define requirements for TS#6 as a RANK4 SU-MIMO test case.

Conclusions

From our simulation results, we have the following observations and proposals:

Observation 1:

Testable performance gain of the R-ML receiver can be achieved over the LMMSE receiver under the 3 scenarios for performance requirements for FDD.

Observation 2:

For the RANK2 test case with 64QAM , the performance gain is only testable in Medium correlation channel.

Observation 3:

Testable performance gain of the R-ML receiver can be achieved over the LMMSE receiver under the 3 scenarios for performance requirements for TDD, the performance gains are comparable with these of FDD.

Observation 4:

For TM9 with 16QAM scenario, the R-ML receiver can provide testable gain under the XPOL Medium A channel condition.

From the above, we propose：

Proposal 1:

Keep TC#1 with 16QAM for requirement definition from test feasibility point of view

Proposal 2:

Define requirements for TS#6 as a RANK4 SU-MIMO test case.

# References

- RP-161877 “New Work Item Proposal: Enhanced CRS and SU-MIMO Interference Mitigation Performance Requirements for LTE”, Intel Corporation, Huawei, LGE, RAN #73, September 2016

- R4-1704237  “WF on enhanced SU-MIMO performance requirements”, Huawei, HiSilicon, LGE, CATT, Intel, RAN4 #82bis, April 2017

- R4-1703254  “Summary of simulation results for enhanced SU-MIMO”, LGE, RAN4 #82bis, April 2017

# Annex-simulation assumptions

- Common test parameters (FDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| Bandwidth | MHz | 10 |
| Duplex mode |  | FDD |
| Number of control OFDM symbols |  | 2 |
| Number of HARQ processes per component carrier | Processes | 8 |
| Maximum number of HARQ transmission |  | 4 |
| Redundancy version coding sequence |  | {0,1,2,3} for QPSK and 16QAM |
|  |  | {0,0,1,2} for 64QAM and 256QAM |
| Number of OFDM symbols for PDCCH | OFDM symbols | 2 for 10 MHz |
| PMI |  | PUSCH 3-1 for TM4/TM9 |
|  |  | Follow wideband PMI feedback for TM4/TM9 |
| CSI-RS ports |  | port 15, 16 for 2 Tx antennas port 15,…18 for 4 Tx antennas (port 0,1 for CRS) |
| CSI-RS periodicity and subframe offset TCSI-RS / ΔCSI-RS (for TM9) | Subframe | 5 / 2 |
| CSI reference signal configuration (for TM9) |  | 8 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap (for TM9) |  | 3 / 0010000000000000 |
| Cyclic Prefix |  | Normal |
| Cell_ID |  | 0 |
| Tx EVM |  | 6% for 16QAM and 64QAM; |
|  |  | 3% for 256QAM |

- Common test parameters (TDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| Bandwidth | MHz | 10 |
| Duplex mode |  | TDD |
| Uplink-Downlink Configuration |  | 1 |
| Special Subframe Configuration |  | 4 |
| Number of control OFDM symbols |  | 2 |
| Number of HARQ processes per component carrier | Processes | 7 |
| Maximum number of HARQ transmission |  | 4 |
| Redundancy version coding sequence |  | {0,1,2,3} for QPSK and 16QAM |
|  |  | {0,0,1,2} for 64QAM and 256QAM |
| Number of OFDM symbols for PDCCH | OFDM symbols | 2 for 10 MHz |
| PMI |  | PUSCH 3-1 for TM4/TM9 |
|  |  | Follow wideband PMI feedback for TM4/TM9 |
| CSI-RS ports |  | port 15, 16 for 2 Tx antennas port 15,…18 for 4 Tx antennas (port 0,1 for CRS) |
| CSI-RS periodicity and subframe offset TCSI-RS / ΔCSI-RS (for TM9) | Subframe | 5 / 4 |
| CSI reference signal configuration (for TM9) |  | 8 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap (for TM9) |  | 3 / 0000000000000000 |
| Cyclic Prefix |  | Normal |
| Cell_ID |  | 0 |
| Tx EVM |  | 6% for 16QAM and 64QAM; |
|  |  | 3% for 256QAM |
| Reference receiver |  | R-ML, interesting companies are encouraged to provide full ML results |

- Antenna correlation for evaluation：

|  | Antenna Correlation |
| --- | --- |
| AC #1 | ULA Low correlation |
| AC #2 | ULA Medium correlation (α=0.3, β=0.9) |
| AC #3 | ULA Medium A correlation (α=0.3, β=0.3874) |
| AC #4 | XPOL Medium A correlation (α=0.3, β=0.6, γ=0.2) |

- FRC of test cases for performance requirements:

|  |  | Test case for performance requirements |  |  |
| --- | --- | --- | --- | --- |
| FRC for FDD |  | Rank2 |  | Rank3 |
|  |  | TC#1 | TC#2 | TC#3 |
| Parameter | Unit | TM4 16QAM | TM9 16QAM | TM3 16QAM |
| Reference channel |  |  |  |  |
| Channel bandwidth | MHz | 10 | 10 | 10 |
| Allocated resource blocks (Note 3) |  | 50 | 50 | 50 |
| Allocated subframes per Radio Frame |  | 9 | 9 | 9 |
| Modulation |  | 16QAM | 16QAM | 16QAM |
| Target Coding Rate |  | 1/2 | 1/2 | 1/2 |
| Information Bit Payload |  |  |  |  |
| For Sub-Frames 1,4,6,9 | Bits | 12960 | 11448 | 12960(CW0) 25456(CW1) |
| For Sub-Frames 2,3,7,8 | Bits | 12960 | 11448 | 12960(CW0) 25456(CW1) |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | 12960 | 9528 | 11448(CW0) 22920(CW1) |
| Number of Code Blocks (Note 4) |  |  |  |  |
| For Sub-Frames 1,4,6,9 | Code blocks | 3 | 2 | 3(CW0) 5(CW1) |
| For Sub-Frames 2,3,7,8 | Code blocks | 3 | 2 | 3(CW0) 6(CW1) |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | 3 | 2 | 2(CW0) 4(CW1) |
| Binary Channel Bits |  |  |  |  |
| For Sub-Frames 1,6 | Bits | 26400 | 24000 | 25600(CW0) 51200(CW1) |
| For Sub-Frames 4,9 | Bits | 26400 | 24000 | 25600(CW0) 51200(CW1) |
| For Sub-Frames 2,7 | Bits | 26400 | 23600 | 25600(CW0) 51200(CW1) |
| For Sub-Frames 3,8 | Bits | 26400 | 23200 | 25600(CW0) 51200(CW1) |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | 24768 | 19680 | 24064(CW0) 48128(CW1) |
| Max. Throughput averaged over 1 frame | Mbps | 11.664 | 10.1112 | 11.513(CW0) 22.657(CW1) |
| Note 1: 2 symbols allocated to PDCCH. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211. Note 3: For TM9, 50 resource blocks are allocated in sub-frames 1, 2, 3, 4, 6, 7, 8, 9 and 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0. Note 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |  |

|  |  | scenarios for further evaluation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FRC for FDD |  | Rank2 |  |  |  | Rank4 |  |
|  |  | TS#1 | TS#2 | TS#3 | TS#4 | TS#5 | TS#6 |
| Parameter | Unit | TM4 64QAM | TM9 64QAM | TM4 256QAM | TM9 256QAM | TM4 16QAM | TM9 16QAM |
| Reference channel |  |  |  |  |  |  |  |
| Channel bandwidth | MHz | 10 | 10 | 10 | 10 | 10 | 10 |
| Allocated resource blocks (Note 3) |  | 50 | 50 | 50 | 50 | 50 | 50 |
| Allocated subframes per Radio Frame |  | 9 | 9 | 9 | 9 | 9 | 9 |
| Modulation |  | 64QAM | 64QAM | 256QAM | 256QAM | 16QAM | 16QAM |
| Target Coding Rate |  | 1/2 | 1/2 | 0.62 | 0.67 | 1/2 | 0.57 |
| Information Bit Payload |  |  |  |  |  |  |  |
| For Sub-Frames 1,4,6,9 | Bits | 18336 | 18336 | 31704 | 31704 | 25456 | 25456 |
| For Sub-Frames 2,3,7,8 | Bits | 18336 | 18336 | 31704 | 31704 | 25456 | 25456 |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | 18336 | 14688 | 31704 | 26416 | 22920 | 21384 |
| Number of Code Blocks (Note 4) |  |  |  |  |  |  |  |
| For Sub-Frames 1,4,6,9 | Code blocks | 3 | 3 | 6 | 6 | 5 | 5 |
| For Sub-Frames 2,3,7,8 | Code blocks | 3 | 3 | 6 | 6 | 5 | 5 |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | 3 | 3 | 6 | 5 | 4 | 4 |
| Binary Channel Bits |  |  |  |  |  |  |  |
| For Sub-Frames 1,6 | Bits | 38400 | 36000 | 51200 | 48000 | 51200 | 43200 |
| For Sub-Frames 4,9 | Bits | 38400 | 36000 | 51200 | 48000 | 51200 | 43200 |
| For Sub-Frames 2,7 | Bits | 38400 | 34800 | 51200 | 46400 | 51200 | 41600 |
| For Sub-Frames 3,8 | Bits | 38400 | 34800 | 51200 | 46400 | 51200 | 41600 |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | 36096 | 29520 | 48128 | 39360 | 48128 | 35424 |
| Max. Throughput averaged over 1 frame | Mbps | 16.502 | 16.138 | 28.534 | 28.005 | 22.657 | 22.503 |
| Note 1: 2 symbols allocated to PDCCH. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211. Note 3: For TM9, 50 resource blocks are allocated in sub-frames 1, 2, 3, 4, 6, 7, 8, 9 and 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0. Note 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |  |  |  |  |

| FRC for TDD |  | Test case for performance requirements |  |  |
| --- | --- | --- | --- | --- |
|  |  | Rank2 |  | Rank3 |
|  |  | TC#1-TDD | TC#2-TDD | TC#3-TDD |
| Parameter | Unit | TM4 16QAM | TM9 16QAM | TM3 16QAM |
| Reference channel |  |  |  |  |
| Channel bandwidth | MHz | 10 | 10 | 10 |
| Allocated resource blocks |  | 50 | 50 5 | 50 |
| Uplink-Downlink Configuration (Note 3) |  | 1 | 1 | 1 |
| Special Subframe Configuration (Note 3) |  | 4 | 4 | 4 |
| Allocated subframes per Radio Frame (D+S) |  | 2+2 | 3+2 | 2+2 |
| Modulation |  | 16QAM | 16QAM | 16QAM |
| Target Coding Rate |  | 1/2 | 1/2 | 1/2 |
| Information Bit Payload |  |  |  |  |
| For Sub-Frames 4,9 | Bits | 12960 | 11448 | 12960(CW0) 25456(CW1) |
| For Sub-Frames 1,6 |  | 9528 | 7736 | 9528(CW0) 19080(CW1) |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | N/A | 9528 | N/A |
| Number of Code Blocks (Notes 4) |  |  |  |  |
| For Sub-Frames 4,9 |  | 3 | 2 | 3(CW0) 5(CW1) |
| For Sub-Frames 1,6 |  | 2 | 2 | 2(CW0) 4(CW1) |
| For Sub-Frame 5 |  | N/A | N/A | N/A |
| For Sub-Frame 0 |  | N/A | 2 | N/A |
| Binary Channel Bits |  |  |  |  |
| For Sub-Frames 4,9 | Bits | 26400 | 23600 | 25600(CW0) 51200(CW1) |
| For Sub-Frames 1,6 |  | 21312 | 15744 | 20512(CW0) 41024(CW1) |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | N/A | 19680 | N/A |
| Max. Throughput averaged over 1 frame | Mbps | 4.498 | 4.79 | 4.498(CW0) 8.907(CW1) |
| UE Category |  | ≥ 2 | ≥ 2 | ≥ 2 |