3GPP TSG-RAN WG4 #83	R4- 1704950

Hangzhou, China, 15th –19th May 2017

Source: 	Ericsson

Title:  	Further elaboration on NR bandwidths for sub 6 GHz bands

Agenda Item:	10.3.1

Document for:	Approval

# Introduction

In [1], the discussion on NR bandwidths and a few general considerations for bands below 6 GHz was initiated. As the size of SS transmission indicating the miniumum bandwidths as well as maximum bandwidths is settled, this paper consider possible candidate bandwidths for some the existing 3GPP bands which are included in the objective of NR WID

# Discussion

The list of agreed NR sub- 6 GHz bands and the supporting operators in the NR WID [2] is as following:

Table 1: Frequency ranges and LTE bands for NR in the WI

| Frequency range /LTE band | REL-indep. from | other supporting companies (min. 3) |
| --- | --- | --- |
| 3.3-4.2 GHz | REL-15 | NTT DOCOMO, KDDI, SBM, CMCC, China Unicom, China Telecom, KT, SK Telecom, LG Uplus, Etisalat, Orange, Telecom Italia, British Telecom, Deutsche Telekom |
| 4.4-4.99 GHz | REL-15 | NTT DOCOMO, KDDI, SBM, CMCC, China Unicom, China Telecom, |
| 1.427-1.518G | REL-15 | Etisalat |
| Band 3 | REL-15 | CMCC, China Telecom |
| Band 7 | REL-15 | CHTTL, British Telecom |
| Band 8 | REL-15 | CMCC |
| Band 20 | REL-15 | Orange |
| Band 28 | REL-15 | Orange, Swisscom, Telecom Italia, Telefonica, Vodafone |
| Band 41 | REL-15 | Sprint, China Telecom, C-Spire, China Unicom |
| Band 66 | REL-15 | T-mobile, DISH |
| Band 1 | REL-15 | China Unicom, China Telecom |

As there is a large variation in band sizes not only in the bands listed above but also in some cases due to regional allocation, the supported NR bandwidths would be also be band dependent similar to E-UTRA as stated in [1].

Considering the supported E-UTRA bandwidths, Table 2 summarizes the supported bandwidths for existing bands in the WID:

Table 2	Extract from E-UTRA supported channel bandwidths

| E-UTRA band / Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 1 |  |  | Yes | Yes | Yes | Yes |
| 3 | Yes | Yes | Yes | Yes | Yes1 | Yes1 |
| 7 |  |  | Yes | Yes | Yes3 | Yes1, 3 |
| 8 | Yes | Yes | Yes | Yes1 |  |  |
| 20 |  |  | Yes | Yes1 | Yes1 | Yes1 |
| 28 |  | Yes | Yes | Yes1 | Yes1 | Yes1, 2 |
| 41 |  |  | Yes | Yes | Yes | Yes |
| 66 | Yes | Yes | Yes | Yes | Yes | Yes |
| NOTE 1: 1 refers to the bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (subclause 7.3) is allowed. NOTE 2: 2 For the 20 MHz bandwidth, the minimum requirements are specified for E-UTRA UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz NOTE 3: 3 refers to the bandwidth for which the uplink transmission bandwidth can be restricted by the network for some channel assignments in FDD/TDD co-existence scenarios in order to meet unwanted emissions requirements (Clause 6.6.3.2). NOTE 4: 4 For the 20 MHz bandwidth, the minimum requirements are restricted to E-UTRA operation when carrier aggregation is configured. |  |  |  |  |  |  |

For NR following should be considered:

The NR minimum bandwidth based on SS block (127 sub-carriers) and PBCH (288 sub-carriers) transmission for sub-6 GHz bands is settled as following:

  - 15 kHz subcarrier spacing NR-SS transmission results in minimum bandwidth of 5 MHz.

  - 30 kHz subcarrier spacing NR-SS transmission results in minimum bandwidth of 10 MHz.

Thus, the E-UTRA bandwidths of 1.4 MHz and 3 MHz can not be supported for NR.

On the other hand, for some bands, the supported NR bandwidths can be larger than  for E-UTRA but would possibly imply some additional modification of UE requirements such as sensitivity, A-MPR or restrictions of UL transmission bandwidth for other confinements.

Table 2 summarizes some possible candidate bandwidths for NR for existing bands that are covered in WID.

Table 2	Initial proposal for NR supported channel bandwidths in some existing bands

| NR band / Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 80 MHz | 100 MHz |
| 1 | Yes | Yes | Yes | Yes | Yes |  |  |
| 3 | Yes | Yes | Yes | Yes | Yes |  |  |
| 7 | Yes | Yes | Yes | Yes | Yes |  |  |
| 8 | Yes | Yes | Yes |  |  |  |  |
| 20 | Yes | Yes | Yes | Yes |  |  |  |
| 28 | Yes | Yes | Yes | Yes |  |  |  |
| 41 |  | Yes |  | Yes | Yes | Yes | Yes |
| 66 | Yes | Yes | Yes | Yes | Yes |  |  |
| NOTE 1: |  |  |  |  |  |  |  |

For some bands depending on their size, additional supported bandwidths could be added or excluded (depending on operator license allocations) but as any addition would result in increased number of permutations, the supported bandwidths should be chosen carefully.

In addition, as previously stated, feasibility of certain bandwidth from UE perspective and impact on sensitivity, A-MPR or other restrictions should be considered.

For the new bands around 3.5 GHz and 4.5 GHz, the maximum BW of 100 MHz as a BW option is feasible as the size of the concerned bands is sufficient.

# Conclusion

In this paper, the selection of supported NR bandwidths for the candidate bands below 6 GHz was further elaborated. The choice of minimum bandwidth depends on the frequency band and sub-carrier spacing for SS block transmission while the maximum BW and additional bandwidth options which need to be kept to a level which would avoid excessive permutations. The supported bandwidths can be based on the concerned band size, operator allocations and other factorts to minimize the impact on UE performance requirements.

Based on the above, an initial proposal on supported NR bandwidths for 3GPP existing bands included in the WID was given. We encourage RAN4 to further discuss and consider the aspects brought up in this paper and conclude on supported bandwidths.

# References

[1]	R4-1703082, “On NR bandwidths for sub 6 GHz”, Ericsson

[2]	RP-170823, “WI on New Radio (NR) access technology”