3GPP TSG-RAN WG4 Meeting #83 	R4-1705082

# Hangzhou, China, 15th – 19th May 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 4943 | rev | - | Current version: | 13.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PHR reporting requirements for NB1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-Perf |  |  |  |  |  |  | Date: |  | 2017-05-15 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Currently there are two PHR tables defined in section 9.1.23 based on the coverage level. It is stated in table that normal coverage PHR table applies when enhanced coverage level 0 is selected during RA and the enhanced coverage PHR table applies when enhanced coverage level other than 0 is selected. This dependency on the CE level limits the different implementations since since some implemenation may support enhanced coverage using the CE level 0 by applying higher number repetitions which are supported within that level, while others will use a higher CE level. Hence, it is proposed to remove this CE level dependncy and instead only choose the PHR tables based on the coverage levels (normal or enhanced coverage) which are clearly defined in section 3.6.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The text related to the selected CE level tis removed from the PHR reporting tables for category NB1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A big limitation in the implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.23 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 36.521 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

-----------------------------------------------------------Beginning of Change ----------------------------------------------------------

### 9.1.23	Power Headroom for UE Category NB1

The requirements in this clause shall apply for power headroom for UE Category NB1 as defined in [31].

The power headroom provides the serving eNB with information about the differences between the UE configured maximum output power (PCMAX,) defined in TS 36.101 [5] and the estimated power for UL-NSCH transmission of the serving cell [3].

#### 9.1.23.1	Period

The reported power headroom shall be estimated over 1 slot of NPUSCH transmissions.

#### 9.1.23.2	Reporting Delay

The power headroom reporting delay is defined as the time between the beginning of the power headroom reference period and the time when the UE starts transmitting the power headroom over the radio interface. The reporting delay of the power headroom shall be 0 ms, which is applicable for all configured triggering mechanisms for power headroom reporting.

#### 9.1.23.3	Report Mapping for UE Category NB1

##### 9.1.23.3.1	Report Mapping for UE Category NB1 in normal coverage

The power headroom reporting range is from -23 ...+11 dB for UE category NB1 in normal coverage and the report maping is defined in Table 9.1.23.3.1-1..

Table 9.1.23.3-1: Power headroom report mapping for UE category NB1 in normal coverage

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -23 PH 5 |
| POWER_HEADROOM_1 | 5 PH 8 |
| POWER_HEADROOM_2 | 8 PH 11 |
| POWER_HEADROOM_3 | PH ≥ 11 |

##### 9.1.23.3.2	Report Mapping for UE Category NB1 in enhanced coverage

The power headroom reporting range is from -23 ...+6 dB for UE category NB1 in enhanced coverage and the report mapping is defined in Table 9.1.23.3.2-1.

Table 9.1.23.3.2-1: NB-IOT power headroom report mapping in enhanced coverage

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -23 PH -10 |
| POWER_HEADROOM_1 | -10 PH -2 |
| POWER_HEADROOM_2 | -2 PH 6 |
| POWER_HEADROOM_3 | PH ≥ 6 |

---------------------------------------------------------------End of Change---------------------------------------------------------------