3GPP TSG-RAN WG4 #83	R4-1705094

Hangzhou, China, 14-18 May 2017

Source: 	Ericsson

Title:  	Joint probability for receiver blocking in NR BS

Agenda Item:	10.5.4.2

Document for:	Approval

# Introduction

BS receiver blocking has been studied in RAN4 in the Rel-14 NR SI phase. In this contribution, we provide a methodology based on joint probability of receiver blocking where probability of both wanted signal and blocking signal above a level are considered.

# Joint distribution of angle and distances

In NR, beamforming will play an important role in terms of directivity between wanted signals and blocker signals. The directivity of the signals and the positions of the UEs will actually determine the blocker signal levels. In the following figures, we provide joint distribution of angle and distances of being both the victim UE and blocker UE transmitting in the same direction, in addition to joint distribution of being in certain distance with respect to the victim BS. We provide distributions for both collocated and non-collocated case when urban macro scenario in 30GHz is considered.

Figure 1	Joint distribution of angle and distances between victims and blockers in UMa collocated deployment

Figure 2	Joint distribution of angle and distances between victims and blockers in UMa non-collocated deployment

From the above figures, we can see very clearly that, the potential stronger blockers occur only at certain distance and certain directivity to a level below 3%. It is worth mentioning here that, the above plots do not indicate the blocker levels, however only indicates potential blockers at certain distance and angle distribution. In order to get a more detailed picture of strongest blocker levels we provide the UL interference statistics, by providing the cdf of top five blockers in UL. These results provide a good indication of the levels of the blockers that are seen at the NR BS.

Figure 3	The 5 strongest blocking signal power CDFs together with the wanted signal power CDF for UMa collocated (right) and non-collocated (left) deployments.

A summary of the 5 strongest blockers statistics is given below.

Table 1   99%-tile for signal power of 1st -5th largest blockers for UMa collocated

|  | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| Signal power [dBm] | -70.8 | -72.8 | -75.6 | -78.8 | -80.8 |

Table 2   99%-tile for signal power of 1st -5th largest blockers for UMa non-collocated

|  | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| Signal power [dBm] | -66 | -67.6 | -71.2 | -74.2 | -74.8 |

The above results identify that the NR blocker requirements could be potentially much less strict compared to E-UTRA systems.

# Joint probability for receiver blocking

Receiver blocking level is usually defined for EUTRA and UTRA as the level at which the receiver can sustain a blocker with 99.99% probability. For NR, beamforming will be an integral part of the system, thus probability for the blocker level alone is not sufficient, instead joint probability of blocker levels above certain level and the wanted signal below a certain level should be considered.

For a WCDMA system, transmissions are continuous and thus blocking will take down the entire UL for an extended period of time. For NR, a blocking event occurs only in a specific TTI; as soon as the beam changes then there is no longer blocking. So blocking degrades link throughput. It is strange to target 5% throughput loss for ACS but 0.01% throughput loss for blocking.

Blocking occurs when both a blocker is at a high level and a wanted signal is weak (at least, that is true as long as there is AGC). So the joint probability is more relevant.

The following steps can be followed for definition of joint probability for receiver blocking:

- Step-1: Determine a joint blocker probability (P1)

- Step-2: Find out the probability that the wanted signal being below REFSENS+6dB (P2)

- Step-3: Find the interference level for which P3 is satisfied, where P3=P1/P2.

The above steps are described in detail below.

## Step-1: Determine a joint blocker probability (P1)

We determine the joint blocker probability (P1) as the joint probability for blocker signal and wanted signal gives the probability that each of wanted and blocker signal falls in any range or discrete set of values specified for that variable. In general, the blocker signal is expected to be above REFSENS+6dB. In this joint probability case, the wanted signal is expected to be below REFSENS+6dB while blocker signal level is measured at certain level based on assumed joint probability. Thus, P1 is defined as follows:

P1=Probblocking,wanted<(REFSENS+6)

P1 can be assumed to be 1% when blocking probability is 99%. For UTRA and EUTRA, 99.99% blocker probability has been assumed. In case of UTRA, when the blocker level is above the specified level, there is a strong probability that this blocker will block the receiver continuously, since the common transmission from the blocker will sustain for long time duration. The blocker can destroy uplink availability of the BS on all channels during this time. Thus, the blocker probability has been set to enable a high BS availability, 99.99%. The mentioned high level blocking probability is set so that  the receiver is able to tackle the blocker with almost certainty.

In case of NR, beamforming at the transmitter side will ensure that the blocker will affect the receiver only for short while, e.g. for a subframe or a few subframes depending on the scheduling of the blocker and will only impact one or a subset of users during that TTI. Thus, the blocking probability for NR will relate to link degradation more than BS availability and as such we do not need to consider such low blocking probability.

Moreover, we considered up to 5% throughput degradation when we studied the coexistence issues for WP5D response. Thus, it seems unreasonable to assume 99.99% blocker probability.

Considering the above reasoning, we propose to use 1% joint blocker probability for NR.

## Step-2:  Find out the probability that the wanted signal being below REFSENS+6dB (P2)

From the statistics of the wanted signal, we find out the probability P2 that wanted signal is below REFSENS+6dBdefined as:

P2=Probwanted<(REFSENS+6)

Note that we could also consider something other than 6dB if useful. We have considered 6dB since this is used quite extensively in E-UTRA specification for UMa deployment.

We would like to emphasize here that we could get a reasonable estimation of wanted signal probability based on simulations for ITU-R work, as detailed in TR 38.803 [1]. Moreover, it is also worth noting that, the wanted signal probability and blocking probability are statistically independent as they relate to different systems; hence they can be individually derived and just multiplied.

The main challenge is to define the REFSENS in OTA context. As explained in our previous contribution [2], the BS OTA sensitivity can be expressed as:

OTA sensitivity= -174 + NF + 10 log BW + SINR - 10 log (n) - 5 (element gain) + implementation margin

The above formula is valid when the UE is in the boresight of the BS.

SINR can be chosen from the SINR range which is also a parameter requested by ITU-R. For simplification, we would propose SINR = 0 dB. The noise figure value can be used as 10 dB, 12 dB and 14 dB for 30 GHz, 45 GHz and 70 GHz respectively. An implementation margin of ~1.5 dB can be used to calculate the OTA sensitivity.

We have provided some discussions related to possible OTA REFSENS levels in one of our earlier contributions [2].

## Step-3: Find the interference level for which P3 is satisfied, where P3=P1/P2.

P3 is the conditional probability which is defined as below:

P3=Probblockingwanted<(REFSENS+6)=P1P2

From the interference statistics, we find out the interference level for which P3 is satisfied. This will be the blocker level with 99% joint probability.

# Conclusion

Based on the above discussions, we propose the following:

Proposal-1: Follow the following steps for determining the joint probability of receiver blocking levels:

- Determine a joint blocker probability (P1)

- Find out the probability that the wanted signal being below REFSENS+6dB (P2)

- Find the interference level for which P3 is satisfied, where P3=P1/P2.

We also propose the following:

Proposal-2: Adopt a joint probability of blocking as 99%.

We propose to adopt the above two proposals while determining the methodology for receiver blocking for NR BS.

# References

TR 38.803

R4-1700224, BS OTA sensitivity for mm-wave frequencies, Ericsson