3GPP TSG-RAN WG4 Meeting #83	 R4-1705162

Hangzhou, China, 15 – 19 May, 2017

Agenda Item:	9.8.2

Source: 	Samsung …

Title: 	TP on REFSENS for Band 71

Document for:	Approval

Introduction

Several contributions provided evaluation results for REFSENS of Band 71[1-7]. During the offline discussion more results are collected. They are summarized in table 1 below. Furthermore, it is suggested to accept the average value to be defined for Band 71.

Table 1 Evaluation results summary on REFSENS (2RX) for Band 71

| Reference# | REFSENS for 10MHz(dBm) | REFSENS for 15MHz(dBm) | REFSENS for 15MHz(dBm) | REFSENS for 20MHz(dBm) |
| --- | --- | --- | --- | --- |
| [1] | -97 | -94 | -91.6 | -82.2 |
| [2] | -97 | -94 | -92.2 | -87.9 |
| [3] | -97 | -94 | -92.2 | -90 |
| [4] | -97 | -94 | -91.2 | -85.5 |
| [5] | -97 | -94 | -92.2 | -88.1 |
| [7] | -98 | -95 | -92.9 | -89.2 |
| Skyworks |  |  |  | -89.6 |
| Proposed value to be agreed | -97.2 | -94.2 | -92 | -87.5 |

Table 2: Uplink configuration for evluation

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| 71 |  |  | 25 | 25 1 | 20 1 | 20 1 | FDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth |  |  |  |  |  |  |  |

Based on above discussion corresponding TP is provided in section2. It’s proposed to incorporate below TP on UE REFSENS in TR for Band 71.

Text proposal

<Start of change>

- 7.2.X UE REFSENS

Evaluation results for REFSENS of Band 71 are presented in table 7.2.X-1. And it is agreed to use average value to define the corresponding requirements.

Table 7.2.X-1 Evaluation result summary on REFSENS(2RX) for Band 71

| Source# | REFSENS for 10MHz(dBm) | REFSENS for 15MHz(dBm) | REFSENS for 15MHz(dBm) | REFSENS for 20MHz(dBm) |
| --- | --- | --- | --- | --- |
| 1 | -97 | -94 | -91.6 | -82.2 |
| 2 | -97 | -94 | -92.2 | -87.9 |
| 3 | -97 | -94 | -92.2 | -90 |
| 4 | -97 | -94 | -91.2 | -85.5 |
| 5 | -97 | -94 | -92.2 | -88.1 |
| 6 | -98 | -95 | -92.9 | -89.2 |
| 7 |  |  |  | -89.6 |
| Average value | -97.2 | -94.2 | -92 | -87.5 |

Based on above assessments, the reference sensitivity requirement specified in table7.2.X-2 shall be met for E-UTRA Band 71 for an uplink transmission bandwidth less than or equal to the specified in and table 7.2.X-3.

Table7.2.X-2 Reference sensitivity QPSK PREFSENS(2RX)

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 71 | - | - | -97.2 | -94.2 | -92 | -87.5 | FDD |

Table 7.2.X-3 Uplink configuration for reference sensitivity

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| 71 |  |  | 25 | 25 1 | 20 1 | 20 1 | FDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth |  |  |  |  |  |  |  |

<End of change>

Reference

[1] R4-1702797, Discussion on UE RF requirement for Band 71, Samsung

[2]R4-1703065, Reference Sensitivity for 20 MHz channel BW, Ericsson
[3] R4-1703855, Self-interference issues for new US 600 MHz band, Nokia
[4] R4-1703983, Reference sensitivity for wide bandwidth channels in US 600 MHz band, Qualcomm
[5] R4-1703989, REFSENS impact due to TX leakage for the USA 600MHz band, Intel
[6] R4-1704007, Band 71 UE RF Duplexer Architecture and REFSENS Impact, Skyworks
[7] R4-1705238, B71 REFSENS, Dish

[8] R4-1705224, TR36.755: US600MHz band