3GPP TSG-RAN WG4 Meeting #83	R4-175238

Hangzhou, China, 15 - 19 May, 2017

Agenda item:	9.8.2

Source:	Dish Network

Title:	B71 REFSENS

Document for:	Discussion

# 1	Introduction

Several companies provided REFSENS analysis for B71 in RAN4#82bis.  In this contribution we provide our view on B71 REFSENS.

# 2	Discussion

Together with frequency arrangements, filter characteristics that mostly impact the receiver NF are IL and TX/RX isolation.

The TX-RX distance is 46MHz for B71 while it is 30MHz for B12/B17, 31MHz for B13, and 41MHz for B20. We note that the UL RB allocation was shifted in B20 to avoid IMD reaching 15/20MHz DL. In any case, the B71 TX-RX separation is larger than that for B12/B13/B17 and roughly similar to B20. TX-RX separation is an important factor as it basically defines the PA spectrum regrowth power at PA output at RX frequency as well as TX and RX phase noise. B20 RX isolation assumption was 45dB when B20 was created.

A maximum of 1dB difference in REFSENS to bands that use 9dB NF (REFSENS=-100dBm for 5MHz) can be explained by larger IL. The rest of the difference comes from noise rise due to self-interference. The increased NF for the receiver is a result of increased noise from TX; Our opinion is that it’s not correct approach to assume very pessimistic NF and on top of that calculate the impact of additional noise assuming good duplexer isolation. By doing so the additional noise is included kind of twice in calculation. Instead, reasonable NF should be used as a baseline and then the impact of additional noise should be accounted assuming good duplexer isolation.

To highlight the impact of additional noise and duplexer characteristics, we did compare the performance of B20 and B71. Basically, we calculated the REFSENS for both B20 and B71 including the impact of TX noise on top of 10dB NF. B20 duplexer RX isolation assumption was 45dB because it was the assumption during B20 work. We did not find any clear definition for TX isolation but we assumed it as 55dB. Typically, the difference between TX and RX isolation is not more than 5dB but anyway we assumed it as 10dB to be on the safe side. Parameters and analysis results can be found in Table 1 and Table 2 below. Parameters that don’t show band number are common for both bands. The TX-RX isolation is smaller for B20 causing higher PA noise, but still we assumed the PA noise to be at the same level for both bands. Similarly, TX and RX phase noise are higher for B20 but we assumed the same for both bands. The impact of better duplexer slightly exceeds 1dB for both 5MHz and 10MHz CC’s. This analysis illustrates that because the duplexer for B71 is better than that for B20, 1dB tightening in the baseline REFSENS is reasonable. We note that this would not be the only possible comparison; compared with B12/B13/B17 the PA noise and TX/RX phase noise are lower for B71 because the TX-RX separation is larger.

Table 1 RF parameters

Table 2 REFSENS comparison

Our REFSENS proposal is as follows.

| 5MHz | 10MHz | 15MHz | 20MHz |
| --- | --- | --- | --- |
| -98dBm | -95dBm | -92.9dBm | -89.2dBm |

Table 3 REFSENS proposal

# 3	Conclusions

Our REFSENS proposal is as follows.

| 5MHz | 10MHz | 15MHz | 20MHz |
| --- | --- | --- | --- |
| -98dBm | -95dBm | -92.9dBm | -89.2dBm |

# 4	References

- R4-173434, “TP on REFSENS for Band 71”, Samsung, RAN4#82bis