3GPP TSG-RAN WG4 Meeting #83                                                R4-1705302

Huangzhou, China, 15 - 19 May, 2017

Source:		Ericsson

Title:			FRC for 256QAM

Agenda item:		7.12.3

Document for:	Discussion

# 1 Introduction

In this contribution we present FRC for 256 QAM.

# 2 Discussion

We present an FRC calculated in the standard way:

- Payload size from Transport block size table corresponding to MCS 26.

- Code block CRC added if more than 6144 bits in payload size.

- Code block CRC added if more than 1 code block.

- Number of 6144 code blocks.

- Coded block size including 12bits trellis termination (bits) = 
((Payload size + Transport block CRC)/ Number of code blocks)*3+12

- Total number of bits per sub-frame = Total symbols per sub-frame * 8

- Total symbols per sub-frame = Allocated resource blocks * 12 * DFT-OFDM Symbols per subframe

## 2.1 Fixed Reference Channel

# Table 1 FRC parameters for performance requirements (256QAM 5/6)

| Reference channel | A17-1 | A17-2 | A17-3 | A17-4 | A17-5 | A17-6 |
| --- | --- | --- | --- | --- | --- | --- |
| Allocated resource blocks | 6 | 15 | 25 | 50 | 75 | 100 |
| DFT-OFDM Symbols per subframe | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation | 256QAM | 256QAM | 256QAM | 256QAM | 256QAM | 256QAM |
| Code rate | 5/6 | 5/6 | 5/6 | 5/6 | 5/6 | 5/6 |
| Payload size (bits) | 5544 | 14112 | 22920 | 46888 | 68808 | 93800 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 0 | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 1 | 3 | 4 | 8 | 12 | 16 |
| Coded block size including 12bits trellis termination (bits) | 16716 | 14220 | 17292 | 17676 | 17292 | 17676 |
| Total number of bits per sub-frame | 6912 | 17280 | 28800 | 57600 | 86400 | 115200 |
| Total symbols per sub-frame | 864 | 2160 | 3600 | 7200 | 10800 | 14400 |

# 3 Conclusion

FRC for 256QAM is presented.

# 4 References

- R4-1702302, WF on UL enhancement, CMCC, CMCC,  Nokia, Alcatel-Lucent Shanghai Bell, Ericsson, Huawei, HiSilicon.