3GPP TSG-RAN WG4 Meeting #83	R4-1705427

Hangzhou, China, 15 - 19 May, 2017

Agenda Item:	9.14.2.1

Source: 	Nokia, Alcatel-Lucent Shanghai Bell

Title: 	Receiver dynamic range for microcell NB-IoT BS

Document for:	Approval

# 1.	Introduction

The WI proposal on Further NB-IoT enhancements was agreed in RAN#73 [1]. One of the objectives of the WI is to specify necessary support (appropriate eNB classes) for NB-IoT to be used in microcell, picocell, and femtocell deployments. A way forward on new BS classes for NB-IoT was approved in RAN4#82bis [2], where the list of required coexistence simulations and corresponding assumptions were provided.

This contribution provides the simulation results for NB-IoT standalone operation in microcell deployment using the agreed assumptions, and proposes a way forward to specify the receiver dynamic range requirement for microcell NB-IoT BS in the RAN4 specifications.

# 2.	Discussion

It was approved in [2] that coexistence simulations for standalone NB-IoT vs standalone NB-IoT are required to set BS RF requirements. The simulation results for NB-IoT standalone operation in microcell deployment are provided below, using the simulation assumptions for medium-range BS in TR 37.809 [3] and NB-IoT in TR 36.802 [4]. Here the medium range E-UTRA BS noise figure of 10 dB is assumed for the microcell NB-IoT BS. The microcell NB-IoT BS ACS is assumed to be 45 dB (i.e. the same as wide area NB-IoT BS). And the UL power control is as specified in TR 36.942 [5] section 5.1.1.6 (set 1) by bandwidth scale, target SNR at BS is 15 dB. Thus the CLx-ile for microcell NB-IoT UE is set to 130 [=112+10log10(16*180e3/15*e3)-5] for 15 kHz sub-carrier spacing. The carrier frequency is 2 GHz.

The simulation results for microcell victim and aggressor (Micro Vs Micro) case are shown in Figure 1 below. Here IOT is defined as the ratio between the total interference (with or without interfering system) and the BS noise floor.

(a)	(b)

Figure 1: Microcell victim and aggressor (Micro Vs Micro) case

The simulation results for microcell victim and macrocell aggressor (Micro Vs Macro) case are shown in Figures 2 and 3 below, while the simulation results for macrocell victim and microcell aggressor (Macro Vs Micro) case are shown in Figures 4 and 5 below. Here the wide area NB-IoT BS noise figure of 5 dB is assumed, and the CLx-ile for macrocell NB-IoT UE is set to 135 [=112+10log10(16*180e3/15*e3)] for 15 kHz sub-carrier spacing.

(a)	(b)

Figure 2: Microcell victim and 500 m ISD macrocell aggressor (Micro Vs 500 Macro) case

(a)	(b)

Figure 3: Microcell victim and 1732 m ISD macrocell aggressor (Micro Vs 1732 Macro) case

(a)	(b)

Figure 4: 500 m ISD macrocell victim and microcell aggressor (500 Macro Vs Micro) case

(a)	(b)

Figure 5: 1732 m ISD macrocell victim and microcell aggressor (1732 Macro Vs Micro) case

It can be seen from Figures 2 to 5 that the microcell UE transmit much lower (~15 dB for 500 m and ~40 dB for 1732 m) power than the macrocell UE due to the much lower path loss between the UE and microcell BS. Hence significant UE power saving can be achieved with microcell NB-IoT deployment comparing to macrocell deployment.

Moreover, it can be seen from Figures 1 to 3 that adopting the wide area NB-IoT BS receiver dynamic range of 20 dB for microcell NB-IoT BS would provide reasonable assurance of the capability of the receiver to receive a wanted signal in the presence of an interfering signal inside the received channel bandwidth. Further enhancement of the receiver dynamic range of the NB-IoT BS can be considered, but then the required implementation margin should also be considered. Considering the pros and cons, it is proposed (as a trade-off) to adopt the receiver dynamic range of 20 dB (i.e. the same as wide area NB-IoT BS) for microcell NB-IoT BS, and specifying the corresponding requirement with the wanted and interfering signals increased by 5 dB comparing to the wide area NB-IoT BS.

# 3.	Conclusion and proposal

This contribution has provided simulation results for NB-IoT standalone operation in microcell deployment using the agreed assumptions, which show that adopting the medium range E-UTRA BS receiver dynamic range of 20 dB (i.e. the same as wide area NB-IoT BS) for microcell NB-IoT BS would provide reasonable assurance of the capability of the receiver to receive a wanted signal in the presence of an interfering signal inside the received channel bandwidth.

Proposal: To adopt the receiver dynamic range of 20 dB (i.e. the same as wide area NB-IoT BS) for microcell NB-IoT BS, and specifying the corresponding requirement with the wanted and interfering signals increased by 5 dB comparing to the wide area NB-IoT BS.

# References

[1]	RP-170852, “New WID on Further NB-IoT enhancements”, Huawei, HiSilicon, Neul.

[2]	RP-1704047, “WF on new BS classes for NB-IoT”, Ericsson, Nokia, Alcatel-Lucent Shanghai Bell, Huawei.

[3]	3GPP TR 37.809 v11.0.0, “Evolved Universal Terrestrial Radio Access (E-UTRA) medium range and Multi-Standard Radio (MSR) medium range / local area Base Station (BS) class requirements”.

[4]	3GPP TR 36.802 v13.0.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); NB-IOT; Technical Report for BS and UE radio transmission and reception”.

[5]	3GPP TR 36.942 v13.0.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) system scenarios”.