3GPP TSG-WG RAN4 Meeting #83	R4-1705452

# Hangzhou, China,  Date 15th – 19th May, 2017

Source: 	Nokia

Title: 					TP to TR 36.748: Band 72 transmitter requirements

Agenda Item: 			9.2.2

Document for:	Approval

# 1	Introduction

In this contribution we discuss transmitter requriements for band 72. In addition to standard LTE operation MTC and NB-IoT are part of the WI hence those will be discussed as well.

# 2	Discussion

Due to very similar band arrangement with band 31 the transmitter requirements will be very similar for band 72 as those are for band 31.

Maximum output power:

Standard LTE Transmitter output power class will be specified as power class 3 with ± 2 dB tolerances.

UE category M1 Transmitter output power classes will be specified as power classes 3 and 5 with ± 2 dB tolerances.

UE category NB1/NB2 Transmitter output power classes will be specified as power classes 3 and 5 with ± 2 dB tolerances and power class 6 with ± 2.5 dB tolerances.

Spurious emission band UE co-existence:

Spurious emissions towards other 3GPP bands were already agreed in R4-1704068 in RAN4#82bis and are captured below. For UE category M1 and NB1/NB2 standard LTE requirements apply.

| E-UTRA Band |  | Spurious emission |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Protected band |  | Frequency range (MHz) |  |  | Maximum Level (dBm) |  |  |  | MBW (MHz) | NOTE |  |  |
| 1 |  | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 44, 45, 65, 67, 68, 69, [72] |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  | E-UTRA Band 3, 34 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 15 |  |
|  |  | Frequency range |  | 1880 |  |  |  | 1895 |  | -40 |  |  | 1 |  |  | 15, 27 |  |
|  |  | Frequency range |  | 1895 |  |  |  | 1915 |  | -15.5 |  |  | 5 |  |  | 15, 26, 27 |  |
|  |  | Frequency range |  | 1915 |  |  |  | 1920 |  | +1.6 |  |  | 5 |  |  | 15, 26, 27 |  |
| 2 |  | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 66, 70 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  | E-UTRA Band 2, 25 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 15 |  |
|  |  | E-UTRA Band 43 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
| 3 |  | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 39, 40, 41, 43, 44, 45, 65, 67, 68, 69, [72] |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  | E-UTRA Band 3 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 15 |  |
|  |  | E-UTRA Band 11, 18, 19, 21 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 13 |  |
|  |  | E-UTRA Band 22, 42 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  |  | 0.3 |  |  | 13 |  |
| 4 |  | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 41, 43, 48, 66, 70 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  | E-UTRA Band 42 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
| 5 |  | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 65, 66, 70 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  | E-UTRA Band 26 |  | 859 |  | - |  | 869 |  | -27 |  |  | 1 |  |  |  |  |
|  |  | E-UTRA Band 41 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
|  |  | E-UTRA Band 18, 19 |  | FDL_low |  | - |  | FDL_high |  | -40 |  |  | 1 |  |  | 39 |  |
|  |  | E-UTRA Band 11, 21 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 39 |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  |  | 0.3 |  |  | 8, 39 |  |
| 6 |  | E-UTRA Band 1, 9, 11, 34 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 860 |  | - |  | 875 |  | -37 |  |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 875 |  | - |  | 895 |  | -50 |  |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1919.6 |  | -41 |  |  | 0.3 |  |  | 7 |  |
|  |  |  |  | 1884.5 |  | - |  | 1915.7 |  |  |  |  |  |  |  | 8 |  |
| 7 |  | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 20, 22, 26, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 65, 66, 67, 68, [72] |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 |  |  | 5 |  |  | 15, 21, 26 |  |
|  |  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 |  |  | 5 |  |  | 15, 21, 26 |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 |  |  | 1 |  |  | 15, 21 |  |
| 8 |  | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 45, 65, 67, 68, 69, [72] |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  | E-UTRA band 3, 7, 22, 41, 42, 43 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
|  |  | E-UTRA Band 8 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 15 |  |
|  |  | E-UTRA Band 11, 21 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 23 |  |
|  |  | Frequency range |  | 860 |  | - |  | 890 |  | -40 |  |  | 1 |  |  | 15, 23 |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  |  | 0.3 |  |  | 8, 23 |  |
| 9 |  | E-UTRA Band 1, 11, 18, 19, 21, 26, 28, 34 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  | E-UTRA Band 42 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
|  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 1839.9 |  | - |  | 1879.9 |  | -50 |  |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  |  | 0.3 |  |  | 8 |  |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  |  | 1 |  |  |  |  |
| 10 |  | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 41, 43, 66, 70 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  | E-UTRA Band 22, 42 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
| 11 |  | E-UTRA Band 1, 11, 18, 19, 21, 28, 34, 42, 65 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 1839.9 |  | - |  | 1879.9 |  | -50 |  |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  |  | 0.3 |  |  | 8 |  |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  |  | 1 |  |  |  |  |
| 12 |  | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 48, |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  | E-UTRA Band 4, 10, 66, 70 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
|  |  | E-UTRA Band 12 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 15 |  |
| 13 |  | E-UTRA Band 2, 4, 5, 10, 12, 13, 17, 25, 26, 27, 29, 41, 48, 66, 70 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  | E-UTRA Band 14 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 15 |  |
|  |  | E-UTRA Band 24, 30 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
|  |  | Frequency range |  | 769 |  | - |  | 775 |  | -35 |  |  | 0.00625 |  |  | 15 |  |
|  |  | Frequency range |  | 799 |  | - |  | 805 |  | -35 |  |  | 0.00625 |  |  | 11, 15 |  |
| 14 |  | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 23, 24, 25, 26, 27, 29, 30, 41, 48, 66, 70 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 769 |  | - |  | 775 |  | -35 |  |  | 0.00625 |  |  | 12, 15 |  |
|  |  | Frequency range |  | 799 |  | - |  | 805 |  | -35 |  |  | 0.00625 |  |  | 11, 12, 15 |  |
| 17 |  | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 48 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  | E-UTRA Band 4, 10, 66, 70 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
|  |  | E-UTRA Band 12 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 15 |  |
| 18 |  | E-UTRA Band 1, 11, 21, 34, 42, 65 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 758 |  | - |  | 799 |  | -50 |  |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 799 |  | - |  | 803 |  | -40 |  |  | 1 |  |  | 15 |  |
|  |  | Frequency range |  | 860 |  | - |  | 890 |  | -40 |  |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 1839.9 |  | - |  | 1879.9 |  | -50 |  |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  |  | 0.3 |  |  | 8 |  |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  |  | 1 |  |  |  |  |
| 19 |  | E-UTRA Band 1, 11, 21, 28, 34, 42, 65 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 1839.9 |  | - |  | 1879.9 |  | -50 |  |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  |  | 0.3 |  |  | 8 |  |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  |  | 1 |  |  |  |  |
| 20 |  | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 33, 34, 40, 43, 65, 67, [72] |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  | E-UTRA Band 20 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 15 |  |
|  |  | E-UTRA Band 38, 42, 69 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
|  |  | Frequency range |  | 758 |  | - |  | 788 |  | -50 |  |  | 1 |  |  |  |  |
| 21 |  | E-UTRA Band 1, 18, 19, 28, 34, 42, 65 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 1839.9 |  | - |  | 1879.9 |  | -50 |  |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  |  | 0.3 |  |  | 8 |  |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  |  | 1 |  |  |  |  |
| 22 |  | E-UTRA Band 1, 3, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 39, 40, 43, 65, 67, 69, [72] |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 3510 |  | - |  | 3525 |  | -40 |  |  | 1 |  |  | 15 |  |
|  |  | Frequency range |  | 3525 |  | - |  | 3590 |  | -50 |  |  | 1 |  |  |  |  |
| 23 |  | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 23, 24, 26, 27, 29, 30, 41, 66 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
| 24 |  | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 66, 70 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
| 25 |  | E-UTRA Band 4, 5, 10,12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 66, 70 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  | E-UTRA Band 2 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 15 |  |
|  |  | E-UTRA Band 25 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 15 |  |
|  |  | E-UTRA Band 43 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
| 26 |  | E-UTRA Band 1, 2, 3, 4, 5, 10, 11, 12, 13, 14, 17, 18,19, 21, 24, 25, 26, 29, 30, 31, 34, 39, 40, 42, 43, 48, 65, 66, 70 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  | E-UTRA Band 41 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
|  |  | Frequency range |  | 703 |  | - |  | 799 |  | -50 |  |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 799 |  | - |  | 803 |  | -40 |  |  | 1 |  |  | 15 |  |
|  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  |  | 0.3 |  |  | 8 |  |
| 27 |  | E-UTRA Band 1, 2, 3, 4, 5, 7, 10, 12, 13, 14, 17, 25, 26, 27, 29, 30, 31, 38, 40, 41, 42, 43, 65, 66 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  | E-UTRA Band 28 |  | FDL_low |  | - |  | 790 |  | -50 |  |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 799 |  | - |  | 805 |  | -35 |  |  | 0.00625 |  |  |  |  |
| 28 |  | E-UTRA Band 1, 4, 10, 22, 42, 43, 65 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
|  |  | E-UTRA Band 1 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 19, 25 |  |
|  |  | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 20, 25, 26, 27, 31, 34, 38, 40, 41, 66, [72] |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  | E-UTRA Band 11, 21 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 19, 24 |  |
|  |  | Frequency range |  | 470 |  | - |  | 694 |  | -42 |  |  | 8 |  |  | 15, 35 |  |
|  |  | Frequency range |  | 470 |  | - |  | 710 |  | -26.2 |  |  | 6 |  |  | 34 |  |
|  |  | Frequency range |  | 662 |  | - |  | 694 |  | -26.2 |  |  | 6 |  |  | 15 |  |
|  |  | Frequency range |  | 758 |  | - |  | 773 |  | -32 |  |  | 1 |  |  | 15 |  |
|  |  | Frequency range |  | 773 |  | - |  | 803 |  | -50 |  |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  |  | 0.3 |  |  | 8, 19 |  |
| 30 |  | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 38, 41, 48, 66, 70 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
| 31 |  | E-UTRA Band 1, 5, 7, 8, 20, 22, 26, 27, 28, 31, 32, 33, 34, 38, 40, 42, 43, 65, 67, 69 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  | E-UTRA Band 3 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
|  |  | E-UTRA Band [72] |  | FDL_low |  | - |  | FDL_high |  | TBD |  |  | 1 |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 33 |  | E-UTRA Band 1, 7, 8, 20, 22, 28, 31, 32, 34, 38, 40, 42, 43, 65, 67, 69, [72] |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 5 |  |
|  |  | E-UTRA Band 3 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 15 |  |
| 34 |  | E-UTRA Band 1, 3, 7, 8, 11, 18, 19, 20, 21, 22, 26, 28, 31, 32, 33, 38,39, 40, 41, 42, 43, 44, 45, 65, 67, 69, [72] |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 5 |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  |  | 0.3 |  |  | 8 |  |
| 35 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 36 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 37 |  |  |  |  |  | - |  |  |  |  |  |  |  |  |  |  |  |
| 38 |  | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13, 14, 17, 20, 22, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 65, 66, 67, 68, [72] |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 2620 |  | - |  | 2645 |  | -15.5 |  |  | 5 |  |  | 15, 22, 26 |  |
|  |  | Frequency range |  | 2645 |  | - |  | 2690 |  | -40 |  |  | 1 |  |  | 15, 22 |  |
| 39 |  | E-UTRA Band 1, 8, 22, 26, 34, 40, 41, 42, 44, 45 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 1805 |  |  |  | 1855 |  | -40 |  |  | 1 |  |  | 33 |  |
|  |  | Frequency range |  | 1855 |  |  |  | 1880 |  | -15.5 |  |  | 5 |  |  | 15,26,33 |  |
| 40 |  | E-UTRA Band 1, 3, 5, 7, 8, 20, 22, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 42, 43, 44, 45, 65, 67, 68, 69, [72] |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
| 41 |  | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13 , 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44, 45, 48, 65, 66, 70 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  | E-UTRA Band 9, 11, 18, 19, 21 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 30 |  |
|  |  | Frequency range |  | 1884.5 |  |  |  | 1915.7 |  | -41 |  |  | 0.3 |  |  | 8, 30 |  |
| 42 |  | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 11, 18, 19, 20, 21, 25, 26, 27, 28, 31, 32, 33, 34, 38, 40, 41, 44, 45, 65, 66, 67, 69, [72] |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  |  | 0.3 |  |  | 8 |  |
| 43 |  | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 20, 25, 26, 27, 28, 31,32, 33, 34, 38, 40, 65, 66, 67, 69, [72] |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  | E-UTRA Band 22 |  | FDL_low |  | - |  | FDL_high |  | [-50] |  |  | [1] |  |  | 3 |  |
| 44 |  | E-UTRA Band 1, 40, 42, 45 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
|  |  | E-UTRA Band 3, 5, 8, 34, 39, 41 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
| 45 |  | E-UTRA Band 1, 3, 5, 8, 34, 39, 40, 41, 42.44 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 47 |  | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 40, 41, 42, 44, 45, 65, [72] |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  | Frequency range |  | 5925 |  | - |  | 5950 |  | -30 |  |  | 1 |  |  | 38 |  |
|  |  | Frequency range |  | 5815 |  | - |  | 5855 |  | -30 |  |  | 1 |  |  | 38 |  |
|  |  | Frequency range |  | 5795 |  | - |  | 5815 |  | [-65] |  |  | 1 |  |  | 38 |  |
| 48 |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 66, 70 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
| 65 |  | E-UTRA Band 1, 7, 8, 20, 22, 28, 31, 32, 38, 40, 42, 43, 65, 69, [72] |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  | E-UTRA Band 3 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 15 |  |
|  |  | E-UTRA Band 5, 11, 18, 19, 21, 26, 27, 41 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  | E-UTRA Band 34 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 36 |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  |  | 0.3 |  |  | 37 |  |
|  |  | Frequency range |  | 1900 |  | - |  | 1915 |  | -15.5 |  |  | 5 |  |  | 15, 26, 27 |  |
|  |  | Frequency range |  | 1915 |  | - |  | 1920 |  | +1.6 |  |  | 5 |  |  | 15, 26, 27 |  |
| 66 |  | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 66, 70 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  | E-UTRA Band 42, 48 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
| 68 |  | E-UTRA Band 3, 7, 8, 28, 38, 40, [72] |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  | E-UTRA Band 1 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 70 |  | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 66, 70 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| [72] |  | E-UTRA Band 1, 7, 20, 22, 28, 31, 32, 33, 34, 38, 42, 43, 47, 65, 68, [72] |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  | E-UTRA Band 3, 8, 40 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
|  |  | Frequency range |  | 470 |  | - |  | 694 |  | -42 |  |  | 8 |  |  |  |  |

# 3	Conclusion

In this contribution we have discussed band 72 transmitter requirements for standard LTE and UE categories on M! and NB1/NB2 and made attahed TP to TR 36.748.

************************** Start of the TP to TR 36.748. *****************************************

8.3 	UE transmitter requirements

Due to very similar band arrangement with band 31 the transmitter requirements will be very similar for band 72 as those are for band 31.

### 8.3.1	Maximum output power:

Standard LTE Transmitter output power class will be specified as power class 3 with ± 2 dB tolerances.

UE category M1 Transmitter output power classes will be specified as power classes 3 and 5 with ± 2 dB tolerances.

UE category NB1/NB2 Transmitter output power classes will be specified as power classes 3 and 5 with ± 2 dB tolerances and power class 6 with ± 2.5 dB tolerances.

### 8.3.2	Spurious emission band UE co-existence:

Spurious emissions towards other 3GPP bands were already agreed in R4-1704068 in RAN4#82bis and are captured below. For UE category M1 and NB1/NB2 standard LTE requirements apply.

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band |  | Frequency range (MHz) |  | Maximum Level (dBm) |  |  |  | MBW (MHz) | NOTE |  |  |
| 1 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 44, 45, 65, 67, 68, 69, [72] |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  | E-UTRA Band 3, 34 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 15 |  |
|  | Frequency range |  | 1880 |  |  | 1895 |  | -40 |  |  | 1 |  |  | 15, 27 |  |
|  | Frequency range |  | 1895 |  |  | 1915 |  | -15.5 |  |  | 5 |  |  | 15, 26, 27 |  |
|  | Frequency range |  | 1915 |  |  | 1920 |  | +1.6 |  |  | 5 |  |  | 15, 26, 27 |  |
| 2 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 66, 70 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  | E-UTRA Band 2, 25 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 15 |  |
|  | E-UTRA Band 43 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
| 3 | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 39, 40, 41, 43, 44, 45, 65, 67, 68, 69, [72] |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  | E-UTRA Band 3 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 15 |  |
|  | E-UTRA Band 11, 18, 19, 21 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 13 |  |
|  | E-UTRA Band 22, 42 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
|  | Frequency range |  | 1884.5 | - |  | 1915.7 |  | -41 |  |  | 0.3 |  |  | 13 |  |
| 4 | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 41, 43, 48, 66, 70 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  | E-UTRA Band 42 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
| 5 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 65, 66, 70 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  | E-UTRA Band 26 |  | 859 | - |  | 869 |  | -27 |  |  | 1 |  |  |  |  |
|  | E-UTRA Band 41 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
|  | E-UTRA Band 18, 19 |  | FDL_low | - |  | FDL_high |  | -40 |  |  | 1 |  |  | 39 |  |
|  | E-UTRA Band 11, 21 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 39 |  |
|  | Frequency range |  | 1884.5 | - |  | 1915.7 |  | -41 |  |  | 0.3 |  |  | 8, 39 |  |
| 6 | E-UTRA Band 1, 9, 11, 34 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  | Frequency range |  | 860 | - |  | 875 |  | -37 |  |  | 1 |  |  |  |  |
|  | Frequency range |  | 875 | - |  | 895 |  | -50 |  |  | 1 |  |  |  |  |
|  | Frequency range |  | 1884.5 | - |  | 1919.6 |  | -41 |  |  | 0.3 |  |  | 7 |  |
|  |  |  | 1884.5 | - |  | 1915.7 |  |  |  |  |  |  |  | 8 |  |
| 7 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 20, 22, 26, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 65, 66, 67, 68, [72] |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  | Frequency range |  | 2570 | - |  | 2575 |  | +1.6 |  |  | 5 |  |  | 15, 21, 26 |  |
|  | Frequency range |  | 2575 | - |  | 2595 |  | -15.5 |  |  | 5 |  |  | 15, 21, 26 |  |
|  | Frequency range |  | 2595 | - |  | 2620 |  | -40 |  |  | 1 |  |  | 15, 21 |  |
| 8 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 45, 65, 67, 68, 69, [72] |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  | E-UTRA band 3, 7, 22, 41, 42, 43 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
|  | E-UTRA Band 8 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 15 |  |
|  | E-UTRA Band 11, 21 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 23 |  |
|  | Frequency range |  | 860 | - |  | 890 |  | -40 |  |  | 1 |  |  | 15, 23 |  |
|  | Frequency range |  | 1884.5 | - |  | 1915.7 |  | -41 |  |  | 0.3 |  |  | 8, 23 |  |
| 9 | E-UTRA Band 1, 11, 18, 19, 21, 26, 28, 34 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  | E-UTRA Band 42 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
|  | Frequency range |  | 945 | - |  | 960 |  | -50 |  |  | 1 |  |  |  |  |
|  | Frequency range |  | 1839.9 | - |  | 1879.9 |  | -50 |  |  | 1 |  |  |  |  |
|  | Frequency range |  | 1884.5 | - |  | 1915.7 |  | -41 |  |  | 0.3 |  |  | 8 |  |
|  | Frequency range |  | 2545 | - |  | 2575 |  | -50 |  |  | 1 |  |  |  |  |
|  | Frequency range |  | 2595 | - |  | 2645 |  | -50 |  |  | 1 |  |  |  |  |
| 10 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 41, 43, 66, 70 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  | E-UTRA Band 22, 42 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
| 11 | E-UTRA Band 1, 11, 18, 19, 21, 28, 34, 42, 65 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  | Frequency range |  | 945 | - |  | 960 |  | -50 |  |  | 1 |  |  |  |  |
|  | Frequency range |  | 1839.9 | - |  | 1879.9 |  | -50 |  |  | 1 |  |  |  |  |
|  | Frequency range |  | 1884.5 | - |  | 1915.7 |  | -41 |  |  | 0.3 |  |  | 8 |  |
|  | Frequency range |  | 2545 | - |  | 2575 |  | -50 |  |  | 1 |  |  |  |  |
|  | Frequency range |  | 2595 | - |  | 2645 |  | -50 |  |  | 1 |  |  |  |  |
| 12 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 48, |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  | E-UTRA Band 4, 10, 66, 70 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
|  | E-UTRA Band 12 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 15 |  |
| 13 | E-UTRA Band 2, 4, 5, 10, 12, 13, 17, 25, 26, 27, 29, 41, 48, 66, 70 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  | E-UTRA Band 14 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 15 |  |
|  | E-UTRA Band 24, 30 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
|  | Frequency range |  | 769 | - |  | 775 |  | -35 |  |  | 0.00625 |  |  | 15 |  |
|  | Frequency range |  | 799 | - |  | 805 |  | -35 |  |  | 0.00625 |  |  | 11, 15 |  |
| 14 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 23, 24, 25, 26, 27, 29, 30, 41, 48, 66, 70 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  | Frequency range |  | 769 | - |  | 775 |  | -35 |  |  | 0.00625 |  |  | 12, 15 |  |
|  | Frequency range |  | 799 | - |  | 805 |  | -35 |  |  | 0.00625 |  |  | 11, 12, 15 |  |
| 17 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 48 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  | E-UTRA Band 4, 10, 66, 70 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
|  | E-UTRA Band 12 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 15 |  |
| 18 | E-UTRA Band 1, 11, 21, 34, 42, 65 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  | Frequency range |  | 758 | - |  | 799 |  | -50 |  |  | 1 |  |  |  |  |
|  | Frequency range |  | 799 | - |  | 803 |  | -40 |  |  | 1 |  |  | 15 |  |
|  | Frequency range |  | 860 | - |  | 890 |  | -40 |  |  | 1 |  |  |  |  |
|  | Frequency range |  | 945 | - |  | 960 |  | -50 |  |  | 1 |  |  |  |  |
|  | Frequency range |  | 1839.9 | - |  | 1879.9 |  | -50 |  |  | 1 |  |  |  |  |
|  | Frequency range |  | 1884.5 | - |  | 1915.7 |  | -41 |  |  | 0.3 |  |  | 8 |  |
|  | Frequency range |  | 2545 | - |  | 2575 |  | -50 |  |  | 1 |  |  |  |  |
|  | Frequency range |  | 2595 | - |  | 2645 |  | -50 |  |  | 1 |  |  |  |  |
| 19 | E-UTRA Band 1, 11, 21, 28, 34, 42, 65 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  | Frequency range |  | 945 | - |  | 960 |  | -50 |  |  | 1 |  |  |  |  |
|  | Frequency range |  | 1839.9 | - |  | 1879.9 |  | -50 |  |  | 1 |  |  |  |  |
|  | Frequency range |  | 1884.5 | - |  | 1915.7 |  | -41 |  |  | 0.3 |  |  | 8 |  |
|  | Frequency range |  | 2545 | - |  | 2575 |  | -50 |  |  | 1 |  |  |  |  |
|  | Frequency range |  | 2595 | - |  | 2645 |  | -50 |  |  | 1 |  |  |  |  |
| 20 | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 33, 34, 40, 43, 65, 67, [72] |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  | E-UTRA Band 20 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 15 |  |
|  | E-UTRA Band 38, 42, 69 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
|  | Frequency range |  | 758 | - |  | 788 |  | -50 |  |  | 1 |  |  |  |  |
| 21 | E-UTRA Band 1, 18, 19, 28, 34, 42, 65 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  | Frequency range |  | 945 | - |  | 960 |  | -50 |  |  | 1 |  |  |  |  |
|  | Frequency range |  | 1839.9 | - |  | 1879.9 |  | -50 |  |  | 1 |  |  |  |  |
|  | Frequency range |  | 1884.5 | - |  | 1915.7 |  | -41 |  |  | 0.3 |  |  | 8 |  |
|  | Frequency range |  | 2545 | - |  | 2575 |  | -50 |  |  | 1 |  |  |  |  |
|  | Frequency range |  | 2595 | - |  | 2645 |  | -50 |  |  | 1 |  |  |  |  |
| 22 | E-UTRA Band 1, 3, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 39, 40, 43, 65, 67, 69, [72] |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  | Frequency range |  | 3510 | - |  | 3525 |  | -40 |  |  | 1 |  |  | 15 |  |
|  | Frequency range |  | 3525 | - |  | 3590 |  | -50 |  |  | 1 |  |  |  |  |
| 23 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 23, 24, 26, 27, 29, 30, 41, 66 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
| 24 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 66, 70 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
| 25 | E-UTRA Band 4, 5, 10,12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 66, 70 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  | E-UTRA Band 2 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 15 |  |
|  | E-UTRA Band 25 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 15 |  |
|  | E-UTRA Band 43 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
| 26 | E-UTRA Band 1, 2, 3, 4, 5, 10, 11, 12, 13, 14, 17, 18,19, 21, 24, 25, 26, 29, 30, 31, 34, 39, 40, 42, 43, 48, 65, 66, 70 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  | E-UTRA Band 41 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
|  | Frequency range |  | 703 | - |  | 799 |  | -50 |  |  | 1 |  |  |  |  |
|  | Frequency range |  | 799 | - |  | 803 |  | -40 |  |  | 1 |  |  | 15 |  |
|  | Frequency range |  | 945 | - |  | 960 |  | -50 |  |  | 1 |  |  |  |  |
|  | Frequency range |  | 1884.5 | - |  | 1915.7 |  | -41 |  |  | 0.3 |  |  | 8 |  |
| 27 | E-UTRA Band 1, 2, 3, 4, 5, 7, 10, 12, 13, 14, 17, 25, 26, 27, 29, 30, 31, 38, 40, 41, 42, 43, 65, 66 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  | E-UTRA Band 28 |  | FDL_low | - |  | 790 |  | -50 |  |  | 1 |  |  |  |  |
|  | Frequency range |  | 799 | - |  | 805 |  | -35 |  |  | 0.00625 |  |  |  |  |
| 28 | E-UTRA Band 1, 4, 10, 22, 42, 43, 65 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
|  | E-UTRA Band 1 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 19, 25 |  |
|  | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 20, 25, 26, 27, 31, 34, 38, 40, 41, 66, [72] |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  | E-UTRA Band 11, 21 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 19, 24 |  |
|  | Frequency range |  | 470 | - |  | 694 |  | -42 |  |  | 8 |  |  | 15, 35 |  |
|  | Frequency range |  | 470 | - |  | 710 |  | -26.2 |  |  | 6 |  |  | 34 |  |
|  | Frequency range |  | 662 | - |  | 694 |  | -26.2 |  |  | 6 |  |  | 15 |  |
|  | Frequency range |  | 758 | - |  | 773 |  | -32 |  |  | 1 |  |  | 15 |  |
|  | Frequency range |  | 773 | - |  | 803 |  | -50 |  |  | 1 |  |  |  |  |
|  | Frequency range |  | 1884.5 | - |  | 1915.7 |  | -41 |  |  | 0.3 |  |  | 8, 19 |  |
| 30 | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 38, 41, 48, 66, 70 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
| 31 | E-UTRA Band 1, 5, 7, 8, 20, 22, 26, 27, 28, 31, 32, 33, 34, 38, 40, 42, 43, 65, 67, 69 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  | E-UTRA Band 3 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
|  | E-UTRA Band [72] |  | FDL_low | - |  | FDL_high |  | TBD |  |  | 1 |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 33 | E-UTRA Band 1, 7, 8, 20, 22, 28, 31, 32, 34, 38, 40, 42, 43, 65, 67, 69, [72] |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 5 |  |
|  | E-UTRA Band 3 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 15 |  |
| 34 | E-UTRA Band 1, 3, 7, 8, 11, 18, 19, 20, 21, 22, 26, 28, 31, 32, 33, 38,39, 40, 41, 42, 43, 44, 45, 65, 67, 69, [72] |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 5 |  |
|  | Frequency range |  | 1884.5 | - |  | 1915.7 |  | -41 |  |  | 0.3 |  |  | 8 |  |
| 35 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 36 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 37 |  |  |  | - |  |  |  |  |  |  |  |  |  |  |  |
| 38 | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13, 14, 17, 20, 22, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 65, 66, 67, 68, [72] |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  | Frequency range |  | 2620 | - |  | 2645 |  | -15.5 |  |  | 5 |  |  | 15, 22, 26 |  |
|  | Frequency range |  | 2645 | - |  | 2690 |  | -40 |  |  | 1 |  |  | 15, 22 |  |
| 39 | E-UTRA Band 1, 8, 22, 26, 34, 40, 41, 42, 44, 45 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  | Frequency range |  | 1805 |  |  | 1855 |  | -40 |  |  | 1 |  |  | 33 |  |
|  | Frequency range |  | 1855 |  |  | 1880 |  | -15.5 |  |  | 5 |  |  | 15,26,33 |  |
| 40 | E-UTRA Band 1, 3, 5, 7, 8, 20, 22, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 42, 43, 44, 45, 65, 67, 68, 69, [72] |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
| 41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13 , 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44, 45, 48, 65, 66, 70 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 30 |  |
|  | Frequency range |  | 1884.5 |  |  | 1915.7 |  | -41 |  |  | 0.3 |  |  | 8, 30 |  |
| 42 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 11, 18, 19, 20, 21, 25, 26, 27, 28, 31, 32, 33, 34, 38, 40, 41, 44, 45, 65, 66, 67, 69, [72] |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  | Frequency range |  | 1884.5 | - |  | 1915.7 |  | -41 |  |  | 0.3 |  |  | 8 |  |
| 43 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 20, 25, 26, 27, 28, 31,32, 33, 34, 38, 40, 65, 66, 67, 69, [72] |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  | E-UTRA Band 22 |  | FDL_low | - |  | FDL_high |  | [-50] |  |  | [1] |  |  | 3 |  |
| 44 | E-UTRA Band 1, 40, 42, 45 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
|  | E-UTRA Band 3, 5, 8, 34, 39, 41 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
| 45 | E-UTRA Band 1, 3, 5, 8, 34, 39, 40, 41, 42.44 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 47 | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 40, 41, 42, 44, 45, 65, [72] |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  | Frequency range |  | 5925 | - |  | 5950 |  | -30 |  |  | 1 |  |  | 38 |  |
|  | Frequency range |  | 5815 | - |  | 5855 |  | -30 |  |  | 1 |  |  | 38 |  |
|  | Frequency range |  | 5795 | - |  | 5815 |  | [-65] |  |  | 1 |  |  | 38 |  |
| 48 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 66, 70 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
| 65 | E-UTRA Band 1, 7, 8, 20, 22, 28, 31, 32, 38, 40, 42, 43, 65, 69, [72] |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  | E-UTRA Band 3 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 15 |  |
|  | E-UTRA Band 5, 11, 18, 19, 21, 26, 27, 41 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  | E-UTRA Band 34 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 36 |  |
|  | Frequency range |  | 1884.5 | - |  | 1915.7 |  | -41 |  |  | 0.3 |  |  | 37 |  |
|  | Frequency range |  | 1900 | - |  | 1915 |  | -15.5 |  |  | 5 |  |  | 15, 26, 27 |  |
|  | Frequency range |  | 1915 | - |  | 1920 |  | +1.6 |  |  | 5 |  |  | 15, 26, 27 |  |
| 66 | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 66, 70 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  | E-UTRA Band 42, 48 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
| 68 | E-UTRA Band 3, 7, 8, 28, 38, 40, [72] |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  | E-UTRA Band 1 |  | FDL_low | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 70 |  | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 66, 70 | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| [72] |  | E-UTRA Band 1, 7, 20, 22, 28, 31, 32, 33, 34, 38, 42, 43, 47, 65, 68, [72] | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |
|  |  | E-UTRA Band 3, 8, 40 | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |
|  |  | Frequency range | 470 |  | - |  | 694 |  | -42 |  |  | 8 |  |  |  |

************************** End of the TP to TR 36.748. *****************************************