3GPP TSG-RAN WG4 Meeting #83 	R4-1705496

# Hangzhou, China, 15 - 19 May, 2017

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Simulation results for eLAA PUSCH

Agenda Item:	7.14.7

Document for:	Discussion

# Introduction

The scope of eLAA performance work according to the latest WID is to Specify the necessary UE and base station performance requirements to support UL carrier aggregation for an LAA SCell using Frame Structure type 3 [1]. In the Athens RAN4 meeting, a way forward on eLAA demodulation was agreed in [2]. Furthermore, a set of initial simulation assumptions was agreed in [3]. In the last meeting further agreements were made on channel model, modulation and coding schemes, and ending symbol configuration. In this contribution we provide simulation results based on the agreed simulation assumptions.

# Simulation assumptions

Simulation assumptions as agreed in [3] and based on the further agreements in Spokane meeting are listed in Table 1.

Table 1: Parameter setup for PUSCH Performance requirements

| Parameter | Value |
| --- | --- |
| Carrier frequency | 5 GHz |
| System bandwidth for LAA SCell | 20 MHz |
| Channel model | EPA 5Hz |
| Antenna setup | 1Tx, 2Rx and 4Rx LOW |
| Number of interlaces | TBD |
| Number of PRBs per interlace | 10 |
| PUSCH frequency hopping | No |
| Starting PUSCH symbol configuration | 01 |
| Ending symbol configuration | Up to OFDM symbol 13 |
| MCS | QPSK 1/3, 16QAM ¾ |
| CP type | Normal |
| Receiver noise | AWGN |
| Number of user | 1 |
| Resource allocation | contiguous |
| Frequency error | 0 |
| Time error | 0 |
| Interference | no |
| Reference Receiver | MRC |
| Performance Metric | SNR values [dB] for which 70% of maximum throughput is achieved |
| Max number of HARQ Transmissions | 4 |
| LBT model | No |

# Simulation results

In the last meeting, it was proposed that eLAA performance tests should test multiple interlace allocation. In Table 2 we have provided simulation results for eLAA PUSCH with 1, 2 and 5 contiguous interlaces.

Table 2: eLAA PUSCH simulation results for 1 interlace allocation.

| SNR [dB] |  |  | 1 interlace | 2 interlaces (contiguous) | 5 interlaces (contiguous) |
| --- | --- | --- | --- | --- | --- |
| 70 % TP, EPA5 | 2 Rx | QPSK (MCS 5) | -2.37 | -2.72 | -2.75 |
|  |  | 16QAM (MCS 19) | 10.18 | 10.15 | 10.06 |
|  |  | 64QAM (MCS 27) | 16.73 | 16.81 | 16.83 |
|  | 4 Rx | QPSK (MCS 5) | -5.52 | -5.85 | -5.91 |
|  |  | 16QAM (MCS 19) | 6.08 | 6.02 | 5.88 |
|  |  | 64QAM (MCS 27) | 12.46 | 12.53 | 12.56 |

As can be seen from the 70 % throughput SNR results, 1, 2 and 5 interlace allocations have very minor differences in performance. Thus we do not see a need to include tests for multiple interlace allocations, but choose one number of interlaces for the test. We see that 1 interlace allocation should be sufficient for eLAA PUSCH demodulation tests.

Proposal 1: Use 1 interlace allocation in eLAA demodulation test setup.

As discussed in the previous meetings, we do not think LBT modelling needs to be included in the test. In uplink PUSCH, from the demodulation perspective, the impact of LBT is handled in a similar way as the situation where UE misses UL grant and does not transmit because of that. Difference may be in the granularity of UL signals missing, but we do not see that this would impact eNodeB implementation. Thus, we see that the simulation assumptions that have been agreed can be used as final.

Proposal 2: Do not include LBT modelling in eLAA demodulation tests.

# Summary

In this contribution we have provided simulation results for eLAA PUSCH based on the simulation assumptions agreed in the previous meetings. We have made the following proposals based on the simulation results:

Proposal 1: Use 1 interlace allocation in eLAA demodulation test setup.

Proposal 2: Do not include LBT modelling in eLAA demodulation tests.

# References

[1] RP-161832, Work Item on enhanced LAA for LTE, 3GPP TSG RAN Meeting #73, New Orleans, September 2016.

[2] R4-1701086, Way forward on eLAA demodulation performance, 3GPP TSG-RAN WG4 Meeting #82, Athens, Greece, February 2017.

[3] R4-1700990, Simulation assumption for eLAA PUSCH demodulation, Ericsson, 3GPP TSG-RAN WG4 #82, Athens, Greece, February 2017.