3GPP TSG-RAN WG4 Meeting #83 												      R4- 1705620

Hangzhou, China, 15 - 19 May, 2017

Source: 	Huawei, Hisilicon

Title: 	TDD L-Band - Channel numbering for E-UTRA, MSR

Agenda Item:	9.6.1

Document for:	Approval

# Introduction

A work item [1] was agreed to define a new TDD band covering 1427 – 1518 MHz.

This contribution is to provide Channel numbering

# Discussion

## Background

The frequency band 1427-1518 MHz was identified for IMT in WRC-15, named as L-band.

Based on the approval at the 3GPP RAN4 meeting#82-bis in Spokane, US, March 3 to 6 2017, the TDD L-Band arrangement is aligned on the SDL L-Band arrangement. Thus, there is only a TDD L-band challenging arrangements, as follow:

Figure 1 - TDD Arrangement (a)

### The objective of SI or Core part WI or Testing part WI

The objectives of this work item is the following:

- Study the feasible channelling arrangement(s) of L-band (1427 – 1518 MHz) considering the above ITU options and relevant protection requirements of EESS below 1 427 MHz and MSS above 1518 MHz, considering the results of relevant ongoing studies, including:

  - TDD: 90 MHz (1427 – 1517 MHz), considering 1 MHz GB with MSS at the upper edge,

  - Note: the upper edge refinement is based on results of ongoing compatibility studies with MSS. For example option of 88 MHz (1427 – 1515 MHz) considers 3 MHz GB with MSS at the upper edge.

- Specify performance requirements for the feasible plan, band numbering and core requirements, as necessary.

- Support of 3, 5, 10, 15 and 20 MHz channel bandwidths

- Support of 200 KHz and 1.4 MHz for NB-IoT and eMTC, respectively, for development of Category M1 and NB1.

## Channel numbering for E-UTRA

For 1427-1517MHz Band the E-UTRA frequency band is defined as in Table X-1.

Table X-1 E-UTRA frequency bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 49 | 1427 MHz | – | 1517 MHz | 1427 MHz | – | 1517 MHz | TDD |

EARFCN for 1427-1517MHz Band is defined as in Table X-2.

Table X-2: EARFCN allocated for E-UTRA Band [49]

| E-UTRA Operating Band | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FDL_low [MHz] | NOffs-DL | Range of NDL | FUL_low [MHz] | NOffs-UL | Range of NUL |
| 49 | 1427 | 56740 | 56740 - 57639 | 1427 | 56740 | 56740 - 57639 |

# Conclusion

Proposal 1: Add the table above “Table X-1 E-UTRA frequency bands” and “Table X-2: EARFCN allocated for E-UTRA Band 49” in the TR [1]

.

# References

[1] RP-162557 “New E-UTRAN L-band plan 1427-1518MHz”, Etisalat, Huawei, HiSilicon

[2] R4-1704099_WF on UE and BS requirements in FDD L-band

[3] ECC Report 263, “Adjacent band compatibility studies between IMT operating in the frequency band 1492-1518 MHz and the MSS operating in the frequency band 1518-1525 MHz”;

[4] ITU-R Report M.2292 “Characteristics of terrestrial IMT-Advanced systems for frequency sharing/interference analyses”

[5] R4-1704098- Band plan discussion 1 5 GHz TDD

[6] TR 36.892 - 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Introduction of 1447-1467MHz Band for TD-LTE in China

[7] R4-1703068 - UE reference sensitivity