3GPP TSG-RAN WG4 Meeting #83 												      R4- 1705621

Hangzhou, China, 15 - 19 May, 2017

Source: 	Huawei, Hisilicon

Title: 	Study about TDD L-Band - UE reference sensitivity power level

Agenda Item:	9.6.2

Document for:	Approval

# Introduction

A work item [1] was agreed to define a new TDD band covering 1427 – 1518 MHz.

This contribution is to provide a reference sensitivity power level from the TR 36.892, already approved by3GPP

# Discussion

## Background

The frequency band 1427-1518 MHz was identified for IMT in WRC-15, named as L-band.

Based on the approval at the 3GPP RAN4 meeting#82-bis in Spokane, US, March 3 to 6 2017, the TDD L-Band arrangement is aligned on the SDL L-Band arrangement. Thus, there is only a TDD L-band challenging arrangements, as follow:

Figure 1 - TDD Arrangement (a)

### The objective of SI or Core part WI or Testing part WI

The objectives of this work item is the following:

- Study the feasible channelling arrangement(s) of L-band (1427 – 1518 MHz) considering the above ITU options and relevant protection requirements of EESS below 1 427 MHz and MSS above 1518 MHz, considering the results of relevant ongoing studies, including:

  - TDD: 90 MHz (1427 – 1517 MHz), considering 1 MHz GB with MSS at the upper edge,

  - Note: the upper edge refinement is based on results of ongoing compatibility studies with MSS. For example option of 88 MHz (1427 – 1515 MHz) considers 3 MHz GB with MSS at the upper edge.

- Specify performance requirements for the feasible plan, band numbering and core requirements, as necessary.

- Support of 3, 5, 10, 15 and 20 MHz channel bandwidths

- Support of 200 KHz and 1.4 MHz for NB-IoT and eMTC, respectively, for development of Category M1 and NB1.

## UE reference sensitivity power level

In the TR 36.892, there is the paragraph 8.2:

The 1.4GHz band is a TDD band without simultaneous Rx/Tx hence no Rx/Tx isolation requirement is needed. The bandwidth is 20MHz and BW/F0 equals to 1.37% which will not bring difficulty for implementation. The reported insertion loss values from different filter vendors are shown in Table 8.2-1. The proposed reference sensitivity is shown in Table 8.2-2.

Table 8.2-1: IL values for

|  | Typ. IL (dB) | ETC. IL (dB) |
| --- | --- | --- |
| Vendor 1 | 1.2 | 1.9 |
| Vendor 2 | 1.6 | 2.1 |

Table 8.2-2: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 45 |  |  | -100 | -97 | -95.2 | -94 | TDD |

And also the contribution “R4-1703068” [7]:

Table 1: Insertion Loss

|  | Typ. IL (dB) | Note |
| --- | --- | --- |
| Vendor 1 | 1.37 |  |
| Vendor 2 | 2.4 | with 2016 technologies |
|  | 1.6 | with 2018 technologies |

Table 2: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| [TBD] | - | -102.2 | -100 | -97 | -95.2 | -94 | TDD |

# Conclusion

Proposal 1: Merge the table form R4-1703068 [7] and the paragraph 8.2 of the TR 36.892:

The 1.4GHz band is a TDD band without simultaneous Rx/Tx hence no Rx/Tx isolation requirement is needed. The bandwidth is 20MHz and BW/F0 equals to 1.37% which will not bring difficulty for implementation. The reported insertion loss values from different filter vendors are shown in Table X-1. The proposed reference sensitivity is shown in Table X-2.

Table X-1: IL values for

|  | Typ. IL (dB) | ETC. IL (dB) |
| --- | --- | --- |
| Vendor 1 | 1.2 | 1.9 |
| Vendor 2 | 1.6 | 2.1 |
| Vendor 3 | 1.37 |  |
| Vendor 4 | 2.4 | with 2016 technologies |
|  | 1.6 | with 2018 technologies |

Table X-2: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 45 |  | -102.2 | -100 | -97 | -95.2 | -94 | TDD |

Proposal 2: Add to the TR [1] the paragraph of the proposal 1

.

# References

[1] RP-162557 “New E-UTRAN L-band plan 1427-1518MHz”, Etisalat, Huawei, HiSilicon

[2] R4-1704099_WF on UE and BS requirements in FDD L-band

[3] ECC Report 263, “Adjacent band compatibility studies between IMT operating in the frequency band 1492-1518 MHz and the MSS operating in the frequency band 1518-1525 MHz”;

[4] ITU-R Report M.2292 “Characteristics of terrestrial IMT-Advanced systems for frequency sharing/interference analyses”

[5] R4-1704098- Band plan discussion 1 5 GHz TDD

[6] TR 36.892 - 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Introduction of 1447-1467MHz Band for TD-LTE in China

[7] R4-1703068 - UE reference sensitivity